3GPP TSG-RAN5 Meeting #96-e                      	R5-225162

# Electronic Meeting, August 15 - 26 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2659 | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of 7 DL CA Activation and Deactivation of Unknown SCell Test Case 8.16.102 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RRM test case 8.16.96 is incomplete in TS 36.521-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRM test case 8.16.96 has been updated, following items has been added/corrected: Test Tolerance, test procedure, message content and cell configuration were added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM test case 8.16.96 will remain incomplete in TS 36.521-3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.16.96 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

### 8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

#### 8.16.102.1	Test purpose

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with six downlink SCells, when the SCell is unknown by the UE at the time of activation.

#### 8.16.102.2	Test applicability

This test case applies to all types of E-UTRA FDD UE release 14 and forward that support 7 DL CA. Applicability requires support for FGI bit 111.

This test case also applies to all types of E-UTRA TDD UE release 14 and forward that support 7 DL CA. Applicability requires support for FGI bit 111.

This test case also applies to all types of E-UTRA UE release 14 and forward that support E-UTRA FDD, TDD and 7 DL CA. Applicability requires support for FGI bit 111.

#### 8.16.102.3	Minimum conformance requirements

TBD

#### 8.16.102.4	Test description

##### 8.16.102.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-2 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1 for different CA bandwidth classes.

Channel Bandwidth to be tested: The largest aggregated bandwidth combination supported by UE from Table 8.16.87.5-1 as defined in 3GPP TS 36.508 [7] clause 4.3.1, for different CA configurations as defined in 3GPP TS 36.521-1 [10] clause 5.4.2A.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as follows:

-	For UEs supporting only 2Rx in all the bands under test, use TS 36.508 [7] Annex A, Figure group A.63 as appropriate.

-	For UEs supporting 4Rx in any of the bands under test use TS 36.508 [7] Annex A, Figure A.90, without intra-frequency neighbours. For PCell, use the 4Rx module if the UE supports 4Rx for the corresponding band and the 2Rx module otherwise. For all other cells, the 2Rx module can be used.

2.	The general test parameter settings are set up according to Table 8.16.87.4.1-1.

3.	Propagation conditions are set according to Annex B clauses B.0.

4.	Message contents are defined in clause 8.16.87.4.3.

5.	Cell 1 is the PCell on the primary component carrier, Cell 2 is SCell on RF channel number 2, Cell 3 is the deactivated SCell on RF channel number 3, Cell 4 is the deactivated SCell on RF channel number 4 and Cell 5 is the deactivated SCell on RF channel number 5, Cell 6 is the deactivated SCell on RF channel number 6, Cell 7 is the deactivated SCell on RF channel number 7. Cell 1 is the cell used for connection setup with the power levels set according to Annex C.0 and C.1 for this test. Cell 2, Cell 3, Cell 4, Cell 5, Cell 6 and Cell 7 shall be powered OFF.

Table 8.16.102.4.1-1: General test parameters for unknown SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6, 7 | Seven radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell4 |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Configured deactivated SCell5 |  | Cell 6 | Configured deactivated secondary cell on RF channel number 6. |
| Configured deactivated SCell6 |  | Cell 7 | Configured deactivated secondary cell on RF channel number 7. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| Cell-individual offset for cells on RF channel number 6 | dB | 0 | Individual offset for cells on SCC5. |
| Cell-individual offset for cells on RF channel number 7 | dB | 0 | Individual offset for cells on SCC6. |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell, SCell2, SCell3, SCell4, SCell5 and SCell6 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, SCell3, SCell4, SCell5 and SCell6. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, SCell3, SCell4, SCell5 and SCell6. |

##### 8.16.102.4.2	Test procedure

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. All cells have constant signal levels throughout the test.

Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), Cell 3 (deactivated SCell2) on radio channel 3 (SCC2), Cell 4 (deactivated SCell3) on radio channel 4 (SCC3), Cell 5 (deactivated SCell4) on radio channel 5 (SCC4), Cell 6 (deactivated SCell5) on radio channel 6 (SCC5) and Cell 7 (deactivated SCell6) on radio channel 7 (SCC6) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC, SCC2, SCC3, SCC4, SCC5 and SCC6. The UE shall be continuously scheduled in the PCell throughout the whole test.

There are seven carriers, each with one cell. Cell 1, Cell 2, Cell 3, Cell 4, Cell 5, Cell 6 and Cell 7 may operate in either FDD or TDD duplex mode according to test configuration.

1.	Ensure the UE is in State 3A-RF according to 3GPP TS 36.508 [7] clause 7.2A.3.

2.	Configure SCC2, SCC3, SCC4, SCC5 and SCC6 according to Annex C.0, C.1 for all downlink physical channels.

3.	The SS shall configure SCell2 (Cell 3) on the SCC2, SCell3 (Cell 4) on the SCC3, SCell4 (Cell 5) on the SCC4, SCell5 (Cell 6) on the SCC5 and SCell6 (Cell 7) on the SCC6 as per TS 36.508 [7] clause 5.2A.4 with the message content exceptions defined in clause 8.16.94.4.3.

4.	The SS shall configure transmission of PDSCH with a maximum number of 1 HARQ transmission.

5.	Set the parameters according to T1 in Table 8.16.102.5-1. Propagation conditions are set according to Annex B clauses B.1.1. T1 starts. The SS shall configure SCell1 (Cell 2) on the SCC1 as per TS 36.508 [7] clause 5.2A.4 with the message content exceptions defined in clause 8.16.102.4.3.

6.	The SS activates SCell1 by sending the activation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8) in a subframe # denoted m, where m is an even number for FDD PCell and 4 or 9 for TDD PCell. If the SS receives ACK for MAC-CE sent by the UE, T2 starts in subframe m, and the test proceeds to step 7, otherwise go to step 12.

7.	The SS activates SCell2 in subframe (m+10) for FDD PCell and (m+15) for TDD PCell, SCell 3 in subframe (m+20) for FDD PCell and (m+15) for TDD PCell, SCell 4 in subframe (m+30) for FDD PCell and (m+15) for TDD PCell, SCell 5 in subframe (m+40) for FDD PCell and (m+15) for TDD PCell and SCell 6 in subframe (m+50) for FDD PCell and (m+15) for TDD PCell by sending the activation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8).

8.	The UE shall start sending CSI reports for SCell1 and the SS shall monitor CSI reports for SCell1 sent from the UE and ACK/NACK sent in PCell during SCells activation.

8a.	For all intra-band PCell FDD case

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+10) if the subframe (m+8) was subject to interruption

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+59),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+59)

-	and DTX is not observed by the SS anywhere outside the subframes (m+5) to (m+9), the subframes (m+15) to (m+19), the subframes (m+25) to (m+29), the subframes (m+35) to (m+39), the subframes (m+45) to (m+49) and the subframes (m+55) to (m+59) up to the end of T2

-	Then the number of successes for the event "Activation" is increased by one. Otherwise, count a fail for the event "Activation" and go to step 12.

8b.	For the case where the band relation between PCC(FDD) and SCells are inter-band

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+10) if the subframe (m+8) was subject to interruption,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+59),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+59)

-	and DTX is not observed by the SS anywhere outside the subframes (m+5) to (m+9), the subframes (m+15) to (m+19), the subframes (m+25) to (m+29), the subframes (m+35) to (m+39), the subframes (m+45) to (m+49) and the subframes (m+55) to (m+59)  up to the end of T2,

-	and ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (m+5) to (m+13), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (m+15) to (m+23), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (m+25) to (m+33), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (m+35) to (m+43), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (m+45) to (m+53) and ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (m+55) to (m+63)  up to the end of T2,

-	Then the number of successes for the event "Activation" is increased by one. Otherwise, count a fail for the event "Activation" and go to step 12.

8c.	For the case where the band relation between PCC(FDD) and SCell1 is intra-band and the others SCells are inter-band

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+10) if the subframe (m+8) was subject to interruption

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+59),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+59)

-	and DTX is not observed by the SS anywhere outside the subframes (m+5) to (m+9), the subframes (m+15) to (m+19), the subframes (m+25) to (m+29), the subframes (m+35) to (m+39), the subframes (m+45) to (m+49) and the subframes (m+55) to (m+59)  up to the end of T2,

-	and ≤2 non-consecutive DTX subframes are observed by the SS during the subframes (m+15) to (m+23),  ≤2 non-consecutive DTX subframes are observed by the SS during the subframes (m+25) to (m+33), ≤2 non-consecutive DTX subframes are observed by the SS during the subframes (m+35) to (m+43), ≤2 non-consecutive DTX subframes are observed by the SS during the subframes (m+45) to (m+53) and ≤2 non-consecutive DTX subframes are observed by the SS during the subframes (m+55) to (m+63) up to the end of T2,

-	Then the number of successes for the event "Activation" is increased by one. Otherwise, count a fail for the event "Activation" and go to step 12.

8d.	For PCell TDD case

-	If the first CSI report for SCell1 is received by the SS in a subframe (m+8),

-	or (m+9) if the subframe (m+8) was subject to interruption,

-	or in a subframe (m+13) if the subframes (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to subframe (m+59),

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a subframe (m+59).

-	and DTX is not observed by the SS outside the subframes (m+5) to (m+11) and the subframes (m+20) to (m+26)

-	Then the number of successes for the event "Activation" is increased by one. Otherwise, count a fail for the event "Activation" and go to step 12.

9.	When T2 expires, the SS deactivate SCell1 by sending the deactivation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8) in a subframe # denoted n, where n is an even number for FDD PCell and 4 or 9 for TDD PCell. If the SS receives ACK for MAC-CE sent by the UE, T3 starts in subframe n, and the test proceeds to step 10, otherwise go to step 12.

10.	The SS deactivates SCell2 in subframe (n+10) for FDD PCell and (n+15) for TDD PCell, SCell 3 in subframe (n+20) for FDD PCell and (n+30) for TDD PCell, SCell 4 in subframe (n+30) for FDD PCell and (n+45) for TDD PCell, SCell 5 in subframe (n+40) for FDD PCell and (n+60) for TDD PCell and SCell 6 in subframe (n+50) for FDD PCell and (n+75) for TDD PCell by sending the deactivation MAC-CE (Refer TS 36.321 [26], clauses 5.13, 6.1.3.8).

11.	The UE shall stop sending CSI reports for SCell1 and the SS shall monitor CSI reports for SCell1 sent from the UE and ACK/NACK sent in PCell during SCell1 deactivation.

11a.	For all intra-band PCell FDD case

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS anywhere outside the subframes (n+5) to (n+9), the subframes (n+15) to (n+19), the subframes (n+25) to (n+29), (n+35) to (n+39), (n+45) to (n+49) and (n+55) to (n+59), up to the end of T3

-	Then the number of successes for the event "Deactivation" is increased by one. Otherwise, count a fail for the event "Deactivation".

11b.	For the case where the band relation between PCC(FDD) and SCells are inter-band

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS anywhere outside the subframes (n+5) to (n+9), the subframes (n+15) to (n+19), the subframes (n+25) to (n+29), (n+35) to (n+39), (n+45) to (n+49) and (n+55) to (n+59), up to the end of T3

-	and ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+5) to (n+9), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+15) to (n+19), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+25) to (n+29), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+35) to (n+39),  ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+45) to (n+49) and  ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+55) to (n+59) up to the end of T3

-	Then the number of successes for the event "Deactivation" is increased by one. Otherwise, count a fail for the event "Deactivation".

11c.	For the case where the band relation between PCC(FDD) and SCell1 is intra-band and the others SCells are inter-band

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS anywhere outside the subframes (n+5) to (n+9), the subframes (n+15) to (n+19), the subframes (n+25) to (n+29), (n+35) to (n+39), (n+45) to (n+49) and (n+55) to (n+59), up to the end of T3

-	and ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+5) to (n+9), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+15) to (n+19), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+25) to (n+29), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+35) to (n+39), ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+45) to (n+49) ≤2 non-consecutive DTX subframes are observed by the SS anywhere during the subframes (n+55) to (n+59) up to the end of T3

-	Then the number of successes for the event "Deactivation" is increased by one. Otherwise, count a fail for the event "Deactivation".

11d.	For PCell TDD case

-	If the last CSI report is received by the SS earlier than or equal to subframe (n+8),

-	and DTX is not observed by the SS anywhere outside the subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41), (n+50) to (n+56) and (n+65) to (n+71) and (n+80) to (n+86) up to the end of T3,

-	Then the number of successes for the event "Deactivation" is increased by one. Otherwise, count a fail for the event "Deactivation".

11e.	TBD

12.	When T3 expires, or Activation in step 6 was not acknowledged, or a fail was counted for the event "Activation" in step 8, or Deactivation in step 9 was not acknowledged, the SS shall transmit a RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

13.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

14.	After the RRC connection release, the SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to 3GPP TS 36.508 [7] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to 3GPP TS 36.508 [7] clause 7.2A.3); or

-	switches off and on the UE and ensures the UE is in State 3A-RF according to 3GPP TS 36.508 [7] clause 7.2A.3.

15.	Repeat steps 2-14 until a test verdict has been achieved.

##### 8.16.102.4.3	Message contents

Same message contents as in 8.16.94.4.3 with the following exceptions:

Table 8.16.102.4.3-1: MeasObjectEUTRA-GENERIC: Additional 5DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

| Derivation Path: 3GPP TS 36.508 [7] clause 4.6.6 Table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| cellsToAddModList SEQUENCE (SIZE (1..maxCellMeas)) { } | Not present |  |  |
| measCycleSCell-r10 | sf320 |  | Cell 2, Cell 3, Cell 4, Cell 5, Cell 6, Cell 7 |
| } |  |  |  |

#### 8.16.102.5	Test requirement

Table 8.16.102.5-1, table 8.16.102.5-2 and table table 8.16.102.5-3 define the Cell specific test parameters for unknown SCell1 activation.

Table 8.16.102.5-1: Cell specific test parameters for unknown SCell1 activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity | 17 |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | -infinity | 17 |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 5 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 6 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.102.1-3: Cell specific test parameters for unknown SCell1 activation

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  | Cell 6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  | 6 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 5 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 6 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.102.1-4: Cell specific test parameters for unknown SCell1 activation

| Parameter | Unit | Cell 7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  |  |  |  |  |  |
| E-UTRA RF Channel Number |  | 7 |  |  |  |  |  |  |  |  |
| Duplex mode |  | FDD or TDD |  |  |  |  |  |  |  |  |
| TDD special subframe configuration |  | 6 |  |  |  |  |  |  |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  |  |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  |  |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 4 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 5 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 6 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+8) and (m+9) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+59).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29), (m+35) to (m+39), (m+45) to (m+49) and (m+55) to (m+59) for FDD PCell, and not happen outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

During T3 interruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29), (n+35) to (n+39), (n+45) to (n+49) and (n+55) to (n+59) for FDD PCell, and not happen outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41), (n+50) to (n+56), (n+65) to (n+71) and (n+80) to (n+86) for TDD PCell.

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+59) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

///End of changes