3GPP TSG-RAN5 Meeting #96-e                      	R5-225163

# Electronic Meeting, August 15 - 26 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2660 | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Cell Configuration in Annex E for 6 and 7 DL CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Cell configuration in annex E is incorrect or missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Cell configuration for RRM 6 and 7 DL CA test cases has been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Annex E will remain incomplete. RRM 6 and 7 DL CA test cases cannot be implemented correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

Annex E (normative):
Cell configuration mapping

The cells used in 3GPP TS 36.521-3 do not correspond to the cells defined in 3GPP TS 36.508 [7] clause 4.4.2. Tables E-1, E-2, E-3, E-5, E-6, E-7 and E-8 describes the mapping between cells described in 3GPP TS 36.521-3 and those defined in 3GPP TS 36.508 [7]. Table E-4 describe the mapping between Ncells described in 3GPP TS 36.521-3 and those defined in 3GPP TS 36.508 [7] For each test case the cells as defined in 3GPP TS 36.508 [7] clause 4.4.2 and clause 8.1.4.2 are listed in one row. The test case shall apply the RF parameters as defined in 3GPP TS 36.521-3 according to the column heading.

NOTE:	For example if the second cell in a test case is an inter-frequency cell then Cell3 from TS 36.508 [7] clause 4.4.2 is used with the radio parameters as defined for Cell2 in 3GPP TS 36.521-3.

Table E-1: Cell configuration mapping for RRM testing

| Test Case | Description | 36.521-3 Cell1 | 36.521-3 Cell2 | 36.521-3 Cell3 | 36.521-3 Cell4 | CA Type (Note 2) |
| --- | --- | --- | --- | --- | --- | --- |
| 4.2.1 | RRC IDLE / E-UTRAN Cell Reselection / FDD - FDD cell re-selection intra frequency case | Cell1 | Cell11 |  |  |  |
| 4.2.2 | RRC IDLE / E-UTRAN Cell Reselection / TDD - TDD cell re-selection intra frequency case | Cell1 | Cell11 |  |  |  |
| 4.2.3 | RRC IDLE / E-UTRAN Cell Reselection / FDD - FDD cell re-selection inter frequency case | Cell6 | Cell23 |  |  |  |
| 4.2.4 | RRC IDLE / E-UTRAN Cell Reselection / FDD - TDD cell re-selection inter frequency case | Cell1 | Cell31 | Dual mode in single PLMN |  |  |
| 4.2.5 | RRC IDLE / E-UTRAN Cell Reselection / TDD - FDD cell re-selection inter frequency case | Cell31 | Cell1 |  |  |  |
| 4.2.6 | RRC IDLE / E-UTRAN Cell Reselection / TDD - TDD cell re-selection inter frequency case | Cell6 | Cell23 |  |  |  |
| 4.2.7 | RRC IDLE / E-UTRAN Cell Reselection / FDD - FDD Inter frequency case in the existence of non-allowed CSG cell | Cell23 | Cell6 | Cell3 |  |  |
| 4.2.8 | RRC IDLE / E-UTRAN Cell Reselection / TDD - TDD Inter frequency case in the existence of non-allowed CSG cell | Cell23 | Cell6 | Cell3 |  |  |
| 4.2.9 | RRC IDLE / E-UTRAN Cell Reselection / E-UTRAN FDD-FDD intra-frequency Cell Re-selection case for 5MHz Bandwidth | Cell1 | Cell11 |  |  |  |
| 4.2.10 | E-UTRAN FDD - FDD reselection using an increased number of carriers | Cell6 | (Note 9) | (Note 10) (Note15) | (Note 10) (Note15) |  |
| 4.2.11 | E-UTRAN TDD - TDD reselection using an increased number of carriers | Cell6 | (Note 9) | (Note 10) (Note15) | (Note 10) (Note15) |  |
| 4.2.12 | E-UTRAN FDD - FDD Intra frequency case for Cat-M1 UE in normal coverage | Cell1 | Cell11 |  |  |  |
| 4.2.13 | E-UTRAN HD - FDD Intra frequency case for Cat-M1 UE in normal coverage | Cell1 | Cell11 |  |  |  |
| 4.2.14 | E-UTRAN TDD - TDD Intra frequency case for Cat-M1 UE in normal coverage | Cell1 | Cell11 |  |  |  |
| 4.2.15 | E-UTRAN FDD - FDD Intra frequency case for Cat-M1 UE in enhanced coverage | Cell1 | Cell11 |  |  |  |
| 4.2.16 | E-UTRAN HD - FDD Intra frequency case for Cat-M1 UE in enhanced coverage | Cell1 | Cell11 |  |  |  |
| 4.2.17 | E-UTRAN TDD - TDD Intra frequency case for Cat-M1 UE in enhanced coverage | Cell1 | Cell11 |  |  |  |
| 4.2.20 | E-UTRAN FDD - FDD Intra frequency case for UE Category 1bis | Cell1 | Cell11 |  |  |  |
| 4.2.21 | E-UTRAN TDD - TDD Intra frequency case for UE Category 1bis | Cell1 | Cell11 |  |  |  |
| 4.2.22 | E-UTRAN FDD - FDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag | Cell1 | Cell11 |  |  |  |
| 4.2.23 | E-UTRAN TDD - TDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag | Cell1 | Cell11 |  |  |  |
| 4.3.1.1 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA is of higher priority | Cell3 | Cell9 |  |  |  |
| 4.3.1.2 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA is of lower priority | Cell3 | Cell9 |  |  |  |
| 4.3.1.3 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN FDD cell re-selection in fading propagation conditions: UTRA FDD is of lower priority | Cell3 | Cell9 |  |  |  |
| 4.3.1.4 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority for 5MHz Bandwidth | Cell3 | Cell9 |  |  |  |
| 4.3.1.5 | Idle mode FDD to UTRA FDD interRAT reselection | Cell6 | (Note11) | (Note14) (Note15) |  |  |
| 4.3.2 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN TDD cell re-selection | Cell6 | Cell8 |  |  |  |
| 4.3.2A | E-UTRA FDD to UTRA TDD cell re-selection for IncMon | Cell6 | (Note11) | (Note 16) (Note15) |  |  |
| 4.3.3 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN TDD - UTRAN FDD cell re-selection | Cell6 | Cell8 |  |  |  |
| 4.3.3A | Idle mode TDD to UTRA FDD interRAT reselection | Cell6 | (Note11) | (Note 14) (Note 15) |  |  |
| 4.3.4.1 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN TDD - UTRAN TDD cell re-selection: UTRA is of higher priority | Cell6 | Cell8 |  |  |  |
| 4.3.4.2 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN TDD - UTRAN TDD cell re-selection: UTRA is of lower priority | Cell6 | Cell8 |  |  |  |
| 4.3.4.4 | E-UTRA TDD to UTRA TDD cell re-selection for IncMon | Cell6 | (Note11) | (Note 16) (Note 15) |  |  |
| 4.3.4.3 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection /EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority | Cell3 | Cell9 |  |  |  |
| 4.4.1 | RRC IDLE / E-UTRAN to GSM Cell re-selection / E-UTRAN FDD - GSM cell re-selection | Cell1 | Cell26 |  |  |  |
| 4.4.2 | RRC IDLE / E-UTRAN to GSM Cell re-selection / E-UTRAN TDD - GSM cell re-selection | Cell1 | Cell26 |  |  |  |
| 4.5.1.1 | RRC IDLE / E-UTRAN to HRPD Cell re-selection / E-UTRAN FDD - HRPD cell re-selection: HRPD is of lower priority | Cell1 | Cell15 |  |  |  |
| 4.5.2.1 | RRC IDLE / E-UTRAN to HRPD Cell re-selection / E-UTRAN TDD - HRPD Cell Reselection: HRPD is of Lower Priority | Cell1 | Cell15 |  |  |  |
| 4.6.1.1 | RRC IDLE / E-UTRAN to cdma2000 1xRTT Cell re-selection / E-UTRAN FDD - cdma2000 1xRTT cell re-selection: cdma2000 1x is of lower priority | Cell1 | Cell19 |  |  |  |
| 4.6.2.1 | RRC IDLE / E-UTRAN to cdma2000 1xRTT Cell re-selection / E-UTRAN TDD - cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority | Cell1 | Cell19 |  |  |  |
| 5.1.1 | RRC CONNECTED / E-UTRAN Handover / FDD - FDD / Intra frequency case | Cell1 | Cell2 |  |  |  |
| 5.1.2 | RRC CONNECTED / E-UTRAN Handover / TDD - TDD / Intra frequency case | Cell1 | Cell2 |  |  |  |
| 5.1.3 | RRC CONNECTED / E-UTRAN Handover / FDD - FDD / Inter frequency case | Cell6 | Cell3 |  |  |  |
| 5.1.3_2 | E-UTRAN FDD - FDD Inter frequency handover for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 5.1.4 | RRC CONNECTED / E-UTRAN Handover / TDD - TDD / Inter frequency case | Cell6 | Cell3 |  |  |  |
| 5.1.4_2 | E-UTRAN TDD - TDD Inter frequency handover for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 5.1.5 | RRC CONNECTED / E-UTRAN Handover / FDD - FDD / Inter frequency case: unknown target cell | Cell6 | Cell3 |  |  |  |
| 5.1.5_2 | E-UTRAN FDD - FDD Inter frequency handover: unknown target cell for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 5.1.6 | RRC CONNECTED / E-UTRAN Handover / TDD - TDD / Inter frequency case: unknown target cell | Cell6 | Cell3 |  |  |  |
| 5.1.6_2 | E-UTRAN TDD - TDD Inter frequency handover; unknown Target Cell for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 5.1.7 | RRC CONNECTED / E-UTRAN Handover / FDD - TDD / Inter frequency case | Cell1 | Cell10 | Dual mode in multiple PLMN |  |  |
| 5.1.7_2 | E-UTRAN FDD - TDD Inter frequency handover for UE Category 1bis | Cell1 | Cell10 | Dual mode in multiple PLMN |  |  |
| 5.1.8 | RRC CONNECTED / E-UTRAN Handover / TDD - FDD / Inter frequency case | Cell10 | Cell1 |  |  |  |
| 5.1.8_2 | E-UTRAN TDD - FDD Inter frequency handover for UE Category 1bis | Cell10 | Cell1 |  |  |  |
| 5.1.9 | RRC CONNECTED / E-UTRAN Handover / FDD - FDD / Intra frequency handover for 5MHz Bandwidth | Cell1 | Cell2 |  |  |  |
| 5.1.10 | E-UTRAN FDD-FDD Handover intra frequency handover for UE category 0 | Cell1 | Cell2 |  |  |  |
| 5.1.11 | E-UTRAN HD-FDD Handover intra frequency handover for UE category 0 | Cell1 | Cell2 |  |  |  |
| 5.1.12 | E-UTRAN TDD-TDD Handover intra frequency handover for UE category 0 | Cell1 | Cell2 |  |  |  |
| 5.1.13 | E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Cell1 | Cell2 |  |  |  |
| 5.1.14 | E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Cell1 | Cell2 |  |  |  |
| 5.1.15 | E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA | Cell1 | Cell2 |  |  |  |
| 5.1.19 | E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 5.1.20 | E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 5.1.28 | E-UTRAN HD-FDD inter frequency handover for CE UEs in CEModeA | Cell6 | Cell3 |  |  |  |
| 5.1.34 | E-UTRAN HD-FDD intra frequency handover for CE UEs in CEModeA without SFN acquisition | Cell6 | Cell3 |  |  |  |
| 5.1.41 | E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover | Cell6 | Cell3 |  |  |  |
| 5.1.42 | E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover | Cell6 | Cell3 |  |  |  |
| 5.1.43 | E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover | Cell1 | Cell10 |  |  |  |
| 5.1.44 | E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover | Cell1 | Cell10 |  |  |  |
| 5.1.45 | E-UTRAN FDD - FDD Intra frequency DAPS handover | Cell1 | Cell2 |  |  |  |
| 5.1.46 | E-UTRAN TDD - TDD Intra frequency DAPS handover | Cell1 | Cell2 |  |  |  |
| 5.1.53 | E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover | Cell6 | Cell3 |  |  |  |
| 5.1.54 | E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover | Cell1 | Cell10 |  |  |  |
| 5.1.55 | E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover | Cell1 | Cell10 |  |  |  |
| 5.1.56 | E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover | Cell1 | Cell10 |  |  |  |
| 5.1.57 | E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover | Cell1 | Cell10 |  |  |  |
| 5.1.58 | E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover | Cell1 | Cell10 |  |  |  |
| 5.1.47 | E-UTRAN FDD - FDD Intra frequency conditional handover | Cell1 | Cell2 |  |  |  |
| 5.1.48 | E-UTRAN TDD - TDD Intra frequency conditional handover | Cell1 | Cell2 |  |  |  |
| 5.1.49 | E-UTRAN FDD - FDD Inter frequency conditional handover | Cell6 | Cell3 |  |  |  |
| 5.1.50 | E-UTRAN TDD - TDD Inter frequency conditional handover | Cell6 | Cell3 |  |  |  |
| 5.1.51 | E-UTRAN FDD - TDD Inter frequency conditional handover | Cell1 | Cell10 |  |  |  |
| 5.1.52 | E-UTRAN TDD - FDD Inter frequency conditional handover | Cell10 | Cell1 |  |  |  |
| 5.2.1 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to UTRAN / E-UTRAN FDD - UTRAN FDD handover | Cell3 | Cell9 |  |  |  |
| 5.2.2 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to UTRAN / E-UTRAN TDD - UTRAN FDD handover | Cell6 | Cell8 |  |  |  |
| 5.2.3 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to GSM / E-UTRAN FDD - GSM handover | Cell1 | Cell26 |  |  |  |
| 5.2.4 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to UTRAN / E-UTRAN TDD - UTRAN TDD handover | Cell3 | Cell9 |  |  |  |
| 5.2.5 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to UTRAN / E-UTRAN FDD - UTRAN TDD handover | Cell6 | Cell8 |  |  |  |
| 5.2.6 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to GSM / E-UTRA TDD - GSM handover | Cell1 | Cell26 |  |  |  |
| 5.2.7 | RRC CONNECTED / Handover from E-UTRAN to other RATs / E-UTRAN FDD - UTRAN FDD handover: unknown target cell | Cell3 | Cell9 |  |  |  |
| 5.2.8 | RRC CONNECTED / Handover from E-UTRAN to other RATs / E-UTRAN FDD - GSM handover: unknown target cell | Cell1 | Cell26 |  |  |  |
| 5.2.9 | RRC CONNECTED / Handover from E-UTRAN to other RATs / E-UTRAN TDD - GSM Handover: unknown target cell | Cell1 | Cell26 |  |  |  |
| 5.2.10 | RRC CONNECTED / Handover from E-UTRAN to other RATs / E-UTRAN TDD - UTRAN TDD HO test: unknown target cell | Cell6 | Cell8 |  |  |  |
| 5.2.11 | RRC CONNECTED / Handover from E-UTRAN to other RATs / E-UTRAN FDD - UTRAN FDD handover for 5MHz Bandwidth | Cell3 | Cell9 |  |  |  |
| 5.3.1 | RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN FDD - HRPD handover | Cell1 | Cell15 |  |  |  |
| 5.3.2 | RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN FDD - cdma2000 1xRTT handover | Cell1 | Cell19 |  |  |  |
| 5.3.3 | RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN FDD - HRPD Handover: unknown target cell | Cell1 | Cell15 |  |  |  |
| 5.3.4 | RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN FDD - cdma2000 1xRTT Handover: unknown target cell | Cell1 | Cell19 |  |  |  |
| 5.3.5 | RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN TDD-HRPD Handover | Cell1 | Cell15 |  |  |  |
| 5.3.6 | RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN TDD-cdma2000 1X Handover | Cell1 | Cell19 |  |  |  |
| 6.1.1 | RRC Connection Mobility Control / E-UTRAN FDD Intra-frequency RRC Re-establishment | Cell1 | Cell2 |  |  |  |
| 6.1.1_2 | RRC Connection Mobility Control / E-UTRAN FDD Intra-frequency RRC Re-establishment for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 6.1.2 | RRC Connection Mobility Control / E-UTRAN FDD Inter-frequency RRC Re-establishment | Cell6 | Cell3 |  |  |  |
| 6.1.2_2 | RRC Connection Mobility Control / E-UTRAN FDD Inter-frequency RRC Re-establishment for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 6.1.3 | RRC Connection Mobility Control / E-UTRAN TDD Intra-frequency RRC Re-establishment | Cell1 | Cell2 |  |  |  |
| 6.1.3_2 | RRC Connection Mobility Control / E-UTRAN TDD Intra-frequency RRC Re-establishment for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 6.1.4 | RRC Connection Mobility Control / E-UTRAN TDD Inter-frequency RRC Re-establishment | Cell6 | Cell3 |  |  |  |
| 6.1.4_2 | RRC Connection Mobility Control / E-UTRAN TDD Inter-frequency RRC Re-establishment for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 6.1.5 | RRC Connection Mobility Control / E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz Bandwidth | Cell1 | Cell2 |  |  |  |
| 6.1.8 | E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0 | Cell1 | Cell2 |  |  |  |
| 6.1.9 | E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 6.1.10 | E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 6.1.11 | E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 6.1.12 | E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 6.1.13 | E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 6.1.14 | E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 6.1.18 | E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for CE UE in CEModeA | Cell6 | Cell3 |  |  |  |
| 6.2.1 | RRC Connection Mobility Control / Random Access / E-UTRAN FDD - Contention Based Random Access | Cell1 |  |  |  |  |
| 6.2.1_2 | RRC Connection Mobility Control / Random Access / E-UTRAN FDD - Contention Based Random Access for UE Category 1bis | Cell1 |  |  |  |  |
| 6.2.2 | RRC Connection Mobility Control / Random Access / E-UTRAN FDD - Non-Contention Based Random Access | Cell1 |  |  |  |  |
| 6.2.2_2 | RRC Connection Mobility Control / Random Access / E-UTRAN FDD - Non-Contention Based Random Access for UE Category 1bis | Cell1 |  |  |  |  |
| 6.2.3 | RRC Connection Mobility Control / Random Access / E-UTRAN TDD - Contention Based Random Access | Cell1 |  |  |  |  |
| 6.2.3_2 | RRC Connection Mobility Control / Random Access / E-UTRAN TDD - Contention Based Random Access for UE Category 1bis | Cell1 |  |  |  |  |
| 6.2.4 | RRC Connection Mobility Control / Random Access / E-UTRAN TDD - Non-Contention Based Random Access | Cell1 |  |  |  |  |
| 6.2.4_2 | RRC Connection Mobility Control / Random Access / E-UTRAN TDD - Non-Contention Based Random Access for UE Category 1bis | Cell1 |  |  |  |  |
| 6.2.5 | RRC Connection Mobility Control / Random Access / E-UTRAN FDD - Contention Based Random Access Test for 5MHz Bandwidth | Cell1 |  |  |  |  |
| 6.2.6 | RRC Connection Mobility Control / Random Access / E-UTRAN FDD - Non-Contention Based Random Access Test for 5MHz Bandwidth | Cell1 |  |  |  |  |
| 6.2.7 | E-UTRAN FDD - Non-Contention Based Random Access Test For SCell in sTAG | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 6.2.8 | E-UTRAN TDD - Non-Contention Based Random Access Test For SCell in sTAG | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 6.2.10 | E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | Cell1 |  |  |  |  |
| 6.2.11 | E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | Cell1 |  |  |  |  |
| 6.2.12 | RRC Connection Mobility Control / Random Access / E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | Cell1 |  |  |  |  |
| 6.2.13 | E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Cell1 |  |  |  |  |
| 6.2.14 | E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Cell1 |  |  |  |  |
| 6.2.15 | E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Cell1 |  |  |  |  |
| 6.3.1 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to UTRAN FDD | Cell3 | Cell9 |  |  |  |
| 6.3.1_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to UTRAN FDD for UE Category 1bis | Cell3 | Cell9 |  |  |  |
| 6.3.2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN TDD to UTRAN FDD | Cell6 | Cell8 |  |  |  |
| 6.3.2_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN TDD to UTRAN FDD for UE Category 1bis | Cell6 | Cell8 |  |  |  |
| 6.3.3 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to GERAN when System Information is provided | Cell1 | Cell26 |  |  |  |
| 6.3.3_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to GERAN when System Information is provided for UE Category 1bis | Cell1 | Cell26 |  |  |  |
| 6.3.4 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN TDD to GERAN when System Information is provided | Cell1 | Cell26 |  |  |  |
| 6.3.4_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN TDD to GERAN when System Information is provided for UE Category 1bis | Cell1 | Cell26 |  |  |  |
| 6.3.5 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA TDD RRC connection release redirection to UTRA TDD | Cell3 | Cell9 |  |  |  |
| 6.3.5_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA TDD RRC connection release redirection to UTRA TDD for UE Category 1bis | Cell3 | Cell9 |  |  |  |
| 6.3.6 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA FDD RRC connection release redirection to UTRA TDD | Cell6 | Cell8 |  |  |  |
| 6.3.6_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA FDD RRC connection release redirection to UTRA TDD for UE Category 1bis | Cell6 | Cell8 |  |  |  |
| 6.3.7 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided | Cell3 | Cell9 |  |  |  |
| 6.3.7_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided for UE Category 1bis | Cell3 | Cell9 |  |  |  |
| 6.3.8 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided | Cell6 | Cell8 |  |  |  |
| 6.3.8_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided for UE Category 1bis | Cell6 | Cell8 |  |  |  |
| 6.3.9 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to UTRAN FDD without System Information | Cell3 | Cell9 |  |  |  |
| 6.3.9_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to UTRAN FDD without System Information for UE Category 1bis | Cell3 | Cell9 |  |  |  |
| 6.3.10 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to GERAN when System Information is not provided | Cell1 | Cell26 |  |  |  |
| 6.3.10_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN FDD to GERAN when System Information is not provided for UE Category 1bis | Cell1 | Cell26 |  |  |  |
| 6.3.11 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN TDD to GERAN when System Information is not provided | Cell1 | Cell26 |  |  |  |
| 6.3.11_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / Redirection from E-UTRAN TDD to GERAN when System Information is not provided for UE Category 1bis | Cell1 | Cell26 |  |  |  |
| 6.3.12 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided | Cell6 | Cell8 |  |  |  |
| 6.3.12_2 | RRC Connection Mobility Control / RRC Connection Release with Redirection / E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided for UE Category 1bis | Cell6 | Cell8 |  |  |  |
| 7.1.1 | E-UTRAN FDD-UE Transmit Timing Accuracy | Cell1 |  |  |  |  |
| 7.1.1_2 | E-UTRAN FDD - UE Transmit Timing Accuracy for UE category 1bis | Cell1 |  |  |  |  |
| 7.1.2 | E-UTRAN TDD-UE Transmit Timing Accuracy | Cell1 |  |  |  |  |
| 7.1.2_2 | E-UTRAN TDD - UE Transmit Timing Accuracy for UE category 1bis | Cell1 |  |  |  |  |
| 7.1.3 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.3_1 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell (Release 12 and forward) | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.4 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.4A | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz +10 MHz bandwidth | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.4_1 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell (Release 12 and forward) | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.5 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for 5MHz Bandwidth | Cell1 |  |  |  |  |
| 7.1.6 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell in sTAG | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.7 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.7A | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz bandwidth | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.7B | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz bandwidth | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.1.8 | Void |  |  |  |  |  |
| 7.1.9 | Void |  |  |  |  |  |
| 7.1.10 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Cell1 |  |  |  |  |
| 7.1.11 | E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Cell1 |  |  |  |  |
| 7.1.12 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Cell1 |  |  |  |  |
| 7.1.14 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Cell1 |  |  |  |  |
| 7.1.15 | E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Cell1 |  |  |  |  |
| 7.1.16 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Cell1 |  |  |  |  |
| 7.1.21 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeA | Cell1 |  |  |  |  |
| 7.1.22 | E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeA | Cell1 |  |  |  |  |
| 7.1.23 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeA | Cell1 |  |  |  |  |
| 7.1.24 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeB | Cell1 |  |  |  |  |
| 7.2.1 | E-UTRAN FDD-UE Timing Advance Adjustment Accuracy | Cell1 |  |  |  |  |
| 7.2.1_2 | E-UTRAN FDD - UE Timing Advance Adjustment Accuracy for UE category 1bis | Cell1 |  |  |  |  |
| 7.2.2 | E-UTRAN TDD-UE Timing Advance Adjustment Accuracy | Cell1 |  |  |  |  |
| 7.2.2_2 | E-UTRAN TDD - UE Timing Advance Adjustment Accuracy for UE category 1bis | Cell1 |  |  |  |  |
| 7.2.3 | E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for 5MHz Bandwidth | Cell1 |  |  |  |  |
| 7.2.4 | E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test For SCell in sTAG | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.2.5 | E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test For SCell in sTAG | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.2.5A | E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG for 20MHz +20MHz bandwidth | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.2.5B | E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG for 20MHz +10MHz bandwidth | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 7.2.6 | E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Cell1 |  |  |  |  |
| 7.2.7 | E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Cell1 |  |  |  |  |
| 7.2.8 | E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Cell1 |  |  |  |  |
| 7.2.10 | E-UTRAN FDD Timing Advance Adjustment Accuracy Test in CEModeB | Cell1 |  |  |  |  |
| 7.2.11 | E-UTRAN HD-FDD Timing Advance Adjustment Accuracy Test in CEModeB | Cell1 |  |  |  |  |
| 7.2.12 | E-UTRAN TDD Timing Advance Adjustment Accuracy Test in CEModeB | Cell1 |  |  |  |  |
| 7.3.1 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync | Cell1 |  |  |  |  |
| 7.3.1_1 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync with 4 Rx antenna ports | Cell1 |  |  |  |  |
| 7.3.2 | E-UTRAN FDD Radio Link Monitoring Test for In-sync | Cell1 |  |  |  |  |
| 7.3.2_1 | E-UTRAN FDD Radio Link Monitoring Test for In-sync with 4 Rx antenna ports | Cell1 |  |  |  |  |
| 7.3.3 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync | Cell1 |  |  |  |  |
| 7.3.3_1 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync with 4 Rx antenna ports | Cell 1 |  |  |  |  |
| 7.3.4 | E-UTRAN TDD Radio Link Monitoring Test for In-sync | Cell1 |  |  |  |  |
| 7.3.4_1 | E-UTRAN TDD Radio Link Monitoring Test for In-sync with 4 Rx antenna ports | Cell1 |  |  |  |  |
| 7.3.5 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX | Cell1 |  |  |  |  |
| 7.3.6 | E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX | Cell1 |  |  |  |  |
| 7.3.7 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX | Cell1 |  |  |  |  |
| 7.3.8 | E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX | Cell1 |  |  |  |  |
| 7.3.9 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 7.3.10 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 7.3.11 | E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 7.3.12 | E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 7.3.13 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell3 |  |  |  |
| 7.3.14 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell3 |  |  |  |
| 7.3.15 | E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell3 |  |  |  |
| 7.3.16 | E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell3 |  |  |  |
| 7.3.17 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell3 | Cell4 |  |  |
| 7.3.18 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell3 | Cell4 |  |  |
| 7.3.19 | E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell3 | Cell4 |  |  |
| 7.3.20 | E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell3 | Cell4 |  |  |
| 7.3.21 | E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell3 | Cell4 |  |  |
| 7.3.22 | E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell3 | Cell4 |  |  |
| 7.3.23 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth | Cell1 |  |  |  |  |
| 7.3.24 | E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth | Cell1 |  |  |  |  |
| 7.3.25 | E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth | Cell1 |  |  |  |  |
| 7.3.26 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | Cell1 |  |  |  |  |
| 7.3.26_2 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.27 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE category 0 | Cell1 |  |  |  |  |
| 7.3.27_2 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.28 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | Cell1 |  |  |  |  |
| 7.3.28_2 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.29 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Cell1 |  |  |  |  |
| 7.3.29_2 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.30 | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | Cell1 |  |  |  |  |
| 7.3.31 | E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE category 0 | Cell1 |  |  |  |  |
| 7.3.32 | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | Cell1 |  |  |  |  |
| 7.3.33 | E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Cell1 |  |  |  |  |
| 7.3.34 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | Cell1 |  |  |  |  |
| 7.3.34_2 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.35 | E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0 | Cell1 |  |  |  |  |
| 7.3.35_2 | E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.36 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | Cell1 |  |  |  |  |
| 7.3.36_2 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.37 | E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Cell1 |  |  |  |  |
| 7.3.37_2 | E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category 1bis | Cell1 |  |  |  |  |
| 7.3.38 | E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC | Cell1 | Cell10 |  |  |  |
| 7.3.39 | E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC | Cell1 | Cell10 |  |  |  |
| 7.3.40 | E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC | Cell1 | Cell10 |  |  |  |
| 7.3.41 | E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity | Cell1 | Cell10 |  |  |  |
| 7.3.42 | E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC | Cell1 | Cell10 |  |  |  |
| 7.3.43 | E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity | Cell1 | Cell10 |  |  |  |
| 7.3.44 | E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD | Cell1 | Cell10 |  |  |  |
| 7.3.45 | E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD | Cell1 | Cell10 |  |  |  |
| 7.3.46 | E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD | Cell1 | Cell10 |  |  |  |
| 7.3.47 | E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD | Cell1 | Cell10 |  |  |  |
| 7.3.48 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | Cell1 |  |  |  |  |
| 7.3.49 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | Cell1 |  |  |  |  |
| 7.3.50 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A | Cell1 |  |  |  |  |
| 7.3.51 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A | Cell1 |  |  |  |  |
| 7.3.52 | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | Cell1 |  |  |  |  |
| 7.3.53 | E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | Cell1 |  |  |  |  |
| 7.3.54 | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A | Cell1 |  |  |  |  |
| 7.3.55 | E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A | Cell1 |  |  |  |  |
| 7.3.56 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | Cell1 |  |  |  |  |
| 7.3.57 | E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | Cell1 |  |  |  |  |
| 7.3.58 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A | Cell1 |  |  |  |  |
| 7.3.59 | E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A | Cell1 |  |  |  |  |
| 7.3.69 | E-UTRAN HD-FDD Early Out-of-sync reporting Test for CE UE in CEMode A | Cell1 |  |  |  |  |
| 7.4.1 | E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC | Cell1 | Cell10 |  |  |  |
| 7.4.2 | E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC | Cell1 | Cell10 |  |  |  |
| 7.5.1 | E-UTRAN FDD - UE ProSe Direct Discovery Transmission Timing Accuracy Test | Cell 1 |  |  |  |  |
| 7.5.4 | E-UTRAN FDD - UE ProSe Direct Communication Transmission Timing Accuracy Test | Cell 1 |  |  |  |  |
| 7.6.1 | E-UTRAN FDD-TDD CA interruption at SRS carrier based switching | Cell 1 | Cell 10 |  |  |  |
| 7.6.2 | E-UTRAN TDD-TDD CA interruption at SRS carrier based switching | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 8.1.1 | UE Measurement Procedures / E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in asynchronous cells | Cell1 | Cell2 |  |  |  |
| 8.1.2 | UE Measurement Procedures / E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells | Cell1 | Cell2 |  |  |  |
| 8.1.3 | UE Measurement Procedures / E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX | Cell1 | Cell2 |  |  |  |
| 8.1.4 | Void |  |  |  |  |  |
| 8.1.5 | UE Measurement Procedures / E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Cell1 | Cell2 |  |  |  |
| 8.1.6 | UE Measurement Procedures / E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Cell1 | Cell2 |  |  |  |
| 8.1.7 | UE Measurement Procedures / E-UTRAN FDD-FDD Intra-frequency event-triggered reporting under time domain measurement resource restriction with non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 8.1.8 | E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 8.1.9 | E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth | Cell1 | Cell2 |  |  |  |
| 8.1.10 | E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for 5MHz bandwidth | Cell1 | Cell2 |  |  |  |
| 8.1.11 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0 | Cell1 | Cell2 |  |  |  |
| 8.1.11_2 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 1bis | Cell1 | Cell2 |  |  |  |
| 8.1.12 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0 | Cell1 | Cell2 |  |  |  |
| 8.1.12_2 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Cell1 | Cell2 |  |  |  |
| 8.1.13 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 | Cell1 | Cell2 |  |  |  |
| 8.1.13_2 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 1bis | Cell1 | Cell2 |  |  |  |
| 8.1.17 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells | Cell1 | Cell2 |  |  |  |
| 8.1.17_2 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Cell1 | Cell2 |  |  |  |
| 8.1.18 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX | Cell1 | Cell2 |  |  |  |
| 8.1.18_2 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 1bis | Cell1 | Cell2 |  |  |  |
| 8.1.23 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 8.1.24 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 8.1.25 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Cell1 | Cell2 |  |  |  |
| 8.1.26 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 8.1.27 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 8.1.28 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Cell1 | Cell2 |  |  |  |
| 8.1.29 | E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 8.1.30 | E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Cell1 | Cell2 |  |  |  |
| 8.1.31 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.32 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.33 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.34 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.35 | E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.36 | E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.37 | -UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.38 | E-UTRAN HD- FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.39 | E-UTRAN HD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.1.40 | E-UTRAN FDD-FDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells | Cell1 | Cell2 |  |  |  |
| 8.2.1 | UE Measurement Procedures / E-UTRAN TDD-TDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells | Cell1 | Cell2 |  |  |  |
| 8.2.2 | UE Measurement Procedures / E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX | Cell1 | Cell2 |  |  |  |
| 8.2.3 | UE Measurement Procedures / E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Cell1 | Cell2 |  |  |  |
| 8.2.4 | UE Measurement Procedures / E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Cell1 | Cell2 |  |  |  |
| 8.2.5 | UE Measurement Procedures / E-UTRAN TDD-TDD intra-frequency event-triggered Reporting under time domain measurement resource restriction with non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 8.2.6 | E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 8.2.9 | E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.2.10 | E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 8.2.11 | E-UTRAN TDD-TDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells | Cell1 | Cell11 |  |  |  |
| 8.3.1 | UE Measurement Procedures / E-UTRAN FDD-FDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells | Cell6 | Cell3 |  |  |  |
| 8.3.1_2 | UE Measurement Procedures / E-UTRAN FDD-FDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 1bis | Cell6 | Cell3 |  |  |  |
| 8.3.2 | UE Measurement Procedures / E-UTRAN FDD-FDD inter frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells | Cell6 | Cell3 |  |  |  |
| 8.3.2_2 | UE Measurement Procedures / E-UTRAN FDD-FDD inter frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells for UE category 1bis | Cell6 | Cell3 |  |  |  |
| 8.3.3 | UE Measurement Procedures / E-UTRAN FDD-FDD inter frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used | Cell6 | Cell3 |  |  |  |
| 8.3.3_2 | E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used for UE category 1bis | Cell6 | Cell3 |  |  |  |
| 8.3.4 | UE Measurement Procedures / E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Cell6 | Cell3 |  |  |  |
| 8.3.5 | UE Measurement Procedures / E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Cell6 | Cell3 |  |  |  |
| 8.3.6 | UE Measurement Procedures / E-UTRAN FDD-FDD inter frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells | Cell6 | Cell3 |  |  |  |
| 8.3.7 | UE Measurement Procedures / E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group | Cell6 | (Note 8) | (Note 8) | (Note 8) |  |
| 8.3.8 | UE Measurement Procedures / FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, non DRX | Cell6 | (Note 9) | (Note 10) | (Note 10) |  |
| 8.3.9 | UE Measurement Procedures / FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX | Cell6 | (Note 9) | (Note 10) | (Note 10) |  |
| 8.3.12 | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA | Cell6 | Cell3 |  |  |  |
| 8.3.13 | E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA | Cell6 | Cell3 |  |  |  |
| 8.3.14 | E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB | Cell6 | Cell3 |  |  |  |
| 8.3.15 | E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB | Cell6 | Cell3 |  |  |  |
| 8.3.16 | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA when DRX is used | Cell6 | Cell3 |  |  |  |
| 8.3.17 | E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA in DRX | Cell6 | Cell3 |  |  |  |
| 8.3.18 | E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB in DRX | Cell6 | Cell3 |  |  |  |
| 8.3.19 | E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB in DRX | Cell6 | Cell3 |  |  |  |
| 8.4.1 | UE Measurement Procedures / E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Cell6 | Cell3 |  |  |  |
| 8.4.1_2 | UE Measurement Procedures / E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Cell6 | Cell3 |  |  |  |
| 8.4.2 | UE Measurement Procedures / E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells | Cell6 | Cell3 |  |  |  |
| 8.4.2_2 | UE Measurement Procedures / E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells for UE category 1bis | Cell6 | Cell3 |  |  |  |
| 8.4.3 | UE Measurement Procedures / E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used | Cell6 | Cell3 |  |  |  |
| 8.4.3_2 | E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used for UE category 1bis | Cell6 | Cell3 |  |  |  |
| 8.4.4 | UE Measurement Procedures / E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Cell6 | Cell3 |  |  |  |
| 8.4.5 | UE Measurement Procedures / E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Cell6 | Cell3 |  |  |  |
| 8.4.6 | UE Measurement Procedures / E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 | Cell6 | Cell3 |  |  |  |
| 8.4.6_2 | UE Measurement Procedures / E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 for UE category 1bis | Cell6 | Cell3 |  |  |  |
| 8.4.7 | UE Measurement Procedures / E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group | Cell6 | (Note 8) | (Note 8) | (Note 8) |  |
| 8.4.8 | UE Measurement Procedures / TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX | Cell6 | (Note 9) | (Note 10) | (Note 10) |  |
| 8.4.9 | TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX | Cell6 | (Note 9) | (Note 10) | (Note 10) |  |
| 8.4.12 | E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA |  |  |  |  |  |
| 8.4.13 | E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB |  |  |  |  |  |
| 8.4.14 | E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA in DRX |  |  |  |  |  |
| 8.4.15 | E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB in DRX |  |  |  |  |  |
| 8.5.1 | UE Measurement Procedures / E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions | Cell3 | Cell9 |  |  |  |
| 8.5.2 | UE Measurement Procedures / E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions | Cell3 | Cell9 |  |  |  |
| 8.5.3 | UE Measurement Procedures / E-UTRAN FDD - UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions | Cell3 | Cell9 |  |  |  |
| 8.5.4 | UE Measurement Procedures / E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions | Cell3 | Cell9 |  |  |  |
| 8.5.6 | UE Measurement Procedures / E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions | Cell3 | Cell9 |  |  |  |
| 8.5.7 | UE Measurement Procedures / E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions for 5MHz bandwidth | Cell3 | Cell9 |  |  |  |
| 8.5.8 | E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX | Cell6 | (Note 11) | (Note 12) | (Note 12) |  |
| 8.6.1 | UE Measurement Procedures / E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions | Cell1 | Cell8 |  |  |  |
| 8.6.3 | E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX | Cell6 | (Note 11) | (Note 12) | (Note 12) |  |
| 8.7.1 | UE Measurement Procedures / E-UTRAN TDD - UTRAN TDD event triggered reporting under fading propagation conditions | Cell6 | Cell8 |  |  |  |
| 8.7.2 | UE Measurement Procedures / E-UTRAN TDD - UTRAN TDD cell search when DRX is used under fading propagation conditions | Cell6 | Cell8 |  |  |  |
| 8.7.3 | UE Measurement Procedures / E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting under AWGN propagation conditions | Cell3 | Cell9 |  |  |  |
| 8.7.4 | UE Measurement Procedures / E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions | Cell3 | Cell9 |  |  |  |
| 8.7.5 | E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX | Cell6 | (Note 11) | (Note 12) | (Note 13) |  |
| 8.7A.1 | E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX | Cell6 | (Note 11) | (Note 12) | (Note 13) |  |
| 8.8.1 | UE Measurement Procedures / E-UTRAN FDD - GSM event triggered reporting in AWGN | Cell6 | Cell26 |  |  |  |
| 8.8.2 | UE Measurement Procedures / E-UTRAN FDD - GSM event triggered reporting when DRX is used in AWGN | Cell6 | Cell26 |  |  |  |
| 8.9.1 | UE Measurement Procedures / E-UTRAN FDD - UTRAN TDD event triggered reporting under fading propagation conditions | Cell6 | Cell8 |  |  |  |
| 8.9.2 | UE Measurement Procedures / E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions | Cell6 | Cell8 |  |  |  |
| 8.10.1 | UE Measurement Procedures / E-UTRAN TDD - GSM event triggered reporting in AWGN | Cell6 | Cell26 |  |  |  |
| 8.10.2 | UE Measurement Procedures / E-UTRAN TDD - GSM event triggered reporting when DRX is used in AWGN | Cell6 | Cell26 |  |  |  |
| 8.11.1 | UE Measurement Procedures / Monitoring of multiple layers / E-UTRAN FDD - E-UTRAN FDD and E-UTRAN FDD Inter-frequency event triggered reporting under fading propagation conditions | Cell1 | Cell3 | Cell6 |  |  |
| 8.11.2 | UE Measurement Procedures / Monitoring of multiple layers / E-UTRAN TDD - E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions | Cell1 | Cell3 | Cell6 |  |  |
| 8.11.3 | UE Measurement Procedures / Monitoring of multiple layers / InterRAT E-UTRA FDD to E-UTRA FDD and UTRA FDD cell search | Cell1 | Cell6 | Cell8 |  |  |
| 8.11.4 | UE Measurement Procedures / Monitoring of multiple layers / InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search | Cell1 | Cell6 | Cell8 |  |  |
| 8.11.5 | UE Measurement Procedures / Combined E-UTRAN FDD - E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions | Cell6 | Cell3 | Cell24 |  |  |
| 8.11.6 | UE Measurement Procedures / Combined E-UTRAN TDD - E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions | Cell6 | Cell3 | Cell24 |  |  |
| 8.14.1 | UE Measurement Procedures / E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Cell10 | Cell1 |  |  |  |
| 8.14.2 | UE Measurement Procedures / E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells | Cell10 | Cell1 |  |  |  |
| 8.14.3 | UE Measurement Procedures / E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Cell10 | Cell1 |  |  |  |
| 8.15.1 | UE Measurement Procedures / E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Cell1 | Cell10 |  |  |  |
| 8.15.2 | UE Measurement Procedures / E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells | Cell1 | Cell10 |  |  |  |
| 8.15.3 | UE Measurement Procedures / E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Cell1 | Cell10 |  |  |  |
| 8.16.1 | UE Measurement Procedures / E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.2 | UE Measurement Procedures / E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.3 | UE Measurement Procedures / E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.4 | UE Measurement Procedures / E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.5 | UE Measurement Procedures / E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.6 | UE Measurement Procedures / E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.7 | UE Measurement Procedures / E-UTRAN FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.8 | UE Measurement Procedures / E-UTRAN TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.9 | E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.10 | E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.11 | E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.12 | E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.13 | E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.14 | E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.15 | E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5MHz+5MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.16 | E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5MHz+5MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.16.17 | E-UTRAN FDD activation and deactivation of known SCell in non-DRX | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 8.16.17A | E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz +20MHz bandwidth | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 8.16.18 | E-UTRAN TDD activation and deactivation of known SCell in non-DRX | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 8.16.18A | E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz +20MHz bandwidth | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 8.16.23 | UE Measurement Procedures / E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD | Cell1 | Cell10 | Cell30 |  |  |
| 8.16.24 | UE Measurement Procedures / E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD | Cell1 | Cell10 | Cell30 |  |  |
| 8.16.25 | UE Measurement Procedures / E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD | Cell1 | Cell10 | Cell30 |  |  |
| 8.16.26 | UE Measurement Procedures / E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD | Cell1 | Cell10 | Cell30 |  |  |
| 8.16.47 | 2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 8.16.48 | 2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Cell1 | Cell2 |  |  | Intra-band |
|  |  | Cell1 | Cell10 |  |  | Inter-band |
| 8.16.49 | 2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Cell1 | Cell10 |  |  |  |
| 8.16.50 | 2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Cell1 | Cell10 |  |  |  |
| 8.18.1 | UE Measurement Procedures / E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions | Cell1 | Cell15 |  |  |  |
| 8.19.1 | UE Measurement Procedures / E-UTRAN TDD-CDMA2000 1X event triggered reporting under fading propagation conditions | Cell1 | Cell19 |  |  |  |
| 8.20.1 | UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN FDD-FDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells | Cell6 | Cell3 Note6 | Cell13 |  | Intra-band |
|  |  | Cell6 | Cell3 | Cell10 Note4 |  | Inter-band |
| 8.20.2 | UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells | Cell6 | Cell3 Note6 | Cell13 |  | Intra-band |
|  |  | Cell6 | Cell3 | Cell10 Note4 |  | Inter-band |
| 8.20.2A | E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth | Cell6 | Cell3 Note6 | Cell13 |  | Intra-band |
|  |  | Cell6 | Cell3 | Cell10 Note4 |  | Inter-band |
| 8.20.3 | UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions | Cell1 | Cell8 | Cell4 |  | Intra-band |
|  |  | Cell1 | Cell8 | Cell10 |  | Inter-band |
| 8.20.4 | UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions | Cell1 | Cell8 | Cell4 |  | Intra-band |
|  |  | Cell1 | Cell8 | Cell10 |  | Inter-band |
| 8.20.4A | E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 20 MHz bandwidth | Cell1 | Cell8 | Cell4 |  | Intra-band |
|  |  | Cell1 | Cell8 | Cell10 |  | Inter-band |
| 8.22.1 | UE Measurement Procedures / E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal | Cell1 | Cell2 |  |  |  |
| 8.22.2 | UE Measurement Procedures / E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal | Cell1 | Cell2 |  |  |  |
| 8.22.3 | UE Measurement Procedures / E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal | Cell6 | Cell3 |  |  |  |
| 8.22.4 | UE Measurement Procedures / E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal | Cell6 | Cell3 |  |  |  |
| 8.22.5 | UE Measurement Procedures / E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Cell1 | Cell2 |  |  |  |
| 8.22.6 | UE Measurement Procedures / E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Cell1 | Cell2 |  |  |  |
| 8.22.7 | UE Measurement Procedures / E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Cell6 | Cell3 |  |  |  |
| 8.22.8 | UE Measurement Procedures / E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal | Cell6 | Cell3 |  |  |  |
| 8.22.9 | UE Measurement Procedures / E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.22.10 | UE Measurement Procedures / E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.22.11 | UE Measurement Procedures / E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.22.12 | UE Measurement Procedures / E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 8.23.1 | E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC | Cell1 | Cell10 |  |  |  |
| 8.23.2 | E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC | Cell1 | Cell10 |  |  |  |
| 8.23.3 | E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC | Cell1 | Cell10 |  |  |  |
| 8.23.4 | E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC | Cell6 | Cell3 | Cell10 Note4 |  |  |
| 8.23.5 | E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC | Cell6 | Cell3 | Cell10 Note4 |  |  |
| 8.23.6 | E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC | Cell6 | Cell3 | Cell10 Note4 |  |  |
| 8.23.7 | E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC | Cell1 | Cell10 |  |  |  |
| 8.23.8 | E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC | Cell1 | Cell10 |  |  |  |
| 8.23.9 | E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC | Cell1 | Cell10 |  |  |  |
| 8.25.1 | E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN | Cell1 | Cell2 (WLAN) |  |  |  |
| 8.25.2 | E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN | Cell1 | Cell2 (WLAN) |  |  |  |
| 8.26.1 | E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX | Cell1 | Cell10 |  |  | Inter-Band |
| 8.26.2 | E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX | Cell1 | Cell10 |  |  | Inter-Band |
| 8.26.3 | E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 8.26.4 | E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 8.26.5 | E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 8.26.6 | E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 8.26.7 | E-UTRAN FDD-FS3 intra-frequency event triggered reporting in DRX for CRS based discovery signal | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 8.26.8 | E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 8.26.9 | E-UTRAN FDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Cell1 | Cell10 |  |  |  |
| 8.26.10 | E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Cell1 | Cell10 |  |  |  |
| 9.1.1.1 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRP Accuracy / Absolute | Cell1 | Cell2 |  |  |  |
| 9.1.1.1_1 | Measurement Performance Requirements / E-UTRAN / FDD Intra Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Cell1 | Cell2 |  |  |  |
| 9.1.1.1_2 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRP Accuracy / Absolute for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 9.1.1.1_3 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRP Accuracy / Absolute for CA Idle Mode Measurements | Cell1 | Cell2 |  |  |  |
| 9.1.1.2 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRP Accuracy / Relative | Cell1 | Cell2 |  |  |  |
| 9.1.1.2_2 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRP Accuracy / Relative for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 9.1.2.1 | Measurement Performance Requirements / E-UTRAN / TDD Intra Frequency RSRP Accuracy / Absolute | Cell1 | Cell2 |  |  |  |
| 9.1.2.1_1 | Measurement Performance Requirements / E-UTRAN / TDD Intra Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Cell1 | Cell2 |  |  |  |
| 9.1.2.1_2 | Measurement Performance Requirements / E-UTRAN / TDD Intra Frequency RSRP Accuracy / Absolute for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 9.1.2.2 | Measurement Performance Requirements / E-UTRAN / TDD Intra Frequency RSRP Accuracy / Relative | Cell1 | Cell2 |  |  |  |
| 9.1.2.2_2 | Measurement Performance Requirements / E-UTRAN / TDD Intra Frequency RSRP Accuracy / Relative for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 9.1.3.1 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Absolute | Cell6 | Cell3 |  |  |  |
| 9.1.3.1_1 | Measurement Performance Requirements / E-UTRAN / FDD - FDD Inter Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Cell6 | Cell3 |  |  |  |
| 9.1.3.1_2 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Absolute for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.1.3.1_3 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Absolute for CA Idle Mode Measurements for Overlapping Carrier | Cell6 | Cell3 |  |  |  |
| 9.1.3.1_4 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Absolute for CA Idle Mode Measurements for Non-Overlapping Carrier | Cell6 | Cell3 |  |  |  |
| 9.1.3.2 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Relative | Cell6 | Cell3 |  |  |  |
| 9.1.3.2_1 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Relative (Rel-12 and forward) | Cell6 | Cell3 |  |  |  |
| 9.1.3.2_2 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Relative for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.1.4.1 | Measurement Performance Requirements / E-UTRAN / TDD Inter Frequency RSRP Accuracy / Absolute | Cell6 | Cell3 |  |  |  |
| 9.1.4.1_1 | Measurement Performance Requirements / E-UTRAN / TDD - TDD Inter Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Cell6 | Cell3 |  |  |  |
| 9.1.4.1_2 | Measurement Performance Requirements / E-UTRAN / TDD Inter Frequency RSRP Accuracy / Absolute for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.1.4.2 | Measurement Performance Requirements / E-UTRAN / TDD Inter Frequency RSRP Accuracy / Relative | Cell6 | Cell3 |  |  |  |
| 9.1.4.2_1 | Measurement Performance Requirements / E-UTRAN / TDD Inter Frequency RSRP Accuracy / Relative (Rel-12 and forward) | Cell6 | Cell3 |  |  |  |
| 9.1.4.2_2 | Measurement Performance Requirements / E-UTRAN / TDD Inter Frequency RSRP Accuracy / Relative for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.1.5.1 | Measurement Performance Requirements / E-UTRAN / FDD - TDD Inter frequency RSRP Accuracy / Absolute | Cell1 | Cell10 |  |  |  |
| 9.1.5.1_1 | Measurement Performance Requirements / E-UTRAN / FDD - TDD Inter Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Cell1 | Cell10 |  |  |  |
| 9.1.5.1_2 | Measurement Performance Requirements / E-UTRAN / FDD - TDD Inter frequency RSRP Accuracy / Absolute for UE Category 1bis | Cell1 | Cell10 |  |  |  |
| 9.1.5.2 | Measurement Performance Requirements / E-UTRAN / FDD-TDD Inter frequency RSRP Accuracy / Relative | Cell1 | Cell10 |  |  |  |
| 9.1.5.2_1 | Measurement Performance Requirements / E-UTRAN / FDD-TDD Inter frequency RSRP Accuracy / Relative (Rel-12 and forward) | Cell1 | Cell10 |  |  |  |
| 9.1.5.2_2 | Measurement Performance Requirements / E-UTRAN / FDD-TDD Inter frequency RSRP Accuracy / Relative for UE Category 1bis | Cell1 | Cell10 |  |  |  |
| 9.1.6.1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.6.1_1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.6.2 | Measurement Performance Requirements / FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.6.2_1 | Measurement Performance Requirements / FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation (Rel-12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.7.1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.7.1_1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation (Rel-12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.7.2 | Measurement Performance Requirements / TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.7.2_1 | Measurement Performance Requirements / TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation (Rel-12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.8.1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 9.1.8.1_1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) (Rel-12 and forward) | Cell1 | Cell2 |  |  |  |
| 9.1.8.2 | Measurement Performance Requirements / FDD Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 9.1.9.1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 9.1.9.1_1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) (Rel-12 and forward) | Cell1 | Cell2 |  |  |  |
| 9.1.9.2 | Measurement Performance Requirements / TDD Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 9.1.10.1 | Measurement Performance Requirements / FDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.1.10.1_1 | Measurement Performance Requirements / FDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Rel-12 and forward) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.1.10.2 | Measurement Performance Requirements / FDD Relative RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.1.11.1 | Measurement Performance Requirements / TDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.1.11.1_1 | Measurement Performance Requirements / TDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Rel-12 and forward) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.1.11.2 | Measurement Performance Requirements / TDD Relative RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.1.12.1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.12.1_1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.12.2 | Measurement Performance Requirements / FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.13.1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.13.1_1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel-12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.13.2 | Measurement Performance Requirements / TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.14.1 | FDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.1.14.1_1 | FDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Rel-12 and forward) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.1.14.2 | FDD Intra Frequency Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.1.15.1 | TDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.1.15.1_1 | TDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.1.15.2 | TDD Intra Frequency Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.1.16.1 | Measurement Performance Requirements / E-UTRAN / FDD Intra Frequency Absolute RSRP Accuracy for 5MHz Bandwidth | Cell1 | Cell2 |  |  |  |
| 9.1.16.1_1 | Measurement Performance Requirements / E-UTRAN / FDD Intra Frequency Absolute RSRP Accuracy for 5MHz Bandwidth (Rel-12 and forward) | Cell1 | Cell2 |  |  |  |
| 9.1.16.2 | Measurement Performance Requirements / E-UTRAN / FDD Intra Frequency Relative Accuracy of RSRP for 5MHz Bandwidth | Cell1 | Cell2 |  |  |  |
| 9.1.17.1 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRP Accuracy / Absolute for 5 MHz | Cell1 | Cell10 |  |  |  |
| 9.1.17.1_1 | Measurement Performance Requirements / E-UTRAN / FDD - FDD Inter Frequency Absolute RSRP Accuracy for 5MHz Bandwidth (Rel-12 and forward) | Cell1 | Cell10 |  |  |  |
| 9.1.17.2 | Measurement Performance Requirements / E-UTRAN / FDD - FDD Inter Frequency Relative Accuracy of RSRP for 5MHz Bandwidth | Cell1 | Cell10 |  |  |  |
| 9.1.17.2_1 | Measurement Performance Requirements / E-UTRAN / FDD - FDD Inter Frequency Relative Accuracy of RSRP for 5MHz Bandwidth (Rel-12 and forward) | Cell1 | Cell10 |  |  |  |
| 9.1.18.1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA for Carrier Aggregation for 10MHz + 5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.18.1_1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA for Carrier Aggregation for 10MHz + 5MHz (Rel 12 and forward) | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.18.2 | Measurement Performance Requirements / FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.19.1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.19.1_1 | TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz (Rel 12 and forward) | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.19.2 | Measurement Performance Requirements / TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz | Cell6 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.20.1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.20.1_1 | Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth (Rel 12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.20.2 | Measurement Performance Requirements / FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.21.1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.21.1_1 | Measurement Performance Requirements / TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz (Rel 12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.21.2 | Measurement Performance Requirements / TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.22 | FDD-TDD RSRP Accuracy E-UTRA for Carrier Aggregation with PCell in FDD | Cell1 | Cell10 | Cell30 |  |  |
| 9.1.23 | FDD-TDD RSRP Accuracy E-UTRA for Carrier Aggregation with PCell in TDD | Cell1 | Cell10 | Cell30 |  |  |
| 9.1.24.1_1 | TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz (Rel 12 and forward) | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.25 | Measurement Performance Requirements / FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Cell1 | Cell2 |  |  |  |
| 9.1.26 | Measurement Performance Requirements / TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Cell1 | Cell2 |  |  |  |
| 9.1.27 | Measurement Performance Requirements / FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Cell6 | Cell3 |  |  |  |
| 9.1.28 | Measurement Performance Requirements / TDD—TDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Cell6 | Cell3 |  |  |  |
| 9.1.29 | FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Cell1 | Cell2 |  |  |  |
| 9.1.30 | TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Cell1 | Cell2 |  |  |  |
| 9.1.31 | Measurement Performance Requirements / FDD-FDD inter frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Cell6 | Cell3 |  |  |  |
| 9.1.32 | Measurement Performance Requirements / TDD-TDD inter frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Cell6 | Cell3 |  |  |  |
| 9.1.33 | Measurement Performance Requirements / FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.34 | Measurement Performance Requirements / TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.35 | Measurement Performance Requirements / FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.36 | Measurement Performance Requirements / TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.41.1 | FD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Cell1 | Cell2 |  |  |  |
| 9.1.41.2 | FD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Cell1 | Cell2 |  |  |  |
| 9.1.42.1 | HD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Cell1 | Cell2 |  |  |  |
| 9.1.42.2 | HD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Cell1 | Cell2 |  |  |  |
| 9.1.43.1 | TDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Cell1 | Cell2 |  |  |  |
| 9.1.43.2 | TDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Cell1 | Cell2 |  |  |  |
| 9.1.52 | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 9.1.53 | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 9.1.54 | TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Cell1 | Cell2 |  |  |  |
| 9.1.55 | FDD intra frequency absolute and relative RSRP accuracies for SCell with frame structure 3 | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.57 | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 9.1.58 | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 9.1.59 | TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Cell1 | Cell2 |  |  |  |
| 9.1.60 | FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with FDD PCell | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.1.61 | FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with TDD PCell | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.1.1 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRQ Accuracy / Absolute | Cell1 | Cell2 |  |  |  |
| 9.2.1.1_2 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRQ Accuracy / Absolute for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 9.2.1.1_3 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRQ Accuracy / Absolute for CA Idle Mode Measurements | Cell1 | Cell2 |  |  |  |
| 9.2.2.1 | Measurement Performance Requirements / E-UTRAN / TDD Intra Frequency RSRQ Accuracy / Absolute | Cell1 | Cell2 |  |  |  |
| 9.2.2.1_2 | Measurement Performance Requirements / E-UTRAN / TDD Intra Frequency RSRQ Accuracy / Absolute for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 9.2.3.1 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRQ Accuracy / Absolute | Cell6 | Cell3 |  |  |  |
| 9.2.3.1_2 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRQ Accuracy / Absolute for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.2.3.1_3 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRQ Accuracy / Absolute for CA Idle Mode Measurements for Overlapping Carrier | Cell6 | Cell3 |  |  |  |
| 9.2.3.1_4 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRQ Accuracy / Absolute for CA Idle Mode Measurements for Non-Overlapping Carrier | Cell6 | Cell3 |  |  |  |
| 9.2.3.2 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRQ Accuracy / Relative | Cell6 | Cell3 |  |  |  |
| 9.2.3.2_2 | Measurement Performance Requirements / E-UTRAN / FDD Inter frequency RSRQ Accuracy / Relative for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.2.4.1 | Measurement Performance Requirements / E-UTRAN / TDD - TDD Inter Frequency RSRQ Accuracy / Absolute | Cell6 | Cell3 |  |  |  |
| 9.2.4.1_2 | Measurement Performance Requirements / E-UTRAN / TDD - TDD Inter Frequency RSRQ Accuracy / Absolute for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.2.4.2 | Measurement Performance Requirements / E-UTRAN / TDD - TDD Inter Frequency RSRQ Accuracy / Relative | Cell6 | Cell3 |  |  |  |
| 9.2.4.2_2 | Measurement Performance Requirements / E-UTRAN / TDD - TDD Inter Frequency RSRQ Accuracy / Relative for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 9.2.4A.1 | Measurement Performance Requirements / FDD - TDD Inter Frequency Absolute RSRQ Accuracy | Cell1 | Cell10 |  |  |  |
| 9.2.4A.1_2 | Measurement Performance Requirements / FDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis | Cell1 | Cell10 |  |  |  |
| 9.2.4A.2 | Measurement Performance Requirements / FDD - TDD Inter Frequency Relative Accuracy of RSRQ | Cell1 | Cell10 |  |  |  |
| 9.2.4A.2_2 | Measurement Performance Requirements / FDD - TDD Inter Frequency Relative Accuracy of RSRQ for UE Category 1bis | Cell1 | Cell10 |  |  |  |
| 9.2.5.1 | Measurement Performance Requirements / FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.5.2 | Measurement Performance Requirements / FDD Relative RSRQ Accuracy E-UTRA for Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.6.1 | Measurement Performance Requirements / TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.6.2 | Measurement Performance Requirements / TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.7.1 | Measurement Performance Requirements / FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 9.2.8.1 | Measurement Performance Requirements / TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Cell1 | Cell2 |  |  |  |
| 9.2.9.1 | Measurement Performance Requirements / FDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.2.10.1 | Measurement Performance Requirements / TDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Note 3) | Cell1 | Cell6 |  |  |  |
| 9.2.11.1 | Measurement Performance Requirements / FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.11.2 | Measurement Performance Requirements / FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.12.1 | Measurement Performance Requirements / TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.12.2 | Measurement Performance Requirements / TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.15.1 | FDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.2.16.1 | TDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Note 3) | Cell1 | Cell2 | Cell6 |  |  |
| 9.2.17.1 | Measurement Performance Requirements / E-UTRAN / FDD Intra frequency RSRQ Accuracy / Absolute for 5MHz | Cell1 | Cell2 |  |  |  |
| 9.2.18.1 | Measurement Performance Requirements / FDD - FDD Inter Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth | Cell1 | Cell10 |  |  |  |
| 9.2.18.2 | Measurement Performance Requirements / FDD - FDD Inter Frequency Relative Accuracy of RSRQ for 5MHz Bandwidth | Cell1 | Cell10 |  |  |  |
| 9.2.19.1 | Measurement Performance Requirements / FDD-FDD Inter Frequency absolute WB-RSRQ accuracy | Cell6 | Cell3 |  |  |  |
| 9.2.20.1 | Measurement Performance Requirements / TDD-TDD Inter Frequency absolute WB-RSRQ accuracy | Cell6 | Cell3 |  |  |  |
| 9.2.21.1 | Measurement Performance Requirements / FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.21.2 | Measurement Performance Requirements / FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.22.1 | Measurement Performance Requirements / TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.22.2 | Measurement Performance Requirements / TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.23.1 | Measurement Performance Requirements / FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.23.2 | Measurement Performance Requirements / FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.24.1 | Measurement Performance Requirements / TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.24.2 | Measurement Performance Requirements / TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.25.1 | Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD | Cell1 | Cell10 |  |  |  |
| 9.2.25.2 | Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD | Cell1 | Cell10 |  |  |  |
| 9.2.26.1 | Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD | Cell1 | Cell10 |  |  |  |
| 9.2.26.2 | Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD | Cell1 | Cell10 |  |  |  |
| 9.2.28 | Measurement Performance Requirements / FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal | Cell1 | Cell2 |  |  |  |
| 9.2.29 | Measurement Performance Requirements / TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal | Cell1 | Cell2 |  |  |  |
| 9.2.30 | Measurement Performance Requirements / FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal | Cell6 | Cell3 |  |  |  |
| 9.2.31 | Measurement Performance Requirements / TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal | Cell6 | Cell3 |  |  |  |
| 9.2.32 | Measurement Performance Requirements / FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.33 | Measurement Performance Requirements / TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal | Cell1 | Cell2 | Cell4 Note1 |  | Intra-band |
|  |  | Cell1 | Cell10 | Cell30 |  | Inter-band |
| 9.2.51 | FDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 9.2.52 | TDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 | Cell1 | Cell10 | Cell30 |  | Inter-Band |
| 9.3.1 | Measurement Performance Requirements / E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy | Cell3 | Cell9 |  |  |  |
| 9.3.2 | Measurement Performance Requirements / E-UTRAN TDD- UTRA FDD CPICH RSCP absolute accuracy | Cell3 | Cell9 |  |  |  |
| 9.3.3 | Measurement Performance Requirements / E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy for 5MHz bandwidth | Cell3 | Cell9 |  |  |  |
| 9.4.1 | Measurement Performance Requirements / E- UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy | Cell3 | Cell9 |  |  |  |
| 9.4.2 | Measurement Performance Requirements / E- UTRAN TDD - UTRA FDD CPICH Ec/No absolute accuracy | Cell3 | Cell9 |  |  |  |
| 9.4.3 | Measurement Performance Requirements / E-UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy for 5MHz bandwidth | Cell3 | Cell9 |  |  |  |
| 9.5.1 | Measurement Performance Requirements / E- UTRAN FDD - UTRA TDD P-CCPCH RSCP absolute accuracy | Cell3 | Cell9 |  |  |  |
| 9.5.2 | Measurement Performance Requirements / E- UTRAN TDD - UTRA TDD P-CCPCH RSCP absolute accuracy | Cell3 | Cell9 |  |  |  |
| 9.6.1 | Measurement Performance Requirements / GSM RSSI accuracy for E-UTRAN FDD | Cell1 | Cell26 | Cell24 |  |  |
| 9.6.2 | Measurement Performance Requirements / GSM RSSI accuracy for E-UTRAN TDD | Cell1 | Cell26 | Cell24 |  |  |
| 9.9.1.1 | Measurement Performance Requirements / FDD Intra Frequency Serving Cell Absolute RSRP Accuracy | Cell1 |  |  |  |  |
| 9.9.1.1_1 | Measurement Performance Requirements / FDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel-12 and forward) | Cell1 |  |  |  |  |
| 9.9.1.2 | Measurement Performance Requirements / FDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | Cell1 |  |  |  |  |
| 9.9.2.1 | Measurement Performance Requirements / TDD Intra Frequency Serving Cell Absolute RSRP Accuracy | Cell1 |  |  |  |  |
| 9.9.2.1_1 | Measurement Performance Requirements / TDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel-12 and forward) | Cell1 |  |  |  |  |
| 9.9.2.2 | Measurement Performance Requirements / TDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | Cell1 |  |  |  |  |
| 9.11.1 | FS3 average RSSI accuracy case (PCell using FDD) | Cell1 | Cell10 |  |  | Inter-Band |
| 9.11.2 | FS3 average RSSI accuracy case (PCell using TDD) | Cell1 | Cell10 |  |  | Inter-Band |
| 9.12.1 | FS3 channel occupancy test (PCell using FDD) | Cell1 | Cell10 |  |  | Inter-Band |
| 9.12.2 | FS3 channel occupancy test (PCell using TDD) | Cell1 | Cell10 |  |  | Inter-Band |
| 12.3.1 | V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority | Cell1 |  |  |  |  |
| 12.4 | Congestion Control Measurement Test for V2X UE | Cell1 |  |  |  |  |
| 12.5 | Interruptions due to V2X Sidelink Communication | Cell1 |  |  |  |  |
| NOTE 1: Neighbour cell uses same frequency as SCC. NOTE 2: "Intra-band" notation in the table refers to both intra-band contiguous and intra-band non-contiguous CA configuration as applicable. NOTE 3: All cells are intra frequency and shall use the same range (mid) independently of the range information given in 3GPP TS 36.508 Table 4.4.2-1. NOTE 4: Low range shall be used independently on the range information given in 3GPP TS 36.508 Table 4.4.2-1. NOTE 5: The frequency range remains unchanged after any change of Cell IDs in the test procedure. NOTE 6: Mid range shall be used for intra-band non-contiguous CA independently on the range information given in 3GPP TS 36.508 Table 4.4.2-1. NOTE 7: For operating band 66 "High Range" simulated cells for all test cases shall be configured according to "High Range" as defined in 3GPP TS36.508 Table 4.3.1.1.66 when UL is not required on the cell. Otherwise "Paired High Range" shall be configured. NOTE 8: 3 Cells are randomly selected from (Cell 1, Cell 3, Cell 10, Cell 32, Cell 33, Cell 34, Cell 35, Cell 36) NOTE 9: Cell is randomly selected from (Cell 33, Cell 34, Cell 35) NOTE 10: 2 Cells are randomly selected from (Cell 1, Cell 3, Cell 10, Cell 32, Cell 36) NOTE 11: Cell is randomly selected from (Cell 5, Cell 8, Cell 9) NOTE 12: 2 Cells are randomly selected from (Cell 37, Cell 38, Cell 39) NOTE 13: 2 Cells are randomly selected from (Cell 7, Cell 37, Cell 38, Cell 39) NOTE 14: Cell is randomly selected from (Cell 37, Cell 38, Cell 39) NOTE 15: Cell in reduced performance group NOTE 16: Cell is randomly selected from (Cell 7, Cell 37, Cell 38, Cell 39) |  |  |  |  |  |  |  |

Table E-2: Cell configuration mapping for 3DL CA RRM testing

| Test Case | 36.521-3 Cell1 |  |  |  |  | 36.521-3 Cell2 |  |  |  |  | 36.521-3 Cell3 |  |  |  | 36.521-3 Cell4 |  |  |  |  | 36.521-3 Cell5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.26.3A | Default Test Settings for a CA_XA-YC Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.16.83(generic duplex modes) | PCC |  |  |  | SCC1 |  |  |  |  | SCC2 |  |  |  |  | Neigh |  |  |  |  |  |  |  |
| 8.16.84(generic duplex modes) | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  | Band |  |  |  |  |  |  |  |
|  | X | Cell1 |  |  | Y |  |  | Cell2 |  | Y |  |  | Cell3 |  | Y |  | Cell4 |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  |  | SCC1 |  |  |  |  | SCC2 |  |  |  |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  | Band |  |  |  |  |  |  |  |
|  | X | Cell1 |  |  | X |  |  | Cell2 |  | Y |  |  | Cell3 |  | Y |  | Cell4 |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  |  | SCC1 |  |  |  |  | SCC2 |  |  |  |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  | Band |  |  |  |  |  |  |  |
|  | X | Cell1 |  |  | Y |  |  | Cell2 |  | Z |  |  | Cell3 |  | Z |  | Cell4 |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-XA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  |  | SCC1 |  |  |  |  | SCC2 |  |  |  |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  | Band |  |  |  |  |  |  |  |
|  | X | Cell1 |  |  | X |  |  | Cell2 |  | Y |  |  | Cell3 |  | Y |  | Cell4 |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  |  | SCC1 |  |  |  |  | SCC2 |  |  |  |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  | Band |  |  |  |  |  |  |  |
|  | X | Cell1 |  |  | Y |  |  | Cell2 |  | Y |  |  | Cell3 |  | Y |  | Cell4 |  |  |  |  |  |
|  | Default Test Settings for a CA_XD / CA_XA-XC / CA_XC-XA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  |  | SCC1 |  |  |  |  | SCC2 |  |  |  |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  | Band |  |  |  |  |  |  |  |
|  | X | Cell1 |  |  | X |  |  | Cell2 |  | X |  |  | Cell3 |  | X |  | Cell4 |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  |  | SCC1 |  |  |  |  | Neigh |  |  |  |  | SCC2 |  |  |  |  | Neigh |  |  |
|  | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  | Band |  |  |  |  | Band |  |  |
| 9.1.68(generic duplex modes) Note 5 | X | Cell1 |  |  | Y |  |  | Cell2 |  | Y |  |  | Cell3 |  | Y |  | Cell4 |  |  | Y | Cell6 |  |
|  | Default Test Settings for a CA_XC-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  |  |  |  | Neigh |  |  |  |  |  | SCC2 |  |  |  | Neigh |  |  |
|  | Band |  |  | Band |  |  |  |  |  | Band |  |  |  |  |  | Band |  |  |  | Band |  |  |
|  | X | Cell1 |  | X |  |  | Cell2 |  |  | X |  |  | Cell3 |  |  | Y |  |  | Cell4 | Y | Cell6 |  |
|  | Default Test Settings for a XA-YA-ZA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  |  | SCC1 |  |  |  |  | Neigh |  |  |  |  | SCC2 |  |  |  |  | Neigh |  |  |
|  | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  | Band |  |  |  |  | Band |  |  |
|  | X | Cell1 |  |  | Y |  |  | Cell2 |  | Y |  |  | Cell3 |  | Z |  | Cell4 |  |  | Z | Cell6 |  |
|  | Default Test Settings for a XA-XA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  |  |  |  | Neigh |  |  |  |  |  | SCC2 |  |  |  | Neigh |  |  |
|  | Band |  |  | Band |  |  |  |  |  | Band |  |  |  |  |  | Band |  |  |  | Band |  |  |
|  | X | Cell1 |  | X |  |  | Cell2 |  |  | X |  |  | Cell3 |  |  | Y |  |  | Cell4 | Y | Cell6 |  |
|  | Default Test Settings for a XA-YA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  |  | SCC1 |  |  |  |  | Neigh |  |  |  |  | SCC2 |  |  |  |  | Neigh |  |  |
|  | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  | Band |  |  |  |  | Band |  |  |
|  | X | Cell1 |  |  | Y |  |  | Cell2 |  | Y |  |  | Cell3 |  | Y |  | Cell4 |  |  | Y | Cell6 |  |
|  | Default Test Settings for a CA_XD/CA_XA-XC/CA_XC-XA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  |  | SCC1 |  |  |  |  |  | Neigh |  |  | SCC2 |  |  |  |  |  | Neigh |  |  |
|  | Band |  |  |  | Band |  |  |  |  |  | Band |  |  | Band |  |  |  |  |  | Band |  |  |
|  | X |  | Cell1 |  | X |  |  |  | Cell2 |  | X | Cell3 |  | X |  |  |  | Cell4 |  | X |  | Cell6 |
|  | Default Test Settings for a CA_XA-YC Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.16.85(generic duplex modes) Note 3 | PCC |  |  |  | SCC1 |  |  |  |  | SCC2 |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.16.86(generic duplex modes) Note 3 9.2.55(generic duplex modes) Note 5 | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | Cell1 |  |  | Y |  |  | Cell2 |  | Y |  |  | Cell3 |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  |  | SCC1 |  |  |  |  | SCC2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | Cell1 |  |  | X |  |  | Cell2 |  | Y |  |  | Cell3 |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  |  | SCC1 |  |  |  |  | SCC2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | Cell1 |  |  | Y |  |  | Cell2 |  | Z |  |  | Cell3 |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-XA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  |  | SCC1 |  |  |  |  | SCC2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.2.9 | X | Cell1 |  |  | X |  |  | Cell2 |  | Y |  |  | Cell3 |  |  |  |  |  |  |  |  |  |
| 7.1.13 | Default Test Settings for a CA_XA-YA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  |  | SCC1 |  |  |  |  | SCC2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | Cell1 |  |  | Y |  |  | Cell2 |  | Y |  |  | Cell3 |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD / CA_XA-XC / CA_XC-XA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  |  | SCC1 |  |  |  |  | SCC2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  |  |  | Band |  |  |  |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | Cell1 |  |  | X |  |  | Cell2 |  | X |  |  | Cell3 |  |  |  |  |  |  |  |  |  |
| NOTE 1: All cells shall use the same range (mid) independently of the range information given in Table 4.4.2-1 of TS 36.508 [7]. NOTE 2: X, Y and Z correspond to the different bands in the CA Configuration is specified in 5.4.2A.1 of TS 36.521-1 [10]. E.g. for CA_1A-3A-19A, X=1, Y=3, Z=19. For PCell in FDD CA Configuration, FDD bands shall precede TDD bands. E.g. for CA_13A-48A-66A, X=13, Y=66, Z=48. For PCell in TDD CA Configuration, TDD bands shall precede FDD bands. E.g. for CA_13A-48A-66A, X=48, Y=13, Z=66. NOTE 3: For intra band contiguous CA, CC addition shall be done in increasing frequency order to ensure Contiguous CC addition. NOTE 4: For simplicity, this table does not consider CA Type B. For CA Type B, the corresponding Type C default test settings shall be used. For example, for CA_XB_YA, the entry for CA_XC_YA shall be used. NOTE 5: These test cases require to swap the bands so that every band in the CA Configuration is used as PCC. In the most generic case XA-YA-ZA, the test case shall be run three times: CA_XA-YA-ZA, CA_YA-XA-ZA and CA_ZA-XA-YA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table E-3: Cell configuration mapping for 2DL CA 20MHz+10MHz RRM testing

| Test Case |  | 36.521-3 Cell1 |  |  | 36.521-3 Cell2 |  |  | 36.521-3 Cell3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | SCC1 |  |  | Neigh |  |  |  |
|  | Band |  |  | Band |  |  | Band |  |  |  |
|  | X | Cell1 |  | Y |  | Cell10 | Y | Cell30 |  |  |
|  | Y | Cell1 |  | X |  | Cell10 | X | Cell30 |  |  |
| 8.16.21 | Default Test Settings for a CA_XC Configuration |  |  |  |  |  |  |  |  |  |
| 8.16.22 | PCC |  |  | Neigh |  |  | SCC1 |  |  |  |
|  | Band |  |  | Band |  |  | Band |  |  |  |
|  | X | Cell1 |  | X |  | Cell2 | X | Cell4 Note1 |  |  |
|  | Default Test Settings for a CA_XA-XA Configuration |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | Neigh |  |  | SCC1 |  |  |  |
|  | Band |  |  | Band |  |  | Band |  |  |  |
|  | X | Cell1 |  | X |  | Cell2 | X | Cell4 Note1 |  |  |
|  | Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | Neigh |  |  | SCC1 |  |  |  |
|  | Band |  |  | Band |  |  | Band |  |  |  |
|  | X | Cell6 |  | Y | Cell3 |  | Y | Cell10 Note5 |  |  |
|  | Y | Cell6 |  | X | Cell3 |  | X | Cell10 Note5 |  |  |
| 8.20.2B | Default Test Settings for a CA_XC Configuration |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | Neigh |  |  | SCC1 |  |  |  |
|  | Band |  |  | Band |  |  | Band |  |  |  |
|  | X | Cell6 |  | X | Cell3 |  | X | Cell13 |  |  |
|  | Default Test Settings for a CA_XA-XA Configuration |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | Neigh |  |  | SCC1 |  |  |  |
|  | Band |  |  | Band |  |  | Band |  |  |  |
|  | X | Cell6 |  | X | Cell3 Note6 |  | X | Cell13 |  |  |
|  | Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | Neigh (UTRA) |  |  | SCC1 |  |  |  |
|  | Band |  |  | Band |  |  | Band |  |  |  |
|  | X | Cell1 |  | Y | Cell8 |  | Y | Cell10 |  |  |
|  | Y | Cell1 |  | X | Cell8 |  | X | Cell10 |  |  |
| 8.20.4B | Default Test Settings for a CA_XC Configuration |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | Neigh |  |  | SCC1 |  |  |  |
|  | Band |  |  | Band |  |  | Band |  |  |  |
|  | X | Cell1 |  | X | Cell8 |  | X | Cell4 |  |  |
|  | Default Test Settings for a CA_XA-XA Configuration |  |  |  |  |  |  |  |  |  |
|  | PCC |  |  | Neigh |  |  | SCC1 |  |  |  |
|  | Band |  |  | Band |  |  | Band |  |  |  |
|  | X | Cell1 |  | X | Cell8 |  | X | Cell4 |  |  |
|  |  | Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |  |  |
|  |  | PCC |  |  | SCC1 |  |  | Neigh |  |  |
|  |  | Band |  |  | Band |  |  | Band |  |  |
| 9.1.24.1 |  | X |  | Cell1 | Y | Cell10 |  | Y |  | Cell30 |
| 9.1.24.2 |  | Y |  | Cell1 | X | Cell10 |  | X |  | Cell30 |
| 9.2.27.1 |  | Default Test Settings for a CA_XC Configuration |  |  |  |  |  |  |  |  |
| 9.2.27.2 |  | PCC |  |  | SCC1 |  |  | Neigh |  |  |
|  |  | Band |  |  | Band |  |  | Band |  |  |
|  |  | X |  | Cell1 | X | Cell2 |  | X |  | Cell4 Note1 |
|  |  | Default Test Settings for a CA_XA-XA Configuration |  |  |  |  |  |  |  |  |
|  |  | PCC |  |  | SCC1 |  |  | Neigh |  |  |
|  |  | Band |  |  | Band |  |  | Band |  |  |
|  |  | X |  | Cell1 | X | Cell2 |  | X |  | Cell4 Note1 |
| NOTE 1: Neighbour cell uses same frequency as SCC. NOTE 2: X, Y correspond to the different bands in the CA Configuration is specified in 5.4.2A.1 of TS 36.521-1 [10]. E.g. for CA_1A-3A, X=1, Y=3. NOTE 3: For intra-band contiguous and intra-band non-contiguous CA, 20 MHz for PCC, 10MHz for SCC are tested. NOTE 4: For inter-band CA configuration, 20 MHz for band X and 10 MHz for band Y are tested if X supports 20 MHz and Y supports 10 MHz. Otherwise, 10 MHz for band X and 20 MHz for band Y are tested. NOTE 5: Low range shall be used independently on the range information given in 3GPP TS 36.508 Table 4.4.2-1. NOTE 6: Mid range shall be used independently on the range information given in 3GPP TS 36.508 Table 4.4.2-1. NOTE 7: For simplicity, this table does not consider CA Type B. For CA Type B, the corresponding Type C default test settings shall be used. For example, for CA_XB, the entry for CA_XC shall be used. |  |  |  |  |  |  |  |  |  |  |

Table E-4: Ncell and Cell configuration mapping for RRM testing of Category NB1

| Test Case | Description | 36.521-3 Ncell1 | 36.521-3 Ncell2 | 36.521-3 Cell1 | 36.521-3 Cell2 |
| --- | --- | --- | --- | --- | --- |
| 4.2.18 | HD - FDD Intra frequency case for UE category NB1 In-Band mode in normal coverage | Ncell18 | Ncell5 | Cell1 | Cell11 |
| 4.2.19 | HD - FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage | Ncell18 | Ncell5 | Cell1 | Cell11 |
| 4.2.24 | HD - FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage | Ncell18 | Ncell6a | Cell1 |  |
| 4.2.35 | E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage | Ncell18 | Ncell5 | Cell1 | Cell11 |
| 4.2.36 | E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage | Ncell18 | Ncell5 | Cell1 | Cell11 |
| 4.2.37 | E-UTRAN TDD - TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage | Ncell18 | Ncell6a | Cell1 |  |
| 4.2.38 | HD - FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation | Ncell18 | Ncell5 | Cell1 | Cell11 |
| 6.1.15 | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 under normal coverage | Ncell1 | Ncell2 | Cell1 | Cell2 |
| 6.1.16 | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 under enhanced coverage | Ncell18 | Ncell1 | Cell1 |  |
| 6.1.23 | E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage | Ncell18 | Ncell1 | Cell1 |  |
| 6.1.24 | E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage | Ncell18 | Ncell1 | Cell1 | Cell2 |
| 6.2.16 | Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage | Ncell18 |  | Cell1 |  |
| 6.2.17 | Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage | Ncell18 | Ncell1 | Cell1 |  |
| 6.2.18 | Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage | Ncell18 |  | Cell1 |  |
| 6.2.19 | TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage | Ncell18 |  | Cell1 |  |
| 6.2.20 | TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage | Ncell18 | Ncell1 | Cell1 |  |
| 6.2.21 | TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage | Ncell18 |  | Cell1 |  |
| 7.1.17 | E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage | Ncell18 |  | Cell1 |  |
| 7.1.18 | E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage | Ncell18 |  | Cell1 |  |
| 7.1.27 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage | Ncell18 |  | Cell1 |  |
| 7.1.28 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage | Ncell18 |  | Cell1 |  |
| 7.2.9 | HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage | Ncell1 |  |  |  |
| 7.2.15 | E-UTRAN TDD - TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage | Ncell1 |  |  |  |
| 7.3.60 | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage | Ncell18 |  | Cell1 |  |
| 7.3.61 | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in Enhanced Coverage | Ncell18 |  | Cell1 |  |
| 7.3.62 | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage | Ncell18 |  | Cell1 |  |
| 7.3.63 | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage | Ncell18 |  | Cell1 |  |
| 7.3.64 | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage | Ncell18 |  | Cell1 |  |
| 7.3.65 | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage | Ncell18 |  | Cell1 |  |
| 7.3.66 | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | Ncell18 |  | Cell1 |  |
| 7.3.67 | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage | Ncell18 |  | Cell1 |  |
| 7.3.88 | TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage | Ncell18 |  | Cell1 |  |
| 7.3.89 | TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage | Ncell18 |  | Cell1 |  |
| 7.3.90 | TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage | Ncell18 |  | Cell1 |  |
| 7.3.91 | TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage | Ncell18 |  | Cell1 |  |
| 7.3.92 | TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage | Ncell18 |  | Cell1 |  |
| 7.3.93 | TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage | Ncell18 |  | Cell1 |  |
| 7.3.94 | TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | Ncell18 |  | Cell1 |  |
| 7.3.95 | TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage | Ncell18 |  | Cell1 |  |

Table E-5: Cell configuration mapping for 4DL CA RRM testing

| Test Case | 36.521-3 Cell1 |  | 36.521-3 Cell2 |  | 36.521-3 Cell3 |  | 36.521-3 Cell4 |  | 36.521-3 Cell5 |  | 36.521-3 Cell6 |  | 36.521-3 Cell7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.16.87 (generic duplex modes) 8.16.88 (generic duplex modes) | Default Test Settings for a CA_XA-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XD-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XC-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Z | 4 | Z | 6 |  |  |  |  |
|  | Default Test Settings for a CA_XE |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | Neigh |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 |  |  |  |  |
| 9.1.69(generic duplex modes) | Default Test Settings for a CA_XA-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | R | 10 | R | 11 |
|  | Default Test Settings for a CA_XA-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Y | 10 | Y | 11 |
|  | Default Test Settings for a CA_XD-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | Y | 10 | Y | 11 |
|  | Default Test Settings for a CA_XC-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Y | 10 | Y | 11 |
|  | Default Test Settings for a CA_XA-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | Z | 10 | Z | 11 |
|  | Default Test Settings for a CA_XA-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Z | 10 | Z | 11 |
|  | Default Test Settings for a CA_XC-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Z | 10 | Z | 11 |
|  | Default Test Settings for a CA_XE |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | X | 10 | X | 11 |
| 8.16.89(generic duplex modes) Note 5 8.16.90(generic duplex modes) Note 5 | Default Test Settings for a CA_XA-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Z | 3 | Z | 4 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 |  |  |  |  |  |  |
| NOTE 1: All cells shall use the same range (mid) independently of the range information given in Table 4.4.2-1 of TS 36.508 [7]. NOTE 2: X, Y, Z and R correspond to the bands in the CA Configuration is specified in 5.4.2A.1 of TS 36.521-1 [10]. E.g. for CA_2A-2A-4A-5A, X=2, Y=2, Z=4, R=5. For PCell in FDD CA Configuration, FDD bands shall precede TDD bands. E.g. for CA_2A-2A-42A-66A, X=2, Y=2, Z=66, R=42. For PCell in TDD CA Configuration, TDD bands shall precede FDD bands. E.g. for CA_2A-2A-42A-66A, X=42, Y=2, Z=2, R=66. NOTE 3: These test cases require to swap the bands so that every band in the CA Configuration is used as PCC. In the most generic case XA-YA-ZA-RA, the test case shall be run four times: CA_XA-YA-ZA-RA, CA_YA-ZA-RA-XA, CA_ZA-RA- XA-YA and CA_RA- XA-YA-ZA. NOTE 4: For simplicity, this table does not consider CA Type B. For CA Type B, the corresponding Type C default test settings shall be used. For example, for CA_XB-YA-ZA, the entry for CA_XC-YA-ZA shall be used. NOTE 5: For intra band contiguous CA, CC addition shall be done in increasing frequency order to ensure Contiguous CC addition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table E-6: Cell configuration mapping for 5DL CA RRM testing

| Test Case | 36.521-3 Cell1 |  | 36.521-3 Cell2 |  | 36.521-3 Cell3 |  | 36.521-3 Cell4 |  | 36.521-3 Cell5 |  | 36.521-3 Cell6 |  | 36.521-3 Cell7 |  | 36.521-3 Cell8 |  | 36.521-3 Cell9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.16.91 (generic duplex modes) 8.16.92 (generic duplex modes) | Default Test Settings for a CA_XA-YA-ZA-RA-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | S | 6 | S | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Y | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 6 | Y | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 6 | Y | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Y | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 6 | Z | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 6 | Z | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Z | 6 | Z | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | Z | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Z | 4 | Z | 6 | Z | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Z | 6 | Z | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 6 | R | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | R | 6 | R | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | R | 6 | R | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Z | 4 | R | 6 | R | 11 |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | Neigh |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | X | 11 |  |  |  |  |  |  |
| 8.16.93(generic duplex modes) Note 5 8.16.94(generic duplex modes) Note 5 9.2.57(generic duplex modes) | Default Test Settings for a CA_XA-YA-ZA-RA-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | S | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Z | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Z | 4 | Z | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Z | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | R | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | R | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Z | 4 | R | 6 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 |  |  |  |  |  |  |  |  |
| 9.1.70(generic duplex modes) | Default Test Settings for a CA_XA-YA-ZA-RA-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | R | 10 | R | 11 | S | 12 | S | 14 |
|  | Default Test Settings for a CA_XA-YE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Y | 14 |
|  | Default Test Settings for a CA_XE-YA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | X | 10 | X | 11 | Y | 12 | Y | 14 |
|  | Default Test Settings for a CA_XC-YD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Y | 14 |
|  | Default Test Settings for a CA_XD-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | Y | 10 | Y | 11 | Y | 12 | Y | 14 |
|  | Default Test Settings for a CA_XA-YA-ZD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 14 |
|  | Default Test Settings for a CA_XA-YD-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Y | 10 | Y | 11 | Z | 12 | Z | 14 |
|  | Default Test Settings for a CA_XD-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | Y | 10 | Y | 11 | Z | 12 | Z | 14 |
|  | Default Test Settings for a CA_XA-YC-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 14 |
|  | Default Test Settings for a CA_XC-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 14 |
|  | Default Test Settings for a CA_XC-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Y | 10 | Y | 11 | Z | 12 | Z | 14 |
|  | Default Test Settings for a CA_XA-YA-ZA-RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | R | 10 | R | 11 | R | 12 | R | 14 |
|  | Default Test Settings for a CA_XA-YA-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 6 | Z | 10 | Z | 11 | R | 12 | R | 14 |
|  | Default Test Settings for a CA_XA-YC-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 6 | Z | 10 | Z | 11 | R | 12 | R | 14 |
|  | Default Test Settings for a CA_XC-YA-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 6 | Z | 10 | Z | 11 | R | 12 | R | 14 |
|  | Default Test Settings for a CA_XF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 6 | X | 10 | X | 11 | X | 12 | X | 14 |
| NOTE 1: All cells shall use the same range (mid) independently of the range information given in Table 4.4.2-1 of TS 36.508 [7]. NOTE 2: X, Y, Z, R and S correspond to the bands in the CA Configuration is specified in 5.4.2A.1 of TS 36.521-1 [10]. E.g. for CA_2A-2A-4A-4A-5A, X=2, Y=2, Z=4, R=4, S=5. For PCell in FDD CA Configuration, FDD bands shall precede TDD bands. E.g. for CA_2A-2A-41A-42A-66A, X=2, Y=2, Z=66, R=41, S=42. For PCell in TDD CA Configuration, TDD bands shall precede FDD bands. E.g. for CA_2A-2A-41A-42A-66A, X=41, Y=42, Z=2, R=2, S=66. NOTE 3: These test cases require to swap the bands so that every band in the CA Configuration is used as PCC. In the most generic case CA_XA-YA-ZA-RA-SA, the test case shall be run five times: CA_XA-YA-ZA-RA-SA, CA_YA-ZA-RA-SA-XA, CA_ZA-RA-SA-XA-YA, CA_RA-SA-XA-YA-ZA and CA_SA-XA-YA-ZA-RA. NOTE 4: For simplicity, this table does not consider CA Type B. For CA Type B, the corresponding Type C default test settings shall be used. For example, for CA_XB-YA-ZA-RA, the entry for CA_XC-YA-ZA-RA shall be used. NOTE 5: For intra-band contiguous CA, CC addition shall be done in increasing frequency order to ensure Contiguous CC addition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table E-7: Cell configuration mapping for 6DL CA RRM testing

| Test Case | 36.521-3 Cell1 |  | 36.521-3 Cell2 |  | 36.521-3 Cell3 |  | 36.521-3 Cell4 |  | 36.521-3 Cell5 |  | 36.521-3 Cell6 |  | 36.521-3 Cell7 |  | 36.521-3 Cell8 |  | 36.521-3 Cell9 |  | 36.521-3 Cell10 |  | 36.521-3 Cell11 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.16.95 8.16.96 | Default Test Settings for a CA_XA-YA-ZA-RA-SA-TA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | S | 1a | T | 6 | T | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 1a | Y | 6 | Y | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 1a | R | 6 | R | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZD-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | R | 6 | R | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 1a | R | 6 | R | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YD-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YC-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Z | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC-RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | R | 1a | R | 6 | R | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | R | 11 |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RC-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | Neigh |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 1a | S | 6 | S | 11 |  |  |  |  |  |  |  |  |
| 8.16.97 8.16.98 9.2.58 | Default Test Settings for a CA_XA-YA-ZA-RA-SA-TA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | S | 1a | T | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 1a | Y | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 1a | R | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZD-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | R | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 1a | R | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YD-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XD-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YC-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Z | 1a | Z | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC-RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | R | 1a | R | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RC-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 1a | S | 6 |  |  |  |  |  |  |  |  |  |  |
| 9.1.71 | Default Test Settings for a CA_XA-YA-ZA-RA-SA-TA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | R | 10 | S | 11 | S | 12 | T | 14 | T | 23 |
|  | Default Test Settings for a CA_XA-YF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Y | 14 | Y | 23 |
|  | Default Test Settings for a CA_XA-YA-ZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XA-YE-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1 | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XE-YA-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 1a | X | 6 | X | 10 | Y | 11 | Y | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XC-YE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 1a | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Y | 14 | Y | 23 |
|  | Default Test Settings for a CA_XE-YC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 1a | X | 6 | X | 10 | Y | 11 | Y | 12 | Y | 14 | Y | 23 |
|  | Default Test Settings for a CA_XA-YA-ZA-RD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | R | 10 | R | 11 | R | 12 | R | 14 | R | 23 |
|  | Default Test Settings for a CA_XA-YA-ZD-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | R | 14 | R | 23 |
|  | Default Test Settings for a CA_XA-YD-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Z | 11 | Z | 12 | R | 14 | R | 23 |
|  | Default Test Settings for a CA_XA-YC-ZD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XA-YD-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Z | 11 | Z | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XC-YD-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XD-YC-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 1a | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XC-YC-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Z | 11 | Z | 12 | Z | 14 | Z | 23 |
|  | Default Test Settings for a CA_XA-YA-ZC-RC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 10 | R | 11 | R | 12 | R | 14 | R | 23 |
|  | Default Test Settings for a CA_XA-YC-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | R | 14 | R | 23 |
|  | Default Test Settings for a CA_XA-YA-ZA-RC-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | R | 10 | R | 11 | R | 12 | S | 14 | S | 23 |
| NOTE 1: All cells shall use the same range (mid) independently of the range information given in Table 4.4.2-1 of TS 36.508 [7]. NOTE 2: X, Y, Z, R, S and T correspond to the bands in the CA Configuration is specified in 5.4.2A.1 of TS 36.521-1 [10]. E.g. for CA_1A-3A-3A-7A-7A-8A, X=1, Y=3, Z=3, R=7, S=7, T=8. For PCell in FDD CA Configuration, FDD bands shall precede TDD bands. E.g. for CA_2A-2A-41A-42A-66A-66A, X=2, Y=2, Z=66, R=66, S=41, T=42. For PCell in TDD CA Configuration, TDD bands shall precede FDD bands. E.g. for CA_2A-2A-41A-42A-66A-66A, X=41, Y=42, Z=2, R=2, S=66, T=66. NOTE 3: For simplicity, this table does not consider CA Type B. For CA Type B, the corresponding Type C default test settings shall be used. For example, for CA_XB-YE, the entry for CA_XC-YE shall be used. NOTE 4: For intra-band contiguous CA, CC addition shall be done in increasing frequency order to ensure Contiguous CC addition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table E-8: Cell configuration mapping for 7DL CA RRM testing

| Test Case | 36.521-3 Cell1 |  | 36.521-3 Cell2 |  | 36.521-3 Cell3 |  | 36.521-3 Cell4 |  | 36.521-3 Cell5 |  | 36.521-3 Cell6 |  | 36.521-3 Cell7 |  | 36.521-3 Cell8 |  | 36.521-3 Cell9 |  | 36.521-3 Cell10 |  | 36.521-3 Cell11 |  | 36.521-3 Cell12 |  | 36.521-3 Cell13 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.16.99 8.16.100 | Default Test Settings for a CA_XA-YA-ZA-RA-SA-TA-UA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | S | 1a | T | 6 | T | 11 | U | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 11 | Z | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 11 | Z | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 11 | Z | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YE-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Z | 11 | Z | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 1a | Z | 6 | Z | 11 | Z | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 1a | R | 6 | R | 11 | R | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZE-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | Z | 6 | R | 11 | R | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZD-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | R | 11 | R | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 1a | Z | 6 | R | 11 | R | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | R | 11 | R | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC-RC-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | S | 11 | S | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZD-RA-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  | Neigh |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | R | 6 | S | 11 | S | 14 |  |  |  |  |  |  |  |  |  |  |
| 8.16.102 8.16.101 9.2.59 | Default Test Settings for a CA_XA-YA-ZA-RA-SA-TA-UA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | S | 1a | T | 6 | T | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XC-YE-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Z | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XE-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | Y | 1a | Z | 6 | Z | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZA-RE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | R | 4 | R | 1a | R | 6 | R | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZE-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | Z | 6 | R | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YC-ZD-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | R | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YD-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Z | 1a | Z | 6 | R | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YE-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | R | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZC-RC-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | S | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Default Test Settings for a CA_XA-YA-ZD-RA-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  | SCC4 |  | SCC5 |  | SCC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | 1 | Y | 2 | Z | 3 | Z | 4 | Z | 1a | R | 6 | S | 11 |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.72 | Default Test Settings for a CA_XA-YA-ZA-RA-SA-TA-UA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | R | 10 | S | 11 | S | 12 | T | 13 | T | 14 | U | 23 | U | 30 |
|  | Default Test Settings for a CA_XA-YA-ZF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 13 | Z | 14 | Z | 23 | Z | 30 |
|  | Default Test Settings for a CA_XA-YC-ZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 13 | Z | 14 | Z | 23 | Z | 30 |
|  | Default Test Settings for a CA_XA-YE-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Z | 13 | Z | 14 | Z | 23 | Z | 30 |
|  | Default Test Settings for a CA_XC-YE-ZA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Y | 13 | Y | 14 | Z | 23 | Z | 30 |
|  | Default Test Settings for a CA_XE-YA-ZC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | X | 2 | X | 3 | X | 4 | X | 1a | X | 6 | X | 10 | Y | 11 | Y | 12 | Z | 13 | Z | 14 | Z | 23 | Z | 30 |
|  | Default Test Settings for a CA_XA-YA-ZA-RE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | R | 6 | R | 10 | R | 11 | R | 12 | R | 13 | R | 14 | R | 23 | R | 30 |
|  | Default Test Settings for a CA_XA-YA-ZE-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 13 | Z | 14 | R | 23 | R | 30 |
|  | Default Test Settings for a CA_XA-YC-ZD-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | Z | 13 | Z | 14 | R | 23 | R | 30 |
|  | Default Test Settings for a CA_XA-YD-ZC-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Z | 11 | Z | 12 | Z | 13 | Z | 14 | R | 23 | R | 30 |
|  | Default Test Settings for a CA_XA-YE-ZA-RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Y | 4 | Y | 1a | Y | 6 | Y | 10 | Y | 11 | Y | 12 | Z | 13 | Z | 14 | R | 23 | R | 30 |
|  | Default Test Settings for a CA_XA-YA-ZC-RC-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 10 | R | 11 | R | 12 | R | 13 | R | 14 | S | 23 | S | 30 |
|  | Default Test Settings for a CA_XA-YA-ZD-RA-SA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PCC |  | SCC1 |  | Neigh |  | SCC2 |  | Neigh |  | SCC3 |  | Neigh |  | SCC4 |  | Neigh |  | SCC5 |  | Neigh |  | SCC6 |  | Neigh |  |
|  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  | Band |  |
|  | X | 1 | Y | 2 | Y | 3 | Z | 4 | Z | 1a | Z | 6 | Z | 10 | Z | 11 | Z | 12 | R | 13 | R | 14 | S | 23 | S | 30 |
| NOTE 1: All cells shall use the same range (mid) independently of the range information given in Table 4.4.2-1 of TS 36.508 [7]. NOTE 2: X, Y, Z, R, S, T and U correspond to the bands in the CA Configuration is specified in 5.4.2A.1 of TS 36.521-1 [10]. E.g. for CA_1A-1A-3C-7C-28, X=1, Y=1, Z=3, R=3, S=7, T=7, U=28. For PCell in FDD CA Configuration, FDD bands shall precede TDD bands. E.g. for CA_25A-25A-41F, X=25, Y=25, Z=41, R=41, S=41, T=41, U=41. For PCell in TDD CA Configuration, TDD bands shall precede FDD bands. E.g. for CA_25A-25A-41F, X=41, Y=41, Z=41, R=41, S=41, T=25, U=25. NOTE 3: For intra-band contiguous CA, CC addition shall be done in increasing frequency order to ensure Contiguous CC addition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

///End of changes