3GPP TSG-RAN5 Meeting #96-e	R5-225213
Electronic Meeting, 15th August – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1946 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on number of HARQ processes for FDD RRM test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current number of HARQ processes for FDD is 8 based on Table C.1.3-2. The combination of 8 HARQ processes and K1 = 7 for SCS 15 kHz could be insuficient for some scenarios. UCI will be received just in the previous slot where the HARQ process need to re-transmitted. Hence, it could cause on a missing transmission for correspoding HARQ process |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated number of HARQ process for FDD to 16 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC will unfairly fail a conformance UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## C.1.3	Default connection setup

Table C.1.3-1 describes the downlink physical channels that are required for NR connection setup. For EN-DC test cases in clauses 4 and 5, the required E-UTRA downlink physical channels are defined in TS 36.521-3 [26] Annex C.2.

Table C.1.3-1: Downlink physical channels required for NR connection set-up

| Physical Channel | EPRE Ratio | Note |
| --- | --- | --- |
| PBCH | PBCH_RA = 0 dB |  |
|  | PBCH_RB = 0 dB |  |
| PSS | PSS_RA = 0 dB |  |
| SSS | SSS_RA = 0 dB |  |
| PDCCH | PDCCH_RA = 0 dB |  |
|  | PDCCH_RB = 0 dB |  |
|  | MPDCCH_RB = 0 dB |  |
| PDSCH | PDSCH_RA = 0 dB |  |
|  | PDSCH_RB = 0 dB |  |
| DMRS | TBD |  |
| CSI-RS | TBD |  |
| NOTE 1: No boosting is applied |  |  |

As common PDSCH and PDCCH configuration parameters the parameters in Table A.1 and C.1.3-2 shall be used to bring up the connection setup for FR1 and FR2 NR cell.

Table C.1.3-2: PDSCH and PDCCH configuration

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ processes |  | 8 (TDD) 16 (FDD) |
| Maximum number of HARQ transmission |  | 5 |

## <<< Skip unchanged sections >>>

## <<< END OF CHANGES >>>