3GPP TSG-RAN5 Meeting #96-e	R5-225220
Electronic Meeting, 15th August – 26th August 2022

Title: 	On achievable improvements of permitted test methods

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.50.1

Document for:	Discussion and Endorsement

# 1.	Introduction

RAN4 completed a study item whose objectives were to enhance the FR2 RF testing methodology and to quantify the impact of the enhancements on the UE performance among others. The outcome of this study item is being documented in [1].

One of the investigations carried out was around how to overcome the testability issues of a set of FR2 RF test cases requiring either high DL power or low UL power. As part of this investigation, improvements on existing test methodologies were analysed and documented in section 5.1.6 in [1].

This document reviews the progress done by RAN4 in terms of achievable improvements on existing testing methodologies and provides internal views on such improvements in order to evaluate whether near field methodologies defined by RAN4 in [1] should be considered in conformance testing to overcome some of the FR2 testability issues found.

# 2.	Discussion

Improvement of permitted methods were documented as shown below:

### 5.1.6 Improvement of permitted methods Tables 5.1.6-1 and 5.1.6-2 below provide a preliminary list of potential improvement of permitted methods based on the analysis provided by one company and are applicable to the frequency range of 24.25 – 43.5 GHz. Table 5.1.6-1: Summary of potential improvement of permitted methods by Tx test case (24.25 – 43.5 GHz) Clause; Requirement; Testability issue; Test Metric; Regulatory related; TS 38.521-2 Test Requirements; Potential improvement. 6.3.1; Minimum output power; Low UL power; EIRP (Link=TX beam peak direction, Meas=Link angle).; No; No relaxation for PC1. For other power classes, relaxation varies from 0dB to 13.5dB depending on the operating band and channel bandwidth.; Improvements remove required relaxations from TC. 6.3.2; Transmit OFF power; Low UL power; TRP (Link=TX beam peak direction, Meas=TRP grid); Yes; Relaxations for n257: 21.4dB @ 50MHz, 24.4dB @ 100MHz, 27.4dB @ 200MHz and 30.4dB @ 400MHz. Relaxations for n258 and n261: [21.4]dB @ 50MHz, [24.4]dB @ 100MHz, [27.4]dB @ 200MHz and [30.4]dB @ 400MHz. Relaxations for n260: [24.1]dB @ 50MHz, [27.1]dB @ 100MHz, [30.1]dB @ 200MHz and [33.1]dB @ 400MHz.; ~ 10dB for FR2a and FR2b. 6.5.1; Occupied bandwidth; Low UL power; OBW (Link=TX beam peak direction, Meas=Link angle); Yes; No relaxations for FR2a and FR2b; N/A for FR2a and FR2b. 6.5.2.3; Adjacent channel leakage ratio; Low UL power; TRP (Link=TX beam peak direction, Meas=TRP grid).; Yes; Relaxation for n257, n258 and n261: 0dB, except for 200MHz (1.5dB in two test IDs) and 400MHz (between 0 and 5.5dB); Improvements remove required relaxations from TC TC coverage is extended for FR2b. 6.5.3.2; Additional spurious emissions; Low UL power; TRP (Link=TX beam peak direction, Meas=TRP grid).; Yes; Between 0.3dB and 13dB relaxation depending on the combination of NR Band and Protected band.; TBD. Table 5.1.6-2: Summary of potential improvement of permitted methods by Rx test case (24.25 – 43.5 GHz) Clause; Requirement; Testability issue; Test Metric; Regulatory related; TS 38.521-2 Test Requirements; Potential improvement. 7.4; Maximum input power; High DL power; EIS (Link=RX beam peak direction, Meas=Link angle).; No; 26dB relaxation for 24.25 ~ 29.5 GHz and 34 dB relaxation for 37 ~ 40 GHz with respect to minimun requirements.; ~ 12dB for FR2a ~16dB for FR2b. 7.5; Adjacent channel selectivity (case 1); High DL power; EIS (Link=RX beam peak direction, Meas=Link angle); Yes; 50MHz: 1.8dB relaxation for power in transmission BW and interferer for band n260. 100MHz: 4.8dB relaxation for power in transmission BW and interferer for band n260. 200MHz and 400MHz are deemed not testable.; Similar improvements as for TC 7.4 Improvements remove required relaxations from TC. 7.5; Adjacent channel selectivity (case 2); High DL power; EIS (Link=RX beam peak direction, Meas=Link angle); No; Decision not test ACS case 2.; Interferer need ~ 15-22dB relaxation. 7.6.2; In-band blocking; High DL power; EIS (Link=RX beam peak direction, Meas=Link angle); Yes; 50MHz: 1.8dB relaxation for power in transmission BW and interferer for band n260. 100MHz: 4.8dB relaxation for power in transmission BW and interferer for band n260. 200MHz and 400MHz are deemed not testable.; Similar improvements as for TC 7.4 Improvements remove required relaxations from TC. 7.9; Receiver spurious emissions; Low UL power; TRP (Link=TX beam peak direction, Meas=TRP grid).; Yes; Relaxations for n257, n258, n260, and n261: 10.2dB between 6-20GHz, 17.2dB between 20-40GHz and 33.1dB between 40GHz and the 2nd harmonic. Relaxations for other bands are still TBD.; TBD. For a given test case, NF based solutions should only be considered if the improvement for current methods is not enough to remove the relaxations determined by RAN5.

Observation 1: Despite Table 5.1.6-1 and Table 5.1.6-2 titles define that the frequency range for the analysis of achievable improvements for either Tx and Rx test cases is (24.25 – 43.5 GHz), conclusions inside tables are for achievable relaxation improvements for only FR2a and FR2b, i.e (24.25 – 40.8 GHz).

As described in observation 3 in [2], the analysis in section 5.1.6 in [1] was provided by a single company:

The source of achievable improvements in existing test methodologies, despite not documented in [1], can be found in [3] as the evolution of the signal conditioning unit to a highly integrated circuits version of it:

- Enhancement of current permitted methods Test systems defined according to current permitted methods, and more precisely using the IFF methodology as the de-facto standard for RF conformance, are based on a high level block diagram like the following: Radio Communication Tester Remote Radio Head(s) Signal Conditioning OTA Chamber Figure 3-1: High level system diagram for FR2 conformance testing Functional blocks on the left (i.e. radio communication tester and RRH) have physical limitations on the potential improvements that can be achieved since the design requirements are fixed based on other needs (control signals, cooling, etc.). On the chamber side, physics are the limiting factor considering the test volume size and required minimum footprint, although system with shorter focal length have an advantage in terms of path loss. Therefore, the intermediate step (i.e. signal conditioning) becomes the critical piece defining most of the dynamic range / SNR limitations driving the testability issues shown in previous section. This “signal conditioning” may comprise elements like switches, combiners, power amplifiers / LNAs, etc., and thus they become an expensive and complex piece of equipment, having many drawbacks (RF noise, heat, etc.) in case discrete RF components are used and connected by cables. The natural evolution of such “signal conditioning” boxes is the usage of highly integrated circuits. With this approach and based on state of the art components, the overall test system dynamic range / SNR could be improved and several relaxations could be either improved or removed completely.

After internal analyses, Keysight confirms that achievable relaxations improvements on permitted test methods described in section 5.1.6 in [1] are achievable for FR2a and FR2b.

Observation 2: Keysight confirms that achievable relaxations improvements on permitted test methods described in section 5.1.6 in [1] are achievable for FR2a and FR2b.

However, as it can be seen in section 5.1.6 in [1], relaxations cannot be completely removed for all analyzed test cases with only achievable relaxation improvements in permitted test methodologies as mentioned above: refer to at least Transmit OFF power, Adjacent channel leakage ratio for FR2b, Maximum input power or adjacent channel selectivity (case 2).

Observation 3: Relaxations cannot be completely removed for all analyzed test cases with only achievable relaxation improvements in permitted test methodologies so far analyzed.

On top of that, testability analysis has not been concluded yet for n259, n262, CA or even PC1, which could lead to new relaxation values or even new test cases needing relaxation.

Proposal 1: Continue further investigations on near field methodologies described in TR 38.884 [1] as a potential solution for FR2 testability issues.

# 3. 	Conclusion

This document reviews the progress done by RAN4 in terms of achievable improvements on existing testing methodologies and provides internal views on such improvements in order to evaluate whether near field methodologies defined by RAN4 in [1] should be considered in conformance testing to overcome some of the FR2 testability issues found.

The following observations and proposals are made:

Observation 1: Despite Table 5.1.6-1 and Table 5.1.6-2 titles define that the frequency range for the analysis of achievable improvements for either Tx and Rx test cases is (24.25 – 43.5 GHz), conclusions inside tables are for achievable relaxation improvements for only FR2a and FR2b, i.e (24.25 – 40.8 GHz).

Observation 2: Keysight confirms that achievable relaxations improvements on permitted test methods described in section 5.1.6 in [1] are achievable for FR2a and FR2b.

Observation 3: Relaxations cannot be completely removed for all analyzed test cases with only achievable relaxation improvements in permitted test methodologies so far analyzed.

Proposal 1: Continue further investigations on near field methodologies described in TR 38.884 [1] as a potential solution for FR2 testability issues.

# 4.	References

[1] 3GPP TR38.884 v18.1.0, “Study on enhanced test methods for FR NR UEs”

[2] R5-223644, “Discussion on improvements of permitted test methods”, Keysight Technologies UK Ltd, RAN5#95-e, May 2022.

[3] R4-2016562, “Views on test methods for high DL power and low UL power TCs”, Rohde & Schwarz, RAN4#97-e, Nov. 2020.