3GPP TSG-RAN5 Meeting #96-e	R5-225226
Electronic Meeting, 15th August – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2561 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test frequencies for CA_n258I |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test frequencies for CA_n258I are undefined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of test frequencies for CA_n258I. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA_n258I could not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.2.3.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES>>>

4.3.1.2.3.2.8	CA_n258I

Table 4.3.1.2.3.2.8-1: NR Intra-Band contiguous CA configuration CA_n258I (PCC=CC1, SCC=CC2, CC3, CC4), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 60 | 66 | Downlink | Low | 24275.04 | 2017083 | 24251.28 | 2016687 | 0 | 120 |  | 22257 | 2016955 |  | 4 |  | 2 | 0 (0) | 2 |
| +100+100 |  |  |  |  | & | Mid | 25725 | 2041249 | 25627.8 | 2039629 | 102 |  |  | 22341 | 2041147 |  | 6 |  | 4 | 0 (0) | 106 |
|  |  |  |  |  | Uplink | High | 27175.68 | 2065427 | 26789.04 | 2058983 | 504 |  |  | 22425 | 2065339 |  | 8 |  | 5 | 0 (0) | 509 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 24349.44 | 2018323 | 24301.92 | 2017531 | 0 | 120 |  | 22260 | 2017819 |  | 0 |  | 4 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 25799.4 | 2042489 | 25678.44 | 2040473 | 102 |  |  | 22344 | 2042011 |  | 2 |  | 6 | 0 (0) | 108 |
|  |  |  |  |  | Uplink | High | 27250.08 | 2066667 | 26839.68 | 2059827 | 504 |  |  | 22428 | 2066203 |  | 4 |  | 7 | 0 (0) | 511 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 24449.4 | 2019989 | 24401.88 | 2019197 | 0 | 120 |  | 22266 | 2019547 |  | 2 |  | 1 | 1 (8) | 9 |
|  |  |  |  |  | & | Mid | 25899.36 | 2044155 | 25778.4 | 2042139 | 102 |  |  | 22350 | 2043739 |  | 4 |  | 3 | 1 (8) | 113 |
|  |  |  |  |  | Uplink | High | 27350.04 | 2068333 | 26939.64 | 2061493 | 504 |  |  | 22434 | 2067931 |  | 6 |  | 4 | 1 (8) | 516 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 60 | 132 | Downlink | Low | 24549.36 | 2021655 | 24501.84 | 2020863 | 0 | 120 |  | 22272 | 2021275 |  | 4 |  | 6 | 1 (8) | 14 |
|  |  |  |  |  | & | Mid | 25999.32 | 2045821 | 25878.36 | 2043805 | 102 |  |  | 22356 | 2045467 |  | 6 |  | 8 | 1 (8) | 118 |
|  |  |  |  |  | Uplink | High | 27450 | 2069999 | 27039.6 | 2063159 | 504 |  |  | 22440 | 2069659 |  | 8 |  | 9 | 1 (8) | 521 |
| 100+100 | CC1 | 100 | 60 | 132 | Downlink | Low | 24300 | 2017499 | 24252.48 | 2016707 | 0 | 120 |  | 22257 | 2016955 |  | 8 |  | 0 | 0 (0) | 0 |
| +100+100 |  |  |  |  | & | Mid | 25725 | 2041249 | 25604.04 | 2039233 | 102 |  |  | 22340 | 2040859 |  | 6 |  | 5 | 1 (8) | 115 |
|  |  |  |  |  | Uplink | High | 27150.12 | 2065001 | 26739.72 | 2058161 | 504 |  |  | 22422 | 2064475 |  | 2 |  | 2 | 0 (0) | 506 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 24399.96 | 2019165 | 24352.44 | 2018373 | 0 | 120 |  | 22263 | 2018683 |  | 10 |  | 5 | 0 (0) | 5 |
|  |  |  |  |  | & | Mid | 25824.96 | 2042915 | 25704 | 2040899 | 102 |  |  | 22346 | 2042587 |  | 8 |  | 10 | 1 (8) | 120 |
|  |  |  |  |  | Uplink | High | 27250.08 | 2066667 | 26839.68 | 2059827 | 504 |  |  | 22428 | 2066203 |  | 4 |  | 7 | 0 (0) | 511 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 24499.92 | 2020831 | 24452.4 | 2020039 | 0 | 120 |  | 22269 | 2020411 |  | 0 |  | 3 | 1 (8) | 11 |
|  |  |  |  |  | & | Mid | 25924.92 | 2044581 | 25803.96 | 2042565 | 102 |  |  | 22352 | 2044315 |  | 10 |  | 15 | 1 (8) | 125 |
|  |  |  |  |  | Uplink | High | 27350.04 | 2068333 | 26939.64 | 2061493 | 504 |  |  | 22434 | 2067931 |  | 6 |  | 4 | 1 (8) | 516 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 60 | 132 | Downlink | Low | 24599.88 | 2022497 | 24552.36 | 2021705 | 0 | 120 |  | 22275 | 2022139 |  | 2 |  | 8 | 1 (8) | 16 |
|  |  |  |  |  | & | Mid | 26024.88 | 2046247 | 25903.92 | 2044231 | 102 |  |  | 22357 | 2045755 |  | 0 |  | 5 | 0 (0) | 107 |
|  |  |  |  |  | Uplink | High | 27450 | 2069999 | 27039.6 | 2063159 | 504 |  |  | 22440 | 2069659 |  | 8 |  | 9 | 1 (8) | 521 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.2.8-2: NR Intra-Band contiguous CA configuration CA_n258I (PCC=CC1, SCC=CC2, CC3, CC4), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 120 | 32 | Downlink | Low | 24275.04 | 2017083 | 24252 | 2016699 | 0 | 120 |  | 22257 | 2016955 |  | 8 |  | 21 | 6 (-21) | 0 |
| +100+100 |  |  |  |  | & | Mid | 25725 | 2041249 | 25555.08 | 2038417 | 102 |  |  | 22341 | 2041147 |  | 9 |  | 22 | 6 (-21) | 206 |
|  |  |  |  |  | Uplink | High | 27175.68 | 2065427 | 26426.88 | 2052947 | 504 |  |  | 22425 | 2065339 |  | 4 |  | 23 | 6 (-21) | 1012 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 24349.44 | 2018323 | 24301.92 | 2017531 | 0 | 120 |  | 22260 | 2017819 |  | 0 |  | 23 | 6 (-21) | 4 |
|  |  |  |  |  | & | Mid | 25799.4 | 2042489 | 25605 | 2039249 | 102 |  |  | 22344 | 2042011 |  | 1 |  | 23 | 6 (-20) | 210 |
|  |  |  |  |  | Uplink | High | 27250.08 | 2066667 | 26476.8 | 2053779 | 504 |  |  | 22428 | 2066203 |  | 8 |  | 24 | 6 (-21) | 1014 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 24449.4 | 2019989 | 24401.88 | 2019197 | 0 | 120 |  | 22266 | 2019547 |  | 7 |  | 25 | 6 (-21) | 8 |
|  |  |  |  |  | & | Mid | 25899.36 | 2044155 | 25704.96 | 2040915 | 102 |  |  | 22350 | 2043739 |  | 8 |  | 26 | 6 (-21) | 214 |
|  |  |  |  |  | Uplink | High | 27350.04 | 2068333 | 26576.76 | 2055445 | 504 |  |  | 22434 | 2067931 |  | 3 |  | 27 | 6 (-21) | 1020 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 120 | 66 | Downlink | Low | 24549.36 | 2021655 | 24501.84 | 2020863 | 0 | 120 |  | 22272 | 2021275 |  | 2 |  | 28 | 6 (-21) | 14 |
|  |  |  |  |  | & | Mid | 25999.32 | 2045821 | 25804.92 | 2042581 | 102 |  |  | 22356 | 2045467 |  | 3 |  | 29 | 6 (-21) | 220 |
|  |  |  |  |  | Uplink | High | 27450 | 2069999 | 26676.72 | 2057111 | 504 |  |  | 22440 | 2069659 |  | 10 |  | 29 | 6 (-21) | 1024 |
| 100+100 | CC1 | 100 | 120 | 66 | Downlink | Low | 24300 | 2017499 | 24252.48 | 2016707 | 0 | 120 |  | 22257 | 2016955 |  | 4 |  | 21 | 6 (-21) | 0 |
| +100+100 |  |  |  |  | & | Mid | 25725 | 2041249 | 25530.6 | 2038009 | 102 |  |  | 22340 | 2040859 |  | 9 |  | 27 | 6 (-21) | 216 |
|  |  |  |  |  | Uplink | High | 27150.12 | 2065001 | 26376.84 | 2052113 | 504 |  |  | 22422 | 2064475 |  | 1 |  | 22 | 6 (-21) | 1010 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 24399.96 | 2019165 | 24352.44 | 2018373 | 0 | 120 |  | 22263 | 2018683 |  | 11 |  | 23 | 6 (-21) | 4 |
|  |  |  |  |  | & | Mid | 25824.96 | 2042915 | 25630.56 | 2039675 | 102 |  |  | 22346 | 2042587 |  | 4 |  | 30 | 6 (-21) | 222 |
|  |  |  |  |  | Uplink | High | 27250.08 | 2066667 | 26476.8 | 2053779 | 504 |  |  | 22428 | 2066203 |  | 8 |  | 24 | 6 (-21) | 1014 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 24499.92 | 2020831 | 24452.4 | 2020039 | 0 | 120 |  | 22269 | 2020411 |  | 6 |  | 26 | 6 (-21) | 10 |
|  |  |  |  |  | & | Mid | 25924.92 | 2044581 | 25730.52 | 2041341 | 102 |  |  | 22352 | 2044315 |  | 11 |  | 32 | 6 (-21) | 226 |
|  |  |  |  |  | Uplink | High | 27350.04 | 2068333 | 26576.76 | 2055445 | 504 |  |  | 22434 | 2067931 |  | 3 |  | 27 | 6 (-21) | 1020 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 120 | 66 | Downlink | Low | 24599.88 | 2022497 | 24552.36 | 2021705 | 0 | 120 |  | 22275 | 2022139 |  | 1 |  | 29 | 6 (-21) | 16 |
|  |  |  |  |  | & | Mid | 26024.88 | 2046247 | 25830.48 | 2043007 | 102 |  |  | 22357 | 2045755 |  | 6 |  | 23 | 6 (-21) | 208 |
|  |  |  |  |  | Uplink | High | 27450 | 2069999 | 26676.72 | 2057111 | 504 |  |  | 22440 | 2069659 |  | 10 |  | 29 | 6 (-21) | 1024 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.2.9	CA_n258J

## <<< END OF CHANGES>>>