3GPP TSG-RAN5 Meeting #96-e	R5-225229
Electronic Meeting, 15th August – 26th August 2022

Agenda item:	5.4.17

Source:	Keysight Technologies

Title:	Discussion on L1-RSRP measurement accuracy test cases

Document for:	Discussion and Endorsement

# Introduction

This contribution analyses current L1-RSRP measurement accuracy test cases definition and provide a new proposal for checking absolute and relative requirements.

# Discussion

In [1] clause 5.7.4.1.5 test requirements, the values to be checked to provide a verdict on relative measurement requirements are defined as follows:

Table 5.7.4.1.5-5: evaluation limits for the reported values for T1 and T2 relative accuracy rules R3, R6

| UE power class 3 |  |  |
| --- | --- | --- |
| Normal Conditions | Test 1 All bands | Test 2 All bands |
| Lowest reported value (L1-RSRP SSB#1 – L1-RSRP SSB#0) | - 9 | - 4 |
| Highest reported value ((L1-RSRP SSB#1 – L1-RSRP SSB#0)) | -2 | +4 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands |
| Lowest reported value ((L1-RSRP SSB#1 – L1-RSRP SSB#0)) | - 12+ FFS | - 7+ FFS |
| Highest reported value ((L1-RSRP SSB#1 – L1-RSRP SSB#0)) | + 1+ FFS | + 7+ FFS |

However, it is not clear enough the meaning of above values, what is the mean of these values?

- Difference of absolute L1-RSRP SSB#1 and absolute L1-RSRP SSB#0?

For a good UE, UE would report values pretty close to SSB_RP values defined for SSB#0 and SSB#1 in Table 5.7.4.1.58-2:

SSB#0 ~ -96 dBm/SCS

SSB#1 ~ -85 dBm/SCS

- DIFF RSRP reported by UE in L1-RSRP report following TS 38.533 table 10.1.6.1-2?

If answer is A. values can’t be on the range of -9 to -2 dB because for a good UE the difference between absolute L1-RSRP for SSB#1 and SSB#0 would be ~ -11 dB and this value is out of the current defined range [-9,-2] in Table 5.7.4.1.5-5. Hence, we were failing a good UE.

If answer is B. values can’t be negative as it is based on DIFF RSRP values following [2] table 10.1.6.1-2:

Table 10.1.6.1-2: Differential SS-RSRP and CSI-RSRP measurement (for L1 reporting and L3 reporting) report mapping

| Reported value | Measured quantity value (difference in measured RSRP from strongest RSRP) | Unit |
| --- | --- | --- |
| DIFFRSRP_0 | 0≥ΔRSRP>-2 | dB |
| DIFFRSRP_1 | -2≥ΔRSRP>-4 | dB |
| DIFFRSRP_2 | -4≥ΔRSRP>-6 | dB |
| DIFFRSRP_3 | -6≥ΔRSRP>-8 | dB |
| DIFFRSRP_4 | -8≥ΔRSRP>-10 | dB |
| DIFFRSRP_5 | -10≥ΔRSRP>-12 | dB |
| DIFFRSRP_6 | -12≥ΔRSRP>-14 | dB |
| DIFFRSRP_7 | -14≥ΔRSRP>-16 | dB |
| DIFFRSRP_8 | -16≥ΔRSRP>-18 | dB |
| DIFFRSRP_9 | -18≥ΔRSRP>-20 | dB |
| DIFFRSRP_10 | -20≥ΔRSRP>-22 | dB |
| DIFFRSRP_11 | -22≥ΔRSRP>-24 | dB |
| DIFFRSRP_12 | -24≥ΔRSRP>-26 | dB |
| DIFFRSRP_13 | -26≥ΔRSRP>-28 | dB |
| DIFFRSRP_14 | -28≥ΔRSRP>-30 | dB |
| DIFFRSRP_15 | -30≥ΔRSRP | dB |

On the other hand, Test Tolerances analysis defined in [3] are based on DIFF RSRP reported value using [2] report mapping Table.

Furthermore, L1-RSRP reports interface (as defined in [4] cl 5.2.1.4.3	L1-RSRP Reporting) includes one absolute SS-RSRP value corresponding to the largest measured value of L1-RSRP for SSB/CSI-RS and DIFF RSRP value based on [2] table 10.1.6.1-2. The differential L1-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance. Hence, UE does not report absolute values for both SSBs and DIFF RSRP has a resolution of 2dB.

Considering this explanation, it is proposed to base 5.7.4.1 and 5.7.4.2 test requirements on absolute L1-RSRP value reported by UE corresponding to the largest measured SSB/CSI-RS and DIFF RSRP value based on TS 38.133 table 10.1.6.1-2.

Proposal 1: To update 5.7.4.1 and 5.7.4.2 test procedure and test requirements to consider only one absolute L1-RSRP value which corresponds to the largest measured SSB/CSI-RS reported by UE as part of L1-RSRP report.

Proposal 2: To update 5.7.4.1 and 5.7.4.2 test procedure and test requirements to consider DIFF RSRP value reported by UE as part of L1-RSRP report based on TS 38.133 table 10.1.6.1-2 for relative requirement.

# Conclusion

The following observations and conclusions were made in this contribution.

Proposal 1: To update 5.7.4.1 and 5.7.4.2 test procedure and test requirements to consider only one absolute L1-RSRP value which corresponds to the largest measured SSB/CSI-RS reported by UE as part of L1-RSRP report.

Proposal 2: To update 5.7.4.1 and 5.7.4.2 test procedure and test requirements to consider DIFF RSRP value reported by UE as part of L1-RSRP report based on TS 38.133 table 10.1.6.1-2 for relative requirement.

# References

- 3GPP TS 38.533 v17.3.0, 	NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM)

- 3GPP TS 38.133 v16.12.0 	NR; Requirements for support of radio resource management

- 3GPP TR 38.903 v16.12.0 	NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases

- 3GPP TS 38.214 v15.16.0 NR; Physical layer procedures for data

- R5-225218, Updates on 5.7.4.1 test procedure and test requirements, Keysight Technologies, RAN5#96e

- R5-225228, Updates on 5.7.4.2 test procedure and test requirements, Keysight Technologies, RAN5#96e