3GPP TSG-RAN5 Meeting #96-e	R5-225259

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1507 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to annexure A.2.3 and A.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the contact header field of A.2.3, condition A2 AND A15 and "MT call has been established" and A.3.1, condition A2 AND A5 and "MT call has been established", currently the contact URI to expect is retrieved from the stored dialog information. This can cause a problem as the stored URI can have URL parameters like “transport=” which can vary across transactions within a dialog. For example if a UE sends the initial INVITE establishing a dialog using TCP transport, and the SS later sends a re-INVITE using UDP, then the checking of the contact header in the following 200 OK will fail as it will expect url parameter “transport=tcp” in the contact header but UE will include “transport=udp”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a note in the contact header to allow any url |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.3, A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## A.2.3	183 Session Progress for INVITE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status-Line |  |  |  | RFC 3261 [15] |
| SIP-Version |  | SIP/2.0 |  |  |
| Status-Code |  | 183 |  |  |
| Reason-Phrase |  | Session Progress |  |  |
| Record-Route |  | order of the parameters in this header must be like in the respective rows |  | RFC 3261 [15] |
| rec-route | A1 | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: protected server port of SS;lr> |  |  |
|  | A3 | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: unprotected server port of SS (optional);lr> |  |  |
|  | A2 | same value as received in INVITE |  |  |
|  | A4 | same value as received in INVITE |  |  |
|  | A5 | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:protected server port of SS;lr> |  |  |
|  | A1 AND A15 | not present |  |  |
|  | A2 AND A15 | any value if present |  |  |
| Via |  |  |  | RFC 3261 [15] |
| via-parm |  | same value as received in INVITE message |  |  |
| Require |  | The option tags defined below shall be included additionally to any option tags defined in any specific message content, unless specified otherwise in this specific message content. |  | RFC 3261 [15] |
| option-tag |  | 100rel |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in INVITE message |  |  |
| tag |  | same value as received in INVITE message |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in INVITE message |  |  |
| tag | A1,A3, A5,A6 | common to-tag (invite) |  |  |
|  | A2,A4 | any value |  |  |
| P-Asserted-Identity | A5 |  |  | RFC 3325 [89] |
| addr-spec |  | A tel URI that can be recognized as valid emergency numbers if dialled by the user are specified in 3GPP TS 22.101 [39]. The emergency numbers 112 and 911 are stored on the ME, in accordance with 3GPP TS 22.101 [39] |  |  |
| uri-parameter |  | lr |  |  |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec | A1,A3 | px_IMS_CalleeContactUri |  |  |
|  | A2 AND NOT A9 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
|  | A4 AND NOT A9 | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |
|  | A2 AND A9 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  | RFC 5627 [61] |
|  | A4 AND A9 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  | RFC 5627 [61] |
|  | A1 AND A15 | MO call has been established: px_IMS_CalleeContactUri MT call has been established: contact address sent by SS in dialog creating INVITE |  |  |
|  | A2 AND A15 | MO call has been established: contact address sent by UE in dialog creating INVITE MT call has been established: contact address sent by UE in response to dialog creating INVITE (NOTE : The 'transport' url parameter may change, url parameters are not checked) |  |  |
| feature-param |  | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel"(see NOTE 2, 3) |  |  |
|  | A6 | video |  |  |
|  | A10 | audio |  |  |
|  | A11 AND A12 AND (A13 OR A14) | audio |  |  |
| Rseq |  |  |  | RFC 3262 [33] |
| response-num | A2,A4 | any value |  |  |
|  | A1,A3 | 121 (arbitrarily selected) |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in INVITE message |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | same value as received in INVITE message |  |  |
| Feature-Caps |  |  |  | RFC 6809 [125] |
| feature-param | A8 | +g.3gpp.ps2cs-srvcc-orig-pre-alerting |  | TS 24.237 [110] |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type |  | application/sdp |  |  |
| Content-Length | A1,A3 |  |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | 183 sent by the SS (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A2 | 183 sent by the UE (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A3 | 183 sent by the SS (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) |
| A4 | 183 sent by the UE (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) |
| A5 | 183 sent by the SS for INVITE for a non-UE detectable emergency call |
| A6 | UE supports video media feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A7 | Void |
| A8 | 183 sent by the SS for a voice call and UE supports pre-alerting media feature tag (A.12/36 3GPP TS 34.229-2 [5]) |
| A9 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |
| A10 | Void |
| A11 | Void |
| A12 | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |
| A13 | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A. |
| A14 | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23. |
| A15 | 183 sent for re-INVITE within an established dialog |

NOTE1:	All choices for applicable conditions are described for each header.

NOTE 2:	The “=” may include optional linear white spaces according to the EQUAL definition in chapter 25.1, RFC 3261 [15].

NOTE 3:	URN is the outcome of the URL encoding (“Percent-Encoding” according to RFC 3986 [129]) of urn:urn-7:3gpp-service.ims.icsi.mmtel.

SKIPPED TEXT

# A.3	Generic Common Messages

## A.3.1	200 OK for other requests than REGISTER or SUBSCRIBE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status-Line |  |  |  | RFC 3261 [15] |
| SIP-Version |  | SIP/2.0 |  |  |
| Status-Code |  | 200 |  |  |
| Reason-Phrase |  | OK |  |  |
| Via |  |  |  | RFC 3261 [15] |
| via-parm |  | same value as received in request |  |  |
| Record-Route |  | order of the parameters in this header must be like in the respective rows |  | RFC 3261 [15] |
| rec-route | A1 | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: protected server port of SS;lr> |  |  |
|  | A3 | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: unprotected server port of SS (optional);lr> |  |  |
|  | A2,A4, A5 | same value as received in the request (if present in the request) Note: for requests other than INVITE it is not regulated if and what the UE writes into this header in a response. |  |  |
|  | A6 | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:protected server port of SS;lr> |  |  |
|  | A7 | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:unprotected server port of SS;lr> |  |  |
|  | A1 AND A23 | not present |  |  |
|  | A2 AND A23 | any value if present |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in request |  |  |
| tag |  | same value as received in request |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in request |  |  |
| tag |  | same value as received in request or any value added if missing from request |  |  |
| P-Asserted-Identity | A6 |  |  | RFC 3325 [89] |
| addr-spec |  | A tel URI that can be recognized as valid emergency numbers if dialled by the user are specified in 3GPP TS 22.101 [39]. The emergency numbers 112 and 911 are stored on the ME, in accordance with 3GPP TS 22.101 [39] |  |  |
| uri-parameter |  | lr |  |  |
| Contact | A1,A2 A3,A4 |  |  | RFC 3261 [15] RFC 5627 [61] |
| addr-spec | A1,A3 | px_IMS_CalleeContactUri |  |  |
|  | A2 AND NOT A9 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
|  | A4 AND NOT A9 | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |
|  | A2 AND A9 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  |  |
|  | A4 AND A9 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  |  |
|  | A1 AND A5 | MO call has been established: px_IMS_CalleeContactUri MT call has been established: contact address sent by SS in dialog creating INVITE |  |  |
|  | A2 AND A5 | MO call has been established: contact address sent by UE in dialog creating INVITE MT call has been established: contact address sent by UE in response to dialog creating INVITE (NOTE : The 'transport' url parameter may change, url parameters are not checked) |  |  |
| feature-param | A10 | Audio |  |  |
|  | A18 AND (A19 OR A20) | video |  | IR.94 [134] |
|  | A11 AND A14 AND (A15 OR A16) | audio |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in request |  |  |
| Call-Info | A12 |  |  | RFC 8147 [149] |
| cid URL |  | <cid:psap@3gpp.org> | Rel-14 |  |
| purpose |  | EmergencyCallData.Control |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | same value as received in request |  |  |
| P-Access-Network-Info | A8 |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A21 | access network information and, if applicable, the cell ID |  |  |
|  | A22 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID |  |  |
| Accept | A12 |  |  | RFC 8147 [149] |
| media-range |  | application/sdp, application/pidf+xml, application/EmergencyCallData.Control+xml, application/emergencyCallData.eCall.MSD | Rel-14 |  |
| Recv-Info | A12 |  |  | RFC 8147 [149] |
| Info-package-type |  | emergencyCallData.eCall.MSD | Rel-14 |  |
| Content-Type | A12 |  |  | RFC 8147 [149] |
| media-type |  | multipart/mixed; boundary=boundary1 | Rel-14 |  |
| Content-Length | A10 |  |  | RFC 3261 [15] |
| value | NOT A12 | 0 |  |  |
|  | A12 | length of message body |  |  |
| Message-body | A12 |  |  |  |
|  | A13 | --boundary1 Content-Type: application/EmergencyCallData.eCall.Control+xml Content-ID: <psap@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="true" ref=" addr-spec of the Content-ID of MIME body part containing the MSD sent by the UE in INVITE"/> </EmergencyCallData.control> --boundary1 |  | RFC 8147 [149] |
|  | A14 | --boundary1 Content-Type: application/EmergencyCallData.eCall.Control+xml Content-ID: <psap@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <emergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="false" ref=" addr-spec of the Content-ID of MIME body part containing the MSD sent by the UE in INVITE "/> </EmergencyCallData.control> --boundary1 |  | RFC 8147 [149] |

| Condition | Explanation |
| --- | --- |
| A1 | Response sent by SS for INVITE/UPDATE (IMS security, A.6a/2 TS 34.229-2 [5])) |
| A2 | Response sent by UE for INVITE/UPDATE (IMS security, A.6a/2 TS 34.229-2 [5])) |
| A3 | Response sent by SS for INVITE/UPDATE (GIBA, A.6a/1 TS 34.229-2 [5])) |
| A4 | Response sent by UE for INVITE/UPDATE (GIBA, A.6a/1 TS 34.229-2 [5])) |
| A5 | Any response sent by the UE within a dialog |
| A6 | Response sent by SS for INVITE for emergency call or non-UE detectable emergency call |
| A7 | Response sent by SS for INVITE for emergency call without emergency registration |
| A8 | Any response sent by the UE within a dialog, except for CANCEL requests |
| A9 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |
| A10 | Response sent by SS |
| A11 | Response sent by UE |
| A12 | Response sent by SS for INVITE for eCall over IMS session with either ACK or NACK |
| A13 | Response sent by SS for INVITE for eCall over IMS session with ACK element = TRUE |
| A14 | Response sent by SS for INVITE for eCall over IMS session with ACK element = FALSE |
| A15 | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |
| A16 | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A |
| A17 | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23 |
| A18 | UE supports video feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A19 | Response sent by SS for INVITE |
| A20 | Response sent by UE for INVITE |
| A21 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A22 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A23 | Response sent for re-INVITE within an established dialog |