3GPP TSG-RAN5 Meeting #96-e	R5-225286

# Online, 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-1 | CR | 3938 | rev | 1 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 34123-1 Correct mobile originated short message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-07-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In MO SMS case the UE sends CP-DATA message to NW and NW shall send the CP-ACK message back to the UE. In current spec it is mistakenly says UE send CP-ACK message which need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | At step 13, in MO case, change UE send the CP-ACK message to SS send the CP-ACK message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Typo still in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is outcome of R5-225049 with below revisions. R1: Change the WIC to TEI8_Test R2: Remove “x” from ME in the cover page. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

16.1.10	Test of capabilities of simultaneously receiving a short message whilst sending a mobile originated short message

16.1.10.1	Definition

16.1.10.2	Conformance requirements

An active UE shall be able to receive a short message TPDU (SMS-DELIVER) at any time, independently of whether or not there is an SMS mobile originated call (SMS-SUBMIT or SMS-COMMAND) in progress.

References

3GPP TS 23.040 clauses 3.1, 9.2.3.16.

3GPP TS 24.011 clause 3.2.

16.1.10.3	Test purpose

The test verifies that the UE is capable of simultaneously receiving a network originated SM whilst sending a mobile originated SM.

16.1.10.4	Method of test

Initial Conditions

-	System simulator:

-	1 cell, default parameters.

-	User Equipment:

-	the UE shall be in  MM-state "Idle, updated";

-	the SMS message storage shall be empty.

Related ICS/IXIT Statements

Support for Short message MO/PP and MT/PP.

Support for state U10 of call control.

The value of timer TC1M.

Whether SMS messages are stored in the USIM and/or the ME.

Test procedure

a)	The SS is configured to receive a mobile originated SM. In clause 16.1.2 steps a) and b) are repeated and, using the end of the CP-DATA message from the UE as a trigger, the SS sends a SM to the UE. In this case a new transaction identifier shall be used in the CP messages of SMS mobile terminated.

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  |  | The UE is set up to send an SM |
| 2 | SS |  |  | The SS verifies that the IE “Establishment cause” in the received RRC CONNECTION REQUEST message is set to “Originating Low Priority Signalling”. |
| 3 |  |  | (void) |  |
| 4 |  |  | (void) |  |
| 5 | --> |  | CM SERVICE REQUEST | CM service type set to "Short Message Service" |
| 6 | <-- |  | AUTHENTICATION REQUEST |  |
| 7 | --> |  | AUTHENTICATION RESPONSE |  |
| 8 | SS |  |  | The SS starts integrity protection |
| 9 |  |  | (void) |  |
| 10 | --> |  | CP-DATA | Contains RP-DATA RPDU (SMS SUBMIT TPDU) |
| 11 | SS |  |  | The SS sends an SM to the UE triggered by the end of the CP-DATA message from the UE |
| 12 | <-- |  | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) |
| 13 | UE |  |  | The UE shall correctly receive the SM and indicate that a message has arrived. In the MO case the SS shall send the CP-ACK message with transaction identifier assigned to this transfer. In the MT case the UE shall send a CP-ACK message and a CP-DATA message containing the RP-ACK RPDU. The transaction identifier shall be the same as chosen by the SS for the MT transfer. |
| NOTE: Time values for SS wait times are chosen sufficiently high to be sure that the UE has enough time to respond to the different messages. |  |  |  |  |

Specific Message Contents

SMS SUBMIT TPDU

| Information element | Comment Value |
| --- | --- |
| TP-UDL | as applicable |
| TP-UD (140 octets) | 160 ASCII characters |

16.1.10.5	Test requirements

After step 12 UE shall correctly receive the SM and indicate that a message has arrived.

<<< END OF CHANGES >>>