3GPP TSG-RAN5 Meeting #96-e	R5-225306

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3203 | rev | 1 | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR CAG testcase 6.5.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to current Prose, at Power setting T0, the NR cell 1 and NR Cell 4 are both set to -88 dBm and are active at same time. Acccording to 38.508 Table 6.3.2.2-3, both of these are on frequency NRf1. Also accroding to the configuration of 38.523-3 Table 7.1.2.2.3.1-1 and 38.508-1 Table 4.4.2-2 , NR Cell 1 and NR Cell 4 use SSB index 1 and system information located in the RB0~ RB6, while NR Cell2 use SSB index 0 and system information located in RB (NBWP-7)~ RB(NBWP). Therefore, in T0, NR Cell 1 and NR Cell 4 have same RSRP level, and intra-frequency interference may cause issues for the UE to receive SIB1 of NR Cell 1 and thus UE may not detect and camp on NR Cell 1 in step2. Hence, it is proposed that at T0 setting, NR Cell 4 be set as OFF. Also, the negative test of the UE not camping on NR Cell 4 is performed at Step 24 with T1 setting, so there is no need to keep NR Cell 4 at serving cell power in T2 setting. It is proposed to set NR cell 4 as “Off”. According to the current Prose, at Step 24, the UE should not camp on NR Cell 4. However, the NR Cell 4 is belonging to PLMN3, and the access to this PLMN is not restricted for CAG only. As a result, a conformant UE will access NR Cell 4 at Step 24. This needs to be addressed. It is proposed that in the Registration Accept message at Step 14, in the IE CAG information list, included the entry for NR Cell 4. Basically included the PLMN for NR Cell 4, but with CAG bit set as ‘1’B (Which means PLMN for NR Cell 4 can be accessed via CAG only), and also the CAG ID is set to 4. The CAG IS is set to 4, because the UE will include the CAG ID 4 in allowed CAG list. This will ensure that the UE detects the NR Cell 4 PLMN and finds that access to the PLMN is via CAG only (CAG bit set to 1) UE will detect CAG ID = 2 for NR Cell 4 via SIB1  The UE will not be able to match the CAG ID in allowed CAG list for NR Cell 4 (CAG ID = 4) with what is received in broadcasted SIB1 (CAG ID = 2 ) and thus not camp on NR Cell 4. Also inlcuded the PLMN of NR Cell 2 as EPLMn so that the UE finds this NR cell quickly (If this is not done, then according to 38.304 cl 5.2.4.4 some UEs may take 300s to detect the NR cell). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the T0 and T1 settings in Table 6.5.2.2.3.2-1 In the Registration Accept, added another Entry for NR Cell 4, where CAG bit is set to ‘1’B, and CAG ID is set to 4. Also included the PLMN of NR Cell 2 as EPLMN Also introduced Tables 6.5.2.2.3.3-3 and 6.5.2.2.3.3-4 for SIB1 for NR Cell 2 and NR Cell 4 respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase based on current Prose |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is a revision of R5-225236 |  |  |  |  |  |  |  |  |

#### 6.5.2.2	CAG Selection in Automatic Mode

6.5.2.2.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and provisioned with an empty "CAG information list” }

ensure that {

when { UE is switched on or returned to coverage, and a cell which is not a CAG cell becomes available }

then { UE selects the cell }

}

(2)

with { UE in Automatic network selection mode provisioned with a "CAG information list” }

ensure that {

when { UE is switched on or returned to coverage, and a CAG cell with a CAG-ID present in the "Allowed CAG list" becomes available }

then { UE selects the CAG cell }

}

(3)

with { UE in Automatic network selection mode provisioned with a "CAG information list” }

ensure that {

when { UE is switched on and a cell which is not a CAG cell becomes available, and there is an entry with the PLMN ID of the PLMN in the "CAG information list" but the "indication that the MS is only allowed to access 5GS via CAG cells" is not included in the entry }

then { UE selects the cell }

}

6.5.2.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 3.8, and 4.4.3.1.1. Unless otherwise stated these are Rel-16 requirements.

[TS 23.122, clause 3.8]

The MS may support CAG.

If the MS supports CAG, the MS can be provisioned with a "CAG information list", consisting of zero or more entries, each containing:

a)	a PLMN ID;

b)	an "Allowed CAG list". The "Allowed CAG list" contains zero or more CAG-IDs; and

c)	an optional "indication that the MS is only allowed to access 5GS via CAG cells".

The "CAG information list" is stored in the ME.

NOTE: 	When the UE is registering or registered to a PLMN other than the HPLMN or EHPLMN, then the HPLMN will send a "CAG information list" consisting of CAG subscription information related to the serving PLMN only. When the UE is registering or registered to the HPLMN or EHPLMN then the HPLMN or EHPLMN can send CAG subscription information related to any PLMN in the "CAG information list".

If the MS supports CAG and a PLMN is selected as described in subclause 4.4.3.1.1, the automatic CAG selection is performed as part of subclause 4.4.3.1.1.

If the MS supports CAG and a PLMN is selected as described in subclause 4.4.3.1.2, the manual CAG selection is performed as part of subclause 4.4.3.1.2.

The NAS shall provide the AS with a "CAG information list", if available. If the contents of the "CAG information list" have changed, the NAS shall provide an updated "CAG information list" to the AS.

[TS 23.122, clause 4.4.3.1.1]

The MS selects and attempts registration on other PLMN/access technology combinations, if available and allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present) ;

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order) or stored in the ME (in priority order);

iv)	other PLMN/access technology combinations with received high quality signal in random order;

v)	other PLMN/access technology combinations in order of decreasing signal quality.

When following the above procedure the following requirements apply:

…

m)	In i to v, if the MS supports CAG and:

1)	is provisioned with a non-empty "CAG information list", the MS shall consider a PLMN indicated by an NG-RAN cell only if:

A)	the cell is a CAG cell and broadcasts a CAG-ID for the PLMN such that there exists an entry with the PLMN ID of the PLMN in the "CAG information list" and the CAG-ID is included in the "Allowed CAG list" of the entry; or

B)	the cell is not a CAG cell and:

-	there is no entry with the PLMN ID of the PLMN in the "CAG information list"; or

-	there exists an entry with the PLMN ID of the PLMN in the "CAG information list" but the "indication that the MS is only allowed to access 5GS via CAG cells" is not included in the entry; or

2)	is provisioned with an empty "CAG information list" or is not provisioned with a "CAG information list", the MS shall consider a PLMN indicated by an NG-RAN cell only if the cell is not a CAG cell.

n)	In i to v, if the MS only supports control plane CIoT 5GS optimization (see 3GPP TS 23.501 [62]) and the MS camps on an E-UTRA cell connected to 5GCN, which is not NB-IoT cell (see 3GPP TS 36.304 [43], 3GPP TS 36.331 [22]), the MS shall not consider PLMNs which do not advertise support of 5GS services with control plane CIoT 5GS optimization.

o)	In i to v, if the MS supports CIoT 5GS optimizations, the MS shall not consider the PLMN/access technology combinations for which the MS preferred CIoT network behaviour is not advertised as supported by the PLMN/access technology combination (see 3GPP TS 24.501 [64]).

NOTE 7:	As an implementation option, the MS supporting CIoT 5GS optimizations that was not able to select any PLMN according to o) can perform a second iteration of i to v with no restriction.

p)	In iii, the MS shall use the PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" stored in the ME, if the last received steering of roaming information contains the "list of preferred PLMN/access technology combinations"(see annex C) and is stored in the ME. Otherwise, the MS shall use the "Operator Controlled PLMN Selector with Access Technology" list retrieved from the SIM.

If successful registration is achieved, the MS indicates the selected PLMN.

If registration cannot be achieved because no PLMNs are available and allowable, and the MS does not support access to RLOS, the MS indicates "no service" to the user, waits until a new PLMN is available and allowable and then repeats the procedure.

If there were one or more PLMNs which were available and allowable, but an LR failure made registration on those PLMNs unsuccessful or an entry in any of the lists "forbidden location areas for roaming", "forbidden tracking areas for roaming", "5GS forbidden tracking areas for roaming", "forbidden location areas for regional provision of service", "forbidden tracking areas for regional provision of service", "5GS forbidden tracking areas for regional provision of service", or "CAG information list" prevented a registration attempt, the MS selects the first such PLMN again and enters a limited service state.

…

6.5.2.2.3	Test description

6.5.2.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 2, NR Cell 4 and NR Cell 11.

-	NR Cell 2 and NR Cell 4 are CAG cells.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used for NR Cell 1, NR Cell 2, NR Cell 4 and NR Cell 11.

-	The PLMNs are identified in the test by the identifiers in Table 6.5.2.2.3.1-1 and the PLMN settings are defined in TS 36.523-1 [13] table 6.0.1-1.

Table 6.5.2.2.3.1-1: PLMN identifiers

| NR Cell |  | PLMN name |  | CAG IDs |  |
| --- | --- | --- | --- | --- | --- |
| 2 |  | PLMN2 |  | 1 |  |
| 4 |  | PLMN3 |  | 2 |  |
| 11 |  | PLMN2 |  | - |  |

UE:

-	The UE is in Automatic PLMN selection mode.

-	There is an empty “CAG information list” provisioned in the UE.

Preamble:

-	The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes an empty CAG information list before it is in state Switched OFF (State 0N-B).

6.5.2.2.3.2	Test procedure sequence

Table 6.5.2.2.3.2-1 for FR1 and 6.5.2.2.3.2-2 for FR2 illustrate the downlink power levels to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2" and "T3" are to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 38.508-1 [4] Table 6.2.2.1-3 for FR1 and Table 6.2.2.2-2 for FR2.

Table 6.5.2.2.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | NR Cell 11 | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | "Off" | "Off" |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | -88 | "Off" |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 | "Off" | "Off" |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | "Off" | -88 |  |

Table 6.5.2.2.3.2-2: Cell configuration changes over time for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | NR Cell 11 | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | "Off" | FFS | "Off" |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | FFS | "Off" |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | FFS | FFS | "Off" |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | "Off" | FFS |  |

Table 6.5.2.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - | - | - | - |
| 2 | Check: Does the UE send a RRCSetupRequest on NR Cell 1? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 3-13 | Steps 3 to 13 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 1. NOTE: The UE starts registration. | - | - | - | - |
| 14 | The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing the CAG information list with the CAG ID of NR Cell 2. | <-- | NR RRC: DLInformationTransfer 5G MM: REGISTRATION ACCEPT | - | - |
| 15-20a1 | Steps 15 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 1. NOTE: The UE completes registration and the RRC connection is released. | - | - | - | - |
| 21 | The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4] | - | - | - | - |
| 22 | SS adjusts cell levels according to row T1 of table 6.5.2.2.3.2-1 for FR1 and table 6.5.2.2.3.2-2 for FR2. | - | - | - | - |
| 23 | Power on the UE. | - | - | - | - |
| 24 | Check: Does the UE send a RRCSetupRequest on NR Cell 4 within the next 60 s? | --> | NR RRC: RRCSetupRequest | 2 | F |
| 25 | SS adjusts cell levels according to row T2 of table 6.5.2.2.3.2-1 for FR1 and table 6.5.2.2.3.2-2 for FR2. | - | - | - | - |
| 26 | Check: Does the UE send a RRCSetupRequest on NR Cell 2? | --> | NR RRC: RRCSetupRequest | 2 | P |
| 27-44a1 | Steps 3 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 2. NOTE: The UE completes registration and the RRC connection is released. | - | - | - | - |
| 45 | The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4] | - | - | - | - |
| 46 | SS adjusts cell levels according to row T3 of table 6.5.2.2.3.2-1 for FR1 and table 6.5.2.2.3.2-2 for FR2. | - | - | - | - |
| 47 | Power on the UE. | - | - | - | - |
| 48 | Check: Does the UE send a RRCSetupRequest on NR Cell 11? | --> | NR RRC: RRCSetupRequest | 3 | P |
| 49-66a1 | Steps 3 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 11. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |

6.5.2.2.3.3	Specific message contents

Table 6.5.2.2.3.3-1: REGISTRATION ACCEPT (step 14, Table 6.5.2.2.3.2-3)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  | MCC : 002; MNC : 11; CAG only : 0; CAG-ID 1 : 1 |  |
| Length of CAG information list contents | ‘0000 0000 0001 0010’B |  |  |
| Entry 1 |  |  |  |
| Length of entry contents | ‘0000 1000’B |  |  |
| MCC | MCC of NR Cell 2 | See Table 6.5.2.2.3.1-1 |  |
| MNC | MNC of NR Cell 2 | See Table 6.5.2.2.3.1-1 |  |
| CAG only | ‘0’B |  |  |
| CAG_ID 1 | 1 | CAG ID is coded as a 32 bit BITSTRING |  |
| Entry 1 |  |  |  |
| Length of entry contents | ‘0000 1000’B |  |  |
| MCC | MCC of NR Cell 4 | See Table 6.5.2.2.3.1-1 |  |
| MNC | MNC of NR Cell 4 | See Table 6.5.2.2.3.1-1 |  |
| CAG only | ‘1’B |  |  |
| CAG_ID 1 | 4 | CAG ID is coded as a 32 bit BITSTRING |  |
| Equivalent PLMNs | PLMN2 |  |  |

Table 6.5.2.2.3.3-2: REGISTRATION ACCEPT (Preamble)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  |  |  |
| Length of CAG information list contents | ‘0000 0000’B |  |  |
| Entry 1 | Not present |  |  |

Table 6.5.2.2.3.3-3: SIB1 for NR Cell 2 (all steps in Table 6.5.2.2.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 with condition SNPN and CAG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| CellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellReservedForOtherUse | true |  |  |
| npn-IdentityInfoList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-IdentityInfo-r16 { | 1 entry |  |  |
| NPN-IdentityInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| npn-IdentityList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-Identity-r16 { | 1 entry |  |  |
| NPN-Identity-r16[1] CHOICE { |  | entry 1 |  |
| pni-npn-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc | 002 | PLMN2 MCC |  |
| mnc | 11 | PLMN2 MNC |  |
| } |  |  |  |
| cag-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16 { | 1 entry |  |  |
| CAG-IdentityInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| cag-Identity-r16 | 1 | CAG ID is coded as a 32 bit BITSTRING |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 2 |  |  |
| cellIdentity-r16 | Cell Identity of NR Cell 2 |  |  |
| cellReservedForOperatorUse-r16 | notReserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.2.3.3-4: SIB1 for NR Cell 4 (all steps in Table 6.5.2.2.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 with condition SNPN and CAG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| CellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellReservedForOtherUse | true |  |  |
| npn-IdentityInfoList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-IdentityInfo-r16 { | 1 entry |  |  |
| NPN-IdentityInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| npn-IdentityList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-Identity-r16 { | 1 entry |  |  |
| NPN-Identity-r16[1] CHOICE { |  | entry 1 |  |
| pni-npn-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc | 003 | PLMN3 MCC |  |
| mnc | 21 | PLMN3 MNC |  |
| } |  |  |  |
| cag-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16 { | 1 entry |  |  |
| CAG-IdentityInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| cag-Identity-r16 | 2 | CAG ID is coded as a 32 bit BITSTRING |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 4 |  |  |
| cellIdentity-r16 | Cell Identity of NR Cell 4 |  |  |
| cellReservedForOperatorUse-r16 | notReserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |