3GPP TSG-RAN5 Meeting #96-e	   R5-225355

# Electronic Meeting, 15th Aug – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2453 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Table 4.6.3-96 to add message contents values for SDT in PDCCH-ConfigCommon |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SDT Information Elements values corresponding to PDCCH-ConfigCommon message are needed for 3GPP SDT test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | ##### Updated PDCCH-ConfigCommon message Information elements values for SDT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the work plan will remain unfulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-224332r1 - included other non-SDT related Rel-17 IEs also, with values set to Not present and marked default value of sdt-SearchSpace-r17 as Not present. R5-225355 : Is the final Tdoc for R5-224332r1 |  |  |  |  |  |  |  |  |

### 4.6.3	Radio resource control information elements

<Contents skipped>

<Start of Changes>

#### –	PDCCH-ConfigCommon

Table 4.6.3-96: PDCCH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| controlResourceSetZero | ControlResourceSetZero | Initial BWP |  |
|  | Not present |  | SCell_Add InitialBWP_SIB, BWP-Id1 |
| commonControlResourceSet | ControlResourceSet |  | EN-DC |
|  | Not present |  |  |
| searchSpaceZero | SearchSpaceZero | Initial BWP |  |
|  | Not present |  | SCell_Add InitialBWP_SIB, BWP-Id1 |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[1] | SearchSpace with condition CSS | entry 1 |  |
| SearchSpace[2] | SearchSpace with condition SISS | entry 2 |  |
| } |  |  |  |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 1 entry |  | EN-DC |
| SearchSpace[1] | SearchSpace with condition CSS | entry 1 |  |
| } |  |  |  |
| commonSearchSpaceList | Not present |  | SCell_Add |
| searchSpaceSIB1 | 0 |  |  |
|  | Not present |  | EN-DC, SCell_Add |
| searchSpaceOtherSystemInformation | SearchSpaceId with condition SISS |  |  |
|  | Not present |  | EN-DC, SCell_Add |
| pagingSearchSpace | 0 |  |  |
|  | Not present |  | EN-DC, SCell_Add |
| ra-SearchSpace | SearchSpaceId with condition CSS |  |  |
|  | Not present |  | SCell_Add |
| firstPDCCH-MonitoringOccasionOfPO | Not present |  |  |
| commonSearchSpaceListExt-r16 | Not present |  |  |
| sdt-SearchSpace-r17 | Not present |  |  |
| sdt-SearchSpace-r17 CHOICE { |  |  | SDT |
| existingSearchSpace | SearchSpaceId with condition CSS |  |  |
| } |  |  |  |
| searchSpaceMCCH-r17 | Not present |  |  |
| searchSpaceMTCH-r17 | Not present |  |  |
| commonSearchSpaceListExt2-r17 | Not present |  |  |
| firstPDCCH-MonitoringOccasionOfPO-v1710 | Not present |  |  |
| pei-ConfigBWP-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| SCell_Add | Add Scell |
| InitialBWP_SIB | Configured via DownlinkConfigCommonSIB |
| BWP-Id1 | Additional BWP 1 |
| SDT | For SDT test cases |

<End of Changes>