3GPP TSG-RAN5 Meeting #96-e	R5-225434

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0446 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IMS TC 6.9-Subscription with 503 Service Unavailable |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The presence of registration procedure and IMS PDU session establishment procedure as defined in steps 2-19a1 of TS38.508-1 table 4.5.2.2-2 are currently not reflected in test procedure sequence. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Note to reference the NAS signalling.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC or a non-conformant UE may pass the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-224673r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.9	Subscription / 503 Service Unavailable / 5GS

6.9.1	Test Purpose (TP)

(1)

with { UE subscribing to reg event }

ensure that {

when { UE receives 503 Service unavailable containing a Retry-After header field }

then { UE does not reattempt the request for the indicated time period }

}

6.9.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229 clause 5.1.2.2]:

If the UE receives a 503 (Service Unavailable) response to an initial SUBSCRIBE request containing a Retry-After header field, then the UE shall not automatically reattempt the request until after the period indicated by the Retry-After header field contents.

6.9.3	Test description

6.9.3.1	Pre-test conditions

System Simulator:

-	SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE.

-	SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

-	SS is listening to SIP default port 5060 for both UDP and TCP protocols.

-	1 NR Cell

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS after switch on.

-	The UE is switched off.

Preamble:

-	None.

6.9.3.2	Test procedure sequence

Table 6.9.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. |  |  |  |  |
| 2-5 | Steps 1-4 of Annex A.2 happen. |  |  |  |  |
| 6 | UE subscribes to its registration event package. | --> | SUBSCRIBE |  |  |
| 7 | SS responds with 503 response containing a Retry-After header with period set to T=128s. | <-- | 503 Service Unavailable |  |  |
| 8 | Check: does the SS receive the UE's re-attempt of SUBSCRIBE within the Time T=128s? |  |  | 1 | F |
| 9 | UE reattempts to subscribe to its registration event package. | --> | SUBSCRIBE |  |  |
| 10-12 | Continue with Annex A.2 step 6-8 in order to get the UE in a stable registered state. | - |  |  |  |
| NOTE: The associated NR/5GC signalling is specified in TS 38.508-1 [21] generic procedure 4.5.2. |  |  |  |  |  |

6.9.3.3	Specific message contents

Table 6.9.3.3-1: SUBSCRIBE for reg-event package (step 6, table 6.9.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.1.4, Conditions A1 and A7

Table 6.9.3.3-2: 503 Service Unavailable (step 7, table 6.9.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.4.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Retry-After |  |  |  | RFC 3261 [6] |
| period |  | 128 |  |  |

Table 6.9.3.3-3: SUBSCRIBE for reg-event package (step 9, table 6.9.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.1.4, Conditions A1 and A7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Call-ID |  |  |  | RFC 3261 [6] |
| callid |  | value different from the previous SUBSCRIBE request |  |  |

<End of modified section 1>