3GPP TSG-RAN5 Meeting #96-e                                                                 	R5-225640

# Electronic Meeting, 15th Aug 2022 - 26th Aug 2022

Agenda item:	5.4.17

Source: 	Qualcomm Inc

Title: 	Applicability of FWA devices for Demod and RRM test cases

Document for:	Discussion and Endorsement

## 1.	Introduction

DEMOD spec [1] and RRM spec [2] currently limit the applicability of test cases to UE power class 3. Also Additionally DEMOD spec [1] limits the test case applicability to IFF setup with QZ <=30cm. This contribution is to discuss extending the DEMOD and RRM test cases to other UE power classes and IFF with QZ >30cm.

## 2.	Discussion

DEMOD spec has below wordings in spec to limit test cases to PC3.

It is primarily due to 3 terms.

- UE REFSENS for PC3 device is used for SNR calculation

- Available DL power assumption for SNR calculation is for IFF 30cm QZ

- Test system DL setting uncertainty

In order to expand the applicability to other device power classes esp PC1, the Demod SNR calculator needs to be redone using PC1 UE REFSENS. PC1 UE REFSENS is better than PC3 UE REFSENS. So that would only make the testable SNR higher compared to existing values.

Observation1: In order to expand the applicability to other device power classes esp PC1, the Demod SNR calculator needs to be redone using PC1 UE REFSENS

Proposal1:  Revise 38.903 Demod SNR calculator to include PC1. Revisit PC5  discussion once there is agreement on antenna assumptions for PC5 devices.

For device size > 30cm and <= 40cm, the available DL power at CW 1 dB compression at QZ term needs to be analyzed by TE vendors.

Proposal2: In order to expand the applicability of DEMOD test cases for device size > 30cm and <=40cm, go with the working assumption of existing available DL power at CW 1 dB compression at QZ.

Test System DL setting uncertainty term in the SNR calculator is based on the MU values specified in Annex B of TR 38.903 [3]. Some of the MU values are already specified for PC1. But some are still missing.

Proposal3: Identify the list of MU terms that are still missing for PC1 and are needed to make the DEMOD and RRM test case applicable.

RRM test cases are applicable for DFF based system. Hence QZ size is not relevant. RRM TT analysis uses UE REFSENS assumption for PC3.

The TT analysis needs to account for UE REFSENS for PC1 and other power classes.

Based on input from R&S, below is the list of MU contributors relevant for Demod:

Assumption is for PC1 as well the approach of baseband combining implementation is followed:

- gNB emulator SNR uncertainty

- gNB emulator fading model impairments

- AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig (Note 4)

- SNR uncertainty due to finite test time

- Impact on non-ideal isolation between branches for the wireless cable mode

From these MU contributors the yellow highlighted can depend on the RF properties of the test system. Thus, they need to be assessed for PC1 MU.

For the calculation of achievable SNR the following is required:

- Available DL power at CW 1dB compression at QZ, dBm

- Test system DL setting uncertainty, +/-dB

- other parameters provided by the minimum conformance requirement (Refsens level, CHBW, allocation etc.)

Available DL power at CW 1dB compression at QZ is dependent on the frequency but not on the power class.

Test system DL setting uncertainty depends on power class and frequency range. The dependence of power class is via the antenna pattern assumptions and via the power level range required in demod testing.

Test system DL setting uncertainty has the same MU contributors as Refsens EIS and has been assessed only for IFF systems. These MU contributors have to be assessed for each power class and frequency range.

In the demod SNR calculator, the Refsens MU EIS was reused after checking that the values of MU contributors in Stage 2: DUT measurement (Modulated Interferer Signal specific contributions) of the ACS TC have the same values as Stage 2: DUT measurement for Refsens EIS measurement. In this way, it is ensured that the power level range of the demod TCs is covered which may not be the case if one focuses only on Refsens EIS MU.

The list of MU contributors relevant for the Test system DL setting uncertainty are:

| UID |  | Description of uncertainty contribution |  | Details in clause |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 |  | Positioning misalignment |  | B.2.2.1 |  |  |
| 2 |  | Measure distance uncertainty |  | B.2.2.2 |  |  |
| 3 |  | Quality of Quiet Zone |  | B.2.2.3 |  |  |
| 4 |  | Mismatch |  | B.2.2.4 |  |  |
| 5 |  | Standing wave between the DUT and measurement antenna |  | B.2.2.5 |  |  |
| 6 |  | gNB emulator uncertainty |  | B.2.2.17 |  |  |
| 7 |  | Phase curvature |  | B.2.2.7 |  |  |
| 8 |  | Amplifier uncertainties |  | B.2.2.8 |  |  |
| 9 |  | Random uncertainty |  | B.2.2.9 |  |  |
| 10 |  | Influence of the XPD |  | B.2.2.10 |  |  |
| 11 |  | Insertion Loss Variation |  | B.2.2.11 |  |  |
| 12 |  | RF leakage (from measurement antenna to the receiver/transmitter) |  | B.2.2.12 |  |  |
| 13 |  | Multiple measurement antenna uncertainty |  | B.2.2.25 |  |  |
| 14 |  | DUT repositioning |  | B.2.2.26 |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 16 |  | Mismatch |  | B.2.2.4 |  |  |
| 17 |  | Amplifier Uncertainties |  | B.2.2.8 |  |  |
| 18 |  | Misalignment of positioning System |  | B.2.2.13 |  |  |
| 19 |  | Uncertainty of the Network Analyzer |  | B.2.2.14 |  |  |
| 20 |  | Uncertainty of the absolute gain of the calibration antenna |  | B.2.2.15 |  |  |
| 21 |  | Positioning and pointing misalignment between the reference antenna and the measurement antenna |  | B.2.2.16 |  |  |
| 22 |  | Phase centre offset of calibration antenna |  | B.2.2.18 |  |  |
| 23 |  | Quality of quiet zone for calibration process |  | B.2.2.19 |  |  |
| 24 |  | Standing wave between reference calibration antenna and measurement antenna |  | B.2.2.20 |  |  |
| 25 |  | Influence of the calibration antenna feed cable |  | B.2.2.21 |  |  |
| 26 |  | Insertion Loss Variation |  | B.2.2.11 |  |  |
|  | Systematic uncertainties |  |  |  |  |
|  | 27 |  | Systematic error related to beam peak search |  | B.2.2.28 |

Based on inputs from R&S and Ericsson, below is the preliminary list of open MU items for PC1

- Annex B of TS 38.133 contains FFS for PCs other than PC3. The values in this Annex have been determined in RAN4 mainly by chipset and UE vendors since they depend on the UE implementation such as UE antenna gain. One of the many examples is presented below:

- Uncertainty assessment for DL AWGN absolute power or wanted DL signal absolute power is currently FFS for PC1

- UE gain differences such as the ones listed under Annex B.2.1.5.2, B.2.1.5.3 of TS 38.133 are pending to be defined by RAN4 for PC other than PC3.

- Requirements in RAN4 needs to be double checked as in some test cases values are specified separate for PC1 and separate for other PCs, while the core requirements are separate for PC1 and PC5 and separate for other PCs. And this needs to be done case by case

- AoA Relative angular offset between active probes for power classes other than PC3 not specified

Apart from the UE REFSENS assumptions, the overall Test System DL setting uncertainty also is only defined for PC3. As mentioned in prop3, looking for inputs from RAN5 group on all the missing aspects needed to make DEMOD and RRM test cases applicable for device power class other than PC3 and device sizes > 30cm and <= 40cm.

Given there are many TBD for RRM PC1 in RAN4 spec, more work in RAN4 is needed before RAN5 can update the TT analysis.

To ensure the remaining open issues for RRM PC1 is tracked, as per suggestion from Ericsson, below line item is added to the FR2 RRM test cases: Known Issue List document is maintained every meeting.

| 11 | Expanding RRM test coverage to FWA devices | Various | TBD | Needs research |  | Needs research | R5-224808 | RAN5-98 | Research required what is left in RAN4 and in RAN5 specification to be defined before RRM test cases can be extended to cover FWA devices |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## 3.	Conclusion

Observation1: In order to expand the applicability to other device power classes esp PC1, the Demod SNR calculator needs to be redone using PC1 UE REFSENS

Proposal1:  Revise 38.903 Demod SNR calculator to include PC1. Revisit PC5 discussion once there is agreement on antenna assumptions for PC5 devices.

Proposal2: In order to expand the applicability of DEMOD test cases for device size >30cm and <= 40cm, go with the working assumption of existing available DL power at CW 1 dB compression at QZ.

Proposal3: Identify the list of MU terms that are still missing for PC1 and are needed to make the DEMOD and RRM test case applicable.

Proposal4: Redo RRM TT analysis to include UE REFSENS for PC1 and other power class

## 4.		References

[1]  TS 38.521-4 v16.11.0

[2]  TS 38.533 v17.3.0

[3]  TR 38.903 v16.12