3GPP TSG-RAN5 Meeting #96-e	R5-225688

# Electronic Meeting, Aug 15 - 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0356 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update RF Baseline Implementation Capabilities for PC2 duty cycle |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RF Baseline Implementation Capabilities “maxUplinkDutyCycle-PC2-FR1” is absent and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | “maxUplinkDutyCycle-PC2-FR1” has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE conformance test for PC2 MOP will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The related CR is R5-224209. Table A.4.3.1-4g should be implemented before R5-224209. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

A.4.3	Baseline Implementation Capabilities

<<< UNCHANGED TEXT SKIPPED >>>

Table A.4.3.1-4f: NR FR1 maxNumberSRS-Ports-PerResource RF Baseline Implementation Capabilities (Rel-15)

| Item | NR Band | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported maxNumberSRS-Ports-PerResource |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | n41 | NR_n41 maxNumberSRS-Ports-PerResource | 38.306, 4.2.7.7 | Rel-15 | pc_nrBand41_maxNumberSRS-Ports-PerResource_r15 |  |

Table A.4.3.1-4g: NR FR1 maxUplinkDutyCycle-PC2-FR1 RF Baseline Implementation Capabilities

| Item | NR Band | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic Parameter Name | Parameter Type | Supported Value | Supported UE capability (NOTE 1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n41 | NR_n41 maxUplinkDutyCycle-PC2-FR1 | 38.306, 4.2.7.2 | Rel-15 | pc_nrBand41_maxUplinkDutyCycle_PC2_FR1 | enumerated | n60, n70, n80, n90, n100 |  |
| 2 | n79 | NR_n79 maxUplinkDutyCycle-PC2-FR1 | 38.306, 4.2.7.2 | Rel-15 | pc_nrBand79_maxUplinkDutyCycle_PC2_FR1 | enumerated | n60, n70, n80, n90, n100 |  |
| NOTE 1: The UE supplier shall indicate the supported maxUplinkDutyCycle-PC2-FR1 as per RF-Parameters in TS 38.331 Section 6.3.3 UE capability information elements and choose the supported value. |  |  |  |  |  |  |  |  |

Table A.4.3.1-5: NR SUL FR1 RF Baseline Implementation Capabilities

| Item | NR SUL FR1 RF Baseline Implementation Capabilities |  | Ref. |  | Release |  | Mnemonic |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 1710-1785 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand80_Supp |  | NR SUL FR1 Band n80 |  |
| 2 | NR Frequency band: 880-915 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand81_Supp |  | NR SUL FR1 Band n81 |  |
| 3 | NR Frequency band: 832-862 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand82_Supp |  | NR SUL FR1 Band n82 |  |
| 4 | NR Frequency band: 703-748 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand83_Supp |  | NR SUL FR1 Band n83 |  |
| 5 | NR Frequency band: 1920-1980 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand84_Supp |  | NR SUL FR1 Band n84 |  |
| 6 | NR Frequency band: 1710-1780 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand86_Supp |  | NR SUL FR1 Band n86 |  |
| 6a to 6b | Reserved |  |  |  |  |  |  |  |  |  |
| 6c | NR Frequency band: 824-849 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand89_Supp |  | NR SUL FR1 Band n89 |  |
| 7 | NR Frequency band: 2010-2025 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand95_Supp |  | NR SUL FR1 Band n95 |  |
| 8 | NR Frequency band: 2300 MHz – 2400 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-17 |  | pc_nrBand97_Supp |  | NR SUL FR1 Band n97 |  |
| 9 |  | NR Frequency band: 1626.5-1660.5 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-17 |  | pc_nrBand99_Supp |  | NR SUL FR1 Band n99 |

<< END OF CHANGES >>