3GPP TSG-RAN5 Meeting #96-e	R5-225735

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1868 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General updates of clause 5 for R17 CADC configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Nokia, WE Certification |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some R17 NR CADC combos which are already 100% completion in RAN4 and assigned to interested companies need to be added into operating bands and channel bandwidth of TS38.521-1 clause 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added 13 4CA combinations to Table 5.5A.3.2-1. Added Table 5.5B.1-1. Remove the original Table 5.5A.3.2-1, add a new Table to keep the format consistent with TS 38.101-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The activated R17 CADC configurations in clause 5 will be still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.3 5.2B 5.5B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Multiple WIC covered in the same CR, the R16(NR_CADC_NR_LTE_DC_R16-UEConTest ) is main WIC. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is an outcome from R5-225022r1. |  |  |  |  |  |  |  |  |

<< Start of changes >>

5.5A.3.2	Configurations for inter-band CA (three bands)

Table 5.5A.3.2-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A-n79A4 | CA_n1A-n78A CA_n1A-n79A CA_n78A-n79A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90 |  |  |
|  |  | n79 | 40, 50, 60, 80 |  |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90 |  |  |
|  |  | n79 | 40, 50, 60, 80 |  |  |
| CA_n26A-n66A-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |
| CA_n26A-n66(2A)-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |
| CA_n29A-n66A-n70A | - | n29 | 5, 10 | 0 |  |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |
| CA_n29A-n66B-n70A | - | n29 | 5, 10 | 0 |  |  |
|  |  | n66 | CA_n66B_BCS0. |  |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |
| CA_n29A-n66(2A)-n70A | - | n29 | 5, 10 | 0 |  |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |
| CA_n48A-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |
| CA_n48A-n66(2A)-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |
| CA_n48(2A)-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | CA_n48(2A)_BCS1 | 0 |  |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |
| CA_n48B-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | CA_n48B_BCS2 | 0 |  |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |
| CA_n48A-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
|  |  | n71 | 5, 10, 15, 20 |  |  |  |
| CA_n48A-n66(2A)-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |  |  |
|  |  | n71 | 5, 10, 15, 20 |  |  |  |
| CA_n48(2A)-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | CA_n48(2A)_BCS1 | 0 |  |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
|  |  | n71 | 5, 10, 15, 20 |  |  |  |
| CA_n48B-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | CA_n48B_BCS2 | 0 |  |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
|  |  | n71 | 5, 10, 15, 20 |  |  |  |
| CA_n48A-n66A-n71(2A) | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |  |  |
| CA_n48A-n70A-n71A | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |  |
|  |  | n71 | 5, 10, 15, 20 |  |  |  |  |
| CA_n48(2A)-n70A-n71A | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | CA_n48(2A)_BCS1 | 0 |  |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |  |  |
|  |  | n71 | 5, 10, 15, 20 |  |  |  |  |  |
| CA_n48A-n70A-n71(2A) | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |  |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |  |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |  |  |  |  |
| CA_n66A-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | 5, 10, 15, 20, 40 | 0 |  |  |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |  |  |
|  |  | n71 | 5, 10, 15, 20 |  |  |  |  |  |
| CA_n66A-n70A-n71(2A) | CA_n66A-n71A CA_n70A-n71A | n66 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |  |  |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |  |  |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |  |  |  |  |  |
| CA_n66B-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66B_BCS0 |  | 0 |  |  |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |  |  |  |
|  |  | n71 | 5, 10, 15, 20 |  |  |  |  |  |  |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66(2A)_BCS0 |  | 0 |  |  |  |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |  |  |  |
|  |  | n71 | 5, 10, 15, 20 |  |  |  |  |  |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n78 with an n77 implementation. |  |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

5.2B	Operating bands for DC

The operating bands are specified in clause 5.5B for operation with NR dual connectivity configured, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a band combination, the mandatory simultaneous Rx/Tx capability also applies for the band combination when the applicable band combination is a subset of a higher order band combination.

<<Unchaged sections skipped>>

5.5B	Configurations for DC

For an NR-DC configuration specified in Table 5.5B.1-1, the bandwidth combination sets for the corresponding NR CA configuration in 5.5A.3,i.e.,dual uplink inter-band carrier aggregation with uplink assigned to two NR bands, are applicable to Dual Connectivity. If UE supporting NR-DC configuration do not support the corresponding CA configuration, NR-DC configuration is used in CA test cases.

Table 5.5B.1-1: Inter-band NR-DC configurations (two bands)

| NR-DC configuration | Uplink NR-DC configuration |
| --- | --- |
| DC_n48A-n70A | DC_n48A-n70A |

<<End of changes>>