3GPP TSG-RAN5 Meeting #96-e	R5-225767

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0380 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update PICS for RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, There is no PICS for HD-FDD for RedCap UE, and RF test need this PICS |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Add new PICS to 38.508-2 for RedCap UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no PICS for HD-FDD for RedCap UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revised from R5-225189 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support PDSCH reception based on semi-persistent scheduling | 38.306, 4.2.7.10 | Rel-15 | pc_downlinkSPS | No |  |  |
| 2 | Support 256QAM for PDSCH for FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_256QAM_FR1 | CY |  | Mandatory for non-RedCap UEs and optional for RedCap UEs. |
| 3 | Support 256QAM for PDSCH for at least one NR FR2 band | 38.306, 4.2.7.2 | Rel-15 | pc_pdsch_256QAM_FR2 | No |  |  |
| 4 | Support 256QAM for PUSCH for at least one NR FR1 band | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR1 | No |  |  |
| 4a | Support 256QAM for PUSCH for at least one NR FR2 band | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR2 | No |  |  |
| 5 | Support receiving PDSCH using PDSCH mapping type A with less than seven symbols | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_MappingTypeA | Yes | Yes |  |

<End of modified section 1>

<Start of modified section 2>

A.4.3.12	RedCap Capabilities

According to TS 38.306 [17] clause 4.2.21:

-	PICS associated to the following features shall not be set to 'true':

-	CA: PICS defined in clause A.4.3.2A;

-	MR-DC: PICS defined in clause A.4.3.2B;

-	DAPS: PICS include pc_intraFreqDAPS_r16, interFreqDAPS_r16 and other DAPS related PICS;

-	CPAC: PICS include pc_condPSCellChange_r16 and other CPAC related PICS.

-	A RedCap UE shall not support features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 2 UE Tx branches or more than 2 UL MIMO layers.

Table A.4.3.12-1: RedCap Capabilities

| Item | UE Sidelink Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support of 16 DRBs for RedCap UEs. | 38.306, 4.2.21.2 | Rel-17 | pc_16DRB_RedCap _r17 | No |  |  |
| 2 | Support of RedCap. | 38.306, 4.2.21.2 | Rel-17 | pc_RedCap_r17 | No |  |  |
| 3 | Support of 18 bit length of PDCP sequence for RedCap UEs. | 38.306, 4.2.21.3 | Rel-17 | pc_longSN_RedCap_r17 | No |  |  |
| 4 | Support of AM DRB with 18 bit length of RLC sequence number for RedCap UEs. | 38.306, 4.2.21.4 | Rel-17 | pc_ amWithLongSN_RedCap_r17 | No |  |  |
| 5 | Support of Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE. | 38.306 4.2.21.6.1 | Rel-17 | pc_halfDuplexFDD_TypeA_RedCap_r17 | No |  |  |

<End of modified section 2>

<Start of modified section 3>

A.4.3.3	PDCP Implementation Capabilities

Table A.4.3.3-1: UE PDCP Implementation Capabilities

| Item |  | UE PDCP Implementation Capabilities |  | Ref. |  | Release |  | Mnemonic |  | M |  | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Support 12 bit length of PDCP sequence number |  | 38.306, 4.2.4 |  | Rel-15 |  | pc_shortSN |  | Yes |  | Yes (for RedCap UE) |  | This PICS shall always be true for RedCap UE. |  |
| 2 |  | Supports Out of order delivery of data to upper layers by PDCP |  | 38.306, 4.2.4 |  | Rel-15 |  | pc_outOfOrderDelivery |  | No |  |  |  |  |  |
| 3 |  | Support CA-based PDCP duplication over MCG or SCG DRB |  | 38.306, 4.2.4 |  | Rel-15 |  | pc_pdcp_DuplicationMCG_OrSCG_DRB |  | No |  |  |  |  |  |
| 4 |  | Support PDCP duplication over split DRB |  | 38.306, 4.2.4 |  | Rel-15 |  | pc_pdcp_DuplicationSplitDRB |  | No |  |  |  |  |  |
| 5 |  | Support PDCP duplication with more than two RLC entities |  | 38.306, 4.2.4 |  | Rel-16 |  | pc_pdcp_DuplicationMoreThanTwoRLC_r16 |  | No |  |  |  | specifically for TSC (time sensitive communication) services |  |
| 6 |  | Support PDCP duplication over split SRB1/2 |  | 38.306, 4.2.4 |  | Rel-15 |  | pc_pdcp_DuplicationSplitSRB |  | No |  |  |  |  |  |
| 7 |  | Support EHC (Ethernet header compression) |  | 38.306, 4.2.4 |  | Rel-16 |  | pc_NR_ehc_r16 |  | No |  |  |  | specifically for TSC (time sensitive communication) services |  |

<End of modified section 3>

<Start of modified section 4>

A.4.3.4	RLC Implementation Capabilities

Table A.4.3.4-1: UE RLC Implementation Capabilities

| Item | UE RLC Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support RLC AM with 12 bit length of RLC sequence number | 38.306, 4.2.5 | Rel-15 | pc_am_WithShortSN | Yes | Yes (for RedCap UE) | This PICS shall always be true for RedCap UE. |
| 2 | Support RLC UM with 12 bit length of RLC sequence number | 38.306, 4.2.5 | Rel-15 | pc_um_WIthLongSN | Yes |  |  |
| 3 | Support RLC UM with 6 bit length of RLC sequence number | 38.306, 4.2.5 | Rel-15 | pc_um_WithShortSN | Yes |  |  |

<End of modified section 4>