3GPP TSG-RAN5 Meeting #96-e	R5-223989r1

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0352 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MDT PICS for location reporting of inter-RAT measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As TC 8.1.6.2.1 needs to configure includeCommonLocationInfo in PeriodicalReportConfigInterRAT to support location reporting in inter-RAT measurement as mentioned in TP1.However this is a feature added in R2-2106772 Change 7. Therefore a new PICS is proposed to address this feature. Change#7 (as captured in R2-2104002): In NR, according to the 5.4.1 of 37.320 and the 9.3.1.169 of 38.413, both M1 and location reporting can be configured simultaneously, and it means the location reporting in the inter-RAT measurement is allowed. However, it is impossible for the network to include location configuration in inter-RAT measurements for connected mode, and also impossible for the UE to report the location info correspondingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New PICS was added in Table A.4.4-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC 8.1.6.2.1 will be selected to run incorrectly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0231 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: replace “-” with “_”. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

A.4.4	Additional information

Table A.4.4-1: Additional information

| Item | Additional information | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support of ICMP or ICMP IPv6 | RFC 792 OR RFC 4443, RFC 4884 | NA | pc_IP_Ping | UE supports ICMP or ICMPv6 protocol to enable IP Ping Operation |
| 2 | Support of IMS | 24.229, Annex U | Rel-15 | pc_IMS_5GS |  |
| 3 | Support of rachReport | 38.306, 4.2.17 | Rel-16 | pc_rachReport_r16 | UE supports delivery of rachReport upon request from the network. |
| 4 | Support of GNSS | 38.306, 4.2.18 | Rel-16 | pc_GNSS_location_r16 | UE is equipped with a GNSS or A-GNSS receiver that may be used to provide detailed location information along with SON or MDT related measurements in RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE. |
| 5 | Support of UL PDCP Packet Average Delay | 38.306, 4.2.18 | Rel-16 | pc_PDCP_Delay_r16 | UE supports UL PDCP Packet Average Delay measurement and reporting in RRC_CONNECTED state |
| 6 | Support logged MDT | 38.306, 4.2.18 | Rel-16 | pc_loggedMeasurements_r16 | UE supports logged measurements in RRC_IDLE and RRC_INACTIVE. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The memory size of MDT logged measurements is 64KB. |
| 7 | Support of uncompensated barometric pressure measurement reporting | 38.306, 4.2.18 | Rel-16 | pc_barometer_r16 | UE supports uncompensated barometric pressure measurement reporting upon request from the network. |
| 8 | Support of orientation information reporting | 38.306, 4.2.18 | Rel-16 | pc_orientation_r16 | UE supports orientation information reporting upon request from the network. |
| 9 | Support of speed information reporting | 38.306, 4.2.18 | Rel-16 | pc_speed_r16 | UE supports speed information reporting upon request from the network. |
| 10 | Support of Bluetooth measurements in RRC_CONNECTED state | 38.306, 4.2.18 | Rel-16 | pc_immMeasBT_r16 | UE supports Bluetooth measurements in RRC_CONNECTED state. |
| 11 | Support of WLAN measurements in RRC_CONNECTED state | 38.306, 4.2.18 | Rel-16 | pc_immMeasWLAN_r16 | UE supports WLAN measurements in RRC_CONNECTED state. |
| 12 | Support of Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state | 38.306, 4.2.18 | Rel-16 | pc_loggedMeasBT_r16 | UE supports Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state. |
| 13 | Support of WLAN measurements in RRC_IDLE and RRC_INACTIVE state | 38.306, 4.2.18 | Rel-16 | pc_loggedMeasWLAN_r16 | UE supports WLAN measurements in RRC_IDLE and RRC_INACTIVE state. |
| xx | Support of location reporting in inter-RAT measurement | 38.331, 6.3.2 | Rel-16 | pc_locationReporting_inter_RAT_r16 | UE supports location reporting in inter-RAT measurement. |

[End of change]