3GPP TSG-RAN5 Meeting #96-e	R5-224071r4
Electronic Meeting, 15th August – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0762 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New test case addition: 6.2.4_1 Configured transmitted power with Power Boost |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Rel-16 FR2 RF enhancements related to Power Boost have not been defined in current test specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define a new test case to enable testing of Rel-16 FR2 RF Enhancements related to Power Boost. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specifications will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4_1 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is dependent on R5-224046. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: CR title corrected to remove redundant information on test specification. Revision 2: - Reference to message contents section corrected in test procedure step 1. - Table title added in message contents section. - Step 3 removed and added in ServingCellConfig table in message contents section, test steps renumbered in test procedure and reference corrected in test requirement table accordingly. Revision 3: - Reference corrected in section 6.2.4_1.3 - Message contents table simplified to only show updated values. - Multi-band test requirements for Rel-15 removed and tables renumbered. Revision 4: - Table 6.2.4_1.5-3b title corrected to remove (Rel-16 and forward) |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### 6.2.4	Configured transmitted power

6.2.4.1	Test purpose

To verify the UE configured transmitted power PUMAX,f,c is within the range defined prescribed by the specified nominal maximum output power and tolerance.

6.2.4.2	Test applicability

The requirements of this test are covered in test cases 6.2.1 Maximum output power, 6.2.2 Maximum output power reduction and 6.2.3 UE maximum output power with additional requirements to all types of NR UE release 15 and forward.

6.2.4.3	Minimum conformance requirements

The UE can configure its maximum output power. The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [24].

The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds

PPowerclass + DPIBE – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax

while the corresponding measured total radiated power PTMAX,f,c is bounded by

PTMAX,f,c ≤ TRPmax

with PPowerclass the UE minimum peak EIRP as specified in sub-clause 6.2.1.1.3, EIRPmax the applicable maximum EIRP as specified in sub-clause 6.2.1.1.3, MPRf,c as specified in sub-clause 6.2.2.3, A-MPRf,c as specified in sub-clause 6.2.3.3, ΔMBP,n the peak EIRP relaxation as specified in section 6.2.1.1.3 and TRPmax the maximum TRP for the UE power class as specified in sub-clause 6.2.1.1.3. DPIBE is 1.0 dB if UE declares support for mpr-PowerBoost-FR2-r16, UL transmission is QPSK, MPRf,c = 0 and when NS_200 applies and the network configures the UE to operate with mpr-PowerBoost-FR2-r16, otherwise DPIBE is 0.0 dB. The requirement is verified in beam peak direction.

maxUplinkDutyCycle-FR2 as defined in TS 38.306 [26] is a UEcapability to facilitate electromagnetic power density exposure requirements. This UE capability is applicable to all FR2 power classes.

If the field of UE capability maxUplinkDutyCycle-FR2 is present and the percentage of uplink symbols transmitted within any 1 s evaluation period is larger than maxUplinkDutyCycle-FR2, the UE follows the uplink scheduling and can apply P-MPRf,c.

If the field of UE capability maxUplinkDutyCycle-FR2 is absent, the compliance to electromagnetic power density exposure requirements are ensured by means of scaling down the power density or by other means.

P-MPRf,c is the power management maximum output power reduction. The UE shall apply P-MPRf,c for carrier f of serving cell c only for the cases described below. For UE conformance testing P-MPRf,c shall be 0 dB.

a)	ensuring compliance with applicable electromagnetic power density exposure requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic power density exposure requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

NOTE 1:	P-MPRf,c was introduced in the PCMAX,f,c equation such that the UE can report to the gNB the available maximum output transmit power. This information can be used by the gNB for scheduling decisions.

NOTE 2:	P-MPRf,c and maxUplinkDutyCycle-FR2 may impact the maximum uplink performance for the selected UL transmission path.

NOTE 3:	MPE P-MPR Reporting, as defined in TS 38.306 [26], is an optional UE capability to report P-MPRf,c when the reporting conditions configured by gNB are met. This UE capability is applicable to all FR2 power classes.

The tolerance T(∆P) for applicable values of ∆P (values in dB) is specified in Table 6.2.4.3-1.

Table 6.2.4.3-1: PUMAX,f,c tolerance

| Operating Band | ∆P (dB) | Tolerance T(∆P) (dB) |
| --- | --- | --- |
| n257, n258, n259, n260, n261 | P = 0 | 0 |
|  | 0 < P ≤ 2 | 1.5 |
|  | 2 < P ≤ 3 | 2.0 |
|  | 3 < P ≤ 4 | 3.0 |
|  | 4 < P ≤ 5 | 4.0 |
|  | 5 < P ≤ 10 | 5.0 |
|  | 10 < P ≤ 15 | 7.0 |
|  | 15 < P ≤ X | 8.0 |
| NOTE: X is the value such that Pumax,f,c lower bound, PPowerclass - P – T(P) = minimum output power specified in clause 6.3.1. |  |  |

6.2.4.4	Test description

This test is covered by clause 6.2.1 Maximum output power, 6.2.2 Maximum output power reduction and 6.2.3 UE maximum output power with additional requirements.

6.2.4.5	Test requirements

This test is covered by clause 6.2.1 Maximum output power, 6.2.2 Maximum output power reduction and 6.2.3 UE maximum output power with additional requirements.

### 6.2.4_1	Configured transmitted power with Power Boost

Editor’s note: The following aspects are either missing or not yet determined:

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

-	The test case is incomplete for band n259.

6.2.4_1.1	Test purpose

To verify the UE configured transmitted power PUMAX,f,c is within the range defined prescribed by the specified nominal maximum output power and tolerance.

6.2.4_1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward supporting mpr-PowerBoost-FR2-r16 UE capability.

6.2.4_1.3	Minimum conformance requirements

Same as clause 6.2.4.3.

6.2.4_1.4	Test description

6.2.4_1.4.1	Initial conditions

Same as clause 6.2.1.1.4.1

6.2.4_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Messages to configure the appropriate uplink modulation in section 6.2.4_1.4.3.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2.4_1.5-1 to 6.2.4_1.5-4. EIRP test procedure is defined in Annex K.1.3. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

6.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.2.4_1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config with the following exceptions:

Table 6.2.4_1.4.3-1: ServinCellConfig

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| mpr-PowerBoost-FR2-r16 | True |  |  |
| } |  |  |  |
| } |  |  |  |

6.2.4_1.5	Test requirement

The EIRP derived in step 5 shall not exceed the values specified in Table 6.2.4_1.5-1 to Table 6.2.4_1.5-4.

Table 6.2.4_1.5-1: UE maximum output test requirements for power class 1

| Operating band | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- |
| n257 | 55 | 41.0-TT |
| n258 | 55 | 41.0-TT |
| n260 | 55 | 39.0-TT |
| n261 | 55 | 41.0-TT |

Table 6.2.4_1.5-2: UE maximum output test requirements for power class 2

| Operating band | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- |
| n257 | 43 | 30-TT |
| n258 | 43 | 30-TT |
| n260 |  |  |
| n261 | 43 | 30-TT |

Table 6.2.4_1.5-3: UE maximum output test requirements for power class 3 for single band UE

| Operating band | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- |
| n257 | 43 | 23.4-TT |
| n258 | 43 | 23.4-TT |
| n260 | 43 | 21.6-TT |
| n261 | 43 | 23.4-TT |

Table 6.2.4_1.5-3a: Test Tolerance (Min peak EIRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 2.87 dB (NTC) 3.04 (ETC) | 2.87 dB (NTC) 3.04 (ETC) |

Table 6.2.4_1.5-3b: UE maximum output test requirements for power class 3

| ID | FR2 bands/set | Test requirement (dB) (Note 1) |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n257 | n258 | n259 | n260 | n261 |  |
| 1 | n257 | 23.4-TT-MBP,n |  |  |  |  |  |
| 2 | n258 |  | 23.4-TT-MBP,n |  |  |  |  |
| 3 | n259 |  |  | 19.7-TT-MBP,n |  |  |  |
| 4 | n260 |  |  |  | 21.6-TT-MBP,n |  |  |
| 5 | n261 |  |  |  |  | 23.4-TT-MBP,n |  |
| 6 | n257, n261 | 23.4-TT-MBP,n |  |  |  | 23.4-TT-MBP,n | MBP,n relaxation is 0 dB |
| 7 | n260, n261 |  |  |  | 21.6-TT-MBP,n | 23.4-TT-MBP,n | MBP,n relaxation is 0 dB |
| Note 1: MBP,n is the Multi-band Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

Table 6.2.4_1.5-4: UE maximum output power test requirements for power class 4

| Operating band | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- |
| n257 | 43 | 35-TT |
| n258 | 43 | 35-TT |
| n260 | 43 | 35-TT |
| n261 | 43 | 35-TT |

## 6.2A	Transmit power for CA

### 6.2A.1	UE maximum output power for CA

## <<< END OF CHANGES >>>