3GPP TSG-RAN5 Meeting #96-e	R5-224096r1

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3042 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Inter-System MDT test cases 8.1.6.3.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The former R5-223496 have identified the issue related to UE being not able to clear VarConnEstFailReport in other way than after the 48h expiry or after a successful delivery of the collected data to the network within UEInformationResponse. If due to some reasons UE would experience connection failures during the regression, it will subsequently(constantly) ping the network setting connEstFailInfoAvailable-r1 flag and this may cause a conformant UE to fail the test case |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE-MeasurementsAvailable-r16 was updated to reference the default message content in TS 38.508-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail SON-MDT test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.3.2.1.3.3, 8.1.6.3.2.2.3.3, 8.1.6.3.3.1.3.3, 8.1.6.3.4.1.3.3, 8.1.6.3.4.2.3.3 and 8.1.6.3.4.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-1 CR 2429 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: specify the MDT irrelated IEs as “Not checked” in the specific message contents. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

8.1.6.3.2	Inter-System MDT / Logged MDT

8.1.6.3.2.1	Inter-System MDT / Logged MDT / Logging and reporting / Bluetooth measurement collection

8.1.6.3.2.1.1	Test Purpose (TP)

(1)

with { UE received RRCSetup message }

ensure that {

when { UE has Bluetooth logged measurements available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE transmits the logMeasAvailableBT in the RRCSetupComplete message }

}

(2)

with { UE in NR RRC_CONNECTED state and UE has Bluetooth logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

ensure that {

when { receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r16 including logMeasResultListBT }

}

(3)

with { UE has Bluetooth logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and a Bluetooth beacon is not included in BT-NameList }

ensure that {

when { UE in NR RRC_CONNECTED state and receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r16 not including the measurement result of the Bluetooth beacon not included in BT-NameList }

}

8.1.6.3.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clauses 5.5a.1.3, 5.5a.3.2, 5.3.3.7. Unless otherwise stated these are Rel-16 requirements

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-NameList, if included, in VarLogMeasConfig;

1>	store the received sensor-NameList, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause 5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]); or

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig:

5>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

…

2>	when performing the logging:

…

3>	if location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

8.1.6.3.2.1.3	Test description

8.1.6.3.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, Bluetooth beacon 1 (Cell 40) and Bluetooth beacon 2 (Cell 41)

-	Cell 40 and Cell 41 are configured as per TS 38.508-1 [4] cl 4.4.1.3.2

UE:

None

Preamble:

-	The UE is in state 3N-A on NR Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.3.2.1.3.2	Test procedure sequence

Table 8.1.6.3.2.1.3.2-1/2 illustrates the downlink power levels. Row marked "T0" denotes the conditions after the preamble.

Table 8.1.6.3.2.1.3.2-1: Time instances of cell power level and parameter changes (FR1)

|  | Parameter | Unit | NR Cell 1 | Cell 40 (Bluetooth beacon 1) | Cell 41 (Bluetooth beacon 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 |  |  |  |
|  | BeaconRSSI | dBm |  | -80 | -50 |  |

Table 8.1.6.3.2.1.3.2-2: Time instances of cell power level and parameter changes (FR2)

|  | Parameter | Unit | NR Cell 1 | Cell 40 (Bluetooth beacon 1) | Cell 41 (Bluetooth beacon 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 |  |  |  |
|  | BeaconRSSI | dBm |  | -80 | -50 |  |

Table 8.1.6.3.2.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |
| 3 | Wait 8s to allow UE to activate logging. | - | - | - | - |
| 4 | The SS transmits a Paging message on Cell 1. | <-- | Paging | - | - |
| 5 | The UE transmits an RRCRequest message on Cell 1. | --> | RRCRequest | - | - |
| 6 | SS transmit an RRCSetup message on Cell 1. | <-- | RRCSetup | - | - |
| 7 | Check: Does the UE transmit an RRCSetupComplete message with logMeasAvailableBT on Cell 1? | --> | RRCSetupComplete | 1 | P |
| 8-11 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 12 | The SS transmits a UEInformationRequest message on Cell 1. | <-- | UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfo-r16 with one entry (Bluetooth beacon 1) measurement result in the IE logMeasResultListBT? | --> | UEInformationResponse | 2,3 | P |

8.1.6.3.2.1.3.3	Specific message contents

Table 8.1.6.3.2.1.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.3.2.1.3.2-3)

| Derivation path: TS 38.508-1 [4], table 4.6.1-5AA with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| bt-NameList-r16 SEQUENCE { SIZE (1..maxBT-Name-r16)) OF | 1 entry | Bluetooth beacon 1 |  |
| BT-Name-r16[1] | Set as per Table 4.4.9-1 of 3GPP TS 36.508 | OCTET STRING (SIZE (1..248)) Refers to LOCAL NAME defined in Bluetooth specification |  |
| } |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms10240 | 10.24s |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.2.1.3.3-2: RRCSetupComplete (step 7, Table 8.1.6.3.2.1.3.2-3)

| Derivation path: TS 38.508-1 [4], table 4.6.1-22 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| rrc-TransactionIdentifier |  | Not checked |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| iab-NodeIndication-r16 |  | Not checked |  |  |  |  |  |
| idleMeasAvailable-r16 |  | Not checked |  |  |  |  |  |
| ue-MeasurementsAvailable-r16 |  | UE-MeasurementsAvailable-r16 with condition LOG and BT |  |  |  |  |  |
| mobilityHistoryAvail-r16 |  | Not checked |  |  |  |  |  |
| mobilityState-r16 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.1.6.3.2.1.3.3-3: UEInformationRequest (step 12, Table 8.1.6.3.2.1.3.2-3)

Derivation Path: TS 38.508-1 [4], table 4.6.1-32A with Condition LOG

Table 8.1.6.3.2.1.3.3-4: UEInformationResponse (step 13, Table 8.1.6.3.2.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceRecordingSessionRef-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 { |  |  |  |  |
| LogMeasInfo-r16[1] SEQUENCE { |  |  | 1 entry |  |
| locationInfo-r16 SEQUENCE { |  |  |  |  |
| commonLocationInfo-r16 |  | Not checked |  |  |
| bt-LocationInfo-r16 SEQUENCE { |  |  |  |  |
| LogMeasResultListBT-r16 ::= SEQUENCE (SIZE (1..maxBT-IdReport-r16)) OF LogMeasResultBT-r16 { |  | 1 entry |  |  |
| LogMeasResultBT-r16[1] SEQUENCE { |  |  | Report Bluetooth beacon 1 |  |
| bt-Addr-r16 |  | Bluetooth public address of Bluetooth beacon 1 | BIT STRING (SIZE (48)) |  |
| rssi-BT-r16 |  | Any allowed value | INTEGER (-128..127) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| wlan-LocationInfo-r16 |  | Not present |  |  |
| sensor-LocationInfo-r16 |  | Not present |  |  |
| } |  |  |  |  |
| relativeTimeStamp-r16 |  | Any allowed value | INTEGER (0..7200) |  |
| servCellIdentity-r16 |  | Same as Cell 1 |  |  |
| measResultServingCell-r16 |  | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |  |
| measResultNeighCellListNR |  | Not present |  |  |
| measResultNeighCellListEUTRA |  | Not present |  |  |
| } |  |  |  |  |
| anyCellSelectionDetected-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logMeasAvailable-r16 |  | Not present |  |  |
| logMeasAvailableBT-r16 |  | Not present |  |  |
| logMeasAvailableWLAN-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

8.1.6.3.2.2	Inter-System MDT / Logged MDT / Logging and reporting / WLAN measurement collection

8.1.6.3.2.2.1	Test Purpose (TP)

(1)

with { UE received RRCSetup message }

ensure that {

when { UE has WLAN logged measurements available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE transmits the RRCSetupComplete message with logMeasAvailable-r16 and logMeasAvailableWLAN-r16 included }

}

(2)

with { UE in NR RRC_CONNECTED state and UE has WLAN logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

ensure that {

when { receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r16 including logMeasResultListWLAN }

}

(3)

with { UE has WLAN logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and a WLAN AP is not included in wlan-NameList }

ensure that {

when { UE in NR RRC_CONNECTED state and receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r16 not including the measurement result of the WLAN AP not included in wlan-NameList }

}

8.1.6.3.2.2.2	Conformance requirements

Same as test case 8.1.6.3.2.1.

8.1.6.3.2.2.3	Test description

8.1.6.3.2.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, WLAN AP 1 (Cell 27) and WLAN AP 2 (Cell 27b)

-	Cell 27 and Cell 27b are configured as per TS 38.508-1 [4] cl 4.4.1.3 with the OFFLOAD condition

UE:

None

Preamble:

- The UE is in state 3N-A on NR Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.3.2.2.3.2	Test procedure sequence

Table 8.1.6.3.2.2.3.2-1/2 illustrates the downlink power levels. Row marked "T0" denotes the conditions after the preamble.

Table 8.1.6.3.2.2.3.2-1: Time instances of cell power level and parameter changes (FR1)

|  | Parameter | Unit | NR Cell 1 | Cell 27 (WLAN AP 1) | Cell 27b (WLAN AP 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 |  |  |  |
|  | BeaconRSSI | dBm |  | -60 | -50 |  |

Table 8.1.6.3.2.2.3.2-2: Time instances of cell power level and parameter changes (FR2)

|  | Parameter | Unit | NR Cell 1 | Cell 27 (WLAN AP 1) | Cell 27b (WLAN AP 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 |  |  |  |
|  | BeaconRSSI | dBm |  | -60 | -50 |  |

Table 8.1.6.3.2.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |
| 3 | Wait 8s to allow UE to activate logging. | - | - | - | - |
| 4 | The SS transmits a Paging message on NR Cell 1. | <-- | Paging | - | - |
| 5 | The UE transmits an RRCRequest message on NR Cell 1. | --> | RRCRequest | - | - |
| 6 | SS transmit an RRCSetup message on NR Cell 1. | <-- | RRCSetup | - | - |
| 7 | Check: Does the UE transmit an RRCSetupComplete message with logMeasAvailable-r16 and logMeasAvailableWLAN-r16 on NR Cell 1? | --> | RRCSetupComplete | 1 | P |
| 8-11 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 [4] table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 12 | The SS transmits a UEInformationRequest message on NR Cell 1. | <-- | UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfo-r16 with one entry (WLAN AP 1) measurement result in the IE logMeasResultListWLAN?. | --> | UEInformationResponse | 2, 3 | P |

8.1.6.3.2.2.3.3	Specific message contents

Same as test case 8.1.6.3.1.1 with the following difference:

Table 8.1.6.3.2.2.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.3.2.2.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| wlan-NameList-r16 CHOICE { |  |  |  |
| Setup | WLAN-NameList-r16 |  |  |
| } |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms10240 | 10.24s |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.2.2.3.3-1A: WLAN-NameList-r16 (Table 8.1.6.3.2.2.3.3-1)

| Derivation path: TS 38.508-1 [4], table 4.6.5-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-NameList-r16 ::= SEQUENCE (SIZE (1..maxWLAN-Name-r16)) OF { | 1 entry | WLAN AP1 |  |
| WLAN-Name-r16[1] | Set as per Table 4.4.8-1 of TS 36.508 | OCTET STRING (SIZE (1..32)) Refers to Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| } |  |  |  |

Table 8.1.6.3.2.2.3.3-2: RRCSetupComplete (step 7, Table 8.1.6.3.2.2.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| iab-NodeIndication-r16 | Not present |  |  |  |
| idleMeasAvailable-r16 | Not present |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG and WLAN |  |  |  |
| mobilityHistoryAvail-r16 | Not present |  |  |  |
| mobilityState-r16 | Not present |  |  |  |
| nonCriticalExtension | Not present |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.6.3.2.2.3.3-3: Void

Table 8.1.6.3.2.2.3.3-4: UEInformationResponse (step 13, Table 8.1.6.3.2.2.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 { |  |  |  |
| LogMeasInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| commonLocationInfo-r16 | Not checked |  |  |
| bt-LocationInfo-r1 | Not present |  |  |
| wlan-LocationInfo-r16 SEQUENCE (SIZE (1..maxWLAN-Id-Report-r16)) OF LogMeasResultWLAN-r16 { | 1 entry | Report WLAN AP 1 |  |
| LogMeasResultWLAN-r16[1] SEQUENCE { |  |  |  |
| wlan-Identifiers-r16 SEQUENCE { |  |  |  |
| ssid-r12 | ssid for WLAN AP 1 |  |  |
| bssid-r12 | Not present |  |  |
| hessid-r12 | Not present |  |  |
| } |  |  |  |
| rssiWLAN-r16 | Any allowed value |  |  |
| rtt-WLAN-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| sensor-LocationInfo-r16 | Not present |  |  |
| } |  |  |  |
| relativeTimeStamp-r16 | Any allowed value | INTEGER (0..7200) |  |
| servCellIdentity-r16 | Same as NR Cell 1 |  |  |
| measResultServCell-16 | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| anyCellSelectionDetected-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| …….logMeasAvailableWLAN-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[Skip unchanged text]

8.1.6.3.3	Inter-System MDT / Radio Link Failure

8.1.6.3.3.1	Inter-System MDT / Radio Link Failure / Logging and reporting / Bluetooth measurement collection

8.1.6.3.3.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state detecting radio link failure }

ensure that {

when { UE has radio link failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE sends the RRCReestablishmentComplete message with rlf-InfoAvailable included }

}

(2)

with { UE in RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq }

then { UE sends the UEInformationResponse message with rlf-Report-r16 including LocationInfo-r16 which includes logMeasResultListBT }

}

(3)

with { UE in RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq and a Bluetooth beacon is not included in bt-NameList }

then { UE sends the UEInformationResponse message not including the measurement result of the Bluetooth beacon not included in bt-NameList }

}

8.1.6.3.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clauses 5.3.5.9, 5.3.10.5, 5.3.3.7, 5.7.10.3, 5.3.7.5. Unless otherwise stated these are Rel-16 requirements

[TS 38.331, clause 5.3.5.9]

The UE shall:

1>	if the received otherConfig includes the btNameList:

2>	if btNameList is set to setup, include available Bluetooth measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the wlanNameList:

2>	if wlanNameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report, CEF report and SCGFailureInformation;

1>	if the received otherConfig includes the sensorNameList:

2>	if sensorNameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report, CEF report and SCGFailureInformation;

NOTE 2:	The UE is requested to attempt to have valid Bluetooth measurements, WLAN measurements and Sensor measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth or Sensor hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth or Sensor, are up to UE implementation.

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

…

1>	else if the failure is detected due to radio link failure as described in 5.3.10.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to rlf;

2>	set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.3.10.4;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

2>	if an RRCReconfiguration message including the reconfigurationWithSync was received before the connection failure:

3>	if the last RRCReconfiguration message including the reconfigurationWithSync concerned an intra NR handover:

4>	include the nrPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync;

…

1>	if available, set the locationInfo as in 5.3.3.7.

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

1>	set the content of RRCReestablishmentComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.1.6.3.3.1.3	Test description

8.1.6.3.3.1.3.1	Pre-test conditions

System Simulator:

- NR Cell 1, NR Cell 2, Bluetooth beacon 1 (Cell 40) and Bluetooth beacon 2 (Cell 41)

- System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

- Cell 40 and Cell 41 are configured as per 38.508-1 [4] cl 4.4.1.3.

UE:

None

Preamble:

- The UE is in state 3N-A on NR Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.3.3.1.3.2	Test procedure sequence

Tables 8.1.6.3.3.1.3.2-1/2 illustrate the downlink power levels. Row marked "T0" denotes the conditions after the preamble.

Table 8.1.6.3.3.1.3.2-1: Time instances of cell power level and parameter changes (FR1)

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Cell 40 (Bluetooth beacon 1) | Cell 41 (Bluetooth beacon 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -91 |  |  | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
|  | BeaconRSSI | dBm |  |  | -80 | -50 |  |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | “Off” | -79 |  |  | Only Cell 2 is available. (NOTE 1) |
|  | BeaconRSSI | dBm |  |  | -80 | -50 |  |
| NOTE 1: Power level “Off” is defined in TS 38.508-1 [4], Table 6.2.2.1-3. |  |  |  |  |  |  |  |

Table 8.1.6.3.3.1.3.2-2: Time instances of cell power level and parameter changes (FR2)

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Cell 40 (Bluetooth beacon 1) | Cell 41 (Bluetooth beacon 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 | -94 |  |  | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
|  | BeaconRSSI | dBm |  |  | -80 | -50 |  |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | “Off” | -82 |  |  | Only Cell 2 is available. (NOTE 1) |
|  | BeaconRSSI | dBm |  |  | -80 | -50 |  |
| NOTE 1: Power level “Off” is defined in TS 38.508-1 [4], Table 6.2.2.2.2-2. |  |  |  |  |  |  |  |

Table 8.1.6.3.3.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message on Cell 1. | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS changes NR Cell 1 and NR Cell 2 parameters according to the row "T1" in Tables 8.1.6.3.3.1.3.2-1/2. | - | - | - | - |
| 4 | The UE transmits an RRCReestablishmentRequest message on NR Cell 2. | --> | RRCReestablishmentRequest | - | - |
| 5 | The SS transmits an RRCReestablishment message on NR Cell 2. | <-- | RRCReestablishment | - | - |
| 6 | Check: Does the UE transmit an RRCReestablishmentComplete message on NR Cell 2 with rlf-InfoAvailable included? | --> | RRCReestablishmentComplete | 1 | P |
| 7 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB on NR Cell 2. | <-- | RRCReconfiguration | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message on NR Cell 2. | --> | RRCReconfigurationComplete | - | - |
| 9 | The SS transmits a UEInformationRequest message on NR Cell 2. | <-- | UEInformationRequest | - | - |
| 10 | Check: Does the UE transmit a UEInformationResponse message with rlf-Report-r9 with logMeasResultListBT including one entry (Bluetooth beacon 1) measurement result? | --> | UEInformationResponse | 2, 3 | P |

8.1.6.3.3.1.3.3	Specific message contents

Table 8.1.6.3.3.1.3.3-1: RRCReconfiguration (step 1, Table 8.1.6.3.3.1.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1610 SEQUENCE { |  |  |  |
| idc-AssistanceConfig-r16 | Not present |  |  |
| drx-PreferenceConfig-r16 | Not present |  |  |
| maxBW-PreferenceConfig-r16 | Not present |  |  |
| maxCC-PreferenceConfig-r16 | Not present |  |  |
| maxMIMO-LayerPreferenceConfig-r16 | Not present |  |  |
| minSchedulingOffsetPreferenceConfig-r16 | Not present |  |  |
| releasePreferenceConfig-r16 | Not present |  |  |
| referenceTimePreferenceReporting-r16 | Not present |  |  |
| btNameList-r16 CHOICE { |  |  |  |
| Setup BT-NameList-r16 SEQUENCE (SIZE (1..maxBT-Name-r16)) OF BT-Name-r16{ | 1 entry |  |  |
| BT-Name-r16[1] | Set as per Table 4.4.9-1 of 36.508 | OCTET STRING (SIZE (1..248)) Refers to LOCAL NAME defined in Bluetooth specification |  |
| } |  |  |  |
| } |  |  |  |
| wlanNameList-r16 | Not present |  |  |
| sensorNameList-r16 | Not present |  |  |
| obtainCommonLocation-r16 | Not present |  |  |
| sl-AssistanceConfigNR-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.3.1.3.3-2: RRCReestablishmentComplete (step 6, Table 8.1.6.3.3.1.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| iab-NodeIndication-r16 | Not checked |  |  |
| idleMeasAvailable-r16 | Not checked |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition RLF |  |  |
| mobilityHistoryAvail-r16 | Not checked |  |  |
| mobilityState-r16 | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.3.1.3.3-2A: UEInformationRequest (step 9, Table 8.1.6.3.3.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with Condition RLF

Table 8.1.6.3.3.1.3.3-3: UEInformationResponse (step 10, Table 8.1.6.3.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | Not checked |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 | Present | Specified in Table 8.1.6.3.3.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.3.1.3.3-4: rlf-Report-r16 (Table 8.1.6.3.3.1.3.3-3)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE {} | Not checked |  |  |
| c-RNTI-r16 | Not checked |  |  |
| previousPCellId-r16 CHOICE {} | Not checked |  |  |
| failedPCellId-r16 CHOICE {} | Not checked |  |  |
| reconnectCellId-r16 CHOICE {} | Not checked |  |  |
| timeUntilReconnection-r16 | Not checked |  |  |
| reestablishmentCellId-r16 | Not checked |  |  |
| timeConnFailure-r16 | Not checked |  |  |
| timeSinceFailure-r16 | Not checked |  |  |
| connectionFailureType-r16 | rlf |  |  |
| rlf-Cause-r16 | Not checked |  |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| commonLocationInfo-r16 | Not checked |  |  |
| bt-LocationInfo-r16 SEQUENCE (SIZE (1..maxBT-IdReport-r16)) OF LogMeasResultBT-r16 SEQUENCE { | 1 entry |  |  |
| LogMeasResultBT-r16[1] SEQUENCE { |  | Report Bluetooth beacon 1 |  |
| bt-Addr-r16 | Bluetooth public address of Bluetooth beacon 1 | BIT STRING (SIZE (48)) |  |
| rssi-BT-r16 | Any allowed value | INTEGER (-128..127) |  |
| } |  |  |  |
| } |  |  |  |
| wlan-LocationInfo-r16 | Not checked |  |  |
| sensor-LocationInfo-r16 | Not checked |  |  |
| } |  |  |  |
| noSuitableCellFound-r16 | Not checked |  |  |
| ra-InformationCommon-r16 SEQUENCE {} | Not checked |  |  |
| csi-rsRLMConfigBitmap-v1650 SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

[Skip unchanged text]

8.1.6.3.4	Inter-System MDT / Connection Establishment Failure

8.1.6.3.4.1	Inter-System MDT / Connection Establishment Failure / Logging and reporting / Bluetooth measurement collection

8.1.6.3.4.1.1	Test Purpose (TP)

(1)

with { UE after being set up with LoggedMeasurementConfiguration detects connection establishment failure }

ensure that {

when { UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

then { UE sends the RRCSetupComplete message with logMeasAvailable-r16, logMeasAvailableBT-r16 and connEstFailInfoAvailable-r16 set to true }

}

(2)

with { UE has sent the RRCSetupComplete message with connEstFailInfoAvailable-r16 set to true }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq-r16 set to true }

then { UE sends the UEInformationResponse message containing only the measurement of the Bluetooth beacon(s) that included in bt-NameList of LoggedMeasurementConfiguration }

}

8.1.6.3.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clauses 5.5a.1.3, 5.5a.3.2, 5.3.3.4, 5.3.3.7 and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

- store the received bt-NameList, if included, in VarLogMeasConfig;

[TS 38.331, clause 5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig:

5>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

…

2>	when performing the logging:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

1>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableBT in the RRCSetupComplete message;

...

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable in the RRCSetupComplete message;

…

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.1.6.3.4.1.3	Test description

8.1.6.3.4.1.3.1	Pre-test conditions

System Simulator:

- NR Cell 1, Bluetooth beacon 1 (Cell 40), Bluetooth beacon 2 (Cell 41)

-	Cell 40 and Cell 41 are configured as per 38.508-1 [4] cl 4.4.1.3.2

UE:

None

Preamble:

- The UE is in state 3N-A on NR Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.3.4.1.3.2	Test procedure sequence

Table 8.1.6.3.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |
| 3 | Wait 8s to allow UE to activate logging. | - | - | - | - |
| 4-5 | Steps 1 to 2 of the generic radio bearer establishment procedure in TS 38.508-1 [4] table 4.5.4.2-3. | - | - | - | - |
| 6 | The SS waits for 2sec (T300 expiry). | - | - | - | - |
| 7-9 | Steps 1 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 [4] table 4.5.4.2-3. | - | - | - | - |
| 10 | Check: Does the UE transmit an RRCSetupComplete message with logMeasAvailable-r16, logMeasAvailableBT-r16 and connEstFailInfoAvailable-r16 included? | --> | NR RRC: RRCSetupComplete 5GMM: Service Request | 1 | P |
| 11-14 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 [4] table 4.5.4.2-3. | - | - | - | - |
| 15 | The SS sends a UEInformationRequest message to get connEstFailReportReq-r16. | <-- | UEInformationRequest | - | - |
| 16 | Check: Does the UE transmit a UEInformationResponse message with LogMeasResultListBT including one entry (Bluetooth beacon 1) measurement result? | --> | UEInformationResponse | 2 | P |

8.1.6.3.4.1.3.3	Specific message contents

Table 8.1.6.3.4.1.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.3.4.1.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.6.1-5AA with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| bt-NameList-r16 CHOICE { |  |  |  |
| setup | BT-NameList-r16 |  |  |
| } |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms10240 | 10.24 seconds |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.1.3.3-1A: BT-NameList-r16 (Table 8.1.6.3.4.1.3.3-1)

| Derivation path: TS 38.508-1 [4], table 4.6.5-0C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BT-NameList-r16 ::= SEQUENCE (SIZE (1..maxBT-Name-r16)) OF BT-Name-r16 { | 1 entry | Bluetooth beacon 1 |  |
| BT-Name-r16[1] | Set as per Table 4.4.9-1 of 3GPP TS 36.508 | OCTET STRING (SIZE (1..248)) Refers to LOCAL NAME defined in Bluetooth specification |  |
| } |  |  |  |

Table 8.1.6.3.4.1.3.3-2: RRCSetupComplete (step 10, Table 8.1.6.3.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| iab-NodeIndication-r16 | Not checked |  |  |
| idleMeasAvailable-r16 | Not checked |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG and BT and CEF |  |  |
| mobilityHistoryAvail-r16 | Not checked |  |  |
| mobilityState-r16 | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.1.3.3-3: Void

Table 8.1.6.3.4.1.3.3-4: UEInformationResponse (step 16, Table 8.1.6.3.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| cgi-Info SEQUENCE { |  |  |  |
| plmn-Identity-r16 | plmn-Identity within SIB1 broadcasted in NR Cell 1 |  |  |
| cellIdentity-r16 | cellIdentity within SIB1 broadcasted in NR Cell 1 | BIT STRING (SIZE (36)) |  |
| trackingAreaCode-r16 | Not checked | BIT STRING (SIZE (24)) |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 | Not checked |  |  |
| rsIndexResults-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| bt-LocationInfo-r16 SEQUENCE (SIZE (1..maxBT-IdReport-r16)) OF LogMeasResultBT-r16 { | 1 entry |  |  |
| LogMeasResultBT-r16[1] SEQUENCE { |  | Report Bluetooth beacon 1 |  |
| bt-Addr-r16 | Bluetooth public address of Bluetooth beacon 1 | BIT STRING (SIZE (48)) |  |
| rssi-BT-r16 | Any allowed value | INTEGER (-128..127) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Any allowed value | INTEGER (1..8) |  |
| perRAInfoList-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value | INTEGER (0..172800) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.1.6.3.4.2	Inter-System MDT / Connection Establishment Failure / Logging and reporting / WLAN measurement collection

8.1.6.3.4.2.1	Test Purpose (TP)

(1)

with { UE after being set up with LoggedMeasurementConfiguration detects connection establishment failure }

ensure that {

when { UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

then { UE sends the RRCSetupComplete message with logMeasAvailable-r16, logMeasAvailableWLAN-r16 and connEstFailInfoAvailable-r16 set to true }

}

(2)

with { UE has sent the RRCSetupComplete message with connEstFailInfoAvailable-r16 set to true }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq-r16 set to true }

then { UE sends the UEInformationResponse message containing only the measurement of the WLAN AP(s) that included in wlan-NameList-r16 of LoggedMeasurementConfiguration }

}

8.1.6.3.4.2.2	Conformance requirements

Same as test case 8.1.6.3.4.1.

8.1.6.3.4.2.3	Test description

8.1.6.3.4.2.3.1	Pre-test conditions

Same as test case 8.1.6.3.4.1 with the following differences:

System Simulator:

-	NR Cell 1, WLAN AP 1 (Cell 27) and WLAN AP 2 (Cell 27b)

- Cell 27 and Cell 27b are configured as per TS 38.508-1 [4] cl 4.4.1.3 with the OFFLOAD condition

8.1.6.3.4.2.3.2	Test procedure sequence

Same as test case 8.1.6.3.4.1 with the following differences: logMeasAvailableBT-r16 is replaced by logMeasAvailableWLAN-r16.

Table 8.1.6.3.4.2.3.2-1: Void

8.1.6.3.4.2.3.3	Specific message contents

Table 8.1.6.3.4.2.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.3.4.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.6.1-5AA with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| wlan-NameList-r16 CHOICE { |  |  |  |
| setup | WLAN-NameList-r16 |  |  |
| } |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms10240 | 10.24 seconds |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.2.3.3-1A: WLAN-NameList-r16 (Table 8.1.6.3.4.2.3.3-1)

| Derivation path: TS 38.508-1 [4], table 4.6.5-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-NameList-r16 ::= SEQUENCE (SIZE (1..maxWLAN-Name-r16)) OF WLAN-Name-r16 { | 1 entry | WLAN AP1 |  |
| WLAN-Name-r16[1] | Set as per Table 4.4.8-1 of TS 36.508 | OCTET STRING (SIZE (1..32)) Refers to Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| } |  |  |  |

Table 8.1.6.3.4.2.3.3-2: RRCSetupComplete (step 10, Table 8.1.6.3.4.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| iab-NodeIndication-r16 | Not checked |  |  |
| idleMeasAvailable-r16 | Not checked |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG and WLAN and CEF |  |  |
| mobilityHistoryAvail-r16 | Not checked |  |  |
| mobilityState-r16 | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.2.3.3-3: Void

Table 8.1.6.3.4.2.3.3-4: UEInformationResponse (step 16, Table 8.1.6.3.4.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| cgi-Info SEQUENCE { |  |  |  |
| plmn-Identity-r16 | plmn-Identity within SIB1 broadcasted in NR Cell 1 |  |  |
| cellIdentity-r16 | cellIdentity within SIB1 broadcasted in NR Cell 1 | BIT STRING (SIZE (36)) |  |
| trackingAreaCode-r16 | Not checked | BIT STRING (SIZE (24)) |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 | Not checked |  |  |
| rsIndexResults-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| wlan-LocationInfo-r16 SEQUENCE (SIZE (1..maxWLAN-Id-Report-r16)) OF LogMeasResultWLAN-r16 { | 1 entry |  |  |
| LogMeasResultWLAN-r16[1] SEQUENCE { |  | Report WLAN AP 1 |  |
| wlan-Identifiers-r16 SEQUENCE { |  |  |  |
| ssid-r12 | ssid for WLAN AP 1 |  |  |
| bssid-r12 | Not present |  |  |
| hessid-r12 | Not present |  |  |
| } |  |  |  |
| rssiWLAN-r16 | Any allowed value |  |  |
| rtt-WLAN-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Any allowed value | INTEGER (1..8) |  |
| perRAInfoList-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value | INTEGER (0..172800) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.1.6.3.4.3	Inter-System MDT / Connection Establishment Failure / Logging and reporting / Sensor measurement collection

8.1.6.3.4.3.1	Test Purpose (TP)

(1)

with { UE after being set up with LoggedMeasurementConfiguration detects connection establishment failure }

ensure that {

when { UE has sensor logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

then { UE sends the RRCSetupComplete message with logMeasAvailable-r16 and connEstFailInfoAvailable-r16 set to true }

}

(2)

with { UE has sent the RRCSetupComplete message with connEstFailInfoAvailable-r16 set to true }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq-r16 set to true }

then { UE sends the UEInformationResponse message containing only the measurement of the sensor(s) that included in sensor-NameList-r16 of LoggedMeasurementConfiguration }

}

8.1.6.3.4.3.2	Conformance requirements

Same as test case 8.1.6.3.4.1.

8.1.6.3.4.3.3	Test description

8.1.6.3.4.3.3.1	Pre-test conditions

Same as test case 8.1.6.3.4.1 with the following differences:

System Simulator:

-	NR Cell 1

UE:

-	UE supports at least one of pc_barometer_r16, pc_orientation_r16 and pc_speed_r16.

8.1.6.3.4.3.3.2	Test procedure sequence

Same as test case 8.1.6.3.4.1 with the following differences: logMeasAvailableBT-r16 is not mentioned.

Table 8.1.6.3.4.3.3.2-1: Void

8.1.6.3.4.3.3.3	Specific message contents

Table 8.1.6.3.4.3.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.3.4.3.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.6.1-5AA with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| sensor-NameList-r16 CHOICE { |  |  |  |
| setup | Sensor-NameList-r16 |  |  |
| } |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms10240 | 10.24 second |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.3.3.3-1A: Sensor-NameList-r16 (Table 8.1.6.3.4.3.3.3-1)

| Derivation path: TS 38.508-1 [4], table 4.6.5-13 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Sensor-NameList-r16 ::= SEQUENCE { |  |  |  |  |
| measUncomBarPre-r16 | true |  | pc_barometer_r16 |
|  | Not present |  |  |
| measUeSpeed | true |  | pc_speed_r16 |
|  | Not present |  |  |
| measUeOrientation | true |  | pc_orientation_r16 |
|  | Not present |  |  |
| } |  |  |  |  |

Table 8.1.6.3.4.3.3.3-2: RRCSetupComplete (step 10, Table 8.1.6.3.4.3.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| iab-NodeIndication-r16 | Not checked |  |  |
| idleMeasAvailable-r16 | Not checked |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG and CEF |  |  |
| mobilityHistoryAvail-r16 | Not checked |  |  |
| mobilityState-r16 | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.3.3.3-3: Void

Table 8.1.6.3.4.3.3.3-4: UEInformationResponse (step 16, Table 8.1.6.3.4.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| cgi-Info SEQUENCE { |  |  |  |
| plmn-Identity-r16 | plmn-Identity within SIB1 broadcasted in NR Cell 1 |  |  |
| cellIdentity-r16 | cellIdentity within SIB1 broadcasted in NR Cell 1 | BIT STRING (SIZE (36)) |  |
| trackingAreaCode-r16 | Not checked | BIT STRING (SIZE (24)) |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 | Not checked |  |  |
| rsIndexResults-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| sensor-LocationInfo-r16 SEQUENCE { |  | Report sensor measurement information |  |
| sensor-MeasurementInformation-r16 | Not present |  |  |
|  | OCTET STRING including Sensor-MeasurementInformation-r13 according to TS 37.355 cl 6.5.5.2. |  | pc_barometer_r16 |
| sensor-MotionInformation-r16 | Not present |  |  |
|  | OCTET STRING including Sensor-MotionInformation-r15 according to TS 37.355 cl 6.5.5.2. |  | pc_orientation_r16 OR pc_speed_r16 |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Any allowed value | INTEGER (1..8) |  |
| perRAInfoList-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value | INTEGER (0..172800) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]