3GPP TSG-RAN5 Meeting #96-e		       R5-224141
Electronic Meeting, 15th August – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1392 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test frequencies for LTE Band 103 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Puloli |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_upper_700MHz_A-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of test frequencies for LTE Band 103 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added band 103 to NB-IoT standalone operations - Added section 8.1.3.1.1.89 to 8.1.3.1.1.102 as placeholder - Added 8.1.3.1.1.103 to specify the band 103 test frequencies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test frequencies specified for LTE band 103 testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

#### 8.1.3.1	NB-IoT Test frequencies

NB-IoT is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 25, 26, 28, 31, 41, 42, 43, 65, 66, 70, 71, 72, 73, 74, 85, 87, 88 and 103 which are defined in Table 5.5-1 in [21]. NB-IoT system operates in HD-FDD and TDD duplex mode or in TDD mode.

The test frequencies are based on the E-UTRA frequency bands defined in the core specifications.

The raster spacing is 100 kHz.

E-UTRA/FDD is designed to operate in paired bands of 3GPP TS 36.101 [27]. The reference test frequencies for the RF and Signalling test environment for each of the operating bands are defined in sub clause 8.1.3.1.1 and 8.1.3.1.2.

E-UTRA/TDD is designed to operate in unpaired bands of 3GPP TS 36.101 [27]. The reference test frequencies for the RF and Signalling test environment for each of the operating bands are defined in sub clause 8.1.3.1.2.

USA & Canada emission requirements for specific operating bands are indicated by network signaling value NS_04 on indicated bands in clause 8.1.3.1.1. The normative reference for this requirement is TS 36.101 [27] clause 5.5F.

##### 8.1.3.1.1	NB-IoT FDD Mode Test frequencies

8.1.3.1.1.1	NB-IoT FDD reference test frequencies for operating band 1

Table 8.1.3.1.1.1-1: NB-IoT standalone Test frequencies for operating band 1

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 18001 | 0 | 1920.1 | 1 | -0.5 | 2110.1 |
| Mid Range | 18300 | 0 | 1950.0 | 300 | -0.5 | 2140.0 |
| High Range | 18599 | 0 | 1979.9 | 599 | -0.5 | 2169.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.1-2: NB-IoT in-band Test frequencies for operating band 1

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 18007 | 0 | 1920.7000 | 7 | -2 | 2110.6925 |
| Low Range2 | 18060 | -2 | 1925.9900 | 60 | -1 | 2115.9975 |
| Low Range3 | 18069 | -2 | 1926.8900 | 69 | -1 | 2116.8975 |
| Mid Range1 | 18291 | 0 | 1949.1000 | 291 | -2 | 2139.0925 |
| Mid Range2 | 18310 | -2 | 1950.9900 | 310 | -1 | 2140.9975 |
| Mid Range3 | 18319 | -2 | 1951.8900 | 319 | -1 | 2141.8975 |
| High Range1 | 18593 | 0 | 1979.3000 | 593 | 1 | 2169.3075 |
| High Range2 | 18560 | -2 | 1975.9900 | 560 | -1 | 2165.9975 |
| High Range3 | 18569 | -2 | 1976.8900 | 569 | -1 | 2166.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.1-3: NB-IoT guard-band Test frequencies for operating band 1

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 18001 | 0 | 1920.1000 | 1 | 1 | 2110.1075 |
| Mid Range | 5 | 18276 | 0 | 1947.6000 | 276 | 1 | 2137.6075 |
|  | 10 | 18346 | 0 | 1954.6000 | 346 | 1 | 2144.6075 |
| High Range | Any | 18599 | 0 | 1979.9000 | 599 | -2 | 2169.8925 |
| NOTE 1: Void NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.2	NB-IoT FDD reference test frequencies for operating band 2

Table 8.1.3.1.1.2-1: NB-IoT standalone Test frequencies for operating band 2

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 18601 | 0 | 1850.1 | 601 | -0.5 | 1930.1 |
| Mid Range | 18900 | 0 | 1880.0 | 900 | -0.5 | 1960.0 |
| High Range | 19199 | 0 | 1909.9 | 1199 | -0.5 | 1989.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.2-1a: NB-IoT standalone Test frequencies for operating band 2 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 18602 | 0 | 1850.2 | 602 | -0.5 | 1930.2 |
| Mid Range | 18900 | 0 | 1880.0 | 900 | -0.5 | 1960.0 |
| High Range | 19198 | 0 | 1909.8 | 1198 | -0.5 | 1989.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.2-2: NB-IoT in-band Test frequencies for operating band 2

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 18606 | 0 | 1850.6000 | 606 | -2 | 1930.5925 |
| Low Range2 | 18660 | -2 | 1855.9900 | 660 | -1 | 1935.9975 |
| Low Range3 | 18669 | -2 | 1856.8900 | 669 | -1 | 1936.8975 |
| Mid Range1 | 18891 | 0 | 1879.1000 | 891 | -2 | 1959.0925 |
| Mid Range2 | 18910 | -2 | 1880.9900 | 910 | -1 | 1960.9975 |
| Mid Range3 | 18919 | -2 | 1881.8900 | 919 | -1 | 1961.8975 |
| High Range1 | 19194 | 0 | 1909.4000 | 1194 | 1 | 1989.4075 |
| High Range2 | 19160 | -2 | 1905.9900 | 1160 | -1 | 1985.9975 |
| High Range3 | 19169 | -2 | 1906.8900 | 1169 | -1 | 1986.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.2-2a: NB-IoT in-band Test frequencies for operating band 2 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 18606 | 0 | 1850.6000 | 606 | -2 | 1930.5925 |
| Low Range2 | 18660 | -2 | 1855.9900 | 660 | -1 | 1935.9975 |
| Low Range3 | 18669 | -2 | 1856.8900 | 669 | -1 | 1936.8975 |
| Mid Range1 | 18891 | 0 | 1879.1000 | 891 | -2 | 1959.0925 |
| Mid Range2 | 18910 | -2 | 1880.9900 | 910 | -1 | 1960.9975 |
| Mid Range3 | 18919 | -2 | 1881.8900 | 919 | -1 | 1961.8975 |
| High Range1 | 19194 | 0 | 1909.4000 | 1194 | 1 | 1989.4075 |
| High Range2 | 19160 | -2 | 1905.9900 | 1160 | -1 | 1985.9975 |
| High Range3 | 19169 | -2 | 1906.8900 | 1169 | -1 | 1986.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.2-3: NB-IoT guard-band Test frequencies for operating band 2

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 18601 | 0 | 1850.1000 | 601 | 1 | 1930.1075 |
| Mid Range | 5 | 18876 | 0 | 1877.6000 | 876 | 1 | 1957.6075 |
|  | 10 | 18946 | 0 | 1884.6000 | 946 | 1 | 1964.6075 |
| High Range | Any | 19199 | 0 | 1909.9000 | 1199 | -2 | 1989.8925 |
| NOTE 1: Void NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.2-3a: NB-IoT guard-band Test frequencies for operating band 2 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 18602 | 0 | 1850.2000 | 602 | 1 | 1930.2075 |
| Mid Range | 5 | 18876 | 0 | 1877.6000 | 876 | 1 | 1957.6075 |
|  | 10 | 18946 | 0 | 1884.6000 | 946 | 1 | 1964.6075 |
| High Range | Any | 19198 | 0 | 1909.8000 | 1198 | -2 | 1989.7925 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.3	NB-IoT FDD reference test frequencies for operating band 3

Table 8.1.3.1.1.3-1: NB-IoT standalone Test frequencies for operating band 3

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 19201 | 0 | 1710.1 | 1201 | -0.5 | 1805.1 |
| Mid Range | 19575 | 0 | 1747.5 | 1575 | -0.5 | 1842.5 |
| High Range | 19949 | 0 | 1784.9 | 1949 | -0.5 | 1879.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.3-2: NB-IoT in-band Test frequencies for operating band 3

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 19206 | 0 | 1710.6000 | 1206 | -2 | 1805.5925 |
| Low Range2 | 19260 | -2 | 1715.9900 | 1260 | -1 | 1810.9975 |
| Low Range3 | 19269 | -2 | 1716.8900 | 1269 | -1 | 1811.8975 |
| Mid Range1 | 19566 | 0 | 1746.6000 | 1566 | -2 | 1841.5925 |
| Mid Range2 | 19585 | -2 | 1748.4900 | 1585 | -1 | 1843.4975 |
| Mid Range3 | 19594 | -2 | 1749.3900 | 1594 | -1 | 1844.3975 |
| High Range1 | 19944 | 0 | 1784.4000 | 1944 | 1 | 1879.4075 |
| High Range2 | 19910 | -2 | 1780.9900 | 1910 | -1 | 1875.9975 |
| High Range3 | 19919 | -2 | 1781.8900 | 1919 | -1 | 1876.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.3-3: NB-IoT guard-band Test frequencies for operating band 3

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 19201 | 0 | 1710.1000 | 1201 | 1 | 1805.1075 |
| Mid Range | 5 | 19551 | 0 | 1745.1000 | 1551 | 1 | 1840.1075 |
|  | 10 | 19621 | 0 | 1752.1000 | 1621 | 1 | 1847.1075 |
| High Range | Any | 19949 | 0 | 1784.9000 | 1949 | -2 | 1879.8925 |
| NOTE 1: Void NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.4	NB-IoT FDD reference test frequencies for operating band 4

Table 8.1.3.1.1.4-1: NB-IoT standalone Test frequencies for operating band 4

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 19951 | 0 | 1710.1 | 1951 | -0.5 | 2110.1 |
| Mid Range | 20175 | 0 | 1732.5 | 2175 | -0.5 | 2132.5 |
| High Range | 20399 | 0 | 1754.9 | 2399 | -0.5 | 2154.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.4-1a: NB-IoT standalone Test frequencies for operating band 4 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 19952 | 0 | 1710.2 | 1952 | -0.5 | 2110.2 |
| Mid Range | 20175 | 0 | 1732.5 | 2175 | -0.5 | 2132.5 |
| High Range | 20398 | 0 | 1754.8 | 2398 | -0.5 | 2154.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.4-2: NB-IoT in-band Test frequencies for operating band 4

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 19957 | 0 | 1710.7000 | 1957 | -2 | 2110.6925 |
| Low Range2 | 20010 | -2 | 1715.9900 | 2010 | -1 | 2115.9975 |
| Low Range3 | 20019 | -2 | 1716.8900 | 2019 | -1 | 2116.8975 |
| Mid Range1 | 20166 | 0 | 1731.6000 | 2166 | -2 | 2131.5925 |
| Mid Range2 | 20185 | -2 | 1733.4900 | 2185 | -1 | 2133.4975 |
| Mid Range3 | 20194 | -2 | 1734.3900 | 2194 | -1 | 2134.3975 |
| High Range1 | 20393 | 0 | 1754.3000 | 2393 | 1 | 2154.3075 |
| High Range2 | 20360 | -2 | 1750.9900 | 2360 | -1 | 2150.9975 |
| High Range3 | 20369 | -2 | 1751.8900 | 2369 | -1 | 2151.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.4-2a: NB-IoT in-band Test frequencies for operating band 4 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 19957 | 0 | 1710.7000 | 1957 | -2 | 2110.6925 |
| Low Range2 | 20010 | -2 | 1715.9900 | 2010 | -1 | 2115.9975 |
| Low Range3 | 20019 | -2 | 1716.8900 | 2019 | -1 | 2116.8975 |
| Mid Range1 | 20166 | 0 | 1731.6000 | 2166 | -2 | 2131.5925 |
| Mid Range2 | 20185 | -2 | 1733.4900 | 2185 | -1 | 2133.4975 |
| Mid Range3 | 20194 | -2 | 1734.3900 | 2194 | -1 | 2134.3975 |
| High Range1 | 20393 | 0 | 1754.3000 | 2393 | 1 | 2154.3075 |
| High Range2 | 20360 | -2 | 1750.9900 | 2360 | -1 | 2150.9975 |
| High Range3 | 20369 | -2 | 1751.8900 | 2369 | -1 | 2151.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.4-3: NB-IoT guard-band Test frequencies for operating band 4

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 19951 | 0 | 1710.1000 | 1951 | 1 | 2110.1075 |
| Mid Range | 5 | 20151 | 0 | 1730.1000 | 2151 | 1 | 2130.1075 |
|  | 10 | 20221 | 0 | 1737.1000 | 2221 | 1 | 2137.1075 |
| High Range | Any | 20399 | 0 | 1754.9000 | 2399 | -2 | 2154.8925 |
| NOTE 1: Void NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.4-3a: NB-IoT guard-band Test frequencies for operating band 4 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 19952 | 0 | 1710.2000 | 1952 | 1 | 2110.2075 |
| Mid Range | 5 | 20151 | 0 | 1730.1000 | 2151 | 1 | 2130.1075 |
|  | 10 | 20221 | 0 | 1737.1000 | 2221 | 1 | 2137.1075 |
| High Range | Any | 20398 | 0 | 1754.8000 | 2398 | -2 | 2154.7925 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.5	NB-IoT FDD reference test frequencies for operating band 5

Table 8.1.3.1.1.5-1: NB-IoT standalone Test frequencies for operating band 5

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 20401 | 0 | 824.1 | 2401 | -0.5 | 869.1 |
| Mid Range | 20525 | 0 | 836.5 | 2525 | -0.5 | 881.5 |
| High Range | 20649 | 0 | 848.9 | 2649 | -0.5 | 893.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.5-1a: NB-IoT standalone Test frequencies for operating band 5 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 20402 | 0 | 824.2 | 2402 | -0.5 | 869.2 |
| Mid Range | 20525 | 0 | 836.5 | 2525 | -0.5 | 881.5 |
| High Range | 20648 | 0 | 848.8 | 2648 | -0.5 | 893.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.5-2: NB-IoT in-band Test frequencies for operating band 5

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 20406 | 0 | 824.6000 | 2406 | -2 | 869.5925 |
| Low Range2 | 20460 | -2 | 829.9900 | 2460 | -1 | 874.9975 |
| Low Range3 | 20469 | -2 | 830.8900 | 2469 | -1 | 875.8975 |
| Mid Range1 | 20516 | 0 | 835.6000 | 2516 | -2 | 880.5925 |
| Mid Range2 | 20535 | -2 | 837.4900 | 2535 | -1 | 882.4975 |
| Mid Range3 | 20544 | -2 | 838.3900 | 2544 | -1 | 883.3975 |
| High Range1 | 20644 | 0 | 848.4000 | 2644 | 1 | 893.4075 |
| High Range2 | 20610 | -2 | 844.9900 | 2610 | -1 | 889.9975 |
| High Range3 | 20619 | -2 | 845.8900 | 2619 | -1 | 890.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.5-2a: NB-IoT in-band Test frequencies for operating band 5 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 20406 | 0 | 824.6000 | 2406 | -2 | 869.5925 |
| Low Range2 | 20460 | -2 | 829.9900 | 2460 | -1 | 874.9975 |
| Low Range3 | 20469 | -2 | 830.8900 | 2469 | -1 | 875.8975 |
| Mid Range1 | 20516 | 0 | 835.6000 | 2516 | -2 | 880.5925 |
| Mid Range2 | 20535 | -2 | 837.4900 | 2535 | -1 | 882.4975 |
| Mid Range3 | 20544 | -2 | 838.3900 | 2544 | -1 | 883.3975 |
| High Range1 | 20644 | 0 | 848.4000 | 2644 | 1 | 893.4075 |
| High Range2 | 20610 | -2 | 844.9900 | 2610 | -1 | 889.9975 |
| High Range3 | 20619 | -2 | 845.8900 | 2619 | -1 | 890.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.5-3: NB-IoT guard-band Test frequencies for operating band 5

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 20401 | 0 | 824.1000 | 2401 | 1 | 869.1075 |
| Mid Range | 5 | 20501 | 0 | 834.1000 | 2501 | 1 | 879.1075 |
|  | 10 | 20571 | 0 | 841.1000 | 2571 | 1 | 886.1075 |
| High Range | Any | 20649 | 0 | 848.9000 | 2649 | -2 | 893.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.5-3a: NB-IoT guard-band Test frequencies for operating band 5 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 20402 | 0 | 824.2000 | 2402 | 1 | 869.2075 |
| Mid Range | 5 | 20501 | 0 | 834.1000 | 2501 | 1 | 879.1075 |
|  | 10 | 20571 | 0 | 841.1000 | 2571 | 1 | 886.1075 |
| High Range | Any | 20648 | 0 | 848.8000 | 2648 | -2 | 893.7925 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.6	Void

8.1.3.1.1.7	NB-IoT FDD reference test frequencies for operating band 7

Table 8.1.3.1.1.7-1: NB-IoT standalone Test frequencies for operating band 7

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 20751 | 0 | 2500.1 | 2751 | -0.5 | 2620.1 |
| Mid Range | 21100 | 0 | 2535.0 | 3100 | -0.5 | 2655.0 |
| High Range | 21449 | 0 | 2569.9 | 3449 | -0.5 | 2689.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.7-2: NB-IoT in-band Test frequencies for operating band 7

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 20757 | 0 | 2500.7000 | 2757 | -2 | 2620.6925 |
| Low Range2 | 20810 | -2 | 2505.9900 | 2810 | -1 | 2625.9975 |
| Low Range3 | 20819 | -2 | 2506.8900 | 2819 | -1 | 2626.8975 |
| Mid Range | 21091 | 0 | 2534.1000 | 3091 | -2 | 2654.0925 |
| Mid Range2 | 21110 | -2 | 2535.9900 | 3110 | -1 | 2655.9975 |
| Mid Range3 | 21119 | -2 | 2536.8900 | 3119 | -1 | 2656.8975 |
| High Range | 21443 | 0 | 2569.3000 | 3443 | 1 | 2689.3075 |
| High Range2 | 21410 | -2 | 2565.9900 | 3410 | -1 | 2685.9975 |
| High Range3 | 21419 | -2 | 2566.8900 | 3419 | -1 | 2686.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.7-3: NB-IoT guard-band Test frequencies for operating band 7

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 20751 | 0 | 2500.1000 | 2751 | 1 | 2620.1075 |
| Mid Range | 5 | 21076 | 0 | 2532.6000 | 3076 | 1 | 2652.6075 |
|  | 10 | 21146 | 0 | 2539.6000 | 3146 | 1 | 2659.6075 |
| High Range | Any | 21449 | 0 | 2569.9000 | 3449 | -2 | 2689.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.8	NB-IoT FDD reference test frequencies for operating band 8

Table 8.1.3.1.1.8-1: NB-IoT standalone Test frequencies for operating band 8

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 21451 | 0 | 880.1 | 3451 | -0.5 | 925.1 |
| Mid Range | 21625 | 0 | 897.5 | 3625 | -0.5 | 942.5 |
| High Range | 21799 | 0 | 914.9 | 3799 | -0.5 | 959.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.8-2: NB-IoT in-band Test frequencies for operating band 8

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 21456 | 0 | 880.6000 | 3456 | -2 | 925.5925 |
| Low Range2 | 21510 | -2 | 885.9900 | 3510 | -1 | 930.9975 |
| Low Range3 | 21519 | -2 | 886.8900 | 3519 | -1 | 931.8975 |
| Mid Range | 21616 | 0 | 896.6000 | 3616 | -2 | 941.5925 |
| Mid Range2 | 21635 | -2 | 898.4900 | 3635 | -1 | 943.4975 |
| Mid Range3 | 21644 | -2 | 899.3900 | 3644 | -1 | 944.3975 |
| High Range | 21794 | 0 | 914.4000 | 3794 | 1 | 959.4075 |
| High Range2 | 21760 | -2 | 910.9900 | 3760 | -1 | 955.9975 |
| High Range3 | 21769 | -2 | 911.8900 | 3769 | -1 | 956.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.8-3: NB-IoT guard-band Test frequencies for operating band 8

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 21451 | 0 | 880.1000 | 3451 | 1 | 925.1075 |
| Mid Range | 5 | 21601 | 0 | 895.1000 | 3601 | 1 | 940.1075 |
|  | 10 | 21671 | 0 | 902.1000 | 3671 | 1 | 947.1075 |
| High Range | Any | 21799 | 0 | 914.9000 | 3799 | -2 | 959.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.9 to 8.1.3.1.1.10	Void

8.1.3.1.1.11	NB-IoT FDD reference test frequencies for operating band 11

Table 8.1.3.1.1.11-1: NB-IoT standalone Test frequencies for operating band 11

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 22751 | 0 | 1428.0 | 4751 | -0.5 | 1476.0 |
| Mid Range | 22850 | 0 | 1437.9 | 4850 | -0.5 | 1485.9 |
| High Range | 22949 | 0 | 1447.8 | 4949 | -0.5 | 1495.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.11-2: NB-IoT in-band Test frequencies for operating band 11

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 22757 | 0 | 1428.6000 | 4757 | -2 | 1476.5925 |
| Low Range2 | 22810 | -2 | 1433.8900 | 4810 | -1 | 1481.8975 |
| Low Range3 | 22819 | -2 | 1434.7900 | 4819 | -1 | 1482.7975 |
| Mid Range1 | 22841 | 0 | 1437.0000 | 4841 | -2 | 1484.9925 |
| Mid Range2 | 22860 | -2 | 1438.8900 | 4860 | -1 | 1486.8975 |
| Mid Range3 | 22869 | -2 | 1439.7900 | 4869 | -1 | 1487.7975 |
| High Range1 | 22943 | 0 | 1447.2000 | 4943 | 1 | 1495.2075 |
| High Range2 | 22910 | -2 | 1443.8900 | 4910 | -1 | 1491.8975 |
| High Range3 | 22919 | -2 | 1444.7900 | 4919 | -1 | 1492.7975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.11-3: NB-IoT guard-band Test frequencies for operating band 11

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 22751 | 0 | 1428.0000 | 4751 | 1 | 1476.0075 |
| Mid Range | 22826 | 0 | 1435.5000 | 4826 | 1 | 1483.5075 |
| High Range | 22949 | 0 | 1447.8000 | 4949 | -2 | 1495.7925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.12	NB-IoT FDD reference test frequencies for operating band 12

Table 8.1.3.1.1.12-1: NB-IoT standalone Test frequencies for operating band 12

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23011 | 0 | 699.1 | 5011 | -0.5 | 729.1 |
| Mid Range | 23095 | 0 | 707.5 | 5095 | -0.5 | 737.5 |
| High Range | 23179 | 0 | 715.9 | 5179 | -0.5 | 745.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.12-1a: NB-IoT standalone Test frequencies for operating band 12 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23011 | 0 | 699.1 | 5011 | -0.5 | 729.1 |
| Mid Range | 23095 | 0 | 707.5 | 5095 | -0.5 | 737.5 |
| High Range | 23178 | 0 | 715.8 | 5178 | -0.5 | 745.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.12-2: NB-IoT in-band Test frequencies for operating band 12

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23016 | 0 | 699.6000 | 5016 | -2 | 729.5925 |
| Low Range2 | 23070 | -2 | 704.9900 | 5070 | -1 | 734.9975 |
| Low Range3 | 23079 | -2 | 705.8900 | 5079 | -1 | 735.8975 |
| Mid Range1 | 23086 | 0 | 706.6000 | 5086 | -2 | 736.5925 |
| Mid Range2 | 23105 | -2 | 708.4900 | 5105 | -1 | 738.4975 |
| Mid Range3 | 23114 | -2 | 709.3900 | 5114 | -1 | 739.3975 |
| High Range1 | 23174 | 0 | 715.4000 | 5174 | 1 | 745.4075 |
| High Range2 | 23140 | -2 | 711.9900 | 5140 | -1 | 741.9975 |
| High Range3 | 23149 | -2 | 712.8900 | 5149 | -1 | 742.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.12-2: NB-IoT in-band Test frequencies for operating band 12 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23016 | 0 | 699.6000 | 5016 | -2 | 729.5925 |
| Low Range2 | 23070 | -2 | 704.9900 | 5070 | -1 | 734.9975 |
| Low Range3 | 23079 | -2 | 705.8900 | 5079 | -1 | 735.8975 |
| Mid Range1 | 23086 | 0 | 706.6000 | 5086 | -2 | 736.5925 |
| Mid Range2 | 23105 | -2 | 708.4900 | 5105 | -1 | 738.4975 |
| Mid Range3 | 23114 | -2 | 709.3900 | 5114 | -1 | 739.3975 |
| High Range1 | 23174 | 0 | 715.4000 | 5174 | 1 | 745.4075 |
| High Range2 | 23140 | -2 | 711.9900 | 5140 | -1 | 741.9975 |
| High Range3 | 23149 | -2 | 712.8900 | 5149 | -1 | 742.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.12-3: NB-IoT guard-band Test frequencies for operating band 12

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 23011 | 0 | 699.1000 | 5011 | 1 | 729.1075 |
| Mid Range | 5 | 23071 | 0 | 705.1000 | 5071 | 1 | 735.1075 |
|  | 10 | 23141 | 0 | 712.1000 | 5141 | 1 | 742.1075 |
| High Range | Any | 23179 | 0 | 715.9000 | 5179 | -2 | 745.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.12-3a: NB-IoT guard-band Test frequencies for operating band 12 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 23011 | 0 | 699.1000 | 5011 | 1 | 729.1075 |
| Mid Range | 5 | 23071 | 0 | 705.1000 | 5071 | 1 | 735.1075 |
|  | 10 | 23141 | 0 | 712.1000 | 5141 | 1 | 742.1075 |
| High Range | Any | 23178 | 0 | 715.8000 | 5178 | -2 | 745.7925 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.13	NB-IoT FDD reference test frequencies for operating band 13

Table 8.1.3.1.1.13-1: NB-IoT standalone Test frequencies for operating band 13

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23181 | 0 | 777.1 | 5181 | -0.5 | 746.1 |
| Mid Range | 23230 | 0 | 782.0 | 5230 | -0.5 | 751.0 |
| High Range | 23279 | 0 | 786.9 | 5279 | -0.5 | 755.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.13-1a: NB-IoT standalone Test frequencies for operating band 13 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23181 | 0 | 777.1 | 5181 | -0.5 | 746.1 |
| Mid Range | 23230 | 0 | 782.0 | 5230 | -0.5 | 751.0 |
| High Range | 23278 | 0 | 786.8 | 5278 | -0.5 | 755.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.13-2: NB-IoT in-band Test frequencies for operating band 13

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23187 | 0 | 777.7000 | 5187 | -2 | 746.6925 |
| Low Range2 | 23240 | -2 | 782.9900 | 5240 | -1 | 751.9975 |
| Low Range3 | 23249 | -2 | 783.8900 | 5249 | -1 | 752.8975 |
| Mid Range1 | 23221 | 0 | 781.1000 | 5221 | -2 | 750.0925 |
| Mid Range2 | 23240 | -2 | 782.9900 | 5240 | -1 | 751.9975 |
| Mid Range3 | 23249 | -2 | 783.8900 | 5249 | -1 | 752.8975 |
| High Range1 | 23273 | 0 | 786.3000 | 5273 | 1 | 755.3075 |
| High Range2 | 23240 | -2 | 782.9900 | 5240 | -1 | 751.9975 |
| High Range3 | 23249 | -2 | 783.8900 | 5249 | -1 | 752.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.13-2a: NB-IoT in-band Test frequencies for operating band 13 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23187 | 0 | 777.7000 | 5187 | -2 | 746.6925 |
| Low Range2 | 23240 | -2 | 782.9900 | 5240 | -1 | 751.9975 |
| Low Range3 | 23249 | -2 | 783.8900 | 5249 | -1 | 752.8975 |
| Mid Range1 | 23221 | 0 | 781.1000 | 5221 | -2 | 750.0925 |
| Mid Range2 | 23240 | -2 | 782.9900 | 5240 | -1 | 751.9975 |
| Mid Range3 | 23249 | -2 | 783.8900 | 5249 | -1 | 752.8975 |
| High Range1 | 23273 | 0 | 786.3000 | 5273 | 1 | 755.3075 |
| High Range2 | 23240 | -2 | 782.9900 | 5240 | -1 | 751.9975 |
| High Range3 | 23249 | -2 | 783.8900 | 5249 | -1 | 752.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.13-3: NB-IoT guard-band Test frequencies for operating band 13

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 23181 | 3 | 777.1150 | 5181 | 1 | 746.1075 |
| Mid Range | 5 | 23206 | 3 | 779.6150 | 5206 | 1 | 748.6075 |
|  | 10 | 23276 | 3 | 786.6150 | 5276 | 1 | 755.6075 |
| High Range | Any | 23279 | -3 | 786.8850 | 5279 | -2 | 755.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.13-3a: NB-IoT guard-band Test frequencies for operating band 13 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 23181 | 3 | 777.1150 | 5181 | 1 | 746.1075 |
| Mid Range | 5 | 23206 | 3 | 779.6150 | 5206 | 1 | 748.6075 |
|  | 10 | 23276 | 3 | 786.6150 | 5276 | 1 | 755.6075 |
| High Range | Any | 23278 | -3 | 786.7850 | 5278 | -2 | 755.7925 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.14	NB-IoT FDD reference test frequencies for operating band 14

Table 8.1.3.1.1.14-1: NB-IoT standalone Test frequencies for operating band 14

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23281 | 0 | 788.1 | 5281 | -0.5 | 758.1 |
| Mid Range | 23330 | 0 | 793.0 | 5330 | -0.5 | 763.0 |
| High Range | 23379 | 0 | 797.9 | 5379 | -0.5 | 767.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.14-2: NB-IoT in-band Test frequencies for operating band 14

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23287 | 0 | 788.7000 | 5287 | -2 | 758.6925 |
| Low Range2 | 23340 | -2 | 793.9900 | 5340 | -1 | 763.9975 |
| Low Range3 | 23349 | -2 | 794.8900 | 5349 | -1 | 764.8975 |
| Mid Range1 | 23321 | 0 | 792.1000 | 5321 | -2 | 762.0925 |
| Mid Range2 | 23340 | -2 | 793.9900 | 5340 | -1 | 763.9975 |
| Mid Range3 | 23349 | -2 | 794.8900 | 5349 | -1 | 764.8975 |
| High Range1 | 23373 | 0 | 797.3000 | 5373 | 1 | 767.3075 |
| High Range2 | 23340 | -2 | 793.9900 | 5340 | -1 | 763.9975 |
| High Range3 | 23349 | -2 | 794.8900 | 5349 | -1 | 764.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.14-3: NB-IoT guard-band Test frequencies for operating band 14

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 23281 | 3 | 788.1150 | 5281 | 1 | 758.1075 |
| Mid Range | 5 | 23306 | 3 | 790.6150 | 5306 | 1 | 760.6075 |
|  | 10 | 23376 | 3 | 797.6150 | 5376 | 1 | 767.6075 |
| High Range | Any | 23379 | -3 | 797.8850 | 5379 | -2 | 767.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.15 to 8.1.3.1.1.16	Void

8.1.3.1.1.17	NB-IoT FDD reference test frequencies for operating band 17

Table 8.1.3.1.1.17-1: NB-IoT standalone Test frequencies for operating band 17

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23731 | 0 | 704.1 | 5731 | -0.5 | 734.1 |
| Mid Range | 23790 | 0 | 710.0 | 5790 | -0.5 | 740.0 |
| High Range | 23849 | 0 | 715.9 | 5849 | -0.5 | 745.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.17-1a: NB-IoT standalone Test frequencies for operating band 17 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23732 | 0 | 704.2 | 5732 | -0.5 | 734.2 |
| Mid Range | 23790 | 0 | 710.0 | 5790 | -0.5 | 740.0 |
| High Range | 23848 | 0 | 715.8 | 5848 | -0.5 | 745.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.17-2: NB-IoT in-band Test frequencies for operating band 17

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23737 | 0 | 704.7000 | 5737 | -2 | 734.6925 |
| Low Range2 | 23790 | -2 | 709.9900 | 5790 | -1 | 739.9975 |
| Low Range3 | 23799 | -2 | 710.8900 | 5799 | -1 | 740.8975 |
| Mid Range1 | 23781 | 0 | 709.1000 | 5781 | -2 | 739.0925 |
| Mid Range2 | 23800 | -2 | 710.9900 | 5800 | -1 | 740.9975 |
| Mid Range3 | 23809 | -2 | 711.8900 | 5809 | -1 | 741.8975 |
| High Range1 | 23843 | 0 | 715.3000 | 5843 | 1 | 745.3075 |
| High Range2 | 23810 | -2 | 711.9900 | 5810 | -1 | 741.9975 |
| High Range3 | 23819 | -2 | 712.8900 | 5819 | -1 | 742.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.17-2a: NB-IoT in-band Test frequencies for operating band 17 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23737 | 0 | 704.7000 | 5737 | -2 | 734.6925 |
| Low Range2 | 23790 | -2 | 709.9900 | 5790 | -1 | 739.9975 |
| Low Range3 | 23799 | -2 | 710.8900 | 5799 | -1 | 740.8975 |
| Mid Range1 | 23781 | 0 | 709.1000 | 5781 | -2 | 739.0925 |
| Mid Range2 | 23800 | -2 | 710.9900 | 5800 | -1 | 740.9975 |
| Mid Range3 | 23809 | -2 | 711.8900 | 5809 | -1 | 741.8975 |
| High Range1 | 23843 | 0 | 715.3000 | 5843 | 1 | 745.3075 |
| High Range2 | 23810 | -2 | 711.9900 | 5810 | -1 | 741.9975 |
| High Range3 | 23819 | -2 | 712.8900 | 5819 | -1 | 742.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.17-3: NB-IoT guard-band Test frequencies for operating band 17

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23731 | 0 | 704.1000 | 5731 | 1 | 734.1075 |
| Mid Range | 23766 | 0 | 707.6000 | 5766 | 1 | 737.6075 |
| High Range | 23849 | 0 | 715.9000 | 5849 | -2 | 745.8925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.17-3a: NB-IoT guard-band Test frequencies for operating band 17 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23732 | 0 | 704.2000 | 5732 | 1 | 734.2075 |
| Mid Range | 23766 | 0 | 707.6000 | 5766 | 1 | 737.6075 |
| High Range | 23848 | 0 | 715.8000 | 5848 | -2 | 745.7925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.18	NB-IoT FDD reference test frequencies for operating band 18

Table 8.1.3.1.1.18-1: NB-IoT standalone Test frequencies for operating band 18

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23851 | 0 | 815.1 | 5851 | -0.5 | 860.1 |
| Mid Range | 23925 | 0 | 822.5 | 5925 | -0.5 | 867.5 |
| High Range | 23999 | 0 | 829.9 | 5999 | -0.5 | 874.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.18-2: NB-IoT in-band Test frequencies for operating band 18

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23857 | 0 | 815.7000 | 5857 | -2 | 860.6925 |
| Low Range2 | 23910 | -2 | 820.9900 | 5910 | -1 | 865.9975 |
| Low Range3 | 23919 | -2 | 821.8900 | 5919 | -1 | 866.8975 |
| Mid Range1 | 23916 | 0 | 821.6000 | 5916 | -2 | 866.5925 |
| Mid Range2 | 23935 | -2 | 823.4900 | 5935 | -1 | 868.4975 |
| Mid Range3 | 23944 | -2 | 824.3900 | 5944 | -1 | 869.3975 |
| High Range1 | 23993 | 0 | 829.3000 | 5993 | 1 | 874.3075 |
| High Range2 | 23960 | -2 | 825.9900 | 5960 | -1 | 870.9975 |
| High Range3 | 23969 | -2 | 826.8900 | 5969 | -1 | 871.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.18-3: NB-IoT guard-band Test frequencies for operating band 18

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23851 | 0 | 815.1000 | 5851 | 1 | 860.1075 |
| Mid Range | 23901 | 0 | 820.1000 | 5901 | 1 | 865.1075 |
| High Range | 23999 | 0 | 829.9000 | 5999 | -2 | 874.8925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.19	NB-IoT FDD reference test frequencies for operating band 19

Table 8.1.3.1.1.19-1: NB-IoT standalone Test frequencies for operating band 19

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 24001 | 0 | 830.1 | 6001 | -0.5 | 875.1 |
| Mid Range | 24075 | 0 | 837.5 | 6075 | -0.5 | 882.5 |
| High Range | 24149 | 0 | 844.9 | 6149 | -0.5 | 889.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.19-2: NB-IoT in-band Test frequencies for operating band 19

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 24007 | 0 | 830.7000 | 6007 | -2 | 875.6925 |
| Low Range2 | 24060 | -2 | 835.9900 | 6060 | -1 | 880.9975 |
| Low Range3 | 24069 | -2 | 836.8900 | 6069 | -1 | 881.8975 |
| Mid Range1 | 24066 | 0 | 836.6000 | 6066 | -2 | 881.5925 |
| Mid Range2 | 24085 | -2 | 838.4900 | 6085 | -1 | 883.4975 |
| Mid Range3 | 24094 | -2 | 839.3900 | 6094 | -1 | 884.3975 |
| High Range1 | 24143 | 0 | 844.3000 | 6143 | 1 | 889.3075 |
| High Range2 | 24110 | -2 | 840.9900 | 6110 | -1 | 885.9975 |
| High Range3 | 24119 | -2 | 841.8900 | 6119 | -1 | 886.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.19-3: NB-IoT guard-band Test frequencies for operating band 19

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 24001 | 0 | 830.1000 | 6001 | 1 | 875.1075 |
| Mid Range | 24051 | 0 | 835.1000 | 6051 | 1 | 880.1075 |
| High Range | 24149 | 0 | 844.9000 | 6149 | -2 | 889.8925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.20	NB-IoT FDD reference test frequencies for operating band 20

Table 8.1.3.1.1.20-1: NB-IoT standalone Test frequencies for operating band 20

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 24151 | 0 | 832.1 | 6151 | -0.5 | 791.1 |
| Mid Range | 24300 | 0 | 847.0 | 6300 | -0.5 | 806.0 |
| High Range | 24449 | 0 | 861.9 | 6449 | -0.5 | 820.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.20-2: NB-IoT in-band Test frequencies for operating band 20

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 24157 | 0 | 832.7000 | 6157 | -2 | 791.6925 |
| Low Range2 | 24210 | -2 | 837.9900 | 6210 | -1 | 796.9975 |
| Low Range3 | 24219 | -2 | 838.8900 | 6219 | -1 | 797.8975 |
| Mid Range1 | 24291 | 0 | 846.1000 | 6291 | -2 | 805.0925 |
| Mid Range2 | 24310 | -2 | 847.9900 | 6310 | -1 | 806.9975 |
| Mid Range3 | 24319 | -2 | 848.8900 | 6319 | -1 | 807.8975 |
| High Range1 | 24443 | 0 | 861.3000 | 6443 | 1 | 820.3075 |
| High Range2 | 24410 | -2 | 857.9900 | 6410 | -1 | 816.9975 |
| High Range3 | 24419 | -2 | 858.8900 | 6419 | -1 | 817.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.20-3: NB-IoT guard-band Test frequencies for operating band 20

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 24151 | 3 | 832.1150 | 6151 | 1 | 791.1075 |
| Mid Range | 5 | 24276 | 3 | 844.6150 | 6276 | 1 | 803.6075 |
|  | 10 | 24346 | 3 | 851.6150 | 6346 | 1 | 810.6075 |
| High Range | Any | 24449 | -3 | 861.8850 | 6449 | -2 | 820.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.21	NB-IoT FDD reference test frequencies for operating band 21

Table 8.1.3.1.1.21-1: NB-IoT standalone Test frequencies for operating band 21

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 24451 | 0 | 1448.00 | 6451 | -0.5 | 1496.00 |
| Mid Range | 24525 | 0 | 1455.40 | 6525 | -0.5 | 1503.40 |
| High Range | 24599 | 0 | 1462.80 | 6599 | -0.5 | 1510.80 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.21-2: NB-IoT in-band Test frequencies for operating band 21

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 24457 | -3 | 1448.5850 | 6457 | -2 | 1496.5925 |
| Low Range2 | 24510 | -2 | 1453.8900 | 6510 | -1 | 1501.8975 |
| Low Range3 | 24519 | -2 | 1454.7900 | 6519 | -1 | 1502.7975 |
| Mid Range | 24516 | -3 | 1454.4850 | 6516 | -2 | 1502.4925 |
| Mid Range2 | 24535 | -2 | 1456.3900 | 6535 | -1 | 1504.3975 |
| Mid Range3 | 24544 | -2 | 1457.2900 | 6544 | -1 | 1505.2975 |
| High Range | 24593 | 3 | 1462.2150 | 6593 | 1 | 1510.2075 |
| High Range2 | 24560 | -2 | 1458.8900 | 6560 | -1 | 1506.8975 |
| High Range3 | 24569 | -2 | 1459.7900 | 6569 | -1 | 1507.7975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.21-3: NB-IoT guard-band Test frequencies for operating band 21

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 24451 | 0 | 1448.0000 | 6451 | 1 | 1496.0075 |
| Mid Range | 5 | 24501 | 0 | 1453.0000 | 6501 | 1 | 1501.0075 |
|  | 10 | 24571 | 0 | 1460.0000 | 6571 | 1 | 1508.0075 |
| High Range | Any | 24599 | 0 | 1462.8000 | 6599 | -2 | 1510.7925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.22 to 8.1.3.1.1.24	Void

8.1.3.1.1.25	NB-IoT FDD reference test frequencies for operating band 25

Table 8.1.3.1.1.25-1: NB-IoT standalone Test frequencies for operating band 25

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26041 | 0 | 1850.10 | 8041 | -0.5 | 1930.1 |
| Mid Range | 26365 | 0 | 1882.50 | 8365 | -0.5 | 1962.5 |
| High Range | 26689 | 0 | 1914.90 | 8689 | -0.5 | 1994.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.25-1a: NB-IoT standalone Test frequencies for operating band 25 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26042 | 0 | 1850.20 | 8042 | -0.5 | 1930.2 |
| Mid Range | 26365 | 0 | 1882.50 | 8365 | -0.5 | 1962.5 |
| High Range | 26688 | 0 | 1914.80 | 8688 | -0.5 | 1994.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.25-2: NB-IoT in-band Test frequencies for operating band 25

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 26046 | 0 | 1850.6000 | 8046 | -2 | 1930.5925 |
| Low Range2 | 26100 | -2 | 1855.9900 | 8100 | -1 | 1935.9975 |
| Low Range3 | 26109 | -2 | 1856.8900 | 8109 | -1 | 1936.8975 |
| Mid Range1 | 26356 | 0 | 1881.6000 | 8356 | -2 | 1961.5925 |
| Mid Range2 | 26375 | -2 | 1883.4900 | 8375 | -1 | 1963.4975 |
| Mid Range3 | 26384 | -2 | 1884.3900 | 8384 | -1 | 1964.3975 |
| High Range1 | 26684 | 0 | 1914.4000 | 8684 | 1 | 1994.4075 |
| High Range2 | 26650 | -2 | 1910.99 | 8650 | -1 | 1990.9975 |
| High Range3 | 26659 | -2 | 1911.89 | 8659 | -1 | 1991.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.25-2a: NB-IoT in-band Test frequencies for operating band 25 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 26046 | 0 | 1850.6000 | 8046 | -2 | 1930.5925 |
| Low Range2 | 26100 | -2 | 1855.9900 | 8100 | -1 | 1935.9975 |
| Low Range3 | 26109 | -2 | 1856.8900 | 8109 | -1 | 1936.8975 |
| Mid Range1 | 26356 | 0 | 1881.6000 | 8356 | -2 | 1961.5925 |
| Mid Range2 | 26375 | -2 | 1883.4900 | 8375 | -1 | 1963.4975 |
| Mid Range3 | 26384 | -2 | 1884.3900 | 8384 | -1 | 1964.3975 |
| High Range1 | 26684 | 0 | 1914.4000 | 8684 | 1 | 1994.4075 |
| High Range2 | 26650 | -2 | 1910.99 | 8650 | -1 | 1990.9975 |
| High Range3 | 26659 | -2 | 1911.89 | 8659 | -1 | 1991.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.25-3: NB-IoT guard-band Test frequencies for operating band 25

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26041 | 0 | 1850.1000 | 8041 | 1 | 1930.1075 |
| Mid Range | 26341 | 0 | 1880.1000 | 8341 | 1 | 1960.1075 |
| High Range | 26689 | 0 | 1914.9000 | 8689 | -2 | 1994.8925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.25-3a: NB-IoT guard-band Test frequencies for operating band 25 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26041 | 0 | 1850.1000 | 8041 | 1 | 1930.1075 |
| Mid Range | 26341 | 0 | 1880.1000 | 8341 | 1 | 1960.1075 |
| High Range | 26688 | 0 | 1914.8000 | 8688 | -2 | 1994.7925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.26	NB-IoT FDD reference test frequencies for operating band 26

Table 8.1.3.1.1.26-1: NB-IoT standalone Test frequencies for operating band 26

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26691 | 0 | 814.1 | 8691 | -0.5 | 859.1 |
| Mid Range | 26865 | 0 | 831.5 | 8865 | -0.5 | 876.5 |
| High Range | 27039 | 0 | 848.9 | 9039 | -0.5 | 893.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.26-1a: NB-IoT standalone Test frequencies for operating band 26 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26692 | 0 | 814.2 | 8692 | -0.5 | 859.2 |
| Mid Range | 26865 | 0 | 831.5 | 8865 | -0.5 | 876.5 |
| High Range | 27038 | 0 | 848.8 | 9038 | -0.5 | 893.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.26-2: NB-IoT in-band Test frequencies for operating band 26

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 26696 | 0 | 814.6000 | 8696 | -2 | 859.5925 |
| Low Range2 | 26750 | -2 | 819.9900 | 8750 | -1 | 864.9975 |
| Low Range3 | 26759 | -2 | 820.8900 | 8759 | -1 | 865.8975 |
| Mid Range1 | 26856 | 0 | 830.6000 | 8856 | -2 | 875.5925 |
| Mid Range2 | 26875 | -2 | 832.4900 | 8875 | -1 | 877.4975 |
| Mid Range3 | 26884 | -2 | 833.3900 | 8884 | -1 | 878.3975 |
| High Range1 | 27034 | 0 | 848.4000 | 9034 | 1 | 893.4075 |
| High Range2 | 27000 | -2 | 844.9900 | 9000 | -1 | 889.9975 |
| High Range3 | 27009 | -2 | 845.8900 | 9009 | -1 | 890.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.26-2a: NB-IoT in-band Test frequencies for operating band 26 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 26696 | 0 | 814.6000 | 8696 | -2 | 859.5925 |
| Low Range2 | 26750 | -2 | 819.9900 | 8750 | -1 | 864.9975 |
| Low Range3 | 26759 | -2 | 820.8900 | 8759 | -1 | 865.8975 |
| Mid Range1 | 26856 | 0 | 830.6000 | 8856 | -2 | 875.5925 |
| Mid Range2 | 26875 | -2 | 832.4900 | 8875 | -1 | 877.4975 |
| Mid Range3 | 26884 | -2 | 833.3900 | 8884 | -1 | 878.3975 |
| High Range1 | 27034 | 0 | 848.4000 | 9034 | 1 | 893.4075 |
| High Range2 | 27000 | -2 | 844.9900 | 9000 | -1 | 889.9975 |
| High Range3 | 27009 | -2 | 845.8900 | 9009 | -1 | 890.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.26-3: NB-IoT guard-band Test frequencies for operating band 26

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26691 | 0 | 814.1000 | 8691 | 1 | 859.1075 |
| Mid Range | 26841 | 0 | 829.1000 | 8841 | 1 | 874.1075 |
| High Range | 27039 | 0 | 848.9000 | 9039 | -2 | 893.8925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.26-3a: NB-IoT guard-band Test frequencies for operating band 26 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26692 | 0 | 814.2000 | 8692 | 1 | 859.2075 |
| Mid Range | 26841 | 0 | 829.1000 | 8841 | 1 | 874.1075 |
| High Range | 27038 | 0 | 848.8000 | 9038 | -2 | 893.7925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.27	Void

8.1.3.1.1.28	NB-IoT FDD reference test frequencies for operating band 28

Table 8.1.3.1.1.28-1: NB-IoT standalone Test frequencies for operating band 28

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 27211 | 0 | 703.1 | 9211 | -0.5 | 758.1 |
| Mid Range | 27435 | 0 | 725.5 | 9435 | -0.5 | 780.5 |
| High Range | 27659 | 0 | 747.9 | 9659 | -0.5 | 802.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.28-2: NB-IoT in-band Test frequencies for operating band 28

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 27216 | 0 | 703.6000 | 9216 | -2 | 758.5925 |
| Low Range2 | 27270 | -2 | 708.9900 | 9270 | -1 | 763.9975 |
| Low Range3 | 27279 | -2 | 709.8900 | 9279 | -1 | 764.8975 |
| Mid Range1 | 27426 | 0 | 724.6000 | 9426 | -2 | 779.5925 |
| Mid Range2 | 27420 | -2 | 723.9900 | 9420 | -1 | 778.9975 |
| Mid Range3 | 27429 | -2 | 724.8900 | 9429 | -1 | 779.8975 |
| High Range1 | 27654 | 0 | 747.4000 | 9654 | 1 | 802.4075 |
| High Range2 | 27620 | -2 | 743.9900 | 9620 | -1 | 798.9975 |
| High Range3 | 27629 | -2 | 744.8900 | 9629 | -1 | 799.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.28-3: NB-IoT guard-band Test frequencies for operating band 28

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 27211 | 0 | 703.1000 | 9211 | 1 | 758.1075 |
| Mid Range | 5 | 27411 | 0 | 723.1000 | 9411 | 1 | 778.1075 |
|  | 10 | 27456 | 0 | 727.6000 | 9456 | 1 | 782.6075 |
| High Range | Any | 27659 | 0 | 747.9000 | 9659 | -2 | 802.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.29 to 8.1.3.1.1.30	Void

8.1.3.1.1.31	NB-IoT FDD reference test frequencies for operating band 31

Table 8.1.3.1.1.31-1: NB-IoT standalone Test frequencies for operating band 31

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 27761 | 0 | 452.60 | 9871 | -0.5 | 462.6 |
| Mid Range | 27785 | 0 | 455.00 | 9895 | -0.5 | 465 |
| High Range | 27809 | 0 | 457.40 | 9919 | -0.5 | 467.4 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.31-2: NB-IoT in-band Test frequencies for operating band 31

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 27766 | -3 | 453.0850 | 9876 | -2 | 463.0925 |
| Low Range2 | 27794 | 0 | 455.9000 | 9904 | 1 | 465.9075 |
| Low Range3 | 27803 | 0 | 456.8000 | 9913 | 1 | 466.8075 |
| Mid Range1 | 27776 | -3 | 454.0850 | 9886 | -2 | 464.0925 |
| Mid Range2 | 27794 | 0 | 455.9000 | 9904 | 1 | 465.9075 |
| Mid Range3 | 27803 | 0 | 456.8000 | 9913 | 1 | 466.8075 |
| High Range1 | 27804 | 3 | 456.9150 | 9914 | 1 | 466.9075 |
| High Range2 | 27794 | 0 | 455.9000 | 9904 | 1 | 465.9075 |
| High Range3 | 27803 | 0 | 456.8000 | 9913 | 1 | 466.8075 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 5 MHz and NB-IoT PRB 17 NOTE 3: Related to LTE channel BW 5 MHz and NB-IoT PRB 22 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.31-3: NB-IoT guard-band Test frequencies for operating band 31

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 27761 | 0 | 452.6000 | 9871 | 1 | 462.6075 |
| Mid Range | 27761 | 0 | 452.6000 | 9871 | 1 | 462.6075 |
| High Range | 27809 | 0 | 457.4000 | 9919 | -2 | 467.3925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.32 to 8.1.3.1.1. 64	Void

8.1.3.1.1.65	NB-IoT FDD reference test frequencies for operating band 65

Table 8.1.3.1.1.65-1: NB-IoT standalone Test frequencies for operating band 65

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 131073 | 0 | 1920.1 | 65537 | -0.5 | 2110.1 |
| Mid Range | 131522 | 0 | 1965.0 | 65986 | -0.5 | 2155.0 |
| High Range | 131971 | 0 | 2009.9 | 66435 | -0.5 | 2199.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing NOTE 2: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |

Table 8.1.3.1.1.65-2: NB-IoT in-band Test frequencies for operating band 65

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 131078 | 0 | 1920.6000 | 65542 | -2 | 2110.5925 |
| Low Range2 | 131132 | -2 | 1925.9900 | 65596 | -1 | 2115.9975 |
| Low Range3 | 131141 | -2 | 1926.8900 | 65605 | -1 | 2116.8975 |
| Mid Range1 | 131513 | 0 | 1964.1000 | 65977 | -2 | 2154.0925 |
| Mid Range2 | 131532 | -2 | 1965.9900 | 65996 | -1 | 2155.9975 |
| Mid Range3 | 131541 | -2 | 1966.8900 | 66005 | -1 | 2156.8975 |
| High Range1 | 131966 | 0 | 2009.4000 | 66430 | 1 | 2199.4075 |
| High Range2 | 131932 | -2 | 2005.9900 | 66396 | -1 | 2195.9975 |
| High Range3 | 131941 | -2 | 2006.8900 | 66405 | -1 | 2196.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing NOTE 3: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |

Table 8.1.3.1.1.65-3: NB-IoT guard-band Test frequencies for operating band 65

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 131073 | 0 | 1920.1000 | 65537 | 1 | 2110.1075 |
| Mid Range | 5 | 131498 | 0 | 1962.6000 | 65962 | 1 | 2152.6075 |
|  | 10 | 131568 | 0 | 1969.6000 | 66032 | 1 | 2159.6075 |
| High Range | Any | 131971 | 0 | 2009.9000 | 66435 | -2 | 2199.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing NOTE 3: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |  |

8.1.3.1.1.66	NB-IoT FDD reference test frequencies for operating band 66

Table 8.1.3.1.1.66-1: NB-IoT standalone Test frequencies for operating band 66

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 131973 | 0 | 1710.1 | 66437 | -0.5 | 2110.1 |
| Mid Range | 132322 | 0 | 1745.0 | 66786 | -0.5 | 2145.0 |
| High Range | 132671 | 0 | 1779.9 | 67135 | -0.5 | 2179.9 |
| High Range3 | 132421 | 0 | 1754.9 | 66885 | -0.5 | 2154.9 |
| High Range4 | 132571 | 0 | 1769.0 | 67035 | -0.5 | 2169.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing NOTE 2: Only paired part of the band considered for NB-IoT NOTE 3: Applicable to NB-IoT FDD MFBI where band 66 overlaps band 4 NOTE 4: Applicable to NB-IoT FDD MFBI where band 66 overlaps band 10 |  |  |  |  |  |  |

Table 8.1.3.1.1.66-1a: NB-IoT standalone Test frequencies for operating band 66 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 131974 | 0 | 1710.2 | 66438 | -0.5 | 2110.2 |
| Mid Range | 132322 | 0 | 1745.0 | 66786 | -0.5 | 2145.0 |
| High Range | 132670 | 0 | 1779.8 | 67134 | -0.5 | 2179.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing NOTE 2: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |

Table 8.1.3.1.1.66-2: NB-IoT in-band Test frequencies for operating band 66

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 131978 | 0 | 1710.6000 | 66442 | -2 | 2110.5925 |
| Low Range2 | 132032 | -2 | 1715.9900 | 66496 | -1 | 2115.9975 |
| Low Range3 | 132041 | -2 | 1716.8900 | 66505 | -1 | 2116.8975 |
| Mid Range1 | 132313 | 0 | 1744.1000 | 66777 | -2 | 2144.0925 |
| Mid Range2 | 132432 | -2 | 1755.9900 | 66896 | -1 | 2155.9975 |
| Mid Range3 | 132441 | -2 | 1756.8900 | 66905 | -1 | 2156.8975 |
| High Range1 | 132666 | 0 | 1779.4000 | 67130 | 1 | 2179.4075 |
| High Range2 | 132632 | -2 | 1775.9900 | 67096 | -1 | 2175.9975 |
| High Range3 | 132641 | -2 | 1776.8900 | 67105 | -1 | 2176.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing NOTE 3: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |

Table 8.1.3.1.1.66-2a: NB-IoT in-band Test frequencies for operating band 66 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 131978 | 0 | 1710.6000 | 66442 | -2 | 2110.5925 |
| Low Range2 | 132032 | -2 | 1715.9900 | 66496 | -1 | 2115.9975 |
| Low Range3 | 132041 | -2 | 1716.8900 | 66505 | -1 | 2116.8975 |
| Mid Range1 | 132313 | 0 | 1744.1000 | 66777 | -2 | 2144.0925 |
| Mid Range2 | 132432 | -2 | 1755.9900 | 66896 | -1 | 2155.9975 |
| Mid Range3 | 132441 | -2 | 1756.8900 | 66905 | -1 | 2156.8975 |
| High Range1 | 132666 | 0 | 1779.4000 | 67130 | 1 | 2179.4075 |
| High Range2 | 132632 | -2 | 1775.9900 | 67096 | -1 | 2175.9975 |
| High Range3 | 132641 | -2 | 1776.8900 | 67105 | -1 | 2176.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing NOTE 3: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |

Table 8.1.3.1.1.66-3: NB-IoT guard-band Test frequencies for operating band 66

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 131973 | 0 | 1710.1000 | 66437 | 1 | 2110.1075 |
| Mid Range | 5 | 132298 | 0 | 1742.6000 | 66762 | 1 | 2142.6075 |
|  | 10 | 132468 | 0 | 1759.6000 | 66932 | 1 | 2159.6075 |
| High Range | Any | 132671 | 0 | 1779.9000 | 67135 | -2 | 2179.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing NOTE 3: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |  |

Table 8.1.3.1.1.66-3a: NB-IoT guard-band Test frequencies for operating band 66 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 131974 | 0 | 1710.2000 | 66438 | 1 | 2110.2075 |
| Mid Range | 5 | 132298 | 0 | 1742.6000 | 66762 | 1 | 2142.6075 |
|  | 10 | 132468 | 0 | 1759.6000 | 66932 | 1 | 2159.6075 |
| High Range | Any | 132670 | 0 | 1779.8000 | 67134 | -2 | 2179.7925 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing NOTE 2: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |  |

8.1.3.1.1.67 to 8.1.3.1.1.69	Void

8.1.3.1.1.70	NB-IoT FDD reference test frequencies for operating band 70

Table 8.1.3.1.1.70-1: NB-IoT standalone Test frequencies for operating band 70

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 132973 | 0 | 1695.1 | 68337 | -0.5 | 1995.1 |
| Mid Range | 133047 | 0 | 1702.5 | 68411 | -0.5 | 2002.5 |
| High Range | 133121 | 0 | 1709.9 | 68485 | -0.5 | 2009.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.70-2: NB-IoT in-band Test frequencies for operating band 70

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 132979 | -3 | 1695.6850 | 68343 | -2 | 1995.6925 |
| Mid Range | 3 | 133038 | -3 | 1701.5850 | 68402 | -2 | 2001.5925 |
|  | 10 PRB 30 | 133057 | -2 | 1703.4900 | 68421 | -1 | 2003.4975 |
|  | 10 PRB 35 | 133066 | -2 | 1704.3900 | 68430 | -1 | 2004.3975 |
| High Range | Any | 133115 | 3 | 1709.3150 | 68479 | 1 | 2009.3075 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.70-3: NB-IoT guard-band Test frequencies for operating band 70

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 132973 | 0 | 1695.1000 | 68337 | 1 | 1995.1075 |
| Mid Range | 5 | 133023 | 0 | 1700.1000 | 68387 | 1 | 2000.1075 |
|  | 10 | 133093 | 0 | 1707.1000 | 68457 | 1 | 2007.1075 |
| High Range | Any | 133121 | 0 | 1709.9000 | 68485 | -2 | 2009.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing NOTE 3: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |  |

8.1.3.1.1.71	NB-IoT FDD reference test frequencies for operating band 71

Table 8.1.3.1.1.71-1: NB-IoT standalone Test frequencies for operating band 71

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 133123 | 0 | 663.1000 | 68587 | -0.5 | 617.1000 |
| Mid Range | 133297 | 0 | 680.5000 | 68761 | -0.5 | 634.5000 |
| High Range | 133471 | 0 | 697.9000 | 68935 | -0.5 | 651.9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.71-1a: NB-IoT standalone Test frequencies for operating band 71 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 133124 | 0 | 663.2000 | 68588 | -0.5 | 617.2000 |
| Mid Range | 133297 | 0 | 680.5000 | 68761 | -0.5 | 634.5000 |
| High Range | 133470 | 0 | 697.8000 | 68934 | -0.5 | 651.8000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.71-2: NB-IoT in-band Test frequencies for operating band 71

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 133129 | 0 | 663.7000 | 68593 | -2 | 617.6925 |
| Low Range2 | 133182 | -2 | 668.9900 | 68646 | -1 | 622.9975 |
| Low Range3 | 133191 | -2 | 669.8900 | 68655 | -1 | 623.8975 |
| Mid Range1 | 133288 | 0 | 679.6000 | 68752 | -2 | 633.5925 |
| Mid Range2 | 133307 | -2 | 681.4900 | 68771 | -1 | 635.4975 |
| Mid Range3 | 133316 | -2 | 682.3900 | 68780 | -1 | 636.3975 |
| High Range1 | 133465 | 0 | 697.3000 | 68929 | 1 | 651.3075 |
| High Range2 | 133432 | -2 | 693.9900 | 68896 | -1 | 647.9975 |
| High Range3 | 133441 | -2 | 694.8900 | 68905 | -1 | 648.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.71-2a: NB-IoT in-band Test frequencies for operating band 71 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 133129 | 0 | 663.7000 | 68593 | -2 | 617.6925 |
| Low Range2 | 133182 | -2 | 668.9900 | 68646 | -1 | 622.9975 |
| Low Range3 | 133191 | -2 | 669.8900 | 68655 | -1 | 623.8975 |
| Mid Range1 | 133288 | 0 | 679.6000 | 68752 | -2 | 633.5925 |
| Mid Range2 | 133307 | -2 | 681.4900 | 68771 | -1 | 635.4975 |
| Mid Range3 | 133316 | -2 | 682.3900 | 68780 | -1 | 636.3975 |
| High Range1 | 133465 | 0 | 697.3000 | 68929 | 1 | 651.3075 |
| High Range2 | 133432 | -2 | 693.9900 | 68896 | -1 | 647.9975 |
| High Range3 | 133441 | -2 | 694.8900 | 68905 | -1 | 648.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.71-3: NB-IoT guard-band Test frequencies for operating band 71

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 133123 | 3 | 663. 1150 | 68587 | 1 | 617.1075 |
| Mid Range | 5 | 133273 | 3 | 678. 1150 | 68737 | 1 | 632.1075 |
|  | 10 | 133343 | 3 | 685. 1150 | 68807 | 1 | 639.1075 |
| High Range | Any | 133471 | -3 | 697. 8850 | 68935 | -2 | 651.8925 |
| NOTE 1: Void. NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.71-3a: NB-IoT guard-band Test frequencies for operating band 71 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 133124 | 3 | 663. 2150 | 68588 | 1 | 617.2075 |
| Mid Range | 5 | 133273 | 3 | 678. 1150 | 68737 | 1 | 632.1075 |
|  | 10 | 133343 | 3 | 685. 1150 | 68807 | 1 | 639.1075 |
| High Range | Any | 133470 | -3 | 697. 7850 | 68934 | -2 | 651.7925 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.72	Void

8.1.3.1.1.73	NB-IoT FDD reference test frequencies for operating band 73

Table 8.1.3.1.1.73-1: NB-IoT standalone Test frequencies for operating band 73

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 133523 | 0 | 450.1000 | 68997 | -0.5 | 461.1000 |
| Mid Range | 133547 | 0 | 452.5000 | 69011 | -0.5 | 462.5000 |
| High Range | 133571 | 0 | 454.9000 | 69035 | -0.5 | 464.9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.73-2: NB-IoT in-band Test frequencies for operating band 73

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 133556 | 0 | 453.4000 | 69020 | 1 | 463.4000 |
| Mid Range | 133556 | 0 | 453.4000 | 69020 | 1 | 463.4000 |
| High Range | 133556 | 0 | 453.4000 | 69020 | 1 | 463.4000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing NOTE 2: Defined for 5 MHz LTE Cell Bandwidth. |  |  |  |  |  |  |

Table 8.1.3.1.1.73-3: NB-IoT guard-band Test frequencies for operating band 73

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 133523 | 0 | 450.1000 | 68997 | 1 | 461.1000 |
| Mid Range | 5 | 133523 | 0 | 450.1000 | 68997 | 1 | 461.1000 |
| High Range | Any | 133571 | 0 | 454.9000 | 69035 | 1 | 464.9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.74 to 8.1.3.1.1.84	Void

8.1.3.1.1.85	NB-IoT FDD reference test frequencies for operating band 85

Table 8.1.3.1.1.85-1: NB-IoT standalone Test frequencies for operating band 85

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134003 | 0 | 698,1000 | 70367 | -0.5 | 728,1000 |
| Mid Range | 134082 | 0 | 706,0000 | 70446 | -0.5 | 736,0000 |
| High Range | 134181 | 0 | 715,9000 | 70545 | -0.5 | 745,9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.85-1a: NB-IoT standalone Test frequencies for operating band 85 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134004 | 0 | 698,2000 | 70368 | -0.5 | 728,2000 |
| Mid Range | 134082 | 0 | 706,0000 | 70446 | -0.5 | 736,0000 |
| High Range | 134180 | 0 | 715,8000 | 70544 | -0.5 | 745,8000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.85-2: NB-IoT in-band Test frequencies for operating band 85

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 134062 | -2 | 703,9900 | 70426 | -1 | 733,9975 |
| Low Range2 | 134071 | -2 | 704,8900 | 70435 | -1 | 734,8975 |
| Mid Range1 | 134102 | -2 | 707,9900 | 70466 | -1 | 737,9975 |
| Mid Range2 | 134111 | -2 | 708,8900 | 70475 | -1 | 738,8975 |
| High Range1 | 134142 | -2 | 711,9900 | 70506 | -1 | 741,9975 |
| High Range2 | 134151 | -2 | 712,8900 | 70515 | -1 | 742,8975 |
| NOTE 1: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 3: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.85-2a: NB-IoT in-band Test frequencies for operating band 85 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 134062 | -2 | 703,9900 | 70426 | -1 | 733,9975 |
| Low Range2 | 134071 | -2 | 704,8900 | 70435 | -1 | 734,8975 |
| Mid Range1 | 134102 | -2 | 707,9900 | 70466 | -1 | 737,9975 |
| Mid Range2 | 134111 | -2 | 708,8900 | 70475 | -1 | 738,8975 |
| High Range1 | 134142 | -2 | 711,9900 | 70506 | -1 | 741,9975 |
| High Range2 | 134151 | -2 | 712,8900 | 70515 | -1 | 742,8975 |
| NOTE 1: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 3: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.85-3: NB-IoT guard-band Test frequencies for operating band 85

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 134098 | 0 | 707,6000 | 70462 | 1 | 737,6075 |
| Mid Range | 10 | 134138 | 0 | 711,6000 | 70502 | 1 | 741,6075 |
| High Range | Any | 134178 | 0 | 715,6000 | 70542 | -2 | 745,6075 |
| NOTE 1: Void. NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.85-3a: NB-IoT guard-band Test frequencies for operating band 85 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 134004 | 0 | 698.2000 | 70368 | 1 | 728.2075 |
| Mid Range | 10 | 134092 | 0 | 707.0000 | 70456 | 1 | 737.0075 |
| High Range | Any | 134180 | 0 | 715.8000 | 70544 | -2 | 745.7925 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.86	FFS

8.1.3.1.1.87	NB-IoT FDD reference test frequencies for operating band 87

Table 8.1.3.1.1.87-1: NB-IoT standalone Test frequencies for operating band 87

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134183 | 0 | 410.1000 | 70547 | -0.5 | 420.1000 |
| Mid Range | 134207 | 0 | 412.5000 | 70571 | -0.5 | 422.5000 |
| High Range | 134231 | 0 | 414.9000 | 70595 | -0.5 | 424.9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.87-2: NB-IoT in-band Test frequencies for operating band 87

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134216 | 0 | 413.4000 | 70580 | 1 | 423.4000 |
| Mid Range | 134216 | 0 | 413.4000 | 70580 | 1 | 423.4000 |
| High Range | 134216 | 0 | 413.4000 | 70580 | 1 | 423.4000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing NOTE 2: Defined for 5 MHz LTE Cell Bandwidth. |  |  |  |  |  |  |

Table 8.1.3.1.1.87-3: NB-IoT guard-band Test frequencies for operating band 87

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 134183 | 0 | 410.1000 | 70547 | 1 | 420.1000 |
| Mid Range | 5 | 134183 | 0 | 410.1000 | 70547 | 1 | 4210.1000 |
| High Range | Any | 134231 | 0 | 414.9000 | 70595 | 1 | 424.9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.88	NB-IoT FDD reference test frequencies for operating band 88

Table 8.1.3.1.1.88-1: NB-IoT standalone Test frequencies for operating band 88

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134233 | 0 | 412.1000 | 70597 | -0.5 | 422.1000 |
| Mid Range | 134257 | 0 | 414.5000 | 70621 | -0.5 | 424.5000 |
| High Range | 134281 | 0 | 416.9000 | 70645 | -0.5 | 426.9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.88-2: NB-IoT in-band Test frequencies for operating band 88

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134266 | 0 | 415.4000 | 70630 | 1 | 425.4000 |
| Mid Range | 134266 | 0 | 415.4000 | 70630 | 1 | 425.4000 |
| High Range | 134266 | 0 | 415.4000 | 70630 | 1 | 425.4000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing NOTE 2: Defined for 5 MHz LTE Cell Bandwidth. |  |  |  |  |  |  |

Table 8.1.3.1.1.88-3: NB-IoT guard-band Test frequencies for operating band 88

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 134233 | 0 | 412.1000 | 70597 | 1 | 422.1000 |
| Mid Range | 5 | 134233 | 0 | 412.1000 | 70597 | 1 | 422.1000 |
| High Range | Any | 134281 | 0 | 416.9000 | 70645 | 1 | 426.9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.89 to 8.1.3.1.1.102	Void

8.1.3.1.1.103	NB-IoT FDD reference test frequencies for operating band 103

Table 8.1.3.1.1.103-1: NB-IoT standalone Test frequencies for operating band 103

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134283 | 0 | 787.1 | 70647 | -0.5 | 757.1 |
| Mid Range | 134287 | 0 | 787.5 | 70651 | -0.5 | 757.5 |
| High Range | 134291 | 0 | 787.9 | 70655 | -0.5 | 757.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.103-1a: NB-IoT standalone Test frequencies for operating band 103 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134284 | 0 | 787.2 | 70648 | -0.5 | 757.2 |
| Mid Range | 134287 | 0 | 787.5 | 70651 | -0.5 | 757.5 |
| High Range | 134290 | 0 | 787.8 | 70654 | -0.5 | 757.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

[End of change]