3GPP TSG-RAN5 Meeting #96-e	R5-224240r1

# Electronic Meeting, 15th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1775 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of additional tolerance to test requirement of R15 Transmitter power test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Additional tolerance ΔTIB,c for NR-DC was missing in the test requirement of some R15 Transmitter power test cases (6.2.4 and 6.2C.1). - The “DC” in Additional tolerance ΔTIB,c description shall be “EN-DC”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The missing additional tolerance ΔTIB,c for NR-DC was added to the test requirement of some R15 Transmitter power test cases (6.2.4 and 6.2C.1). - The “DC” in Additional tolerance ΔTIB,c description was revised to “EN-DC”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Good UE may fail the test (6.2.4 or 6.2C.1) without considering the additional tolerance ΔTIB,c for NR-DC. - The editorial error will remain in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.5, 6.2.2.5, 6.2.3.5, 6.2.4.5, 6.2A.1.1.5, 6.2A.2.1.5, 6.2A.3.1.5, 6.2A.4.1.5, 6.2B.1.1.5, 6.2B.2.1.5, 6.2B.3.1.5, 6.2B.4.1.5, 6.2C.1.5, 6.2C.3.5, 6.2C.4.5, 6.2C.5.5, 6.2D.1.5, 6.2D.2.5, 6.2D.3.5, 6.2D.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: The coversheet was updated for Reason for change, Summary of change, and Consequences if not approved. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1 for Power Class 3, Table 6.2.1.5-2 for Power Class 2, Table 6.2.1.5-2a for Power Class 1.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1.

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±2±TT |
| n2 |  |  |  |  | 23 | ±23±TT |
| n3 |  |  |  |  | 23 | ±23±TT |
| n5 |  |  |  |  | 23 | ±2±TT |
| n7 |  |  |  |  | 23 | ±23±TT |
| n8 |  |  |  |  | 23 | ±23±TT |
| n12 |  |  |  |  | 23 | ±23±TT |
| n14 |  |  |  |  | 23 | ±2±TT |
| n20 |  |  |  |  | 23 | ±23±TT |
| n24 |  |  |  |  | 23 | +2+TT/-3.03-TT |
| n25 |  |  |  |  | 23 | ±23±TT |
| n26 |  |  |  |  | 23 | ±23±TT |
| n28 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n30 |  |  |  |  | 23 | ±2±TT |
| n34 |  |  |  |  | 23 | ±2±TT |
| n38 |  |  |  |  | 23 | ±2±TT |
| n39 |  |  |  |  | 23 | ±2±TT |
| n40 |  |  |  |  | 23 | ±2±TT |
| n41 |  |  |  |  | 23 | ± 23±TT |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  | 23 | ±2±TT |
| n51 |  |  |  |  | 23 | ±2±TT |
| n53 |  |  |  |  | 23 | ±2±TT |
| n65 |  |  |  |  | 23 | ±2±TT |
| n66 |  |  |  |  | 23 | ±2±TT |
| n70 |  |  |  |  | 23 | ±2±TT |
| n71 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n74 |  |  |  |  | 23 | ±2±TT |
| n77 |  |  |  |  | 23 | +2+TT/-3-TT |
| n78 |  |  |  |  | 23 | +2+TT/-3-TT |
| n79 |  |  |  |  | 23 | +2+TT/-3-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

Table 6.2.1.5-2: Maximum Output Power test requirement for Power Class 2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n34 |  |  | 26 | +2+TT/-3-TT |  |  |
| n39 |  |  | 26 | +2+TT/-3-TT |  |  |
| n41 |  |  | 26 | +2+TT/-33-TT |  |  |
| n77 |  |  | 26 | +2+TT/-3-TT |  |  |
| n78 |  |  | 26 | +2+TT/-3-TT |  |  |
| n79 |  |  | 26 | +2+TT/-3-TT |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

Table 6.2.1.5-2a: Maximum Output Power test requirement for Power Class 1

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n14 | 31 | +2+TT/-3-TT |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

Table 6.2.1.5-2b: Void

Table 6.2.1.5-3: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 to Table 6.2.2.5-9a.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 and Table 6.2.2.5-3.

Table 6.2.2.5-1a: UE Power Class test requirements Band 24 for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 20.5 - TT | （18.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 – TT | （15.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （15.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （15.5 - TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 25.0 + TT | 18.5 - TT | （17.0 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT | （19.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-1: UE Power Class test requirements(for Bands n1, n2, n3, n5, n7, n8, n12, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n65, n66, n70, n74) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | （1.52） | 25.8 | （24.32） | 2.0 |  | 2 | 28.0 + TT | 23.8 - TT | （22.3 - TT2） |
| 2 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 3 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 4 | 23 | -3 | 1.2 | 0 | （1.52） | 24.8 | （23.32） | 2.0 |  | 2 | 28.0 + TT | 22.8 - TT | （21.3 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 2 | 25.0 + TT | 19.5 - TT | （17.5 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | (19.0 - TT2) |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT | （19.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 6 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 7 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2.5 | 25.0 + TT | 19.0 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2a: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 6 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 7 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 8 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 9 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 10 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 11 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 12 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 13 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 16 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 17 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 18 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 19 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 20 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 21 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 22 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 23 | 23 | 0 | 2 | 0 | 21 | 2 | 2.5 | 25.0 + TT | 18.5-TT |
| 24 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 25 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 26 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 27 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 28 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 29 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 30 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 31 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 32 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 33 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 34 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 35 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 36 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 37 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 38 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 39 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-3: UE Power Class test requirements (for Bands n48, n77, n78, n79) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | 25.8 | 2.0 | 3 | 28.0 + TT | 22.8 - TT |
| 2 | 23 | -3 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 - TT |
| 3 | 23 | -3 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 - TT |
| 4 | 23 | -3 | 1.2 | 0 | 24.8 | 2.0 | 3 | 28.0 + TT | 21.8 - TT |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| 6 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 7 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 3 | 25.0 + TT | 18.5 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4: UE Power Class test requirements (for Bands n34, n39, n41, n77, n78, n79) for Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 2.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0- TT2） |
| 4 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 2.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 5 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 2.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 8 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 2.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 9 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 2.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 12 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 2.0 |  | 3 | 28.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0- TT2） |
| 15 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 16 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 17 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 18 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 19 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 2.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 20 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 21 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 22 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 28.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 23 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 2.0 |  | 3 | 28.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 24 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 26 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 28.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 27 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 28 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 29 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 30 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 31 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4a: UE Power Class test requirements for Band n14 for Power Class 1 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| 2 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 3 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 4 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 5 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| 6 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 7 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 8 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 9 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 10 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 11 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 12 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 13 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 14 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 15 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 16 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 17 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 18 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 19 | 31 | 0 | 1.5 | 0 | 29.5 | 2.0 | 3 | 33.0 + TT | 26.5 — TT |
| 20 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 21 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 22 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 23 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 24 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 25 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 26 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 27 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 28 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 29 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 30 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 31 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 32 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 33 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 34 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 35 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4b: Void

Table 6.2.2.5-5: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.2.5-6: UE Power Class test requirements (for Bands n1, n2, n3, n5, n7, n8, n12, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n65, n66, n70, n74) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 23 | 0 | 1.5 | 1.5 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 25.0 + TT | 17.5 - TT | 14.5 - TT2 |
| 15 | 23 | 0 | 1.5 | 1 | 0 | 1.52 | 20.5 | 192 | 2.5 | 3.52 | 2 | 25.0 + TT | 18 - TT | 15.5 - TT2 |
| 26 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 27 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 25.0 + TT | 15.5 - TT | 12.5 - TT2 |
| 34 | 23 | 0 | 2 | 1.5 | 0 | 1.52 | 19.5 | 182 | 3.5 | 42 | 2 | 25.0 + TT | 16 - TT | 14 - TT2 |
| 35 | 23 | 0 | 2 | 1 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 25.0 + TT | 17.5 - TT | 14.5 - TT2 |
| 46 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 47 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 25.0 + TT | 15.5 - TT | 12.5 - TT2 |
| 56 | 23 | 0 | 3.5 | 1.5 | 0 | 1.52 | 18 | 16.52 | 4 | 52 | 2 | 25.0 + TT | 14 - TT | 11.5 - TT2 |
| 57 | 23 | 0 | 3.5 | 1 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 66 | 23 | 0 | 6.5 | 1.5 | 0 | 1.52 | 15 | 13.52 | 5 | 52 | 2 | 25.0 + TT | 10 - TT | 8.5- TT2 |
| 67 | 23 | 0 | 6.5 | 1 | 0 | 1.52 | 15.5 | 142 | 5 | 52 | 2 | 25.0 + TT | 10.5 - TT | 9 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-6a: Void

Table 6.2.2.5-7: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 23 | 0 | 1.5 | 1.5 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 14 | 23 | 0 | 1.5 | 1 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18 - TT |
| 25 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 26 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5 - TT |
| 33 | 23 | 0 | 2 | 1.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16 - TT |
| 34 | 23 | 0 | 2 | 1 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 45 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 46 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5 - TT |
| 55 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 56 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 65 | 23 | 0 | 6.5 | 1.5 | 0 | 15 | 5 | 2.5 | 25.0 + TT | 10 - TT |
| 66 | 23 | 0 | 6.5 | 1 | 0 | 15.5 | 5 | 2.5 | 25.0 + TT | 10.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 4: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 5: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 6: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-7a: Void

Table 6.2.2.5-7b: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 24 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 43 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 44 | 23 | 0 | 3.5 | 1 | 0 | 17.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 53 | 23 | 0 | 4 | 1.5 | 0 | 17.5 | 5 | 2.5 | 25.0 + TT | 12.5 - TT |
| 54 | 23 | 0 | 4 | 1 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 63 | 23 | 0 | 7 | 1.5 | 0 | 14.5 | 5 | 2.5 | 25.0 + TT | 9.5 - TT |
| 64 | 23 | 0 | 7 | 1 | 0 | 15 | 5 | 2.5 | 25.0 + TT | 10 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW=30MHz when SCS=60kHz. NOTE 4: Applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz when SCS=30kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-8: UE Power Class test requirements (for Bands n48, n77, n78, n79) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 23 | 0 | 1.5 | 1.5 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17 - TT |
| 14 | 23 | 0 | 1.5 | 1 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 26 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 3 | 25.0 + TT | 15.5 - TT |
| 33 | 23 | 0 | 2 | 1.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16 - TT |
| 34 | 23 | 0 | 2 | 1 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17 - TT |
| 45 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 46 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 3 | 25.0 + TT | 15.5 - TT |
| 55 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 3 | 25.0 + TT | 14 - TT |
| 56 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 65 | 23 | 0 | 6.5 | 1.5 | 0 | 15 | 5 | 3 | 25.0 + TT | 10 - TT |
| 66 | 23 | 0 | 6.5 | 1 | 0 | 15.5 | 5 | 3 | 25.0 + TT | 10.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 4: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 5: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 6: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-8a: Void

Table 6.2.2.5-9: UE Power Class test requirements (for Bands n34, n39, n41, n77, n78, n79) for Power Class 2 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 26 | 0 | 1.5 | 1.5 | 0 | 1.52 | 23 | 21.52 | 2 | 22 | 3 | 28.0 + TT | 20 - TT | 18 - TT2 |
| 15 | 26 | 0 | 1.5 | 1 | 0 | 1.52 | 23.5 | 222 | 2 | 22 | 3 | 28.0 + TT | 20.5 - TT | 19.0 - TT2 |
| 26 | 26 | 0 | 3 | 1.5 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 28.0 + TT | 18.5 - TT | 17.0 - TT2 |
| 27 | 26 | 0 | 3 | 1 | 0 | 1.52 | 22 | 20.52 | 2 | 2.52 | 3 | 28.0 + TT | 19 - TT | 17.5 - TT2 |
| 34 | 26 | 0 | 2 | 1.5 | 0 | 1.52 | 22.5 | 212 | 2 | 22 | 3 | 28.0 + TT | 19.5 - TT | 18.0 - TT2 |
| 35 | 26 | 0 | 2 | 1 | 0 | 1.52 | 23 | 21.52 | 2 | 22 | 3 | 28.0 + TT | 20 - TT | 18.5 - TT2 |
| 46 | 26 | 0 | 3 | 1.5 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 28.0 + TT | 18.5 - TT | 17.0 - TT2 |
| 47 | 26 | 0 | 3 | 1 | 0 | 1.52 | 22 | 20.52 | 2 | 2.52 | 3 | 28.0 + TT | 19 - TT | 17.5 - TT2 |
| 56 | 26 | 0 | 3.5 | 1.5 | 0 | 1.52 | 21 | 19.52 | 2 | 3.52 | 3 | 28.0 + TT | 18 - TT | 16.0 - TT2 |
| 57 | 26 | 0 | 3.5 | 1 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 28.0 + TT | 18.5 - TT | 17.0 - TT2 |
| 66 | 26 | 0 | 6.5 | 1.5 | 0 | 1.52 | 18 | 16.52 | 4 | 52 | 3 | 28.0 + TT | 14 - TT | 11.5 - TT2 |
| 67 | 26 | 0 | 6.5 | 1 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 3 | 28.0 + TT | 14.5 - TT | 12.0 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 8: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-9a: Void

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2.3.5-1. The allowed A-MPR values specified in table 6.2.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2.1.3-1 apply.

Table 6.2.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.3.5-1: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 2 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 3 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 4 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 5 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 6 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 7 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 8 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 9 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 10 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 11 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 13 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 14 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 16 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 17 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 18 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 19 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 20 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 21 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 23 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 24 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 25 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 26 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-2: UE Power Class 2 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 26 | 3.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 8 | 26 | 3.5 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 9 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 10 | 26 | 0.5 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 11 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 12 | 26 | 0.5 | 0 | 0 | 25.5 | 2.0 | 3 | 28+TT | 22.5-TT |
| 13 | 26 | 3.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 14 | 26 | 3.5 | 4.5 | 0 | 21 | 2.0 | 3 | 28+TT | 18.5-TT |
| 15 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 16 | 26 | 1 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 17 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 18 | 26 | 1 | 0 | 0 | 25 | 2.0 | 3 | 28+TT | 22-TT |
| 19 | 26 | 3.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 20 | 26 | 3.5 | 5 | 0 | 19.5 | 2.0 | 3 | 28+TT | 18-TT |
| 21 | 26 | 3.5 | 0 | 0 | 21 | 2.0 | 3 | 28+TT | 19.5-TT |
| 22 | 26 | 2 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 23 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 24 | 26 | 2 | 0 | 0 | 24 | 2.0 | 3 | 28+TT | 21-TT |
| 25 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 26 | 26 | 3.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 27 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 28 | 26 | 2.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 29 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 30 | 26 | 2.5 | 0 | 0 | 23.5 | 2.0 | 3 | 28+TT | 20.5-TT |
| 31 | 26 | 4.5 | 8 | 0 | 18 | 4.0 | 3 | 28+TT | 14-TT |
| 32 | 26 | 4.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 33 | 26 | 4.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 34 | 26 | 4.5 | 0 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 35 | 26 | 4.5 | 0 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 36 | 26 | 4.5 | 0 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 37 | 26 | 3.5 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 38 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 39 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 40 | 26 | 3 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 41 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 42 | 26 | 3 | 0 | 0 | 23 | 2.0 | 3 | 28+TT | 20-TT |
| 43 | 26 | 3.5 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 44 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 45 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 46 | 26 | 3 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 47 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 48 | 26 | 3 | 0 | 0 | 23 | 2.0 | 3 | 28+TT | 20-TT |
| 49 | 26 | 3.5 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 50 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 51 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 52 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 53 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 54 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 55 | 26 | 6.5 | 10 | 0 | 16 | 5.0 | 3 | 28+TT | 11-TT |
| 56 | 26 | 6.5 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 57 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 58 | 26 | 6.5 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 59 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 60 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-3: UE Power Class 3 test requirements (NS_04) for band n41 (Step 3) and UE Power Class 2 test requirements (NS_04) for band n41 (Step 4)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 3.5 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 2 | 23 | -3 | 3.5 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 3 | 23 | -3 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 4 | 23 | -3 | 1.2 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 5 | 23 | -3 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 6 | 23 | -3 | 1.2 | 0 | 0 | 24.8 | 2.0 | 3 | 28+TT | 21.8-TT |
| 7 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 8 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 9 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 2.0 | 3 | 25+TT | 19.5-TT |
| 10 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 11 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 2.0 | 3 | 25+TT | 19.5-TT |
| 12 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 2.0 | 3 | 25+TT | 19.5-TT |
| 13 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 14 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 15 | 23 | 0 | 1 | 0 | 0 | 22 | 2.0 | 3 | 25+TT | 19-TT |
| 16 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 17 | 23 | 0 | 1 | 0 | 0 | 22 | 2.0 | 3 | 25+TT | 19-TT |
| 18 | 23 | 0 | 1 | 0 | 0 | 22 | 2.0 | 3 | 25+TT | 19-TT |
| 19 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 20 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 21 | 23 | 0 | 2 | 0 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 22 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 23 | 23 | 0 | 2 | 0 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 24 | 23 | 0 | 2 | 0 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 25 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 26 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 27 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 28 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 29 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 30 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 31 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 32 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 33 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 34 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 35 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 36 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 38 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 40 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 41 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 42 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 43 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 44 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 45 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 46 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 47 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 48 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 3 | 25+TT | 17.5-TT |
| 49 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 50 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 51 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 52 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 53 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 54 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 55 | 23 | 0 | 6.5 | 8 | 0 | 15 | 5.0 | 3 | 25+TT | 10-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 57 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 58 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 59 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 60 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-4: UE Power Class 3 test requirements (NS_03 and NS_03U) for band n66, n70

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | NS_03, NS_03U | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | NS_03, NS_03U | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | NS_03, NS_03U | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 9 | NS_03, NS_03U | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10, 11 | NS_03, NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 12 | NS_03, NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 13, 14 | NS_03, NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 15 | NS_03, NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 16, 17 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25, 26 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 27 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-5: UE Power Class 3 test requirements (NS_03 and NS_03U) for band n2, n25

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 1.5 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | NS_03; NS_03U | 23 | 1 | 2 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 6 | NS_03; NS_03U | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | NS_03; NS_03U | 23 | 2 | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 9 | NS_03; NS_03U | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10, 11 | NS_03; NS_03U | 23 | 2.5 | 3.5 | 1.5 | 18 | 4 | 2 | 25+TT | 14-TT |
| 12 | NS_03; NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 13, 14 | NS_03; NS_03U | 23 | 4.5 | 5.5 | 1.5 | 16 | 5 | 2 | 25+TT | 11-TT |
| 15 | NS_03; NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 16, 17 | NS_03; NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 18 | NS_03; NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | NS_03; NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 21 | NS_03; NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | NS_03; NS_03U | 23 | 3.5 | 4.5 | 1.5 | 17 | 5 | 2 | 25+TT | 12-TT |
| 24 | NS_03; NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25, 26 | NS_03; NS_03U | 23 | 6.5 | 7.5 | 1.5 | 14 | 5 | 2 | 25+TT | 9-TT |
| 27 | NS_03; NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-6: UE Power Class 3 test requirements (NS_05) for bands n1, n65

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 2 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 6 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 12 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 16 | 23 | 0 | 1.0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 17 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 21 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 22 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 23 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 24 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 25 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 26 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 27 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 28 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 29 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 30 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 31 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 32 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 33 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 34 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 35 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 36 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 37 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 38 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 40 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 41 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 42 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 43 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 44 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 45 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 46 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 47 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 48 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 49 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 50 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 51 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 52 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 53 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 55 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 56 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 57 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 58 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 59 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 61 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 62 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 63 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 64 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 65 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 66 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 67 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 68 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 69 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 70 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 71 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 72 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 73 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 74 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 75 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 76 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 77 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 78 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 79 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 80 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 81 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 82 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 83 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 84 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 85 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 86 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 87 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 88 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 89 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 90 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 91 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 92 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 93 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 94 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 95 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 96 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 97 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 98 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 99 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 100 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 101 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 102 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 103 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 104 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 105 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 106 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 107 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 108 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 109 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 110 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 111 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 112 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 113 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 114 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 115 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-7: UE Power Class 3 test requirements (NS_05U) for bands n1, n65

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 2 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 6 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 12 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 16 | 23 | 0 | 1.0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 17 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 21 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 25 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 26 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 27 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 28 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 31 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 32 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 33 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 36 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 37 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 40 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 41 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 42 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 43 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 44 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 46 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 47 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 48 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 49 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 50 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 51 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 52 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 53 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 55 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 56 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 57 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 58 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 59 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 61 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 62 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 63 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 64 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 65 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 66 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 67 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 68 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 69 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 70 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 71 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 72 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 73 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 74 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 75 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 76 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 77 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 78 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 79 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 80 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 81 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 82 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 83 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 84 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 85 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 86 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 87 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 88 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 89 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 90 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 91 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 92 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 93 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 94 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 95 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 96 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 97 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 98 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 99 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 100 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 101 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 102 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 103 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 104 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 105 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 106 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 107 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 108 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 109 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 110 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 111 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 112 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 113 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 114 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 115 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 116 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 117 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 118 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 119 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 121 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 122 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 123 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 124 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 125 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 126 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 127 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 128 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 129 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 130 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 131 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-8: UE Power Class 3 test requirements (NS_18)

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 5 | 23 | 1 | 2 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
|  | 10, 15, 20 | 23 | 1 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 3, 4 | 5 | 23 | 2 | 3 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
|  | 10, 15, 20 | 23 | 2 | 6 | 0 | 17 | 5 | 2.5 | 25+TT | 12-TT |
| 5, 6 | 5 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 2.5 | 25+TT | 15.5-TT |
|  | 10, 15, 20 | 23 | 2.5 | 7 | 0 | 16 | 5 | 2.5 | 25+TT | 11-TT |
| 7, 8 | 5 | 23 | 4.5 | 6 | 0 | 17 | 5 | 2.5 | 25+TT | 12-TT |
|  | 10, 15, 20 | 23 | 4.5 | 9 | 0 | 14 | 5 | 2.5 | 25+TT | 9-TT |
| 9, 10 | 5 | 23 | 3 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
|  | 10, 15, 20 | 23 | 3 | 6.5 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 11, 12 | 5 | 23 | 3 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
|  | 10, 15, 20 | 23 | 3 | 7 | 0 | 16 | 5 | 2.5 | 25+TT | 11-TT |
| 13, 14 | 5 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
|  | 10, 15, 20 | 23 | 3.5 | 8.5 | 0 | 14.5 | 5 | 2.5 | 25+TT | 9.5-TT |
| 15, 16 | 5 | 23 | 6.5 | 8.5 | 0 | 14.5 | 5 | 2.5 | 25+TT | 9.5-TT |
|  | 10, 15, 20 | 23 | 6.5 | 11.5 | 0 | 11.5 | 6 | 2.5 | 25+TT | 5.5-TT |
| 17, 18, 19 | 30 | 23 | 5 | 3 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 20 | 30 | 23 | 5 | 8 | 0 | 15 | 5 | 2.5 | 25+TT | 10–TT |
| 21 | 30 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 30 | 23 | 3 | 4.5 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 23 | 30 | 23 | 5 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
| 24, 25, 26 | 30 | 23 | 7 | 4.5 | 0 | 16 | 5 | 2.5 | 25+TT | 11–TT |
| 27 | 30 | 23 | 7 | 9.5 | 0 | 13.5 | 5 | 2.5 | 25+TT | 8.5–TT |
| 28 | 30 | 23 | 3.5 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 29 | 30 | 23 | 4 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
| 30 | 30 | 23 | 7 | 7.5 | 0 | 15.5 | 5 | 2.5 | 25+TT | 10.5-TT |

Table 6.2.3.5-10: UE Power Class 3 test requirements (NS_43)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 2 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 3 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 4 | 23 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 5 | 23 | 1.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 6 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 7 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 8 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 10 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 11 | 23 | 2.5 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 13 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 14 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 15 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 16 | 23 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 17 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 19 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 20 | 23 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 21 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 23 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 24 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 25 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 26 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 27 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 28 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-11: UE Power Class 3 test requirements (NS_43U)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 2 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 4 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 5 | 23 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 7 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 8 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 9 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 11 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 12 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 13 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 14 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 15 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 16 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 17 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 18 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 19 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 20 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 21 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 24 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 25 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 26 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 27 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 28 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 29 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 32 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 33 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 34 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 35 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 36 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-12: UE Power Class 3 test requirements (NS_100) for band n1, n5, n18, n65, n66

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10, 11 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 12 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13, 14 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 16, 17 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 24 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 25, 26 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-13: UE Power Class 3 test requirements (NS_100) for band n2, n3, n8, n25 and n26

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 0.5 | 2 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 3 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | 23 | 1 | 1 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 6 | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | 23 | 2 | 2.5 | 1.5 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 9 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10, 11 | 23 | 2.5 | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 12 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13, 14 | 23 | 4.5 | 4.5 | 1.5 | 17 | 5 | 2 | 25+TT | 12-TT |
| 15 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 16, 17 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 18 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 21 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | 23 | 3.5 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 24 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 25, 26 | 23 | 6.5 | 6.5 | 1.5 | 15 | 5 | 2 | 25+TT | 10-TT |
| 27 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-14: UE Power Class 3 test requirements (NS_37)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 1.0 | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 2 | 23 | 0 | 0.5 | 1.0 | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 3 | 23 | 0 | 0.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 0.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 5 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 6 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 7 | 23 | 0 | 1.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 8 | 23 | 0 | 1.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 9 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 11 | 23 | 0 | 2.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 12 | 23 | 0 | 2.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13 | 23 | 0 | 2.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 14 | 23 | 0 | 2.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 15 | 23 | 0 | 2.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 16 | 23 | 0 | 2.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 17 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 18 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 19 | 23 | 0 | 3.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 20 | 23 | 0 | 3.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 21 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 22 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 23 | 23 | 0 | 3.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 24 | 23 | 0 | 3.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-15: UE Power Class 3 test requirements (NS_38)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 2 | 23 | 0 | 0.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 3 | 23 | 0 | 0.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 4 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 5 | 23 | 0 | 0.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 6 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 1.0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 8 | 23 | 0 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 9 | 23 | 0 | 1.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 10 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 1.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 12 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 2.0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 14 | 23 | 0 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 15 | 23 | 0 | 2.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 16 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 17 | 23 | 0 | 2.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 18 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 19 | 23 | 0 | 2.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 20 | 23 | 0 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 21 | 23 | 0 | 2.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 22 | 23 | 0 | 2.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 23 | 23 | 0 | 2.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 24 | 23 | 0 | 2.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 25 | 23 | 0 | 4.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 26 | 23 | 0 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 27 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 28 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 29 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 30 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 31 | 23 | 0 | 3.0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 32 | 23 | 0 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 33 | 23 | 0 | 3.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 34 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 35 | 23 | 0 | 3.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 36 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 37 | 23 | 0 | 3.0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 38 | 23 | 0 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 39 | 23 | 0 | 3.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 40 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 41 | 23 | 0 | 3.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 42 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 43 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 44 | 23 | 0 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 45 | 23 | 0 | 3.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46 | 23 | 0 | 3.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 47 | 23 | 0 | 3.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 48 | 23 | 0 | 3.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 49 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 50 | 23 | 0 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 51 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 52 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 53 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 54 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-16: UE Power Class 3 test requirements (NS_39)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 2 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 3 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 5 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 6 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 7 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 10 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 11 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 12 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 13 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 15 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 16 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 17 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 20 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 21 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 22 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 24 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 25 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 26 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-17: UE Power Class 3 test requirements (NS_24) for n65

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-3 | 23 | N/A | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 4-6 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 7-15 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 16-TT |
| 16-18 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 19-21 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 22-30 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31-39 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 40-42 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
| 43-51 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 52-60 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 61-63 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
| 64-72 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 73-75 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
| 76-84 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 85-87 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
| 88 | 23 | N/A | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 89 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 90-92 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 16-TT |
| 93 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 94 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 95-97 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 98-100 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 101-102 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 103-105 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 106-108 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 109 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 110-112 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 113 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 114-116 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 117 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 118 | 23 | 4.5 |  | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 119 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 120-122 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | 4.5 |  | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 123 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 124 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 125-127 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | 4.5 |  | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 128-130 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 131-132 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 133-135 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | 4.5 |  | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 136-138 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 139 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 140-142 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 143 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 144-146 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 147 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 148-150 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 151-153 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 154-162 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 16-TT |
| 163-165 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 166-168 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 169-177 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 178-186 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 187-189 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 190-198 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 199-207 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 208-210 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 211-219 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 220-222 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 223-231 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 232-234 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 235 | 23 | 6.5 |  | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 236 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 237-239 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | 6.5 |  | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 240 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 241 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 242-244 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | 6.5 |  | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 245-247 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 248-249 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 250-252 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | 6.5 |  | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 253-255 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 256 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 257-259 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 260 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 261-263 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 264 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 265 | 23 | 1 |  | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 266 | 23 | 2 |  | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 267 | 23 | 1 |  | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-18: UE Power Class 3 test requirements (NS_27)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 2 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 3 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 4 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 5 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 6 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 7 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 9 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 10 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 11 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 12 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 14 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 15 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 16 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 17 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 18 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 19 | 23 | 0 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 20 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 21 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 22 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 23 | 23 | 0 | 0.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 24 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 25 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 26 | 23 | 0 | 0.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 28 | 23 | 0 | 0.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 29 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 30 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 31 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 32 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 33 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 34 | 23 | 0 | 1 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 35 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 36 | 23 | 0 | 1 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 37 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 38 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 39 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 40 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 41 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 42 | 23 | 0 | 1 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 43 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 44 | 23 | 0 | 1 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 45 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 46 | 23 | 0 | 1 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 47 | 23 | 0 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 48 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 49 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 50 | 23 | 0 | 1 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 51 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 52 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 53 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 54 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 55 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 56 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 57 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 58 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 59 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 61 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 62 | 23 | 0 | 2 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 63 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 64 | 23 | 0 | 2 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 65 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 66 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 67 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 68 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 69 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 70 | 23 | 0 | 2 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 71 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 72 | 23 | 0 | 2 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 73 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 74 | 23 | 0 | 2 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 75 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 76 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 77 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 78 | 23 | 0 | 2 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 79 | 23 | 0 | 2 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 80 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 81 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 82 | 23 | 0 | 2 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 83 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 84 | 23 | 0 | 2 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 85 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 86 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 87 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 88 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 89 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 90 | 23 | 0 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 91 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 92 | 23 | 0 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 93 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 94 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 95 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 96 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 97 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 98 | 23 | 0 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 99 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 100 | 23 | 0 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 101 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 102 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 103 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 104 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 105 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 106 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 107 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 108 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 109 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 110 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 111 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 112 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 113 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 114 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 115 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 116 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 117 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 118 | 23 | 0 | 4.5 | 2 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 119 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 23 | 0 | 4.5 | 2 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 121 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 122 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 123 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 124 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 125 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 126 | 23 | 0 | 4.5 | 2 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 127 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 128 | 23 | 0 | 4.5 | 2 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 129 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 130 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 131 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 132 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 133 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 134 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 135 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 136 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 137 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 138 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 139 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 140 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 141 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 142 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 143 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 144 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 145 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 146 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 147 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 148 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 149 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 150 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 151 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 152 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 153 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 154 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 155 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 156 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 157 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 158 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 159 | 23 | 0 | 1.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 160 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 161 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 162 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 163 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 164 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 165 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 166 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 167 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 168 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 169 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 170 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 171 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 172 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 173 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 174 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 175 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 176 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 177 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 178 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 179 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 180 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 181 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 182 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 183 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 184 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 185 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 186 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 187 | 23 | 0 | 2 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 188 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 189 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 190 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 191 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 192 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 193 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 194 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 195 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 196 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 197 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 198 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 199 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 200 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 201 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 202 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 203 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 204 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 205 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 206 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 207 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 208 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 209 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 210 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 211 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 212 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 213 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 214 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 215 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 216 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 217 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 218 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 219 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 220 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 221 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 222 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 223 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 224 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 225 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 226 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 227 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 228 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 229 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 230 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 231 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 232 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 233 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 234 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 235 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 236 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 237 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 238 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 239 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 240 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 241 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 242 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 243 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 244 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 245 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 246 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 247 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 248 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 249 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 250 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 251 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 252 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-19: UE Power Class 3 test requirements (NS_40)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 15.5 | 0 | 7.5 | 7.0 | 2 | 25+TT | 0.5-TT |
| 2 | 23 | 0 | 0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 3 | 23 | 0 | 1 | 15.5 | 0 | 7.5 | 7.0 | 2 | 25+TT | 0.5-TT |
| 4 | 23 | 0 | 2 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 5 | 23 | 0 | 1 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 6 | 23 | 0 | 2 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 7 | 23 | 0 | 2.5 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 8 | 23 | 0 | 2.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 9 | 23 | 0 | 2.5 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 10 | 23 | 0 | 4.5 | 12.5 | 0 | 10.5 | 6.0 | 2 | 25+TT | 4.5-TT |
| 11 | 23 | 0 | 4.5 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 12 | 23 | 0 | 4.5 | 12.5 | 0 | 10.5 | 6.0 | 2 | 25+TT | 4.5-TT |
| 13 | 23 | 0 | 3 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 14 | 23 | 0 | 1.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 15 | 23 | 0 | 3 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 16 | 23 | 0 | 3 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 17 | 23 | 0 | 2 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 3 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 19 | 23 | 0 | 3.5 | 14 | 0 | 9 | 6.0 | 2 | 25+TT | 3-TT |
| 20 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 21 | 23 | 0 | 3.5 | 14 | 0 | 9 | 6.0 | 2 | 25+TT | 3-TT |
| 22 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 23 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 24 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-20: UE Power Class 3 test requirements (NS_41)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 2 | 23 | 0 | 0.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 3 | 23 | 0 | 0.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 4 | 23 | 0 | 0.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 5 | 23 | 0 | 0.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 6 | 23 | 0 | 0.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 7 | 23 | 0 | 0.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 8 | 23 | 0 | 0.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 9 | 23 | 0 | 1 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 10 | 23 | 0 | 1 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 11 | 23 | 0 | 1 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 12 | 23 | 0 | 1 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 13 | 23 | 0 | 1 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 14 | 23 | 0 | 1 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 15 | 23 | 0 | 1 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 16 | 23 | 0 | 1 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 17 | 23 | 0 | 2 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 18 | 23 | 0 | 2 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 19 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 20 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 21 | 23 | 0 | 2 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 22 | 23 | 0 | 2 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 23 | 23 | 0 | 2 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 24 | 23 | 0 | 2 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 25 | 23 | 0 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 26 | 23 | 0 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 27 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 28 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 29 | 23 | 0 | 2.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 30 | 23 | 0 | 2.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 31 | 23 | 0 | 2.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 32 | 23 | 0 | 2.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 33 | 23 | 0 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 34 | 23 | 0 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 35 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 36 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 37 | 23 | 0 | 4.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 38 | 23 | 0 | 4.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 39 | 23 | 0 | 4.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 40 | 23 | 0 | 4.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 41 | 23 | 0 | 3 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 42 | 23 | 0 | 3 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 43 | 23 | 0 | 3 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 44 | 23 | 0 | 3 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 45 | 23 | 0 | 3 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 46 | 23 | 0 | 3 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 47 | 23 | 0 | 3 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 48 | 23 | 0 | 3 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 49 | 23 | 0 | 3 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 50 | 23 | 0 | 3 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 51 | 23 | 0 | 3 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 52 | 23 | 0 | 3 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 53 | 23 | 0 | 3 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 54 | 23 | 0 | 3 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 55 | 23 | 0 | 3 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 56 | 23 | 0 | 3 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 57 | 23 | 0 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 58 | 23 | 0 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 59 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 61 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 62 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 63 | 23 | 0 | 3.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 64 | 23 | 0 | 3.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 65 | 23 | 0 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 66 | 23 | 0 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 67 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 68 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 69 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 70 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 71 | 23 | 0 | 6.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 72 | 23 | 0 | 6.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-21: UE Power Class 3 test requirements (NS_42)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 2 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 3 | 23 | 0 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 4 | 23 | 0 | 0.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 5 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 7 | 23 | 0 | 1 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 8 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 9 | 23 | 0 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 10 | 23 | 0 | 1 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 11 | 23 | 0 | 1 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 13 | 23 | 0 | 2 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 14 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | 0 | 1 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 16 | 23 | 0 | 2 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 17 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 18 | 23 | 0 | 1 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 19 | 23 | 0 | 2.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 20 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 0 | 2.5 | 1.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 22 | 23 | 0 | 2.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 23 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 2.5 | 1.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 25 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 26 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 4.5 | 1.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 29 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 4.5 | 1.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 31 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 32 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 33 | 23 | 0 | 1.5 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 34 | 23 | 0 | 3 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 35 | 23 | 0 | 3 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 1.5 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 37 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 38 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 39 | 23 | 0 | 2 | 1.5 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 40 | 23 | 0 | 3 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 41 | 23 | 0 | 3 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 42 | 23 | 0 | 2 | 1.5 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 43 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 44 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 45 | 23 | 0 | 3.5 | 1.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 46 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 47 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 48 | 23 | 0 | 3.5 | 1.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 49 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 50 | 23 | 0 | 6.5 | 4 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 51 | 23 | 0 | 6.5 | 1.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 53 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 54 | 23 | 0 | 6.5 | 1.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-22: UE Power Class 3 test requirements for NS_47 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 3.5 | 7 | 0 | 19 | 3.5 | 2 | 28+TT | 15.5-TT |
| 2 | 23 | -3 | 0.2 | 5.5 | 0 | 20.5 | 2.5 | 2 | 28+TT | 18-TT |
| 3 | 23 | -3 | 3.5 | 2 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 4 | 23 | -3 | 1.2 | 5.5 | 0 | 20.5 | 2.5 | 2 | 28+TT | 18-TT |
| 5 | 23 | -3 | 1.2 | 3 | 0 | 23 | 2.0 | 2 | 28+TT | 21-TT |
| 6 | 23 | -3 | 1.2 | 3 | 0 | 23 | 2.0 | 2 | 28+TT | 21-TT |
| 7 | 23 | -3 | 1.2 | 5.5 | 0 | 20.5 | 2.5 | 2 | 28+TT | 18-TT |
| 8 | 23 | 0 | 0.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 9 | 23 | 0 | 0.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 10 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 11 | 23 | 0 | 0.5 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 12 | 23 | 0 | 0.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13 | 23 | 0 | 0.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 14 | 23 | 0 | 0.5 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 15 | 23 | 0 | 1.0 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 16 | 23 | 0 | 0.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 17 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 18 | 23 | 0 | 1.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 19 | 23 | 0 | 1.0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 20 | 23 | 0 | 1.0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 21 | 23 | 0 | 1.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 22 | 23 | 0 | 2.0 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 23 | 23 | 0 | 1.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 24 | 23 | 0 | 2.0 | 0 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 25 | 23 | 0 | 2.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 26 | 23 | 0 | 2.0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 27 | 23 | 0 | 2.0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 28 | 23 | 0 | 2.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 29 | 23 | 0 | 2.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 30 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 31 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 32 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 33 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 34 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 35 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 36 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 37 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 39 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 40 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 41 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 42 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 43 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 44 | 23 | 0 | 1.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 45 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 46 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 47 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 48 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 49 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 50 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 51 | 23 | 0 | 2.0 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 52 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 53 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 54 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 55 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 56 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 57 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 58 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 59 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 61 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 62 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 63 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 64 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 65 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 66 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 67 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 68 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 69 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 70 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-22a: UE Power Class 3 test requirements for NS_47 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | MPR & A-MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7 | 1.0 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 2 | 23 | 0 | 3.0 | 7 | 1.0 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 3 | 23 | 0 | 3.5 | 7 | 1.0 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 4 | 23 | 0 | 6.5 | 7 | 1.0 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-23: UE Power Class 2 test requirements for NS_47 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 2 | 26 | 0 | 0.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 3 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 4 | 26 | 0 | 0.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 5 | 26 | 0 | 0.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 6 | 26 | 0 | 0.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 7 | 26 | 0 | 0.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 8 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 9 | 26 | 0 | 0.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 10 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 11 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 12 | 26 | 0 | 1.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 13 | 26 | 0 | 1.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 14 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 15 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 16 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 17 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 18 | 26 | 0 | 2.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 19 | 26 | 0 | 2.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 20 | 26 | 0 | 2.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 21 | 26 | 0 | 2.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 22 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 23 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 24 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 25 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 26 | 26 | 0 | 2.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 27 | 26 | 0 | 2.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 28 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 29 | 26 | 0 | 4.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 30 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 31 | 26 | 0 | 4.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 32 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 33 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 34 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 35 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 36 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 37 | 26 | 0 | 1.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 38 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 39 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 40 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 41 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 42 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 43 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 44 | 26 | 0 | 2.0 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 45 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 46 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 47 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 48 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 49 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 50 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 51 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 52 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 53 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 54 | 26 | 0 | 3.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 55 | 26 | 0 | 3.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 56 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 57 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 58 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 59 | 26 | 0 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 61 | 26 | 0 | 6.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 62 | 26 | 0 | 6.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 63 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-23a: UE Power Class 2 test requirements for NS_47 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | MPR & A-MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 3.0 | 10 | 1.0 | 0 | 16 | 5.0 | 3 | 28+TT | 11-TT |
| 2 | 26 | 0 | 3.0 | 10 | 1.0 | 0 | 16 | 5.0 | 3 | 28+TT | 11-TT |
| 3 | 26 | 0 | 3.5 | 10 | 1.0 | 0 | 16 | 5.0 | 3 | 28+TT | 11-TT |
| 4 | 26 | 0 | 6.5 | 10 | 1.0 | 0 | 16 | 5.0 | 3 | 28+TT | 11-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-24: UE Power Class 3 test requirements for NS_48 (PC3 contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 6 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 7 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 12 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 16 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 17 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 21 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 22 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 23 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 26 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 30 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 31 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 32 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 33 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 36 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 37 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 38 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 39 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 40 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 41 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 42 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 43 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 44 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 46 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 47 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 48 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 49 | 23 | 0 | 1.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 50 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 51 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 52 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 53 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 54 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 55 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 56 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 57 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 58 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 59 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 60 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 61 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 62 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 63 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 64 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 65 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 66 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 67 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 68 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 69 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 70 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 71 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 72 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 73 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 74 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 75 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 76 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 77 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 78 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 79 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 80 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-25: UE Power Class 3 test requirements (NS_12) for n26

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-3 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 5.5 | 1.5 | 16 | 5 | 2 | 25+TT | 11-TT |
|  | 23 | N/A | 9.5 | 1.5 | 12 | 6 | 2 | 25+TT | 6-TT |
| 4-6 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
|  | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 7-9 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 5.5 | 1.5 | 16 | 5 | 2 | 25+TT | 11-TT |
|  | 23 | N/A | 9.5 | 1.5 | 12 | 6 | 2 | 25+TT | 6-TT |
| 10-12 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
|  | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 13 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 14 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 15-16 | 23 | N/A | 7 | 1.5 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
|  | 23 | N/A | 9.5 | 1.5 | 12 | 6 | 2 | 25+TT | 6-TT |
| 17-18 | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
|  | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 19-20 | 23 | N/A | 7 | 1.5 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
|  | 23 | N/A | 9.5 | 1.5 | 12 | 6 | 2 | 25+TT | 6-TT |
| 21-22 | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
|  | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 23 | 23 | 1.5 | 0 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 24 | 23 | 1.5 | 0 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-26: UE Power Class 3 test requirements (NS_13) for n26

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-3 | 23 | N/A | 3.5 | 1.5 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4.5 | 1.5 | 17 | 5 | 2 | 25+TT | 12-TT |
|  | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 4-6 | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 7-9 | 23 | N/A | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
|  | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
|  | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 10 | 23 | N/A | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 11 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 12-14 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 6 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
|  | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 15-16 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 17-19 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
|  | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 20 | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 21 | 23 | 3 | N/A | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-27a: UE Power Class 3 test requirements (NS_14) for n26 low range

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 2 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | N/A | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 4 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 5-6 | 23 | N/A | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
|  | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 7 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 8 | 23 | 0 | N/A | 1.5 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 9 | 23 | 0 | N/A | 1.5 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 10 | 23 | 0 | N/A | 1.5 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 11-13 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 6 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
|  | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 14 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15-17 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 6 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
|  | 23 | N/A | 7 | 1.5 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 18 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19-21 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 6 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
|  | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 22 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 1.5 | N/A | 1.5 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 24 | 23 | 1.5 | N/A | 1.5 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 25 | 23 | 1.5 | N/A | 1.5 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-27b: UE Power Class 3 test requirements (NS_14) for n26 high range

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | N/A | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 5-6 | 23 | N/A | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 7 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 8 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 9 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 10 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 11-13 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
|  | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 14 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15-17 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
|  | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 18 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19-21 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
|  | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 22 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 1.5 | N/A | 1.5 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 24 | 23 | 1.5 | N/A | 1.5 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 25 | 23 | 1.5 | N/A | 1.5 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-28: UE Power Class 3 test requirements (NS_15) for n26

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 9 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 2 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 3 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 4-5 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 6-7 | 23 | N/A | 9 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
|  | 23 | N/A | 13.5 | 0 | 8 | 6 | 2 | 25+TT | 2-TT |
| 8 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 9 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 10-11 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 12-13 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 14 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 15 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 16-17 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 18-19 | 23 | N/A | 9 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
|  | 23 | N/A | 13.5 | 0 | 8 | 6 | 2 | 25+TT | 2-TT |
| 20 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 21 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 22-23 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 24-25 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
|  | 23 | N/A | 13.5 | 0 | 9.5 | 6 | 2 | 25+TT | 3.5-TT |
| 26 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 27 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 28-29 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 30-31 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 32 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 33 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 34-35 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 36-37 | 23 | N/A | 9 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
|  | 23 | N/A | 13.5 | 0 | 8 | 6 | 2 | 25+TT | 2-TT |
| 38 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 39 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 40-41 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 42-43 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
|  | 23 | N/A | 13.5 | 0 | 9.5 | 6 | 2 | 25+TT | 3.5-TT |
| 44 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 45 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46-47 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 48 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 49 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 50 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 51 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 52 | 23 | N/A | 10.5 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 53 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 54 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 55-56 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 57-58 | 23 | N/A | 10.5 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
|  | 23 | N/A | 13.5 | 0 | 8 | 6 | 2 | 25+TT | 2-TT |
| 59 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 60 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 61-62 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 63-64 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
|  | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 65 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 66 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 67-68 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 69-70 | 23 | N/A | 10.5 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
|  | 23 | N/A | 13.5 | 0 | 8 | 6 | 2 | 25+TT | 2-TT |
| 71 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 72 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 73-74 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 75-76 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
|  | 23 | N/A | 13.5 | 0 | 9.5 | 6 | 2 | 25+TT | 3.5-TT |
| 77 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 78 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 79-80 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 81-82 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
|  | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 83 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 84 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 85-86 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 87-88 | 23 | N/A | 10.5 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
|  | 23 | N/A | 13.5 | 0 | 8 | 6 | 2 | 25+TT | 2-TT |
| 89 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 90 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 91-92 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 93-94 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
|  | 23 | N/A | 13.5 | 0 | 9.5 | 6 | 2 | 25+TT | 3.5-TT |
| 95 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 96 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 97-98 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 99 | 23 | 3 | N/A | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 100 | 23 | 3 | N/A | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 101 | 23 | 1.5 | N/A | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 102 | 23 | 1.5 | N/A | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-29: UE Power Class 3 test requirements for NS_45 (contiguous allocation)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 3 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 4, 5 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10, 11 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 12 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-30: UE Power Class 3 test requirements for NS_46 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 2 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 3 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 4 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 2 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 0 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 7 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 9 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 10 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 11 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 12 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 13 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 14 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 16 | 23 | 0 | 3.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 17 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 18 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 19 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 20 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 21 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 22 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-31: UE Power Class 3 test requirements (NS_44) for band n38

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 4 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 5 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 6 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 7 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 8 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 9 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 10 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 11 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 12 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 13 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 14 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 15 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 16 | 23 | 0 | 2.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 17 | 23 | 0 | 2.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 18 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 19 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 20 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 21 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 22 | 23 | 0 | 4.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 23 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 25 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 26 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 29 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 31 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 32 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 33 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 34 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 35 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 36 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 37 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 38 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 39 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 40 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 41 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 42 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 43 | 23 | 0 | 6.5 | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 44 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-32: UE Power Class 3 test requirements (NS_21)

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) |  | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 5 | 23 | 0.5 | 0 | 0 |  | 22.5 | 2 | 2 | 25+TT | 20.5-TT |
|  | 10 | 23 | 0.5 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 3 | 5 | 23 | 0.5 | 0 | 0 |  | 22.5 | 2 | 2 | 25+TT | 20.5-TT |
|  | 10 | 23 | 0.5 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 4, 5 | 10 | 23 | 3 | 3 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 6, 7 | 5 | 23 | 1 | 0 | 0 |  | 22 | 2 | 2 | 25+TT | 20-TT |
|  | 10 | 23 | 1 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 5 | 23 | 1 | 0 | 0 |  | 22 | 2 | 2 | 25+TT | 20-TT |
|  | 10 | 23 | 1 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 9, 10 | 10 | 23 | 1 | 3 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 11, 12 | 5 | 23 | 2 | 0 | 0 |  | 21 | 2 | 2 | 25+TT | 19-TT |
|  | 10 | 23 | 2 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 13 | 5 | 23 | 2 | 0 | 0 |  | 21 | 2 | 2 | 25+TT | 19-TT |
|  | 10 | 23 | 2 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 14, 15 | 10 | 23 | 2 | 3 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 16, 17 | 5 | 23 | 2.5 | 0 | 0 |  | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
|  | 10 | 23 | 2.5 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 18 | 5 | 23 | 2.5 | 0 | 0 |  | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
|  | 10 | 23 | 2.5 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | 10 | 23 | 2.5 | 3 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 21, 22 | 5 | 23 | 4.5 | 0 | 0 |  | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 4.5 | 6 | 0 |  | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 5 | 23 | 4.5 | 0 | 0 |  | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 4.5 | 4 | 0 |  | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 24, 25 | 10 | 23 | 4.5 | 3 | 0 |  | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 26, 27 | 5 | 23 | 3 | 0 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 28 | 5 | 23 | 3 | 0 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 29, 30 | 10 | 23 | 3 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31, 32 | 5 | 23 | 3 | 0 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 33 | 5 | 23 | 3 | 0 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 34, 35 | 10 | 23 | 3 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 36, 37 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
|  | 10 | 23 | 3.5 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 38 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
|  | 10 | 23 | 3.5 | 5.5 | 0 |  | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 39, 40 | 10 | 23 | 3.5 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 41, 42 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 6 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 43 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 44, 45 | 10 | 23 | 6.5 | 5.5 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 4 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-33: UE Power Class 3 test requirements (NS_49)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 5 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 7 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 10 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 16 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 17 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 18 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 19 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 20 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 21 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 23 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 26 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 27 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 29 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 30 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 31 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 32 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 33 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 34 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 35 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 37 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 38 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 40 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 41 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 42 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 43 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 44 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 45 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 46 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 47 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 48 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 49 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 50 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 51 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 52 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 53 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 55 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 56 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 57 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 58 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 59 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 61 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 62 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 63 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 64 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 65 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 66 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 67 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 68 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 69 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 70 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 71 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 72 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 73 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 74 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 75 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 76 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 77 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 78 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 79 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 80 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 81 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 82 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 83 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 84 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-34: UE Power Class 3 test requirements (NS_06) for band n12 and n14

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20.5-TT |
| 2 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20.5-TT |
| 3 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20.5-TT |
| 4 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 20-TT |
| 5 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 20-TT |
| 6 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 20-TT |
| 7 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 19-TT |
| 8 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 19-TT |
| 9 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 19-TT |
| 10 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 11 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 13 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 14 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 16 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 17 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 18 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 19 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 20 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 21 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 23 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 24 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 25 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 26 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-35: UE Power Class 1 test requirements (NS_06) for band n14

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 2 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 3 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 4 | 31 | 1 | 0 | 0 | 30 | 2 | 3 | 33+TT | 27-TT |
| 5 | 31 | 1 | 0 | 0 | 30 | 2 | 3 | 33+TT | 27-TT |
| 6 | 31 | 1 | 0 | 0 | 30 | 2 | 3 | 33+TT | 27-TT |
| 7 | 31 | 2 | 0 | 0 | 29 | 2 | 3 | 33+TT | 26-TT |
| 8 | 31 | 2 | 0 | 0 | 29 | 2 | 3 | 33+TT | 26-TT |
| 9 | 31 | 2 | 0 | 0 | 29 | 2 | 3 | 33+TT | 26-TT |
| 10 | 31 | 2.5 | 0 | 0 | 28.5 | 2 | 3 | 33+TT | 25.5-TT |
| 11 | 31 | 2.5 | 0 | 0 | 28.5 | 2 | 3 | 33+TT | 25.5-TT |
| 12 | 31 | 2.5 | 0 | 0 | 28.5 | 2 | 3 | 33+TT | 25.5-TT |
| 13 | 31 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 23.5-TT |
| 14 | 31 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 23.5-TT |
| 15 | 31 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 23.5-TT |
| 16 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 17 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 18 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 19 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 20 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 21 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 22 | 31 | 3.5 | 0 | 0 | 27.5 | 2 | 3 | 33+TT | 24.5-TT |
| 23 | 31 | 3.5 | 0 | 0 | 27.5 | 2 | 3 | 33+TT | 24.5-TT |
| 24 | 31 | 3.5 | 0 | 0 | 27.5 | 2 | 3 | 33+TT | 24.5-TT |
| 25 | 31 | 6.5 | 0 | 0 | 24.5 | 2 | 3 | 33+TT | 21.5-TT |
| 26 | 31 | 6.5 | 0 | 0 | 24.5 | 2 | 3 | 33+TT | 21.5-TT |
| 27 | 31 | 6.5 | 0 | 0 | 24.5 | 2 | 3 | 33+TT | 21.5-TT |
| 28 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 29 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 30 | 31 | 0 | 0 | 0 | 31 | 2 | 3 | 33+TT | 28-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-36: UE Power Class 3 test requirements (NS_56) for band n24

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 2 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 3 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 5 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 6 | 23 | 1 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 7 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 8 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 9 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 10 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 11 | 23 | 2 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 12 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 13 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 14 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 15 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 16 | 23 | 2.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 17 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 18 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 19 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 20 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 21 | 23 | 4.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 22 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 23 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 24 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 25 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 26 | 23 | 0.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 27 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 28 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 29 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 30 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 31 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 32 | 23 | 0.5 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 33 | 23 | 0.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 34 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 35 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 36 | 23 | 0.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 37 | 23 | 1 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 38 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 39 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 40 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 41 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 42 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 43 | 23 | 1 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 44 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 45 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 46 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 47 | 23 | 1 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 48 | 23 | 2 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 49 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 50 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 51 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 52 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 53 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 54 | 23 | 2 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 55 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 56 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 57 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 58 | 23 | 2 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 59 | 23 | 2.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 61 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 62 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 63 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 64 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 65 | 23 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 66 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 67 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 68 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 69 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 70 | 23 | 4.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 71 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 72 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 73 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 74 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 75 | 23 | 4.5 | 4 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 76 | 23 | 4.5 | 2 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 77 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 78 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 79 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 80 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 81 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 82 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 83 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 84 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 85 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 86 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 87 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 88 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 89 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 90 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 91 | 23 | 3.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 92 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 93 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 94 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 95 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 96 | 23 | 6.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 97 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 98 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 99 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 100 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 101 | 23 | 3 | 12 | 0 | 9 | 6 | 3 | 25+TT | 5-TT |
| 102 | 23 | 3 | 8 | 0 | 11 | 5 | 3 | 25+TT | 10-TT |
| 103 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 104 | 23 | 1.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 105 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 106 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 107 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 108 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 109 | 23 | 1.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 110 | 23 | 1.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 111 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 112 | 23 | 3 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 113 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 114 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 115 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 116 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 117 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 118 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 119 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 121 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 122 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 123 | 23 | 3.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 124 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 125 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 126 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 127 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 128 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 129 | 23 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 130 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 131 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 132 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 133 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 134 | 23 | 6.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 135 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 136 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 137 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 138 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 139 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 140 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 141 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 142 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 143 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 144 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2.4.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2.4.5-1.

Table 6.2.4.5-1: PCMAX configured UE output power

|  | Maximum output power |  |
| --- | --- | --- |
|  | Test ID 1,2 | Test ID 3 |
| Measured UE output power test point 1 | -10 dBm ± (7+TT) | -10 dBm +7+TT/-7.2-TT |
| Measured UE output power test point 2 | 10 dBm ± (6+TT) | 10 dBm +6+TT/-6.2-TT |
| Measured UE output power test point 3 | 15 dBm ± (5+TT) | 15 dBm +5+TT/-5.2-TT |
| Measured UE output power test point 4 | Note 3 | Note 4 |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. Note 2: Power class 3 is default power class unless otherwise stated. Note 3: The maximum output power shall be within the range in Table 6.2.4.5-1a. Note 4: The maximum output power shall be within the range in Table 6.2.4.5-1b. |  |  |

Table 6.2.4.5-1a: Measured UE output power test point 4 for Test ID 1,2

| NR band | Tolerance (dB) |
| --- | --- |
| n1 | 20 dBm ±(2.5+TT) |
| n2 | 20 dBm ±(2.51+TT) |
| n3 | 20 dBm ±(2.51+TT) |
| n5 | 20 dBm ±2.5±TT |
| n7 | 20 dBm ±(2.51+TT) |
| n8 | 20 dBm ±(2.51+TT) |
| n12 | 20 dBm ±(2.51+TT) |
| n14 | 20 dBm ±(2.5+TT) |
| n20 | 20 dBm ±(2.51+TT) |
| n24 | 20 dBm + 2.5+TT/-31-TT |
| n25 | 20 dBm ±(2.5+TT) |
| n26 | 20 dBm ±(2.51+TT) |
| n28 | 20 dBm ±(2.5+TT) |
| n30 | 20 dBm ±(2.5+TT) |
| n34 | 20 dBm ±(2.5+TT) |
| n38 | 20 dBm ±(2.5+TT) |
| n39 | 20 dBm ±(2.5+TT) |
| n40 | 20 dBm ±(2.5+TT) |
| n41 | 20 dBm ±(2.51+TT) |
| n50 | 20 dBm ±(2.5+TT) |
| n51 | 20 dBm ±(2.5+TT) |
| n65 | 20 dBm ±(2.5+TT) |
| n66 | 20 dBm ±(2.5+TT) |
| n70 | 20 dBm ±(2.5+TT) |
| n71 | 20 dBm ±(2.5+TT) |
| n74 | 20 dBm ±(2.5+TT) |
| n77 | 20 dBm + 2.5+TT/-3-TT |
| n78 | 20 dBm + 2.5+TT/-3-TT |
| n79 | 20 dBm + 2.5+TT/-3-TT |
| NOTE 1: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.0 dB. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. |  |

Table 6.2.4.5-1b: Measured UE output power test point 4 for Test ID 3

| NR band | Tolerance (dB) |
| --- | --- |
| n40 | 23 dBm +2+TT/-2.2-TT |
| n41 | 23 dBm +2+TT/-2.2-TT |
| n77 | 23 dBm +2+TT/-3.2-TT |
| n78 | 23 dBm +2+TT/-3.2-TT |
| n79 | 23 dBm +2+TT/-3.2-TT |
| NOTE 1: Void NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. |  |

Table 6.2.4.5-2: Test Tolerance (Configured transmitted power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in clause 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clauses 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and clause 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clauses 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and clause 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2A.1.1.5	Test requirement

The maximum output power for CA, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.1.1.5-1 for Inter-band 2 UL CA configuration. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.1.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n8A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n5A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n48A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n5A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n7A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n7A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n8A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n24A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n24A-n48A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n24A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n26A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n26A-n70A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n39A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n40A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n41A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n48A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n48A-n70A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n48A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n50A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n66A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n66A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| NOTE 1: Void. NOTE 2: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1.3-1 is allowed to reduce the lower tolerance limit by 1.5dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The TT for 2UL CA Maximum Output Power is in the maximum TT among all UL CCs. For TT of each UL CC refers to Table 6.2A.1.1.5-2. |  |  |  |  |  |  |  |  |

Table 6.2A.1.1.5-1a: Maximum Output Power test requirement for Power Class except class 3

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| CA_n3A-n41A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| CA_n3A-n78A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| CA_n28A-n41A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| CA_n28A-n79A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| CA_n40A-n41A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| NOTE 1: Void. NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The TT for 2UL CA Maximum Output Power is in the maximum TT among all UL CCs. For TT of each UL CC refer to Table6.2A.1.1.5-2. NOTE 7: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. |  |  |  |  |  |  |  |  |

Table 6.2A.1.1.5-2: Test Tolerance (UE maximum output power for CA)

| TT for overall output power (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |
|  |  |  | BW ≤ 40MHz |  | 40MHz < BW ≤ 100MHz |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
|  | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 |
| SCell |  | 3.0GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2A.2.1.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.2.1.5-1 to Table 6.2A.2.1.5-3.

Table 6.2A.2.1.5-1: UE Output Power for inter-band CA (2 UL CA) test requirements

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0.2 | 0.2 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 3 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 4 | 23 | 1.2 | 1.2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 5 | 23 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 6 | 23 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 7 | 23 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 8 | 23 | 0.5 | 0.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 9 | 23 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 10 | 23 | 1 | 1 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 11 | 23 | 1 | 1 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 12 | 23 | 1 | 1 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 13 | 23 | 1 | 1 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 14 | 23 | 2 | 2 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 15 | 23 | 2 | 2 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 16 | 23 | 2 | 2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 17 | 23 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 18 | 23 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 19 | 23 | 2.5 | 2.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 20 | 23 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 202 | 62 | 25+TT | 14-TT2 |
| 21 | 23 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 202 | 62 | 25+TT | 14-TT2 |
| 22 | 23 | 4.5 | 4.5 | 0 | 0 | 3 | 21.5 | 5 | 25+TT | 16.5-TT |
| 23 | 23 | 1.5 | 1.5 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 24 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 25 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 26 | 23 | 3 | 3 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 27 | 23 | 2 | 2 | 0 (1.52) | 0 (1.52) | 3 | 23 (22.52) | 3 (52) | 25+TT | 20-TT (17.5-TT2) |
| 28 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 29 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 30 | 23 | 3 | 3 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 31 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 32 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 33 | 23 | 3.5 | 3.5 | 0 | 0 | 3 | 22.5 | 5 2 | 25+TT | 17.5-TT |
| 34 | 23 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 182 | 52 | 25+TT | 13-TT2 |
| 35 | 23 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 182 | 52 | 25+TT | 13-TT2 |
| 36 | 23 | 6.5 | 6.5 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 6.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1a: UE Output Power for intra-band contiguous 2 UL CA (contiguous RB allocation) test requirements (for CA_n41C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL=2 | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 2 | 2 | 2.5 | 25+TT | 20-TT | 25+TT | 18-TT |
| 2 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 3 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 2 | 2 | 2.5 | 25+TT | 20-TT | 25+TT | 18-TT |
| 4 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 5 | 23 | 1.5 | 2.5 | 0 | 21.5 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19.5-TT | 25+TT | 18-TT |
| 6 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 7 | 23 | 3 | 5 | 0 | 20 | 18 | 2 | 2.5 | 4 | 25+TT | 17.5-TT | 25+TT | 14-TT |
| 8 | 23 | 4 | 7 | 0 | 19 | 16 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 11-TT |
| 9 | 23 | 5.5 | 7 | 0 | 17.5 | 16 | 2 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 11-TT |
| 10 | 23 | 6 | 7.5 | 0 | 17 | 15.5 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 10.5-TT |
| 11 | 23 | 2 | 3.5 | 0 | 21 | 19.5 | 2 | 2 | 3.5 | 25+TT | 19-TT | 25+TT | 16-TT |
| 12 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 13 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 14 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 15 | 23 | 3.5 | 5 | 0 | 19.5 | 18 | 2 | 3.5 | 4 | 25+TT | 16-TT | 25+TT | 14-TT |
| 16 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 17 | 23 | 6.5 | 7 | 0 | 16.5 | 16 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11-TT |
| 18 | 23 | 6.5 | 8 | 0 | 16.5 | 15 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 10-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1b: UE Output Power for intra-band contiguous 2 UL CA (contiguous RB allocation) test requirements (for CA_n48B, CA_n77C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL=2 | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 3 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 17.5-TT |
| 2 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 3 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 3 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 17.5-TT |
| 4 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 5 | 23 | 1.5 | 2.5 | 0 | 21.5 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18.5-TT | 25+TT | 17.5-TT |
| 6 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 7 | 23 | 3 | 5 | 0 | 20 | 18 | 3 | 2.5 | 4 | 25+TT | 17-TT | 25+TT | 14-TT |
| 8 | 23 | 4 | 7 | 0 | 19 | 16 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 11-TT |
| 9 | 23 | 5.5 | 7 | 0 | 17.5 | 16 | 3 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 11-TT |
| 10 | 23 | 6 | 7.5 | 0 | 17 | 15.5 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 10.5-TT |
| 11 | 23 | 2 | 3.5 | 0 | 21 | 19.5 | 3 | 2 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 12 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 13 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 14 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 15 | 23 | 3.5 | 5 | 0 | 19.5 | 18 | 3 | 3.5 | 4 | 25+TT | 16-TT | 25+TT | 14-TT |
| 16 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 17 | 23 | 6.5 | 7 | 0 | 16.5 | 16 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11-TT |
| 18 | 23 | 6.5 | 8 | 0 | 16.5 | 15 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 10-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1c: UE Output Power for intra-band contiguous 2 UL CA (non-contiguous RB allocation) test requirements (for CA_n41C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 18-TT |
| 2 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 2 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 3 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 4 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 18-TT |
| 5 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 2 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 6 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 7 | 23 | 2.5 | 3 | 0 | 20.5 | 20 | 2 | 2.5 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 8 | 23 | 5.5 | 6 | 0 | 17.5 | 17 | 2 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 12-TT |
| 9 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 10 | 23 | 4.5 | 5 | 0 | 18.5 | 18 | 2 | 4 | 4 | 25+TT | 14.5-TT | 25+TT | 14-TT |
| 11 | 23 | 6 | 6 | 0 | 17 | 17 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 12-TT |
| 12 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 13 | 23 | 6 | 6.5 | 0 | 17 | 16.5 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 11.5-TT |
| 14 | 23 | 6.5 | 6.5 | 0 | 16.5 | 16.5 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11.5-TT |
| 15 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 16 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 17 | 23 | 6.5 (4.5)1 | 7 (5)1 | 0 | 16.5 (18.5)1 | 16 (18)1 | 2 | 5 (4)1 | 5 (4)1 | 25+TT | 11.5-TT (14.5-TT)1 | 25+TT | 11-TT (14-TT)1 |
| 18 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 19 | 23 | 3 | 3.5 | 0 | 20 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 20 | 23 | 7 | 7 | 0 | 16 | 16 | 2 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 21 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 22 | 23 | 5 | 5 | 0 | 18 | 18 | 2 | 4 | 4 | 25+TT | 14-TT | 25+TT | 14-TT |
| 23 | 23 | 7 | 7 | 0 | 16 | 16 | 2 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 24 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 25 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 2 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 26 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 2 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 27 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1d: UE Output Power for intra-band contiguous 2 UL CA (non-contiguous RB allocation) test requirements (for CA_n48B, CA_n77C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 2 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 3 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 3 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 4 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 5 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 3 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 6 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 7 | 23 | 2.5 | 3 | 0 | 20.5 | 20 | 3 | 2.5 | 2.5 | 25+TT | 17.5-TT | 25+TT | 17-TT |
| 8 | 23 | 5.5 | 6 | 0 | 17.5 | 17 | 3 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 12-TT |
| 9 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 10 | 23 | 4.5 | 5 | 0 | 18.5 | 18 | 3 | 4 | 4 | 25+TT | 14.5-TT | 25+TT | 14-TT |
| 11 | 23 | 6 | 6 | 0 | 17 | 17 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 12-TT |
| 12 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 13 | 23 | 6 | 6.5 | 0 | 17 | 16.5 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 11.5-TT |
| 14 | 23 | 6.5 | 6.5 | 0 | 16.5 | 16.5 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11.5-TT |
| 15 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 16 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 17 | 23 | 6.5 (4.5)1 | 7 (5)1 | 0 | 16.5 (18.5)1 | 16 (18)1 | 3 | 5 (4)1 | 5 (4)1 | 25+TT | 11.5-TT (14.5-TT)1 | 25+TT | 11-TT (14-TT)1 |
| 18 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 19 | 23 | 3 | 3.5 | 0 | 20 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17-TT | 25+TT | 16-TT |
| 20 | 23 | 7 | 7 | 0 | 16 | 16 | 3 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 21 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 22 | 23 | 5 | 5 | 0 | 18 | 18 | 3 | 4 | 4 | 25+TT | 14-TT | 25+TT | 14-TT |
| 23 | 23 | 7 | 7 | 0 | 16 | 16 | 3 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 24 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 25 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 3 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 26 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 3 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 27 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1e: UE Output Power for intra-band non-contiguous 2 UL CA test requirements (MPRIM3 to meet -13dBm/MHz)

| Test ID | PPowerClass，CA (dBm) | MPR (dB) | ΔTC (dB) | PCMAX_L (dBm) | T(PCMAX_L) (dB) | TL (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 3 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 4 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 5 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 6 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 7,8 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 9,10 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 11 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 12 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 13 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 14 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 15,16 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 17,18 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 19 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 20 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 21 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 22 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 23,24 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 25,26 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 27 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 28 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 29 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 30 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 31,32 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 33,34 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 35 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 36 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 37 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 38 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 39,40 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 41,42 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 43 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 44 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 45 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 46 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 47,48 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 49,50 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 51 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 52 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 53 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 54 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 55,56 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 57,58 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 59 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 60 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 61 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 62 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 63,64 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 65,66 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 67 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 68 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 69 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 70 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 71,72 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |

Table 6.2A.2.1.5-1f: UE Output Power for intra-band non-contiguous 2 UL CA test requirements (MPRIM3 to meet - 30dBm/MHz)

| Test ID | PPowerClass，CA (dBm) | MPR (dB) | ΔTC (dB) | PCMAX_L (dBm) | T(PCMAX_L) (dB) | TL (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 3 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 4 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 5 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 6,7 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 8,9 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 10 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 11 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 12 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 13,14 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 15,16 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 17 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 18 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 19 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 20,21 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 22,23 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 24 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 25 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 26 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 27,28 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 29,30 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 31 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 32 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 33 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 34,35 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 36,37 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 38 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 39 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 40 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 41,42 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 43,44 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 45 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 46 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 47 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 48,49 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 50,51 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 52 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 53 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 54 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 55,56 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 57,58 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 59 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 60 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 61 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 62,63 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |

Table 6.2A.2.1.5-2: Test Tolerance for inter-band CA (Maximum Power Reduction (MPR))

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell |  | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | BW ≤ 40MHz | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.2.1.5-3: Test Tolerance for intra-band CA (Maximum Power Reduction (MPR))

| TT for overall output power (dB) |  |  |
| --- | --- | --- |
| Aggregation BW | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |
| 100MHz < BW ≤ 200MHz | FFS | FFS |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2A.3.1.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.3.1.5-1, Table 6.2A.3.1.5-2, Table 6.2A.3.1.5-3 as appropriate. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in 6.2A.4.0 apply. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.3.1.5-0: Test Tolerance (Additional Maximum Power Reduction (A-MPR))

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.3.1.5-1: UE Power Class 3 test requirement (network signalling value NS_100/NS_01) for CA_n3-n78 and CA_n8-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 23 (22.72) | 3 (52) | 25+TT | 20-TT (17.7-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-2: UE Power Class 3 test requirement (network signalling value NS_43/NS_01, NS_43U/NS_01) for CA_n8-n78

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 9.0 | 0 | 0 | 0 | 3 | 18.4 | 5 | 25+TT | 13.4-TT |
| 3 | 23 | 6.5 | 0 | 9.0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3: UE Power Class 3 test requirement (network signalling value NS_47/NS_01) for CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20+TT |
| 2 | 23 | 6.5 | 6.5 | 7 | 0 | 0 | 0 | 3 | 9.8 | 7 | 25+TT | 2.8+TT |
| 3 | 23 | 6.5 | 0 | 7 | 0 | 0 | 0 | 3 | 20 | 6 | 25+TT | 14+TT |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2A.4.1.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2A.4.1.5-1 to Table 6.2A.4.1.5-3.

Table 6.2A.4.1.5-1: Configured UE Output Power for Inter-band CA

|  | Maximum output power |  |
| --- | --- | --- |
|  | Lower limit | Upper limit |
| Measured UE output power test point 1 | -14 dBm - TT | 0 dBm + TT |
| Measured UE output power test point 2 | 7 dBm - TT | 19 dBm + TT |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.4.1.5-2. |  |  |

Table 6.2A.4.1.5-1a: Configured UE Output Power for Intra-band contiguous CA (contiguous RB allocation)

|  | Maximum output power |  |
| --- | --- | --- |
|  | Lower limit | Upper limit |
| Measured UE output power test point 1 | -14 dBm - TT | 0 dBm + TT |
| Measured UE output power test point 2 | 8 dBm - TT | 18 dBm + TT |
| Measured UE output power test point 3 | 2 dBm - TT | 14 dBm + TT |
| Measured UE output power test point 4 | 14 dBm - TT | 22 dBm + TT |
| Note 1: TT for each intra-band UL CA configuration is in table 6.2A.4.1.5-3. |  |  |

Table 6.2A.4.1.5-2: Test Tolerance for inter-band CA (Configured transmitted power for CA)

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.4.1.5-3: Test Tolerance for intra-band CA (Configured transmitted power for CA)

| TT for overall output power (dB) |  |  |
| --- | --- | --- |
| Aggregation BW | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |
| 100MHz < BW ≤ 200MHz | FFS | FFS |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2B.1.1.5	Test requirement

The maximum output power for NR-DC, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2B.1.1.5-1 for NR-DC configuration.

Table 6.2B.1.1.5-1: Maximum Output Power test requirement for NR-DC Power Class 3

| Uplink DC Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_n48A-n70A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1.3-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass is the maximum UE power specified without account of the tolerance NOTE 3: The maximum power requirement applies to the total transmitted power over both the MCG and SCG. NOTE 4: Power class 3 is the default power class unless otherwise stated. NOTE 5: The TT for NR-DC Maximum Output Power is in the maximum TT among all UL CCs. For TT of each UL CC refers to Table 6.2B.1.1.5-2. |  |  |  |  |  |  |  |  |

Table 6.2B.1.1.5-2: Test Tolerance (UE maximum output power)

| TT for overall output power (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |
|  |  |  | BW ≤ 40MHz |  | 40MHz < BW ≤ 100MHz |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| PSCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2B.2.1.5	Test requirement

The maximum output power for NR-DC, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table in clause 6.2.2.5.

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2B.3.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table in clause 6.2.3.5. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2.1.3-1 apply.

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2B.4.1.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2B.4.1.5-1.

Table 6.2B.4.1.5-1: Configured UE Output Power for Inter-band NR-DC

|  | Maximum output power |  |
| --- | --- | --- |
|  | Lower limit | Upper limit |
| Measured UE output power test point 1 | -14 dBm - TT | 0 dBm + TT |
| Measured UE output power test point 2 | 7 dBm - TT | 19 dBm + TT |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.5-2. |  |  |

Table 6.2B.4.1.5-2: Test Tolerance for Configured transmitted power for Inter-band NR-DC

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| PSCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2C.1.5	Test requirement

The maximum output power measured on SUL carrier shall not exceed the values specified in Table 6.2C.1.5-1.

Table 6.2C.1.5-1: PCMAX configured UE output power

|  | Channel bandwidth / maximum output power |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Measured UE output power test point 1 | -10 dBm ± (7+TT) |  |  |  |  |  |
| Measured UE output power test point 2 | 10 dBm ± (6+TT) |  |  |  |  |  |
| Measured UE output power test point 3 | 15 dBm ± (5+TT) |  |  |  |  |  |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. Note 2: In addition note 3 in Table 6.2.1.3-1 shall apply to the tolerances. |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2C.3.5	Test requirement

The maximum output power measured on SUL carrier shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2C.3.5-1.

Table 6.2C.3.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n80 |  |  |  |  | 23 | ±2±TT |
| n81 |  |  |  |  | 23 | ±2±TT |
| n82 |  |  |  |  | 23 | ±2±TT |
| n83 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n84 |  |  |  |  | 23 | ±2±TT |
| n86 |  |  |  |  | 23 | ±2±TT |
| n97 |  |  |  |  | 23 | ±2±TT |
| n99 |  |  |  |  | 23 | 2+TT/-3.03-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2C.4.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2C.4.5-1 and Table 6.2C.4.5-2.

Table 6.2C.4.5-1: UE Power Class test requirements (for Bands n80, n81, n82, n84, n86, n97) for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2 | 25.0 + TT | 21.0 - TT |
| 2 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 3 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 4 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2 | 25.0 + TT | 21.0 - TT |
| 6 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 7 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 8 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 9 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 10 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 11 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 12 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 13 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 16 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 17 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 18 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 19 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25.0 + TT | 19.5 - TT |
| 20 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 21 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 22 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 23 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 27 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 28 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 29 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 30 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25.0 + TT | 11.5 - TT |
| 31 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25.0 + TT | 11.5 - TT |
| 32 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25.0 + TT | 11.5 - TT |
| 33 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5- TT |
| 34 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5- TT |
| 35 | 23 | 0 | 0 | 0 | 23 | 2.0 | 2 | 25.0 + TT | 21- TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2C.4.5-3. |  |  |  |  |  |  |  |  |  |

Table 6.2C.4.5-2: UE Power Class test requirements (for Bands n83 with channel bandwidth other than 30MHz) for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 2 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 3 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 4 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 6 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 7 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 8 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 9 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 10 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 11 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 12 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 13 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 16 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 17 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 18 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 19 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2.5 | 25.0 + TT | 19.0 - TT |
| 20 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 21 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 22 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 23 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 27 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 28 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 29 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 30 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 31 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 32 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 33 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 34 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 35 | 23 | 0 | 0 | 0 | 23 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2C.4.5-3: UE Power Class test requirements (for Bands n83 with 30MHz channel bandwidth) for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 2 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 3 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 4 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 5 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 6 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 7 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 8 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 9 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 10 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 11 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 12 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 13 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 14 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 15 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 16 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 17 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 18 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 19 | 23 | 0 | 2 | 0 | 21 | 2 | 2.5 | 25.0 + TT | 18.5-TT |
| 20 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 21 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 22 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 23 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 24 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 25 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 26 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 27 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 28 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 29 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 30 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 31 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 32 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2C.4.5-4: UE Power Class test requirements for Band n99 for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （17.0 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （16.5 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （16.5 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 20.5 - TT | （16.5 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （17.0 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （15.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （15.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （15.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 – TT | （14.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 25.0 + TT | 18.5 - TT | （15.5 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.0 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.0 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.0 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （15.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.0 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.0 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.0 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （13.5 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （13.5 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （13.5 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 19.5 - TT | （16.5 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 19.5 - TT | （16.5 - TT2） |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 20.0 - TT | （17.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2C.5.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2C.5.5-1 to table 6.2C.5.5-8. The allowed A-MPR values specified in table 6.2.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2C.4. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2.1.3-1 apply.

Table 6.2C.5.5-1: UE Power Class 3 test requirements (NS_03) for band n86

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 4, 5 | NS_03 | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7 | NS_03 | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | NS_03 | 23 | 2 | 2.5 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 9 | NS_03 | 23 | 2 | 2.5 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10, 11 | NS_03 | 23 | 2.5 | 3 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 12 | NS_03 | 23 | 2.5 | 3 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 13, 14 | NS_03 | 23 | 4.5 | 4.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 15 | NS_03 | 23 | 4.5 | 4.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 16, 17 | NS_03 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | NS_03 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | NS_03 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | NS_03 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | NS_03 | 23 | 3.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | NS_03 | 23 | 3.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25, 26 | NS_03 | 23 | 6.5 | 6.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 27 | NS_03 | 23 | 6.5 | 6.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-2: UE Power Class 3 test requirements (NS_18) for band n83

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 5 | 23 | 1 | 2 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
|  | 10, 15, 20 | 23 | 1 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 3, 4 | 5 | 23 | 2 | 3 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
|  | 10, 15, 20 | 23 | 2 | 6 | 0 | 17 | 5 | 2.5 | 25+TT | 12-TT |
| 5, 6 | 5 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 2.5 | 25+TT | 15.5-TT |
|  | 10, 15, 20 | 23 | 2.5 | 7 | 0 | 16 | 5 | 2.5 | 25+TT | 11-TT |
| 7, 8 | 5 | 23 | 4.5 | 6 | 0 | 17 | 5 | 2.5 | 25+TT | 12-TT |
|  | 10, 15, 20 | 23 | 4.5 | 9 | 0 | 14 | 5 | 2.5 | 25+TT | 9-TT |
| 9, 10 | 5 | 23 | 3 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
|  | 10, 15, 20 | 23 | 3 | 6.5 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 11, 12 | 5 | 23 | 3 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
|  | 10, 15, 20 | 23 | 3 | 7 | 0 | 16 | 5 | 2.5 | 25+TT | 11-TT |
| 13, 14 | 5 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
|  | 10, 15, 20 | 23 | 3.5 | 8.5 | 0 | 14.5 | 5 | 2.5 | 25+TT | 9.5-TT |
| 15, 16 | 5 | 23 | 6.5 | 8.5 | 0 | 14.5 | 5 | 2.5 | 25+TT | 9.5-TT |
|  | 10, 15, 20 | 23 | 6.5 | 11.5 | 0 | 11.5 | 6 | 2.5 | 25+TT | 5.5-TT |
| 17 | 30 | 23 | 5 | 3 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 18 | 30 | 23 | 5 | 8 | 0 | 15 | 5 | 2.5 | 25+TT | 10–TT |
| 19 | 30 | 23 | 7 | 4.5 | 0 | 16 | 5 | 2.5 | 25+TT | 11–TT |
| 20 | 30 | 23 | 7 | 9.5 | 0 | 13.5 | 5 | 2.5 | 25+TT | 8.5–TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-3: UE Power Class 3 test requirements (NS_48) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 6 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 7 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 12 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 16 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 17 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 18 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 19 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 21 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 22 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 23 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 24 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 26 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 27 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 31 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 32 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 33 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 34 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 36 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 38 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 39 | 23 | 0 | 1.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 40 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 41 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 42 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 43 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 44 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 46 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 47 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 48 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 49 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 51 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 53 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 54 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 55 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 56 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 57 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 58 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 59 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 60 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-4: UE Power Class 3 test requirements (NS_49) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 5 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 7 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 10 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 16 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 17 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 18 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 19 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 20 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 21 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 23 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 26 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 27 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 29 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 30 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 31 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 32 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 33 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 34 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 35 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 37 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 38 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 40 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 41 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 42 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 43 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 44 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 45 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 46 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 47 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 48 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 49 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 50 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 51 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 52 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 53 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 55 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 56 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 57 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 58 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 59 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 61 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 62 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 63 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 64 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 65 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 66 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 67 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 68 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 69 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 70 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 71 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 72 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 73 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 74 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 75 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 76 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 77 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 78 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 79 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 80 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 81 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 82 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 83 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 84 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-5: UE Power Class 3 test requirements (NS_05) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 2 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 6 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 12 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 16 | 23 | 0 | 1.0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 17 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 21 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 22 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 23 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 24 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 25 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 26 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 27 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 28 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 29 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 30 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 31 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 32 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 33 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 34 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 35 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 36 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 37 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 38 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 40 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 41 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 42 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 43 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 44 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 45 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 46 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 47 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 48 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 49 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 50 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 51 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 52 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 53 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 55 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 56 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 57 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 58 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 59 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 61 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 62 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 63 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 64 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 65 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 66 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 67 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 68 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 69 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 70 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 71 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 72 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 73 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 74 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 75 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 76 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 77 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 78 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 79 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 80 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 81 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 82 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 83 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 84 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 85 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 86 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 87 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 88 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 89 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 90 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 91 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 92 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 93 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 94 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 95 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 96 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 97 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 98 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 99 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 100 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 101 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 102 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 103 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 104 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 105 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 106 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 107 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 108 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 109 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 110 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 111 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 112 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 113 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 114 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 115 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-6: UE Power Class 3 test requirements (NS_05U) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 2 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 6 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 12 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 16 | 23 | 0 | 1.0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 17 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 21 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 25 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 26 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 27 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 28 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 31 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 32 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 33 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 36 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 37 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 40 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 41 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 42 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 43 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 44 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 46 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 47 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 48 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 49 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 50 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 51 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 52 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 53 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 55 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 56 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 57 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 58 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 59 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 61 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 62 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 63 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 64 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 65 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 66 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 67 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 68 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 69 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 70 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 71 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 72 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 73 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 74 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 75 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 76 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 77 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 78 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 79 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 80 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 81 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 82 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 83 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 84 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 85 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 86 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 87 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 88 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 89 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 90 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 91 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 92 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 93 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 94 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 95 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 96 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 97 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 98 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 99 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 100 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 101 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 102 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 103 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 104 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 105 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 106 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 107 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 108 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 109 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 110 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 111 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 112 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 113 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 114 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 115 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 116 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 117 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 118 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 119 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 120 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 121 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 122 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 123 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 124 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 125 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 126 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 127 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 128 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 129 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 130 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 131 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-7: UE Power Class 3 test requirements (NS_100) for band n84

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10, 11 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 12 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13, 14 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 16, 17 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 24 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 25, 26 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-8: UE Power Class 3 test requirements (NS_56) for band n99

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 2 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 3 | 23 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 4 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 5 | 23 | 1 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 6 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 7 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 9 | 23 | 2 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 10 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 11 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 12 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 13 | 23 | 2.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 14 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 15 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 16 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 17 | 23 | 4.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 18 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 19 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 20 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 21 | 23 | 0.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 22 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 23 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 24 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 25 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 26 | 23 | 0.5 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 27 | 23 | 0.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 28 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 29 | 23 | 0.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 30 | 23 | 1 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 31 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 32 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 33 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 34 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 35 | 23 | 1 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 36 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 37 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 38 | 23 | 1 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 39 | 23 | 2 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 40 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 41 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 42 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 43 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 44 | 23 | 2 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 45 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 46 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 47 | 23 | 2 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 48 | 23 | 2.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 49 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 50 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 51 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 52 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 53 | 23 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 54 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 55 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 56 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 57 | 23 | 4.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 59 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 60 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 61 | 23 | 4.5 | 4 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 62 | 23 | 4.5 | 2 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 63 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 64 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 65 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 66 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 67 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 68 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 69 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 70 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 71 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 72 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 73 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 74 | 23 | 3.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 75 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 76 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 77 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 78 | 23 | 6.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 79 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 80 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 81 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 82 | 23 | 3 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 83 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 84 | 23 | 1.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 85 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 86 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 87 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 88 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 89 | 23 | 1.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 90 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 91 | 23 | 3 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 92 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 93 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 94 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 95 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 96 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 97 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 98 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 99 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 100 | 23 | 3.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 101 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 102 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 103 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 104 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 105 | 23 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 106 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 107 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 108 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 109 | 23 | 6.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 110 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 111 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 112 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 113 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 114 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 115 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 116 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 117 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2D.1.5	Test requirement

The maximum output power, derived in step 3 or step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.1.5-1.

Table 6.2D.1.5-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n2 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n3 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n7 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n25 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n30 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n34 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n38 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n39 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n40 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n41 |  |  | 29 | _+2+TT/-31-TT | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n48 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n66 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n70 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n71 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n77 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n78 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n79 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n97 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| NOTE 1: 1 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.1.5-2 |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.1.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2D.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-1 and Table 6.2D.2.5-2.

Table 6.2D.2.5-1: UE Power Class test requirements (for Band n1, n2, n3, n7, n25, n30, n34, n38, n39, n40, n41, n48, n66, n70, n71, n77, n78, n79, n97) for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 | （5.02） | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n25, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2: UE Power Class test requirements (for Bands n41, n77, n78, n79) for Power Class 2

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |  |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |  |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |  |
| 4 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |  |
| 5 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |  |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |  |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |  |
| 8 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |  |
| 9 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |  |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |  |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |  |
| 12 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |  |
| 13 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |  |
| 14 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-2a and Table 6.2D.2.5-2b.

Table 6.2D.2.5-2a: UE Power Class test requirements (for Band n1, n2, n3, n7, n25, n30, n34, n38, n41, n48, n66, n70, n71, n77, n78, n79) for Power Class 3 with supporting ULFPTx

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 | 25.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 2 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 4 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 | 25.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 6 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | (6.02) | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 7 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | (6.02) | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 8 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | (6.02) | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 9 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | (6.02) | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 10 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 11 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 12 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 13 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | (5.02) | 3 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 14 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | (5.02) | 3 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 15 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | (5.02) | 3 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 16 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 17 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 18 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 19 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | (6.02) | 3 | 25.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 20 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | (5.02) | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 21 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | (5.02) | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 22 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | (5.02) | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 23 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | (5.02) | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | (5.02) | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | (5.02) | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 27 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 28 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 29 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 30 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 31 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 32 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2b: UE Power Class test requirements (for Bands n41, n77, n78, n79) for Power Class 2 with supporting ULFPTx

TBD

Table 6.2D.2.5-3: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2D.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3.5-1 to table 6.2D.3.5-14. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

Table 6.2D.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

Table 6.2D.3.5-1: UE Power Class 2 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 2 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 3 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 4 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 5 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 6 | 26 | 1.5 | 0 | 0 | 24.5 | 3 | 3 | 28+TT | 21.3-TT |
| 7 | 26 | 3 | 0 | 0 | 23 | 3 | 3 | 28+TT | 20.0-TT |
| 8 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 9 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 10 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 11 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 12 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 13 | 26 | 2 | 0 | 0 | 24 | 3 | 3 | 28+TT | 21.0-TT |
| 14 | 26 | 3 | 0 | 0 | 23 | 3 | 3 | 28+TT | 20.0-TT |
| 15 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 16 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 17 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 18 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 19 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 20 | 26 | 0 | [10] | 0 | 16 | 5 | 3 | 28+TT | [11.0-TT] |
| 21 | 26 | 0 | [10] | 0 | 16 | 5 | 3 | 28+TT | [11.0-TT] |
| 22 | 26 | 0 | 7.5 | 0 | 18.5 | 5 | 3 | 28+TT | 13.5-TT |
| 23 | 26 | 6.5 | 0 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 24 | 26 | 6.5 | 0 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2: UE Power Class 3 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 2 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 4 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 5 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 6 | 23 | 1.5 | 0 | 0 | 21.5 | 5 | 3 | 25+TT | 16.5-TT |
| 7 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 8 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 10 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 11 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 12 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 13 | 23 | 2.0 | 0 | 0 | 21.0 | 5 | 3 | 25+TT | 16.0-TT |
| 14 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 15 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 16 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 17 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 18 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 19 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 20 | 23 | 0 | 8 | 0 | 15.0 | 6 | 3 | 25+TT | 9.0-TT |
| 21 | 23 | 0 | 8 | 0 | 15.0 | 6 | 3 | 25+TT | 9.0-TT |
| 22 | 23 | 0 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 23 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 24 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-3: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 2 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 3 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 4 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 5 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 6 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 7 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 8 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 9 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 10 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| 11 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| 12 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-4: UE Power Class 3 test requirements (NS_05) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 26 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 36 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 37 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 38 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 39 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 40 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 41 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 42 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 43 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 44 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 45 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 46 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 47 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 48 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 49 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 50 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-5: UE Power Class 3 test requirements (NS_05U) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 26 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 36 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 37 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 40 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 41 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 42 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 43 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 44 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 46 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 47 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 48 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 49 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 50 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 51 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 53 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 54 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 55 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-6: UE Power Class 3 test requirements (NS_48) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 2 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 4 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 6 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 9 | 23 | 0 | 1.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 10 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 11 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 12 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 13 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 14 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 16 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 17 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 18 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 19 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 20 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 21 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 22 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 23 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 24 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 25 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 26 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 27 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 29 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 30 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-7: UE Power Class 3 test requirements (NS_49) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 2 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 4 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 5 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 9 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 10 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 11 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 13 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 14 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 16 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 17 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 18 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 19 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 20 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 21 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 22 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 23 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 24 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 25 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 26 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 29 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 30 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 31 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 32 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 33 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 34 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 35 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 36 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 37 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 38 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 39 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 40 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 41 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 42 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-8: UE Power Class 3 test requirements (NS_100) for band n1

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 4, 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 6 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 7, 8 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 9 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 10, 11 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 12 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-9: UE Power Class 3 test requirements (NS_100) for band n2, n3, n25

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 4, 5 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 6 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 7, 8 | 23 | 3.5 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 9 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 10, 11 | 23 | 6.5 | 6.5 | 1.5 | 15 | 5 | 2 | 25+TT | 10-TT |
| 12 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-10: UE Power Class 3 test requirements (NS_03/NS_03U) for band n66, n70

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 3 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 4,5 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 6 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 7,8 | NS_03, NS_03U | 23 | 3.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 9 | NS_03, NS_03U | 23 | 3.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 10,11 | NS_03, NS_03U | 23 | 6.5 | 6.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 12 | NS_03, NS_03U | 23 | 6.5 | 6.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-11: UE Power Class 3 test requirements (NS_03/NS_03U) for band n2, n25

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | NS_03, NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 3 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 4,5 | NS_03, NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 6 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 7,8 | NS_03, NS_03U | 23 | 3.5 | 4 | 1.5 | 17 | 5 | 2 | 25+TT | 12-TT |
| 9 | NS_03, NS_03U | 23 | 3.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 10,11 | NS_03, NS_03U | 23 | 6.5 | 6.5 | 1.5 | 14 | 5 | 2 | 25+TT | 9-TT |
| 12 | NS_03, NS_03U | 23 | 6.5 | 6.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-12: UE Power Class 3 test requirements (NS_46) for band n7

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 2 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 3 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 4 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 3 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 6 | 23 | 0 | 3.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 9 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 10 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 11 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 12 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-13: UE Power Class 3 test requirements (NS_21) for band n30

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) |  | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 5 | 23 | 3 | 0 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 3 | 5 | 23 | 3 | 0 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 4,5 | 10 | 23 | 3 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 6,7 | 5 | 23 | 3 | 0 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 5 | 23 | 3 | 0 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 9,10 | 10 | 23 | 3 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 11,12 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
|  | 10 | 23 | 3.5 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 13 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
|  | 10 | 23 | 3.5 | 5.5 | 0 |  | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 14,15 | 10 | 23 | 3.5 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 16,17 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 6 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 18 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 19,20 | 10 | 23 | 6.5 | 5.5 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 4 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-14: UE Power Class 3 test requirements (NS_44) for band n38

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 2 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 3 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 4 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 5 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 6 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 7 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 8 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 10 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 11 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 12 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 13 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 14 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 16 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 17 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 18 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 19 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 20 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 21 | 23 | 0 | 6.5 | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 22 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-15: UE Power Class 3 test requirements (NS_27) for band n48

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 2 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 3 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 5 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 6 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 7 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 9 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 10 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 11 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 12 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 13 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 14 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 15 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 16 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 17 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 18 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 19 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 20 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 21 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 22 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 23 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 24 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 25 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 26 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 28 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 29 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 30 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 31 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 32 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 33 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 34 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 35 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 36 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 37 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 39 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 40 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 41 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 42 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 43 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 44 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 45 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 46 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 47 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 48 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 49 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 50 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 51 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 52 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 53 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 54 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 55 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 56 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 57 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 58 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 59 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 60 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 61 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 62 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 63 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 64 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 65 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 66 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 67 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 68 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 69 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 70 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 71 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 72 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 73 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 74 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 75 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 76 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 77 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 78 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 79 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 80 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 81 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 82 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 83 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 84 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 85 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 86 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 87 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 88 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 89 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 90 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 91 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 92 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 93 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 94 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 95 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 96 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 97 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 98 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 99 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 100 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 101 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 102 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 103 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 104 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 105 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 106 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 107 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 108 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 109 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 110 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 111 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 112 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2D.4.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2D.4.5-1.

Table 6.2D.4.5-1: PCMAX configured UE output power

| Configured transmitted power |  |
| --- | --- |
| Measured UE output power test point 1 | 0 dBm ± (7+TT) |
| Measured UE output power test point 2 | 14 dBm ± (6+TT) |
| Measured UE output power test point 3 | 18 dBm ± (5+TT) |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2D.4.5-2. Note 2: In addition note 2 in Table 6.2D.1.3-1 shall apply to the tolerances. |  |

Table 6.2D.4.5-2: Test Tolerance (Configured transmitted power for UL MIMO)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< End of changes >>