# Title:	Test Tolerances for L1 SL-RSRP measurement for resource re-evaluation tests

Source:	Huawei, Hisilicon

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for L1 SL-RSRP measurement for resource re-evaluation tests. The test cases in question, from TS 38.533, are:

- 9.1.4.3	NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation

# 2. 	Test case in TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v16.12.0. Changes to be made in RAN4 #104 is yellow highlighted:

#### A.9.1.4.3	Test for V2X UE Resource Re-evaluation

##### A.9.1.4.3.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to autonomous resource re-evaluation for V2X UE in mode 2 defined in clause 12.5. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

The test parameters are given in Table A.9.1.4.3.1-1, A.9.1.4.3.1-2 and A.9.1.4.3.1-3 below. There are 130 active V2X sidelink UEs in this test. The first 100 active V2X sidelink UEs are scheduled with 50ms periodicity. The last 30 active V2X sidelink Ues are aperiodic service UE with retransmission reservation period equaling 15ms.

Both the UE under test and active V2X sidelink Ues select GNSS as synchronization reference source. The test system can emulate and send the GNSS signal to the test UE and active V2X sidelink Ues. The test parameters for GNSS signals are defined in B.4.1.

The test consists of three duration T0, T1, T2.

During T0, the signal from Test Equipement are configured. The resource occupied by the active V2X sidelink UEs is expected to be excluded in the resource selection procedure such that the measured PSSCH-RSRP is above the measurement threshold. The test equipment shall just configure the resource pool for the test UE without the MAC PDU for transmission channel configuration.

During T1, the signal from Test Equipement are configured. Some of the resource occupied by the active V2X sidelink Ues is expected to be excluded in the resource selection procedure such that the measured PSSCH-RSRP is above the measurement threshold and some of the resource occupied by the active V2X sidelink Ues is expected to be included in the resource selection procedure such that the measured PSSCH-RSRP is below the measurement threshold. The test system shall emulate the active V2X sidelink Ues to transmit PSCCH/PSSCH every 50ms according to the RSRP level specified in the Table A. 9.1.4.3.1-2, but UE #0~29 will be silent during T2

At the end of T1, where slot index mod 100 = 99, the test equipment shall send one message with a SL-SCH MAC PDU as specified in Clause 6.1.6 in TS 38.321[7], in order to make sure that the UE under test shall be scheduled to periodically transmit PSCCH/PSSCH.

During T2, the additional aperiodic active V2X sidelink UEs from Test Equipement are configured in the beginning 30 slots, and the resource occupied by these active V2X sidelink UEs is expected to be excluded in the resource re-evaluation procedure such that the measured PSSCH-RSRP is above the measurement threshold shown in Table A. 9.1.4.3.1-2. The test system shall emulate the active V2X sidelink UEs to transmit PSCCH/PSSCH with the maximum number of reserved PSCCH/PSSCH resources equalling n2 and time resource assignment interval as 15ms.

During T2, the test UE is expeted to reselect the resources and transmit the PSCCH/PSSCH in the newly re-evaluated resources.

Table A.9.1.4.3.1-1: Test Parameters for V2X UE Resource Selection Tests for Re-evaluation

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | HD carrier in Band n47 and n38 |
| Channel Bandwidth (BWchannel) Note 2 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS |  | kHz | 30 |  |
| V2X sidelink communication pre-configuration |  |  | As specified in Table A.3.21.2-2 | IE values unless specified otherwise in this test. |
| sl-TimeResource-r16 included in SL-ResourcePool |  |  | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[3]) |
| sl-NumSubchannel-r16 included in SL-ResourcePool |  |  | 1 | Indicates the number of sub-channels for TX resource pool |
| sl-SubchannelSize-r16 included in SL-ResourcePool |  |  | 10 |  |
| sl-StartRB-Subchannel-r16 |  |  | 0 |  |
| Number of Active Sidelink UEs |  |  | 130 | Active Sidelink UE i = 0, .., 129 |
| SL-Thres-RSRP-r16 |  |  | 17 | Corresponding -96 dBm as defined in Section 6.3.5 in TS38.331[2] |
| Active Sidelink UEs(UE i=0-99) | V2X sidelink Communication preconfiguration |  | As specified in Table A.3.21.2-2 | IE values unless specified otherwise in this test. |
|  | sl-TimeResource-r16 included in SL-ResourcePool |  | {1i}Note1 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213 [3]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-SubchannelSize-r16 included in SL-ResourcePool |  | 10 | Indicates the size of sub-channels for TX resource pool |
|  | sl-ResourceReservePeriod2-r16 | ms | 50 |  |
| Active Sidelink UEs(UE i= 100-129) | V2X sidelink Communication preconfiguration |  | As specified in Table A.3.21.2-2 | IE values unless specified otherwise in this test. |
|  | sl-TimeResource-r16 included in SL-ResourcePool |  | {1i}Note1 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213 [3]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-SubchannelSize included in SL-ResourcePool |  | 10 | Indicates the size of sub-channels for TX resource pool |
|  | sl-MultiReserveResource-r16 |  | enabled |  |
|  | sl-MaxNumPerReserve-r16 |  | n2 |  |
|  | sl-ResourceReservePeriod2-r16 |  | 0 | Unit:ms |
| Timing offset among Active Sidelink UEs |  | s | CP/2 | Synchronous |
| T0 |  | s | 1 |  |
| T1 |  | ms | 50 |  |
| T2 |  | ms | 50 |  |
| Note 1: {1i} is a sequence of ninety-nine 0’s with one 1 in (mod(i,100)+1)’th position. Note 2: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |

Table A.9.1.4.3.1-2: Active Sidelink UE Specific Test Parameters for V2X UE Resource Selection Tests for Re-evaluation (UE #0...99)

| Parameter | Unit | Active Sidelink UE i (i = 0, .., 99) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number | - | 1 |  |
| Channel Bandwidth (BWchannel) Note 7 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| PSCCH RMC (defined in A.3.21.3) | - | CC.1A HD |  |
| PSSCH RMC (defined in A.3.21.3) | - | CD.1A HD |  |
| Note1 | dBm/SCS | -103 |  |
| PSSCH1 Note 3 | dB | 25.5 | 25.5 |
| PSSCH2 Note 4 | dB | 2 | 2 |
| PSSCH3 Note 5 | dB | 12 | -infinity |
| PSSCH4 Note 6 | dB | 12 | 12 |
| PSSCH1 Note2,3,10 | dB | 25.5 | -0.01 |
| PSSCH1 Note2,3,11 |  | 25.5 | 25.5 |
| PSSCH2 Note2,4 | dB | 2 | -23.51 |
| PSSCH3 Note2,5 | dB | 12 | -infinity |
| PSSCH4 Note2,6 | dB | 12 | 12 |
| PSSCH -RSRP1 Note 2, 3 | dBm/SCS | -77.5 | -77.5 |
| PSSCH -RSRP2 Note 2, 4 | dBm/SCS | -101 | -101 |
| PSSCH -RSRP3 Note 2, 5 | dBm/SCS | -91 | -infinity |
| PSSCH -RSRP4 Note 2, 6 | dBm/SCS | -91 | -91 |
| Io Note 2, 8 | dBm/18 MHz | -69.94 | -56.70 |
|  | dBm/36 MHz | -69.94 | -56.70 |
| Io Note 2, 9 | dBm/18 MHz | -78.08 | -56.68 |
|  | dBm/36 MHz | -78.08 | -56.68 |
| Io Note 2, 10 | dBm/18 MHz | -56.70 | -53.69 |
|  | dBm/36 MHz | -56.70 | -53.69 |
| Io Note 2, 11 | dBm/18 MHz | -56.70 | -56.70 |
|  | dBm/36 MHz | -56.70 | -56.70 |
| Io Note 2, 12 | dBm/18 MHz | -69.94 | -69.94 |
|  | dBm/36 MHz | -69.94 | -69.94 |
| Antenna Configuration | - | 1x2 |  |
| Propagation Condition | - | AWGN |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: , PSSCH-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: UE #50~64 and UE #85~99 will periodically occupy the subchannels on the slot with “#slot index mod 100” = #50-64 and #85-99. Note 4: UE #30~49 will periodically occupy the subchannels on the slot with “#slot index mod 100” = #30-49. Note 5: UE #0~29 will periodically occupy the subchannels on the slot with “#slot index mod 100” = #0-29. Note 6: UE #65~84 will periodically occupy the subchannels on the slot with “#slot index mod 100” = #65-84. Note 7: The UE is only required to be tested in one of the supported configurations. Note 8: Calculated on slot with “#slot index mod 100” = #0~29. Note 9: Calculated on slot with “#slot index mod 100” = #30~49. Note 10: Calculated on slot with “#slot index mod 100” = #50~59. Note 11: Calculated on slot with “#slot index mod 100” = #60~64 and #85~99. Note 12: Calculated on slot with “#slot index mod 100” = #65~84. |  |  |  |

Table A.9.1.4.3.1-3: Active Sidelink UE Specific Test Parameters for V2X UE Resource Selection Tests for Re-evaluation (UE #100…129)

| Parameter | Unit | Active Sidelink UE i (i = 100, .., 129) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number | - | 1 |  |
| Channel Bandwidth (BWchannel) Note 4 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| PSCCH RMC (defined in A.3.21.3) | - | CC.1A HD |  |
| PSSCH RMC (defined in A.3.21.3) | - | CD.1 A HD |  |
| Note1 | dBm/SCS | -103 |  |
| PSSCH | dB | -infinity | 25.5 |
| PSSCH Note2,5 | dB | -infinity | 25.5 |
| PSSCH Note2,6 |  | -infinity | 21.38 |
| PSSCH Note2,7 |  | -infinity | -0.11 |
| PSSCH-RSRP Note 2, Note 3 | dBm/SCS | -infinity | -77.5 |
| Io Note 2, 5 | dBm/18 MHz | -69.94 | -56.70 |
|  | dBm/36 MHz | -69.94 | -56.70 |
| Io Note 2, 6 | dBm/18 MHz | -78.08 | -56.68 |
|  | dBm/36 MHz | -78.08 | -56.68 |
| Io Note 2, 7 | dBm/18 MHz | -56.70 | -53.69 |
|  | dBm/36 MHz | -56.70 | -53.69 |
| Antenna Configuration | - | 1x2 |  |
| Propagation Condition | - | AWGN |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, PSSCH-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: UE #100~129 will occupy the subchannels on the slots with “#slot index mod 100”= #0-29 during T2. Note 4: The UE is only required to be tested in one of the supported configurations. Note 5: Calculated on slot with “#slot index mod 100” = #0~29. Note 6: Calculated on slot with “#slot index mod 100” = #30~49. Note 7: Calculated on slot with “#slot index mod 100” = #50~59. |  |  |  |

##### A.9.1.4.3.2	Test Requirements

The rate of PSSCH transmissions on the resources of the subchannels which are occupied by UE #65-84 shall be more than 90% during T2.

# 3. 	Discussion

During the whole test, the UE under test shall keep sensing SL transmission in resource sensing window. To be more specific, UE shall perform SL-RSRP measurement on every candidate resource in sensing window and compare the measurement results against the threshold SL-Thres-RSRP.

During T1, periodic sidelink UEs (UE 0~99) are powered on and aperiodic sidelink UEs (UE 100~129) are powered off. SL UE 0~99 keep performing sidelink transmitting with 50ms periodicity and with three different power levels:

- SL UE #30~49 transmit PSCCH/PSSCH with the power of -101 dBm/SCS (referred as "low power" hereinafter) in slot #30~49, which is 5dB lower than sl-Thres-RSRP;

- SL UE #0~29 and #65~84 transmit PSCCH/PSSCH with the power of -91 dBm/SCS (referred as "mid power" hereinafter) in slot #0~29 and #65~84, which is 5dB higher than sl-Thres-RSRP;

- SL UE #50~64 and #85~99 transmit PSCCH/PSSCH with the power of -77.5 dBm/SCS (referred as “high power” hereinafter) in slot #50~64 and #85~99, which is 18.5dB higher than sl-Thres-RSRP;

During the begining of T2, SL UE #0~29 are powered off. Aperiodic sidelink UEs (UE #100~129) are powered on and starts to transmit with high power. Each SL UE among UE #100~129 reserves two PSCCH/PSSCH resources with a single SCI 1-A. and the time interval between two PSCCH/PSSCH resources are 30 slots. So we have:

- The first PSCCH/PSSCH scheduled by UE #100~129 are transmitted with high power in slot #0~29;

- The second PSSCH scheduled by UE #100~129 are transmitted with high power in slot #30~59;

- SL UE #65~84 transmit PSCCH/PSSCH with the mid power in slot #65-84;

- SL UE #50~64 and #85~99 transmit PSCCH/PSSCH with high power in slot #50~64 and #85~99;

It is helpful to visualise the transmit power of SL UEs as below.

Resource sensing and re-evaluation

During T1, the UE under test keeps performing resource sensing in sensing window according to 38.214 clause 8.1.4. It is expected to excluded slots occupied by SL UEs with transmit power higher than SL-Thres-RSRP (i.e. slot #0~29 and #50~99) and only includes slots occupied by low power SL UEs (i.e. slot 30~49) in candidate resource set. At the end of T1 DUT is configured to perform SL transmission. DUT is expected to select some candidate resource in slot #30~49 to transmit.

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot t'mSL, and 'Resource reservation period' field, if present, and 'Priority' field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX;

...

The UE shall report set SA to higher layers.

If a resource ri from the set (r0,r1,r2,…) is not a member of SA, then the UE shall report re-evaluation of the resource ri to higher layers.

During T2, the UE is expected to perform resource re-evaluation in sensing window before it actually performs SL transmission. Then it shall detect the SCI 1-A sent by active sidelink UE #100~129and realize that slot #30~59 will also be occupied by SL UE #100~129. Then it shall stop sending PSSCH on slot #0~29 and performs resource sensing again.

However, The UE finds that there are no found no available candidate resources since all slots are occupied by mid/high power UEs. Then it shall increase SL-Thres-RSRP by 3dB and repeat resource sensing procedure until mid power is below the threshold. Then the UE is expected to select the slots occupied by mid power SL UEs as it candidate resource set (i.e. slot 65~84).

To ensure this. PSSCH-RSRP measurement result of high-power SL UE shall be even higher than increased SL-Thres-RSRP in T2.

It is helpful to show the power levels for each SL UE versus time, together with the thresholds, as shown below:

With nominal signal levels the condition for resource pre-emption is easily met. but the effect of uncertainties on the test verdict needs to be checked. A decision can then be made on whether Test Tolerances need to be applied.

# 4. 	Calculation of Test Tolerances

Two test configurations are given in test case A.9.1.4.2 (20 MHz CBW and 40 MHz CBW). Analysis given in this clause are based on the test configuration of 20 MHz CBW.

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from 38.133 Table A.9.1.4.2.1-1 and A.9.1.4.2.1-2. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

In this test case the UE is expected to evaluate every candidate resource in resource sensing window. The UE makes SL-RSRP measurements of active sidelink UE and compares them with the threshold SL-Thres-RSRP. A further step is done to calculate SL-RSRP power difference:

- (measured sidelink UE SL-RSRP - SL-Thres-RSRP)

c) Uncertainties

The SS provides AWGN, and 1 active sidelink UE to the UE. We propose to control the following parameters:

- AWGN Absolute power averaged over BWConfig, Noc ±1.5 dB

- AWGN Absolute power averaged over measurement PRB, Noc ±1.5 dB

- Ratio of active sidelink UE i signal to noise averaged over BWConfig, Êsi / Noc ±0.3 dB, i = 0,1,...,129;

- Ratio of active sidelink UE i signal to noise averaged over measurement PRB, Êsi / Noc ±0.3 dB, i = 0,1,...,129;

This choice forms a minimum set, so the superposition principle can be applied if necessary.

In addition, the UE SL-RSRP absolute measurement accuracy needs to be considered, because when the UE compares active sidelink UE SL-RSRP against the threshold SL-Thres-RSRP, it has some inaccuracy which could affect the test verdict. The UE measurement accuracy is defined in the following extract from TS 38.133:

10.4.4.1	Absolute L1 SL-RSRP Accuracy

The requirements for absolute accuracy of L1 SL-RSRP in this clause apply to a UE performing PSCCH-RSRP and/or PSSCH-RSRP measurements on the same frequency as used by operating V2X sidelink communication.

The accuracy requirements in Table 10.4.4.1-1 are valid under the following conditions:

-	Demodulation reference signals for PSCCH and/or PSSCH are transmitted from one port.

-	Conditions defined in clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	PSCCH-RSRP|dBm and/or PSSCH-RSRP|dBm according to Annex B.4.4 for a corresponding Band are fulfilled.

Table 10.4.4.1-1: Intra-frequency L1 SL-RSRP absolute accuracy for UE capable of V2X sidelink communication

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm/SCS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCS = 15kHz | SCS = 30kHz | SCS = 60kHz |  |  |
| 4.5 | 9 | 0 dB | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -70 |
| 8 | 11 | 0 dB | NR_TDD_FR1_B NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS and/or PSSCH-DMRS. |  |  |  |  |  |  |  |  |

Band group NR_TDD_FR1_J has the tightest Io requirement. which is no less than -113.5dBm/30kHz and no bigger than -50dBm/Ch BW. To ensure SL-RSRP absolute measurement accuracy, the conditions of PSSCH-DMRS_RP and PSSCH-DMRS Es/Iot in Annex B.4.4 need to be also fulfilled.

## B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements

This clause defines the following condtions for L1 SL-RSRP measurement accuracy requirements applicable for a corresponding operating band.

The conditions are defined in Table B.4.4-1 for FR1.

Table B.4.4-1: Conditions for L1 SL-RSRP measurements in FR1

| Parameter | NR V2X operating band groups Note1 | Minimum L1 SL-RSRP |  |  | Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/SCS |  |  | dB |
|  |  | SCS= 15kHz | SCS= 30kHz | SCS = 60kHz |  |
|  | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | 0 |
|  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 |  |
| NOTE 1: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 2: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS and/or PSSCH-DMRS. NOTE 3: The SCS is for PSCCH and/or PSSCH. |  |  |  |  |  |

In summary,

- Io shall be no less than -113.5dBm/30kHz and no bigger than -70dBm/Ch BW in slot 0~49 and slot 65~84 in T1;

- Io shall be no less than -113.5dBm/30kHz and no bigger than -50dBm/Ch BW in slot 50~64 and slot 85~99 in T1;

- Io shall be no less than -113.5dBm/30kHz and no bigger than -70dBm/Ch BW in slot 65~84 in T2;

- Io shall be no less than -113.5dBm/30kHz and no bigger than -50dBm/Ch BW in slot 0~64 and slot 85~99 in T2;

- PSSCH-DMRS_RP of active sidelink UEs shall be no less than -113.5 dBm/30kHz;

- PSSCH-DMRS Es/Iot of active sidelink UEs shall be no less than 0 dB;

- Measured low power SL UE SL-RSRP shall be lower than SL-Thres-RSRP and Measured mid/high power SL UE SL-RSRP shall be higher than SL-Thres-RSRP.

- Measured low/mid power SL UE SL-RSRP shall be lower than increased SL-Thres-RSRP and Measured high power SL UE SL-RSRP shall be higher than increased SL-Thres-RSRP.;

d) Controlled parameters critical to verdict

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and the relevant clauses in TS 38.133.

Having identified the parameters which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB.

Denoting critical parameter y as a function of test equipment uncertainties x1, x2, ..., i.e. y=fx1,x2,⋯. Then sensitivity factors of xi on y can be expressed as:

sensitivity factors of xi on y=∂y∂xi

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE measurement accuracies where relevant. This process follows the superposition principle.

For the test system uncertainties the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE measurement accuracies are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE measurement accuracies. If all the uncertainties were combined root-sum square, the resulting smaller test limits could case a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

Nominal value of low power active sidelink UE PSSCH-DMRS Es/Iot is 2dB, minimum PSSCH-DMRS Es/Iot requirement is satisfied with 2 dB margin. The overall uncertainty of PSSCH-DMRS Es/Iot is ±0.3 dB in T1 and 0.42dB in T2 as calculated in step d) of the accompanying spreadsheet. After including uncertainty the minimum Es/Iot requirement can still be satisfied. No offset is needed.

Nominal value of mid power active sidelink UE PSSCH-DMRS Es/Iot is 12dB, minimum PSSCH-DMRS Es/Iot requirement is satisfied with 12 dB margin. The overall uncertainty of PSSCH-DMRS Es/Iot is no more than 0.42dB as calculated in step d) of the accompanying spreadsheet. After including uncertainty, the minimum Es/Iot requirement can still be satisfied. No offset is needed.

Nominal value of high power active sidelink UEs other than UE 100~129 PSSCH-DMRS Es/Iot is 25.5dB, minimum PSSCH-DMRS Es/Iot requirement is satisfied with 25.5 dB margin. The overall uncertainty of PSSCH-DMRS Es/Iot is no more than 0.42dB as calculated in step d) of the accompanying spreadsheet. After including uncertainty the minimum Es/Iot requirement can still be satisfied. No offset is needed.

For UE 100~129, the Nominal value of PSSCH-DMRS Es/Iot in T2 is -0.01 dB. minimum PSSCH-DMRS Es/Iot requirement is satisfied. The overall uncertainty of PSSCH-DMRS Es/Iot is ±0.42 dB in T2 as calculated in step d) of the accompanying spreadsheet. So we suggest to increase Es/Noc of UE 100~129 in T2 by a amount to ensure minimum Es/Iot requirements are satisfied.

Nominal value of low/mid/high power active sidelink UE PSSCH-DMRS_RP is -101/-91/-77.5 dBm/30kHz, minimum PSSCH-DMRS_RP requirement is satisfied with at least 12.5 dB margin. The overall uncertainty of PSSCH-DMRS_RP is ±1.53 dB as calculated in step d) of the accompanying spreadsheet. After including uncertainty the minimum PSSCH-DMRS_RP requirement can still be satisfied. No offset is needed.

Nominal value of Io is:

- Slot #0~29: -69.94 dBm/Ch BW in T1 and -56.70 dBm/Ch BW in T2;

- Slot #30~49: -78.08 dBm/Ch BW in T1 and -56.68 dBm/Ch BW in T2;

- Slot #50~59: -56.70 dBm/Ch BW in T1 and -53.69 dBm/Ch BW in T2;

- Slot #60~64, 85~99: -56.70 dBm/Ch BW in T1 and T2;

- Slot #65~84: -69.94 dBm/Ch BW in T1 and T2;

The overall uncertainty for Io is ±1.53 dB. After including uncertainty, the Io range requirements can’t  be satisfied in slot #0~29 in T1 and slot #65~84 in T1 and T2. We purpose to lower Noc by an amount to reduce Io level in these positions.

Nominal value of low power active sidelink UE (measured SL-RSRP - SL-Thres-RSRP) is -5dB, So the resource selection requirements (<0dB) is satisfied by 5dB. And the overall uncertainty for active sidelink UE (measured SL-RSRP - SL-Thres-RSRP) is ±6.03 dB. After including uncertainty, the resource selection requirements can’t be satisfied. We suggest to lower Noc level by an amount to leave enough margin for SL-RSRP measurement uncertainty.

Nominal value of mid power active sidelink UE (measured SL-RSRP - SL-Thres-RSRP) is 5dB, So the resource selection requirements (>0dB) is satisfied by 5dB. And the overall uncertainty for active sidelink UE (measured SL-RSRP - SL-Thres-RSRP) is ±6.03 dB. After including uncertainty, the resource selection requirements can’t be satisfied. We suggest to raise mid power SL UE Es/Noc level by an amount to leave enough margin for SL-RSRP measurement uncertainty.

Nominal value of high power active sidelink UE (measured SL-RSRP – increased SL-Thres-RSRP) is 3.5dB, So the resource exclusion requirements (>0dB) is satisfied by 3.5dB. And the overall uncertainty for active sidelink UE (measured SL-RSRP - SL-Thres-RSRP) is ±9.53 dB. After including uncertainty the resource exclusion requirements can’t be satisfied. We suggest to raise high power SL UE Es/Noc level by an amount to leave enough margin for SL-RSRP measurement uncertainty .

The check that all other controlled parameters meet their required range is done in step g). In theory it is possible for steps e) to g) to be iterative, or possibly even steps c) to g) to be iterative, but this test case can be done without such iteration.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

- Noc level is decreased by 7.05 dB to satisfy Io requirements

- SL-Thres-RSRP is decreased by 6dB and mid power SL UE Es/Noc is increased by 2.1dB to ensure mid power UE SL-RSRP > SL-Thres-RSRP;

- High power SL UE (except UE 100~129) Es/Noc is increased by 7.1dB to ensure high power UE SL-RSRP > increased SL-Thres-RSRP.

- SL UE 100~129 Es/Noc is increased by 7.55 dB to ensure high power UE SL-RSRP > increased SL-Thres-RSRP and Es/Iot > 0dB.

Re-derived parameters are calculated using the same methods as were used in step b). In general all the values are listed for each time interval T1 and T2 but as NR Cell 3 is not present during T1 those entries are greyed out (the formulae are left in place for use by future test cases).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated.

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements. It is not necessary to calculate all parameters, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

It can be seen that with the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TC 9.1.4.3 also contains configurations with different channel bandwidths. However, difference on CBW has no impact on TT analysis since only 10 RBs are used in test. So the analysis based on 20MHz CBW given in clause 4 also applies.