3GPP TSG-RAN5 Meeting #96-e	R5-224601r1

# Online, ,  15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2475 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of MDT message and information element |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The R17 MDT IE LoggedMeasurementConfiguration-v1700-IEs is missing in TS 38.508-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Updated LoggedMeasurementConfiguration message to include R17 IE 2.Added IE LoggedMeasurementConfiguration-v1700-IEs in 4.6.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1, 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Corrected IE name: LoggedMeasurementConfiguration-v17xy-IEs -> LoggedMeasurementConfiguration-v1700-IEs - Corrected table number from 4.6.3-66A to 4.6.3-65A according to alphabetical order. Also adjust the position of new added table accordingly - Updated the declaration of LoggedMeasurementConfiguration-v1700-IEs with the addition of ‘LoggedMeasurementConfiguration-v1700-IEs::= SEQUENCE { }’ - Update condition ‘sigLogMDT’ to ‘SigLogMDT’ in Table 4.6.3-65A to align with the convention in TS 38.508-1 (capitalize the first letter) - Added condition LOG-MEAS-R17 for option IE to be present in Table 4.6.1-5AA |  |  |  |  |  |  |  |  |

<Start of modified section>

4.6.1	Contents of RRC messages

< Unchanged sections omitted >

–	LoggedMeasurementConfiguration

Table 4.6.1-5AA: LoggedMeasurementConfiguration (Thres1)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |  |
| plmn-Identity-r16 |  | plmn-Identity in USIM | MCC/MNC=MCC/MNC in USIM |  |
| traceId-r16 |  | ‘0EF’H | OCTET STRING (SIZE (3)) |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r16 |  | ‘1A’H | OCTET STRING (SIZE (2)) |  |
| tce-Id-r16 |  | ‘5’H | OCTET STRING (SIZE (1)) |  |
| absoluteTimeInfo-r16 |  | Set to value corresponding to the absolute time when the message is sent | BIT STRING (SIZE (48)) |  |
| areaConfiguration-r16 |  | Not present |  |  |
| plmn-IdentityList-r16 |  | Not present |  |  |
| bt-NameList-r16 |  | Not present |  |  |
| wlan-NameList-r16 |  | Not present |  |  |
| sensor-NameList-r16 |  | Not present |  |  |
| loggingDuration-r16 |  | min120 |  |  |
| reportType CHOICE { |  |  |  |  |
| periodical SEQUENCE { |  |  |  | PERIODICAL |
| loggingInterval-r16 |  | ms2560 | 2.56s |  |
| } |  |  |  |  |
| eventTriggered SEQUENCE { |  |  |  | EVENT |
| eventType-r16 CHOICE { |  |  |  |  |
| outOfCoverage |  | NULL |  | OUT-OF-COVERAG |
| eventL1 SEQUENCE { |  |  |  | EVENTL1 |
| l1-Threshold CHOICE { |  |  |  |  |
| rsrp |  | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |  |
| hysteresis |  | 0 |  |  |
| timeToTrigger |  | ms100 | 0.1s |  |
| } |  |  |  |  |
| } |  |  |  |  |
| loggingInterval-r16 |  | ms2560 | 2.56s |  |
| } |  |  |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
|  |  | LoggedMeasurementConfiguration-v1700-IEs |  | LOG-MEAS-R17 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| PERIODICAL | Configuration of LoggedPeriodicalReport |
| EVENT | Configuration of LoggedEventTrigger |
| OUT-OF-COVERAGE | Configuration of outOfCoverage |
| EVENTL1 | Configuration of eventL1 |
| LOG-MEAS-R17 | Configuration of LoggedMeasurement in R17 |

< Unchanged sections omitted >

4.6.3	Radio resource control information elements

< Unchanged sections omitted >

LocationMeasurementInfo

Table 4.6.3-65: LocationMeasurementInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LocationMeasurementInfo ::= CHOICE { |  |  |  |
| eutra-RSTD SEQUENCE (SIZE (1..maxInterRAT-RSTD-Freq)) OF EUTRA-RSTD-Info { | 1 entry |  |  |
| EUTRA-RSTD-Info[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | ARFCN-ValueEUTRA |  |  |
| measPRS-Offset | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.6.3-65A: LoggedMeasurementConfiguration-v1700-IEs

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-v1700-IEs::= SEQUENCE { |  |  |  |  |
| sigLoggedMeasType-r17 |  | Not present |  |  |
|  |  | true |  | SigLogMDT |
| earlyMeasIndication-r17 |  | Not present |  |  |
|  |  | true |  | EarlyMeas |
| areaConfiguration-v1700 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SigLogMDT | Signalling based logged measurements |
| EarlyMeas | Log measurements on early measurement |

–	LogicalChannelConfig

Table 4.6.3-66: LogicalChannelConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LogicalChannelConfig ::= SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| priority | 1 |  |  |
|  | 3 |  | SRB2 |
| prioritisedBitRate | infinity |  |  |
| bucketSizeDuration | ms50 |  |  |
| allowedServingCells | Not present |  |  |
| allowedSCS-List | Not present |  |  |
| maxPUSCH-Duration | Not present |  |  |
| configuredGrantType1Allowed | Not present |  |  |
| logicalChannelGroup | 1 |  | HI |
|  | 2 |  | LO |
|  | 0 |  | SRB1, SRB2, SRB3 |
| schedulingRequestID | SchedulingRequestId |  |  |
| logicalChannelSR-Mask | false |  |  |
| logicalChannelSR-DelayTimerApplied | false |  |  |
| bitRateQueryProhibitTimer | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HI | Used for DRBs with high logical channel priority |
| LO | Used for DRBs with low logical channel priority |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| SRB3 | Establishment of SRB3 |

<End of modified section>