3GPP TSG-RAN5 Meeting #96-e	R5-224807r2

# Electronic Meeting, 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0566 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 demod testability update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current version of the spec, the testability of 256QAM Demod and CSI test points are currently FFS. The testability of AWGN only scenarios are in [] |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the testability of 256QAM Demod and CSI test points. Updated the testability for AWGN only scenarios |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 256QAM and AWGN only test points will remain untestable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1_1, 8.1.1_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Dependent on discussion paper R5-224806 Values needs to be revised based on agreement in discussion paper |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Editorial corrections to newly added table# and its references Based on agreement prop1a in discussion paper R5-224806 for 256QAM, max testable SNR values are revised. R2: since Table 7.1.1_1-2 already exists, numbered the new table as 7.1.1_1-1a. Corrected typo in Testable SNR value for n260/n261 200MHz |  |  |  |  |  |  |  |  |

### 7.1.1_1	Applicability of test requirements due to maximum achievable SNR

Table 7.1.1_1-1 and Table 7.1.1_1-1a specifies the current assumption of maximum testable SNRBB for indirect farfield (IFF), PC3, Max device size ≤ 30 cm under fading conditions.

Table 7.1.1_1-1: maximum testable SNRBB under fading conditions for modulations up to 64 QAM

| Operating Band / Frequency | Maximum testable SNRBB (dB) |  |  |
| --- | --- | --- | --- |
|  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n257 mid | 30.6 | 27.5 | 24.4 |
| n258 mid | 30.6 | 27.5 | 24.4 |
| n259 mid | 20.4 | 17.2 | 14.1 |
| n260 mid | 24.4 | 21.2 | 18.2 |
| n261 mid | 30.6 | 27.5 | 24.4 |

Table 7.1.1_1-1a: maximum testable SNRBB under fading conditions for 256QAM modulation for DEMOD scenarios

| Operating Band / Frequency | Maximum testable SNRBB (dB) |  |  |
| --- | --- | --- | --- |
|  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n257 mid | 29.1 | 25.9 | 22.9 |
| n258 mid | 29.1 | 25.9 | 22.9 |
| n259 mid | 18.8 | 15.6 | 12.5 |
| n260 mid | 22.9 | 19.7 | 16.6 |
| n261 mid | 29.1 | 25.9 | 22.9 |

Based on the current assumption of maximum testable SNRBB, the applicability of test points is defined in Table 7.1.1_1-2 for indirect farfield (IFF), PC3, Max device size size ≤ 30 cm under fading conditions.

Table 7.1.1_1-2: Testability of test requirements due to maximum achievable SNR per band

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 7.2.2.2.1_1 | 1-1 | 100 | Yes | 1.4 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 3.6 | x | x | x | x | x |
|  | 1-3 | 100 | Yes | 14.2 | x | x | x | x | x |
|  | 2-1 | 100 | Yes | 5.8 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 16.0 | x | x | x | x | x |
|  | 2-3 | 50 | Yes | 15.7 | x | x | x | x | x |
|  | 2-4 | 200 | Yes | 15.8 | x | x | - | x | x |
|  | 2-5 | 50 | Yes | 16 | x | x | x | x | x |
|  | 2-6 | 100 | Yes | 20.3 | x | x | - | x | x |
| 7.2.2.2.1_2 | 3-1 | 100 | Yes | 20.7 | x | x | - | x | x |
| 7.2.2.2.1_3 | 1-4 | 50 | Yes | 22.0 | x | x | - | x | x |
| 7.2.2.2.2_1 | 1-1 | 100 | Yes | TBD | TBD | TBD | TBD | TBD | TBD |
| 7.2.2.2.3_1 | 1-1 | 100 | Yes | 3.1 | x | x | x | x | X |
| 7.2A.2.1 | 1,2 | BWagg 200 | Yes | 12.0 | x | x | x | x | x |
|  | 3,4 | BWagg 200 | Yes | 12.0 | TBD | TBD | TBD | TBD | TBD |
| 7.2A.2.2 | 1,2 | BWagg ≤ 200 | Yes | 12.0 | x | x | x | x | x |
|  | 3,4 | BWagg ≥200 | Yes | 12.0 | TBD | TBD | TBD | TBD | TBD |
| 7.3.2.2.1 | 1-1 | 100 | Yes | 7.7 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 4.3 | x | x | x | x | x |
| 7.3.2.2.2 | 2-1 | 100 | Yes | 3.2 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 0.2 | x | x | x | x | x |
| 7.3.2.2.3 | 1-1 | 100 | Yes | 4.7 | x | x | x | x | x |
| 7.5.1 | - | 100 | No | NA | x | x | x | x | x |
|  | - | 200 | No | NA | x | x | x | x | x |
|  | - | 400 | No | NA | TBD | TBD | TBD | TBD | TBD |
| 7.5A.1 | - | BWagg ≤ 200 | No | NA | x | x | x | x | x |
|  | - | BWagg ≤ 200 | No | NA | TBD | TBD | TBD | TBD | TBD |

### Several sections skipped

### 8.1.1_1	Applicability of test requirements due to maximum achievable SNR

The current assumption of maximum testable SNRBB for PC3, Max device size ≤ 30 cm under fading conditions is specified in Tables 7.1.1_1-1 and 7.1.1_1-1a.

The current assumption of maximum testable SNRBB for indirect far field (IFF), PC3, Max device size ≤ 30 cm without fading conditions is specified in Tables  8.1.1_1-0 and 8.1.1_1-1.

Table 8.1.1_1-0: Current assumption of maximum testable SNRBB under fading for modulations up to 256 QAM for CSI scenarios

| Operating Band / Frequency | Maximum testable SNRBB (dB) |  |  |
| --- | --- | --- | --- |
|  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n257 mid | 26.3 | 23.2 | 20.1 |
| n258 mid | 26.3 | 23.2 | 20.1 |
| n259 mid | 16.0 | 12.8 | 9.6 |
| n260 mid | 20.1 | 16.9 | 13.8 |
| n261 mid | 26.3 | 23.2 | 20.1 |

Table 8.1.1_1-1: Current assumption of maximum testable SNRBB without fading for modulations up to 256 QAM

| Operating Band / Frequency | Maximum testable SNRBB (dB) |  |  |
| --- | --- | --- | --- |
|  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n257 mid | 28.7 | 25.5 | 22.5 |
| n258 mid | 28.7 | 25.5 | 22.5 |
| n259 mid | 18.4 | 15.2 | 12.1 |
| n260 mid | 22.5 | 19.3 | 16.3 |
| n261 mid | 28.7 | 25.5 | 22.5 |

Based on the current assumption of maximum testable SNRBB, the applicability of test points is defined in Table 8.1.1_1-2 for indirect far field (IFF), PC3, Max device size ≤ 30 cm under fading conditions.

Table 8.1.1_1-2: Testability of test requirements due to maximum achievable SNR per band

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 |  | n260 |  | n261 |
| 8.2.2.2.1.1 | 1 | 100 | No | 9 | x | x | x |  | x |  | x |
|  | 2 | 100 | No | 15 | x | x | x |  | x |  | x |
| 8.2.2.2.2.1 | 1 | 100 | Yes | 7 | x | x | x |  | x |  | x |
|  | 2 | 100 | Yes | 13 | x | x | x |  | x |  | x |
| 8.2.2.2.2.1_1 | 3 | 50 | Yes | 8 | x | x | x | x |  | x |  |  |
|  | 4 | 50 | Yes | 21 | x | x | TBD | TBD |  | x |  |  |
| 8.4.2.2.1 | 1 | 100 | Yes | 0 | x | x | X |  | x |  | x |
|  | 2 | 100 | Yes | 16 | x | x | x |  | x |  | x |
|  | 3 | 100 | Yes | 16 | x | x | x |  | x |  | x |