3GPP TSG-RAN5 Meeting #96-e	R5-225013r2

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1397 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of default SIB message content for Rel-17 NTN IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Abbreviation of NTN is necessary. To support NTN IoT test cases, SIB31/32 and new SIB combination are needed. SIB1/SIB1-NB needs to be updated for NTN case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Abbreviation updated to add NTN. SIB31(-NB) and SIB32(-NB) were added and SIB1(-NB) were updated. SIB combination 32(8) were added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN IoT PICS will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.4.3 and 8.1.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: Add more SI combination for eMTC and NB-IoT cases; Updated SIB31, SIB1-NB and SIB31-NB r2: update the SIB31 periodicity in 4.4.3.4 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

3.3	Abbreviations

For the purposes of the present document, the abbreviations specified in TR 21.905 [1] apply, with any additional abbreviations specified below:

1xRTT	1x Radio Transmission Technology

BR	Bandwidth Reduced

BR-BCCH	Bandwidth Reduced Broadcast Control Channel

BL	Bandwidth reduced Low complexity

CA	Carrier Aggregation

CC	Component Carrier

CE	Coverage Enhancement

DRB	(user) Data Radio Bearer

EARFCN 	E-UTRA Absolute Radio Frequency Channel Number

ECM	EPS Connection Management

eIMTA	Enhanced Interference Management and Traffic Adaptation

EMM	EPS Mobility Management

ENB	Evolved Node B

EPRE	Energy Per Resource Element

ESM	EPS Session Management

HRPD	High Rate Packet Data

MAC	Media Access Control

MBS	Metropolitan Beacon System

MCS	Mission Critical Services

MTC	Machine Type Communication

NTN	Non-Terrestrial Networks

OFDM	Orthogonal Frequency Division Multiplexing

RBs	Resource Blocks

ROHC 	Robust Header Compression

SS	System Simulator

TH	Temperature High

TL	Temperature Low

TP	Transmission Point

V2X	Vehicle-to-Everything

VH	Higher extreme Voltage

VL	Lower extreme Voltage

xCH_RA	xCH-to-RS EPRE ratio for the channel xCH in all transmitted OFDM symbols not containing RS

xCH_RB	xCH-to-RS EPRE ratio for the channel xCH in all transmitted OFDM symbols containing RS

[Skip unchange text]

4.4.3	Common parameters for simulated E-UTRA cells

The parameters specified in this sub clause apply to all simulated E-UTRA cells unless otherwise specified.

4.4.3.1	Common configurations of system information blocks

For BL/CE testing the same content of system information blocks SIB2 to SIB20 are used for broadcasting on BCCH and/or BCCH-BR.

4.4.3.1.1	Combinations of system information blocks

The combination of system information blocks required by a test case depends on the test case scenario. In this clause, the following combinations of system information blocks are defined.

Combination 1 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario

- E-UTRA TDD single cell scenario

- E-UTRA FDD intra-frequency multi cell scenario

- E-UTRA TDD intra-frequency multi cell scenario

- E-UTRA FDD and E-UTRA TDD dual mode multi cell roaming scenario

- 3GPP inter-RAT E-UTRA FDD + NR FDD multi cell scenario for RRM

- 3GPP inter-RAT E-UTRA FDD + NR TDD multi cell scenario for RRM

- 3GPP inter-RAT E-UTRA TDD + NR FDD multi cell scenario for RRM

- 3GPP inter-RAT E-UTRA TDD + NR TDD multi cell scenario for RRM

Combination 2 applies to the following test case scenarios:

- E-UTRA FDD intra-frequency multi cell scenario with neighbouring cell related information

- E-UTRA TDD intra-frequency multi cell scenario with neighbouring cell related information

Combination 3 applies to the following test case scenarios:

- E-UTRA FDD inter-frequency multi cell scenario

- E-UTRA TDD inter-frequency multi cell scenario

- E-UTRA FDD inter-band multi cell scenario

- E-UTRA TDD inter-band multi cell scenario

- E-UTRA FDD and E-UTRA TDD dual mode multi cell non-roaming scenario

- E-UTRA FDD intra-band carrier aggregation component carriers cell scenario

- E-UTRA FDD inter-band carrier aggregation component carriers cell scenario

- E-UTRA TDD intra-band carrier aggregation component carriers cell scenario

- E-UTRA FDD and E-UTRA TDD inter-band carrier aggregation component carriers cell scenario

- E-UTRA FDD dual connectivity cell scenario

- E-UTRA TDD dual connectivity cell scenario

Combination 4 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + UTRA FDD multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + UTRA LCR TDD multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + UTRA FDD multi cell scenario

Combination 5 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + GERAN multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + GERAN multi cell scenario

Combination 6 applies to the following test case scenarios:

- 3GPP2 inter-RAT E-UTRA FDD + HRPD multi cell scenario

- 3GPP2 inter-RAT E-UTRA TDD + HRPD multi cell scenario

- 3GPP2 inter-RAT E-UTRA FDD + 1xRTT multi cell scenario

- 3GPP2 inter-RAT E-UTRA TDD + 1xRTT multi cell scenario

Combination 7 applies to the following test case scenarios:

- E-UTRA FDD + home eNB multi cell scenario

- E-UTRA TDD + home eNB multi cell scenario

Combination 8 applies to the following test case scenarios:

- E-UTRA FDD ETWS single cell scenario

- E-UTRA TDD ETWS single cell scenario

Combination 9 applies to the following test case scenarios:

- E-UTRA FDD inter-frequency + 3GPP inter-RAT UTRA multi-cell scenario

- E-UTRA TDD inter-frequency + 3GPP inter-RAT UTRA multi-cell scenario

Combination 10 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + UTRA FDD + GERAN multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + UTRA LCR TDD + GERAN multi cell scenario.

Combination 10a applies to the following test case scenarios:

- E-UTRA FDD inter-frequency + 3GPP inter-RAT E-UTRA FDD + UTRA FDD + GERAN multi cell scenario

- E-UTRA TDD inter-frequency + 3GPP inter-RAT E-UTRA TDD + UTRA LCR TDD + GERAN multi cell scenario

Combination 11 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + UTRA FDD + home eNB multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + UTRA LCR TDD + home eNB multi cell scenario

Combination 12 applies to the following test case scenarios:

- E-UTRA FDD inter-frequency + GERAN cell scenario

- E-UTRA TDD inter-frequency + GERAN cell scenario

Combination 13 applies to the following test case scenarios:

- E-UTRA FDD inter-frequency + home eNB

- E-UTRA TDD inter-frequency + home eNB

Combination 14 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + GERAN + home eNB multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + GERAN + home eNB multi cell scenario

Combination 15 applies to the following test case scenarios:

- 3GPP E-UTRA FDD single cell scenario + MBMS

- 3GPP E-UTRA TDD single cell scenario + MBMS

- 3GPP E-UTRA FDD intra-frequency multi cell scenario + MBMS

- 3GPP E-UTRA TDD intra-frequency multi cell scenario + MBMS

Combination 16 applies to the following test case scenarios:

- 3GPP E-UTRA FDD inter-frequency multi cell scenario + MBMS

- 3GPP E-UTRA TDD inter-frequency multi cell scenario + MBMS

Combination 17 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + CMAS single cell scenario

- 3GPP E-UTRA TDD + CMAS single cell scenario

Combination 18 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + MBMS inter-frequency multi cell scenario (non-MBMS cell broadcasting SIB15)

- 3GPP E-UTRA TDD + MBMS inter-frequency multi cell scenario (non-MBMS cell broadcasting SIB15)

- 3GPP E-UTRA FDD + SC-PTM inter-frequency multi cell scenario (non- SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA TDD + SC-PTM inter-frequency multi cell scenario (non- SC-PTM cell broadcasting SIB15)

Combination 19 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + MBMS intra-frequency multi cell scenario (MBMS cell broadcasting SIB15)

- 3GPP E-UTRA TDD + MBMS intra-frequency multi cell scenario (MBMS cell broadcasting SIB15)

Combination 20 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + MBMS inter-frequency multi cell scenario (MBMS cell broadcasting SIB15)

- 3GPP E-UTRA TDD + MBMS inter-frequency multi cell scenario (MBMS cell broadcasting SIB15)

- 3GPP E-UTRA FDD + MBMS inter-band multi cell scenario (MBMS cell broadcasting SIB15)

- 3GPP E-UTRA TDD + MBMS inter-band multi cell scenario (MBMS cell broadcasting SIB15)

Combination 21 applies to the following test case scenarios:

- E-UTRA FDD SIMTC single cell scenario

- E-UTRA TDD SIMTC single cell scenario

Combination 22 applies to the following test case scenarios:

- RAN assisted WLAN offload: E-UTRA FDD + WLAN AP scenarios

- RAN assisted WLAN offload: E-UTRA TDD + WLAN AP scenarios

Combination 23 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario + sidelink direct communication

- E-UTRA TDD single cell scenario + sidelink direct communication

- E-UTRA FDD intra-frequency multi cell scenario + sidelink direct communication

- E-UTRA TDD intra-frequency multi cell scenario + sidelink direct communication

Combination 24 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario + sidelink direct discovery

- E-UTRA TDD single cell scenario + sidelink direct discovery

- E-UTRA FDD intra-frequency multi cell scenario + sidelink direct discovery

- E-UTRA TDD intra-frequency multi cell scenario + sidelink direct discovery

Combination 25 applies to the following test case scenarios:

- 3GPP E-UTRA FDD single cell scenario + SC-PTM

- 3GPP E-UTRA TDD single cell scenario + SC-PTM

- 3GPP E-UTRA FDD intra-frequency multi cell scenario + SC-PTM

- 3GPP E-UTRA TDD intra-frequency multi cell scenario + SC-PTM

Combination 26 applies to the following test case scenarios:

- 3GPP E-UTRA FDD inter-frequency multi cell scenario + SC-PTM

- 3GPP E-UTRA TDD inter-frequency multi cell scenario + SC-PTM

Combination 27 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + SC-PTM intra-frequency multi cell scenario (SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA TDD + SC-PTM intra-frequency multi cell scenario (SC-PTM cell broadcasting SIB15)

Combination 28 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + SC-PTM inter-frequency multi cell scenario (SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA TDD + SC-PTM inter-frequency multi cell scenario (SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA FDD + SC-PTM inter-band multi cell scenario (SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA TDD + SC-PTM inter-band multi cell scenario (SC-PTM cell broadcasting SIB15)

Combination 29 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario + V2X sidelink communication.

- E-UTRA TDD single cell scenario + V2X sidelink communication.

- E-UTRA FDD intra-frequency multi cell scenario + V2X sidelink communication.

- E-UTRA TDD intra-frequency multi cell scenario + V2X sidelink communication.

- E-UTRA FDD inter-frequency + V2X sidelink communication with inter-frequency scheduling

- E-UTRA TDD inter-frequency + V2X sidelink communication with inter-frequency scheduling

- E-UTRA FDD inter-band + V2X sidelink communication with inter-frequency scheduling

- E-UTRA TDD inter-band + V2X sidelink communication with inter-frequency scheduling

Combination 30 applies to the following test case scenarios:

- E-UTRA FDD intra-band carrier aggregation component carriers cell scenario + E-UTRA FDD intra-frequency neighbour.

- E-UTRA FDD inter-band carrier aggregation component carriers cell scenario+ E-UTRA FDD intra-frequency neighbour.

- E-UTRA TDD intra-band carrier aggregation component carriers cell scenario+ E-UTRA FDD intra-frequency neighbour.

- E-UTRA FDD and E-UTRA TDD inter-band carrier aggregation component carriers cell scenario+ E-UTRA FDD intra-frequency neighbour.

Combination 31 applies to the following test case scenarios:

-	3GPP inter-RAT E-UTRA FDD + NR FDD multi cell scenario

-	3GPP inter-RAT E-UTRA FDD + NR TDD multi cell scenario

-	3GPP inter-RAT E-UTRA TDD + NR FDD multi cell scenario

-	3GPP inter-RAT E-UTRA TDD + NR TDD multi cell scenario

Combination 32 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario + NTN

Combination 33 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD intra-frequency multi cell scenario with neighbouring cell related information + NTN

- E-UTRA TDD intra-frequency multi cell scenario with neighbouring cell related information + NTN

Table 4.4.3.1.1-1: Combinations of system information blocks

|  | System information block type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Combination No. | SIB2 | SIB3 | SIB4 | SIB5 | SIB6 | SIB7 | SIB8 | SIB9 | SIB10 | SIB11 | SIB12 | SIB13 | SIB14 | SIB 15 | SIB 17 | SIB18 | SIB19 | SIB20 | SIB21 | SIB24 | SIB31 |
| 1 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | X | X |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | X | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | X | X |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | X | X | X |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | X | X |  |  |  |  |  |  | X | X |  |  |  |  |  |  |  |  |  |  |  |
| 9 | X | X |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | X | X |  |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10a | X | X |  | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | X | X | X |  | X |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | X | X |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | X | X | X | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | X | X | X |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| 16 | X | X |  | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| 17 | X | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| 18 | X | X |  | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| 19 | X | X |  |  |  |  |  |  |  |  |  | X |  | X |  |  |  |  |  |  |  |
| 20 | X | X |  | X |  |  |  |  |  |  |  | X |  | X |  |  |  |  |  |  |  |
| 21 | X | X |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| 22 | X | X |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| 23 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| 24 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| 25 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| 26 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| 27 | X | X |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  | X |  |  |  |
| 28 | X | X |  | X |  |  |  |  |  |  |  |  |  | X |  |  |  | X |  |  |  |
| 29 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| 30 | X | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| 32 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |
| 33 | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |

4.4.3.1.2	Scheduling of system information blocks

The scheduling configurations for combinations of system information blocks are defined in the following tables.

Table 4.4.3.1.2-1: Scheduling for combination 1

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |

Table 4.4.3.1.2-2: Scheduling for combination 2

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |

Table 4.4.3.1.2-3: Scheduling for combination 3

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |

Table 4.4.3.1.2-4: Scheduling for combination 4

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB6 |

Table 4.4.3.1.2-5: Scheduling for combination 5

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB7 |

Table 4.4.3.1.2-6: Scheduling for combination 6

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB8 |

Table 4.4.3.1.2-7: Scheduling for combination 7

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB9 |

Table 4.4.3.1.2-8: Scheduling for combination 8

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB11 |
| 4 | See sub clause 4.4.3.4 | SIB10 |

Table 4.4.3.1.2-9: Scheduling for combination 9

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5, SIB6 |

Table 4.4.3.1.2-10: Scheduling for combination 10

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB6 |
| 4 | See sub clause 4.4.3.4 | SIB7 |

Table 4.4.3.1.2-10a: Scheduling for combination 10a

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5, SIB6 |
| 4 | See sub clause 4.4.3.4 | SIB7 |

Table 4.4.3.1.2-11: Scheduling for combination 11

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB6 |
| 5 | See sub clause 4.4.3.4 | SIB9 |

Table 4.4.3.1.2-12: Scheduling for combination 12

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB7 |

Table 4.4.3.1.2-13: Scheduling for combination 13

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB5 |

Table 4.4.3.1.2-14: Scheduling for combination 14

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB7 |
| 5 | See sub clause 4.4.3.4 | SIB9 |

Table 4.4.3.1.2-15: Scheduling for combination 15

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB13 |

Table 4.4.3.1.2-16: Scheduling for combination 16

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB13 |

Table 4.4.3.1.2-17: Scheduling for combination 17

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB12 |

Table 4.4.3.1.2-18: Scheduling for combination 18

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB15 |

Table 4.4.3.1.2-19: Scheduling for combination 19

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB13 |
| 4 | See sub clause 4.4.3.4 | SIB15 |

Table 4.4.3.1.2-20: Scheduling for combination 20

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB13 |
| 5 | See sub clause 4.4.3.4 | SIB15 |

Table 4.4.3.1.2-21: Scheduling for combination 21

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB14 |

Table 4.4.3.1.2-22: Scheduling for combination 22

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB17 |

Table 4.4.3.1.2-23: Scheduling for combination 23

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB18 |

Table 4.4.3.1.2-24: Scheduling for combination 24

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB19 |

Table 4.4.3.1.2-25: Scheduling for combination 25

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB20 |

Table 4.4.3.1.2-26: Scheduling for combination 26

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB20 |

Table 4.4.3.1.2-27: Scheduling for combination 27

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB15 |
| 4 | See sub clause 4.4.3.4 | SIB20 |

Table 4.4.3.1.2-28: Scheduling for combination 28

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB15 |
| 5 | See sub clause 4.4.3.4 | SIB20 |

Table 4.4.3.1.2-29: Scheduling for combination 29

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB21 |

Table 4.4.3.1.2-30: Scheduling for combination 30

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB5 |

Table 4.4.3.1.2-31: Scheduling for combination 31

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB24 |

Table 4.4.3.1.2-32: Scheduling for combination 32

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB31 |

Table 4.4.3.1.2-33: Scheduling for combination 33

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB31 |

4.4.3.2	Common contents of system information messages

-	MasterInformationBlock

The MasterInformationBlock includes the system information transmitted on BCH.

Table 4.4.3.2-1: MasterInformationBlock

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock ::= SEQUENCE { |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| phich-Config SEQUENCE {} | PHICH-Config-DEFAULT | See subclause 4.6.3 |  |
| systemFrameNumber | A valid value as defined in TS 36.331 [17] |  |  |
| schedulingInfoSIB1-BR-r13 | 0 | Value 0 means that SystemInformationBlockType1-BR is not scheduled. |  |
| schedulingInfoSIB1-BR-r13 | 10 | 4 PDSCH repetitions and 504 bits TBS carrying for PDSCH carrying SystemInformationBlockType1-BR (TS 36.213 [23] Table 7.1.6.1 and Table 7.1.7.2.7-1) | CEmodeA, CEmodeB |
| spare | '0000 0000 00'B |  |  |
| spare | '0000 0'B |  | CEmodeA, CEmodeB |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |

-	SystemInformation

The SystemInformation message is used to convey one or more System Information Blocks. All the SIBs included are transmitted with the same periodicity.

Table 4.4.3.2-2: SystemInformation

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| systemInformation-r8 SEQUENCE { |  |  |  |
| sib-TypeAndInfo SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {} | See subclause 4.4.3.1 |  |  |
| criticalExtensionsFuture SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	SystemInformation-BR-r13

The SystemInformation-BR-r13 message is used to convey one or more System Information Blocks for Bandwith Redused cell envirionment . All the SIBs included are transmitted with the same periodicity.

Table 4.4.3.2-2A: SystemInformation-BR-r13

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformation-BR-r13 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| systemInformation-r8 SEQUENCE { |  |  |  |
| sib-TypeAndInfo SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {} | See subclause 4.4.3.1 |  |  |
| criticalExtensionsFuture SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	SystemInformationBlockType1

SystemInformationBlockType1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information.

Table 4.4.3.2-3: SystemInformationBlockType1

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| plmn-Identity[1] SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | See table 4.4.2-2 | For NAS test cases, see table 6.3.2.2-1. |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | See table 4.4.2-2 | For NAS test cases, see table 6.3.2.2-1. |  |
| } |  |  |  |
| cellReservedForOperatorUse[1] | notReserved |  |  |
| } |  |  |  |
| trackingAreaCode | See table 4.4.2-2 | For NAS test cases, see table 6.3.2.2-1. |  |
| cellIdentity | Cell ID for the simulated cell |  |  |
| cellBarred | notBarred |  |  |
|  | barred |  | NTN |
| intraFreqReselection | notAllowed |  |  |
| csg-Indication | FALSE |  |  |
| csg-Identity | Not present |  |  |
| } |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 (-140 dBm) | For RF/RRM test cases | RF |
|  | -53 (-106 dBm) | For signalling test cases in normal coverage |  |
|  | -27 (-54dBm) | For signalling test cases in enhanced coverage The value has been selected such that the UE at signal levels for ”Serving cell” and “Suitable neighbour intra-frequency cell” are applied then the cell selection criterion for normal coverage is not fullfilled. | SIG AND ( CEmodeA OR CEmodeB) |
| q-RxLevMinOffset | Not present |  |  |
| } |  |  |  |
| p-Max | Not present |  |  |
| freqBandIndicator | Operating band under test. |  |  |
| freqBandIndicator | 64 | maxFBI | Band > 64 |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE {} | See subclause 4.4.3.1 |  |  |
| tdd-Config SEQUENCE {} | Not present |  | FDD |
| tdd-Config SEQUENCE {} | TDD-Config-DEFAULT | See subclause 4.6.3 | TDD |
| si-WindowLength | ms20 | To allow sufficient number of retransmissions. |  |
| systemInfoValueTag | 0 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| freqBandIndicator-v9e0 | Operating band under test. |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | True | Support IMS emergency call in limited service mode. |  |
| cellSelectionInfo-v920 SEQUENCE {} | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  | QBASED |  |
| q-QualMin-r9 | -20 (-20dB) |  |  |  |
| q-QualMinOffset-r9 | Not present |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { | Not present |  |  |
| tdd-Config-v1130 | Not present |  |  |  |  |
| cellSelectionInfo-v1130 | Not present |  |  |  |  |
| nonCriticalExtension | Not present |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UECAT0 |  |  |
| cellAccessRelatedInfo-v1250 SEQUENCE { |  |  |  |  |  |
| category0Allowed-r12 | True | Cat 0 Allowed |  |  |  |
| } |  |  |  |  |  |
| cellSelectionInfo-v1250 | Not present |  |  |  |  |
| freqBandIndicatorPriority-r12 | Not present |  |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |  |  |
| hyperSFN-r13 | Not present |  |  |  |  |
| eDRXAllowed-r13 | Not present |  |  |  |  |
| cellSelectionInfoCE-r13 | Not present |  |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 | Not present |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | eCalloverIMS |  |  |
| eCallOverIMS-Support-r14 | True | Support eCall over IMS services. |  |  |  |
| tdd-Config-v1430 | Not present |  |  |  |  |
| cellAccessRelatedInfoList-r14 SEQUENCE { } | Not present |  |  |  |  |
| nonCriticalExtension | Not present |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | NTN |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| cellAccessRelatedInfo-v1700 SEQUENCE { |  |  |  |  |  |
| cellBarred-NTN-r17 | notBarred |  |  |  |  |
| } |  |  |  |  |  |
| nonCriticalExtension | Not present |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| QBASED | This condition applies to Quality based cell (re)selection signalling test cases. |
| UECAT0 | This condition applies when UE under test is a UE of Category 0. |
| RF | For RF, performance and RRM testing |
| SIG | For protocol testing |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |
| Band > 64 | If band > 64 is selected |
| eCalloverIMS | eCall over IMS test environment |
| NTN | Non-Terrestrial Networks test environment |

-	SystemInformationBlockType1-BR-r13

SystemInformationBlockType1-BR-r13 contains information relevant when evaluating if a UE in a Bandwith Reduced cell environment is allowed to access a cell and defines the scheduling of other system information.

Table 4.4.3.2-3A: SystemInformationBlockType1-BR-r13

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| plmn-Identity[1] SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | See table 4.4.2-2 | For NAS test cases, see table 6.3.2.2-1. |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | See table 4.4.2-2 | For NAS test cases, see table 6.3.2.2-1. |  |
| } |  |  |  |
| cellReservedForOperatorUse[1] | notReserved |  |  |
| } |  |  |  |
| trackingAreaCode | See table 4.4.2-2 | For NAS test cases, see table 6.3.2.2-1. |  |
| cellIdentity | Cell ID for the simulated cell |  |  |
| cellBarred | notBarred |  |  |
| intraFreqReselection | notAllowed |  |  |
| csg-Indication | FALSE |  |  |
| csg-Identity | Not present |  |  |
| } |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 (-140 dBm) | For RF/RRM test cases | RF |
|  | -27 (-54 dBm) | For signalling test cases | SIG and ENHANCED COVERAGE |
|  | -53 (-106 dBm) | For signalling test cases | SIG and NORMAL COVERAGE |
| q-RxLevMinOffset | Not present |  |  |
| } |  |  |  |
| p-Max | Not present |  |  |
| freqBandIndicator | Operating band under test. |  |  |
| freqBandIndicator | 64 | maxFBI | Band > 64 |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE {} | See subclause 4.4.3.1 |  |  |
| tdd-Config SEQUENCE {} | Not present |  | FDD |
| tdd-Config SEQUENCE {} | TDD-Config-DEFAULT | See subclause 4.6.3 | TDD |
| si-WindowLength | ms20 | To allow sufficient number of retransmissions. |  |
| systemInfoValueTag | 0 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| freqBandIndicator-v9e0 | Operating band under test. |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | True | Support IMS emergency call in limited service mode. |  |
| cellSelectionInfo-v920 SEQUENCE {} | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  | QBASED |
| q-QualMin-r9 | -20 (-20dB) |  |  |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| tdd-Config-v1130 | Not present |  |  |
| cellSelectionInfo-v1130 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-v1250 SEQUENCE { |  |  |  |
| category0Allowed-r12 | Not Present |  |  |
| category0Allowed-r12 | True | Cat 0 Allowed | UECAT0 |
| } |  |  |  |
| cellSelectionInfo-v1250 | Not present |  |  |
| freqBandIndicatorPriority-r12 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| hyperSFN-r13 | Not present |  |  |
| eDRX-Allowed-r13 | Not present |  |  |
| eDRX-Allowed-r13 | True |  | eDRX |
| cellSelectionInfoCE-r13 | CellSelectionInfoCE-r13-DEFAULT |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| si-WindowLength-BR-r13 | ms20 | To allow sufficient number of retransmissions. |  |
| si-RepetitionPattern-r13 | every2ndRF | ENUMERATED {everyRF, every2ndRF, every4thRF, every8thRF} |  |
| schedulingInfoList-BR-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE { | n entries | n is the same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| si-Narrowband-r13[1] | 1 |  |  |
| si-Narrowband-r13[1] | 2 |  | 20MHz BW |
| si-TBS-r13[1] | b712 |  |  |
| …. |  |  |  |
| si-Narrowband-r13[n] | 1 |  |  |
| si-Narrowband-r13[n] | 2 |  | 20MHz BW |
| si-TBS-r13[n] | b712 |  |  |
| } |  |  |  |
| fdd-DownlinkOrTddSubframeBitmapBR-r13 | Not present | All non-MBSFN subframes are considered as valid subframes for FDD downlink or TDD transmissions |  |
| fdd-UplinkSubframeBitmapBR-r13 | Not present | All FDD uplink subframes are considered as valid subframes |  |
| startSymbolBR-r13 | 3 |  | FDD |
|  | 2 |  | TDD |
| si-HoppingConfigCommon-r13 | off |  |  |
| si-ValidityTime-r13 | True |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | n entries | n is the same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | 0 |  |  |
| …. |  |  |  |
| SystemInfoValueTagSI-r13[n] | 0 |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | FREQ HOPPING |
| freqHoppingParametersDL-r13 SEQUENCE { |  |  |  |
| mpdcch-pdsch-HoppingNB-r13 | nb4 | 4 narrowbands |  |
| interval-DLHoppingConfigCommonModeA-r13 CHOICE { |  |  |  |
| interval-FDD-r13 | int4 |  | FDD |
| interval-TDD-r13 | Int5 |  | TDD |
| } |  |  |  |
| interval-DLHoppingConfigCommonModeB-r13 CHOICE { |  |  |  |
| interval-FDD-r13 | int16 |  | FDD |
| interval-TDD-r13 | int20 |  | TDD |
| } |  |  |  |
| mpdcch-pdsch-HoppingOffset-r13 | 2 | INTEGER (1..16 (maxAvailNarrowBands-r13)) |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| QBASED | This condition applies to Quality based cell (re)selection signalling test cases. |
| UECAT0 | This condition applies when UE under test is a UE of Category 0. |
| eDRX | This condition applies when UE under test is in enhanced coverage and eDRX is allowed. |
| RF | For RF, performance and RRM testing |
| SIG | For Signalling tests |
| FREQ HOPPING | This condition applies when MPDCCH/PDSCH frequency hopping is used |
| NORMAL COVERAGE | Normal coverage testing |
| ENHANCED COVERAGE | This condition applies for enhanced coverage testing |
| 20MHz BW | Used for cells with 20MHz bandwidth |
| Band > 64 | If band > 64 is selected |

4.4.3.3	Common contents of system information blocks

-	SystemInformationBlockType2

The IE SystemInformationBlockType2 contains radio resource configuration information that is common for all UEs.

Table 4.4.3.3-1: SystemInformationBlockType2

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringInfo SEQUENCE {} | Not present |  |  |
| radioResourceConfigCommon SEQUENCE {} | RadioResourceConfigCommonSIB-DEFAULT | See subclause 4.6.3 |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t300 | ms1000 | Typical value in real network |  |
| t301 | ms1000 | Typical value in real network |  |
| t310 | ms1000 | Typical value in real network |  |
| n310 | n1 |  |  |
| t311 | ms10000 | Typical value in real network |  |
| n311 | n1 |  |  |
| } |  |  |  |
| freqInfo SEQUENCE { |  |  |  |
| ul-CarrierFreq | Not present | Default UL EARFCN applies |  |
|  | EARFCN of UL in test frequency Table 4.3.1.1.24-1 |  | Band 24 High range |
| ul-Bandwidth | Not Present |  |  |
| additionalSpectrumEmission | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4-1. |  |
| additionalSpectrumEmission | 32 | When IE AdditionalSpectrumEmission - v10l0 is used, 32 should be set to IE AdditionalSpectrumEmission. | NS value > 32 |
| } |  |  |  |
| mbsfn-SubframeConfigList | Not present |  |  |
| mbsfn-SubframeConfigList SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF SEQUENCE { |  |  | MBMS |
| radioframeAllocationPeriod | n4 |  |  |
| radioframeAllocationOffset | 1 |  | FDD |
|  | 0 |  | TDD |
| subframeAllocation CHOICE{ |  |  |  |
| oneFrame | ‘100000’ B |  | FDD |
|  | ‘000010’ B |  | TDD |
| } |  |  |  |
| } |  |  |  |
| timeAlignmentTimerCommon | sf750 | ‘sf750’ is applicable to the widest range of mobility (up to about 360km/h). |  |
| lateNonCriticalExtension SEQUENCE { | Not present |  |  |
| lateNonCriticalExtension SEQUENCE { |  | OCTET STRING (CONTAINING SystemInformationBlockType2-v8h0-IEs) | NS value > 32 |
| multiBandInfoList | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  | OCTET STRING (CONTAINING SystemInformationBlockType2-v10m0-IEs) |  |
| freqInfo-v10l0 SEQUENCE { |  |  |  |
| additionalSpectrumEmission-v10l0 | Specify the network signaling value corresponding to the E-UTRA Band defined in TS 36.101 table 6.2.4-1 |  |  |
| multiBandInfoList-v10l0 | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| Dummy | Not present | This field is not used in the specification. If received it shall be ignored by the UE. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| unicastFreqHoppingInd-r13 | true | Enabled only for CE Mode test cases requiring frequency hopping. | CEfreqHopp |
| cIoT-EPS-OptimisationInfo-r13 SEQUENCE (SIZE (1.. maxPLMN-r11)) OF SEQUENCE { |  |  | CIoT-test |
| up-CIoT-EPS-Optimisation-r13 | true |  |  |
| cp-CIoT-EPS-Optimisation-r13 | true |  |  |
| attachWithoutPDN-Connectivity-r13 | false |  |  |
| } |  |  |  |
| useFullResumeID-r13 | Not present |  |  |
| plmn-InfoList-r15 | Not present |  |  |
| plmn-InfoList-r15 SEQUENCE (SIZE (1..maxPLMN-r11)) OF SEQUENCE { | 1 entry |  | 5G |
| upperLayerIndication-r15[1] | true |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition |  | Explanation |  |
| --- | --- | --- | --- |
| MBMS |  | MBMS cell environment |  |
| FDD |  | FDD cell environment |  |
| TDD |  | TDD cell environment |  |
| CEfreqHopp |  | Used when frequency hopping is used in CE test cases |  |
| CIoT-test |  | CIoT test environment |  |
| NS value > 32 |  | When A-MPR applies with Network Signaling value > 32 |  |
| 5G |  | Applicable to 5G test cases defined in TS 38.523-1 [71] |  |
| Band 24 High range |  | Applicable for test IDs with Band 24 High range |  |

-	SystemInformationBlockType3

The IE SystemInformationBlockType3 contains cell re-selection information common for intra-frequency, inter-frequency and/or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

Table 4.4.3.3-2: SystemInformationBlockType3

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |  |
| q-Hyst | dB0 | To reduce interference between intra-frequency multiple cells |  |  |
| speedStateReselectionPars SEQUENCE {} | Not present |  |  |  |
| } |  |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |  |
| s-NonIntraSearch | Not present |  |  |  |
| threshServingLow | 0 (0 dB) | Typical value in real network |  |  |
| cellReselectionPriority | 4 | A middle value in the range has been selected. |  |  |
| } |  |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin | -70 (-140 dBm) | For RF/RRM test cases |  |  |
|  | -106 dBm | For signalling test cases |  |  |
| p-Max | Not present |  |  |  |
| s-IntraSearch | Not present |  |  |  |
| allowedMeasBandwidth | Not present | The downlink bandwidth of the serving cell applies. |  |  |
| presenceAntennaPort1 | FALSE |  |  |  |
|  | TRUE | At least two cell-specific antenna ports are used in all neighbouring cells. | All neighCells with port1 |  |
| neighCellConfig | '01'B (No MBSFN subframes are present in all neighbour cells) | MBSFN doesn’t apply by default. |  |  |
| t-ReselectionEUTRA | 0 | Typical value in real network |  |  |
| t-ReselectionEUTRA-SF | Not present |  |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| s-IntraSearch-v920 SEQUENCE {} | Not present |  |  |
| s-NonIntraSearch-v920 SEQUENCE {} | Not present |  |  |
| q-QualMin-r9 | Not present |  |  |
|  | -20 (-20dB) |  | QBASED |
| threshServingLowQ-r9 | Not present |  |  |
|  | 3 (3dB) |  | QBASED |
| cellSelectionInfoCE-r13 | CellSelectionInfoCE-r13-DEFAULT |  | CEmodeA, CEmodeB |
|  | Not present |  |  |
| t-ReselectionEUTRA-CE-r13 | 0 |  | CEmodeA, CEmodeB |
|  | Not present |  |  |
| } |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| QBASED | See the definition below table 4.4.3.2-3. |
| All neighCells with port1 | Used for all neighbouring cells with at least two cell-specific antenna ports |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |

-	SystemInformationBlockType4

The IE SystemInformationBlockType4 contains neighbouring cell related information relevant only for intra-frequency cell re-selection. The IE includes cells with specific re-selection parameters as well as blacklisted cells.

Table 4.4.3.3-3: SystemInformationBlockType4

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE {} | Not present | Not required unless Qoffset configuration is tested. When Qoffset configuration is tested, see table 6.3.1.1-1. |  |
| intraFreqBlackCellList SEQUENCE (SIZE (1..maxCellBlack)) OF SEQUENCE {} | Not present | Not required unless Blacklisted cell list configuration is tested. When Blacklisted cell list configuration is tested, see table 6.3.1.1-1. |  |
| csg-PhysCellIdRange SEQUENCE {} | Not present |  | NonCSG |
| csg-PhysCellIdRange SEQUENCE { |  |  | CSG |
| start | Physical Cell ID of the cell on which this SIB is transmitted |  |  |
| range | Not present | The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies. |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NonCSG | SIB4 transmitted on a non-CSG cell |
| CSG | SIB4 transmitted on a CSG cell |

-	SystemInformationBlockType5

The IE SystemInformationBlockType5 contains information relevant only for inter-frequency cell re-selection i.e. information about other E-UTRA frequencies and inter-frequency neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

Table 4.4.3.3-4: SystemInformationBlockType5

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | The same number of entries as the configured inter-freq carriers. For Signalling test cases except NAS, see table 6.3.1.2-1. For NAS test cases when cells are on same PLMN, see table 6.3.2.3.2-1. | n denotes the index of the entry |  |  |
| dl-CarrierFreq[n] | Downlink EARFCN under test For Signalling test cases except NAS, see table 6.3.1.2-1. For NAS test cases when cells are on same PLMN, see table 6.3.2.3.2-1. |  |  |  |
| dl-CarrierFreq[n] | 65535 | maxEARFCN | Band > 64 |  |
| q-RxLevMin[n] | -65 (-130 dBm) | For RF/RRM test cases |  |  |
|  | -106 dBm | For signalling test cases |  |  |
| p-Max[n] | Not present |  |  |  |
| t-ReselectionEUTRA[n] | 0 | Typical value in real network |  |  |
| t-ReselectionEUTRA-SF[n] | Not present | Not required unless speed-dependent cell re-selection is tested. |  |  |
| threshX-High[n] | 2 (4 dB) | This value should be higher than threshServingLow of the serving cell to avoid ping-pong with lower priority cells. |  |  |
| threshX-Low[n] | 1 (2 dB) |  |  |  |
| allowedMeasBandwidth[n] | See subclause 4.4.3.4 | Channel-bandwidth-dependent parameter |  |  |
| presenceAntennaPort1[n] | FALSE |  |  |  |
|  | TRUE | At least two cell-specific antenna ports are used in all neighbouring cells. | All neighCells with port1 |  |
| cellReselectionPriority[n] | 4 | The same priority as the one used for serving cell in SIB 3. |  |  |
| neighCellConfig[n] | '01'B (No MBSFN subframes are present in all neighbour cells) | MBSFN doesn’t apply by default. |  |  |
| q-OffsetFreq[n] | dB0 | Qoffset doesn’t apply by default. |  |  |
| interFreqNeighCellList[n] SEQUENCE (SIZE (1..maxCellInter)) OF SEQUENCE {} | Not present | Not required unless Qoffset configuration is tested. |  |  |
| interFreqBlackCellList[n] SEQUENCE (SIZE (1..maxCellBlack)) OF SEQUENCE {} | Not present | Not required unless Blacklisted cell list configuration is tested. |  |  |
| q-QualMin-r9[n] | Not present |  |  |
|  | -20 (-20dB) |  | QBASED |
| threshX-Q-r9[n] SEQUENCE {} | Not present |  |  |
| threshX-Q-r9[n] SEQUENCE { |  |  | QBASED |
| threshX-HighQ-r9 | 5 (5dB) |  |  |
| threshX-LowQ-r9 | 5 (5dB) |  |  |
| } |  |  |  |
| } |  |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE { (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |  |
| dl-CarrierFreq-v9e0[n] | Downlink EARFCN under test For Signalling test cases except NAS, see table 6.3.1.2-1. For NAS test cases when cells are on same PLMN, see table 6.3.2.3.2-1 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| interFreqCarrierFreqList-v1250[n] SEQUENCE {} | Not present |  |  |  |
| interFreqCarrierFreqListExt-r12[n] SEQUENCE {} | Not present |  |  |  |
| interFreqCarrierFreqListExt-v1280[n] SEQUENCE {} | Not present |  |  |  |
| interFreqCarrierFreqList-v1310[n] SEQUENCE {} | Not present |  |  |  |
| interFreqCarrierFreqListExt-v1310[n] SEQUENCE {} | Not present |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| QBASED | See the definition below table 4.4.3.2-3. |
| All neighCells with port1 | Used for all neighbouring cells with at least two cell-specific antenna ports |
| Band > 64 | If band > 64 is selected |

-	SystemInformationBlockType6

The IE SystemInformationBlockType6 contains information relevant only for inter-RAT cell re-selection i.e. information about UTRA frequencies and UTRA neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

Table 4.4.3.3-5: SystemInformationBlockType6

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE {} | Not present |  | UTRA-TDD |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | The same number of entries as the configured UTRA FDD carriers For Signalling test cases, see table 6.3.1.3-1 | n denotes the index of the entry | UTRA-FDD |  |
| carrierFreq[n] | Downlink UARFCN under test For Signalling test cases, see table 6.3.1.3-1 |  |  |  |
| cellReselectionPriority[n] | 3 |  |  |  |
| threshX-High[n] | 2 (4 dB) |  |  |  |
| threshX-Low[n] | 1 (2 dB) |  |  |  |
| q-RxLevMin[n] | -40 (-79 dBm) | The same value as defined in TS 34.108 [5], table 6.1.1. |  |  |
| p-MaxUTRA[n] | 21 (21 dBm) | The same value as defined in TS 34.108 [5], table 6.1.1. |  |  |
| q-QualMin[n] | -24 (-24 dB) | The same value as defined in TS 34.108 [5], table 6.1.1. |  |  |
| threshX-Q-r9[n] SEQUENCE {} | Not present |  |  |
| threshX-Q-r9[n] SEQUENCE { |  |  | QBASED |
| threshX-HighQ-r9 | 9 (9dB) |  |  |
| threshX-LowQ-r9 | 9 (9dB) |  |  |
| } |  |  |  |
| } |  |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE {} | Not present |  | UTRA-FDD |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | The same number of entries as the configured UTRA TDD carriers For Signalling test cases, see table 6.3.1.3-1 | n denotes the index of the entry | UTRA-TDD |  |
| carrierFreq[n] | Downlink UARFCN under test For Signalling test cases, see table 6.3.1.3-1 |  |  |  |
| cellReselectionPriority[n] | 3 |  |  |  |
| threshX-High[n] | 2 (4 dB) |  |  |  |
| threshX-Low[n] | 1 (2 dB) |  |  |  |
| q-RxLevMin[n] | -41 (-81 dBm) | The same value as defined in TS 34.108 [5], table 6.1.6a |  |  |
| p-MaxUTRA[n] | 21 (21 dBm) | The same value as defined in TS 34.108 [5], table 6.1.6a |  |  |
| } |  |  |  |  |
| t-ReselectionUTRA | 0 | Typical value in real network |  |  |
| t-ReselectionUTRA-SF | Not present |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |
| QBASED | See the definition below table 4.4.3.2-3. |

-	SystemInformationBlockType7

The IE SystemInformationBlockType7 contains information relevant only for inter-RAT cell re-selection i.e. information about GERAN frequencies relevant for cell re-selection. The IE includes cell re-selection parameters for each frequency.

Table 4.4.3.3-6: SystemInformationBlockType7

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType7 ::= SEQUENCE { |  |  |  |
| t-ReselectionGERAN | 0 |  |  |
| t-ReselectionGERAN-SF | Not present |  |  |
| carrierFreqsInfoListSEQUENCE (SIZE (1..maxGNFG)) OF SEQUENCE { | The same number of entries as the configured GERAN carriers For RRM test cases, see table 7.2.2.1-1 For Signalling test cases, see table 6.3.1.4-1 | n denotes the index of the entry |  |
| carrierFreqs [n] SEQUENCE { |  |  |  |
| startingARFCN[n] | Set the corresponding starting ARFCN of the GERAN cells under test. For RRM test cases, see table 7.2.2.1-1 For Signalling test cases, see table 6.3.1.4-1 |  |  |
| bandIndicator[n] | Set according to the band used for GERAN cells under test |  |  |
| followingARFCNs[n] CHOICE { |  |  |  |
| explicitListOfARFCNs[n] | Set the corresponding ARFCN of GERAN cells under test For RRM test cases, see table 7.2.2.1-1 For Signalling test cases, see table 6.3.1.4-1 |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo[n] SEQUENCE { |  |  |  |
| cellReselectionPriority[n] | 2 |  |  |
| ncc-Permitted[n] | ‘01000000'B | NCC=1 permitted |  |
| q-RxLevMin[n] | 2 |  |  |
| p-MaxGERAN[n] | 0 |  |  |
| threshX-High[n] | 2 |  |  |
| threshX-Low[n] | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	SystemInformationBlockType8

The IE SystemInformationBlockType8 contains information relevant only for inter-RAT cell re-selection i.e. information about CDMA2000 frequencies and CDMA2000 neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

Table 4.4.3.3-7: SystemInformationBlockType8

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType8 ::= SEQUENCE { |  |  |  |
| systemTimeInfo SEQUENCE { |  |  |  |
| cdma-EUTRA-Synchronisation | TRUE |  |  |
| cdma-SystemTime CHOICE { |  |  |  |
| synchronousSystemTime | A valid value as per TS 36.331 and calculated by the SS |  |  |
| } |  |  |  |
| } |  |  |  |
| searchWindowSize | 5 |  |  |
| parametersHRPD SEQUENCE {} | Not present |  | 1XRTT |
| parametersHRPD SEQUENCE { |  |  | HRPD |
| preRegistrationInfoHRPD SEQUENCE { |  |  |  |
| preRegistrationAllowed | FALSE |  |  |
| preRegistrationZoneId | Not present |  |  |
| secondaryPreRegistrationZoneIdList | Not present |  |  |
| } |  |  |  |
| cellReselectionParametersHRPD SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | 1 entry |  |  |
| bandClass | Operating band class under test. | ENUMERATED {bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| cellReselectionPriority | 1 |  |  |
| threshX-High | 30(-30) | INTEGER (0..63) |  |
| threshX-Low | 32(-32) | INTEGER (0..63) |  |
| } |  |  |  |
| neighCellList SEQUENCE (SIZE (1..16)) OF SEQUENCE { | 1 entry |  |  |
| bandClass | Operating band class under test | ENUMERATED {bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| neighCellsPerFreqList SEQUENCE (SIZE (1..16)) OF SEQUENCE { | The same number of entries as the configured CDMA2000 HRPD frequencies For Signalling test cases, see table 6.3.1.5-1 | n denotes the index of the entry |  |
| arfcn[n] | ARFCN of the CDMA2000 HRPD frequency For Signalling test cases, see table 6.3.1.5-1 | INTEGER (0..2047) |  |
| physCellIdList[n] SEQUENCE (SIZE (1..16)) OF {INTEGER (0..maxPNOffset) } | Physical Cell ID List of the CDMA2000 HRPD frequency For Signalling test cases, see table 6.3.1.5-1 | INTEGER (0..maxPNOffset) |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 0 | INTEGER (0..7) |  |
| t-ReselectionCDMA2000-SF | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| parameters1XRTT SEQUENCE {} | Not present |  | HRPD |
| parameters1XRTT SEQUENCE { |  |  | 1XRTT |
| csfb-RegistrationParam1XRTT SEQUENCE { |  |  |  |
| longCodeState1XRTT | A valid value, calculated and updated by the SS | BIT STRING (SIZE (42)) OPTIONAL |  |
| cellReselectionParameters1XRTT SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | 1 entry |  |  |
| bandClass | Operating band class under test | ENUMERATED {bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| cellReselectionPriority | 0 |  |  |
| threshX-High | 30(-30) | INTEGER (0..63) |  |
| threshX-Low | 32(-32) | INTEGER (0..63) |  |
| } |  |  |  |
| neighCellList SEQUENCE (SIZE (1..16)) OF SEQUENCE { | 1 entry |  |  |
| bandClass | Operating band class under test | ENUMERATED {bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| neighCellsPerFreqList SEQUENCE (SIZE (1..16)) OF SEQUENCE { | The same number of entries as the configured CDMA2000 1xRTT frequencies For Signalling test cases, see table 6.3.1.6-1 | n denotes the index of the entry |  |
| arfcn[n] | ARFCN of the CDMA2000 1xRTT frequency For Signalling test cases, see table 6.3.1.6-1 | INTEGER (0..2047) |  |
| physCellIdList[n] SEQUENCE (SIZE (1..16)) OF {INTEGER (0..maxPNOffset) } | Physical Cell ID List of the CDMA2000 1xRTT frequency For Signalling test cases, see table 6.3.1.6-1 | INTEGER (0..maxPNOffset) |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 0 | INTEGER (0..7) |  |
| t-ReselectionCDMA2000-SF | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HRPD | CDMA2000 HRPD cell environment |
| 1XRTT | CDMA2000 1XRTT cell environment |

-	SystemInformationBlockType9

The IE SystemInformationBlockType9 contains a home eNB name (HNB Name).

Table 4.4.3.3-8: SystemInformationBlockType9

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType9 ::= SEQUENCE { |  |  |  |
| hnb-Name | “3gppTest” |  |  |
| } |  |  |  |

-	SystemInformationBlockType10

The IE SystemInformationBlockType10 contains an ETWS primary notification.

Table 4.4.3.3-9: SystemInformationBlockType10

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType10 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message [see TS 23.041] |  |
| serialNumber - Geographical Scope (2 bit) - Message Code (10 bit) - Emergency User Alert (1 bit) - Popup (1 bit) - Update Number (4 bit) | '0011 0000 0000 0000'B Octet 1 bit 7 ~ 6 Octet 1 bit 5 ~ Octet 2 bit 4 Octet 1 bit 5 Octet 1 bit 4 Octet 2 bit 3 ~ 0 | Cell wide; '11 0000 0000'B Activate emergency user alert; Activate popup; For each update, incremented by one. [see TS 23.041] |  |
| warningType - Warning Type Value (7 bit) - Emergency User Alert (1 bit) - Popup (1 bit) - Padding (7 bit) | '0000 0101 1000 0000'B Octet 1 bit 7 ~ 1 Octet 1 bit 0 Octet 2 bit 7 Octet 2 bit 6 ~ bit 0 | Earthquake and Tsunami; Activate emergency user alert; Activate Popup. [see TS 23.041] '000 0000'B |  |
| warningSecurityInfo | Not present |  |  |
| } |  |  |  |

-	SystemInformationBlockType11

The IE SystemInformationBlockType11 contains an ETWS secondary notification.

Table 4.4.3.3-10: SystemInformationBlockType11 (1st Segment)

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType11 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message [see TS 23.041] |  |
| serialNumber - Geographical Scope (2 bit) - Message Code (10 bit) - Emergency User Alert (1 bit) - Popup (1 bit) - Update Number (4 bit) | '0011 0000 0000 0000'B Octet 1 bit 7 ~ 6 Octet 1 bit 5 ~ Octet 2 bit 4 Octet 1 bit 5 Octet 1 bit 4 Octet 2 bit 3 ~ 0 | Cell wide; '11 0000 0000'B Activate emergency user alert; Activate popup; For each update, incremented by one. [see TS 23.041] |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 0 |  |  |
| warningMessageSegment | Octetstring of N | where N ≥ 1 and less than 1246. [see TS 23.041] |  |
| dataCodingScheme | Bitstring (8) ID of the alphabet/coding and the applied language [see TS 23.041] | Provided as PIXITs [see TS 36.523-3 [20] cl. 9] |  |
| } |  |  |  |

Table 4.4.3.3-11: SystemInformationBlockType11 (2nd Segment)

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType11 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message [see TS 23.041] |  |
| serialNumber - Geographical Scope (2 bit) - Message Code (10 bit) - Emergency User Alert (1 bit) - Popup (1 bit) - Update Number (4 bit) | '0011 0000 0000 0000'B Octet 1 bit 7 ~ 6 Octet 1 bit 5 ~ Octet 2 bit 4 Octet 1 bit 5 Octet 1 bit 4 Octet 2 bit 3 ~ 0 | Cell wide; '11 0000 0000'B Activate emergency user alert; Activate popup; For each update, incremented by one. [see TS 23.041] |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 1 |  |  |
| warningMessageSegment | Octetstring of N | where N ≥ 1 and less than 1246. [see TS 23.041] |  |
| dataCodingScheme | Not present |  |  |
| } |  |  |  |

Table 4.4.3.3-12: SystemInformationBlockType11 (3rd Segment)

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType11 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message [see TS 23.041] |  |
| serialNumber - Geographical Scope (2 bit) - Message Code (10 bit) - Emergency User Alert (1 bit) - Popup (1 bit) - Update Number (4 bit) | '0011 0000 0000 0000'B Octet 1 bit 7 ~ 6 Octet 1 bit 5 ~ Octet 2 bit 4 Octet 1 bit 5 Octet 1 bit 4 Octet 2 bit 3 ~ 0 | Cell wide; '11 0000 0000'B Activate emergency user alert; Activate popup; For each update, incremented by one. [see TS 23.041] |  |
| warningMessageSegmentType | lastSegment |  |  |
| warningMessageSegmentNumber | 2 |  |  |
| warningMessageSegment | Octetstring of N | where N ≥ 1 and less than 1246. [see TS 23.041] |  |
| dataCodingScheme | Not present |  |  |
| } |  |  |  |

-	SystemInformationBlockType12

The IE SystemInformationBlockType12contains an CMAS  notification.

Table 4.4.3.3-12a: SystemInformationBlockType12 (1st Segment)

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType11 ::= SEQUENCE { |  |  |  |
| messageIdentifier-r9 | '0001 0001 0001 0010'B | CMAS CBS Message Identifier for CMAS Presidential Level Alerts (see TS 23.041]) |  |
| serialNumber-r9 | '0011 0000 0000 0000'B |  |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 0 |  |  |
| warningMessageSegment | Octetstring of N | where N ≥ 1 and less than 1246. (see TS 23.041) |  |
| dataCodingScheme | -Bitstring (8) ID of the alphabet/coding and the applied language [see TS 23.041] | Provided as PIXITs [see TS 36.523-3 [20] cl. 9] |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 4.4.3.3-12b: SystemInformationBlockType12 (2nd Segment)

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType11 ::= SEQUENCE { |  |  |  |
| messageIdentifier-r9 | '0001 0001 0001 0010'B | CMAS CBS Message Identifier for CMAS Presidential Level Alerts (see TS 23.041) |  |
| serialNumber-r9 | '0011 0000 0000 0000'B |  |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 1 |  |  |
| warningMessageSegment | Octetstring of N | where N ≥ 1 and less than 1246. (see TS 23.041) |  |
| dataCodingScheme | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 4.4.3.3-12c: SystemInformationBlockType12 (3rd Segment)

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType11 ::= SEQUENCE { |  |  |  |
| messageIdentifier-r9 | '0001 0001 0001 0010'B | CMAS CBS Message Identifier for CMAS Presidential Level Alerts (see TS 23.041) |  |
| serialNumber-r9 | '0011 0000 0000 0000'B |  |  |
| warningMessageSegmentType | lastSegment |  |  |
| warningMessageSegmentNumber | 2 |  |  |
| warningMessageSegment | Octetstring of N | where N ≥ 1 and less than 1246. (see TS 23.041) |  |
| dataCodingScheme | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

-	SystemInformationBlockType13

The IE SystemInformationBlockType13 contains the information required to acquire the MBMS control information associated with one or more MBSFN areas.

Table 4.4.3.3-13: SystemInformationBlockType13

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType13 ::= SEQUENCE { |  |  |  |
| mbsfn-AreaInfoList-r9 SEQUENCE (SIZE(1..maxMBSFN-Area)) OF SEQUENCE { |  |  |  |
| mbsfn-AreaId-r9 | 0 |  |  |
| non-MBSFNregionLength | 2 |  |  |
| notificationIndicator-r9 | 0 |  |  |
| mcch-Config-r9 SEQUENCE { |  |  |  |
| mcch-RepetitionPeriod-r9 | rf32 |  |  |
| mcch-Offset-r9 | 1 |  | FDD |
|  | 0 |  | TDD |
| mcch-ModificationPeriod-r9 | rf512 |  |  |
| sf-AllocInfo-r9 | ‘100000’B |  | FDD |
|  | ‘000010’B |  | TDD |
| signallingMCS-r9 | n2 |  |  |
| } |  |  |  |
| } |  |  |  |
| notificationConfig-r9 SEQUENCE { |  |  |  |
| notificationRepetitionCoeff-r9 | n4 |  |  |
| notificationOffset-r9 | 1 |  | FDD |
|  | 0 |  | TDD |
| notificationSF-Index-r9 | 1 | Subframe #1 | FDD |
|  | 5 | Subframe #9 | TDD |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

-	SystemInformationBlockType14

The IE SystemInformationBlockType14 contains the EAB parameters.

Table 4.4.3.3-13A: SystemInformationBlockType14

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType14-r11 ::= SEQUENCE { |  |  |  |
| eab-Param-r11 CHOICE { |  |  |  |
| EAB-Config-r11 ::= SEQUENCE { |  |  |  |
| eab-Category-r11 | a | EAB applies to all UEs in the PLMN ENUMERATED {a, b, c} |  |
| eab-BarringBitmap-r11 | '1111111111'B | All UE classes 0-9 BIT STRING (SIZE (10)) |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not Present |  |  |
| } |  |  |  |

-	SystemInformationBlockType15

The IE SystemInformationBlockType15 contains the MBMS Service Area Identities (SAI) of the current and/ or neighbouring carrier frequencies.

Table 4.4.3.3-14: SystemInformationBlockType15

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType15 ::= SEQUENCE { |  |  |  |
| mbms-SAI-IntraFreq-r11[1] SEQUENCE (SIZE (1..maxSAI-MBMS-r11)) OF { INTEGER (0..65535) } | 1 | 1 entry INTEGER (0..65535) |  |
| mbms-SAI-InterFreqList-r11 | Not present |  | MBMS_intraFreq, SCPTM_intraFreq |
| mbms-SAI-InterFreqList-r11[n] SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | The same number of entries as the configured inter-freq carriers providing the MBMS service in the test case (including additional bands, if any) | n denotes the index of the entry | MBMS_interFreq, SCPTM_interFreq |
| dl-CarrierFreq-r11 | Downlink EARFCN under test, see table 6.3.1.2-1. |  |  |
| mbms-SAI-List-r11[n] SEQUENCE (SIZE (1..maxSAI-MBMS-r11)) OF { INTEGER (0..65535) } | The same number of entries as the configured MBMS SAIs providing the MBMS service in the test case for the specific frequency | n denotes the index of the entry INTEGER (0..65535) |  |
| lateNonCriticalExtension | Not present |  | MBMS_intraFreq, MBMS_interFreq, SCPTM_intraFreq, SCPTM_interFreq |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MBMS_intraFreq | SIB15 transmitted for a MBMS intra-band cell environment (MBMS service only provided for intra-frequency cells). |
| MBMS_interFreq | SIB15 transmitted for a MBMS inter-frequency cell environment (MBMS service provided on multiple frequencies within a band). |
| SCPTM_intraFreq | SIB15 transmitted for a SC-PTM intra-band cell environment (SC-PTM service only provided for intra-frequency cells). |
| SCPTM_interFreq | SIB15 transmitted for a SC-PTM inter-frequency cell environment (SC-PTM service provided on multiple frequencies within a band). |

-	SystemInformationBlockType17

The IE SystemInformationBlockType17 contains information relevant for RAN Assisted WLAN interworking

Table 4.4.3.3-15: SystemInformationBlockType17

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType17-r12 ::= SEQUENCE { |  |  |  |
| wlan-OffloadInfoPerPLMN-List-r12 [1] SEQUENCE (SIZE (1..maxPLMN-r11)) OF { |  | 1 Entry |  |
| wlan-OffloadConfigCommon-r12 | WLAN-OffloadConfig-DEFAULT |  |  |
| wlan-Id-List-r12 [1] SEQUENCE (SIZE (1..maxWLAN-Id-r12)) OF { |  | 1 Entry |  |
| wlan-Identifiers SEQUENCE{ |  |  |  |
| ssid-r12 | Set as per Table 4.4.8-1 | OCTET STRING (SIZE (1..32)) Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| bssid-r12 | Not present |  |  |
| hessid-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

-	SystemInformationBlockType18

The IE SystemInformationBlockType18 contains the resource pool information for sidelink direct communication.

Table 4.4.3.3-16: SystemInformationBlockType18

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType18-r12 ::= SEQUENCE { |  |  |  |
| commConfig-r12 SEQUENCE { |  |  |  |
| commRxPool-r12 SEQUENCE (SIZE (1..maxSL-RxPool-r12)) OF SL-CommResourcePool-r12 { |  | 3 Rx pools |  |
| SL-CommResourcePool-r12[1] SEQUENCE { |  | RxPool 1 A monitoring UE can receive on the resources of this pool when a transmitting UE uses the TxPoolNormal 1 |  |
| sc-CP-Len-r12 | normal |  |  |
| sc-Period-r12 | sf40 |  |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 11000000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| data-CP-Len-r12 | normal |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  |  |  |
| hoppingParameter-r12 | 0 |  |  |
| numSubbands-r12 | ns1 |  |  |
| rb-Offset-r12 | 0 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| data-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000000 00000000 11110000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| trpt-Subset-r12 | 001 |  |  |
| } |  |  |  |
| rxParametersNCell-r12 | Not present |  |  |
| txParameters-r12 | Not present |  |  |
| } |  |  |  |
| SL-CommResourcePool-r12[2] SEQUENCE { |  | RxPool 2 A monitoring UE can receive on the resources of this pool when a transmitting UE uses the TxPoolNormal 2 |  |
| sc-CP-Len-r12 | normal |  |  |
| sc-Period-r12 | sf40 |  |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00110000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| data-CP-Len-r12 | normal |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  |  |  |
| hoppingParameter-r12 | 0 |  |  |
| numSubbands-r12 | ns1 |  |  |
| rb-Offset-r12 | 0 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| data-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000000 00000000 00111100 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| trpt-Subset-r12 | 001 |  |  |
| } |  |  |  |
| rxParametersNCell-r12 SEQUENCE { |  |  |  |
| tdd-Config-r12 SEQUENCE {} | Not present |  |  |
| syncConfigIndex-r12 | 1 | Entry 2 in discSyncConfig-r12 (SL-SyncConfig-r12[2]) |  |
| } |  |  |  |
| txParameters-r12 | Not present |  |  |
| } |  |  |  |
| SL-CommResourcePool-r12[3] SEQUENCE { |  | RxPool 3 A monitoring UE can receive on the resources of this pool when a transmitting UE uses the TxPoolExceptional 1 |  |
| sc-CP-Len-r12 | normal |  |  |
| sc-Period-r12 | sf40 |  |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00001100 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| data-CP-Len-r12 | normal |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  |  |  |
| hoppingParameter-r12 | 0 |  |  |
| numSubbands-r12 | ns1 |  |  |
| rb-Offset-r12 | 0 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| data-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000000 00000000 00001111 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| trpt-Subset-r12 | 001 |  |  |
| } |  |  |  |
| rxParametersNCell-r12 | Not present |  |  |
| txParameters-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| commTxPoolNormalCommon-r12 SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-CommResourcePool-r12 { |  | 2 Tx pools Normal |  |
| SL-CommResourcePool-r12[1] SEQUENCE { |  | TxPool 1 |  |
| sc-CP-Len-r12 | normal |  |  |
| sc-Period-r12 | sf40 |  |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 11000000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| data-CP-Len-r12 | normal |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  |  |  |
| hoppingParameter-r12 | 0 |  |  |
| numSubbands-r12 | ns1 |  |  |
| rb-Offset-r12 | 0 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| data-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000000 00000000 11110000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| trpt-Subset-r12 | 001 |  |  |
| } |  |  |  |
| rxParametersNCell-r12 | Not present |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| sc-TxParameters-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| dataTxParameters-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| SL-CommResourcePool-r12[2] SEQUENCE { |  | TxPool 2 |  |
| sc-CP-Len-r12 | normal |  |  |
| sc-Period-r12 | sf40 |  |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00110000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| data-CP-Len-r12 | normal |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  |  |  |
| hoppingParameter-r12 | 0 |  |  |
| numSubbands-r12 | ns1 |  |  |
| rb-Offset-r12 | 0 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| data-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000000 00000000 00111100 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| trpt-Subset-r12 | 001 |  |  |
| } |  |  |  |
| rxParametersNCell-r12 | Not present |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| sc-TxParameters-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| dataTxParameters-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| commTxPoolExceptional-r12 SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-CommResourcePool-r12 { |  | 1 Tx pool Exceptional |  |
| SL-CommResourcePool-r12[1] SEQUENCE { |  | TxExceptionalPool 1 |  |
| sc-CP-Len-r12 | normal |  |  |
| sc-Period-r12 | sf40 |  |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00001100 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| data-CP-Len-r12 | normal |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  |  |  |
| hoppingParameter-r12 | 0 |  |  |
| numSubbands-r12 | ns1 |  |  |
| rb-Offset-r12 | 0 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| data-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000000 00000000 00001111 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| trpt-Subset-r12 | 001 |  |  |
| } |  |  |  |
| rxParametersNCell-r12 | Not present |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| sc-TxParameters-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| dataTxParameters-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| commSyncConfig-r12 SEQUENCE (SIZE (1..maxSL-SyncConfig-r12)) OF SL-SyncConfig-r12 { |  | 2 configuration reception/transmission of synchronisation signals |  |
| SL-SyncConfig-r12[1] SEQUENCE { |  | SyncConfig 1 To be used for transmission |  |
| syncCP-Len-r12 | normal |  |  |
| syncOffsetIndicator-r12 | 2 |  |  |
| slssid-r12 | 30 |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| syncTxParameters-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| syncTxThreshIC-r12 | 7 | -85dBm The threshold for starting transmission of SLSS |  |
| } |  |  |  |
| rxParamsNCell-r12 | Not present |  |  |
| } |  |  |  |
| SL-SyncConfig-r12[2] SEQUENCE { |  | SyncConfig 2 To be used for reception. RxPool Pool 2 is refering to this SyncConfig 2 |  |
| syncCP-Len-r12 | normal |  |  |
| syncOffsetIndicator-r12 | 25 |  |  |
| slssid-r12 | 40 |  |  |
| txParameters-r12 | Not Present |  |  |
| rxParamsNCell-r12 SEQUENCE { |  |  |  |
| physCellId-r12 | Set to the physical cell identity of the PCell |  |  |
| discSyncWindow-r12 | w1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: For convenience the difference between the different pools is only in the relevant setting of the subframeBitmap. Note 2: Note that the resources defined by the first entry in preconfigComm defined in clause 6.8.1.1 (used for transmission by an out of coverage UE) should not overlap (see TS 36.331 [17], clause 9.3.2) with those of the pool(s) covering scheduled transmissions by in coverage UEs which are defined here for SIB18. |  |  |  |

| Condition | Explanation |
| --- | --- |
| BW5 | 5 MHz channel bandwidth cell environment |
| BW10 | 10 MHz channel bandwidth cell environment |
| FDD | FDD cell environment |

-	SystemInformationBlockType19

The IE SystemInformationBlockType19 contains the resource pool information for sidelink direct discovery.

Table 4.4.3.3-17: SystemInformationBlockType19

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType19-r12 ::= SEQUENCE { |  |  |  |
| discConfig-r12 SEQUENCE { |  |  |  |
| discRxPool-r12 SEQUENCE SIZE (1..maxSL-TxPool-r12) OF SL-DiscResourcePool-r12 { |  | 2 RxPools For convenience the difference between the pools is only in subframeBitmap. |  |
| SL-DiscResourcePool-r12[1] SEQUENCE { |  | RxPool 1 A monitoring UE can receive on the resources of this pool when a transmitting UE uses the TxPoolCommon 1 |  |
| cp-Len-r12 | normal |  |  |
| discPeriod-r12 | rf128 |  |  |
| numRetx-r12 | 0 |  |  |
| numRepetition-r12 | 1 |  |  |
| tf-ResourceConfig-r12 SEQUENCE { |  | SL-TF-ResourceConfig-r12 |  |
| prb-Num-r12 | 12 |  | BW5 |
|  | 24 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 23 |  | BW5 |
|  | 48 |  | BW10 |
| offsetIndicator-r12 | 160 | small-r12 | FDD |
|  | 163 |  | TDD |
| subframeBitmap-r12 | 11000000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
|  | 11000000 00000000 | bs16-r12 | TDD |
| } |  |  |  |
| txParameters-r12 | Not present |  |  |
| rxParameters-r12 | Not present |  |  |
| } |  |  |  |
| SL-DiscResourcePool-r12[2] SEQUENCE { |  | RxPool 2 A monitoring UE can receive on the resources of this pool when a transmitting UE uses the TxPoolCommon 2 |  |
| cp-Len-r12 | normal |  |  |
| discPeriod-r12 | rf128 |  |  |
| numRetx-r12 | 0 |  |  |
| numRepetition-r12 | 1 |  |  |
| tf-ResourceConfig-r12 SEQUENCE { |  | SL-TF-ResourceConfig-r12 |  |
| prb-Num-r12 | 12 |  | BW5 |
|  | 24 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 23 |  | BW5 |
|  | 48 |  | BW10 |
| offsetIndicator-r12 | 160 | small-r12 | FDD |
|  | 163 |  | TDD |
| subframeBitmap-r12 | 00110000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
|  | 00110000 00000000 | bs16-r12 | TDD |
| } |  |  |  |
| txParameters-r12 | Not present |  |  |
| rxParameters-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| discTxPoolCommon-r12 SEQUENCE SIZE (1..maxSL-TxPool-r12) OF SL-DiscResourcePool-r12 { |  | 2 TxPools For convenience the difference between the pools is only in subframeBitmap. |  |
| SL-DiscResourcePool-r12[1] SEQUENCE { |  | TxPool 1 |  |
| cp-Len-r12 | normal |  |  |
| discPeriod-r12 | rf128 |  |  |
| numRetx-r12 | 0 |  |  |
| numRepetition-r12 | 1 |  |  |
| tf-ResourceConfig-r12 SEQUENCE { |  | SL-TF-ResourceConfig-r12 |  |
| prb-Num-r12 | 12 |  | BW5 |
|  | 24 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 23 |  | BW5 |
|  | 48 |  | BW10 |
| offsetIndicator-r12 | 160 | small-r12 | FDD |
|  | 163 |  | TDD |
| subframeBitmap-r12 | 11000000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| bs42-r12 | 11000000 00000000 | bs16-r12 | TDD |
| } |  |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| txParametersGeneral-r12 SEQUENCE { |  | SL-TxParameters-r12 |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 | P0-SL-r12 |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| poolSelection-r12 | NULL | random-r12 |  |
| txProbability-r12 | p100 |  |  |
| } |  |  |  |
| } |  |  |  |
| rxParameters-r12 | Not present |  |  |
| } |  |  |  |
| SL-DiscResourcePool-r12[2] SEQUENCE { |  | TxPool 2 |  |
| cp-Len-r12 | normal |  |  |
| discPeriod-r12 | rf128 |  |  |
| numRetx-r12 | 0 |  |  |
| numRepetition-r12 | 1 |  |  |
| tf-ResourceConfig-r12 SEQUENCE { |  | SL-TF-ResourceConfig-r12 |  |
| prb-Num-r12 | 12 |  | BW5 |
|  | 24 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 23 |  | BW5 |
|  | 48 |  | BW10 |
| offsetIndicator-r12 | 160 | small-r12 | FDD |
|  | 163 |  | TDD |
| subframeBitmap-r12 | 00110000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
|  | 00110000 00000000 | bs16-r12 | TDD |
| } |  |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| txParametersGeneral-r12 SEQUENCE { |  | SL-TxParameters-r12 |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 | P0-SL-r12 |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| poolSelection-r12 | NULL | random-r12 |  |
| txProbability-r12 | p100 |  |  |
| } |  |  |  |
| } |  |  |  |
| rxParameters-r12 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| discTxPowerInfo-r12 SEQUENCE (SIZE (maxSL-DiscPowerClass-r12)) OF SL-DiscTxPowerInfo-r12 { |  |  |  |
| SL-DiscTxPowerInfo-r12[1] SEQUENCE { |  | UE range class ‘short’ |  |
| discMaxTxPower-r12 | 23 |  |  |
| } |  |  |  |
| SL-DiscTxPowerInfo-r12[2] SEQUENCE { |  | UE range class ‘medium’ |  |
| discMaxTxPower-r12 | 16 |  |  |
| } |  |  |  |
| SL-DiscTxPowerInfo-r12[3] SEQUENCE { |  | UE range class ‘long’ |  |
| discMaxTxPower-r12 | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| discSyncConfig-r12 SEQUENCE (SIZE (1..maxSL-SyncConfig-r12)) OF SL-SyncConfig-r12 { | 2 configurations reception/transmission of synchronisation signals |  |  |
| SL-SyncConfig-r12[1] SEQUENCE { | SyncConfig 1 | To be used for transmission |  |
| syncCP-Len-r12 | normal |  |  |
| syncOffsetIndicator-r12 | 35 |  |  |
| slssid-r12 | 30 |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| syncTxParameters-r12 SEQUENCE { |  | SL-TxParameters-r12 |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| syncTxThreshIC-r12 | 0 | (-infinity) SLSS will not be transmitted |  |
| syncInfoReserved-r12 | Not Present |  |  |
| } |  |  |  |
| rxParamsNCell-r12 SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| SL-SyncConfig-r12[2] SEQUENCE { | SyncConfig 2 |  |  |
| syncCP-Len-r12 | normal |  |  |
| syncOffsetIndicator-r12 | 25 |  |  |
| slssid-r12 | 40 |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| syncTxParameters-r12 SEQUENCE { |  | SL-TxParameters-r12 |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 4 |  |  |
| } |  |  |  |
| syncTxThreshIC-r12 | 7 | -85dBm The threshold for starting transmission of SLSS |  |
| syncInfoReserved-r12 | Not Present |  |  |
| } |  |  |  |
| rxParamsNCell-r12 SEQUENCE { |  |  |  |
| physCellId-r12 | Set to the physical cell identity of the PCell |  |  |
| discSyncWindow-r12 | w1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| discInterFreqList-r12 | Not present |  |  |
| lateNonCriticalExtension |  |  |  |
| ... |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment (TDD config 1) |
| BW5 | 5 MHz channel bandwidth cell environment |
| BW10 | 10 MHz channel bandwidth cell environment |

-	SystemInformationBlockType20

The IE SystemInformationBlockType20 contains the information required to acquire the SC-PTM control information.

Table 4.4.3.3-18: SystemInformationBlockType20

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType20-r13 ::= SEQUENCE { |  |  |  |
| sc-mcch-RepetionPeriod-r13 | Rf32 |  |  |
| sc-mcch-Offset-r13 | 1 |  |  |
| sc-mcch-FirstSubframe-r13 | 0 |  |  |
| sc-mcch-duration-r13 | Not present |  |  |
| sc-mcch-ModificationPeriod-r13 | Rf512 |  |  |
| lateNonCriticalExtension |  |  |  |
| } |  |  |  |

-	SystemInformationBlockType21

The IE SystemInformationBlockType21 contains the resource pool information for V2X sidelink communication.

Table 4.4.3.3-19: SystemInformationBlockType21

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType21-r14 ::= SEQUENCE { |  |  |  |
| sl-V2X-ConfigCommon-r14 SEQUENCE { |  |  |  |
| v2x-CommRxPool-r14 | Not present |  |  |
| v2x-CommTxPoolNormalCommon-r14 | Not present |  |  |
| p2x-CommTxPoolNormalCommon-r14 | Not present |  |  |
| v2x-CommTxPoolExceptional-r14 | Not present |  |  |
| v2x-SyncConfig-r14 | Not present |  |  |
| v2x-InterFreqInfoList-r14 SEQUENCE (SIZE (0..maxFreqV2X-1-r14)) OF SL-InterFreqInfoV2X-r14 { |  | 1 entry |  |
| SL-InterFreqInfoV2X-r14[1] | SL-InterFreqInfoV2X-r14-DEFAULT using condition SIB |  |  |
| } |  |  |  |
| v2x-ResourceSelectionConfig-r14 | Not present |  |  |
| zoneConfig-r14 SEQUENCE {} | Not present |  |  |
| typeTxSync-r14 | enb | ENUMERATED {gnss, enb, ue, spare1} |  |
| thresSL-TxPrioritization-r14 | Not present |  |  |
| anchorCarrierFreqList-r14 | Not present |  |  |
| offsetDFN-r14 | Not present |  |  |
| cbr-CommonTxConfigList-r14 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

-	SystemInformationBlockType24

The IE SystemInformationBlockType24 contains information relevant only for inter-RAT cell re-selection i.e. information about NR frequencies and NR neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

Table 4.4.3.3-20: SystemInformationBlockType24

| Derivation Path: TS 36.331 [17], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | The same number of entries as the configured NR carriers For Signalling test cases, see Table 6.3.1.8-1. | n denotes the index of the entry |  |
| carrierFreq-r15[n] | Downlink NR ARFCN of SSB under test | For signalling test cases see TS 38.508-1 [67] subclause 6.2.3. Otherwise, see TS 38.508-1 [67] subclause 4.3.1. |  |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF SEQUENCE {} | Not present |  |  |
| multiBandInfoListSUL-r15[n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF SEQUENCE {} | Not present |  |  |
| measTimingConfig-r15[n] SEQUENCE { |  |  |  |
| periodicityAndOffset-r15 CHOICE { |  |  |  |
| sf20 | 0 |  | FR1 |
| sf160 | 0 |  | FR2 |
| } |  |  |  |
| ssb-Duration-r15 | sf2 |  | FR1 |
|  | sf3 |  | FR2 |
| } |  |  |  |
| subcarrierSpacingSSB-r15[n] | kHz15 |  | SCS_15kHz |
|  | kHz30 |  | SCS_30kHz |
|  | kHz120 |  | SCS_120kHz |
| ss-RSSI-Measurement-r15[n] | Not present |  |  |
| cellReselectionPriority-r15[n] | 5 |  |  |
| cellReselectionSubPriority-r15[n] | Not present |  |  |
| threshX-High-r15[n] | 2 (4 dB) |  |  |
| threshX-Low-r15[n] | 1 (2 dB) |  |  |
| threshX-Q-r15[n] | Not present |  |  |
| threshX-Q-r15[n] SEQUENCE { |  |  | QBASED |
| threshX-HighQ-r15 | 5 (5 dB) |  |  |
| threshX-LowQ-r15 | 5 (5 dB) |  |  |
| } |  |  |  |
| q-RxLevMin-r15[n] | -70 (-140 dBm) | For RF/RRM test cases |  |
|  | -55 (-110 dBm) | For signalling test cases |  |
| q-RxLevMinSUL-r15[n] | -70 (-140 dBm) | For RF/RRM test cases | SUL |
|  | -55(-110 dBm) | For signalling test cases |  |
| p-MaxNR-r15[n] | 23 |  | FR1 |
|  | 26 |  | FR2 FR1_RF_PC2 |
| ns-PmaxListNR-r15[n] | Not present |  |  |
| q-QualMin-r15[n] | Not present |  |  |
|  | -20 (-20dB) |  | QBASED |
| deriveSSB-IndexFromCell-r15[n] | False |  |  |
| maxRS-IndexCellQual-r15[n] | Not present |  |  |
| threshRS-Index-r15[n] | Not present |  |  |
| } |  |  |  |
| t-ReselectionNR-r15 | 0 |  |  |
| t-ReselectionNR-SF-r15 | Not present | Not required unless speed-dependent cell re-selection is tested. |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for frequency of the cell according to TS 38.508-1 [67] clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |
| SCS_30kHz | SCS=30kHz for frequency of the cell according to TS 38.508-1 [67] clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |
| SCS_120kHz | SCS=120kHz for frequency of the cell according to TS 38.508-1 [67] clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |
| QBASED | This condition applies to Quality based cell (re)selection signalling test cases. |
| FR1_RF_PC2 | FR1 RF testing with Power Class 2 |

-	SystemInformationBlockType31

The IE SystemInformationBlockType31 contains satellite assistance information for the serving cell.

Table 4.4.3.3-21: SystemInformationBlockType31

| Derivation Path: TS 36.331 [17], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  | NTN |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -11605245 |  |  |
| positionY-r17 | -30240830 |  |  |
| positionZ-r17 | -1854603 |  |  |
| velocityVX-r17 | 44 |  |  |
| velocityVY-r17 | -31 |  |  |
| velocityVZ-r17 | 195 |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 7681821 |  |  |
| nta-CommonDrift-r17 | -37 |  |  |
| nta-CommonDriftVariation-r17 | 0 |  |  |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s240 |  |  |
| epochTime-r17 | Not present |  |  |
| k-Offset-r17 | 550 |  |  |
| k-Mac-r17 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NTN | Non-Terrestrial Networks test environment |

-	SystemInformationBlockType32

The IE SystemInformationBlockType32 contains satellite assistance information for prediction of discontinuous coverage.

Table 4.4.3.3-22: SystemInformationBlockType32

| Derivation Path: TS 36.331 [17], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType32-r17 ::= SEQUENCE { |  |  |  |
| satelliteInfoList-r17 | FFS |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

4.4.3.4	Channel-bandwidth-dependent parameters in system information blocks

The default values of parameters in system information blocks which depend on the channel bandwidth are defined in table 4.4.3.4-1.

Table 4.4.3.4-1: Channel-bandwidth-dependent parameters

| Information Element | Channel bandwidth |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20MHz |  |
| SIB3 periodicity | 64 | 64 | 32 | 32 | 32 | 32 |  |
| SIB4 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| SIB5 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| SIB6 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| SIB7 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| SIB8 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| measurementBandwidth in SIB5 | mbw6 | mbw15 | mbw25 | mbw50 | mbw75 | mbw100 |  |
| SIB9 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| SIB10 periodicity | 64 | 64 | 32 | 32 | 32 | 32 |  |
| SIB11 periodicity | 64 | 64 | 32 | 32 | 32 | 32 |  |
| SIB12 periodicity | 64 | 64 | 32 | 32 | 32 | 32 |  |
| SIB13 periodicity | 64 | 64 | 32 | 32 | 32 | 32 |  |
| SIB15 periodicity | 64 | 64 | 32 | 32 | 32 | 32 |  |
| SIB17 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| SIB18 periodicity |  |  | 32 | 32 | - | 32 |  |
| SIB19 periodicity |  |  | 64 | 64 | 64 | 64 |  |
| SIB20 periodicity | 64 | 64 | 32 | 32 | 32 | 32 |  |
| SIB21 periodicity |  |  |  | 32 |  | 32 |  |
| SIB24 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| SIB31 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |

[Skip unchange text]

8.1.4.3	NB-IoT Common parameters for simulated cells

The parameters specified in this sub clause apply to all simulated cells unless otherwise specified.

8.1.4.3.1	NB-IoT Common configurations of system information blocks

8.1.4.3.1.1	NB-IoT Combinations of system information blocks

The combination of system information blocks required by a test case depends on the test case scenario. In this clause, the following combinations of system information blocks are defined.

Combination 1 is the default combination which applies to the following test case scenarios:

-	single cell scenario

Combination 2 applies to the following test case scenarios:

-	intra-frequency multi cell scenario

-	intra-frequency multi cell scenario with neighbouring cell related information

Combination 3 applies to the following test case scenarios:

-	inter-frequency multi cell scenario

Combination 4 applies to the following test case scenarios:

- access barring single cell scenario

Combination 5 applies to the following test case scenarios:

-	SC-PTM single cell scenario (SC-PTM cell broadcasting SIB20-NB)

Combination 6 applies to the following test case scenarios:

-	single cell on multi carriers scenario

Combination 7 applies to the following test case scenarios:

-	single cell NPRACH resources using preamble format 2 on non-anchor carriers scenario

Combination 8 is the default combination which applies to the following test case scenarios:

-	single cell scenario + NTN

Combination 9 is the default combination which applies to the following test case scenarios:

-	intra-frequency multi cell scenario + NTN

-	intra-frequency multi cell scenario with neighbouring cell related information + NTN

Table 8.1.4.3.1.1-1: Combinations of system information blocks

|  | System information block type |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Combination No. |  | SIB2-NB | SIB3-NB | SIB4-NB | SIB5-NB | SIB14-NB | SIB 16-NB | SIB20-NB | SIB22-NB | SIB23-NB | SIB31-NB |
| 1 |  | X |  |  |  |  |  |  |  |  |  |
| 2 |  | X | X | X |  |  |  |  |  |  |  |
| 3 |  | X | X |  | X |  |  |  |  |  |  |
| 4 |  | X | X |  |  | X |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  | X |  |  |  |
| 6 |  | X |  |  |  |  |  |  | X |  |  |
| 7 |  | X |  |  |  |  |  |  | X | X |  |
| 8 |  | X |  |  |  |  |  |  |  |  | X |
| 9 |  | X | X | X |  |  |  |  |  |  | X |

8.1.4.3.1.2	NB-IoT Scheduling of system information blocks

The scheduling configurations for combinations of system information blocks are defined in the following tables.

Table 8.1.4.3.1.2-1: Scheduling for combination 1

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |

Table 8.1.4.3.1.2-2: Scheduling for combination 2

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB4-NB |

Table 8.1.4.3.1.2-3: Scheduling for combination 3

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB5-NB |

Table 8.1.4.3.1.2-4: Scheduling for combination 4

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB14-NB |

Table 8.1.4.3.1.2-5: Scheduling for combination 5

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB20-NB |

Table 8.1.4.3.1.2-6: Scheduling for combination 6

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB22-NB |

Table 8.1.4.3.1.2-7: Scheduling for combination 7

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB22-NB |
| 3 | 64 | every16thRF | 256 | SIB23-NB |

Table 8.1.4.3.1.2-8: Scheduling for combination 8

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB31-NB |

Table 8.1.4.3.1.2-9: Scheduling for combination 9

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB4-NB |
| 4 | 64 | every16thRF | 256 | SIB31-NB |

8.1.4.3.2	NB-IoT Common contents of system information messages

-	MasterInformationBlock-NB

The MasterInformationBlock-NB includes the system information transmitted on BCH.

Table 8.1.4.3.2-1: MasterInformationBlock-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-NB ::= SEQUENCE { |  |  |  |
| systemFrameNumber-MSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| hyperSFN-LSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| schedulingInfoSIB1-r13 | 2 | TBS = 208 bits, 16 repetitions |  |
| systemInfoValueTag-r13 | 0 |  |  |
| ab-Enabled-r13 | FALSE |  |  |
| operationModeInfo-r13 CHOICE { |  |  |  |
| inband-SamePCI-r13 SEQUENCE { |  |  | Inband-Same |
| eutra-CRS-SequenceInfo-r13 | 23 | As specified in TS 36.213 [29] Clause 16.8 | PRB30 |
|  | 24 | As specified in TS 36.213 [29] Clause 16.8 | PRB35 |
|  | 7 | As specified in TS 36.213 [29] Clause 16.8 | PRB17 |
|  | 8 | As specified in TS 36.213 [29] Clause 16.8 | PRB22 |
| } |  |  |  |
| inband-DifferentPCI-r13 SEQUENCE { |  |  | Inband_Different |
| eutra-NumCRS-Ports-r13 | same | same number of ports as NRS |  |
| rasterOffset-r13 | 2.5*(2MDL+1) kHz | MDL See subclause 8.1.3.1 |  |
| Spare | ’00’B |  |  |
| } |  |  |  |
| guardband-r13 SEQUENCE { |  |  | Guardband |
| rasterOffset-r13 | 2.5*(2MDL+1) kHz | MDL See subclause 8.1.3.1 |  |
| spare | ‘000’B |  |  |
| } |  |  |  |
| standalone-r13 SEQUENCE { |  |  | Standalone |
| spare | ‘0000 0’B |  |  |
| } |  |  |  |
| } |  |  |  |
| additionalTransmissionSIB1-r15 | FALSE | A Rel-13/Rel-14 UE will interpret FALSE as spare ‘0’ |  |
| spare | ‘0000 0000 00’B |  | Rel-15 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Inband-Same | In-band with same PCI test environment |
| Inband-Different | In-band with different PCI test environment |
| PRB30 | For 10 MHz LTE Cell with In-Band NB-IoT in PRB 30 |
| PRB35 | For 10 MHz LTE Cell with In-Band NB-IoT in PRB 35 |
| PRB17 | For 5 MHz LTE Cell with In-Band NB-IoT in PRB 17 |
| PRB22 | For 5 MHz LTE Cell with In-Band NB-IoT in PRB 22 |
| Standalone | Standalone test environment |
| Guardband | Guard-band test environment |

-	MasterInformationBlock-TDD-NB

The MasterInformationBlock-TDD-NB includes the system information transmitted on BCH in TDD.

Table 8.1.4.3.2-1A: MasterInformationBlock-TDD-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-TDD-NB ::= SEQUENCE { |  |  |  |
| systemFrameNumber-MSB-r15 | A valid value as defined in TS 36.331 [17] |  |  |
| hyperSFN-LSB-r15 | A valid value as defined in TS 36.331 [17] |  |  |
| schedulingInfoSIB1-r15 | 2 | 16 repetitions |  |
| systemInfoValueTag-r15 | 0 |  |  |
| ab-Enabled-r15 | FALSE |  |  |
| operationModeInfo-r15 CHOICE { |  |  |  |
| inband-SamePCI-r15 SEQUENCE { |  |  | Inband-Same |
| eutra-CRS-SequenceInfo-r15 | 23 | As specified in TS 36.213 [29] Clause 16.8 | PRB30 |
|  | 24 | As specified in TS 36.213 [29] Clause 16.8 | PRB35 |
|  | 7 | As specified in TS 36.213 [29] Clause 16.8 | PRB17 |
|  | 8 | As specified in TS 36.213 [29] Clause 16.8 | PRB22 |
| sib-InbandLocation-r15 | lower |  |  |
| } |  |  |  |
| inband-DifferentPCI-r15 SEQUENCE { |  |  | Inband_Different |
| eutra-NumCRS-Ports-r15 | same | same number of ports as NRS |  |
| rasterOffset-r15 | 2.5*(2MDL+1) kHz | MDL See subclause 8.1.3.1 |  |
| sib-InbandLocation-r15 | lower |  |  |
| spare | ’00’B |  |  |
| } |  |  |  |
| guardband-r15 SEQUENCE { |  |  | Guardband |
| rasterOffset-r15 | 2.5*(2MDL+1) kHz | MDL See subclause 8.1.3.1 |  |
| sib-GuardbandInfo-r15 CHOICE { |  |  |  |
| sib-GuardbandAnchor-r15 SEQUENCE { |  |  | sib-GuardbandAnchor |
| spare | '0'B |  |  |
| } |  |  |  |
| sib-GuardbandGuardband-r15 SEQUENCE { |  |  | sib-GuardbandGuardband |
| spare | '0'B |  |  |
| } |  |  |  |
| sib-GuardbandInbandSamePCI-r15 SEQUENCE { |  |  | sib-GuardbandInbandSamePCI |
| spare | '0'B |  |  |
| } |  |  |  |
| sib-GuardbandinbandDiffPCI-r15 SEQUENCE { |  |  | sib-GuardbandinbandDiffPCI |
| sib-EUTRA-NumCRS-Ports-r15 | same |  |  |
| } |  |  |  |
| eutra-Bandwitdh-r15 | bw5or10 |  | EUTRA system bandwidth Value 5 or 10 MHz |
|  | Bw15or20 |  | EUTRA system bandwidth Value 15 or 20 MHz |
| } |  |  |  |
| standalone-r15 SEQUENCE { |  |  | Standalone |
| sib-InbandLocation-r15 | lower |  |  |
| spare | ‘0000 0’B |  |  |
| } |  |  |  |
| } |  |  |  |
| sib1-CarrierInfo-r15 | anchor |  |  |
| spare | ‘0000 0000 0’B |  |  |
| } |  |  |  |

-	SystemInformation-NB

The SystemInformation-NB message is used to convey one or more System Information Blocks. All the SIBs included are transmitted with the same periodicity.

Table 8.1.4.3.2-2: SystemInformation-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformation-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| systemInformation-r13 SEQUENCE { |  |  |  |
| sib-TypeAndInfo-r13 SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {} | See subclause 8.1.4.3.1 |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	SystemInformationBlockType1-NB

SystemInformationBlockType1-NB contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information.

Table 8.1.4.3.2-3: SystemInformationBlockType1-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| hyperSFN-MSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| cellAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| plmn-IdentityList-r13 SEQUENCE (SIZE (1.. maxPLMN-r11)) OF SEQUENCE { | 1 entry |  |  |
| plmn-Identity-r13[1] SEQUENCE { |  |  |  |
| mcc | See table 8.1.4.2-2 |  |  |
| mnc | See table 8.1.4.2-2 |  |  |
| } |  |  |  |
| cellReservedForOperatorUse-r13[1] | notReserved |  |  |
| attachWithoutPDN-Connectivity-r13[1] | true |  | ATTACH_WITHOUT_PDN |
|  | Not present |  | ATTACH_WITH_PDN |
| } |  |  |  |
| trackingAreaCode-r13 | See table 8.1.4.2-2 |  |  |
| cellIdentity-r13 | Cell ID for the simulated cell |  |  |
| cellBarred-r13 | notBarred |  |  |
|  | barred |  | NTN |
| intraFreqReselection-r13 | notAllowed |  |  |
| } |  |  |  |
| cellSelectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -70 (-140 dBm) | For RF/RRM test cases | RF |
|  | -106 dBm | For signalling test cases | Signaling |
| q-QualMin-r13 | -20 (-20dB) |  |  |
| } |  |  |  |
| p-Max-r13 | Not Present |  |  |
| freqBandIndicator-r13 | Operating band under test. |  |  |
| freqBandInfo-r13 | Not Present |  |  |
| multiBandInfoList-r13 | Not Present |  |  |
| downlinkBitmap-r13 | Not Present |  |  |
| eutraControlRegionSize-r13 | n2 |  | Inband-Same, Inband- Different |
|  | Not Present |  | Standalone, Guard |
| nrs-CRS-PowerOffset-r13 | dB0 |  | Inband- Same |
|  | Not Present |  | Inband-Different, Standalone, Guardband |
| schedulingInfoList-r13 | See subclause 8.1.4.3.1 |  |  |
| si-WindowLength-r13 | ms160 |  |  |
| si-RadioFrameOffset-r13 | Not Present |  |  |
| systemInfoValueTagList-r13 | Not Present |  |  |
| lateNonCriticalExtension | Not Present |  |  |
| nonCriticalExtension SEQUENCE { | Not Present |  | FDD |
| cellSelectionInfo-v1350 | Not Present |  | TDD |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellSelectionInfo-v1430 | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nrs-CRS-PowerOffset-v1450 | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| tdd-Parameters-r15 SEQUENCE { |  |  |  |
| tdd-Config-r15 SEQUENCE { |  |  |  |
| subframeAssignment-r15 | sa1 |  |  |
| specialSubframePatterns-r15 | ssp6 |  |  |
| } |  |  |  |
| tdd-SI-CarrierInfo-r15 | anchor |  |  |
| tdd-SI-SubframesBitmap-r15 | Not Present |  |  |
| } |  |  |  |
| schedulingInfoList-v1530 | Not Present |  |  |
| nonCriticalExtension SEQUENCE { | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | NTN |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | notBarred |  |  |
| } |  |  |  |
| nonCriticalExtension | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| ATTACH_WITH_PDN | If the UE shall perform ATTACH with PDN connectivity procedure. |
| ATTACH_WITHOUT_PDN | If the UE may perform ATTCH without PDN connectivity procedure. |
| Inband_Same | In-band with same PCI test environment |
| Inband_Different | In-band with different PCI test environment |
| Standalone | Standalone test environment |
| Guardband | Guard-band test environment |
| RF | For RF/RRM test cases |
| Signaling | For Signaling test cases |
| NTN | Non-Terrestrial Networks test environment |

8.1.4.3.3	NB-IoT Common contents of system information blocks

-	SystemInformationBlockType2-NB

The IE SystemInformationBlockType2-NB contains radio resource configuration information that is common for all UEs.

Table 8.1.4.3.3-1: SystemInformationBlockType2-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon-r13 SEQUENCE {} | RadioResourceConfigCommonSIB-NB-DEFAULT | See subclause 8.1.6 |  |
| ue-TimersAndConstants-r13 SEQUENCE { |  |  |  |
| t300-r13 | ms10000 |  |  |
| t301-r13 | ms10000 |  |  |
| t310-r13 | ms1000 |  |  |
| n310-r13 | n2 |  |  |
| t311-r13 | ms1000 |  |  |
| n311-r13 | n1 |  |  |
| } |  |  |  |
| freqInfo-r13 SEQUENCE { |  |  |  |
| ul-CarrierFreq-r13 | Not present | Default UL EARFCN applies | Standalone |
| ul-CarrierFreq-r13 | See subclause 8.1.3.1 |  | Inband_Same, Inband_Different, Guardband |
| additionalSpectrumEmission-r13 | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4F -1. |  |
|  | 4 (NS_04) |  | NS_04 |
| } |  |  |  |
| timeAlignmentTimerCommon-r13 | infinity |  |  |
| multiBandInfoList-r13 SEQUENCE {} | Not present |  |  |
| lateNonCriticalExtension SEQUENCE { | Not present |  | FDD |
| freqInfo-v1530 SEQUENCE { |  |  | TDD |
| tdd-UL-DL-AlignmentOffset-r15 | khz0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Inband_Same | In-band with same PCI test environment |
| Inband_Different | In-band with different PCI test environment |
| Standalone | Standalone test environment |
| Guardband | Guard-band test environment |
| NS_04 | When using the NS_04 frequency table variants in subclause 8.1.3.1 |

-	SystemInformationBlockType3-NB

The IE SystemInformationBlockType3-NB contains cell re-selection information common for intra-frequency, inter-frequency as well as intra-frequency cell re-selection information other than neighbouring cell related.

Table 8.1.4.3.3-2: SystemInformationBlockType3-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |  |  |
| cellReselectionInfoCommon-r13 SEQUENCE { |  |  |  |  |
| q-Hyst-r13 | dB0 | To reduce interference between intra-frequency multiple cells |  |  |
| } |  |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE-r13 { |  |  |  |  |
| s-NonIntraSearch-r13 | 31 (62dB) |  |  |  |
| } |  |  |  |  |
| intraFreqCellReselectionInfo-r13 SEQUENCE { |  |  |  |  |
| q-RxLevMin-r13 | -70 (-140 dBm) | For RF/RRM test cases |  |  |
|  | -106 dBm | For signalling test cases |  |  |
| q-QualMin-r13 | Not present |  |  |  |
| p-Max-r13 | Not present |  |  |  |
| s-IntraSearchP-r13 | 31 (62dB) |  |  |  |
| t-Reselection-r13 | 0 |  |  |  |
| } |  |  |  |  |
| freqBandInfo-r13 | Not present |  |  |
| multiBandInfoList-r1 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |  |

-	SystemInformationBlockType4-NB

The IE SystemInformationBlockType4-NB contains neighbouring cell related information relevant only for intra-frequency cell re-selection. The IE includes cells with specific re-selection parameters.

Table 8.1.4.3.3-3: SystemInformationBlockType4-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4-NB-r13 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList-r13 SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE {} | Not present | Not required unless Qoffset configuration is tested. When Qoffset configuration is tested, see table 8.3.3.1.1-1 |  |
| intraFreqBlackCellList SEQUENCE-r13 (SIZE (1..maxCellBlack)) OF SEQUENCE {} | Not present | Not required unless Blacklisted cell list configuration is tested. When Blacklisted cell list configuration is tested, see table FFS. |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

-	SystemInformationBlockType5-NB

The IE SystemInformationBlockType5-NB contains information relevant only for inter-frequency cell re-selection i.e. information about other NB-IOT frequencies and inter-frequency neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

Table 8.1.4.3.3-4: SystemInformationBlockType5-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType5-NB-r13 ::= SEQUENCE { |  |  |  |  |
| interFreqCarrierFreqList-r13 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | The same number of entries as the configured inter-freq carriers. For Signalling test cases see clause 8.3.2.3 | n denotes the index of the entry |  |  |
| dl-CarrierFreq-r13[n] SEQUENCE { | Downlink EARFCN under test. For Signalling test cases see clause 8.3.2.3 |  |  |  |
| carrierFreq-r13[n] | Downlink EARFCN under test. For Signalling test cases see clause 8.3.2.3 |  |  |  |
| carrierFreqOffset-r13 [n] | Not present |  |  |  |
| carrierFreqOffset-r13 [n] | Offset of the NB-IOT channel number under test to the EARFCN under test |  | RRM-InterFreq |  |
| } |  |  |  |  |
| q-RxLevMin-r13[n] | -65 (-130 dBm) | For RF/RRM test cases |  |  |
|  | -106 dBm | For signalling test cases |  |  |
| q-QualMin-r13[n] | Not present |  |  |  |
| p-Max-r13[n] | Not present |  |  |  |
| q-OffsetFreq-r13[n] | dB0 | Qoffset doesn’t apply by default. |  |  |
| interFreqNeighCellList-r13[n] SEQUENCE (SIZE (1..maxCellInter)) OF SEQUENCE {} | Not present | Not required unless Qoffset configuration is tested |  |  |
| interFreqBlackCellList-r13[n] SEQUENCE (SIZE (1..maxCellBlack)) OF SEQUENCE {} | Not present | Not required unless Blacklisted cell list configuration is tested |  |  |
| multiBandInfoList-r13 | Not present |  |  |
| } |  |  |  |
| t-Reselection-r13 | s0 |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| RRM-InterFreq | Used for RRM NB-IOT inter-frequency mobility scenarios |

-	SystemInformationBlockType14-NB

The IE SystemInformationBlockType14-NB contains the AB parameters.

Table 8.1.4.3.3-5: SystemInformationBlockType14-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType14-NB-r13 ::= SEQUENCE { |  |  |  |
| ab-Param-r13 CHOICE { |  |  |  |
| ab-Common-r13 SEQUENCE { |  |  |  |
| ab-Category-r13 | a | AB applies to all UEs |  |
| ab-BarringBitmap-r13 | '1111111111'B | All UE classes 0-9 |  |
| ab-BarringExceptionData-r13 | Not present |  |  |
| ab-BarringForSpecialAC-r13 | '11111'B | All UE classes 11-15 |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

-	SystemInformationBlockType15-NB

The IE SystemInformationBlockType15-NB contains the MBMS Service Area Identities (SAI) of the current and/ or neighbouring carrier frequencies.

Table 8.1.4.3.3-5A: SystemInformationBlockType15-NB

FFS

-	SystemInformationBlockType16-NB

The IE SystemInformationBlockType16-NB contains information related to GPS time and Coordinated Universal Time (UTC). The UE may use the parameters provided in this system information block to obtain the UTC, the GPS and the local time.

Table 8.1.4.3.3-6: SystemInformationBlockType16-NB

FFS

-	SystemInformationBlockType20-NB

The IE SystemInformationBlockType20-NB contains the information required to acquire the control information associated with transmission of MBMS using SC-PTM.

Table 8.1.4.3.3-7: SystemInformationBlockType20-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType20-NB-r14 ::= SEQUENCE { |  |  |  |
| npdcch-SC-MCCH-Config-r14 SEQUENCE { |  |  |  |
| npdcch-NumRepetitions-SC-MCCH-r14 | r8 |  |  |
| npdcch-StartSF-SC-MCCH-r14 | v1dot5 |  |  |
| npdcch-Offset-SC-MCCH-r14 | zero |  |  |
| } |  |  |  |
| sc-mcch-CarrierConfig-r14 CHOICE { |  |  |  |
| dl-CarrierIndex-r14 | 0 |  |  |
| } |  |  |  |
| sc-mcch-RepetitionPeriod-r14 | rf32 |  |  |
| sc-mcch-Offset-r14 | 1 |  |  |
| sc-mcch-ModificationPeriod-r14 | rf512 |  |  |
| sc-mcch-SchedulingInfo-r14 | Not present |  |  |
| lateNonCriticalExtension |  |  |  |
| } |  |  |  |

-	SystemInformationBlockType22-NB

The IE SystemInformationBlockType22-NB contains radio resource configuration for paging and random access procedure on non-anchor carriers.

Table 8.1.4.3.3-8: SystemInformationBlockType22-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType22-NB-r14 ::= SEQUENCE { |  |  |  |
| dl-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF DL-ConfigCommon-NB-r14 SEQUENCE { |  |  |  |
| DL-ConfigCommon-NB-r14[1] SEQUENCE { |  |  |  |
| dl-CarrierConfig-r14 | DL-CarrierConfigCommon-NB-DEFAULT |  |  |
| pcch-Config-r14 | Not present |  |  |
| wus-Config-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommon-NB-r14 SEQUENCE { |  |  |  |
| UL-ConfigCommon-NB-r14[1] SEQUENCE { |  |  |  |
| ul-CarrierFreq-r14 | See subclause 8.1.3.1 |  |  |
| nprach-ParametersList-r14 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| pagingWeightAnchor-r14 | Not present |  |  |
| nprach-ProbabilityAnchorList-r14 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

-	SystemInformationBlockType23-NB

The IE SystemInformationBlockType23-NB contains radio resource configuration for NPRACH resources using preamble format 2 on non-anchor carriers.

Table 8.1.4.3.3-9: SystemInformationBlockType23-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType23-NB-r15 ::= SEQUENCE { |  |  |  |
| ul-ConfigList-v1530 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommon-NB-v1530 SEQUENCE { | 1 entry |  |  |
| nprach-ParametersListFmt2-r15 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ParametersFmt2-NB-r15 SEQUENCE { | 1 entry |  |  |
| nprach-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-Periodicity-r15 | ms640 |  |  |
| nprach-StartTime-r15 | ms10 |  |  |
| nprach-SubcarrierOffset-r15 | n12 |  |  |
| nprach-NumSubcarriers-r15 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r15 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r15 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r15 | v4 |  |  |
| npdcch-Offset-RA-r15 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r15 | n8 |  |  |
| npdcch-CarrierIndex-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| nprach-ParametersListFmt2EDT-r15 | Not present |  |  |
| } |  |  |  |
| ul-ConfigListMixed-v1530 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

-	SystemInformationBlockType31-NB

The IE SystemInformationBlockType31-NB contains satellite assistance information for the serving cell.

Table 8.1.4.3.3-10: SystemInformationBlockType31-NB

| Derivation Path: TS 36.331 [17], clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  | NTN |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -11605245 |  |  |
| positionY-r17 | -30240830 |  |  |
| positionZ-r17 | -1854603 |  |  |
| velocityVX-r17 | 44 |  |  |
| velocityVY-r17 | -31 |  |  |
| velocityVZ-r17 | 195 |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 7681821 |  |  |
| nta-CommonDrift-r17 | -37 |  |  |
| nta-CommonDriftVariation-r17 | 0 |  |  |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s240 |  |  |
| epochTime-r17 | Not present |  |  |
| k-Offset-r17 | 550 |  |  |
| k-Mac-r17 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NTN | Non-Terrestrial Networks test environment |

-	SystemInformationBlockType32-NB

The IE SystemInformationBlockType32-NB contains satellite assistance information for prediction of discontinuous coverage.

Table 8.1.4.3.3-11: SystemInformationBlockType32-NB

| Derivation Path: TS 36.331 [17], clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType32-NB-r17 ::= SEQUENCE { |  |  |  |
| satelliteInfoList-r17 | FFS |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

[End of change]