3GPP TSG-RAN5 Meeting #96-e	R5-225050r1

# Electronic, 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3188 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test case 9.1.10.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, eNS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. The UE might have some S-NSSAI from previous test case executions that can interfere with the test case. 2. As per 38.508-1/Table 4.7.1-7A the UE is set up with 4 alloved S-NSSAIs that the UE may or may not store during switch-off as per 4.6.2.2/24.501: " NOTE 2: Whether the UE stores the allowed NSSAI and the mapped S-NSSAI(s) for the allowed NSSAI also when the UE is switched off is implementation specific." The test case does not expect the UE to have any allowed NSSAI at switch-on. 3. According to TS 24.501/5.5.1.2.2 the UE can decide what subset of S-NSSAI is included i nthe REQUESTED NSSAI IE: " If the UE has allowed NSSAI or configured NSSAI or both for the current PLMN or SNPN, the requested NSSAI shall be either: a) the configured NSSAI for the current PLMN or SNPN, or a subset thereof as described below; b) the allowed NSSAI for the current PLMN or SNPN, or a subset thereof as described below; or" The test case sends PENDING NSSAI in step 12 and that is only allowed if the UE has sent REQUESTED NSSAI IE that included the S-NSSAI(s) in the PENDING NSSAI as per 4.6.1/24.501: “When the network slice-specific authentication and authorization procedure is to be initiated for one or more S-NSSAIs in the requested NSSAI ..., these S-NSSAI(s) will be included in the pending NSSAI.” The SS has to follow allowed NW behaviour as above as otherwise the UE behaviour is indeterministic 4. In the specific message contents NSSAI and S-NSSI IEs are not aligned with TS 24.501. 5. Specific message contents for steps 28a1-28a17 was missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Added sentence to indicate that the pre-amble needs to scrub the UE from all exisiting S-NSSAIs. 2. Updated REGISTRATION ACCEPT message for pre-amble to remove Allowed NSSAI and other NSSSAI in case they are later added to 38.508-1. 3. Updated specific message contents to make sure that the SS authenticates an S-NSSAI requested by the UE. 4. Aligned specific message contens with 24.501. 5. Specific message contents for steps 28a1 - 28a17 was added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.10.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-225050->R5-225050r1 - Added sentence to indicate that the pre-amble needs to scrub the UE from all exisiting S-NSSAIs. - Updated specific message contents to make sure that the SS authenticates an S-NSSAI requested by the UE - Configured NSSAI is added in the pre-amble. - Specific message contents for steps 28a1 - 28a17 was added |  |  |  |  |  |  |  |  |

#### 9.1.10.6	NSSAA / UE configuration update / Rejected NSSAI

9.1.10.6.1	Test Purpose (TP)

(1)

with { UE is in 5GMM-REGISTERED state and NSSAA failure }

ensure that {

when { SS sends the rejected NSSAI in the CONFIGURATION UPDATE COMMAND message with rejection cause of "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" }

then { UE adds the rejected S-NSSAI(s) in the rejected NSSAI and shall not attempt to use this S-NSSAI in the current PLMN over any access }

}

(2)

with { UE receives the rejected NSSAI in the CONFIGURATION UPDATE COMMAND message with rejection cause of "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" }

ensure that {

when { UE has been switched off, then switched on }

then { UE deletes the stored Rejected NSAAI for the current PLMN }

}

(3)

with { UE receives the rejected NSSAI in the CONFIGURATION UPDATE COMMAND message with rejection cause of "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" }

ensure that {

when { The UICC containing the USIM is removed }

then { UE deletes the stored Rejected NSAAI for the current PLMN }

}

9.1.10.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clause 5.4.4.3, 5.4.7.2.2, 5.5.1.2.2, 5.5.1.2.4 and TS 23.502, clause 4.2.9.2. Unless otherwise stated these are Rel-16 requirements.

[TS 24.501 clause 5.4.4.3]

The UE receiving the rejected NSSAI in the CONFIGURATION UPDATE COMMAND message takes the following actions based on the rejection cause in the rejected S-NSSAI(s):

"S-NSSAI not available in the current PLMN or SNPN"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN as specified in subclause 4.6.2.2 and shall not attempt to use this S-NSSAI(s) in the current PLMN until switching off the UE, the UICC containing the USIM is removed, the entry of the "list of subscriber data" with the SNPN identity of the current SNPN is updated, or the rejected S-NSSAI(s) are removed or deleted as described in subclause 4.6.2.2.

"S-NSSAI not available in the current registration area"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current registration area as specified in subclause 4.6.2.2 and shall not attempt to use this S-NSSAI(s) in the current registration area until switching off the UE, the UE moving out of the current registration area, the UICC containing the USIM is removed, the entry of the "list of subscriber data" with the SNPN identity of the current SNPN is updated, or the rejected S-NSSAI(s) are removed or deleted as described in subclause 4.6.2.2.

"S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the failed or revoked NSSAA as specified in subclause 4.6.2.2 and shall not attempt to use this S-NSSAI in the current PLMN over any access until switching off the UE, the UICC containing the USIM is removed, the entry of the "list of subscriber data" with the SNPN identity of the current SNPN is updated, or the rejected S-NSSAI(s) are removed or deleted as described in subclause 4.6.1 and 4.6.2.2.

9.1.10.6.3	Test description

9.1.10.6.3.1	Pre-test conditions

System Simulator:

NGC Cell A belongs to Home PLMN and TAI1 and set as serving cell;

UE:

None.

Preamble:

The UE has no other S-NSSAI(s) than the one(s) added in the pre-amble of this test case.

The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4].

9.1.10.6.3.2	Test procedure sequence

Table 9.1.10.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits a REGISTRATION REQUEST message including NSSAA bit | --> | REGISTRATION REQUEST | - | - |
| 3-11 | Steps 5 to 13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 12 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI and Pending NSSAI | <-- | REGISTRATION ACCEPT | - | - |
| 13 | The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. | --> | REGISTRATION COMPLETE | - | - |
| - | EXCEPTION: Step 14a1 is performed if pc_noOf_PDUsSameConnection > 0. | - | - | - | - |
| 14a1 | The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. | - | - | - | - |
| 15 | The SS transmits a NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND message with an EAP-Request message. | <-- | 5GMM: NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND |  |  |
| 16 | The UE responds with a NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE message, with an EAP-Response message and NSSAI | --> | 5GMM: NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE |  |  |
| 17 | The SS transmits a NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT message with an EAP-failure message. | <-- | 5GMM: NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT |  |  |
| 18 | The SS transmits a CONFIGURATION UPDATE COMMAND message including a new rejected NSSAI list. | <-- | CONFIGURATION UPDATE COMMAND | - | - |
| 19 | The UE transmits a CONFIGURATION UPDATE COMPLETE message. | --> | CONFIGURATION UPDATE COMPLETE | - | - |
| 20 | The SS transmits an RRCRelease message. | - | - | - | - |
| 21 | Check: Is S-NSSAI=2 in the Rejected NSSAI list with cause ‘S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization’’ associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? | - | - | 1 | P |
| 22 | Cause the UE to request establishment of PDU session with S-NSSAI with SST of the first S-NSSAI in the REQUESTED NSSAI IE in step 2.(Note 1) | - | - | - | - |
| 23 | Check: Does the UE transmit an RRCSetupRequest message? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 24 | Switch off procedure in RRC_IDLE specified in TS 38.508-1 subclause 4.9.6.1 is performed. | - | - | - | - |
| 25 | The SS configures NGC Cell A as the "Non-suitable cell" | - | - | - | - |
| 26 | The UE is switched on. | - | - | - | - |
| 27 | Check: Using AT/MMI command (+C5GNSSAIRDP) verify that S-NSSAI with SST of the first S-NSSAI in the REQUESTED NSSAI IE in step 2 is not in the Rejected NSSAI list associated with current PLMN |  |  | 2 | P |
| 27 | The SS configures NGC Cell A as the "Serving cell". | - | - | - | - |
| - | EXCEPTION: Steps 28a1 to 28a27 take place if pc_USIM_Removal = TRUE | - | - | - | - |
| 28a1-a21 | Steps 2 to 22 above are repeated | - | - | - | - |
| 28a22 | Cause removal of USIM from the UE without powering down. | - | - | - | - |
| 28a23 | Steps 1a2 to 1a5 of generic procedure for Switch off in RRC_IDLE specified in TS 38.508-1[4], subclause 4.9.6.1 is performed. | - | - | - | - |
| 28a24 | The SS configures NGC Cell A as the "Non-suitable cell" | - | - | - | - |
| 28a25 | The USIM is inserted into the UE. | - | - | - | - |
| 28a26 | Check: Using AT/MMI command (+C5GNSSAIRDP) verify that S-NSSAI with SST of the first S-NSSAI in the REQUESTED NSSAI IE in step 28a1 is not in the Rejected NSSAI list associated with current PLMN | - | - | 3 | P |
| 28a27 | The SS configures NGC Cell A as the "Serving cell". | - | - | - | - |
| 29-47 | Steps 2 to 20a1 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| Note1: The request to establish a PDU session may be performed by MMI or AT Command. |  |  |  |  |  |

9.1.10.6.3.3	Specific message contents

Table 9.1.10.6.3.3-1: REGISTRATION REQUEST (step 2, Table 9.1.10.6.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘001’B | Initial registration |  |
| 5GMM capability |  |  |  |  |
| NSSAA |  | ‘1’B | Network slice-specific authentication and authorization supported |  |

Table 9.1.10.6.3.3-2: REGISTRATION ACCEPT (preamble, step 12 and step 28a11, Table 9.1.10.6.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result |  |  |  |  |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| NSSAA Performed |  | ‘1’B | Network slice-specific authentication and authorization is to be performed |  |
| Allowed NSSAI |  | Not Present |  | preamble |
| Allowed NSSAI |  | 1 S-NSSAI |  | Step 12, Step 28a11 |
| Allowed NSSAI IEI |  |  | NSSAI value 1 |  |
| Length of Allowed NSSAI |  | '0000 0010'B | 2 octets |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | SST value 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Configured NSSAI |  |  | 2 S-NSSAI | Preamble |
| S-NSSAI IEI |  |  | NSSAI value 2 |  |
| Length of Configured NSSAI |  | '0000 010'B | 2 octets |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | SST value 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000010’B | SST value 2 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Pending NSSAI |  |  |  | Step 12, Step 28a11 |
| Pending NSSAI IEI |  |  | NSSAI value 3 |  |
| Length of Pending NSSAI contents |  | '0000 0010'B | 2 octets |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | SST value 1 or 2 | SST of the first S-NSSAI in the REQUESTED NSSAI IE in step 2. |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |

Table 9.1.10.6.3.3-3: NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND (step 15 and step 28a14, Table 9.1.10.6.3.2-1)

| Derivation Path: TS 38.508-1 Table 4.7.1-31 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | SST value 1 or 2 | SST of the first S-NSSAI in the REQUESTED NSSAI IE in step 2. |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |

Table 9.1.10.6.3.3-4: NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE (step 16 and step 28a15, Table 9.1.10.6.3.2-1)

| Derivation Path: TS 38.508-1 Table 4.7.1-32 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | SST value 1 or 2 | SST of the first S-NSSAI in the REQUESTED NSSAI IE in step 2. |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |

Table 9.1.10.6.3.3-5: NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT (step 17 and step 28a16, Table 9.1.10.6.3.2-1)

| Derivation Path: TS 38.508-1, Table 4.7.1-33 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  | Set according to specific message content |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | SST value 1 or 2 | SST of the first S-NSSAI in the REQUESTED NSSAI IE in step 2. |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| EAP message |  | EAP-failure |  |  |

Table 9.1.10.6.3.3-6: CONFIGURATION UPDATE COMMAND (step 18 and step 28a17, Table 9.1.10.6.3.2-1)

| Derivation path: TS 38.508-1, Table 4.7.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Configuration update indication | 0001 | Acknowledgement requested |  |
| Rejected NSSAI |  |  |  |
| Rejected NSSAI IEI |  | Rejected S-NSSAI value 1 |  |
| Length of Rejected NSSAI contents | '00000101'B | 5 octets |  |
| Length of rejected S-NSSAI[1] | '0100'B | 4 octest |  |
| Cause value | ‘0010’B | S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization |  |
| SST |  | SST value 1 or 2 |  | SST of the first S-NSSAI in the REQUESTED NSSAI IE in step 2. |  |  |  |
| SD | Not present |  |  |