3GPP TSG-RAN5 Meeting #96-e	R5-225060r1

# Electronic, 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3193 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test case for RRC downlink segmentation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At RAN5#90-e it was agreed that that test cases are required to test RRC DL segmentation(R5-210581) and at RAN5#95(R5-223261) the test method was agreed. No test case has been added yet for DL segmeantation of the RRCReconfiguration message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case for the RRC reconfiguration / DL segment transfer. Test case was added to re-named sub-clause 8.1.5.9 to keep UL and DL segmentation test cases in one sub-clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage for DL message segmentation will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.9.2[New] |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-225060->R5-225060r1 - Removed "Check" from step 7 as it is not testing a TP - Removed references to RRC message segments from the "Message" column - Correcting various typos |  |  |  |  |  |  |  |  |

#### 8.1.5.9	Message Segment transfer

#### [TEXT SKIPPED]

##### 8.1.5.9.2	RRC reconfiguration / DL segment transfer

8.1.5.9.2.1	Test Purpose (TP)

(1)

with { UE having completed the RRC connection establishment procedure}

ensure that {

when { SS initiates the DL Dedicated Message Segment transfer procedure to send an RRCReconfiguration message to establish a DRB and SRB2}

then { UE successfully completes the reconfiguration }

}

(2)

with { UE in NR RRC_CONNECTED state with SRB2 and has previously received the first and second  segment of an RRCReconfiguration message including SRB2 configuration }

ensure that {

when { SS releases the RRC connection }

then { UE discards all segments }

}

8.1.5.9.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in:  TS 38.331, clauses 5.3.11 and 5.7.6.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.11]

The UE shall:

...

1>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

[TS 38.331, clause 5.7.6.3]

Upon receiving DLDedicatedMessageSegment message, the UE shall:

1>	store the segment included in rrc-MessageSegmentContainer;

1>	if all segments of the message have been received:

2>	assemble the message from the received segments and process the message according to 5.3.5.3 for the RRCReconfiguration message or 5.3.13.4 for the RRCResume message;

2>	discard all segments.

8.1.5.9.2.3	Test description

8.1.5.9.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

-	None

Preamble:

-	If pc_IP_Ping is set to TRUE then the UE is in 5GS state 1N-A, PDU SESSION ACTIVE according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-1

-	Else, the UE is in 5GS state 1N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.9.2.3.2	Test procedure sequence

Table 8.1.5.9.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. | <-- | NR RRC: Paging | - | - |
| 2 | UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | SS transmits an RRCSetup message | <-- | NR RRC: RRCSetup | - | - |
| 4 | The UE transmits an RRCSetupComplete to confirm the successful completion of the connection establishment. | --> | NR RRC: RRCSetupComplete | - | - |
| 5 | SS transmits a SecurityModeCommand message to activate AS security. | <-- | NR RRC: SecurityModeCommand | - | - |
| 6 | Using the same slot as the SecurityModeCommand message in step 5, the SS transmits an DLDedicatedMessageSegment containing the first segment of the RRCReconfiguration message to establish a data radio bearer, DRB1 and signalling radio bearer, SRB2. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 7 | The UE transmit a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 8 | The SS transmits a DLDedicatedMessageSegment containing the second(last) segment of the RRCReconfiguration message. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 9 | Check: Does the UE transmit an RRCReconfigurationComplete message to confirm the establishment of data radio bearer, DRB1 and signalling radio bearer, SRB2? | --> | NR RRC: RRCReconfigurationComplete | 1 | P |
| 10 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#1 established in Step 8? | - | - | 1 | - |
| 11 | The SS transmits a DLDedicatedMessageSegment containing the first segment of the RRCReconfiguration message to modify SRB and DRB configuration. | <-- | NR RRC: DLDedicatedMessageSegment |  |  |
| 12 | The SS transmits the second segment of the RRCReconfiguration message to modify SRB and DRB configuration. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 13 | The SS releases the RRC connection. | - | - | - | - |
| 14 | Step 1-8 of generic test procedure in TS 38.508-1 [4] subclause 4.5.4 to bring the UE in RRC_CONNECTED mode. | - | - | - | - |
| 15 | The SS transmits the third(last) segment of the RRCReconfiguration message to modify SRB2 and DRB configuration. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 16 | Check: Does the UE transmit an RRCReconfigurationComplete message in the next 3s? | --> | NR RRC: RRCReconfigurationComplete | 2 | F |
| 17 | The SS releases the RRC connection. | - | - | - | - |
| 18 | Step 1-8 of generic test procedure in TS 38.508-1 [4] subclause 4.5.4 to bring the UE in RRC_CONNECTED mode. | - | - | - | - |
| 19 | The SS transmits an DLDedicatedMessageSegment containing the first segment of the RRCReconfiguration message to modify SRB and DRB configuration. |  | NR RRC: DLDedicatedMessageSegment |  |  |
| 20 | The SS transmits the second segment of the RRCReconfiguration message to modify SRB and DRB configuration. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 21 | The SS transmits the third(last) segment of the RRCReconfiguration message to modify SRB2 and DRB configuration. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 22 | Check: Does the UE transmit an RRCReconfigurationComplete message in the next 3s? | --> | NR RRC: RRCReconfigurationComplete | 1 | P |
| 23 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#1 established in Steps 12-21? | - | - | 1 |  |
| 24 | The SS transmits an OPEN UE TEST LOOP Message. | <-- | OPEN UE TEST LOOP | - | - |
| 25 | The UE transmits an OPEN UE TEST LOOP COMPLETE Message. | --> | OPEN UE TEST LOOP COMPLETE | - | - |
| 26 | The SS transmits DEACTIVATE TEST MODE message. | <-- | DEACTIVATE TEST MODE | - | - |
| 27 | The UE transmits DEACTIVATE TEST MODE COMPLETE message. | --> | DEACTIVATE TEST MODE COMPLETE | - | - |
| NOTE 1: The request to establish a PDU session may be performed by MMI or AT Command. |  |  |  |  |  |

8.1.5.9.2.3.3	Specific message contents

Table 8.1.5.9.2.3.3-1: DLDedicatedMessageSegment (Step 6,8, 11, 17 Table 8.1.5.9.2.3.2-1)

| Derivation Path: TS 38.508-1[4], table 4.6.1-2B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlDedicatedMessageSegment-r16 SEQUENCE { |  |  |  |  |
| segmentNumber-r16 |  | 0 |  | Step 6, Step 11, Step 19 |
| segmentNumber-r16 |  | 1 |  | Step 8, Step 12, Step 20 |
| segmentNumber-r16 |  | 2 |  | Step 15, Step 21 |
| rrc-MessageSegmentContainer-r16 |  | First segment of the RRCReconfiguration message | Table 8.1.5.9.2.3.3-2 | Step 6 |
| rrc-MessageSegmentContainer-r16 |  | Last segment of the RRCReconfiguration message | Table 8.1.5.9.2.3.3-2 | Step 8 |
| rrc-MessageSegmentContainer-r16 |  | First segment of the RRCReconfiguration message | Table 8.1.5.9.2.3.3-4 | Step 11, Step 19 |
| rrc-MessageSegmentContainer-r16 |  | Second segment of the RRCReconfiguration message | Table 8.1.5.9.2.3.3-4 | Step 12, Step 20 |
| rrc-MessageSegmentContainer-r16 |  | Last (third) segment of the RRCReconfiguration message | Table 8.1.5.9.2.3.3-4 | Step 15, Step 21 |
| rrc-MessageSegmentType-r16 |  | notLastSegment |  | Step 6, Step 11, Step 19, Step 12, Step 20 |
| rrc-MessageSegmentType-r16 |  | LastSegment |  | Step 8, Step 15, Step 21 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.5.9.2.3.3-2: RRCReconfiguration (Table 8.1.5.9.2.3.3-1)

| Derivation Path: TS 38.508-1[4], table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with conditions SRB2 and DRB1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig Table 8.1.5.9.2.3.3-3) |  |
| lateNonCriticalExtension |  | '00,,,00'o | Add enough padding that: maximum PDCP SDU size < encoded DLDedicatedMessageSegment size < 2 x maximum PDCP SDU size |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.5.9.2.3.3-3: CellGroupConfig (Table 8.1.5.9.2.3.3-2)

| Derivation Path: TS 38.508-1[4], table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { |  | 2 entries |  |  |
| RLC-Bearer-Config[1] SEQUENCE { |  |  | entry 1 |  |
| logicalChannelIdentity |  | LogicalChannelIdentity |  |  |
| servedRadioBearer CHOICE { |  |  |  |  |
| drb-Identity |  | DRB-Identity with condition DRB1 |  |  |
| } |  |  |  |  |
| reestablishRLC |  | Not present |  |  |
| RLC-Config |  | RLC-Config with condition AM according to 38.508-1 [4], table 4.6.3-149 |  |  |
| mac-LogicalChannelConfig |  | LogicalChannelConfig according to 38.508-1 [4], table 4.6.3-66 |  |  |
| } |  |  |  |  |
| RLC-Bearer-Config[2] SEQUENCE { |  |  | entry 2 |  |
| logicalChannelIdentity |  | LogicalChannelIdentity |  |  |
| servedRadioBearer CHOICE { |  |  |  |  |
| srb-Identity |  | SRB-Identity with condition SRB2 |  |  |
| } |  |  |  |  |
| reestablishRLC |  | Not present |  |  |
| RLC-Config |  | Not present |  |  |
| mac-LogicalChannelConfig |  | LogicalChannelConfig according to 38.508-1 [4], table 4.6.3-66 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| mac-CellGroupConfig |  | Not present |  |  |
| physicalCellGroupConfig |  | Not present |  |  |
| spCellConfig SEQUENCE |  | Not present |  |  |
| } |  |  |  |  |

Table 8.1.5.9.2.3.3-4: RRCReconfiguration (Step 11, Table 8.1.5.9.2.3.2-1)

| Derivation Path: TS 38.508-1[4], table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig Table 8.1.5.9.2.3.3-5) |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | '00,,,00'o | dd enough padding that the size of the encoded DLDedicatedMessageSegment = 3 x maximum PDCP SDU size |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.5.9.2.3.3-5: CellGroupConfig (Table 8.1.5.9.2.3.3-3)

| Derivation Path: TS 38.508-1, table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { |  | 2 entries |  |  |
| RLC-Bearer-Config[1] SEQUENCE { |  |  | entry 1 |  |
| logicalChannelIdentity |  | LogicalChannelIdentity |  |  |
| servedRadioBearer CHOICE { |  |  |  |  |
| srb-Identity |  | SRB-Identity with condition SRB2 |  |  |
| } |  |  |  |  |
| reestablishRLC |  | true |  |  |
| RLC-Config |  | RLC-Config with condition AM according to 38.508-1 [4], table 4.6.3-149 |  |  |
| mac-LogicalChannelConfig |  | LogicalChannelConfig according to 38.508-1 [4], table 4.6.3-66 |  |  |
| } |  |  |  |  |
| RLC-Bearer-Config[2] SEQUENCE { |  |  | entry 2 |  |
| logicalChannelIdentity |  | LogicalChannelIdentity |  |  |
| servedRadioBearer CHOICE { |  |  |  |  |
| drb-Identity |  | DRB-Identity with condition DRB1 |  |  |
| } |  |  |  |  |
| reestablishRLC |  | true |  |  |
| RLC-Config |  | Not present |  |  |
| mac-LogicalChannelConfig |  | LogicalChannelConfig according to 38.508-1 [4], table 4.6.3-66 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| mac-CellGroupConfig |  | Not present |  |  |
| physicalCellGroupConfig |  | Not present |  |  |
| spCellConfig SEQUENCE |  | Not present |  |  |
| } |  |  |  |  |