3GPP TSG-RAN5 Meeting #96-e	R5-225192r1

# Online, 15th August 2022 - 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0796 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to FR2 CA MPR test case 6.2A.2.1 to prevent SCell drop by using UE PHR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to discussion Paper R5-225191 there is a need to include test coverage with PHR test method for Rel-15 UE - Test procedure for testing single CC MPR requirement is incorrect. When only allocating one UL CC there is no need to configure test mode with a power backoff in the UE. - Test tolerance is only complete for aggregated BW ≤ 400MHz. The test case configures different number of DL CC and UL CC, meaning the UL and DL BW is not the same. It is not clear if aggregated BW is UL BW or also including DL CCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test Procedure steps updated to account for agreed changes to ensure UE can be tested in UL-CA mode - Test procedure updated for single UL CC test points - In Table 6.2A.2.1.5-13 clarified that the BW applicability for Test tolerance is only considering UL CCs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test procedure for FR2 UL-CA test cases in Rel-15 will stay incomplete Test procedure remains incorrect for single UL CC test points Test case completeness is clarified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No message exceptions needed to enable PHR as this is already default configured in TS38.508-1 with the appropriate parameters (10 subframe period, multiple-pHR) Associated discussion paper: R5-225191 R5-225192r1: Cover page issue fixed. Aligned test procedure updates with discussion paper R5-225122r1 |  |  |  |  |  |  |  |  |

#### 6.2A.2.1	UE maximum output power reduction for CA (2UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The UPLF test mode is applicable to UEs Release 16 and forward.

-	This test case is incomplete for Power classes 1, 2, 4 Release 15.

-		The power limit request message values set by the UPLF test mode is currently applicable to equal PSD (equal channel bandwidths on all component carriers) only. Message values are pending for unequal channel bandwidths

-	Whether additional check is needed in the test procedure to ensure UE continues transmissions on the SCell is FFS

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

-	Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and intra-band non-contiguous CA are TBD.

-	Whether to further divide this test case considering the number of DL CC is FFS

-	TP analysis needs further update to reflect the selection of applicable cumulative aggregated BW

-	Following aspects are pending RAN4

Minimum requirements for cumulative aggregated bandwidth >=800MHz are within brackets

6.2A.2.1.1	Test purpose

The number of RB identified in 6.2.2.3 is based on meeting the requirements for the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2A.2.1.2	Test applicability

The requirements of this test apply to all types of NR UE release 15 and forward supporting 2UL CA.

6.2A.2.1.3	Minimum conformance requirements

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2A.2.0.

6.2A.2.1.4	Test description

6.2A.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in Table 6.2A.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.2.1.4.1-1: Intra-band Contiguous UL CA Test Configuration Table (Power Class 1, MPRnarrow)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Refer to “Test frequency” column |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation (NOTE 1) |
| Default Test Settings for a CA_nXB, CA_nXD, CA_nXG, CA_nXO Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Low | - | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC/CC2 |  | Low |  | - | - |
| 2 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC/CC2 |  | High |  | - | - |
| 3 | PCC/CC1 |  | Low |  | CP-OFDM 64QAM | 7@0 |
|  | SCC/CC2 |  | Low |  | - | - |
| 4 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | 7@NRB-7 |
|  | SCC/CC2 |  | High |  | - | - |
| Default Test Settings for a CA_nX(D-G)_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Low | - | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
| 2 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
| 3 | PCC/CC1 |  | Low |  | CP-OFDM 64QAM | 7@0 |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
| 4 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | 7@NRB-7 |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(E-O)_UL_nXO Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Low | - | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
|  | SCC4/CC5 |  | Low |  | N/A | N/A |
| 2 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
|  | SCC4/CC5 |  | High |  | N/A | N/A |
| 3 | PCC/CC1 |  | Low |  | CP-OFDM 64QAM | 7@0 |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
|  | SCC4/CC5 |  | Low |  | N/A | N/A |
| 4 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | 7@NRB-7 |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
|  | SCC4/CC5 |  | High |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Low | - | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
|  | SCC4/CC5 |  | Low |  | N/A | N/A |
|  | SCC5/CC6 |  | Low |  | N/A | N/A |
| 2 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
|  | SCC4/CC5 |  | High |  | N/A | N/A |
|  | SCC5/CC6 |  | High |  | N/A | N/A |
| 3 | PCC/CC1 |  | Low |  | CP-OFDM 64QAM | 7@0 |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
|  | SCC4/CC5 |  | Low |  | N/A | N/A |
|  | SCC5/CC6 |  | Low |  | N/A | N/A |
| 4 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | 7@NRB-7 |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
|  | SCC4/CC5 |  | High |  | N/A | N/A |
|  | SCC5/CC6 |  | High |  | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-2: Intra-band Contiguous UL CA Test Configuration Table (Power Class 1, single CC MPR requirement)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Low range, High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| Default Test Settings for a CA_nXG, CA_nXO Configuration (Cumulative aggregated BWchannel <= 200MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM PI/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | - | - |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Inner_Full_Region1 |
|  | SCC/CC2 |  |  |  | - | - |
| Default Test Settings for a CA_nXD Configuration (Cumulative aggregated BWchannel <= 400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM PI/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | - | - |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Full_Region1 |
|  | SCC/CC2 |  |  |  | - | - |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Inner_Full_Region1 |
|  | SCC/CC2 |  |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-3: Intra-band Contiguous UL CA Test Configuration Table (Power Class 1, MPRC_CA)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 and 4.3.1.2.4 for different CA bandwidth classes |  |  |  | For intra-band contiguous CA: Mid range. For intra-band non-contiguous CA: FFS. |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 and 4.3.1.2.4 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC& Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| Default Test Settings for a CA_nXB, CA_nXC_UL_nXB Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
| 4 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
| 5 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nXD Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nXB Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 | 400MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 | 400MHz |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 | 400MHz |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nXG, CA_nXO Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nXD Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 | 200MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nX(D-G)_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 4 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 5 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(E-O)_UL_nXO Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| 4 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| 5 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| 4 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| 5 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-G)_UL_nXD, CA_nX(D-O)_UL_nXD Configuration (400MHz <= Cumulative aggregated BWchannel <800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXO Configuration (400MHz <= Cumulative aggregated BWchannel <800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 200MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 200MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 200MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(O-E)_UL_nXO Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 200MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 200MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 200MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(G-I)_UL_nXG Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-O)_UL_nXD Configuration (Cumulative aggregated BWchannel <400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 50MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 50MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 50MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 50MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 50MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 50MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-O)_UL_nXO Configuration (Cumulative aggregated BWchannel <400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 50MHz | Default | - | N/A | N/A |
|  | SCC1/CC2 | 200MHz |  |  | N/A | N/A |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 | 50MHz |  |  | N/A | N/A |
|  | SCC1/CC2 | 200MHz |  |  | N/A | N/A |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 | 50MHz |  |  | N/A | N/A |
|  | SCC1/CC2 | 200MHz |  |  | N/A | N/A |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  | CP-OFDM 64QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-4: Intra-band Contiguous UL CA Test Configuration Table (Power Class 1, Non-contiguous allocation)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 and 4.3.1.2.4 for different CA bandwidth classes |  |  |  | For intra-band contiguous CA: Mid range. For intra-band non-contiguous CA: FFS. |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 and 4.3.1.2.4 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| Default Test Settings for a CA_nXB, CA_nXD, CA_XG, CA_nXO Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC/CC2 |  |  |  | CP-OFDM 64QAM | Outer_1RB_Right |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Left] |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Right] |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Left] |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Right] |
| Default Test Settings for a CA_nX(D-G), CA_nX(D-O) Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM QPSK | Outer_1RB_Left |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Left] |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Right] |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Left] |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Right] |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-5: Intra-band Contiguous UL CA Test Configuration Table (Power Class 2, 3 and 4, single CC MPR requirement)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Low range, High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| Default Test Settings for a CA_nXG, CA_nXO Configuration (Cumulative aggregated BWchannel <= 200MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM QPSK | Inner_Full |
|  | SCC/CC2 |  |  |  | - | - |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | - | - |
| Default Test Settings for a CA_nXD Configuration (Cumulative aggregated BWchannel <= 400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM QPSK | Inner_Full |
|  | SCC/CC2 |  |  |  | - | - |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-6: Intra-band Contiguous UL CA Test Configuration Table (Power Class 2, 3 and 4, MPRC_CA)

| Default Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 and 4.3.1.2.4 for different CA bandwidth classes |  |  |  |  |  |  | For intra-band contiguous CA: Mid range. For intra-band non-contiguous CA: FFS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 and 4.3.1.2.4 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | 120 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency |  |  |  | DL RB allocation |  |  |  |  |  | UL Modulation |  |  |  |  | UL RB allocation |  |  |  |
| Default Test Settings for a CA_nXB, nXC_UL_nXB Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default |  |  |  | - |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 2 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
| 3 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 4 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
| 5 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
| Default Test Settings for a CA_nXD Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default |  |  |  | - |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 2 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 3 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
| 4 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
| Default Test Settings for a CA_nXB Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz | Default |  |  |  | - |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 | 400MHz |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 2 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 | 400MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 3 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 | 400MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
| 4 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 | 400MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
| Default Test Settings for a CA_nXG, CA_nXO Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default |  |  |  | - |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 2 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
| 3 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
| Default Test Settings for a CA_nXD Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default |  |  |  | - |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 2 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
| 3 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
| Default Test Settings for a CA_nX(D-G) )_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default |  |  |  | - |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
| 2 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
| 3 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
| 4 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
| 5 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
| Default Test Settings for a CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(E-O)_UL_nXO Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default |  |  |  | Default |  |  |  |  |  | - |  |  |  |  | DFT-s-OFDM QPSK |  | Outer_Full |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  | Outer_Full |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
| 2 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  | Outer_Full |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  | Outer_Full |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
| 3 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  | Outer_Full |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  | Outer_Full |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
| 4 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  | Outer_Full |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  | Outer_Full |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
| 5 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  | Outer_Full |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  | Outer_Full |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
| Default Test Settings for a CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default |  |  | Default |  |  |  |  |  | - |  |  |  |  | DFT-s-OFDM QPSK |  |  |  | Outer_Full |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  | Outer_Full |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC5/CC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| 2 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  |  |  | Outer_Full |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  |  |  | Outer_Full |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC5/CC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| 3 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | Outer_Full |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | Outer_Full |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC5/CC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| 4 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  | Outer_Full |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  | Outer_Full |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC5/CC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| 5 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  | Outer_Full |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  | Outer_Full |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC5/CC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| Default Test Settings for a CA_nX(D-G)_UL_nXD, CA_nX(D-O)_UL_nXD Configuration (400MHz <= Cumulative aggregated BWchannel <800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz |  |  | Default |  |  |  |  |  | - |  |  |  |  | DFT-s-OFDM QPSK |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| 2 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| 3 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| 4 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| Default Test Settings for a CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXO Configuration (400MHz <= Cumulative aggregated BWchannel <800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz |  | Default |  |  |  |  |  | - |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| 2 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| 3 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| 4 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | c |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| Default Test Settings for a CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz |  | Default |  |  |  |  |  | - |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| 2 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| 3 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| 4 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| Default Test Settings for a CA_nX(O-E)_UL_nXO Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default |  |  |  |  |  | - |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 2 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 3 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 4 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| Default Test Settings for a CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default |  |  |  |  |  | - |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 2 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 3 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 4 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| Default Test Settings for a CA_nX(G-I)_UL_nXG Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default |  |  |  |  |  | - |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 2 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 3 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 4 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| Default Test Settings for a CA_nX(D-O)_UL_nXD Configuration (Cumulative aggregated BWchannel <400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 50MHz | Default |  |  |  |  | - |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | Outer_Full |
|  | Wgap | 40MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC2/CC3 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC3/CC4 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
| 2 | PCC/CC1 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  |  | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  |  | Outer_Full |
|  | Wgap | 40MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC2/CC3 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC3/CC4 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
| 3 | PCC/CC1 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  |  |  | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  |  |  | Outer_Full |
|  | Wgap | 40MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC2/CC3 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC3/CC4 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
| Default Test Settings for a CA_nX(D-O)_UL_nXO Configuration (Cumulative aggregated BWchannel <400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 50MHz | Default |  |  |  |  | - |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | Wgap | 40MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC2/CC3 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | Outer_Full |
| 2 | PCC/CC1 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | Wgap | 40MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC2/CC3 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  |  | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  |  | Outer_Full |
| 3 | PCC/CC1 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | Wgap | 40MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC2/CC3 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  |  |  | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  |  |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-7: Intra-band Contiguous UL CA Test Configuration Table (Power Class 2, 3 and 4, Non-contiguous allocation)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Mid range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| Default Test Settings for a CA_XG, CA_nXO Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM QPSK | Outer_1RB_Left |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Left] |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Right] |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Left] |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Right] |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.2A.2.1.4.1-1 to Table 6.2A.2.1.4.1-7.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2A.2.1.4.3.

6.2A.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 and Annex C.3.0 for all downlink physical channels

2.	The SS shall configure SCC as per TS 38.508-1 [10] subclause 5.5.1 Procedure to configure SCC(s) for NR RF CA testing. Message contents are defined in subclause 6.2A.2.1.4.3.

3.	Apply the test step based on the 5G NR UE Release:

3a.	For Release 16 and forward 5G NR UEs supporting the UPLF test mode: SS applies a backoff on the PCell power by activating the UE Power Limit Function (UPLF).  The ACTIVATE POWER LIMIT REQUEST procedure is performed as specified in TS 38.508-1 [10] clause 4.9.32 using condition ‘NR FR2 2CA’. UE shall transmit ACTIVATE POWER LIMIT RESPONSE to SS. Go to step 4.

3b.	For Release 15 5G NR UEs and 5G NR UEs not supporting the UPLF test mode: No action

3c. For testing single CC MPR requirement: No action

4.  SS activates SCC by sending the activation MAC CE (Refer TS 38.321, clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[25], clause 9.3).

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.2.1.4.1-1 to Table 6.2A.2.1.4.1-7. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

7.	Apply the test step based on the 5G NR UE Release:

7a. For Release 16 and forward 5G NR UEs supporting the UPLF test mode: Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

7b. For Release 15 5G NR UEs and 5G NR UEs not supporting the UPLF test mode: Send uplink power control commands in uplink scheduling information to the UE per UL CC until the Power Headroom Report (PHR)  from the UE for each UL CC is at the target value according to Table 6.2A.2.1.4.2-1.

Table 6.2A.2.1.4.2-1: Power target values per UL CC for test procedure using PHR

| BW ratio (Note 1) | Xmax [dB] (Note 2) | Target PHR | ΔPHR [dB] (Note 3) | BW combination examples |
| --- | --- | --- | --- | --- |
| 1/2 | 3.0 | POWER_HEADROOM_36 (3  PH  4) | 1 | 2CC equal BW |
| 1/3 | 4.8 | POWER_HEADROOM_38 (5  PH  6) | 1.2 | 2CC 50+100 MHz CC1 |
| 2/3 | 1.8 | POWER_HEADROOM_35 (2  PH  3) | 1.2 | 2CC 50+100 MHz CC2 |
| 1/5 | 7.0 | POWER_HEADROOM_40 (7  PH  8) | 1.0 | 2CC 50+200 MHz CC1 |
| 4/5 | 1.0 | POWER_HEADROOM_34 (1  PH  2) | 1.0 | 2CC 50+200 MHz CC2 |
| 1/9 | 9.5 | POWER_HEADROOM_43 (10  PH  11) | 1.5 | 2CC 50+400 MHz CC1 |
| 8/9 | 0.5 | POWER_HEADROOM_34 (1  PH  2) | 1.5 | 2CC 50+400 MHz CC2 |
| Note 1: The BW ratio is the ratio of BW of the CC over the total Aggregated UL BW Note 2: Xmax = 10log(BW ratio) Note 3: ΔPHR is the worst case UE output power decrease due to Xmax and 1 dB reporting granularity of PHR according to TS38.133 [25]. |  |  |  |  |

7c. For testing single CC MPR requirement: Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

8.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

9.	Measure UE EIRP in the Tx beam peak direction in the accumulative aggregated channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in 6.2A.2.1.5. EIRP test procedure is defined in Annex K.1.3. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

10.	Apply the test step based on the 5G NR UE Release:

10a. For Release 16 and forward 5G NR UEs supporting the UPLF test mode: SS deactivates the UE Power Limit Function (UPLF) by performing the DEACTIVATE POWER LIMIT REQUEST procedure as specified in TS 38.508-1 [10] clause 4.9.33.

10b. For Release 15 5G NR UEs and 5G NR UEs not supporting the UPLF test mode: No action

10c. For testing single CC MPR requirement: No action

11.	 SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

NOTE 2:	When switching to DFT-s-OFDM waveform, as specified in Table 6.2A.2.1.4.1-1 to Table 6.2A.2.1.4.1-7, send an NR RRCReconfiguration message according to TS 38.508-1 [10] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2A.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with the following exceptions for UE not supporting test mode.

Table 6.2A.2.1.4.3-1: PUSCH-PowerControl

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-120 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| tpc-Accumulation | disabled |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF SEQUENCE { | 1 entry |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  |  |  |
| alpha | alpha0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.2A.2.1.5	Test requirement

The EIRP derived in step 8 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2A.2.1.5-1 to Table 6.2A.2.1.5-17.

Table 6.2A.2.1.5-1: MPR requirements for Intra-band Contiguous UL CA (Power Class 1, MPRnarrow)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258, n261 | 40.0 | 14.4 | [7.0] | [18.6]-TT | 55 |
| 1 | n260 | 38.0 | 14.4 | [7.0] | [16.6]-TT | 55 |
| 2 | n257, n258, n261 | 40.0 | 14.4 | [7.0] | [18.6]-TT | 55 |
| 2 | n260 | 38.0 | 14.4 | [7.0] | [16.6]-TT | 55 |
| 3 | n257, n258, n261 | 40.0 | 10 | [5] | [25.0]-TT | 55 |
| 3 | n260 | 38.0 | 10 | [5] | [23.0]-TT | 55 |
| 4 | n257, n258, n261 | 40.0 | 10 | [5] | [25.0]-TT | 55 |
| 4 | n260 | 38.0 | 10 | [5] | [23.0]-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.2.1.5-5. |  |  |  |  |  |  |

Table 6.2A.2.1.5-2: MPR requirements for Intra-band Contiguous UL CA (Power Class 1, single CC MPR requirement)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Test requirements for a CA_nXG, CA_nXO Configuration (Cumulative aggregated BWchannel <= 200MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 40.0 | 5.5 | [5.0] | [29.5]-TT | 55 |
| 1 | n260 | 38.0 | 5.5 | [5.0] | [27.5]-TT | 55 |
| 2 | n257, n258, n261 | 40.0 | 3.0 | [2.0] | [35.0]-TT | 55 |
| 2 | n260 | 38.0 | 3.0 | [2.0] | [33.0]-TT | 55 |
| Test requirements for a CA_nXD Configuration (Cumulative aggregated BWchannel <= 400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 40.0 | 5.5 | [5.0] | [29.5]-TT | 55 |
| 1 | n260 | 38.0 | 5.5 | [5.0] | [27.5]-TT | 55 |
| 2 | n257, n258, n261 | 40.0 | 3.0 | [2.0] | [35.0]-TT | 55 |
| 2 | n260 | 38.0 | 3.0 | [2.0] | [33.0]-TT | 55 |
| 3 | n257, n258, n261 | 40.0 | 3.5 | [3.0] | [33.5]-TT | 55 |
| 3 | n260 | 38.0 | 3.5 | [3.0] | [31.5]-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.2.1.5-5. |  |  |  |  |  |  |

Table 6.2A.2.1.5-3: MPR requirements for Intra-band Contiguous UL CA (Power Class 1, MPRC_CA)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Test requirements for a CA_nXB, CA_nXC_UL_nXB Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 40.0 | 8.2 | [5.0] | [26.8]-TT | 55 |
| 1 | n260 | 38.0 | 8.2 | [5.0] | [24.8]-TT | 55 |
| 2 | n257, n258, n261 | 40.0 | 9.7 | [5.0] | [25.3]-TT | 55 |
| 2 | n260 | 38.0 | 9.7 | [5.0] | [23.3]-TT | 55 |
| 3 | n257, n258, n261 | 40.0 | 9.2 | [5.0] | [25.8]-TT | 55 |
| 3 | n260 | 38.0 | 9.2 | [5.0] | [23.8]-TT | 55 |
| 4 | n257, n258, n261 | 40.0 | 8.7 | [5.0] | [26.3]-TT | 55 |
| 4 | n260 | 38.0 | 8.7 | [5.0] | [24.3]-TT | 55 |
| 5 | n257, n258, n261 | 40.0 | 11.2 | [7.0] | [21.8]-TT | 55 |
| 5 | n260 | 38.0 | 11.2 | [7.0] | [19.8]-TT | 55 |
| Test requirements for a CA_nXD, CA_nXB Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 40.0 | 7.7 | [5.0] | [27.3]-TT | 55 |
| 1 | n260 | 38.0 | 7.7 | [5.0] | [25.3]-TT | 55 |
| 2 | n257, n258, n261 | 40.0 | 8.7 | [5.0] | [26.3]-TT | 55 |
| 2 | n260 | 38.0 | 8.7 | [5.0] | [24.3]-TT | 55 |
| 3 | n257, n258, n261 | 40.0 | 10.7 | [7.0] | [22.3]-TT | 55 |
| 3 | n260 | 38.0 | 10.7 | [7.0] | [20.3]-TT | 55 |
| Test requirements for a CA_nXG, CA_nXO, CA_nXD Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 40.0 | 5.5 | [5.0] | [29.5]-TT | 55 |
| 1 | n260 | 38.0 | 5.5 | [5.0] | [27.5]-TT | 55 |
| 2 | n257, n258, n261 | 40.0 | 6.5 | [5.0] | [28.5]-TT | 55 |
| 2 | n260 | 38.0 | 6.5 | [5.0] | [26.5]-TT | 55 |
| 3 | n257, n258, n261 | 40.0 | 9.0 | [5.0] | [26.0]-TT | 55 |
| 3 | n260 | 38.0 | 9.0 | [5.0] | [24.0]-TT | 55 |
| Test requirements for a CA_nX(D-G)_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO, CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(E-O)_UL_nXO, CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 40.0 | 8.2 | [5.0] | [26.8] –TT | 55 |
| 1 | n260 | 38.0 | 8.2 | [5.0] | [24.8] –TT | 55 |
| 2 | n257, n258, n261 | 40.0 | 9.7 | [5.0] | [25.3] –TT | 55 |
| 2 | n260 | 38.0 | 9.7 | [5.0] | [23.3] –TT | 55 |
| 3 | n257, n258, n261 | 40.0 | 9.2 | [5.0] | [25.8] –TT | 55 |
| 3 | n260 | 38.0 | 9.2 | [5.0] | [23.8] –TT | 55 |
| 4 | n257, n258, n261 | 40.0 | 8.7 | [5.0] | [26.3] –TT | 55 |
| 4 | n260 | 38.0 | 8.7 | [5.0] | [24.3] –TT | 55 |
| 5 | n257, n258, n261 | 40.0 | 11.2 | [7.0] | [21.8] –TT | 55 |
| 5 | n260 | 38.0 | 11.2 | [7.0] | [19.8] –TT | 55 |
| Test requirements for a CA_nX(D-G)_UL_nXD, CA_nX(D-O)_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXO, CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(O-E)_UL_nXO, CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration (400MHz <= Cumulative aggregated BWchannel <800MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 40.0 | 7.7 | [5.0] | [27.3]-TT | 55 |
| 1 | n260 | 38.0 | 7.7 | [5.0] | [25.3]-TT | 55 |
| 2 | n257, n258, n261 | 40.0 | 8.7 | [5.0] | [26.3]-TT | 55 |
| 2 | n260 | 38.0 | 8.7 | [5.0] | [24.3]-TT | 55 |
| 3 | n257, n258, n261 | 40.0 | 10.7 | [7.0] | [22.3]-TT | 55 |
| 3 | n260 | 38.0 | 10.7 | [7.0] | [20.3]-TT | 55 |
| Test requirements for a CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO Configuration (Cumulative aggregated BWchannel <400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 40.0 | 5.5 | [5.0] | [29.5]-TT | 55 |
| 1 | n260 | 38.0 | 5.5 | [5.0] | [27.5]-TT | 55 |
| 2 | n257, n258, n261 | 40.0 | 6.5 | [5.0] | [28.5]-TT | 55 |
| 2 | n260 | 38.0 | 6.5 | [5.0] | [26.5]-TT | 55 |
| 3 | n257, n258, n261 | 40.0 | 9.0 | [5.0] | [26.0]-TT | 55 |
| 3 | n260 | 38.0 | 9.0 | [5.0] | [24.0]-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.2.1.5-5. |  |  |  |  |  |  |

Table 6.2A.2.1.5-4: MPR requirements for Intra-band Contiguous UL CA (Power Class 1, Non-contiguous allocation)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Test requirements for a CA_nXB, CA_nXD, CA_XG, CA_nXO Configuration |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 40.0 | [14.4] | [7.0] | [18.6] –TT | 55 |
| 1 | n260 | 38.0 | [14.4] | [7.0] | [16.6] –TT | 55 |
| 2 | n257, n258, n261 | FFS | FFS | FFS | FFS | FFS |
| 2 | n260 | FFS | FFS | FFS | FFS | FFS |
| 3 | n257, n258, n261 | FFS | FFS | FFS | FFS | FFS |
| 3 | n260 | FFS | FFS | FFS | FFS | FFS |
| Test requirements for a CA_nX(D-G), CA_nX(D-O) Configuration |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 40.0 | [14.4] | [7.0] | [18.6] –TT | 55 |
| 1 | n260 | 38.0 | [14.4] | [7.0] | [16.6] –TT | 55 |
| 2 | n257, n258, n261 | FFS | FFS | FFS | FFS | FFS |
| 2 | n260 | FFS | FFS | FFS | FFS | FFS |
| 3 | n257, n258, n261 | FFS | FFS | FFS | FFS | FFS |
| 3 | n260 | FFS | FFS | FFS | FFS | FFS |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.2.1.5-5. |  |  |  |  |  |  |

Table 6.2A.2.1.5-5: Test Tolerance (MPR for CA for Power class 1)

FFS

Table 6.2A.2.1.5-6: MPR requirements for Intra-band Contiguous UL CA (Power Class 2, single CC MPR requirement)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Test requirements for a CA_nXG, CA_nXO Configuration (Cumulative aggregated BWchannel <= 200MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 29 | 0 | 0 | 29.0-TT | 43 |
| 2 | n257, n258, n261 | 29 | 2 | [1.5] | [25.5]-TT | 43 |
| Test requirements for a CA_nXD Configuration (Cumulative aggregated BWchannel <= 400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 29 | 0 | 0 | 29.0-TT | 43 |
| 2 | n257, n258, n261 | 29 | 3 | [2.0] | [24.0]-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.2.1.5-9. |  |  |  |  |  |  |

Table 6.2A.2.1.5-7: MPR requirements for Intra-band Contiguous UL CA (Power Class 2, MPRC_CA)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Test requirements for a CA_nXB, nXC_UL_nXB Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 29 | 8.2 | [5.0] | [15.8]-TT | 43 |
| 2 | n257, n258, n261 | 29 | 9.3 | [5.0] | [14.7]-TT | 43 |
| 3 | n257, n258, n261 | 29 | 8.0 | [5.0] | [16.0]-TT | 43 |
| 4 | n257, n258, n261 | 29 | 9.2 | [5.0] | [14.8]-TT | 43 |
| 5 | n257, n258, n261 | 29 | 11.2 | [7.0] | [10.8]-TT | 43 |
| Test requirements for a CA_nXD, CA_nXB Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 29 | 7.7 | [5.0] | [16.3]-TT | 43 |
| 2 | n257, n258, n261 | 29 | 7.5 | [5.0] | [16.5]-TT | 43 |
| 3 | n257, n258, n261 | 29 | 8.7 | [5.0] | [15.3]-TT | 43 |
| 4 | n257, n258, n261 | 29 | 10.7 | [7.0] | [11.3]-TT | 43 |
| Test requirements for a CA_nXG, CA_nXO, CA_nXD Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 29 | 5 | [4.0] | [20.0]-TT | 43 |
| 2 | n257, n258, n261 | 29 | 6.5 | [5.0] | [17.5]-TT | 43 |
| 3 | n257, n258, n261 | 29 | 9 | [5.0] | [15.0]-TT | 43 |
| Test requirements for a CA_nX(D-G) )_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO, CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(E-O)_UL_nXO, CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 29 | 8.2 | [5.0] | [15.8]-TT | 43 |
| 2 | n257, n258, n261 | 29 | 9.3 | [5.0] | [14.7]-TT | 43 |
| 3 | n257, n258, n261 | 29 | 8.0 | [5.0] | [16.0]-TT | 43 |
| 4 | n257, n258, n261 | 29 | 9.2 | [5.0] | [14.8]-TT | 43 |
| 5 | n257, n258, n261 | 29 | 11.2 | [7.0] | [10.8]-TT | 43 |
| Test requirements for a CA_nX(D-G)_UL_nXD, CA_nX(D-O)_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXO, CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(O-E)_UL_nXO, CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 29 | 7.7 | [5.0] | [16.3]-TT | 43 |
| 2 | n257, n258, n261 | 29 | 7.5 | [5.0] | [16.5]-TT | 43 |
| 3 | n257, n258, n261 | 29 | 8.7 | [5.0] | [15.3]-TT | 43 |
| 4 | n257, n258, n261 | 29 | 10.7 | [7.0] | [11.3]-TT | 43 |
| Test requirements for a CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 29 | 7.7 | [5.0] | [16.3]-TT | 43 |
| 2 | n257, n258, n261 | 29 | 7.5 | [5.0] | [16.5]-TT | 43 |
| 3 | n257, n258, n261 | 29 | 8.7 | [5.0] | [15.3]-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.2.1.5-9. |  |  |  |  |  |  |

Table 6.2A.2.1.5-8: MPR requirements for Intra-band Contiguous UL CA (Power Class 2, Non-contiguous allocation)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Test requirements for a CA_nXB, CA_nXD, CA_XG, CA_nXO Configuration |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 29 | 7 | [5.0] | [17.0] –TT | 43 |
| 2 | n257, n258, n261 | FFS | FFS | FFS | FFS | FFS |
| 3 | n257, n258, n261 | FFS | FFS | FFS | FFS | FFS |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.2.1.5-9. |  |  |  |  |  |  |

Table 6.2A.2.1.5-9: Test Tolerance (MPR for CA for Power class 2)

FFS

Table 6.2A.2.1.5-10: MPR requirements for Intra-band Contiguous UL CA (Power Class 3, single CC MPR requirement)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Test requirements for a CA_nXG, CA_nXO Configuration (Cumulative aggregated BWchannel <= 200MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 22.4 | 0 | 0 | 22.4-TT | 43 |
| 1 | n260 | 20.6 | 0 | 0 | 20.6-TT | 43 |
| 2 | n257, n258, n261 | 22.4 | 2 | [1.5] | [18.9]-TT | 43 |
| 2 | n260 | 20.6 | 2 | [1.5] | [17.1]-TT | 43 |
| Test requirements for a CA_nXD Configuration (Cumulative aggregated BWchannel <= 400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 22.4 | 0 | 0 | 22.4-TT | 43 |
| 1 | n260 | 20.6 | 0 | 0 | 20.6-TT | 43 |
| 2 | n257, n258, n261 | 22.4 | 3 | [2.0] | [17.4]-TT | 43 |
| 2 | n260 | 20.6 | 3 | [2.0] | [15.6]-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.2.1.5-13. |  |  |  |  |  |  |

Table 6.2A.2.1.5-11: MPR requirements for Intra-band Contiguous UL CA (Power Class 3, MPRC_CA)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) |  | Lower limit for test procedure with UPLF test mode (variant a) |  | Lower limit for test procedure with PHR (variant b) |  |  |  |  |  | Upper limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T(MPR) (dB) | Lower limit (dBm) | T(MPR+ ΔPHR) (dB) |  |  |  | Lower limit PHR(dBm) |  |  |  |
| Test requirements for a CA_nXB, nXC_UL_nXB Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 22.4 | 8.2 |  | [5.0] | [9.2]-TT | [5.0] |  |  |  | [9.2]-ΔPHR-TT |  | 43 |  |
| 1 | n260 | 20.6 | 8.2 |  | [5.0] | [7.4]-TT | [5.0] |  |  |  | [7.4]- ΔPHR-TT |  | 43 |  |
| 2 | n257, n258, n261 | 22.4 | 9.3 |  | [5.0] | [8.1]-TT | [5.0]+2 |  |  |  | [6.1]- ΔPHR-TT |  | 43 |  |
| 2 | n260 | 20.6 | 9.3 |  | [5.0] | [6.3]-TT | [5.0]+2 |  |  |  | [4.3]- ΔPHR-TT |  | 43 |  |
| 3 | n257, n258, n261 | 22.4 | 8.0 |  | [5.0] | [9.4]-TT | [5.0] |  |  |  | [9.4]- ΔPHR-TT |  | 43 |  |
| 3 | n260 | 20.6 | 8.0 |  | [5.0] | [7.6]-TT | [5.0] |  |  |  | [7.6]- ΔPHR-TT |  | 43 |  |
| 4 | n257, n258, n261 | 22.4 | 9.2 |  | [5.0] | [8.2]-TT | [5.0]+2 |  |  |  | [6.2]- ΔPHR-TT |  | 43 |  |
| 4 | n260 | 20.6 | 9.2 |  | [5.0] | [6.4]-TT | [5.0]+2 |  |  |  | [4.4]- ΔPHR-TT |  | 43 |  |
| 5 | n257, n258, n261 | 22.4 | 11.2 |  | [7.0] | [4.2]-TT | [7.0] |  |  |  | [4.2]- ΔPHR-TT |  | 43 |  |
| 5 | n260 | 20.6 | 11.2 |  | [7.0] | [2.4]-TT | [7.0] |  |  |  | [2.4]- ΔPHR-TT |  | 43 |  |
| Test requirements for a CA_nXD, CA_nXB Configuration (Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 22.4 | 7.7 |  | [5.0] | [9.7]-TT | [5.0] |  |  |  | [9.7]- ΔPHR-TT |  | 43 |  |
| 1 | n260 | 20.6 | 7.7 |  | [5.0] | [7.9]-TT | [5.0] |  |  |  | [7.9]- ΔPHR-TT |  | 43 |  |
| 2 | n257, n258, n261 | 22.4 | 7.5 |  | [5.0] | [9.9]-TT | [5.0] |  |  |  | [9.9]- ΔPHR-TT |  | 43 |  |
| 2 | n260 | 20.6 | 7.5 |  | [5.0] | [8.1]-TT | [5.0] |  |  |  | [8.1]- ΔPHR-TT |  | 43 |  |
| 3 | n257, n258, n261 | 22.4 | 8.7 |  | [5.0] | [8.7]-TT | [5.0] |  |  |  | [8.7]- ΔPHR-TT |  | 43 |  |
| 3 | n260 | 20.6 | 8.7 |  | [5.0] | [6.9]-TT | [5.0] |  |  |  | [6.9]- ΔPHR-TT |  | 43 |  |
| 4 | n257, n258, n261 | 22.4 | 10.7 |  | [7.0] | [4.7]-TT | [7.0] |  |  |  | [4.7]- ΔPHR-TT |  | 43 |  |
| 4 | n260 | 20.6 | 10.7 |  | [7.0] | [2.9]-TT | [7.0] |  |  |  | [2.9]- ΔPHR-TT |  | 43 |  |
| Test requirements for a CA_nXG, CA_nXO, CA_nXD Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 22.4 | 5 |  | [4.0] | [13.4]-TT | [4.0]+1 |  |  |  | [12.4]- ΔPHR-TT |  | 43 |  |
| 1 | n260 | 20.6 | 5 |  | [4.0] | [11.6]-TT | [4.0]+1 |  |  |  | [10.6]- ΔPHR-TT |  | 43 |  |
| 2 | n257, n258, n261 | 22.4 | 6.5 |  | [5.0] | [10.9]-TT | [5.0] |  |  |  | [10.9]- ΔPHR-TT |  | 43 |  |
| 2 | n260 | 20.6 | 6.5 |  | [5.0] | [9.1]-TT | [5.0] |  |  |  | [9.1]- ΔPHR-TT |  | 43 |  |
| 3 | n257, n258, n261 | 22.4 | 9 |  | [5.0] | [8.4]-TT | [5.0] |  |  |  | [8.4]- ΔPHR-TT |  | 43 |  |
| 3 | n260 | 20.6 | 9 |  | [5.0] | [6.6]-TT | [5.0] |  |  |  | [6.6]- ΔPHR-TT |  | 43 |  |
| Test requirements for a CA_nX(D-G) )_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO, CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(E-O)_UL_nXO, CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 22.4 | 8.2 |  | [5.0] | [9.2]-TT | [5.0] |  |  |  | [9.2]- ΔPHR-TT |  | 43 |  |
| 1 | n260 | 20.6 | 8.2 |  | [5.0] | [7.4]-TT | [5.0] |  |  |  | [7.4]- ΔPHR-TT |  | 43 |  |
| 2 | n257, n258, n261 | 22.4 | 9.3 |  | [5.0] | [8.1]-TT | [5.0]+2 |  |  |  | [6.1]- ΔPHR-TT |  | 43 |  |
| 2 | n260 | 20.6 | 9.3 |  | [5.0] | [6.3]-TT | [5.0]+2 |  |  |  | [4.3]- ΔPHR-TT |  | 43 |  |
| 3 | n257, n258, n261 | 22.4 | 8.0 |  | [5.0] | [9.4]-TT | [5.0] |  |  |  | [9.4]- ΔPHR-TT |  | 43 |  |
| 3 | n260 | 20.6 | 8.0 |  | [5.0] | [7.6]-TT | [5.0] |  |  |  | [7.6]- ΔPHR-TT |  | 43 |  |
| 4 | n257, n258, n261 | 22.4 | 9.2 |  | [5.0] | [8.2]-TT | [5.0]+2 |  |  |  | [6.2]- ΔPHR-TT |  | 43 |  |
| 4 | n260 | 20.6 | 9.2 |  | [5.0] | [6.4]-TT | [5.0]+2 |  |  |  | [4.4]- ΔPHR-TT |  | 43 |  |
| 5 | n257, n258, n261 | 22.4 | 11.2 |  | [7.0] | [4.2]-TT | [7.0] |  |  |  | [4.2]- ΔPHR-TT |  | 43 |  |
| 5 | n260 | 20.6 | 11.2 |  | [7.0] | [2.4]-TT | [7.0] |  |  |  | [2.4]- ΔPHR-TT |  | 43 |  |
| Test requirements for a CA_nX(D-G)_UL_nXD, CA_nX(D-O)_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXO, CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(O-E)_UL_nXO, CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration (Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 22.4 | 7.7 |  | [5.0] | [9.7]-TT | [5.0] |  |  |  | [9.7]- ΔPHR-TT |  | 43 |  |
| 1 | n260 | 20.6 | 7.7 |  | [5.0] | [7.9]-TT | [5.0] |  |  |  | [7.9]- ΔPHR-TT |  | 43 |  |
| 2 | n257, n258, n261 | 22.4 | 7.5 |  | [5.0] | [9.9]-TT | [5.0] |  |  |  | [9.9]- ΔPHR-TT |  | 43 |  |
| 2 | n260 | 20.6 | 7.5 |  | [5.0] | [8.1]-TT | [5.0] |  |  |  | [8.1]- ΔPHR-TT |  | 43 |  |
| 3 | n257, n258, n261 | 22.4 | 8.7 |  | [5.0] | [8.7]-TT | [5.0] |  |  |  | [8.7]- ΔPHR-TT |  | 43 |  |
| 3 | n260 | 20.6 | 8.7 |  | [5.0] | [6.9]-TT | [5.0] |  |  |  | [6.9]- ΔPHR-TT |  | 43 |  |
| 4 | n257, n258, n261 | 22.4 | 10.7 |  | [7.0] | [4.7]-TT | [7.0] |  |  |  | [4.7]- ΔPHR-TT |  | 43 |  |
| 4 | n260 | 20.6 | 10.7 |  | [7.0] | [2.9]-TT | [7.0] |  |  |  | [2.9]- ΔPHR-TT |  | 43 |  |
| Test requirements for a CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 22.4 | 5 |  | [4.0] | [13.4]-TT | [4.0]+1 |  |  |  | [12.4]- ΔPHR-TT |  | 43 |  |
| 1 | n260 | 20.6 | 5 |  | [4.0] | [11.6]-TT | [4.0]+1 |  |  |  | [10.6]- ΔPHR-TT |  | 43 |  |
| 2 | n257, n258, n261 | 22.4 | 6.5 |  | [5.0] | [10.9]-TT | [5.0] |  |  |  | [10.9]- ΔPHR-TT |  | 43 |  |
| 2 | n260 | 20.6 | 6.5 |  | [5.0] | [9.1]-TT | [5.0] |  |  |  | [9.1]- ΔPHR-TT |  | 43 |  |
| 3 | n257, n258, n261 | 22.4 | 9 |  | [5.0] | [8.4]-TT | [5.0] |  |  |  | [8.4]- ΔPHR-TT |  | 43 |  |
| 3 | n260 | 20.6 | 9 |  | [5.0] | [6.6]-TT | [5.0] |  |  |  | [6.6]- ΔPHR-TT |  | 43 |  |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.2.1.5-13. NOTE 2: ΔPHR is defined in Table 6.2A.2.1.4.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-12: MPR requirements for Intra-band Contiguous UL CA (Power Class 3, Non-contiguous allocation)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | Lower limit for test procedure with UPLF test mode (variant a) |  | Lower limit for test procedure with PHR (variant b) |  | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T(MPR) (dB) | Lower limit (dBm) | T(MPR+ ΔPHR) (dB) | Lower limit PHR(dBm) |  |
| Test requirements for a CA_nXB, CA_nXD, CA_XG, CA_nXO Configuration |  |  |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 22.4 | 7 | [5.0] | [10.4]-TT | [5.0] | [10.4]- ΔPHR -TT | 43 |
| 1 | n260 | 20.6 | 7 | [5.0] | [8.6]-TT | [5.0] | [8.6]- ΔPHR -TT | 43 |
| 2 | n257, n258, n261 | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| 2 | n260 | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| 3 | n257, n258, n261 | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| 3 | n260 | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.2.1.5-13. NOTE 2: ΔPHR is defined in Table 6.2A.2.1.4.2-1 |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-13: Test Tolerance (MPR for CA for Power class 3) (Aggregated UL BW ≤ 400MHz)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 3.11 dB | 3.11 dB |

Table 6.2A.2.1.5-14: MPR requirements for Intra-band Contiguous UL CA (Power Class 4, single CC MPR requirement)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Test requirements for a CA_nXG, CA_nXO Configuration (Cumulative aggregated BWchannel <= 200MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 34 | 0 | 0 | 34.0-TT | 43 |
| 1 | n260 | 31 | 0 | 0 | 31.0-TT | 43 |
| 2 | n257, n258, n261 | 34 | 2 | [1.5] | [30.5]-TT | 43 |
| 2 | n260 | 31 | 2 | [1.5] | [27.5]-TT | 43 |
| Test requirements for a CA_nXD Configuration (Cumulative aggregated BWchannel <= 400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 34 | 0 | 0 | 34.0-TT | 43 |
| 1 | n260 | 31 | 0 | 0 | 31.0-TT | 43 |
| 2 | n257, n258, n261 | 34 | 3 | [2.0] | [29.0]-TT | 43 |
| 2 | n260 | 31 | 3 | [2.0] | [26.0]-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.2.1.5-17. |  |  |  |  |  |  |

Table 6.2A.2.1.5-15: MPR requirements for Intra-band Contiguous UL CA (Power Class 4, MPRC_CA)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Test requirements for a CA_nXB, nXC_UL_nXB Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 34 | 8.2 | [5.0] | [20.8]-TT | 43 |
| 1 | n260 | 31 | 8.2 | [5.0] | [17.8]-TT | 43 |
| 2 | n257, n258, n261 | 34 | 9.3 | [5.0] | [19.7]-TT | 43 |
| 2 | n260 | 31 | 9.3 | [5.0] | [16.7]-TT | 43 |
| 3 | n257, n258, n261 | 34 | 8.0 | [5.0] | [21.0]-TT | 43 |
| 3 | n260 | 31 | 8.0 | [5.0] | [18.0]-TT | 43 |
| 4 | n257, n258, n261 | 34 | 9.2 | [5.0] | [19.8]-TT | 43 |
| 4 | n260 | 31 | 9.2 | [5.0] | [16.8]-TT | 43 |
| 5 | n257, n258, n261 | 34 | 11.2 | [7.0] | [15.8]-TT | 43 |
| 5 | n260 | 31 | 11.2 | [7.0] | [12.8]-TT | 43 |
| Test requirements for a CA_nXD, CA_nXB Configuration (Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 34 | 7.7 | [5.0] | [21.3]-TT | 43 |
| 1 | n260 | 31 | 7.7 | [5.0] | [18.3]-TT | 43 |
| 2 | n257, n258, n261 | 34 | 7.5 | [5.0] | [21.5]-TT | 43 |
| 2 | n260 | 31 | 7.5 | [5.0] | [18.5]-TT | 43 |
| 3 | n257, n258, n261 | 34 | 8.7 | [5.0] | [20.3]-TT | 43 |
| 3 | n260 | 31 | 8.7 | [5.0] | [17.3]-TT | 43 |
| 4 | n257, n258, n261 | 34 | 10.7 | [7.0] | [16.3]-TT | 43 |
| 4 | n260 | 31 | 10.7 | [7.0] | [13.3]-TT | 43 |
| Test requirements for a CA_nXG, CA_nXO, CA_nXD Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 34 | 5 | [4.0] | [25.0]-TT | 43 |
| 1 | n260 | 31 | 5 | [4.0] | [22.0]-TT | 43 |
| 2 | n257, n258, n261 | 34 | 6.5 | [5.0] | [22.5]-TT | 43 |
| 2 | n260 | 31 | 6.5 | [5.0] | [19.5]-TT | 43 |
| 3 | n257, n258, n261 | 34 | 9 | [5.0] | [20.0]-TT | 43 |
| 3 | n260 | 31 | 9 | [5.0] | [17.0]-TT | 43 |
| Test requirements for a CA_nX(D-G) )_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO, CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(E-O)_UL_nXO, CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 34 | 8.2 | [5.0] | [20.8]-TT | 43 |
| 1 | n260 | 31 | 8.2 | [5.0] | [17.8]-TT | 43 |
| 2 | n257, n258, n261 | 34 | 9.3 | [5.0] | [19.7]-TT | 43 |
| 2 | n260 | 31 | 9.3 | [5.0] | [16.7]-TT | 43 |
| 3 | n257, n258, n261 | 34 | 8.0 | [5.0] | [21.0]-TT | 43 |
| 3 | n260 | 31 | 8.0 | [5.0] | [18.0]-TT | 43 |
| 4 | n257, n258, n261 | 34 | 9.2 | [5.0] | [19.8]-TT | 43 |
| 4 | n260 | 31 | 9.2 | [5.0] | [16.8]-TT | 43 |
| 5 | n257, n258, n261 | 34 | 11.2 | [7.0] | [15.8]-TT | 43 |
| 5 | n260 | 31 | 11.2 | [7.0] | [12.8]-TT | 43 |
| Test requirements for a CA_nX(D-G)_UL_nXD, CA_nX(D-O)_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXO, CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(O-E)_UL_nXO, CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration (Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 34 | 7.7 | [5.0] | [21.3]-TT | 43 |
| 1 | n260 | 31 | 7.7 | [5.0] | [18.3]-TT | 43 |
| 2 | n257, n258, n261 | 34 | 7.5 | [5.0] | [21.5]-TT | 43 |
| 2 | n260 | 31 | 7.5 | [5.0] | [18.5]-TT | 43 |
| 3 | n257, n258, n261 | 34 | 8.7 | [5.0] | [20.3]-TT | 43 |
| 3 | n260 | 31 | 8.7 | [5.0] | [17.3]-TT | 43 |
| 4 | n257, n258, n261 | 34 | 10.7 | [7.0] | [16.3]-TT | 43 |
| 4 | n260 | 31 | 10.7 | [7.0] | [13.3]-TT | 43 |
| Test requirements for a CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 34 | 5 | [4.0] | [25.0]-TT | 43 |
| 1 | n260 | 31 | 5 | [4.0] | [22.0]-TT | 43 |
| 2 | n257, n258, n261 | 34 | 6.5 | [5.0] | [22.5]-TT | 43 |
| 2 | n260 | 31 | 6.5 | [5.0] | [19.5]-TT | 43 |
| 3 | n257, n258, n261 | 34 | 9 | [5.0] | [20.0]-TT | 43 |
| 3 | n260 | 31 | 9 | [5.0] | [17.0]-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.2.1.5-17. |  |  |  |  |  |  |

Table 6.2A.2.1.5-16: MPR requirements for Intra-band Contiguous UL CA (Power Class 4, Non-contiguous allocation)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Test requirements for a CA_nXB, CA_nXD, CA_XG, CA_nXO Configuration |  |  |  |  |  |  |
| 1 | n257, n258, n261 | 34 | 7 | [5.0] | [22.0]-TT | 43 |
| 1 | n260 | 31 | 7 | [5.0] | [19.0]-TT | 43 |
| 2 | n257, n258, n261 | FFS | FFS | FFS | FFS | FFS |
| 2 | n260 | FFS | FFS | FFS | FFS | FFS |
| 3 | n257, n258, n261 | FFS | FFS | FFS | FFS | FFS |
| 3 | n260 | FFS | FFS | FFS | FFS | FFS |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.2.1.5-17. |  |  |  |  |  |  |

Table 6.2A.2.1.5-17: Test Tolerance (MPR for CA for Power class 4)

FFS

#### 6.2A.2.2	UE maximum output power reduction for CA (3UL CA)