3GPP TSG-RAN5 Meeting #96-e	R5-225223r1
Electronic Meeting, 15th August – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0798 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.2.2_1 for FR2 MPR enhancements in 38.521-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - MPR Core requirements have been updated for Rel-16 and onwards PC3 devices supporting modifiedMPRbehaviour bit 0 capability. New MPR test should be added. - Rel-15 MPR core requirements will be updated according to R4-2207883. - Rel-16 MPR core requirements will be updated according to R4-2207884. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test 6.2.2: - Minimum conformance requirements have been updated to align with core requirements in TS 38.101-2 v15.17.0, including the update in R4-2207883. - Removal of editor’s note related to modifiedMPRbehaviour - Applicability udated - New test 6.2.2_1 defined using 6.2.2 as baseline but limiting it to the testing of PC3 and covering MPR enhancements. - Applicability updated - Minimum conformance requirements aligned to TS38.101-2 v16.11.0 - Initial conditions table (Table 6.2.2.4.1-7 in 6.2.2) has been extended to test MPR narrow = 2dB in case of PC3 device supporting modifiedMPRbehaviour bit 0 capability. - Initial conditions table added in R5-222437r1 was also used as part of the baseline. - PC3 test requirements defined accordingly. - Editor’s note update to indicate that modifiedMPRbehaviour is now defined but its applicability to PC2, PC4 and PC5 is still FFS. - Measurement uncertainties and test tolerances defined same as test 6.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-16 MPR enhancements will not be properly tested in Rel-16 and onwards PC3 devices supporting modifiedMPRbehaviour bit 0 capability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.2.2_1(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on the following RAN4 CRs:R4-2207883, R4-2207884. This CR depends on the following RAN5 Tdocs: R5-222439, R5-222440, R5-222438 andR5-2224440. This is a revision of R5-222472 keeping minimum conformance requirements mapping with Rel-15 and Rel-16 core requirements and defining a new test case for new Rel-16 requirements as requested in RAN5#95-e. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: CR title corrected to remove test specification redundant information. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

### 6.2.2	UE maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for PC1, PC2 and PC4.

- Measurement grid for PC2/4 in Annex M.4 is FFS.

#### 6.2.2.0	General

The requirements in section 6.2.2 only apply when both UL and DL of a UE are configured for single CC operation, and they are of the same bandwidth. A UE may reduce its maximum output power due to modulation orders, transmit bandwidth configurations, waveform types and narrow allocations. This Maximum Power Reduction (MPR) is defined in subclauses below. The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation. When the maximum output power of a UE is modified by MPR, the power limits specified in subclause 6.2.4 apply.

For a UE that is configured for single CC operation with different channel bandwidths in UL and DL, the requirements in section 6.2A.2 apply.

For all power classes, the waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

6.2.2.1	Test purpose

The number of RB identified in 6.2.2.3 is based on meeting the requirements for the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2.2.2	Test applicability

The requirements of this test apply to all types of Rel-15 PC3 devices which do not support modifiedMPRbehaviour bit 0 capability (according to Annex P.1) and to all types of Rel-15 and forward PC1, PC2 and PC4 devices.

6.2.2.3	Minimum conformance requirements

##### 6.2.2.3.1	UE maximum output power reduction for power class 1

For power class 1, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB ≤ 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Tables 6.2.2.3.1-1 and 6.2.2.3.1-2.

Table 6.2.2.3.1-1: MPRWT for power class 1, BWchannel ≤ 200 MHz

| Modulation |  | MPRWT (dB), BWchannel ≤ 200 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.0 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.0 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 4.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 7.0 | ≤ 5.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |

Table 6.2.2.3.1-2: MPRWT for power class 1, BWchannel = 400 MHz

| Modulation |  | MPRWT (dB), BWchannel = 400 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.0 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 7.0 | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 | ≤ 9.0 |

Where the following parameters are defined to specify valid RB allocation ranges for the RB allocations regions in Tables 6.2.2.3.1-1 and 6.2.2.3.1-2:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBend = RBStart + LCRB - 1

RBStart,Low = Max(1, Floor(LCRB/2))

RBStart,High = NRB – RBStart,Low – LCRB

An RB allocation is an Outer RB allocation if

RBStart < RBStart,Low OR RBStart > RBStart,High OR LCRB > Ceil(NRB/2)

An RB allocation belonging to Table 6.2.2.3.1-1 is a Region 1 inner RB allocation if

RBstart ≥ Ceil(1/3 NRB) AND RBend < Ceil(2/3 NRB)

An RB allocation belonging to Table 6.2.2.3.1-2 is a Region 1 inner RB allocation if

RBstart ≥ Ceil(1/4 NRB) AND RBend < Ceil(3/4 NRB) AND LCRB ≤ Ceil(1/4 NRB)

An RB allocation is a Region 2 inner allocation if it is NOT an Outer allocation AND NOT a Region 1 inner allocation.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.4 apply.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.2.1.

##### 6.2.2.3.2	UE maximum output power reduction for power class 2

For power class 2, MPR specified in subclause 6.2.2.3.3 applies.

Table 6.2.2.3.2-1: Void

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.2.2.

##### 6.2.2.3.3	UE maximum output power reduction for power class 3

For power class 3, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

MPRnarrow = 2.5 dB, BWalloc,RB ≤ 1.44 MHz, and 0 ≤ RBstart < Ceil(1/3 NRB) or Ceil((2/3NRB) -LCRB) ≤ RBstart ≤ NRB-LCRB, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2.3.3-1 and Table 6.2.2.3.3-2.

Table 6.2.2.3.3-1: MPRWT for power class 3, BWchannel ≤ 200 MHz

| Modulation |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 2.0 |
|  | QPSK | 0.0 | ≤ 2.0 |
|  | 16 QAM | ≤ 3.0 | ≤ 3.5 |
|  | 64 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 5.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 |

Table 6.2.2.3.3-2: MPRWT for power class 3, BWchannel = 400 MHz

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 3.0 |
|  | QPSK | 0.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 |

Where the following parameters are defined to specify valid RB allocation ranges for RB allocations in Tables 6.2.2.3.3-1 and 6.2.2.3.3-2:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBend = RBStart + LCRB - 1

An RB allocation belonging to Table 6.2.2.3.3-1 is a Region 1 inner RB allocation if

RBstart ≥ Ceil(1/3 NRB) AND RBend < Ceil(2/3 NRB)

An RB allocation belonging to Table 6.2.2.3.3-2 is a Region 1 inner RB allocation if

RBstart ≥ Ceil(1/4 NRB) AND RBend < Ceil(3/4 NRB) AND LCRB ≤ Ceil(1/4 NRB)

An RB allocation is an Edge allocation if it is NOT a Region 1 inner allocation.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.2.3.

##### 6.2.2.3.4	UE maximum output power reduction for power class 4

For power class 4, MPR specified in sub-clause 6.2.2.3.3 applies.

Table 6.2.2.3.4-1: Void

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.2.4.

6.2.2.4	Test description

6.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.2.2.4.1-1 to Table 6.2.2.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.2.4.1-1: Test Configuration Table (Power Class 1, MPRnarrow)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 2 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 3 | Low |  |  |  | CP-OFDM 64 QAM | 2@0 |
| 4 | High |  |  |  | CP-OFDM 64 QAM | 2@NRB-2 |
| 5 | Low |  |  |  | CP-OFDM 64 QAM | 7@0 |
| 6 | High |  |  |  | CP-OFDM 64 QAM | 7@NRB-7 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-2. |  |  |  |  |  |  |

Table 6.2.2.4.1-2: Test Configuration Table (Power Class 1, MPRWT, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@0 |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@NRB-8 |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Mid |  |  |  | DFT-s-OFDM QPSK | Inner_Full_Region2 |
| 5 | Low |  |  |  | DFT-s-OFDM QPSK | 8@0 |
| 6 | High |  |  |  | DFT-s-OFDM QPSK | 8@NRB-8 |
| 7 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 8 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full_Region2 |
| 9 | Low |  |  |  | DFT-s-OFDM 16 QAM | 8@0 |
| 10 | High |  |  |  | DFT-s-OFDM 16 QAM | 8@NRB-8 |
| 11 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 12 | Low |  |  |  | DFT-s-OFDM 64 QAM | 8@0 |
| 13 | High |  |  |  | DFT-s-OFDM 64 QAM | 8@NRB-8 |
| 14 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 15 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Inner_Full_Region2 |
| 16 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full_Region2 |
| 17 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 18 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 19 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 20 | Low |  |  |  | CP-OFDM 16 QAM | 8@0 |
| 21 | High |  |  |  | CP-OFDM 16 QAM | 8@NRB-8 |
| 22 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 23 | Mid |  |  |  | CP-OFDM 16 QAM | Inner_Full_Region2 |
| 24 | Low |  |  |  | CP-OFDM 64 QAM | 8@0 |
| 25 | High |  |  |  | CP-OFDM 64 QAM | 8@NRB-8 |
| 26 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in clause 6.1-2. |  |  |  |  |  |  |

Table 6.2.2.4.1-3: Test Configuration Table (Power Class 1, MPRWT, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@0 |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@NRB-8 |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Full_Region2 |
| 5 | Mid |  |  |  | DFT-s-OFDM QPSK | Inner_Full_Region2 |
| 6 | Low |  |  |  | DFT-s-OFDM QPSK | 8@0 |
| 7 | High |  |  |  | DFT-s-OFDM QPSK | 8@NRB-8 |
| 8 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full_Region2 |
| 10 | Low |  |  |  | DFT-s-OFDM 16 QAM | 8@0 |
| 11 | High |  |  |  | DFT-s-OFDM 16 QAM | 8@NRB-8 |
| 12 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 13 | Low |  |  |  | DFT-s-OFDM 64 QAM | 8@0 |
| 14 | High |  |  |  | DFT-s-OFDM 64 QAM | 8@NRB-8 |
| 15 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 16 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full_Region2 |
| 17 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 18 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 19 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 20 | Low |  |  |  | CP-OFDM 16 QAM | 8@0 |
| 21 | High |  |  |  | CP-OFDM 16 QAM | 8@NRB-8 |
| 22 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 23 | Mid |  |  |  | CP-OFDM 16 QAM | Inner_Full_Region2 |
| 24 | Low |  |  |  | CP-OFDM 64 QAM | 8@0 |
| 25 | High |  |  |  | CP-OFDM 64 QAM | 8@NRB-8 |
| 26 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in clause 6.1-2. |  |  |  |  |  |  |

Table 6.2.2.4.1-4: Void

Table 6.2.2.4.1-5: Void

Table 6.2.2.4.1-6: Void

Table 6.2.2.4.1-7: Test Configuration Table (Power Class 2, 3 and 4, MPRnarrow, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 4 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2.2.4.1-8: Test Configuration Table (Power Class 2, 3 and 4, MPRWT, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 2 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 3 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full |
| 4 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 7 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Inner_Full |
| 8 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 9 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 10 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 11 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full |
| 12 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 13 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 14 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 15 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 16 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 17 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 18 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 19 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 20 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2.2.4.1-8a: Test Configuration Table (Power Class 2, 3, 4, MPRnarrow, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Right |
| 3 | Low |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Left |
| 4 | High |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Right |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2.2.4.1-9: Test Configuration Table (Power Class 2, 3 and 4, MPRWT, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 12 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 13 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 14 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 15 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 16 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 17 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 18 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 19 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 21 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.2.2.4.1-1 to Table 6.2.2.4.1-9.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

6.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.2.4.1-1 to Table 6.2.2.4.1-9. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in 6.2.2.5. EIRP test procedure is defined in Annex K.1.3. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

6.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

NOTE 2:	When switching to DFT-s-OFDM waveform, as specified in Table 6.2.2.4.1-1 to Table 6.2.2.4.1-9, send an NR RRCReconfiguration message according to TS 38.508-1 [10] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.2.2.5	Test requirement

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in following tables.

Table 6.2.2.5-1: UE Power Class test requirements for Power Class 1 (for Bands n257, n258, n261)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.4.1-1 | 1 | 40 | 14.4 | 7 | 18.6-TT | 55 |
|  | 2 | 40 | 14.4 | 7 | 18.6-TT | 55 |
|  | 3 | 40 | 10 | 5 | 25-TT | 55 |
|  | 4 | 40 | 10 | 5 | 25-TT | 55 |
|  | 5 | 40 | 10 | 5 | 25-TT | 55 |
|  | 6 | 40 | 10 | 5 | 25-TT | 55 |
| Table 6.2.2.4.1-2 | 1 | 40 | 5.5 | 5 | 29.5-TT | 55 |
|  | 2 | 40 | 5.5 | 5 | 29.5-TT | 55 |
|  | 3 | 40 | 5.5 | 5 | 29.5-TT | 55 |
|  | 4 | 40 | 3 | 2 | 35-TT | 55 |
|  | 5 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 6 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 7 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 8 | 40 | 4 | 3 | 33-TT | 55 |
|  | 9 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 10 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 11 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 12 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 13 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 14 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 15 | 40 | 5 | 4 | 31-TT | 55 |
|  | 16 | 40 | 4.5 | 4 | 31.5-TT | 55 |
|  | 17 | 40 | 7 | 5 | 28-TT | 55 |
|  | 18 | 40 | 7 | 5 | 28-TT | 55 |
|  | 19 | 40 | 7 | 5 | 28-TT | 55 |
|  | 20 | 40 | 7 | 5 | 28-TT | 55 |
|  | 21 | 40 | 7 | 5 | 28-TT | 55 |
|  | 22 | 40 | 7 | 5 | 28-TT | 55 |
|  | 23 | 40 | 5.5 | 5 | 29.5-TT | 55 |
|  | 24 | 40 | 7.5 | 5 | 27.5-TT | 55 |
|  | 256 | 40 | 7.5 | 5 | 27.5-TT | 55 |
|  | 26 | 40 | 7.5 | 5 | 27.5-TT | 55 |
| Table 6.2.2.4.1-3 | 1 | 40 | 5.5 | 5 | 29.5-TT | 55 |
|  | 2 | 40 | 5.5 | 5 | 29.5-TT | 55 |
|  | 3 | 40 | 5.5 | 5 | 29.5-TT | 55 |
|  | 4 | 40 | 3 | 2 | 35-TT | 55 |
|  | 5 | 40 | 3.5 | 3 | 33.5-TT | 55 |
|  | 6 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 7 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 8 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 9 | 40 | 4.5 | 4 | 31.5-TT | 55 |
|  | 10 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 11 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 12 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 13 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 14 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 15 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 16 | 40 | 5 | 4 | 31-TT | 55 |
|  | 17 | 40 | 7 | 5 | 28-TT | 55 |
|  | 18 | 40 | 7 | 5 | 28-TT | 55 |
|  | 19 | 40 | 7 | 5 | 28-TT | 55 |
|  | 20 | 40 | 7 | 5 | 28-TT | 55 |
|  | 21 | 40 | 7 | 5 | 28-TT | 55 |
|  | 22 | 40 | 7 | 5 | 28-TT | 55 |
|  | 23 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 24 | 40 | 9 | 5 | 26-TT | 55 |
|  | 25 | 40 | 9 | 5 | 26-TT | 55 |
|  | 26 | 40 | 9 | 5 | 26-TT | 55 |

Table 6.2.2.5-1a: UE Power Class test requirements for Power Class 1 (for Bands n260)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.4.1-1 | 1 | 38 | 14.4 | 7 | 16.6-TT | 55 |
|  | 2 | 38 | 14.4 | 7 | 16.6-TT | 55 |
|  | 3 | 38 | 10 | 5 | 23-TT | 55 |
|  | 4 | 38 | 10 | 5 | 23-TT | 55 |
|  | 5 | 38 | 10 | 5 | 23-TT | 55 |
|  | 6 | 38 | 10 | 5 | 23-TT | 55 |
| Table 6.2.2.4.1-2 | 1 | 38 | 5.5 | 5 | 27.5-TT | 55 |
|  | 2 | 38 | 5.5 | 5 | 27.5-TT | 55 |
|  | 3 | 38 | 5.5 | 5 | 27.5-TT | 55 |
|  | 4 | 38 | 3 | 2 | 33-TT | 55 |
|  | 5 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 6 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 7 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 8 | 38 | 4 | 3 | 31-TT | 55 |
|  | 9 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 10 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 11 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 12 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 13 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 14 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 15 | 38 | 5 | 4 | 29-TT | 55 |
|  | 16 | 38 | 4.5 | 4 | 29.5-TT | 55 |
|  | 17 | 38 | 7 | 5 | 26-TT | 55 |
|  | 18 | 38 | 7 | 5 | 26-TT | 55 |
|  | 19 | 38 | 7 | 5 | 26-TT | 55 |
|  | 20 | 38 | 7 | 5 | 26-TT | 55 |
|  | 21 | 38 | 7 | 5 | 26-TT | 55 |
|  | 22 | 38 | 7 | 5 | 26-TT | 55 |
|  | 23 | 38 | 5.5 | 5 | 27.5-TT | 55 |
|  | 24 | 38 | 7.5 | 5 | 25.5-TT | 55 |
|  | 25 | 38 | 7.5 | 5 | 25.5-TT | 55 |
|  | 26 | 38 | 7.5 | 5 | 25.5-TT | 55 |
| Table 6.2.2.4.1-3 | 1 | 38 | 5.5 | 5 | 27.5-TT | 55 |
|  | 2 | 38 | 5.5 | 5 | 27.5-TT | 55 |
|  | 3 | 38 | 5.5 | 5 | 27.5-TT | 55 |
|  | 4 | 38 | 3 | 2 | 33-TT | 55 |
|  | 5 | 38 | 3.5 | 3 | 31.5-TT | 55 |
|  | 6 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 7 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 8 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 9 | 38 | 4.5 | 4 | 29.5-TT | 55 |
|  | 10 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 11 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 12 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 13 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 14 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 15 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 16 | 38 | 5 | 4 | 29-TT | 55 |
|  | 17 | 38 | 7 | 5 | 26-TT | 55 |
|  | 18 | 38 | 7 | 5 | 26-TT | 55 |
|  | 19 | 38 | 7 | 5 | 26-TT | 55 |
|  | 20 | 38 | 7 | 5 | 26-TT | 55 |
|  | 21 | 38 | 7 | 5 | 26-TT | 55 |
|  | 22 | 38 | 7 | 5 | 26-TT | 55 |
|  | 23 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 24 | 38 | 9 | 5 | 24-TT | 55 |
|  | 25 | 38 | 9 | 5 | 24-TT | 55 |
|  | 26 | 38 | 9 | 5 | 24-TT | 55 |

Table 6.2.2.5-2: UE Power Class test requirements for Power Class 2

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.4.1-7 | 1 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 2 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 3 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 4 | 29 | 2.5 | 2 | 24.5-TT | 43 |
| Table 6.2.2.4.1-8 | 1 | 29 | 2 | 1.5 | 25.5-TT | 43 |
|  | 2 | 29 | 2 | 1.5 | 25.5-TT | 43 |
|  | 3 | 29 | 3 | 2 | 24-TT | 43 |
|  | 4 | 29 | 3.5 | 3 | 22.5-TT | 43 |
|  | 5 | 29 | 3.5 | 3 | 22.5-TT | 43 |
|  | 6 | 29 | 3.5 | 3 | 22.5-TT | 43 |
|  | 7 | 29 | 5 | 4 | 20-TT | 43 |
|  | 8 | 29 | 5.5 | 5 | 18.5-TT | 43 |
|  | 9 | 29 | 5.5 | 5 | 18.5-TT | 43 |
|  | 10 | 29 | 5.5 | 5 | 18.5-TT | 43 |
|  | 11 | 29 | 3.5 | 3 | 22.5-TT | 43 |
|  | 12 | 29 | 4 | 3 | 22-TT | 43 |
|  | 13 | 29 | 4 | 3 | 22-TT | 43 |
|  | 14 | 29 | 4 | 3 | 22-TT | 43 |
|  | 15 | 29 | 5 | 4 | 20-TT | 43 |
|  | 16 | 29 | 5 | 4 | 20-TT | 43 |
|  | 17 | 29 | 5 | 4 | 20-TT | 43 |
|  | 18 | 29 | 7.5 | 5 | 16.5-TT | 43 |
|  | 19 | 29 | 7.5 | 5 | 16.5-TT | 43 |
|  | 20 | 29 | 7.5 | 5 | 16.5-TT | 43 |
| Table 6.2.2.4.1-8a | 1 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 2 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 3 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 4 | 29 | 2.5 | 2 | 24.5-TT | 43 |
| Table 6.2.2.4.1-9 | 1 | 29 | 3 | 2 | 24-TT | 43 |
|  | 2 | 29 | 3 | 2 | 24-TT | 43 |
|  | 3 | 29 | 3 | 2 | 24-TT | 43 |
|  | 4 | 29 | 3 | 2 | 24-TT | 43 |
|  | 5 | 29 | 3 | 2 | 24-TT | 43 |
|  | 6 | 29 | 3 | 2 | 24-TT | 43 |
|  | 7 | 29 | 4.5 | 4 | 20.5-TT | 43 |
|  | 8 | 29 | 4.5 | 4 | 20.5-TT | 43 |
|  | 9 | 29 | 4.5 | 4 | 20.5-TT | 43 |
|  | 10 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 11 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 12 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 13 | 29 | 5 | 4 | 20-TT | 43 |
|  | 14 | 29 | 5 | 4 | 20-TT | 43 |
|  | 15 | 29 | 5 | 4 | 20-TT | 43 |
|  | 16 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 17 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 18 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 19 | 29 | 9 | 5 | 15-TT | 43 |
|  | 20 | 29 | 9 | 5 | 15-TT | 43 |
|  | 21 | 29 | 9 | 5 | 15-TT | 43 |

Table 6.2.2.5-3: UE Power Class test requirements for Power Class 3 (n257, 258, 261)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.4.1-7 | 1 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
| Table 6.2.2.4.1-8 | 1 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 5 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 6 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 7 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 8 | 22.4 | 5.5 | 5 | 11.9-TT-ΔMBP,n | 43 |
|  | 9 | 22.4 | 5.5 | 5 | 11.9-TT-ΔMBP,n | 43 |
|  | 10 | 22.4 | 5.5 | 5 | 11.9-TT-ΔMBP,n | 43 |
|  | 11 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 12 | 22.4 | 4 | 3 | 15.4-TT-ΔMBP,n | 43 |
|  | 13 | 22.4 | 4 | 3 | 15.4-TT-ΔMBP,n | 43 |
|  | 14 | 22.4 | 4 | 3 | 15.4-TT-ΔMBP,n | 43 |
|  | 15 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 16 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 17 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 18 | 22.4 | 7.5 | 5 | 9.9-TT-ΔMBP,n | 43 |
|  | 19 | 22.4 | 7.5 | 5 | 9.9-TT-ΔMBP,n | 43 |
|  | 20 | 22.4 | 7.5 | 5 | 9.9-TT-ΔMBP,n | 43 |
| Table 6.2.2.4.1-8a | 1 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
| Table 6.2.2.4.1-9 | 1 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 5 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 6 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 7 | 22.4 | 4.5 | 4 | 13.9-TT-ΔMBP,n | 43 |
|  | 8 | 22.4 | 4.5 | 4 | 13.9-TT-ΔMBP,n | 43 |
|  | 9 | 22.4 | 4.5 | 4 | 13.9-TT-ΔMBP,n | 43 |
|  | 10 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 11 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 12 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 13 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 14 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 15 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 16 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 17 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 18 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 19 | 22.4 | 9 | 5 | 8.4-TT-ΔMBP,n | 43 |
|  | 20 | 22.4 | 9 | 5 | 8.4-TT-ΔMBP,n | 43 |
|  | 21 | 22.4 | 9 | 5 | 8.4-TT-ΔMBP,n | 43 |
| Note 1: ΔMBP,n is the Multiband Relaxation factor declared by the UE for the tested band in table A.4.3.9-2 of TS38.508-2. This declaration shall fulfil the requirements in clause 6.2.1.1.3.3. Note 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant. Note 3: Max allowed sum of ΔMBP,n over all supported FR2 bands as defined in clause 6.2.1.1.3.3. Note 4: ΔMBP,n is 0 for single band UE. |  |  |  |  |  |  |

Table 6.2.2.5-3a: UE Power Class test requirements for Power Class 3 (n260)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.4.1-7 | 1 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 4 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
| Table 6.2.2.4.1-8 | 1 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 4 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 5 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 6 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 7 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 8 | 20.6 | 5.5 | 5 | 10.1-TT-ΔMBP,n | 43 |
|  | 9 | 20.6 | 5.5 | 5 | 10.1-TT-ΔMBP,n | 43 |
|  | 10 | 20.6 | 5.5 | 5 | 10.1-TT-ΔMBP,n | 43 |
|  | 11 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 12 | 20.6 | 4 | 3 | 13.6-TT-ΔMBP,n | 43 |
|  | 13 | 20.6 | 4 | 3 | 13.6-TT-ΔMBP,n | 43 |
|  | 14 | 20.6 | 4 | 3 | 13.6-TT-ΔMBP,n | 43 |
|  | 15 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 16 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 17 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 18 | 20.6 | 7.5 | 5 | 8.1-TT-ΔMBP,n | 43 |
|  | 19 | 20.6 | 7.5 | 5 | 8.1-TT-ΔMBP,n | 43 |
|  | 20 | 20.6 | 7.5 | 5 | 8.1-TT-ΔMBP,n | 43 |
| Table 6.2.2.4.1-8a | 1 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 4 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
| Table 6.2.2.4.1-9 | 1 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 4 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 5 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 6 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 7 | 20.6 | 4.5 | 4 | 12.1-TT-ΔMBP,n | 43 |
|  | 8 | 20.6 | 4.5 | 4 | 12.1-TT-ΔMBP,n | 43 |
|  | 9 | 20.6 | 4.5 | 4 | 12.1-TT-ΔMBP,n | 43 |
|  | 10 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 11 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 12 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 13 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 14 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 15 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 16 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 17 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 18 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 19 | 20.6 | 9 | 5 | 6.6-TT-ΔMBP,n | 43 |
|  | 20 | 20.6 | 9 | 5 | 6.6-TT-ΔMBP,n | 43 |
|  | 21 | 20.6 | 9 | 5 | 6.6-TT-ΔMBP,n | 43 |
| Note 1: ΔMBP,n is the Multiband Relaxation factor declared by the UE for the tested band in table A.4.3.9-2 of TS38.508-2. This declaration shall fulfil the requirements in clause 6.2.1.1.3.3. Note 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant. Note 3: Max allowed sum of ΔMBP,n over all supported FR2 bands as defined in clause 6.2.1.1.3.3. Note 4: ΔMBP,n is 0 for single band UE. |  |  |  |  |  |  |

Table 6.2.2.5-3b: Test Tolerance (Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 3.11 dB | 3.11 dB |

Table 6.2.2.5-4: UE Power Class test requirements for Power Class 4 (n257, 258, 261)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.4.1-7 | 1 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 2 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 3 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 4 | 34 | 2.5 | 2 | 29.5-TT | 43 |
| Table 6.2.2.4.1-8 | 1 | 34 | 2 | 1.5 | 30.5-TT | 43 |
|  | 2 | 34 | 2 | 1.5 | 30.5-TT | 43 |
|  | 3 | 34 | 3 | 2 | 29-TT | 43 |
|  | 4 | 34 | 3.5 | 3 | 27.5-TT | 43 |
|  | 5 | 34 | 3.5 | 3 | 27.5-TT | 43 |
|  | 6 | 34 | 3.5 | 3 | 27.5-TT | 43 |
|  | 7 | 34 | 5 | 4 | 25-TT | 43 |
|  | 8 | 34 | 5.5 | 5 | 23.5-TT | 43 |
|  | 9 | 34 | 5.5 | 5 | 23.5-TT | 43 |
|  | 10 | 34 | 5.5 | 5 | 23.5-TT | 43 |
|  | 11 | 34 | 3.5 | 3 | 27.5-TT | 43 |
|  | 12 | 34 | 4 | 3 | 27-TT | 43 |
|  | 13 | 34 | 4 | 3 | 27-TT | 43 |
|  | 14 | 34 | 4 | 3 | 27-TT | 43 |
|  | 15 | 34 | 5 | 4 | 25-TT | 43 |
|  | 16 | 34 | 5 | 4 | 25-TT | 43 |
|  | 17 | 34 | 5 | 4 | 25-TT | 43 |
|  | 18 | 34 | 7.5 | 5 | 1.5-TT | 43 |
|  | 19 | 34 | 7.5 | 5 | 1.5-TT | 43 |
|  | 20 | 34 | 7.5 | 5 | 1.5-TT | 43 |
| Table 6.2.2.4.1-8a | 1 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 2 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 3 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 4 | 34 | 2.5 | 2 | 29.5-TT | 43 |
| Table 6.2.2.4.1-9 | 1 | 34 | 3 | 2 | 29-TT | 43 |
|  | 2 | 34 | 3 | 2 | 29-TT | 43 |
|  | 3 | 34 | 3 | 2 | 29-TT | 43 |
|  | 4 | 34 | 3 | 2 | 29-TT | 43 |
|  | 5 | 34 | 3 | 2 | 29-TT | 43 |
|  | 6 | 34 | 3 | 2 | 29-TT | 43 |
|  | 7 | 34 | 4.5 | 4 | 25.5-TT | 43 |
|  | 8 | 34 | 4.5 | 4 | 25.5-TT | 43 |
|  | 9 | 34 | 4.5 | 4 | 25.5-TT | 43 |
|  | 10 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 11 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 12 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 13 | 34 | 5 | 4 | 25-TT | 43 |
|  | 14 | 34 | 5 | 4 | 25-TT | 43 |
|  | 15 | 34 | 5 | 4 | 25-TT | 43 |
|  | 16 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 17 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 18 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 19 | 34 | 9 | 5 | 20-TT | 43 |
|  | 20 | 34 | 9 | 5 | 20-TT | 43 |
|  | 21 | 34 | 9 | 5 | 20-TT | 43 |

Table 6.2.2.5-4a: UE Power Class test requirements for Power Class 4 (n260)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.4.1-7 | 1 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 2 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 3 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 4 | 31 | 2.5 | 2 | 26.5-TT | 43 |
| Table 6.2.2.4.1-8 | 1 | 31 | 2 | 1.5 | 27.5-TT | 43 |
|  | 2 | 31 | 2 | 1.5 | 27.5-TT | 43 |
|  | 3 | 31 | 3 | 2 | 26-TT | 43 |
|  | 4 | 31 | 3.5 | 3 | 24.5-TT | 43 |
|  | 5 | 31 | 3.5 | 3 | 24.5-TT | 43 |
|  | 6 | 31 | 3.5 | 3 | 24.5-TT | 43 |
|  | 7 | 31 | 5 | 4 | 22-TT | 43 |
|  | 8 | 31 | 5.5 | 5 | 20.5-TT | 43 |
|  | 9 | 31 | 5.5 | 5 | 20.5-TT | 43 |
|  | 10 | 31 | 5.5 | 5 | 20.5-TT | 43 |
|  | 11 | 31 | 3.5 | 3 | 24.5-TT | 43 |
|  | 12 | 31 | 4 | 3 | 24-TT | 43 |
|  | 13 | 31 | 4 | 3 | 24-TT | 43 |
|  | 14 | 31 | 4 | 3 | 24-TT | 43 |
|  | 15 | 31 | 5 | 4 | 22-TT | 43 |
|  | 16 | 31 | 5 | 4 | 22-TT | 43 |
|  | 17 | 31 | 5 | 4 | 22-TT | 43 |
|  | 18 | 31 | 7.5 | 5 | 18.5-TT | 43 |
|  | 19 | 31 | 7.5 | 5 | 18.5-TT | 43 |
|  | 20 | 31 | 7.5 | 5 | 18.5-TT | 43 |
| Table 6.2.2.4.1-8a | 1 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 2 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 3 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 4 | 31 | 2.5 | 2 | 26.5-TT | 43 |
| Table 6.2.2.4.1-9 | 1 | 31 | 3 | 2 | 26-TT | 43 |
|  | 2 | 31 | 3 | 2 | 26-TT | 43 |
|  | 3 | 31 | 3 | 2 | 26-TT | 43 |
|  | 4 | 31 | 3 | 2 | 26-TT | 43 |
|  | 5 | 31 | 3 | 2 | 26-TT | 43 |
|  | 6 | 31 | 3 | 2 | 26-TT | 43 |
|  | 7 | 31 | 4.5 | 4 | 22.5-TT | 43 |
|  | 8 | 31 | 4.5 | 4 | 22.5-TT | 43 |
|  | 9 | 31 | 4.5 | 4 | 22.5-TT | 43 |
|  | 10 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 11 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 12 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 13 | 31 | 5 | 4 | 22-TT | 43 |
|  | 14 | 31 | 5 | 4 | 22-TT | 43 |
|  | 15 | 31 | 5 | 4 | 22-TT | 43 |
|  | 16 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 17 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 18 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 19 | 31 | 9 | 5 | 17-TT | 43 |
|  | 20 | 31 | 9 | 5 | 17-TT | 43 |
|  | 21 | 31 | 9 | 5 | 17-TT | 43 |

### 6.2.2_1	UE maximum output power reduction enhancements

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for PC1, PC2 and PC4.

- Measurement grid for PC2/4 in Annex M.4 is FFS.

- Applicability of modifiedMPRbehaviour bit 0 to PC2, PC4 and PC5 is FFS

#### 6.2.2_1.0	General

The requirements in section 6.2.2 only apply when both UL and DL of a UE are configured for single CC operation, and they are of the same bandwidth. A UE may reduce its maximum output power due to modulation orders, transmit bandwidth configurations, waveform types and narrow allocations. This Maximum Power Reduction (MPR) is defined in subclauses below. The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation. When the maximum output power of a UE is modified by MPR, the power limits specified in subclause 6.2.4 apply.

For a UE that is configured for single CC operation with different channel bandwidths in UL and DL, the requirements in section 6.2A.2 apply.

For all power classes, the waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

6.2.2_1.1	Test purpose

The number of RB identified in 6.2.2.3 is based on meeting the requirements for the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2.2_1.2	Test applicability

The requirements of this test apply to all types of Rel-15 PC3 devices which supports modifiedMPRbehaviour bit 0 capability (according to Annex P.1) and Rel-16 and forward PC3 devices.

6.2.2_1.3	Minimum conformance requirements

##### 6.2.2_1.3.1	FFS

##### 6.2.2_1.3.2	FFS

##### 6.2.2_1.3.3	UE maximum output power reduction for power class 3

For transmission bandwidth configuration less than or equal to 200MHz, and 0 ≤ RBstart < Ceil(1/3 NRB) or Ceil((2/3NRB) LCRB) < RBstart ≤ NRB-LCRB:

-	MPRnarrow = 2.5 dB, when BWalloc,RB is less than or equal to 1.44 MHz,

-	MPRnarrow = 2.0 dB, when 1.44 MHz < BWalloc,RB <= 4.32 MHz,

-	otherwise MPRnarrow = 0 dB.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2_1.3.3-1.

Table 6.2.2_1.3.3-1 MPRWT for power class 3, BWchannel ≤ 200 MHz

| Modulation |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 2.0 |
|  | QPSK | 0.0 | ≤ 2.0 |
|  | 16 QAM | ≤ 3.0 | ≤ 3.5 |
|  | 64 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 5.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 |

Where the following parameters are defined to specify valid RB allocation ranges for RB allocations in Table 6.2.2_1.3.3-1:

-	RBStart,Low = max(1, LCRB), where max() indicates the largest value of all arguments.

-	RBStart,High = NRB – RBStart,Low – LCRB,

An RB allocation belonging to table 6.2.2_1.3.3-1 is a Region 1 inner RB allocation if:

-	RBStart,Low  ≤  RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(NRB/3), where ceil(x) is the smallest integer greater than or equal to x.

For transmission bandwidth configuration equal to 400MHz,

MPRnarrow = 2.5 dB, when BWalloc,RB is less than or equal to 1.44 MHz, and 0 ≤ RBstart < Ceil(1/3 NRB) or Ceil(2/3NRB) ≤ RBstart ≤ NRB-LCRB, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2_1.3.3-2.

Table 6.2.2_1.3.3-2 MPRWT for power class 3, BWchannel = 400 MHz

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 3.0 |
|  | QPSK | 0.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 |

Where the following parameters are defined to specify valid RB allocation ranges for RB allocations in Table 6.2.2_1.3.3-2:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBend = RBStart + LCRB - 1

An RB allocation belonging to table 6.2.2_1.3.3-2 is a Region 1 inner RB allocation if

RBstart ≥ Ceil(1/4 NRB) AND RBend < Ceil(3/4 NRB) AND LCRB ≤ Ceil(1/4 NRB)

For all transmission bandwidth configurations, an RB allocation is an Edge allocation if it is NOT a Region 1 inner allocation.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.2.3

##### 6.2.2_1.3.4	FFS

6.2.2_1.4	Test description

Same as in clause 6.2.2.1.4.1 with following exceptions: Instead of Tables 6.2.2.1.4.1-1 to 6.2.2.1.4.1-9 use Tables 6.2.2_1.1.4.1-1 and 6.2.2_1.1.4.1-4

Table 6.2.2_1.4.1-1: Test Configuration Table (Power Class 2, 3 and 4, MPRnarrow, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 4 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 5 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial2_Left |
| 6 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial2_Right |
| 7 | Low |  |  |  | DFT-s-OFDM QPSK | Inner_Partial2_Left |
| 8 | High |  |  |  | DFT-s-OFDM QPSK | Inner_Partial2_Right |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2.2_1.4.1-2: Test Configuration Table (Power Class 3, MPRWT, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 2 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 3 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full |
| 4 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 7 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Inner_Full |
| 8 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 9 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 10 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 11 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full |
| 12 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 13 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 14 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 15 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 16 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 17 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 18 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 19 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 20 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2.2_1.4.1-3: Test Configuration Table (Power Class 3, MPRnarrow, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Right |
| 3 | Low |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Left |
| 4 | High |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Right |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2.2_1.4.1-4: Test Configuration Table (Power Class 3, MPRWT, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 12 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 13 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 14 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 15 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 16 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 17 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 18 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 19 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 21 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

6.2.2.5	Test requirement

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in following tables.

Table 6.2.2_1.5-1: UE Power Class test requirements for Power Class 3 (n257, 258, 261)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| 6.2.2_1.4.1-1 | 1 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 5 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
|  | 6 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
|  | 7 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
|  | 8 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
| 6.2.2_1.4.1-2 | 1 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 5 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 6 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 7 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 8 | 22.4 | 5.5 | 5 | 11.9-TT-ΔMBP,n | 43 |
|  | 9 | 22.4 | 5.5 | 5 | 11.9-TT-ΔMBP,n | 43 |
|  | 10 | 22.4 | 5.5 | 5 | 11.9-TT-ΔMBP,n | 43 |
|  | 11 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 12 | 22.4 | 4 | 3 | 15.4-TT-ΔMBP,n | 43 |
|  | 13 | 22.4 | 4 | 3 | 15.4-TT-ΔMBP,n | 43 |
|  | 14 | 22.4 | 4 | 3 | 15.4-TT-ΔMBP,n | 43 |
|  | 15 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 16 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 17 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 18 | 22.4 | 7.5 | 5 | 9.9-TT-ΔMBP,n | 43 |
|  | 19 | 22.4 | 7.5 | 5 | 9.9-TT-ΔMBP,n | 43 |
|  | 20 | 22.4 | 7.5 | 5 | 9.9-TT-ΔMBP,n | 43 |
| 6.2.2_1.4.1-3 | 1 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
| 6.2.2_1.4.1-4 | 1 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 5 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 6 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 7 | 22.4 | 4.5 | 4 | 13.9-TT-ΔMBP,n | 43 |
|  | 8 | 22.4 | 4.5 | 4 | 13.9-TT-ΔMBP,n | 43 |
|  | 9 | 22.4 | 4.5 | 4 | 13.9-TT-ΔMBP,n | 43 |
|  | 10 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 11 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 12 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 13 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 14 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 15 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 16 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 17 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 18 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 19 | 22.4 | 9 | 5 | 8.4-TT-ΔMBP,n | 43 |
|  | 20 | 22.4 | 9 | 5 | 8.4-TT-ΔMBP,n | 43 |
|  | 21 | 22.4 | 9 | 5 | 8.4-TT-ΔMBP,n | 43 |
| Note 1: ΔMBP,n is the Multiband Relaxation factor declared by the UE for the tested band in table A.4.3.9-2 of TS38.508-2. This declaration shall fulfil the requirements in clause 6.2.1.1.3.3. Note 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant. Note 3: Max allowed sum of ΔMBP,n over all supported FR2 bands as defined in clause 6.2.1.1.3.3. Note 4: ΔMBP,n is 0 for single band UE. |  |  |  |  |  |  |

Table 6.2.2_1.5-2: UE Power Class test requirements for Power Class 3 (n260)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| 6.2.2_1.4.1-1 | 1 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  |  | 4 |  | 20.6 |  | 2.5 |  | 2 |  | 16.1-TT-ΔMBP,n |  | 43 |  |
|  | 5 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
|  | 6 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
|  | 7 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
|  | 8 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
| 6.2.2_1.4.1-2 | 1 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 4 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 5 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 6 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 7 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 8 | 20.6 | 5.5 | 5 | 10.1-TT-ΔMBP,n | 43 |
|  | 9 | 20.6 | 5.5 | 5 | 10.1-TT-ΔMBP,n | 43 |
|  | 10 | 20.6 | 5.5 | 5 | 10.1-TT-ΔMBP,n | 43 |
|  | 11 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 12 | 20.6 | 4 | 3 | 13.6-TT-ΔMBP,n | 43 |
|  | 13 | 20.6 | 4 | 3 | 13.6-TT-ΔMBP,n | 43 |
|  | 14 | 20.6 | 4 | 3 | 13.6-TT-ΔMBP,n | 43 |
|  | 15 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 16 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 17 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 18 | 20.6 | 7.5 | 5 | 8.1-TT-ΔMBP,n | 43 |
|  | 19 | 20.6 | 7.5 | 5 | 8.1-TT-ΔMBP,n | 43 |
|  | 20 | 20.6 | 7.5 | 5 | 8.1-TT-ΔMBP,n | 43 |
| 6.2.2_1.4.1-3 | 1 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
| 6.2.2_1.4.1-4 | 1 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 4 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 5 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 6 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 7 | 20.6 | 4.5 | 4 | 12.1-TT-ΔMBP,n | 43 |
|  | 8 | 20.6 | 4.5 | 4 | 12.1-TT-ΔMBP,n | 43 |
|  | 9 | 20.6 | 4.5 | 4 | 12.1-TT-ΔMBP,n | 43 |
|  | 10 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 11 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 12 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 13 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 14 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 15 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 16 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 17 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 18 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 19 | 20.6 | 9 | 5 | 6.6-TT-ΔMBP,n | 43 |
|  | 20 | 20.6 | 9 | 5 | 6.6-TT-ΔMBP,n | 43 |
|  | 21 | 20.6 | 9 | 5 | 6.6-TT-ΔMBP,n | 43 |
| Note 1: ΔMBP,n is the Multiband Relaxation factor declared by the UE for the tested band in table A.4.3.9-2 of TS38.508-2. This declaration shall fulfil the requirements in clause 6.2.1.1.3.3. Note 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant. Note 3: Max allowed sum of ΔMBP,n over all supported FR2 bands as defined in clause 6.2.1.1.3.3. Note 4: ΔMBP,n is 0 for single band UE. |  |  |  |  |  |  |

Table 6.2.2_1.5-3: Test Tolerance (Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 3.11 dB | 3.11 dB |

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±4.89 dB (FR2a, NTC testing) ±5.09 dB (FR2b, NTC testing) ±5.17 dB (FR2a, ETC testing) ±5.37 dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.3-1 in TR 38.903) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP Max Device size ≤ 30 cm ±4.42 dB (FR2a, NTC testing) ±4.62 dB (FR2b, NTC testing) ±4.70 dB (FR2a, ETC testing) ±4.90 dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.3-2 in TR 38.903) |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC3 Max Device size ≤ 30 cm ±4.60 dB (FR2a) ±5.20 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-3 in TR 38.903) |
| 6.2.2 UE maximum output power reduction | PC3 Max Device size ≤ 30 cm ±4.92 dB (FR2a), NTC testing ±5.10 dB (FR2b), NTC testing ±5.20 dB (FR2a, ETC testing) ±5.38 dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.4-1 in TR 38.903) |
| 6.2.2_1 UE maximum output power reduction enhancements | Same as 6.2.2 |  |
| 6.2.3 UE maximum output power with additional requirements | Same as 6.2.2 |  |

## <<< END OF CHANGES 2>>>

## <<< START OF CHANGES 3>>>

## F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 2.87 dB (FR2a, NTC) 2.87 dB (FR2b, NTC) 3.04 dB (FR2a, ETC) 3.04 dB (FR2b, ETC) Max EIRP 0 dB | Minimum peak EIRP TT = 0.60 x (MTSUIFF - 0.1) (FR2a) TT = 0.60 x (MTSUIFF - 0.3) (FR2b) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP IFF (Max Device size ≤ 30 cm) 2.65 dB (FR2a, NTC) 2.77 dB (FR2b, NTC) 2.82 dB (FR2a, ETC) 2.94 dB (FR2b, ETC) | Max TRP TT = 0.60 x MTSUIFF |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC1 TBD PC2 TBD PC3 IFF (Max Device size ≤ 30 cm) 2.58 dB (FR2a) 2.58 dB (FR2b) PC4 TBD | PC3 TT = 0.60 x (MTSUIFF - 0.3) (FR2a) TT = 0.60 x (MTSUIFF - 0.9) (FR2b) |
| 6.2.2 UE maximum output power reduction | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.11 dB (FR2a, NTC) 3.11 dB (FR2b, NTC) 3.30 dB (FR2a, ETC) 3.30 dB (FR2b, ETC) | Minimum peak EIRP TT = 0.65 x (MTSUIFF - 0.13) (FR2a) TT = 0.65 x (MTSUIFF - 0.31) (FR2b) |
| 6.2.2_1 UE maximum output power reduction enhancements | Same as 6.2.2 |  |
| 6.2.3 UE maximum output power with additional requirements | Same as 6.2.2 |  |

## <<< END OF CHANGES 3>>>