3GPP TSG-RAN5 Meeting #97	R5-225930

# Toulouse, France, 14th Nov 2022 - 18nd Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0469 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test case 7.26 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Apply default EVS configuration according AP#96.02. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case has been updated to use the generic procedure A.4.1a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The action point cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

## 7.26	Mobile Originating CAT / Forking Model / MO Voice Call / 5GS

7.26.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and configured to use preconditions and having initiated an MO voice call with preconditions up to the last step before 180 Ringing }

ensure that {

when { UE receives 183 Session Progress on a forked dialog indicating Customized Alerting Tones }

then { UE moves forked dialog forward until up to the last step before 180 Ringing }

}

(2)

with { UE having moved both dialogs forward up to the last step before 180 Ringing }

ensure that {

when { UE receives 200 OK for INVITE for the first dialog }

then { UE acks reception of 200 OK for INVITE }

}

7.26.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.182, clause 4.5.5.1.1]:

The UE shall follow the procedures specified in 3GPP TS 24.229 [4] for session initiation and termination.

[TS 24.628, clause 4.7.2.1]:

Procedures according to 3GPP TS 24.229 [1] shall apply.

Certain services require the usage of the Alert-Info header field, Call-Info header field and Error-Info header field according to procedures specified by IETF RFC 3261 [4].

If the UE detects that in-band information is received from the network as early media, the in-band information received from the network shall override locally generated communication progress information.

NOTE 1:	In-band information received from the network overrides any locally generated communication progress information also when the most recently received P-Early-Media header fields of all early dialogs contain "inactive" or "recvonly".

NOTE 2:	When multiple early dialogs exist with authorization as "sendrecv" or "sendonly", the mechanism used by the UE to associate the received early media with the correct early dialog is unspecified in this version of this specification.

The UE shall not generate the locally generated communication progress information if an early dialog exists where the last received P-Early-Media header field as described in IETF RFC 5009 [12] contains "sendrecv" or "sendonly".

If an early dialog exists where a SIP 18x response to the SIP INVITE request other than 183 (Session Progress) response was received, no early dialog exists where the last received P-Early-Media header field as described in IETF RFC 5009 [12] contained "sendrecv" or "sendonly" and in-band information is not received from the network, then the UE is expected to render the locally generated communication progress information.

NOTE 3:	According to 3GPP TS 22.173 [23] the UE for an MMTel session generates the communication progress information specified in clause F.2 of 3GPP TS 22.001 [24], with parameters applicable for the home network of the UE.

If the UE supports the P-Early-Media header field as defined in IETF RFC 5009 [12], and at least one P-Early-Media header field has been received on at least one early dialog, then the UE shall send any available user generated media, e.g. speech or DTMF, on media stream(s) associated with the early dialog for which the most recent P-Early-Media header field, as described in IETF RFC 5009 [12], contained a "sendrecv" header field value. If there is more than one such early dialog, the UE shall use the early dialog where the P-Early-Media header field was most recently received.

If the UE receives a re-INVITE request containing no SDP offer, the UE shall send a 200 (OK) response containing an SDP offer according to 3GPP TS 24.229 [1] indicating the directionality used by UE as

-	"sendonly" if the re-INVITE request is received on a dialog where the associated communication session has been put on hold by the user or has been put on hold by both users at both ends; and

-	"sendrecv" otherwise.

7.26.3	Test description

7.26.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	UE is configured to use preconditions.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1 [21]) and registered to IMS.

7.26.3.2	Test procedure sequence

Table 7.26.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to initiate a voice call. | - | - | - | - |
| 1A-1F | Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel with Step 2, parallel behaviour defined in table 7.26.3.2-2 takes place | - | - | - | - |
| 2-6 | Steps 1-5 of generic procedures of MO voice call with preconditions defined in A.4.1a. | - | Setup dialog 1 | - | - |
| 6A | Step 10 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] is performed. | - | - | - | - |
| - | EXCEPTION: In parallel to steps 6B and 6C below, step 7occurs. | - | - | - | - |
| 6B-6C | Steps 11-12 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 7-8 | Steps 6-7 of generic procedures of MO voice call with preconditions defined in A.4.1a. | - | Setup dialog 1 | - | - |
| 9 | SS sends an SDP answer. (Step 3 of A.4.1a) | <-- | 183 Progress (dialog 2) | - | - |
| 10 | Check: Does the UE acknowledge reception of 183 Session Progress? (Step 4 of A.4.1a) | --> | PRACK (dialog 2) | 1 | P |
| 11 | SS responds to PRACK. (Step 5 of A.4.1a) | <-- | 200 OK (dialog 2) | - | - |
| 12-13 | Void | - |  | - | - |
| 14 | The SS sends 200 OK for INVITE sent in step 1 above | <-- | 200 OK | - | - |
| 15 | Check: Does the UE send the ACK to the 200 OK for the INVITE in step 1? | --> | ACK | 2 | P |
| 16 | The UE is made to release the call | - | - | - | - |
| 17 | The UE releases the call with BYE | --> | BYE | - | - |
| 18 | The SS sends 200 OK for BYE | <-- | 200 OK | - | - |

Table 7.26.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |

7.26.3.3	Specific message contents

Table 7.26.3.3-1: 183 Session Progress with an SDP offer (step 9, table 7.26.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in annex A.2.3, Condition A1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To |  |  |  |  |
| tag |  | any value different from what is used in steps 1-5 |  |  |
| Contact |  |  |  |  |
| addr-spec |  | <sip:cat-as.home1.net;+g.3gpp.icsi ref="urn%3Aurn-7%3gpp-service.ims.icsi.mmtel"> |  |  |
| P-Early-Media |  |  |  |  |
| em-param |  | sendonly |  |  |
| Require |  |  |  | TS 24.229 [7] |
| option-tag |  | precondition |  |  |
| Message-body |  | Session description: v=0 o=- 1111111112 1111111111 IN (addrtype) (unicast-address for SS for early-media) s=- c=IN (addrtype) (connection-address for SS for early-media) b=AS:37 Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos mandatory remote sendrecv a=conf:qos remote sendrecv Other attributes: a=content:g.3gpp.cat |  | TS 24.229 [7] RFC 4566 [38] |

Table 7.26.3.3-2: PRACK (step 10, table 7.26.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in annex A.2.4, Conditions A1 and A7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Require |  |  |  |  |
| option-tag |  | precondition |  |  |
| Message-body |  | Contents is copied from step 6 of annex A.4.1a with the following exceptions: Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote sendrecv a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv |  | TS 24.229 [7] |

Table 7.26.3.3-3: 200 OK (step 11, table 7.26.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in annex A.3.1, Conditions A10 and A22 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To |  |  |  |  |
| tag |  | Same value as used in step 9 |  |  |
| Require |  |  |  |  |
| option-tag |  | precondition |  |  |
| Content-Type |  |  |  |  |
| media-type |  | application/sdp |  |  |
| Content-Length |  |  |  |  |
| value |  | length of message-body |  |  |
| Message-body |  | SDP body of the 200 OK response copied from the received PRACK and modified as follows: - IP address on "c=" lines and transport port on "m=" lines changed to indicate to which IP address and port the UE should start sending the media (same as used in step 9 above); - "o=" line identical to previous SDP sent by SS except that sess-version is incremented. |  | TS 24.229 [7] |

<End of modified section>