3GPP TSG-RAN5 Meeting #97	R5-225963

# Toulouse, France, 14 – 18 Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0384 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional UE declared capabilities for new NR bands n91, n92, n93 and n94 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing UE declared capabilities for NR bands n91, n92, n93 and n94 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UE declared capabilities for NR bands n91, n92, n93 and n94 into Table A.4.3.9-4c: 2 Rx antenna ports Capabilities |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE declared capabilities for NR bands n91, n92, n93 and n94 will not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### A.4.3.9	Additional capabilities for UE declared capability

Table A.4.3.9-1: UE declared capabilities

| Item | UE declared capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Enhanced Type 1 Receiver for NR | 38.101-4, 5 | Rel-15 | pc_nr_enh_type1_receiver | Support for Enhanced Type 1 Receiver (SU-MIMO Interference Mitigation advanced receiver) |
| 2 | Vehicular UE | 38.101-1, 3 | Rel-15 | pc_nr_vehicular_ue |  |

Table A.4.3.9-2: UE declared multi-band peak EIRP relaxation factors for Rel-15 FR2 power class 3 UE

| Item |  | Supported FR2 bands set |  | Ref. |  | Release |  | peak EIRP relaxation factor per band, MBp (dB) (Note 1) |  |  |  |  |  |  |  | Maximum sum of MBp, ∑MBP (dB) (Note 2) |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | n257 |  | n258 |  | n260 |  | n261 |  |  |  |  |  |
| 1 |  | n257, n258 |  | 38.101-2, 6.2.1.3 |  | Rel-15 |  |  |  |  |  | N/A |  | N/A |  | 1.3 |  |  |  |
| 2 |  | n257, n260 |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  | 1.0 |  |  |  |
| 3 |  | n258, n260 |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  | 1.0 |  |  |  |
| 4 |  | n258, n261 |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  | 1.0 |  |  |  |
| 5 |  | n260, n261 |  |  |  |  |  | N/A |  | N/A |  |  |  |  |  | 0.0 |  | No relaxation factor allowed |  |
| 6 |  | n257, n258, n260 |  |  |  |  |  |  |  |  |  |  |  | N/A |  | 1.7 |  |  |  |
| 7 |  | n257, n258, n261 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | 1.7 |  |  |  |
| 8 |  | n257, n260, n261 |  |  |  |  |  |  |  | N/A |  |  |  |  |  | 0.5 |  |  |  |
| 9 |  | n258, n260, n261 |  |  |  |  |  | N/A |  |  |  |  |  |  |  | 1.5 |  |  |  |
| 10 |  | n257, n258, n260, n261 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.7 |  |  |  |
| 11 |  | n257, n261 |  |  |  |  |  |  |  | N/A |  | N/A |  |  |  | 0.0 |  | No relaxation factor allowed |  |
| Note 1: UE vendor to fill in the needed relaxation factor per band that is ≥0 for Rel-15 UE supporting only Rel-15 FR2 bands. One row to be filled in, the one matching the supported FR2 bands of the UE as declared in Table A.4.3.1-3. Note 2: Max allowed sum of MBp over all supported FR2 bands as defined in TS 38.521-2 clause 6.2.1.1.3.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.3.9-3: UE declared multi-band peak EIRP Spherical coverage relaxation factors for Rel-15 FR2 power class 3 UE

| Item |  | Supported FR2 bands set |  | Ref. |  | Release |  | EIRP Spherical coverage relaxation factor per band, MBs (dB) (Note 1) |  |  |  |  |  |  |  | Maximum sum of MBs, ∑MBs (dB) (Note 2) |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | n257 |  | n258 |  | n260 |  | n261 |  |  |  |  |  |
| 1 |  | n257, n258 |  | 38.101-2, 6.2.1.3 |  | Rel-15 |  |  |  |  |  | N/A |  | N/A |  | 1.25 |  |  |  |
| 2 |  | n257, n260 |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  | 0.75 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 3 |  | n258, n260 |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  | 0.75 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 4 |  | n258, n261 |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  | 1.25 |  |  |  |
| 5 |  | n260, n261 |  |  |  |  |  | N/A |  | N/A |  |  |  |  |  | 0.75 |  | No relaxation allowed for n260 |  |
| 6 |  | n257, n258, n260 |  |  |  |  |  |  |  |  |  |  |  | N/A |  | 1.75 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 7 |  | n257, n258, n261 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | 1.75 |  |  |  |
| 8 |  | n257, n260, n261 |  |  |  |  |  |  |  | N/A |  |  |  |  |  | 1.25 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 9 |  | n258, n260, n261 |  |  |  |  |  | N/A |  |  |  |  |  |  |  | 1.25 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 10 |  | n257, n258, n260, n261 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.75 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 11 |  | n257, n261 |  |  |  |  |  |  |  | N/A |  | N/A |  |  |  | 0.0 |  | No relaxation factor allowed |  |
| Note 1: UE vendor to fill in the needed relaxation factor per band that is ≥0 for Rel-15 UE supporting only Rel-15 FR2 bands. One row to be filled in, the one matching the supported FR2 bands of the UE as declared in Table A.4.3.1-3 Note 2: Max allowed sum of MBs over all supported FR2 bands as defined in TS 38.521-2 clause 6.2.1.1.3.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.3.9-4a: FDD 4 Rx antenna ports Capabilities

| Item |  | Band |  | Ref. |  | Release |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | FDD Band n1 |  | 38.101-1, 7.3.2 |  | Rel-15 |  |  |  |
| 2 |  | FDD Band n2 |  | 38.101-1, 7.3.2 |  | Rel-15 |  |  |  |
| 3 |  | FDD Band n3 |  | 38.101-1, 7.3.2 |  | Rel-15 |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 7 |  | FDD Band n7 |  | 38.101-1, 7.3.2 |  | Rel-15 |  | NOTE 2 |  |
| 8 |  | FDD Band n8 |  | 38.101-1.7.3.2 |  | Rel-17 |  | 4 Rx operation is targeted for FWA form factor |  |
| ... |  |  |  |  |  |  |  |  |  |
| 28 |  | FDD Band n28 |  | 38.101-1, 7.3.2 |  | Rel-16 |  | 4 Rx operation is targeted for FWA form factor |  |
| ... |  |  |  |  |  |  |  |  |  |
| 30 |  | FDD Band n30 |  | 38.101-1, 7.3.2 |  | Rel-16 |  |  |  |
| ... |  |  |  |  |  |  |  |  |  |
| 66 |  | FDD Band n66 |  | 38.101-1, 7.3.2 |  | Rel-15 |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 70 |  | FDD Band n70 |  | 38.101-1, 7.3.2 |  | Rel-15 |  |  |  |
| 71 |  | FDD Band n71 |  | 38.101-1, 7.3.2 |  | Rel-16 |  | 4 Rx operation is targeted for FWA form factor |  |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/2. NOTE 2: Support of 4 Rx for this band is mandatory for non-vehicular UEs i.e. if support has NOT been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |  |  |  |  |  |

Table A.4.3.9-4b: TDD 4 Rx antenna ports Capabilities

| Item |  | Band | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 34 |  | TDD Band n34 | 38.101-1, 7.3.2 | Rel-15 |  |
| ... |  |  |  |  |  |
| 38 |  | TDD Band n38 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 39 |  | TDD Band n39 | 38.101-1, 7.3.2 | Rel-15 |  |
| ... |  |  |  |  |  |
| 40 |  | TDD Band n40 | 38.101-1, 7.3.2 | Rel-15 |  |
| 41 |  | TDD Band n41 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| ... |  |  |  |  |  |
| 48 |  | TDD Band n48 | 38.101-1, 7.3.2 | Rel-16 | NOTE 2 |
| … |  |  |  |  |  |
| 77 |  | TDD Band n77 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 78 |  | TDD Band n78 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 79 |  | TDD Band n79 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/3. NOTE 2: Support of 4 Rx for this band is mandatory for non-vehicular UEs i.e. if support has NOT been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |  |

Table A.4.3.9-4c: 2 Rx antenna ports Capabilities

| Item |  | Band |  | Ref. |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | FDD Band n1 |  | 38.101-1, 7.3.2 |  |  |  |
| 2 |  | FDD Band n2 |  | 38.101-1, 7.3.2 |  |  |  |
| 3 |  | FDD Band n3 |  | 38.101-1, 7.3.2 |  |  |  |
| 4 |  | FDD Band n5 |  | 38.101-1, 7.3.2 |  |  |  |
| 5 |  | FDD Band n7 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 6 |  | FDD Band n8 |  | 38.101-1, 7.3.2 |  |  |  |
| 7 |  | FDD Band n12 |  | 38.101-1, 7.3.2 |  |  |  |
| 7b |  | FDD Band n14 |  | 38.101-1, 7.3.2 |  |  |  |
| 8 |  | FDD Band n20 |  | 38.101-1, 7.3.2 |  |  |  |
| 8d |  | FDD Band n24 |  | 38.101-1, 7.3.2 |  |  |  |
| 9 |  | FDD Band n25 |  | 38.101-1, 7.3.2 |  |  |  |
| 9a |  | FDD Band n26 |  | 38.101-1, 7.3.2 |  |  |  |
| 10 |  | FDD Band n28 |  | 38.101-1, 7.3.2 |  |  |  |
| 10a |  | SDL Band n29 |  | 38.101-1, 7.3.2 |  |  |  |
| 10b |  | FDD Band n30 |  | 38.101-1, 7.3.2 |  |  |  |
| 11 |  | TDD Band n34 |  | 38.101-1, 7.3.2 |  |  |  |
| 12 |  | TDD Band n38 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 13 |  | TDD Band n39 |  | 38.101-1, 7.3.2 |  |  |  |
| 14 |  | TDD Band n40 |  | 38.101-1, 7.3.2 |  |  |  |
| 15 |  | TDD Band n41 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 16 |  | TDD Band n48 |  | 38.101-1, 7.3.2 |  |  |  |
| 17 |  | TDD Band n50 |  | 38.101-1, 7.3.2 |  |  |  |
| 18 |  | TDD Band n51 |  | 38.101-1, 7.3.2 |  |  |  |
| 18a |  | Reserved |  |  |  |  |  |
| 18b |  | TDD Band n53 |  | 38.101-1, 7.3.2 |  |  |  |
| 19 |  | FDD Band n65 |  | 38.101-1, 7.3.2 |  |  |  |
| 20 |  | FDD Band n66 |  | 38.101-1, 7.3.2 |  |  |  |
| 21 |  | FDD Band n70 |  | 38.101-1, 7.3.2 |  |  |  |
| 22 |  | FDD Band n71 |  | 38.101-1, 7.3.2 |  |  |  |
| 23 |  | FDD Band n74 |  | 38.101-1, 7.3.2 |  |  |  |
| 24 |  | TDD Band n77 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 25 |  | TDD Band n78 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 26 |  | TDD Band n79 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 27 |  | FDD Band n91 |  | 38.101-1, 7.3.2 |  |  |  |
| 28 |  | FDD Band n92 |  | 38.101-1, 7.3.2 |  |  |  |
| 29 |  | FDD Band n93 |  | 38.101-1, 7.3.2 |  |  |  |
| 30 |  | FDD Band n94 |  | 38.101-1, 7.3.2 |  |  |  |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/1. NOTE 2: Support of 2 Rx for this band is allowed only for vehicular UEs i.e. if support has been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |  |  |  |

Table A.4.3.9-4d: Enhanced transient capabilities

| Item | Band | Ref | Release | enhanced transient capability per band |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2us | 4us | 7us |  |
| 1 | FDD Band n1 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 2 | FDD Band n2 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 3 | FDD Band n3 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 4 | FDD Band n5 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 5 | FDD Band n7 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 6 | FDD Band n8 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 7 | FDD Band n12 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 8 | FDD Band n14 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 9 | FDD Band n20 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 10 | FDD Band n24 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 11 | FDD Band n25 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 12 | FDD Band n26 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 13 | FDD Band n28 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 14 | FDD Band n30 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 15 | TDD Band n34 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 16 | TDD Band n38 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 17 | TDD Band n39 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 18 | TDD Band n40 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 19 | TDD Band n41 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 20 | TDD Band n46 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 21 | TDD Band n48 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 22 | TDD Band n50 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 23 | TDD Band n51 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 24 | TDD Band n53 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 25 | TDD Band n65 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 26 | TDD Band n66 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 27 | FDD Band n70 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 28 | FDD Band n71 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 29 | FDD Band n74 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 30 | TDD Band n77 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 31 | TDD Band n78 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 32 | TDD Band n79 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 33 | TDD Band n96 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| NOTE 1: At least one band from those listed in the present table needs to be supported with enhanced transient capability of 2us, 4us or 7us if UE has indicated support of the capability defined in Table A.4.3.2-1/79. NOTE 2: Indicate transient capability for each band by ticking the cell corresponding to the smallest enhanced transient capability that the UE supports for that band. |  |  |  |  |  |  |  |

-------- End of changes --------