3GPP TSG-RAN5 Meeting #97	R5-226064

# Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0276 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.12 - MSRP Messages for MCData |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.5.12	MSRP Messages for MCData

#### 5.5.12.1	MSRP SEND

##### 5.5.12.1.1	MSRP SEND from the UE

Table 5.5.12.1.1-1: MSRP SEND from the UE

| Derivation Path: RFC 4975 [120] clause 9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Transaction Identifier |  |  |  |  |
| value | any allowed value |  |  |  |
| To-Path |  |  |  |  |
| value | MSRP URI as provided by the SS in its SDP message sent to the UE during call establishment |  |  |  |
| From-Path |  |  |  |  |
| value | MSRP URI as provided by the UE during call establishment |  |  |  |
| Message-ID |  |  |  |  |
| value | any allowed value | In case of chunking the same Message-ID shall be used for all chunks of the message |  |  |
| Byte-Range |  |  |  |  |
| range-start | 1 for the first chunk of a message, length of all previous chunks for a second or later chunk of the message |  |  |  |
|  | 1 |  |  | EMPTY_SEND_REQ |
| range-end | any allowed value |  |  |  |
|  | 0 |  |  | EMPTY_SEND_REQ |
| total length | any allowed value | may be a specific length or "*" |  |  |
|  | 0 |  |  | EMPTY_SEND_REQ |
| Content-Type | as specified by the test case |  | TS 24.582 [89], clause 6.4 |  |
|  | not present |  |  | EMPTY_SEND_REQ |
| data | as specified by the test case |  |  |  |
|  | not present |  |  | EMPTY_SEND_REQ |
| End-line |  |  |  |  |
| transact-id | same value as used in Transaction Identifier field |  |  |  |
| continuation-flag | "+" in case of chunking when further chunks will follow; "$" in case of the message's last chunk or if the MSRP SEND request contains the entire message |  |  |  |
|  | "$" |  |  | EMPTY_SEND_REQ |

| Condition | Explanation |
| --- | --- |
| EMPTY_SEND_REQ | Empty SEND request to bind the TCP connection to an MSRP session |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.12.1.1-2..4: Void

##### 5.5.12.1.2	MSRP SEND from the SS

Table 5.5.12.1.2-1: MSRP SEND from the SS

| Derivation Path: RFC 4975 [120] clause 9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Transaction Identifier |  |  |  |  |
| value | value assigned by the SS | The SS shall use a different value for each SEND request sent to the UE during a test case |  |  |
| To-Path |  |  |  |  |
| value | MSRP URI as provided by the UE in its SDP message sent to the SS during call establishment |  |  |  |
| From-Path |  |  |  |  |
| value | MSRP URI as provided by the SS in its SDP message sent to the UE during call establishment |  |  |  |
| Message-ID |  |  |  |  |
| value | value assigned by the SS | The SS shall use a different value for each message sent to the UE during a test case (NOTE 1) |  |  |
| Byte-Range |  |  |  |  |
| range-start | 1 |  |  |  |
| range-end | length of the message in bytes | NOTE 1 |  |  |
|  | 0 |  |  | EMPTY_SEND_REQ |
| total length | length of the message in bytes | NOTE 1 |  |  |
|  | 0 |  |  | EMPTY_SEND_REQ |
| Content-Type | as specified by the test case |  | TS 24.582 [89], clause 6.4 |  |
|  | not present |  |  | EMPTY_SEND_REQ |
| data | as specified by the test case |  |  |  |
|  | not present |  |  | EMPTY_SEND_REQ |
| End-line |  |  |  |  |
| transact-id | same value as used in Transaction Identifier field |  |  |  |
| continuation-flag | "$" | NOTE 1 |  |  |
| NOTE 1: It is assumed that in general there is no chunking in DL for MCData test cases. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EMPTY_SEND_REQ | Empty SEND request to bind the TCP connection to an MSRP session |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.12.1.2-2: Void

#### 5.5.12.2	MSRP 200 (OK)

##### 5.5.12.2.1	MSRP 200 (OK) from the UE

Table 5.5.12.2.1-1: MSRP 200 (OK) from the UE

| Derivation Path: RFC 4975 [120] clause 9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Transaction Identifier |  |  |  |  |
| value | same value as received in the MSRP SEND request |  |  |  |
| To-Path |  |  |  |  |
| value | Same value as received in the From-Path of the MSRP SEND request | According to Table 5.5.12.1.2-1 the SS sends only one URI in its SEND requests | RFC 4975 clause 7.2 |  |
| From-Path |  |  |  |  |
| value | MSRP URI of the UE (as provided by the UE in its SDP message sent to the SS during call establishment) |  | RFC 4975 clause 7.2 |  |
| End-line |  |  |  |  |
| transact-id | same value as used in Transaction Identifier field |  |  |  |
| continuation-flag | "$" |  |  |  |

##### 5.5.12.2.2	MSRP 200 (OK) from the SS

Table 5.5.12.2.2-1: MSRP 200 (OK) from the SS

| Derivation Path: RFC 4975 [120] clause 9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Transaction Identifier |  |  |  |  |
| value | same value as received in the MSRP SEND message |  |  |  |
| To-Path |  |  |  |  |
| value | same value as received in the From-Path of the MSRP SEND request | According to Table 5.5.12.1.1-1 it is assumed that the UE sends only one URI in its SEND requests | RFC 4975 clause 7.2 |  |
| From-Path |  |  |  |  |
| value | MSRP URI of the SS (as provided by the SS in its SDP message sent to the UE during call establishment) |  | RFC 4975 clause 7.2 |  |
| End-line |  |  |  |  |
| transact-id | same value as used in Transaction Identifier field |  |  |  |
| continuation-flag | "$" |  |  |  |