3GPP TSG-RAN5 Meeting #97	R5-226065

# Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0277 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.2 - Default SIP message and other information elements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In Table 5.5.2.5.1-1 and Table 5.5.2.12-1 the comments of the Resource-Priority refer to Table 5.5.8.4-1 which is applicable for MCPTT only. - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarification added to Resource-Priority in Table 5.5.2.5.1-1 and Table 5.5.2.12-1 that the values of Table 5.5.8.4-1 shall be used for MCPTT but the values of Table 5.5.8.8-1 for MCVIdeo. - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.5.2	Default SIP message and other information elements

#### 5.5.2.1	SIP ACK

##### 5.5.2.1.1	SIP ACK from the UE

Table 5.5.2.1.1-1: SIP ACK from the UE

| Derivation Path: TS 24.229 [16], clause A.2.1.4.2, A.2.2.4.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] |  |
| Method | "ACK" |  |  |  |
| Request-URI | same URI as the SS has sent earlier in the Contact header of a response within the same dialog |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] |  |
| sent-protocol | "SIP/2.0/UDP" |  |  | UDP |
|  | "SIP/2.0/TCP" |  |  | TCP |
| sent-by | Same value as in INVITE message |  |  |  |
| via-branch | Value starting with 'z9hG4bK' |  |  |  |
| Route |  |  | RFC 3261 [22] |  |
| route-param list | URIs of the Record-Route header sent to the UE in the response which has established the dialog, in reverse order |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | same value as in the INVITE message | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | same value as in the INVITE | Local tag of the dialog ID (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] |  |
| addr-spec | same value as in the INVITE | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | same tag as in the To-header of the response which has established the dialog | Remote tag of the dialog ID (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | same value as in INVITE message |  |  |  |
| Cseq |  |  | RFC 3261 [22] |  |
| value | same value as in INVITE message |  |  |  |
| method | "ACK" |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | any allowed value | Non-zero value |  |  |
| Content-Length | if present |  | RFC 3261 [22] |  |
| value | "0" | No message body included |  |  |

##### 5.5.2.1.2	SIP ACK from the SS

Table 5.5.2.1.2-1: SIP ACK from the SS

| Derivation Path: TS 24.229 [16], clause A.2.1.4.2, A.2.2.4.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] |  |
| Method | "ACK" |  |  |  |
| Request-URI | same URI as the UE has sent earlier in the Contact header of a response within the same dialog | Contact URI of the UE ("callee") |  |  |
|  | same value as in the INVITE |  |  | NON-2XX |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via | same as in the INVITE but with updated via-branches in case of an ACK for 2xx response | see Table 5.5.2.5.2-1 | RFC 3261 [22] |  |
|  | same as in the INVITE (with the same via-branches) |  |  | NON-2XX |
| Route | not present |  | RFC 3261 [22] |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | same URI as in the From-header of the INVITE | remote URI of the dialog (from the UE's point of view) |  |  |
| tag | same tag as in the From-header of the INVITE | remote tag of the dialog (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] |  |
| addr-spec | same URI as in the To-header of the INVITE | local URI of the dialog (from the UE's point of view) |  |  |
| tag | same tag as in the To-header of the response which has established the dialog | local tag of the dialog (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | Same value as in INVITE | Call-Id of the dialog |  |  |
| Cseq |  |  | RFC 3261 [22] |  |
| value | Same value as in INVITE |  |  |  |
| method | "ACK" |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | "68" | The recommended initial value is 70 in RFC 3261. Assuming 2 hops as according to the Via header this results in a value of 68 in the message sent to the UE |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included |  |  |

| Condition | Explanation |
| --- | --- |
| NON-2XX | ACK for non-2xx response |
| NOTE: For further conditions see table 5.5.1-1 |  |

#### 5.5.2.2	SIP BYE

##### 5.5.2.2.1	SIP BYE from the UE

Table 5.5.2.2.1-1: SIP BYE from the UE

| Derivation Path: TS 24.229 [16], clause A.2.1.4.3, A.2.2.4.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] |  |
| Method | "BYE" |  |  |  |
| Request-URI | same URI as the SS has sent earlier in the Contact header of a message within the same dialog | Contact URI of the recipient of the BYE |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] |  |
| sent-protocol | "SIP/2.0/UDP" |  |  | UDP |
|  | "SIP/2.0/TCP" |  |  | TCP |
| sent-by | same value as in INVITE message |  |  |  |
| sent-by |  |  |  | MT_CALL |
| host | IP address or FQDN | Either the UE’s IP address or its home domain name |  |  |
| port | protected server port of the UE | as assigned during registration |  |  |
| via-branch | Value starting with 'z9hG4bK' |  |  |  |
| Route |  |  | RFC 3261 [22] |  |
| route-param list | URIs of the Record-Route header sent to the UE in the response which has established the dialog, in reverse order |  |  |  |
|  | URIs of the Record-Route header sent to the UE in the INVITE |  |  | MT_CALL |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | Same URI of the UE as used earlier in the dialog | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the UE as used earlier in the dialog | Local tag of the dialog ID (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] |  |
| addr-spec | Same URI of the SS as used earlier in the dialogURI | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the SS as used earlier in the dialog | Remote tag of the dialog ID (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | same value as in INVITE message |  |  |  |
| CSeq |  |  | RFC 3261 [22] |  |
| value | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  |  |
| method | "BYE" |  |  |  |
| Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Proxy-Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Security-Verify |  |  | RFC 3329 [53] |  |
| sec-mechanism | same value as Security -Server header sent by SS during registration |  |  |  |
| Max-Forwards |  |  | RFC 3261[22] |  |
| value | any allowed value | Non-zero value |  |  |
| P-Access-Network-Info |  |  | RFC 7315 [52] RFC 7913 [51] |  |
| access-net-spec | Access network technology and, if applicable, the cell ID |  |  |  |
| Content-Length | if present |  | RFC 3261 [22] |  |
| value | "0" | No message body included |  |  |

##### 5.5.2.2.2	SIP BYE from the SS

Table 5.5.2.2.2-1: SIP BYE from the SS

| Derivation Path: TS 24.229 [16], clause A.2.1.4.3, A.2.2.4.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] |  |
| Method | "BYE" |  |  |  |
| Request-URI | same URI as the UE has sent earlier in the Contact header of a response within the same dialog | Contact URI of the UE ("callee") |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via | same as specified for INVITE sent by the SS in Table 5.5.2.5.2- |  | RFC 3261 [22] | MO_CALL |
| Via | same as in INVITE but with updated via-branches |  | RFC 3261 [22] |  |
| Route | Not present |  | RFC 3261 [22] |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | Same URI of the SS as used earlier in the dialog | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the SS as used earlier in the dialog | Remote tag of the dialog (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] |  |
| addr-spec | Same URI of the UE as used earlier in the dialog | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the UE as used earlier in the dialog | Local tag of the dialog (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | same value as in INVITE message |  |  |  |
| CSeq |  |  | RFC 3261 [22] |  |
| value | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  |  |
| method | "BYE" |  |  |  |
| Max-Forwards |  |  | RFC 3261[22] |  |
| value | "68" | The recommended initial value is 70 in RFC 3261. Assuming 2 hops as according to the Via header this results in a value of 68 in the message sent to the UE |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included |  |  |

#### 5.5.2.3	SIP CANCEL

This message is sent by the SS.

Table 5.5.2.3-1: SIP CANCEL

| Derivation Path: TS 24.229 [16], clause A.2.1.4.4, A.2.2.4.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] |  |
| Method | "CANCEL" |  |  |  |
| Request-URI | same value as in the INVITE being cancelled |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] |  |
| via-parm | same value as in the INVITE being cancelled |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | same value as in the INVITE being cancelled |  |  |  |
| tag | same value as in the INVITE being cancelled |  |  |  |
| To |  |  | RFC 3261 [22] |  |
| addr-spec | same value as in the INVITE being cancelled |  |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| Callid | same value as in the INVITE being cancelled |  |  |  |
| CSeq |  |  | RFC 3261 [22] |  |
| value | same value as in the INVITE being cancelled |  |  |  |
| Method | "CANCEL" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included |  |  |

#### 5.5.2.4	SIP INFO

This message is sent by the SS.

Table 5.5.2.4-1: SIP INFO

| Derivation Path: TS 24.229 [16], clause A.2.1.4.6, A.2.2.4.6 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Method | "INFO" |  |  |  |
| Request-URI | px_MCPTT_Client_A_ID |  |  |  |
|  | px_MCVideo_Client_A_ID |  |  | MCVIDEO |
|  | px_MCData_Client_A_ID |  |  | MCDATA |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol | "SIP/2.0/UDP" |  |  |  |
| sent-by | any allowed value | IP address or FQDN and protected server port of the UE |  |  |
| via-branch | any allowed value | Value starting with 'z9hG4bK' |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | px_MCPTT_Client_A_ID |  |  |  |
|  | px_MCVideo_Client_A_ID |  |  | MCVIDEO |
|  | px_MCData_Client_A_ID |  |  | MCDATA |
| tag | "1" |  |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec | tsc_MCPTT_PublicServiceId_A |  |  |  |
|  | tsc_MCVideo_PublicServiceId_A |  |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A |  |  | MCDATA |
| Call-ID |  |  | RFC 3261 [22] |  |
| Callid | same value as in the INVITE |  |  |  |
| CSeq |  |  | RFC 3261 [22] |  |
| value | value of CSeq sent by the SS within its previous request in the same dialog but increased by one |  |  |  |
| Method | "INFO" |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | "70" | The recommended initial value is 70 in RFC 3261. Editor's Note: to be changed to realistic value taking into account number of hops |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | length of message body |  |  |  |
| Message Body | any allowed value |  |  |  |

Editor’s note: Table 5.5.2.4-1 needs to be reviewed

#### 5.5.2.5	SIP INVITE

##### 5.5.2.5.1	SIP INVITE from the UE

Table 5.5.2.5.1-1: SIP INVITE from the UE

| Derivation Path: TS 24.229 [16], clause A.2.1.4.7, A.2.2.4.7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "INVITE" |  |  |  |
| Request-URI | tsc_MCPTT_PublicServiceId_A | The public service identity identifying the participating MCPTT function serving the MCPTT user |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A | The public service identity identifying the participating MCVideo function serving the MCVideo user |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A | The public service identity identifying the participating MCData function serving the MCData user |  | MCDATA |
| Request-URI | same URI as the SS has sent earlier in the Contact header of a message within the same dialog | Contact URI of the recipient of the BYE |  | re_INVITE |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol | "SIP/2.0/UDP" | UE accesses the server via UDP |  | UDP |
|  | "SIP/2.0/TCP" | UE accesses the server via TCP |  | TCP |
| sent-by |  |  |  |  |
| host | IP address or FQDN | Either the UE’s IP address or its home domain name |  |  |
| port | protected server port of the UE | as assigned during registration |  |  |
| via-branch | Value starting with 'z9hG4bK' |  |  |  |
| Route |  |  | RFC 3261 [22] |  |
| addr-spec[1] | SIP URI |  |  |  |
| user-info and host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | protected server port of the SS | as assigned during registration |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[2] | SIP URI |  |  |  |
| user-info and host | "scscf.3gpp.org" | same value as in the Service-Route header field of the 200 OK response to REGISTER |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| Route |  |  | RFC 3261 [22] | re_INVITE |
| route-param list | URIs of the Record-Route header sent to the UE in the response which has established the dialog, in reverse order |  |  |  |
|  | URIs of the Record-Route header sent to the UE in the INVITE |  |  | MT_CALL |
| From |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | Default public user id (px_MCX_SIP_PublicUserId_A_1) |  |  |  |
| port | not present |  |  |  |
| tag | any value |  |  |  |
| From |  |  | RFC 3261 [22] | re_INVITE |
| addr-spec | Same URI of the UE as used earlier in the dialog | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the UE as used earlier in the dialog | Local tag of the dialog ID (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec |  |  |  |  |
| user-info and host | Same URI as Request-URI |  |  |  |
| port | not present |  |  |  |
| tag | not present |  |  |  |
| To |  |  | RFC 3261 [22] | re_INVITE |
| addr-spec | Same URI of the SS as used earlier in the dialogURI | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the SS as used earlier in the dialog | Remote tag of the dialog ID (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | any allowed value |  |  |  |
| callid | same value as in INVITE creating the dialog |  |  | re_INVITE |
| CSeq |  |  | RFC 3261 [22] |  |
| value | any allowed value |  |  |  |
| value | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  | re_INVITE |
| method | "INVITE" |  |  |  |
| Supported |  |  | RFC 3261 [22] |  |
| option-tag | "timer" |  |  |  |
| Session-Expires |  |  | RFC 4028 [30] |  |
| delta-seconds | any allowed value |  |  |  |
| Require |  |  | RFC 3261 [22] RFC 3312 [56] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Proxy-Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Security-Verify |  |  | RFC 3329 [53] |  |
| sec-mechanism | same value as Security -Server header sent by SS during registration |  |  |  |
| Contact |  |  | RFC 3261 [22 RFC 3840 [33] |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | IP address or FQDN |  |  |  |
| port | protected server port of UE | as assigned during registration |  |  |
| feature-param | "+g.3gpp.mcptt" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Push To Talk (MCPTT) communication. |  | MCPTT |
|  | "+g.3gpp.mcvideo" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Video (MCVideo) communication. |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports mission critical data (MCData) service.communication. |  | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports mission critical data (MCData) service.communication. |  | MCDATA_FD |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" | This URN indicates that the device has the capabilities to support the mission critical push to talk (MCPTT) service. |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" | This URN indicates that the device has the capabilities to support the Mission Critical Video (MCVideo) communication. |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" | This URN indicates that the device has the capabilities to support the mission critical data (MCData) service. |  | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" | This URN indicates that the device has the capabilities to support the mission critical data (MCData) service. |  | MCDATA_FD |
| feature-param | "audio" | This feature tag indicates that the device supports audio as a streaming media type. |  | MCPTT OR MCVIDEO |
| feature-param | "video" | This feature tag indicates that the device supports video as a streaming media type. |  | MCVIDEO |
| feature-param | "text" | This feature tag indicates that the device supports text as a streaming media type. |  | MCDATA |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | any allowed value | Non-zero value |  |  |
| P-Access-Network-Info |  |  | RFC 7315 [52] |  |
| access-net-specs | Access network technology and, if applicable, the cell ID | AUTO |  |  |
| Accept |  |  | RFC 3261 [22] |  |
| media-range[1] | "application/sdp” |  |  |  |
| media-range[2] | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | application/vnd.3gpp.mcvideo-info+xml |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| P-Preferred-Service |  |  | RFC 6050 [31] |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA_SDS |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  | MCDATA_FD |
| P-Preferred-Identity | if present |  | RFC 3325 [32] |  |
| PPreferredID-value | same URI as in From-header |  |  |  |
| Accept-Contact |  |  | RFC 3841 [29] |  |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| Priv-Answer-Mode | not present |  |  |  |
| Answer-Mode | not present |  | RFC 5373 [34] | re_INVITE |
| Answer-Mode |  |  | RFC 5373 [34] |  |
| answer-mode-value | "Auto" |  |  |  |
| answer-mode-value | "Manual" |  |  | MANUAL |
| Resource-Priority |  |  | RFC 4412 [40] RFC 7134 [57] RFC 8101 [45] | EMERGENCY-CALL or IMMPERIL-CALL |
| r-value |  |  |  | EMERGENCY-CALL |
| namespace | value of the <resource-priority-namespace> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration documents | As configured in Table 5.5.8.4-1 for MCPTT and in Table 5.5.8.8-1 for MCVIdeo |  |  |
| r-priority | value of the <resource-priority-priority> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration document | As configured in Table 5.5.8.4-1 for MCPTT and in Table 5.5.8.8-1 for MCVIdeo |  |  |
| r-value |  |  |  | IMMPERIL-CALL |
| namespace | value of the <resource-priority-namespace> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration documents | As configured in Table 5.5.8.4-1 for MCPTT and in Table 5.5.8.8-1 for MCVIdeo |  |  |
| r-priority | value of the <resource-priority-priority> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration document | As configured in Table 5.5.8.4-1 for MCPTT and in Table 5.5.8.8-1 for MCVIdeo |  |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| Content-Length | present in case of TCP and when there is a message body (otherwise optional) |  | RFC 3261 [22] |  |
| value | any value | length of message-body |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/sdp" |  | RFC 4566 [27] |  |
| MIME-part-body | SDP Message as described in Table 5.5.3.1.1-1 |  |  | MCPTT |
|  | SDP Message as described in Table 5.5.3.1.1-2 |  |  | MCVIDEO |
|  | SDP Message as described in Table 5.5.3.1.1-3 |  |  | MCDATA |
| MIME body part |  | MCPTT Info/MCVideo/MCData |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 | MCPTT |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  | TS 24.282 [87] clause D.1 | MCDATA |
| MIME body part |  | Resource list | RFC 5366 [35] | PRIVATE-CALL OR MCD_1to1 |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/resource-lists+xml" |  |  |  |
| Content-ID | any value | Unique URL identifying the Resource-lists XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | As described in Table 5.5.3.3.1-1 |  |  | MCPTT |
|  | As described in Table 5.5.3.3.1-2 |  |  | MCVIDEO |
|  | As described in Table 5.5.3.3.1-3 |  |  | MCDATA |
| MIME body part |  | Location info |  | (EMERGENCY-CALL AND ALERT_IND) OR LOCATION-INFO |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" | This MIME part shall be included if the MCPTT-Info 'alert-ind' element sent in the MCPTT-Info is set to true. |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-location-info+xml" | This MIME part shall be included if the MCVideo-Info 'alert-ind' element sent in the MCVideo-Info is set to true. |  | MCVIDEO |
| Content-ID | any value | Unique URL identifying the Location-info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Location-info as described in Table 5.5.3.4.1-1 |  | TS 24.379 [9] clause F.3 | MCPTT |
|  | Location-info as described in Table 5.5.3.4.1-2 |  | TS 24.281 [86] clause F.3 | MCVIDEO |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| MANUAL | Call establishment with manual commencement mode |
| MCD_1to1 | A one-to-one MCData call |
| MCDATA_SDS | SDS message or SDS disposition notification |
| MCDATA_FD | FD message or FD disposition notification |
| re_INVITE | INVITE within a dialog |
| ALERT_IND | MCPTT emergency alert is required as specified for the test case or automatically initiated by the client for an emergency call (in case of condition EMERGENCY-CALL when pc_MCX_EmergencyIndWithAlertInd=true); <alert-ind> is set to true in the mcptt-info. |
| NOTE: For further conditions see table 5.5.1-1 |  |

##### 5.5.2.5.2	SIP INVITE from the SS

Table 5.5.2.5.2-1: SIP INVITE from the SS

| Derivation Path: TS 24.229 [16], clause A.2.1.4.7, A.2.2.4.7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "INVITE" |  |  |  |
| Request-URI | SIP URI of the UE’s contact address as provided in the Contact-header of the REGISTER message |  |  |  |
| Request-URI | same URI as the UE has sent earlier in the Contact header of a response within the same dialog | Contact URI of the UE |  | re_INVITE |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol[1] | "SIP/2.0/TCP" |  |  |  |
| sent-by[1] |  | Address of the P-CSCF that communicates with the called party |  |  |
| host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | protected server port of the SS | as assigned during registration |  |  |
| via-branch[1] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| sent-protocol[2] | "SIP/2.0/UDP" |  |  |  |
| sent-by[2] |  | Address of the other endpoint (the caller) |  |  |
| host | Host name of the SIP URI being used in the From header |  |  |  |
| port | Same port number as in Contact-header | Caller’s port number |  |  |
| via-branch[2] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| Record-Route |  | Record-Route corresponding to the Via header | RFC 3261 [22] |  |
| addr-spec[1] | SIP URI | SIP URI corresponding to first entry of Via header |  |  |
| user-info and host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | protected server port of the SS | as assigned during registration |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[2] | SIP URI |  |  |  |
| user-info and host | “term@scscf1.3gpp.org” |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[3] | SIP URI |  |  |  |
| user-info and host | “orig@scscf2.3gpp.org” |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[4] | SIP URI |  |  |  |
| user-info and host | “pcscf2.3gpp.org” |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| Record-Route | same as in the 180, 183 or 200 response sent to the UE during MO call establishment in reverse order |  | RFC 3261 [22] | re_INVITE AND MO_CALL |
| From |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_PublicServiceId_A | SIP URI of the calling UE |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A | SIP URI of the calling UE |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A | SIP URI of the calling UE |  | MCDATA |
| port | not present |  |  |  |
| tag | Value assigned by the SS |  |  |  |
| From |  |  | RFC 3261 [22] | re_INVITE |
| addr-spec | Same URI of the SS as used earlier in the dialog | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the SS as used earlier in the dialog | Remote tag of the dialog (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec |  |  |  |  |
| user-info and host | px_MCX_SIP_PublicUserId_A_1 | Default public user ID (IMPU) as stored in the UICC |  |  |
| port | not present |  |  |  |
| tag | not present |  |  |  |
| To |  |  | RFC 3261 [22] | re_INVITE |
| addr-spec | Same URI of the UE as used earlier in the dialog | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the UE as used earlier in the dialog | Local tag of the dialog (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | Value assigned by the SS |  |  |  |
| Call-ID |  |  | RFC 3261 [22] | re_INVITE |
| callid | same value as in INVITE creating the dialog |  |  |  |
| CSeq |  |  | RFC 3261 [22] |  |
| value | Value assigned by the SS |  |  |  |
| value | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  | re_INVITE |
| method | "INVITE" |  |  |  |
| Supported |  |  | RFC 3261 [22] |  |
| option-tag | "100rel" | This option tag indicates that the UA can send or receive reliable provisional responses. |  |  |
| option-tag | "timer" |  |  |  |
| option-tag | "tdialog" |  |  |  |
| option-tag | "norefersub" |  |  |  |
| P-Called-Party-ID |  |  | RFC 7315 [52] |  |
| called-pty-id-spec | Same public user ID as used in the To-header |  |  |  |
| Session-Expires |  |  | RFC 4028 [30] |  |
| generic-param | "1800" | The recommended initial value is 1800 in RFC 4028 [30]. |  |  |
| P-Early-Media |  |  | RFC 5009 [60] |  |
| em-parm | "inactive" |  |  |  |
| Require |  |  | RFC 3261 [22] RFC 3312 [56] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Proxy-Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| P-Asserted-Identity |  |  | RFC 3325 [32] |  |
| addr-spec |  |  |  |  |
| user-info and host | same URI as in From-header |  |  |  |
| port | not present |  |  |  |
| Contact |  |  | RFC 3261 [22] RFC 3840 [33] |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | tsc_MCPTT_SessionId |  |  | MCPTT |
|  | tsc_MCVideo_SessionId |  |  | MCVIDEO |
|  | tsc_MCData_SessionId |  |  | MCDATA |
| port | Value assigned by the SS |  |  |  |
| feature-param | "+g.3gpp.mcptt" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Push To Talk (MCPTT) communication. | RFC 3840 [33] clause 9 | MCPTT |
|  | "+g.3gpp.mcvideo" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Video (MCVideo) communication. | RFC 3840 [33] clause 9 | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Data (MCData) communication. | RFC 3840 [33] clause 9 | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Data (MCData) communication. | RFC 3840 [33] clause 9 | MCDATA_FD |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" | This URN indicates that the device has the capabilities to support the mission critical push to talk (MCPTT) service. | RFC 3840 [33] clause 9 | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" | This URN indicates that the device has the capabilities to support the mission critical video (MCVideo) service. | RFC 3840 [33] clause 9 | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" | This URN indicates that the device has the capabilities to support the mission critical data (MCData) SDS service. | RFC 3840 [33] clause 9 | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" | This URN indicates that the device has the capabilities to support the mission critical data (MCData) FD service. | RFC 3840 [33] clause 9 | MCDATA_FD |
| feature-param | "audio" | This feature tag indicates that the device supports audio as a streaming media type. | RFC 3840 [33] clause 10.1 | MCPTT OR MCVIDEO |
| feature-param | "video" | This feature tag indicates that the device supports video as a streaming media type. |  | MCVIDEO |
| feature-param | "text" | This feature tag indicates that the device supports text as a streaming media type. |  | MCDATA |
| feature-param | "isfocus" |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | "68" | The recommended initial value is 70 in RFC 3261 [22]. Assuming 2 hops as according to the Via header this results in a value of 68 in the message sent to the UE |  |  |
| Accept |  |  | RFC 3261 [22] |  |
| media-range[1] | "application/sdp " |  |  |  |
| media-range[2] | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Accept-Contact |  |  | RFC 3841 [29] |  |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| Answer-Mode | not present |  | RFC 5373 [34] TS 24.379 [9] clause 6.3.2.2.6.3 | re-INVITE OR FIRST-TO-ANSWER |
| Answer-Mode |  |  | RFC 5373 [34] |  |
| answer-mode-value | "Auto" |  |  |  |
| answer-mode-value | "Manual" |  |  | MANUAL |
| Priv-Answer-Mode |  |  |  | FIRST-TO-ANSWER |
| answer-mode-value | "Manual" |  |  |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| Value | length of message-body |  |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/sdp" |  |  |  |
| MIME-part-body | SDP Message as described in Table 5.5.3.1.2-1 |  | RFC 4566 [27] | MCPTT |
|  | SDP Message as described in Table 5.5.3.1.2-2 |  | RFC 4566 [27] | MCVIDEO |
|  | SDP Message as described in Table 5.5.3.1.2-3 |  | RFC 4566 [27] | MCDATA |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.2-1 |  |  | MCPTT |
|  | MCVideo-Info as described in Table 5.5.3.2.2-2 |  |  | MCVIDEO |
|  | As described in Table 5.5.3.2.2-3 |  |  | MCDATA |
| MIME body part |  | Location info |  | LOCATION-INFO |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-location-info+xml" |  |  | MCVIDEO |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the Location-info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Location-info as described in Table 5.5.3.4.2-1 |  | TS 24.379 [9] clause F.3 | MCPTT |
|  | Location-info as described in Table 5.5.3.4.2-2 |  | TS 24.281 [86] clause F.3 | MCVIDEO |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-2 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| MANUAL | Call establishment with manual commencement mode |
| re_INVITE | INVITE within a dialog |
| MCD_1to1 | A one-to-one MCData call |
| MCDATA_SDS | SDS message or SDS disposition notification |
| MCDATA_FD | FD message or FD disposition notification |
| For further conditions see table 5.5.1-1 |  |

#### 5.5.2.6	Void

#### 5.5.2.7	SIP MESSAGE

##### 5.5.2.7.1	SIP MESSAGE from the UE

Table 5.5.2.7.1-1: SIP MESSAGE from the UE

| Derivation Path: TS 24.229 [16], clause A.2.1.4.7a, A.2.2.4.7a |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "MESSAGE" |  |  |  |
| Request-URI | tsc_MCPTT_PublicServiceId_A | The public service identity identifying the originating participating MCPTT function serving the MCPTT user |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A | The public service identity identifying the originating participating MCVideo function serving the MCVideo user |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A | The public service identity identifying the originating participating MCData function serving the MCData user |  | MCDATA |
|  | same URI as provided in the Asserted-Identity header field of the SIP MESSAGE for location reporting configuration |  |  | LOCATION_REPORT |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol | "SIP/2.0/UDP" |  |  | UDP |
|  | "SIP/2.0/TCP" |  |  | TCP |
| sent-by |  |  |  |  |
| host | IP address or FQDN | Either the UE’s IP address or its home domain name |  |  |
| port | protected server port of the UE | as assigned during registration |  |  |
| via-branch | Value starting with 'z9hG4bK' |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | Default public user id (px_MCX_SIP_PublicUserId_A_1) | The URI of the UE |  |  |
| port | not present |  |  |  |
| tag | any allowed value |  |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_PublicServiceId_A | The URI of the SS |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A | The URI of the SS |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A | The URI of the SS |  | MCDATA |
| port | not present |  |  |  |
| tag | not present |  |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | any allowed value |  |  |  |
| Cseq |  |  | RFC 3261 [22] |  |
| value | any allowed value |  |  |  |
| method | "MESSAGE" |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | any allowed value | Non-zero value |  |  |
| P-Access-Network-Info |  |  | RFC 7315 [52] |  |
| access-net-spec | Access network technology and, if applicable, the cell ID |  |  |  |
| Route | same as specified for INVITE sent by the UE in Table 5.5.2.5.1-1 |  | RFC 3261 [22] |  |
| Accept-Contact |  |  | RFC 3841 [29] |  |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata" |  |  | MCDATA |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  | MCDATA_SDS, MCDATA_FD |
| feature-param | "+g.3gpp.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Service |  |  | RFC 6050 [31] |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata" |  |  | MCDATA |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA_SDS |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  | MCDATA_FD |
| P-Preferred-Identity | if present |  | RFC 3325 [32] |  |
| PPreferredID-value | same URI as in From-header |  |  |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| Content-Length | present in case of TCP and when there is a message body (otherwise optional) |  | RFC 3261 [22] |  |
| value | any value | length of message-body |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 | MCPTT |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  |  | MCDATA |
| MIME body part |  | Affiliation-Command |  | AFFILIATION |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp. mcvideo-affiliation-command+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp. mcdata-affiliation-command+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the affiliation-command XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Affiliation-Command as described in Table 5.5.3.7-1 |  | TS 24.379 [9] clause F.4 | MCPTT |
|  | MCVideo-Affiliation-Command as described in Table 5.5.3.7-2 |  | TS 24.281 [86] clause F.4 | MCVIDEO |
|  | MCData-Affiliation-Command as described in Table 5.5.3.7-3 |  | TS 24.282 [87] clause D.3 | MCDATA |
| MIME body part |  | Resource lists | RFC 5366 [35] | RESOURCE_LISTS |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/resource-lists+xml" |  |  |  |
| Content-ID | any value | Unique URL identifying the Resource-lists XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Resource-lists as described in Table 5.5.3.3.1-1 |  |  | MCPTT |
|  | Resource-lists as described in Table 5.5.3.3.1-2 |  |  | MCVIDEO |
|  | Resource-lists as described in Table 5.5.3.3.1-3 |  |  | MCDATA |
| MIME body part |  | Location info | TS 24.379 [9] clause F.3 | LOCATION-INFO, LOCATION_REPORT |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" | This MIME part shall be included if the MCPTT-Info 'alert-ind' element sent in the MCPTT-Info is set to true. |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-location-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-location-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the Location-info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Location-info as described in Table 5.5.3.4.1-1 |  |  | MCPTT |
|  | Location-info as described in Table 5.5.3.4.1-2 |  |  | MCVIDEO |
|  | Location-info as described in Table 5.5.3.4.1-3 |  |  | MCDATA |
| MIME body part |  | MIKEY message |  | MIKEY |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/mikey" |  |  |  |
| MIME-part-body | As described in Table 5.5.9.1-2A | MIKEY message, containing the PSK | TS 33.180 [30] TS 24.282 [87] |  |
| MIME body part |  | MCData Data signalling message |  | MCDATA_SIGNALLING |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| MIME-part-body | SIGNALLING_PAYLOAD as described in Table 5.5.3.8.1-1 |  | TS 24.282 [87] |  |
| MIME body part |  | MCData Data message |  | MCDATA_PAYLOAD |
| MIME-part-headers |  |  |  |  |
| Content-Type | application/vnd.3gpp.mcdata-payload |  |  |  |
| MIME-part-body | DATA_PAYLOAD as described in Table 5.5.3.9.1-1 |  | TS 24.282 [87] |  |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| RESOURCE_LISTS | Message-body contains Resource lists |
| LOCATION_REPORT | Message-body contains location information report according to TS 24.379 [2] clause 13.3.4.2 |
| MIKEY | Message-body contains MIKEY message (e.g. for MCData 1-to-1 communication) |
| MCDATA_SIGNALLING | Message-body contains MCData Data signalling message |
| MCDATA_PAYLOAD | Message-body contains MCData Data message (DATA PAYLOAD) |
| MCDATA_SDS | SDS message or SDS disposition notification |
| MCDATA_FD | FD message or FD disposition notification |
| For further conditions see table 5.5.1-1 |  |

##### 5.5.2.7.2	SIP MESSAGE from the SS

Table 5.5.2.7.2-1: SIP MESSAGE from the SS

| Derivation Path: TS 24.229 [16], clause A.2.1.4.7a, A.2.2.4.7a |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "MESSAGE" |  |  |  |
| Request-URI | Public user id associated to the MC service id | px_MCX_SIP_PublicUserId_A_1 (in general) |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol[1] | "SIP/2.0/TCP" |  |  |  |
| sent-by[1] |  | Address of the P-CSCF that communicates with the called party |  |  |
| ….host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | protected server port of the SS | as assigned during registration |  |  |
| via-branch[1] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| sent-protocol[2] | "SIP/2.0/UDP" |  |  |  |
| sent-by[2] |  |  |  |  |
| ….host | “scscf.3gpp.org“ |  |  |  |
| port | Value assigned by the SS | Caller’s port number |  |  |
| via-branch[2] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| sent-protocol[3] | "SIP/2.0/UDP" |  |  |  |
| sent-by[3] |  |  |  |  |
| host | host name of the MC server |  |  |  |
| port | not present |  |  |  |
| via-branch[3] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_PublicServiceId_A |  |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A |  |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A |  |  | MCDATA |
| port | not present |  |  |  |
| tag | Value assigned by the SS |  |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec |  |  |  |  |
| user-info and host | same URI as used as Request URI |  |  |  |
| port | not present |  |  |  |
| tag | not present |  |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | Value assigned by the SS |  |  |  |
| Cseq |  |  | RFC 3261 [22] |  |
| value | Value assigned by the SS |  |  |  |
| method | "MESSAGE" |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | "67" | The recommended initial value is 70 in RFC 3261. Assuming 3 hops as according to the Via header this results in a value of 67 in the message sent to the UE |  |  |
| P-Asserted-Service |  |  | RFC 6050 [31] | MCDATA_SDS, MCDATA_FD |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA_SDS |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  | MCDATA_FD |
| P-Asserted-Service |  |  | RFC 6050 [31] | AFFILIATION, LOCATION-CONFIG |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata" |  |  | MCDATA |
| Accept-Contact |  |  | RFC 3841 [29] |  |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata" |  |  | MCDATA |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  | ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.mcdata" |  |  | MCDATA |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  | MCDATA_SDS, MCDATA_FD |
| feature-param | "+g.3gpp.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Asserted-Identity |  |  | RFC 3325 [32] | MCDATA_SDS, MCDATA_FD |
| name-addr | px_MCX_SIP_PublicUserId_B | The public user identity of the originating MCData User |  |  |
| P-Asserted-Identity |  |  | RFC 3325 [32] | LOCATION-CONFIG |
| name-addr | tsc_MCPTT_PublicServiceId_PF_A | URI of the participating MCPTT function which configures the location reporting at the UE |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_PF_A | URI of the participating MCVideo function which configures the location reporting at the UE |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_PF_A | URI of the participating MCData function which configures the location reporting at the UE |  | MCDATA |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | length of message-body |  |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.2-1 |  | TS 24.379 [9] clause F.1 | MCPTT |
|  | MCVideo-Info as described in Table 5.5.3.2.2-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.2-3 |  | TS 24.282 [87] clause D.1.2 | MCDATA |
| MIME body part |  | Affiliation-Command |  | AFFILIATION |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-affiliation-command+xml" |  |  | MCVIDEO |
|  | "vnd.3gpp.mcdata-affiliation-command+xml" |  |  | MCDATA |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the affiliation-command XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Affiliation-Command as described in Table 5.5.3.7-1 |  | TS 24.379 [9] clause F.4 | MCPTT |
|  | MCVideo-Affiliation-Command as described in Table 5.5.3.7-2 |  | TS 24.281 [86] clause F.4 | MCVIDEO |
|  | MCData-Affiliation-Command as described in Table 5.5.3.7-3 |  | TS 24.282 [87] clause D.3 | MCDATA |
| MIME body part |  | Resource lists | RFC 5366 [35] | RESOURCE_LISTS |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/resource-lists+xml" |  |  |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the Resource-lists XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Resource-lists as described in Table 5.5.3.3.2-1 |  |  | MCPTT |
|  | Resource-lists as described in Table 5.5.3.3.2-2 |  |  | MCVIDEO |
|  | Resource-lists as described in Table 5.5.3.3.2-3 |  |  | MCDATA |
| MIME body part |  | Location info |  | LOCATION-INFO, LOCATION_CONFIG |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-location-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-location-info+xml" |  |  | MCDATA |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the Location-info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Location-info as described in Table 5.5.3.4.2-1 |  | TS 24.379 [9] clause F.3 | MCPTT |
|  | Location-info as described in Table 5.5.3.4.2-2 |  | TS 24.281 [86] clause F.3 | MCVIDEO |
|  | Location-info as described in Table 5.5.3.4.2-3 |  | TS 24.282 [87] clause D.3 | MCDATA |
| MIME body part |  | MIKEY message |  | MIKEY |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/mikey" |  |  |  |
| MIME-part-body | As described in Table 5.5.9.1-2 | MIKEY message, containing the PSK | TS 33.180 [30] TS 24.282 [87] |  |
| MIME body part |  | MCData Data signalling message |  | MCDATA_SIGNALLING |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| MIME-part-body | SIGNALLING PAYLOAD as described in Table 5.5.3.8.2-1 |  | TS 24.282 [87] |  |
| MIME body part |  | MCData Data message |  | MCDATA_PAYLOAD |
| MIME-part-headers |  |  |  |  |
| Content-Type | application/vnd.3gpp.mcdata-payload |  |  |  |
| MIME-part-body | DATA_PAYLOAD as described in Table 5.5.3.9.1-2 |  | TS 24.282 [87] |  |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-2 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| RESOURCE_LISTS | Message-body contains Resource lists |
| LOCATION_CONFIG | Message-body contains location reporting configuration according to TS 24.379 [2] clause 13.2.2 |
| MIKEY | Message-body contains MIKEY message (e.g. for MCData 1-to-1 communication) |
| MCDATA_SIGNALLING | Message-body contains MCData Data signalling message |
| MCDATA_PAYLOAD | Message-body contains MCData Data message (DATA PAYLOAD) |
| MCDATA_SDS | SDS message or SDS disposition notification |
| MCDATA_FD | FD message or FD disposition notification |
| ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG | Accept-Contact header field contains media feature tag ("+g.3gpp.mcptt", "+g.3gpp.mcvideo" or "+g.3gpp.mcdata") |
| For further conditions see table 5.5.1-1 |  |

#### 5.5.2.8	SIP NOTIFY

This message is sent by the SS.

Table 5.5.2.8-1: SIP NOTIFY

| Derivation Path: TS 24.229 [16] clause A.2.1.4.8, A2.2.4.8 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] |  |
| Method | "NOTIFY" |  |  |  |
| Request-URI | same URI as the UE has provided earlier in the Contact header of the SUBSCRIBE |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] |  |
| sent-protocol[1] | "SIP/2.0/TCP" |  |  |  |
| sent-by[1] |  |  |  |  |
| host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | protected server port of the SS |  |  |  |
| via-branch[1] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| sent-protocol[2] | "SIP/2.0/UDP" |  |  |  |
| sent-by[2] |  |  |  |  |
| host | “scscf.3gpp.org“ |  |  |  |
| port | not present |  |  |  |
| via-branch[2] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| sent-protocol[3] | "SIP/2.0/UDP" |  |  |  |
| sent-by[3] |  |  |  |  |
| host | host name of the MC server |  |  |  |
|  | tsc_MCX_CMS_Hostname |  |  | CONFIG |
|  | tsc_MCX_GMS_Hostname |  |  | GROUPCONFIG |
| port | not present |  |  |  |
| via-branch[3] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | same URI as received in the To header of the SUBSCRIBE message | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | same tag as in the To-header of the response which has established the dialog | Remote tag of the dialog (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] |  |
| addr-spec | same URI as received in the From header of the SUBSCRIBE message | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | same value as received in From tag of the SUBSCRIBE message | Local tag of the dialog (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | same as value received in SUBSCRIBE message |  |  |  |
| Cseq |  |  | RFC 3261 [22] |  |
| value | value of CSeq sent by the SS within its previous request in the same dialog but increased by one |  |  |  |
| method | "NOTIFY" |  |  |  |
| Contact |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | Same URI as used as Contact-URI in the 200 (OK) for the SUBSCRIBE message |  |  |  |
| port | not present |  |  |  |
| Event |  |  | RFC 6665 [39] RFC 3842 [61] |  |
| event-type | "presence" |  |  | PRESENCE-EVENT |
|  | "xcap-diff" |  |  | CONFIG. GROUPCONFIG |
|  | "poc-settings" |  |  | POC-SETTINGS-EVENT |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | "67" | The recommended initial value is 70 in RFC 3261. Assuming 3 hops as according to the Via header this results in a value of 67 in the message sent to the UE |  |  |
| Subscription-State |  |  | RFC 6665 [39] |  |
| substate-value | "active" |  |  |  |
| expires | "7200" |  |  |  |
| Content-Type |  |  | RFC 3261 [22] RFC 3842 [61] |  |
| media-type | “multipart/mixed" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | length of message-body |  |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | PIDF |  | PRESENCE-EVENT |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/pidf+xml" |  |  |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the PIDF XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | PIDF as described in Table 5.5.3.5.2-1 |  | TS 24.379 [9] clause 9.3.1 | MCPTT |
|  | PIDF as described in Table 5.5.3.5.2-2 |  | TS 24.281 [86] clause 8.3.1 | MCVIDEO |
|  | PIDF as described in Table 5.5.3.5.2-3 |  | TS 24.282 [87] clause 8.4.1 | MCDATA |
| MIME body part |  | xcap-diff |  | CONFIG, GROUPCONFIG |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/xcap-diff+xml" |  |  |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the xcap-diff XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | xcap-diff document as described in Table 5.5.3.12-1 |  |  | CONFIG |
|  | xcap-diff document as described in Table 5.5.3.12-2 |  |  | GROUPCONFIG |
| MIME body part |  | PoC-Settings |  | POC-SETTINGS-EVENT |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/poc-settings+xml" |  | RFC 4354 [103] |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the PoC-Settings XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | PoC-Settings document as described in Table 5.5.3.11.2-1 |  |  |  |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-2 |  | TS 24.379 [9] |  |

#### 5.5.2.9	SIP OPTIONS

Editor’s note: It shall be specified who is sending the message.

Table 5.5.2.9-1: SIP OPTIONS

| Derivation Path: TS 24.229 [16] clause A.2.1.4.9, A2.2.4.9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Method | "OPTIONS" |  |  |  |
| Request-Disposition | px_MCPTT_Client_A_ID |  |  |  |
|  | px_MCVideo_Client_A_ID |  |  | MCVIDEO |
|  | px_MCData_Client_A_ID |  |  | MCDATA |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol | "SIP/2.0/UDP" |  |  |  |
| sent-by | any allowed value | IP address or FQDN and protected server port of the UE |  |  |
| via-branch | any allowed value | Value starting with 'z9hG4bK' |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | px_MCPTT_Client_A_ID |  |  |  |
|  | px_MCVideo_Client_A_ID |  |  | MCVIDEO |
|  | px_MCData_Client_A_ID |  |  | MCDATA |
| tag | "1" |  |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec | tsc_MCPTT_PublicServiceId_A |  |  |  |
|  | tsc_MCVideo_PublicServiceId_A |  |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A |  |  | MCDATA |
| Call-ID |  |  | RFC 3261 [22] |  |
| Callid | same value as in the INVITE |  |  |  |
| CSeq |  |  | RFC 3261 [22] |  |
| value | value of CSeq sent by the SS within its previous request in the same dialog but increased by one |  |  |  |
| Method | "INFO" |  |  |  |
| Contact |  |  | RFC 3261 [22 RFC 3840 [33] |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | IP address or FQDN (px_MCPTT_Client_A_ID) |  |  |  |
|  | IP address or FQDN (px_MCVideo_Client_A_ID) |  |  | MCVIDEO |
|  | IP address or FQDN (px_MCData_Client_A_ID) |  |  | MCDATA |
| feature-param | "+g.3gpp.mcptt" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Push To Talk (MCPTT) communication. |  |  |
|  | "+g.3gpp.mcvideo" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Video (MCVideo) communication. |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Data (MCData) communication. |  | MCDATA |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" | This URN indicates that the device has the capabilities to support the mission critical push to talk (MCPTT) service. |  |  |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" | This URN indicates that the device has the capabilities to support the mission critical video (MCVideo) service. |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" | This URN indicates that the device has the capabilities to support the mission critical data (MCData) service. |  | MCDATA |
| feature-param | "audio" | This feature tag indicates that the device supports audio as a streaming media type. |  | MCPTT OR MCVIDEO |
| feature-param | "video" | This feature tag indicates that the device supports video as a streaming media type. |  | MCVIDEO |
| feature-param | "text" | This feature tag indicates that the device supports text as a streaming media type. |  | MCDATA |
| Accept |  |  |  |  |
| media-range | "application/sdp" |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | any allowed value | Non-zero value |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included - end of SIP message |  |  |

Editor’s note: Table 5.5.2.9-1 needs to be reviewed

#### 5.5.2.10	SIP PRACK

##### 5.5.2.10.1	SIP PRACK from the UE

Table 5.5.2.10.1-1: SIP PRACK from the UE

| Derivation Path: TS 24.229 [16] clause A.2.1.4.10, A2.2.4.10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  | RFC 3261 [22] |  |
| Method | "PRACK" |  |  |  |
| Request-URI | same URI as the SS has sent earlier in the Contact header of a response within the same dialog |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] |  |
| sent-protocol | "SIP/2.0/UDP" |  |  | UDP |
|  | "SIP/2.0/TCP" |  |  | TCP |
| sent-by | same value as in INVITE message |  |  |  |
| via-branch | Value starting with 'z9hG4bK' |  |  |  |
| Route |  |  | RFC 3261 [22] |  |
| route-param list | URIs of the Record-Route header sent to the UE in the response which has established the dialog, in reverse order |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | same value as in the INVITE message | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | same value as in the INVITE | Local tag of the dialog ID (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] |  |
| addr-spec | same value as in the INVITE | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | same tag as in the To-header of the response which has established the dialog | Remote tag of the dialog ID (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | same value as in INVITE message |  |  |  |
| CSeq |  |  | RFC 3261 [22] |  |
| value | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  |  |
| method | "PRACK" |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | any allowed value | Non-zero value |  |  |
| RAck |  |  | RFC 3261 [22] |  |
| response-num | same value as in RSeq header of the reliable response |  |  |  |
| cseq-num | same value as in CSeq of reliable response |  |  |  |
| method | same value as in CSeq of reliable response |  |  |  |
| P-Access-Network-Info |  |  | RFC 7315 [52] |  |
| access-net-spec | Access network technology and, if applicable, the cell ID |  |  |  |
| Content-Length | if present |  | RFC 3261 [22] |  |
| value | "0" | No message body included |  |  |

##### 5.5.2.10.2	SIP PRACK from the SS

Table 5.5.2.10.2-1: SIP PRACK from the SS

| Derivation Path: TS 24.229 [16] clause A.2.1.4.10, A2.2.4.10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  | RFC 3261 [22] |  |
| Method | "PRACK" |  |  |  |
| Request-URI | same URI as the UE has sent earlier in the Contact header of a response within the same dialog | Contact URI of the UE ("callee") |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via | same as in the INVITE but with updated via-branches | see Table 5.5.2.5.2-1 | RFC 3261 [22] |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | same URI as in the From-header of the INVITE | remote URI of the dialog (from the UE's point of view) |  |  |
| tag | same tag as in the From-header of the INVITE | remote tag of the dialog (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] |  |
| addr-spec | same URI as in the To-header of the INVITE | local URI of the dialog (from the UE's point of view) |  |  |
| tag | same tag as in the To-header of the response which has established the dialog | local tag of the dialog (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | Same value as in INVITE | Call-Id of the dialog |  |  |
| CSeq |  |  | RFC 3261 [22] |  |
| value | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  |  |
| method | "PRACK" |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | "68" | The recommended initial value is 70 in RFC 3261. Assuming 2 hops as according to the Via header this results in a value of 68 in the message sent to the UE |  |  |
| RAck |  |  | RFC 3261 [22] |  |
| response-num | same value as in RSeq header of the reliable response |  |  |  |
| cseq-num | same value as in CSeq of reliable response |  |  |  |
| method | same value as in CSeq of reliable response |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included |  |  |

#### 5.5.2.11	SIP PUBLISH

This message is sent by the UE.

Table 5.5.2.11-1: SIP PUBLISH

| Derivation Path: TS 24.229 [16] clause A.2.1.4.10A, A.2.2.4.10A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "PUBLISH" |  |  |  |
| Request-URI | tsc_MCPTT_PublicServiceId_A | The public service identity identifying the originating participating MCPTT function serving the MCPTT user |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A | The public service identity identifying the originating participating MCVideo function serving the MCVideo user |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A | The public service identity identifying the originating participating MCData function serving the MCData user |  | MCDATA |
| SIP-Version | "SIP/2.0" |  |  |  |
| Route |  |  | RFC 3261 [22] |  |
| addr-spec[1] | SIP URI |  |  |  |
| user-info and host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | protected server port of the SS | as assigned during registration |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[2] | SIP URI |  |  |  |
| user-info and host | "scscf.3gpp.org" |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol | "SIP/2.0/UDP" |  |  | UDP |
|  | "SIP/2.0/TCP" |  |  | TCP |
| sent-by |  |  |  |  |
| user-info and host | IP address or FQDN | Either the UE’s IP address or its home domain name |  |  |
| port | protected server port of the UE | as assigned during registration |  |  |
| via-branch | Value starting with 'z9hG4bK' |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | Default public user id (px_MCX_SIP_PublicUserId_A_1) |  |  |  |
| port | not present |  |  |  |
| tag | any value |  |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec |  |  |  |  |
| user-info and host | same URI as used as Request URI |  |  |  |
| port | not present |  |  |  |
| tag | not present |  |  |  |
| Expires |  |  | RFC 3261 [22] RFC 3903 [43] |  |
| delta-seconds | "4294967295" |  |  |  |
| Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Proxy-Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Security-Verify |  |  | RFC 3329 [53] |  |
| sec-mechanism | same value as Security -Server header sent by SS during registration |  |  |  |
| Cseq |  |  | RFC 3261 [22] |  |
| value | any allowed value |  |  |  |
| method | "PUBLISH" |  |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | any allowed value |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | any allowed value |  |  |  |
| P-Access-Network-Info |  |  | RFC 7315 [52] RFC 7913 [51] |  |
| access-net-spec | Access network technology and, if applicable, the cell ID |  |  |  |
| Event |  |  | RFC 3903 [43] |  |
| event-type | "presence" |  |  | PRESENCE-EVENT |
|  | "poc-settings" |  |  | CONFIG OR POC-SETTINGS-EVENT |
| P-Preferred-Service |  |  | RFC 6050 [31] |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  | TS 24.379 [9] clause 7.2.1A | MCPTT |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  | TS 24.281 [86] clause 7.2.1A | MCVIDEO |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata" |  | TS 24.282 [87] clause 7.2.1A | MCDATA |
| Accept |  |  | RFC 3261 [22] | PRESENCE-EVENT |
| media-range | "application/pidf+xml" |  |  |  |
| port | not present |  |  |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| Content-Length | present in case of TCP and when there is a message body (otherwise optional)length of message-body |  | RFC 3261 [22] |  |
| value | any value |  |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 | MCPTT |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  | TS 24.282 [87] clause D.1 | MCDATA |
| MIME body part |  | PIDF |  | PRESENCE-EVENT |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/pidf+xml" |  |  |  |
| MIME-part-body | PIDF as described in Table 5.5.3.5.1-1 |  | TS 24.379 [9] clause 9.3.1 | MCPTT |
|  | PIDF as described in Table 5.5.3.5.1-2 |  | TS 24.281 [86] clause 8.3.1 | MCVIDEO |
|  | PIDF as described in Table 5.5.3.5.1-3 |  | TS 24.282 [87] clause 8.3.1 | MCDATA |
| MIME body part |  | MIKEY |  | CONFIG |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/mikey" |  | RFC 3830 [24] |  |
| MIME-part-body | MIKEY message as described in Table 5.5.9.1-1 | MIKEY message, containing the CSK | TS 33.180 [94] |  |
| MIME body part |  | PoC-Settings |  | CONFIG OR POC-SETTINGS-EVENT |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/poc-settings+xml" |  | RFC 4354 [103] |  |
| Content-ID | any value | Unique URL identifying the PoC-settings XML MIME body; used as reference in the signature MIME body |  |  |
| MIME-part-body | PoC Settings as described in Table 5.5.3.11.1-1 |  | TS 24.379 [9] |  |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

#### 5.5.2.12	SIP REFER

This message is sent by the UE outside of a dialog.

Table 5.5.2.12-1: SIP REFER

| Derivation Path: TS 24.229 [16] clause A.2.1.4.11, A.2.2.4.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| Request-Line |  |  |  |  |  | RFC 3261 [22] RFC 5031 [54] |  |  |  |
| Method |  | "REFER" |  |  |  |  |  |  |  |
| Request-URI |  | tsc_MCX_SessionID_B |  | session identity of the pre-established session |  |  |  |  |  |
| SIP-Version |  | "SIP/2.0" |  |  |  |  |  |  |  |
| Via |  |  |  |  |  | RFC 3261 [22] RFC 3581 [55] |  |  |  |
| sent-protocol |  | "SIP/2.0/UDP" |  |  |  |  |  | UDP |  |
|  |  | "SIP/2.0/TCP" |  |  |  |  |  | TCP |  |
| sent-by |  |  |  |  |  |  |  |  |  |
| host |  | IP address or FQDN |  | Either the UE’s IP address or its home domain name |  |  |  |  |  |
| port |  | protected server port of the UE |  |  |  |  |  |  |  |
| via-branch |  | Value starting with 'z9hG4bK' |  |  |  |  |  |  |  |
| Route |  |  |  |  |  | RFC 3261 [22] |  |  |  |
| addr-spec[1] |  | SIP URI |  |  |  |  |  |  |  |
| user-info and host |  | P-CSCF address of the SS |  | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |  |  |  |
| port |  | protected server port of the SS |  | as assigned during registration |  |  |  |  |  |
| uri-parameters |  | "lr" |  |  |  |  |  |  |  |
| addr-spec[2] |  | SIP URI |  |  |  |  |  |  |  |
| user-info and host |  | "scscf.3gpp.org" |  |  |  |  |  |  |  |
| port |  | not present |  |  |  |  |  |  |  |
| uri-parameters |  | "lr" |  |  |  |  |  |  |  |
| From |  |  |  |  |  | RFC 3261 [22] |  |  |  |
| addr-spec |  |  |  |  |  |  |  |  |  |
| user-info and host |  | Default public user id (px_MCX_SIP_PublicUserId_A_1) |  |  |  |  |  |  |  |
| port |  | not present |  |  |  |  |  |  |  |
| tag |  | any allowed value |  |  |  |  |  |  |  |
| To |  |  |  |  |  | RFC 3261 [22] RFC 5031 [54] |  |  |  |
| addr-spec |  |  |  |  |  |  |  |  |  |
| user-info and host |  | Same URI as used in the INVITE creating the pre-established session |  |  |  |  |  |  |  |
| port |  | not present |  |  |  |  |  |  |  |
| tag |  | not present |  |  |  |  |  |  |  |
| Call-ID |  |  |  |  |  | RFC 3261 [22] |  |  |  |
| callid |  | any allowed value |  |  |  |  |  |  |  |
| CSeq |  |  |  |  |  | RFC 3261 [22] |  |  |  |
| value |  | any allowed value |  |  |  |  |  |  |  |
| method |  | "REFER" |  |  |  |  |  |  |  |
| Supported |  |  |  |  |  | RFC 3261 [22] RFC 6442 [62] RFC 4488 [36] |  |  |  |
| option-tag |  | "norefersub" |  |  |  |  |  |  |  |
| Refer-Sub |  |  |  |  |  | RFC 4488 [36] |  |  |  |
| refer-sub-value |  | "false" |  |  |  |  |  |  |  |
| Target-Dialog |  |  |  |  |  | RFC 4538 [37] |  |  |  |
| callid |  | Callid of the pre-established session |  | Callid as used by the UE in the INVITE for establishment of the pre-established session |  |  |  |  |  |
| Require |  |  |  |  |  | RFC 3261 [22] RFC 3312 [56] RFC 3329 [53] |  |  |  |
| option-tag |  | "sec-agree" |  |  |  |  |  |  |  |
| option-tag |  | "multiple-refer" |  |  |  |  |  |  |  |
| Proxy-Require |  |  |  |  |  | RFC 3261 [22] RFC 3329 [53] |  |  |  |
| option-tag |  | "sec-agree" |  |  |  |  |  |  |  |
| Security-Verify |  |  |  |  |  | RFC 3329 [53] |  |  |  |
| sec-mechanism |  | same value as Security -Server header sent by SS during registration |  |  |  |  |  |  |  |
| Contact |  |  |  |  |  | RFC 3261 [22 RFC 3840 [33] |  |  |  |
| addr-spec |  | SIP URI |  |  |  |  |  |  |  |
| user-info and host |  | IP address or FQDN |  |  |  |  |  |  |  |
| feature-param |  | "+g.3gpp.mcptt" |  | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Push To Talk (MCPTT) communication. |  |  |  | MCPTT |  |
|  |  | "+g.3gpp.mcvideo" |  | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Video (MCVideo) communication. |  |  |  | MCVIDEO |  |
|  |  | "+g.3gpp.mcdata.sds" |  | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Data (MCData) communication. |  |  |  | MCDATA |  |
| feature-param |  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  | This URN indicates that the device has the capabilities to support the mission critical push to talk (MCPTT) service. |  |  |  | MCPTT |  |
|  |  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  | This URN indicates that the device has the capabilities to support the mission critical video (MCVideo) service. |  |  |  | MCVIDEO |  |
|  |  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  | This URN indicates that the device has the capabilities to support the mission critical data (MCData) service. |  |  |  | MCDATA |  |
| feature-param |  | "audio" |  | This feature tag indicates that the device supports audio as a streaming media type. |  |  |  | MCPTT OR MCVIDEO |  |
| feature-param |  | "video" |  | This feature tag indicates that the device supports video as a streaming media type. |  |  |  | MCVIDEO |  |
| feature-param |  | "text" |  | This feature tag indicates that the device supports text as a streaming media type. |  |  |  | MCDATA |  |
| Refer-To |  |  |  |  |  | RFC 3515 [38] |  |  |  |
| addr-spec |  | a Content-ID ("cid") Uniform Resource Locator (URL) as specified in IETF RFC 2392 that points to an application/resource-lists+xml MIME body as specified in IETF RFC 5366 |  |  |  |  |  |  |  |
| Refer-To |  |  |  |  |  | RFC 3515 [38] |  | METHOD-BYE |  |
| addr-spec |  |  |  |  |  |  |  |  |  |
| user-info and host |  | tsc_MCX_SessionID_B |  | The session identity of the pre-established session to leave. |  |  |  |  |  |
| uri-parameters |  |  |  |  |  |  |  |  |  |
| id[1] |  | method |  |  |  |  |  |  |  |
| value[1] |  | "BYE" |  |  |  |  |  |  |  |
| Max-Forwards |  |  |  |  |  | RFC 3261 [22] |  |  |  |
| value |  | any allowed value |  | Non-zero value |  |  |  |  |  |
| P-Access-Network-Info |  |  |  |  |  | RFC 7315 [52] |  |  |  |
| access-net-specs |  | Access network technology and, if applicable, the cell ID |  |  |  |  |  |  |  |
| P-Preferred-Service |  |  |  |  |  | RFC 6050 [31] |  |  |  |
| Service-ID |  | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |  |  | MCPTT |  |
|  |  | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  |  |  |  | MCVIDEO |  |
|  |  | "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  |  |  |  | MCDATA |  |
| P-Preferred-Identity |  | If present |  |  |  | RFC 3325 [32] |  |  |  |
| PPreferredID-value |  | same URI as in From-header |  |  |  |  |  |  |  |
| Resource-Priority |  |  |  |  |  | RFC 4412 [40] RFC 7134 [57] RFC 8101 [45] TS 24.379 [9] clause 6.2.8.1.15 |  | EMERGENCY-CALL AND (GROUP-CALL OR PRIVATE-CALL) |  |
| r-value |  |  |  |  |  |  |  |  |  |
| namespace |  | value of the <resource-priority-namespace> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration documents |  | As configured in Table 5.5.8.4-1 for MCPTT and in Table 5.5.8.8-1 for MCVIdeo |  | TS 24.484 [14] |  |  |  |
| r-priority |  | value of the <resource-priority-priority> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration document |  | As configured in Table 5.5.8.4-1 for MCPTT and in Table 5.5.8.8-1 for MCVIdeo |  | TS 24.484 [14] |  |  |  |
| Resource-Priority |  |  |  |  |  | RFC 4412 [40] RFC 7134 [57] RFC 8101 [45] TS 24.379 [9] clause 6.2.8.1.15 |  | IMMPERIL-CALL AND (GROUP-CALL OR PRIVATE-CALL) |  |
| r-value |  |  |  |  |  |  |  |  |  |
| namespace |  | value of the <resource-priority-namespace> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration documents |  | As configured in Table 5.5.8.4-1 for MCPTT and in Table 5.5.8.8-1 for MCVIdeo |  | TS 24.484 [14] |  |  |  |
| r-priority |  | value of the <resource-priority-priority> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration document |  | As configured in Table 5.5.8.4-1 for MCPTT and in Table 5.5.8.8-1 for MCVIdeo |  | TS 24.484 [14] |  |  |  |
| Content-Type |  | not present |  |  |  |  |  | METHOD-BYE |  |
| Content-Type |  |  |  |  |  | RFC 5621 [58] |  |  |  |
| media-type |  | "multipart/mixed" |  |  |  |  |  |  |  |
| Content-Length |  | present in case of TCP and when there is a message body (otherwise optional) |  |  |  | RFC 3261 [22] |  |  |  |
| Value |  | any value |  | length of message-body |  |  |  |  |  |
| Message-body |  | not present |  |  |  |  |  | METHOD-BYE |  |
| Message-body |  |  |  |  |  | RFC 3261 [22] |  |  |  |
| MIME body part |  |  |  | Resource list |  | RFC 5366 [35] |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/resource-lists+xml" |  |  |  |  |  |  |  |
| Content-ID |  | same value as the cid URL in the Refer-To header field |  | Unique URL identifying the Resource-lists XML MIME body; used as reference in the signature MIME body too |  | TS 24.379 [9] clause 6.6.3.1 |  |  |  |
| MIME-part-body |  | Resource-lists as described in Table 5.5.3.3.1-1 with condition PRE-ESTABLISH and the uri attribute of the single <entry> element extended with the headers of Table 5.5.2.12-2 |  |  |  |  |  | MCPTT |  |
|  |  | Resource-lists as described in Table 5.5.3.3.1-2 |  |  |  |  |  | MCVIDEO |  |
|  |  | Resource-lists as described in Table 5.5.3.3.1-3 |  |  |  |  |  | MCDATA |  |
| MIME body part |  |  |  | Location info |  |  |  | LOCATION-INFO |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcptt-location-info+xml" |  |  |  |  |  | MCPTT |  |
|  |  | "application/vnd.3gpp.mcvideo-location-info+xml" |  |  |  |  |  | MCVIDEO |  |
| Content-ID |  | any value |  | Unique URL identifying the Location-info XML MIME body; used as reference in the signature MIME body |  | TS 24.379 [9] clause 6.6.3.1 |  |  |  |
| MIME-part-body |  | Location-info as described in Table 5.5.3.4.1-1 |  |  |  | TS 24.379 [9] clause F.3 |  | MCPTT |  |
|  |  | Location-info as described in Table 5.5.3.4.1-2 |  |  |  | TS 24.281 [86] clause F.3 |  | MCVIDEO |  |
| MIME body part |  |  |  | Signature |  |  |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcptt-signed+xml" |  |  |  | TS 24.379 [9] |  |  |  |
| MIME-part-body |  | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  |  |  | TS 24.379 [9] |  |  |  |

Table 5.5.2.12-2: SIP header fields extending the uri attribute of the resource-lists' single entry

| Derivation Path: TS 24.379 [9] clause 10.1.1.2.2.1, 10.1.2.2.2.1, 11.1.1.2.2.1, 11.1.6.2.2.1 Editor's note: references for MCVIDEO and MCDATA to be added |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  | RFC 3841 [29] | GROUP-CALL OR CHAT-GROUP-CALL |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" |  |  | MCDATA |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| Answer-Mode | not present |  |  |  |
| Answer-Mode |  |  | RFC 5373 [34] TS 24.379 [9] cl. 11.1.1.2.2.1, 8) | PRIVATE-CALL AND (NOT FORCE) |
| answer-mode-value | "Auto" |  |  |  |
| answer-mode-value | "Manual" |  |  | MANUAL |
| Priv-Answer-Mode | not present |  |  |  |
| Priv-Answer-Mode |  |  | RFC 5373 [34] TS 24.379 [9] clause 11.1.1.2.2.1, 8) and clause 11.1.6.2.2.1, 8) | PRIVATE-CALL AND FORCE |
| answer-mode-value | "Auto" | if force of automatic commencement mode at the invited MCPTT client is requested by the MCPTT user, |  |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| body |  | NOTE: Characters that are not formatted as ASCII characters are escaped in the following parameters in the headers portion of the SIP URI. | RFC 3261 [22] |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 | MCPTT |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  | TS 24.282 [87] clause D.1 | MCDATA |
| MIME body part |  | Location info |  | (MCPTT OR MCVIDEO) AND ALLOW-LOCATION-INFO |
| MIME-part-headers |  |  |  |  |
| Content-Type | “application/vnd.3gpp.mcptt-location-info+xml” |  |  | MCPTT |
|  | “application/vnd.3gpp.mcvideo-location-info+xml” |  |  | MCVIDEO |
| Content-ID | any value | Unique URL identifying the Location-info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Location-info as described in Table 5.5.3.4.1-1 |  | TS 24.379 [9] clause F.3 | MCPTT |
|  | Location-info as described in Table 5.5.3.4.1-2 |  | TS 24.281 [86] clause F.3 | MCVIDEO |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | “application/vnd.3gpp.mcptt-signed+xml” |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| MANUAL | Call establishment with manual commencement mode |
| FORCE | force of automatic commencement mode at the invited MCPTT client is requested by the MCPTT user |
| ALLOW-LOCATION-INFO | Implicit floor control is requested AND <allow-location-info-when-talking> element of the <ruleset> element of the MCPTT user profile document set to “true” in TS 36.579-1 [2] Table 5.5.8.3-1 |
| For further conditions see table 5.5.1-1 |  |

#### 5.5.2.13	SIP REGISTER

This message is sent by the UE.

Table 5.5.2.13-1: SIP REGISTER

| Derivation Path: TS 24.229 [16] clause A.2.1.4.12, A.2.2.4.12 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] |  |
| Method | "REGISTER" |  |  |  |
| Request-URI | SIP URI of the home domain name (px_MCX_SIP_HomeDomain_A) if available at the UE or derived from the IMSI otherwise | Depending on the UE configuration the UE may know the home domain name of the SIP core (e.g. when there is an ISIM) or the UE needs to derive it from the IMSI as according to 23.003 [69] clause 13.2 (e.g. when there is a USIM only) |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Route | Not present |  | RFC 3261 [22] |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol | "SIP/2.0/UDP" | UE uses UDP for registration |  | UDP |
|  | "SIP/2.0/TCP | UE uses TCP for registration |  | TCP |
| sent-by |  |  |  |  |
| host | IP address or FQDN |  |  |  |
| port | any value if present |  |  | SIP_REGISTER_INITIAL |
|  | any value if present |  |  | TCP |
|  | protected server port of the UE when using UDP |  |  | UDP |
| via-branch | Value starting with 'z9hG4bK' |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | same value as in the initial REGISTER |  |  |  |
|  | Default public user id (px_MCX_SIP_PublicUserId_A_1) if available at the UE or derived from the IMSI otherwise | Depending on the UE configuration the UE may know the default public user id (e.g. when there is an ISIM) or the UE needs to derive it from the IMSI as according to 23.003 [69] clause 13.4B (e.g. when there is a USIM only) |  | SIP_REGISTER_INITIAL |
| port | not present |  |  |  |
| tag | any value |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as in From-header |  |  |  |
| tag | Not present |  |  |  |
| Contact |  |  | RFC 3261 [22] |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | IP address or FQDN |  |  |  |
| port | any value if present |  |  | SIP_REGISTER_INITIAL |
|  | protected server port of the UE |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Video (MCVideo) communication. |  | MCVIDEO |
| feature-param | g.3gpp.mcdata.sds | SDS is supported | TS 24.282 [87] clause 7.2.1 | MCDATA AND pc_MCData_SDS |
| feature-param | g.3gpp.mcdata.fd | FD is supported | TS 24.282 [87] clause 7.2.1 | MCDATA AND pc_MCData_FD |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" | This URN indicates that the device has the capabilities to support the mission critical video (MCVideo) service. |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata " | This URN indicates that the device has the capabilities to support the mission critical data (MCData) service. |  | MCDATA |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" | SDS is supported | TS 24.282 [87] clause 7.2.1 | MCDATA AND pc_MCData_SDS |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" | FD is supported | TS 24.282 [87] clause 7.2.1 | MCDATA AND pc_MCData_FD |
| feature-param | "audio" |  |  | MCPTT OR MCVIDEO |
| feature-param | "video" | This feature tag indicates that the device supports video as a streaming media type. |  | MCVIDEO |
| feature-param | "text" | This feature tag indicates that the device supports text as a streaming media type. |  | MCDATA |
| feature-param | “expires=600000” if present |  |  |  |
| Expires | Present if no expires parameter in Contact header |  | RFC 3261 [22] RFC 3903 [43] |  |
| value | "600000" |  |  |  |
| Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Proxy-Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Supported |  |  | RFC 3261 [22] RFC 6442 [62] RFC 4488 [36] |  |
| option-tag | "path" |  |  |  |
| option-tag | "timer" |  |  |  |
| Cseq |  |  | RFC 3261 [22] |  |
| value | any allowed value |  |  | SIP_REGISTER_INITIAL |
|  | value sent by the UE in previous REGISTER incremented by one |  |  |  |
| method | "REGISTER" |  |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | any value |  |  |  |
| Security-Client |  |  | RFC 7315 [52] |  |
| mechanism-name | "ipsec-3gpp" |  |  |  |
| algorithm | "hmac-sha-1-96" |  |  |  |
| protocol | "esp" (if present) |  |  |  |
| mode | "trans" (if present) |  |  |  |
| encrypt-algorithm | "des-ede3-cbc" or "aes-cbc" |  |  |  |
| spi-c | SPI number of the inbound SA at the protected client port |  |  |  |
| spi-s | SPI number of the inbound SA at the protected server port |  |  |  |
| port-c | protected client port |  |  |  |
| port-s | protected server port |  |  |  |
| Security-Verify | Not present |  | RFC 3329 [53] | SIP_REGISTER_INITIAL |
| Security-Verify |  |  | RFC 3329 [53] |  |
| sec-mechanism | same value as Security Server header sent by SS |  |  |  |
| Authorization |  |  | RFC 2617 [72], RFC 3310 [96] | SIP_REGISTER_INITIAL |
| username | Private user id (px_MCX_SIP_PrivateUserId_A) if available at the UE or derived from the IMSI otherwise | Depending on the UE configuration the UE may know the private public user id (e.g. when there is an ISIM) or the UE needs to derive it from the IMSI as according to 23.003 [69] clause 13.3 (e.g. when there is a USIM only) |  |  |
| realm | same home domain name as used in Request-URI |  |  |  |
| nonce | "" | Empty string |  |  |
| digest-uri | same SIP-URI as used as Request-URI |  |  |  |
| opaque | any value if present |  |  |  |
| qop | any value if present |  |  |  |
| cnonce | any value if present |  |  |  |
| nc | any value if present |  |  |  |
| algorithm | any value if present |  |  |  |
| response | "" | Empty string |  |  |
| Authorization |  |  | RFC 2617 [72], RFC 3310 [96] |  |
| username | same value as for condition SIP_REGISTER_INITIAL |  |  |  |
| realm | same value as received in the realm directive in the WWW Authenticate header sent by SS |  |  |  |
| nonce | same value as in WWW-Authenticate header sent by SS |  |  |  |
| digest-uri | same SIP-URI as used as Request-URI |  |  |  |
| opaque | same value as sent by the server in “401 Unauthorized for REGISTER” |  |  |  |
| qop | "auth" |  |  |  |
| cnonce | any value | value assigned by UE affecting the response calculation |  |  |
| nc | nonce-count value | counter to indicate how many times the UE has sent the same value of nonce within successive REGISTERs, initial value shall be 1 |  |  |
| algorithm | “AKAv1-MD5” |  |  |  |
| response | Digest response | calculated by the client according to RFC 2617 |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | any allowed value | Non-zero value |  |  |
| P-Access-Network-Info |  |  | RFC 7315 [52] |  |
| access-net-specs | Access network technology and, if applicable, the cell ID |  |  |  |
| Content-Type |  |  | RFC 5621 [58] | CONFIG |
| media-type | "multipart/mixed" |  |  |  |
| Content-Length | present in case of TCP and when there is a message body (otherwise optional) |  | RFC 3261 [22] |  |
| value | any value | length of the message body |  |  |
| Message-body |  |  | RFC 3261 [22] | CONFIG |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 | MCPTT |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  | TS 24.282 [87] clause D.1 | MCDATA |
| MIME body part |  | MIKEY |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/mikey" |  | RFC 3830 [24] |  |
| MIME-part-body | MIKEY message as described in Table 5.5.9.1-1 | MIKEY message, containing the CSK | TS 33.180 [94] |  |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| SIP_REGISTER_INITIAL | Initial unprotected REGISTER |
| For further conditions see table 5.5.1-1 |  |

#### 5.5.2.14	SIP SUBSCRIBE

This message is sent by the UE.

Table 5.5.2.14-1: SIP SUBSCRIBE

| Derivation Path: TS 24.229 [16] clause A.2.1.4.13, A.2.2.4.13 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "SUBSCRIBE" |  |  |  |
| Request-URI | tsc_MCPTT_PublicServiceId_A | The public service identity identifying the originating participating MCPTT function serving the MCPTT user |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A | The public service identity identifying the originating participating MCVideo function serving the MCVideo user |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A | The public service identity identifying the originating participating MCData function serving the MCData user |  | MCDATA |
|  | "sip:" & tsc_MCX_CMS_Hostname | SIP URI of the CMS's domain name: public service identity (PSI) for performing subscription proxy function of the CMS | TS 24.484 [14] clause 6.3.13.2.2 | CONFIG |
|  | "sip:" & tsc_MCX_GMSURI | public service identity (PSI) for performing subscription proxy function of the GMS as configured in the <GMS-URI> element of the initial UE configuration | TS 24.481 [11] clause 6.3.13.2.1 | GROUPCONFIG |
|  | same URI as the SS has sent earlier in the Contact header of a message within the same dialog | Contact URI of the recipient of the previous 200 OK |  | re_SUBSCRIBE |
| SIP-Version | "SIP/2.0" |  |  |  |
| Route |  |  | RFC 3261 [22] |  |
| addr-spec[1] | SIP URI |  |  |  |
| user-info and host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | protected server port of the SS | as assigned during registration |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[2] | SIP URI |  |  |  |
| user-info and host | "scscf.3gpp.org" |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| Route |  |  | RFC 3261 [22] | re_SUBSCRIBE |
| route-param list | URIs of the Record-Route header sent to the UE in the response which has established the dialog, in reverse order |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol | "SIP/2.0/UDP" |  |  | UDP |
|  | "SIP/2.0/TCP" |  |  | TCP |
| sent-by |  |  |  |  |
| host | IP address or FQDN | Either the UE’s IP address or its home domain name |  |  |
| port | protected server port of the UE | as assigned during registration |  |  |
| via-branch | value starting with 'z9hG4bK' |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | Default public user id (px_MCX_SIP_PublicUserId_A_1) |  |  |  |
| port | not present |  |  |  |
| tag | any value |  |  |  |
| From |  |  | RFC 3261 [22] | re_SUBSCRIBE |
| addr-spec | Same URI of the UE as used earlier in the dialog | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the UE as used earlier in the dialog | Local tag of the dialog ID (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec |  |  |  |  |
| user-info and host | same URI as used as Request URI |  |  |  |
| port | not present |  |  |  |
| tag | not present |  |  |  |
| To |  |  | RFC 3261 [22] | re_SUBSCRIBE |
| addr-spec | Same URI of the SS as used earlier in the dialogURI | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the SS as used earlier in the dialog | Remote tag of the dialog ID (from the UE's point of view) |  |  |
| Contact |  |  | RFC 3261 [22] |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | IP address or FQDN |  |  |  |
| port | protected server port of UE | as assigned during registration |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" | Mandatory media feature tag according to TS 24.481 [11] clause 6.3.13.2.1 and TS 24.484 [14] clause 6.3.13.2.2 |  | CONFIG OR GROUPCONFIG |
| feature-param | any (further) feature tags if present | In addition to mandatory feature tags (if any) the UE may provide further feature tags which are not checked |  |  |
| Expires |  |  | RFC 3261 [22] RFC 3903 [43] |  |
| value | any value |  |  |  |
| Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Proxy-Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Security-Verify |  |  | RFC 3329 [53] |  |
| sec-mechanism | same value as Security -Server header sent by SS during registration |  |  |  |
| Cseq |  |  | RFC 3261 [22] |  |
| value | any allowed value |  |  |  |
|  | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  | re_SUBSCRIBE |
| method | "SUBSCRIBE" |  |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | any allowed value |  |  |  |
|  | same value as in SUBSCRIBE creating the dialog |  |  | re_SUBSCRIBE |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | any allowed value | Non-zero value |  |  |
| P-Access-Network-Info |  |  | RFC 7315 [52] RFC 7913 [51] |  |
| access-net-spec | Access network technology and, if applicable, the cell ID | Access network technology and, if applicable, the cell ID |  |  |
| Event |  |  | RFC 6665 [39] |  |
| event-type | "presence" |  |  |  |
|  | "xcap-diff" |  |  | CONFIG GROUPCONFIG |
|  | "poc-settings" |  |  | POC-SETTINGS-EVENT |
| Accept |  |  | RFC 3261 [22] |  |
| media-range | "application/pidf+xml" |  |  |  |
|  | "application/xcap-diff+xml" |  |  | CONFIG, GROUPCONFIG |
|  | "application/poc-settings+xml" |  |  | POC-SETTINGS-EVENT |
| P-Preferred-Service |  |  | RFC 6050 [31] |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT OR CONFIG OR GROUPCONFIG |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata" |  |  | MCDATA |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| Content-Length | present in case of TCP and when there is a message body (otherwise optional) |  | RFC 3261 [22] |  |
| value | any value | length of message-body |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT OR CONFIG OR GROUPCONFIG |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 | MCPTT OR CONFIG OR GROUPCONFIG |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  | TS 24.282 [87] clause D.1 | MCDATA |
| MIME body part |  | SIMPLE-FILTER |  | PRESENCE-EVENT |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/simple-filter+xml" |  |  |  |
| Content-ID | any value | Unique URL identifying the SIMPLE-FILTER XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | SIMPLE-FILTER as described in Table 5.5.3.6-1 |  | TS 24.379 [9] clause 9.3.2 TS 24.281 [86] clause 8.3.2 TS 24.282 [87] clause 8.4.2 |  |
| MIME body part |  | Resource-lists |  | CONFIG, GROUPCONFIG |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/resource-lists+xml" |  |  |  |
| Content-ID | any value | Unique URL identifying the Resource-lists XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Resource-lists as described in Table 5.5.3.3.1-1 |  |  | MCPTT |
|  | Resource-lists as described in Table 5.5.3.3.1-2 |  |  | MCVIDEO |
|  | Resource-lists as described in Table 5.5.3.3.1-3 |  |  | MCDATA |
| MIME body part |  | MIKEY | RFC 3830 [24] | CONFIG, GROUPCONFIG |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/mikey" |  |  |  |
| MIME-part-body | MIKEY message as described in Table 5.5.9.1-1 | MIKEY message, containing the CSK | TS 33.180 [94] |  |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| re_SUBSCRIBE | SUBSCRIBE within a dialog |
| For further conditions see table 5.5.1-1 |  |

#### 5.5.2.15	SIP UPDATE

##### 5.5.2.15.1	SIP UPDATE from the UE

Table 5.5.2.15.1-1: SIP UPDATE from the UE

| Derivation Path: TS 24.229 [16] A.2.1.4.14, A.2.2.4.14 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "UPDATE" |  |  |  |
| Request-URI | The same URI value as the recipient of UPDATE has earlier sent in its Contact header within the same dialog |  |  |  |
| SIP-Version | 'SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol | "SIP/2.0/UDP" |  |  |  |
|  | "SIP/2.0/TCP" |  |  | TCP |
| sent-by | same value as in INVITE message |  |  | MO_CALL |
| sent-by |  |  |  | MT_CALL |
| host | IP address or FQDN | Either the UE’s IP address or its home domain name |  |  |
| port | protected server port of the UE | as assigned during registration |  |  |
| via-branch | Value starting with 'z9hG4bK' |  |  |  |
| Route |  |  | RFC 3261 [22] |  |
| route-param list | URIs of the Record-Route header sent to the UE in the response which has established the dialog, in reverse order |  |  | MO_CALL |
|  | URIs of the Record-Route header sent to the UE in the INVITE |  |  | MT_CALL |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | Same URI of the UE as used earlier in the dialog | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the UE as used earlier in the dialog | Local tag of the dialog ID (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec | Same URI of the SS as used earlier in the dialog | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the SS as used earlier in the dialog | Remote tag of the dialog ID (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | Same value as used in the INVITE initiating the dialog |  |  |  |
| Contact | Contact header with the same Contact URI and the same mandatory feature parameters as in the INVITE creating the dialog |  | RFC 3261 [22] | MO_CALL |
|  | Contact header with the same Contact URI and the same mandatory feature parameters as in the response for the INVITE creating the dialog |  |  | MT_CALL |
| CSeq |  |  | RFC 3261 [22] |  |
| value | value of CSeq sent by the UE within its previous request in the same dialog but increased by one |  |  |  |
| method | "UPDATE" |  |  |  |
| Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Proxy-Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Security-Verify |  |  | RFC 3329 [53] |  |
| sec-mechanism | same value as Security -Server header sent by SS during registration |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | any allowed value | Non-zero value |  |  |
| P-Access-Network-Info |  |  | RFC 7315 [52] RFC 7913 [51] |  |
| access-net-spec | Access network technology and, if applicable, the cell ID |  |  |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "application/sdp" |  |  |  |
| Content-Length | present in case of TCP and when there is a message body (otherwise optional) |  | RFC 3261 [22] |  |
| value | any value | length of message-body |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| SDP Message | SDP Message as described in Table 5.5.3.1.1-1 |  |  |  |
|  | SDP Message as described in Table 5.5.3.1.1-2 |  |  | MCVIDEO |
|  | SDP Message as described in Table 5.5.3.1.1-3 |  |  | MCDATA |

##### 5.5.2.15.2	SIP UPDATE from the SS

Table 5.5.2.15.2-1: SIP UPDATE from the SS

| Derivation Path: TS 24.229 [16] A.2.1.4.14, A.2.2.4.14 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "UPDATE" |  |  |  |
| Request-URI | same URI as the UE has sent earlier in the Contact header of a response within the same dialog | Contact URI of the UE ("callee") |  |  |
| SIP-Version | 'SIP/2.0" |  |  |  |
| Via | same as specified for INVITE sent by the SS in Table 5.5.2.5.2-1 |  | RFC 3261 [22] RFC 3581 [55] | MO_CALL |
| Via | same as in INVITE but with updated via-branches |  | RFC 3261 [22] RFC 3581 [55] | MT_CALL |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | Same URI of the SS as used earlier in the dialog | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the SS as used earlier in the dialog | Remote tag of the dialog (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec | Same URI of the UE as used earlier in the dialog | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the UE as used earlier in the dialog | Local tag of the dialog (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | Same value as used in the INVITE initiating the dialog |  |  |  |
| Contact | same as in the response for the INVITE creating the dialog |  | RFC 3261 [22] | MO_CALL |
|  | same as in the INVITE creating the dialog |  |  | MT_CALL |
| CSeq |  |  | RFC 3261 [22] |  |
| value | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  |  |
| method | "UPDATE" |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | "68" | The recommended initial value is 70 in RFC 3261 [22]. Assuming 2 hops as according to the Via header this results in a value of 68 in the message sent to the UE. |  |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "application/sdp" |  |  |  |
| Content-Length | length of message-body |  | RFC 3261 [22] |  |
| value | length of message-body |  |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| SDP Message | SDP Message as described in Table 5.5.3.1.1-2 |  |  |  |
|  | SDP Message as described in Table 5.5.3.1.2-2 |  |  | MCVIDEO |
|  | SDP Message as described in Table 5.5.3.1.2-3 |  |  | MCDATA |

#### 5.5.2.16	SIP 1xx

##### 5.5.2.16.1	SIP 100 (Trying)

This message is sent by the UE or the SS.

Table 5.5.2.16.1-1: SIP 100 (Trying)

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "100" |  |  |  |
| Reason-Phrase | "Trying" |  |  |  |
| Via |  |  |  |  |
| via-parm | same value as received in INVITE message |  |  |  |
| From |  |  |  |  |
| addr-spec | same value as received in INVITE message |  |  |  |
| tag | same value as received in INVITE message |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as received in INVITE message |  |  |  |
| Call-ID |  |  |  |  |
| callid | same value as received in INVITE message |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in INVITE message |  |  |  |
| Content-Length | Optional in case of the message being sent by the UE |  |  |  |
| value | "0" | No message body included - end of SIP message |  |  |

##### 5.5.2.16.2	SIP 180 (Ringing)

5.5.2.16.2.1	SIP 180 (Ringing) from the UE

Table 5.5.2.16.2.1-1: SIP 180 (Ringing) from the UE

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "180" |  |  |  |
| Reason-Phrase | "Ringing" |  |  |  |
| Record-Route |  |  | RFC 3261 [22] |  |
| rec-route | same as received in INVITE message |  |  |  |
| Via | same as received in INVITE message |  | RFC 3261 [22] RFC 3581 [55] |  |
| Require |  |  |  | 100rel |
| option-tag | "100rel" |  |  |  |
| From |  |  |  |  |
| addr-spec | same value as received in INVITE message |  |  |  |
| tag | same value as received in INVITE message |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as received in INVITE message |  |  |  |
| tag | same value as received in the INVITE message or any value if missing in the INVITE message. |  |  |  |
| Contact |  |  |  |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | IP address or FQDN |  |  |  |
| port | protected server port of UE | as assigned during registration |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
| feature-param | "+g.3gpp.icsi-ref= urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
| feature-param | "audio" |  |  | MCPTT OR MCVideo |
| feature-param | "video" |  |  | MCVIDEO |
| Supported |  |  |  |  |
| option-tag | "norefersub" |  |  |  |
| Rseq |  |  | RFC 3262 [97] | 100rel |
| response-num | previous RSeq number sent in the same direction incremented by one |  |  |  |
| Call-ID |  |  |  |  |
| callid | same value as received in INVITE message |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in INVITE message |  |  |  |
| Content-Length | if present |  |  |  |
| value | "0" | No message body included |  |  |

| Condition | Explanation |
| --- | --- |
| 100rel | Reponse sent reliable according to RFC 3262 [97] |

5.5.2.16.2.2	SIP 180 (Ringing) from the SS

Table 5.5.2.16.2.2-1: SIP 180 (Ringing) from the SS

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "180" |  |  |  |
| Reason-Phrase | "Ringing" |  |  |  |
| Record-Route | same as spefied for the SIP 200 (OK) from the SS in table 5.5.2.17.1.2-1 with condition INVITE-RSP |  | RFC 3261 [22] |  |
| Via | same as received in the INVITE message |  | RFC 3261 [22] RFC 3581 [55] |  |
| Require |  |  |  | 100rel |
| option-tag | "100rel" |  |  |  |
| From |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request or To-tag assigned by the SS if missing in the request |  |  |  |
| Contact |  |  |  |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_SessionId |  |  | MCPTT |
|  | tsc_MCVideo_SessionId |  |  | MCVIDEO |
| port | not present |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
| feature-param | "+g.3gpp.icsi-ref= urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
| feature-param | "audio" |  |  | MCPTT OR MCVIDEO |
| feature-param | "video" | This feature tag indicates that the device supports video as a streaming media type. |  | MCVIDEO |
| feature-param | "isfocus" |  |  |  |
| Supported |  |  |  |  |
| option-tag | "norefersub" |  |  |  |
| Rseq |  |  | RFC 3262 [97] | 100rel |
| response-num | previous RSeq number sent in the same direction incremented by one; arbitrarily selected if there is no previous RSeq number |  |  |  |
| Call-ID |  |  |  |  |
| callid | same value as received in INVITE message |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in INVITE message |  |  |  |
| Content-Length |  |  |  |  |
| value | "0" | No message body included |  |  |

| Condition | Explanation |
| --- | --- |
| 100rel | Reponse sent reliable according to RFC 3262 [97] |

##### 5.5.2.16.3	SIP 183 (Session Progress)

5.5.2.16.3.1	SIP 183 (Session Progress) from the UE

Table 5.5.2.16.3.1-1: SIP 183 (Session Progress) from the UE

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "183" |  |  |  |
| Reason-Phrase | "Session progress" |  |  |  |
| Record-Route |  |  | RFC 3261 [22] |  |
| rec-route | same as received in INVITE message |  |  |  |
| Via | same as received in INVITE message |  | RFC 3261 [22] RFC 3581 [55] |  |
| Require |  |  |  | 100rel |
| option-tag | "100rel" |  |  |  |
| From |  |  |  |  |
| addr-spec | same value as received in INVITE message |  |  |  |
| tag | same value as received in INVITE message |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as received in INVITE message |  |  |  |
| tag | same value as received in the INVITE message or any value if missing in the INVITE message. |  |  |  |
| Contact |  |  |  |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | IP address or FQDN |  |  |  |
| port | protected server port of UE | as assigned during registration |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
| feature-param | "+g.3gpp.icsi-ref= urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
| feature-param | "audio" |  |  | MCPTT OR MCVideo |
| feature-param | "video" |  |  | MCVIDEO |
| Supported |  |  |  |  |
| option-tag | "norefersub" |  |  |  |
| Rseq |  |  |  | 100rel |
| response-num | previous RSeq number sent in the same direction incremented by one |  |  |  |
| Call-ID |  |  |  |  |
| callid | same value as received in INVITE message |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in INVITE message |  |  |  |
| P-Answer-State | if present |  |  |  |
| value | "unconfirmed" |  |  |  |
| Content-Length | if present |  | RFC 3261 [22] |  |
| value | "0" | No message body included |  |  |

| Condition | Explanation |
| --- | --- |
| 100rel | Reponse sent reliable according to RFC 3262 [97] |

5.5.2.16.3.2	SIP 183 (Session Progress) from the SS

Table 5.5.2.16.3.2-1: SIP 183 (Session Progress) from the SS

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "183" |  |  |  |
| Reason-Phrase | "Session progress" |  |  |  |
| Record-Route | same as specified for the SIP 200 (OK) from the SS in table 5.5.2.17.1.2-1 with condition INVITE-RSP |  | RFC 3261 [22] |  |
| Via | same as received in the INVITE message |  | RFC 3261 [22] RFC 3581 [55] |  |
| Require |  |  |  | 100rel |
| option-tag | "100rel" |  |  |  |
| From |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request or To-tag assigned by the SS if missing in the request |  |  |  |
| Contact |  |  |  |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_SessionId |  |  | MCPTT |
|  | tsc_MCVideo_SessionId |  |  | MCVIDEO |
| port | not present |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
| feature-param | "+g.3gpp.icsi-ref= urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
| feature-param | "audio" |  |  | MCPTT OR MCVIDEO |
| feature-param | "video" | This feature tag indicates that the device supports video as a streaming media type. |  | MCVIDEO |
| feature-param | "isfocus" |  |  |  |
| Supported |  |  |  |  |
| option-tag | "norefersub" |  |  |  |
| Rseq |  |  |  | 100rel |
| response-num | previous RSeq number sent in the same direction incremented by one; arbitrarily selected if there is no previous RSeq number |  |  |  |
| Call-ID |  |  |  |  |
| callid | same value as received in INVITE message |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in INVITE message |  |  |  |
| P-Answer-State |  |  |  |  |
| value | "unconfirmed" |  |  |  |
| P-Asserted-Identity |  |  | RFC 3325 [32] |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_PublicServiceId_A |  |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A |  |  | MCVIDEO |
| port | not present |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included |  |  |

| Condition | Explanation |
| --- | --- |
| 100rel | Response sent reliable according to RFC 3262 [97] |

#### 5.5.2.17	SIP 2xx

##### 5.5.2.17.1	SIP 200 (OK)

5.5.2.17.1.1	SIP 200 (OK) from the UE

Table 5.5.2.17.1.1-1: SIP 200 (OK) from the UE

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "200" |  |  |  |
| Reason-Phrase | "OK" |  |  |  |
| Via | same as received in the request |  | RFC 3261 [22] RFC 3581 [55] |  |
| Record-Route |  |  | RFC 3261 [22] | INVITE-RSP |
| rec-route | same as received in the request |  |  |  |
| From |  |  |  |  |
| addr-spec | Same value as received in the request |  |  |  |
| tag | same value as received in the request |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as received in the request |  |  |  |
| tag | same value as received in the request or any value if missing in the request. |  |  |  |
| Contact |  |  |  | INVITE-RSP |
| user-info and host | IP address or FQDN |  |  |  |
| port | protected server port of UE | as assigned during registration |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" |  | TS 24.282 [87] clause 9.2.3.2.4 | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" |  | TS 24.282 [87] clause 10.2.5.2.4 | MCDATA_FD |
| feature-param | "+g.3gpp.icsi-ref= urn:urn- 7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  | TS 24.282 [87] clause 9.2.3.2.4 | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  | TS 24.282 [87] clause 10.2.5.2.4 | MCDATA_FD |
| feature-param | "audio" |  |  | MCPTT OR MCVideo |
| feature-param | "video" |  |  | MCVIDEO |
| feature-param | "text" |  |  | MCDATA |
| Call-ID |  |  |  |  |
| callid | same value as received in the request |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in the request |  |  |  |
| Require |  |  |  | INVITE-RSP |
| option-tag | "timer" |  |  |  |
| Session-Expires |  |  |  | INVITE-RSP |
| delta-seconds | Same value as session expires header in SIP INVITE |  | RFC 4028 [30] TS 24.229 [16] cl.5.1.4.1 |  |
| refresher | "uas" |  |  |  |
| Content-Type |  |  | RFC 5621 [58] | INVITE-RSP |
| value | "multipart/mixed" |  |  |  |
| Content-Length | present in case of TCP and when there is a message body (otherwise optional) |  | RFC 3261 [22] |  |
| value | any value | length of message-body |  |  |
| P-Answer-State | If present |  | RFC 4964 [118] TS 24.379 [9] clause 6.2.3.1.2 | INVITE-RSP AND GROUP-CALL |
| answer-type | “confirmed” |  |  |  |
| Message-body | not present |  | RFC 3261 [22] |  |
| Message-body |  |  | RFC 3261 [22] | INVITE-RSP |
| MIME body part |  | SDP message |  |  |
| MIME-part-header |  |  |  |  |
| MIME-Content-Type | "application/sdp" |  | RFC 4566 [27] |  |
| MIME-part-body | SDP message as described in Table 5.5.3.1.1-1 |  |  | MCPTT |
|  | SDP message as described in Table 5.5.3.1.1-2 |  |  | MCVIDEO |
|  | SDP message as described in Table 5.5.3.1.1-3 |  |  | MCDATA |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-header |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 | MCPTT |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  | TS 24.282 [87] clause D.1 | MCDATA |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| INVITE-RSP | 200 OK is the response to the SIP INVITE |
| MCDATA_SDS | INVITE for SDS communication |
| MCDATA_FD | INVITE for FD communication |

5.5.2.17.1.2	SIP 200 (OK) from the SS

Table 5.5.2.17.1.2-1: SIP 200 (OK) from the SS

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "200" |  |  |  |
| Reason-Phrase | "OK" |  |  |  |
| Via | same as received in the request |  | RFC 3261 [22] RFC 3581 [55] |  |
| Record-Route |  |  | RFC 3261 [22] | INVITE-RSP |
| addr-spec[1] | SIP URI |  |  |  |
| user-info and host | pcscf.other.com |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[2] | SIP URI |  |  |  |
| user-info and host | scscf.other.com |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[3] | SIP URI |  |  |  |
| user-info and host | orig@scscf.3gpp.org |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[4] | SIP URI |  |  |  |
| user-info and host | same address as sent by the UE in the first entry of the Route header of the INVITE | P-CSCF address |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| Record-Route |  |  | RFC 3261 [22] | SUBSCRIBE-RSP |
| addr-spec[1] | SIP URI |  |  |  |
| user-info and host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| From |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request or To-tag assigned by the SS if missing in the request |  |  |  |
| Expires |  |  | RFC 3261 [22] RFC 3903 [43] | SUBSCRIBE-RSP, PUBLISH-RSP |
| value | same value as in the request |  |  |  |
| Contact |  |  |  | REGISTER-RSP |
| addr-spec | same value as received in the REGISTER |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
| feature-param | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
| feature-param | "+g.3gpp.mcdata.sds" |  |  | MCDATA |
| feature-param | "+g.3gpp.mcdata.fd" |  |  | MCDATA |
| expires | "600000" |  |  |  |
| Contact |  |  |  | SUBSCRIBE-RSP |
| addr-spec |  |  |  |  |
| user-info and host | Same URI as used as Request-URI of the SUBSCRIBE message |  |  |  |
| port | not present |  |  |  |
| Contact |  |  |  | INVITE-RSP |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_SessionId |  |  | MCPTT |
|  | tsc_MCVideo_SessionId |  |  | MCVIDEO |
|  | tsc_MCData_SessionId |  |  | MCDATA |
| port | not present |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" |  | TS 24.282 [87] clause 9.2.3.2.4 | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" |  | TS 24.282 [87] clause 10.2.5.2.4 | MCDATA_FD |
| feature-param | "+g.3gpp.icsi-ref= urn:urn- 7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  | TS 24.282 [87] clause 9.2.3.2.4 | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  | TS 24.282 [87] clause 10.2.5.2.4 | MCDATA_FD |
| feature-param | “audio” |  |  | MCPTT OR MCVIDEO |
| feature-param | “video” |  |  | MCVIDEO |
| feature-param | “text” |  |  | MCDATA |
| feature-param | "isfocus" |  |  |  |
| Call-ID |  |  |  |  |
| callid | same value as received in the request |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in the request |  |  |  |
| Require |  |  |  | INVITE-RSP |
| option-tag | "timer" |  |  |  |
| Session-Expires |  |  |  | INVITE-RSP |
| generic-param | "3600" |  |  |  |
| refresher | "uac" |  |  |  |
| Supported |  |  |  | INVITE-RSP |
| option-tag | "tdialog" |  |  |  |
| option-tag | "norefersub" |  |  |  |
| option-tag | "explicitsub" |  |  |  |
| option-tag | "nosub" |  |  |  |
| Refer-Sub |  |  | RFC 4488 [36] | REFER-RSP |
| refer-sub-value | “false” |  |  |  |
| P-Associated-URI |  |  | RFC 7315 [52] | REGISTER-RSP |
| addr-spec[1] | SIP URI |  |  |  |
| host | px_MCX_SIP_PublicUserId_A_1 |  |  |  |
| port | not present |  |  |  |
| Service-Route |  |  | RFC 3261 [22] | REGISTER-RSP |
| addr-spec[1] | SIP URI |  |  |  |
| host | scscf.3gpp.org |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| SIP-ETag |  |  | RFC 3903 [43] | PUBLISH-RSP |
| entity-tag | unique value arbitrarily selected by the SS |  |  |  |
| Content-Type |  |  | RFC 4566 [27] | INVITE-RSP |
| media-type | "application/sdp" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | length of message-body |  |  |  |
| Message-body |  |  | RFC 3261 [22] | INVITE-RSP |
| SDP message | SDP message as described in Table 5.5.3.1.2-1 |  |  | MCPTT |
|  | SDP message as described in Table 5.5.3.1.2-2 |  |  | MCVIDEO |
|  | SDP message as described in Table 5.5.3.1.2-3 |  |  | MCDATA |

| Condition | Explanation |
| --- | --- |
| REGISTER-RSP | 200 OK is the response to a SIP REGISTER |
| INVITE-RSP | 200 OK is the response to a SIP INVITE |
| SUBSCRIBE-RSP | 200 OK is the response to a SIP SUBSCRIBE |
| PUBLISH-RSP | 200 OK is the response to a SIP PUBLISH |
| REFER-RSP | 200 OK is the response to a SIP REFER |

##### 5.5.2.17.2	SIP 202 (Accepted)

Table 5.5.2.17.2-1: SIP 202 (Accepted)

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  | RFC 3261 [22] |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "202" |  |  |  |
| Reason-Phrase | "Accepted" |  |  |  |
| Via | same value as received in request |  | RFC 3261 [22] |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | same value as received in request |  |  |  |
| tag | same value as received in request |  |  |  |
| To |  |  | RFC 3261 [22] |  |
| addr-spec | same value as received in request |  |  |  |
| tag | same value as in the request or To-tag assigned by the SS if missing in the request |  |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | same value as received in request |  |  |  |
| CSeq |  |  | RFC 3261 [22] |  |
| value | same value as received in request |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" |  |  |  |

#### 5.5.2.18	SIP 3xx

##### 5.5.2.18.1	SIP 302 (Moved Temporarily)

Table 5.5.2.18.1-1: SIP 302 (Moved Temporarily)

| Delivery Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "302" |  |  |  |
| Reason-Phrase | "Moved Temporarily" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included - end of SIP message |  |  |

Editor’s note: Table 5.5.2.18.1-1 needs to be reviewed

#### 5.5.2.19	SIP 4xx

##### 5.5.2.19.1	SIP 403 (Forbidden)

This message is sent by the SS.

Table 5.5.2.19.1-1: SIP 403 (Forbidden)

| Delivery Path: RFC 3261 [22] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| Status-Line |  |  |  |  |  |  |  |  |  |
| SIP-Version |  | "SIP/2.0" |  |  |  |  |  |  |  |
| Status-Code |  | "403" |  |  |  |  |  |  |  |
| Reason-Phrase |  | "Forbidden" |  |  |  |  |  |  |  |
| Via |  | same as received in the request |  |  |  |  |  |  |  |
| From |  |  |  |  |  |  |  |  |  |
| addr-spec |  | same value as in the request |  |  |  |  |  |  |  |
| tag |  | same value as in the request |  |  |  |  |  |  |  |
| To |  |  |  |  |  |  |  |  |  |
| addr-spec |  | same value as in the request |  |  |  |  |  |  |  |
| tag |  | same value as in the request or To-tag assigned by the SS if missing in the request |  |  |  |  |  |  |  |
| Call-ID |  |  |  |  |  |  |  |  |  |
| callid |  | same value as in the request |  |  |  |  |  |  |  |
| CSeq |  |  |  |  |  |  |  |  |  |
| value |  | same value as in the request |  |  |  |  |  |  |  |
| Warning |  |  |  |  |  | RFC 3261 [22] |  |  |  |
| warn-code[1] |  | "100" |  |  |  |  |  |  |  |
| warn-agent[1] |  |  |  | name or pseudonym of the server adding the Warning header |  |  |  |  |  |
| pseudonym |  | "MCX Server" |  |  |  |  |  |  |  |
| warn-text[1] |  | "function not allowed due to" <detailed reason> |  |  |  |  |  |  |  |
| Content-Length |  |  |  |  |  | RFC 3261 [22] |  |  |  |
| value |  | "0" |  |  |  |  |  |  |  |

##### 5.5.2.19.2	SIP 404 (Not Found)

Table 5.5.2.19.2-1: SIP 404 (Not Found)

| Delivery Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "404" |  |  |  |
| Reason-Phrase | "Not Found" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included - end of SIP message |  |  |

Editor’s note: Table 5.5.2.19.2-1 needs to be reviewed

##### 5.5.2.19.3	SIP 423 (Interval Too Brief)

Table 5.5.2.19.3-1: SIP 423 (Interval Too Brief)

| Delivery Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "423" |  |  |  |
| Reason-Phrase | "Internal Too Brief" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included - end of SIP message |  |  |

Editor’s note: Table 5.5.2.19.3-1 needs to be reviewed

##### 5.5.2.19.4	SIP 480 (Temporarily unavailable)

This message is sent by the UE.

Table 5.5.2.19.4-1: SIP 480 (Temporarily unavailable)

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "480" |  |  |  |
| Reason-Phrase | "Temporarily Unavailable" |  |  |  |
| Via | same as received in request message |  | RFC 3261 [22] RFC 3581 [55] |  |
| From |  |  |  |  |
| addr-spec | same value as received in INVITE message |  |  |  |
| tag | same value as received in request message |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as received in request message |  |  |  |
| tag | same value as received in the INVITE or any value if missing in the INVITE. |  |  |  |
| Warning |  |  | RFC 3261 [22] |  |
| warn-code[1] | "110" |  |  |  |
| warn-agent[1] | any value |  |  |  |
| warn-text[1] | "user declined the call invitation" |  |  |  |
| Call-ID | same value as received in request message |  |  |  |
| CSeq | same value as received in request message |  |  |  |
| Content Length | if present |  |  |  |
| value | "0" | No message body included |  |  |

##### 5.5.2.19.5	SIP 486 (Busy Here)

Table 5.5.2.19.5-1: SIP 486 (Busy Here)

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "486" |  |  |  |
| Reason-Phrase | "Busy Here" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included - end of SIP message |  |  |

Editor’s note: Table 5.5.2.18.5-1 needs to be reviewed

##### 5.5.2.19.6	SIP 488 (Not Acceptable Here)

Table 5.5.2.19.6-1: SIP 488 (Not Acceptable Here)

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "488" |  |  |  |
| Reason-Phrase | "Not Acceptable Here" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included - end of SIP message |  |  |

Editor’s note: Table 5.5.2.19.6-1 needs to be reviewed

##### 5.5.2.19.7	SIP 401 (Unauthorized)

Table 5.5.2.19.7-1: SIP 401 (Unauthorized)

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  | RFC 3261 [22] |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "401" |  |  |  |
| Reason-Phrase | "Unauthorized" |  |  |  |
| Via | Same value as received in the REGISTER message |  | RFC 3261 [22] |  |
| To |  |  | RFC 3261 [22] |  |
| addr-spec | Same value as received in the REGISTER message |  |  |  |
| tag | To-tag assigned by the SS |  |  |  |
| From | Same value as received in the REGISTER message |  | RFC 3261 [22] |  |
| Call-ID | Same value as received in the REGISTER message |  | RFC 3261 [22] |  |
| CSeq | Same value as received in the REGISTER message |  | RFC 3261 [22] |  |
| WWW-Authenticate |  |  | RFC 2617 [72] RFC 3310 [96] |  |
| Realm | px_MCX_DomainName_Organization_A |  |  |  |
| algorithm | "AKAv1-MD5" |  |  |  |
| qop-value | "auth" |  |  |  |
| nonce | Base 64 encoding of RAND and AUTN |  |  |  |
| opaque | arbitrary value (to be returned by the UE in subsequent REGISTER) |  |  |  |
| Security-Server |  |  | RFC 3329 [50] |  |
| mechanism-name | "ipsec-3gpp" |  |  |  |
| algorithm[1] | px_IpSecAlgorithm (hmac-md5-96 or hmac-sha-1-96) |  |  |  |
| spi-c[1] | SPI number of the inbound SA at the protected client port |  |  |  |
| spi-s[1] | SPI number of the inbound SA at the protected server port |  |  |  |
| port-c[1] | protected client port of SS |  |  |  |
| port-s[1] | protected server port of SS |  |  |  |
| Encrypt-algorithm[1] | des-ede3-cbc or aes-cbc |  |  |  |
| q[1] | "0.9" |  |  |  |
| mechanism-name[2] | "Ipsec-3gpp" |  |  |  |
| algorithm[2] | Algorithm not selected by px_IpSecAlgorithm (hmac-sha-1-96 or hmac-md5-96) |  |  |  |
| spi-c[2] | SPI number of the inbound SA at the protected client port |  |  |  |
| spi-s[2] | SPI number of the inbound SA at the protected server port |  |  |  |
| port-c[2] | protected client port of SS |  |  |  |
| port-s[2] | protected server port of SS |  |  |  |
| encrypt-algorithm[2] | des-ede3-cbc or aes-cbc |  |  |  |
| q[2] | "0.7" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | “0” |  |  |  |

##### 5.5.2.19.8	SIP 487 (Request Terminated)

Table 5.5.2.19.8-1: SIP 486 (Request Terminated)

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "487" |  |  |  |
| Reason-Phrase | "Request Terminated" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included - end of SIP message |  |  |

#### 5.5.2.20	SIP 5xx

##### 5.5.2.20.1	SIP 500 (Server Internal Error)

Table 5.5.2.20.1-1: SIP 500 (Server Internal Error)

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "500" |  |  |  |
| Reason-Phrase | "Server Internal Error" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included - end of SIP message |  |  |

Editor’s note: Table 5.5.2.20.1-1 needs to be reviewed

#### 5.5.2.21	SIP 6xx

##### 5.5.2.21.1	SIP 606 (Not Acceptable)

Table 5.5.2.21.1-1: SIP 606 (Not Acceptable)

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "606" |  |  |  |
| Reason-Phrase | "Not Acceptable" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included - end of SIP message |  |  |

Editor’s note: Table 5.5.2.21.1-1 needs to be reviewed