3GPP TSG-RAN5 Meeting #97	     R5-226093
Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0816 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PC1 - MPR test case update in 38.521-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MU and TT analysis has progressed for PC1 devices. Test case needs to be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For 6.2.2: -Editor’s note update and TT table update/addition. -Anex F update: MU and TT definition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomple. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### 6.2.2	UE maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for PC2 and PC4.

- Measurement grid for PC2/4 in Annex M.4 is FFS.

- Once test case 6.2.2_1 is available for certification, the test applicability needs an update to exclude Rel15 UE supporting modifiedMPR bit 0 and Rel16 and later UE.

#### 6.2.2.0	General

The requirements in section 6.2.2 only apply when both UL and DL of a UE are configured for single CC operation, and they are of the same bandwidth. A UE may reduce its maximum output power due to modulation orders, transmit bandwidth configurations, waveform types and narrow allocations. This Maximum Power Reduction (MPR) is defined in subclauses below. The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation. When the maximum output power of a UE is modified by MPR, the power limits specified in subclause 6.2.4 apply.

For a UE that is configured for single CC operation with different channel bandwidths in UL and DL, the requirements in section 6.2A.2 apply.

For all power classes, the waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

6.2.2.1	Test purpose

The number of RB identified in 6.2.2.3 is based on meeting the requirements for the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2.2.2	Test applicability

The requirements of this test apply to all types of NR UE release 15 and forward.

6.2.2.3	Minimum conformance requirements

##### 6.2.2.3.1	UE maximum output power reduction for power class 1

For power class 1, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB ≤ 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Tables 6.2.2.3.1-1 and 6.2.2.3.1-2.

Table 6.2.2.3.1-1: MPRWT for power class 1, BWchannel ≤ 200 MHz

| Modulation |  | MPRWT (dB), BWchannel ≤ 200 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.0 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.0 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 4.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 7.0 | ≤ 5.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |

Table 6.2.2.3.1-2: MPRWT for power class 1, BWchannel = 400 MHz

| Modulation |  | MPRWT (dB), BWchannel = 400 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.0 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 7.0 | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 | ≤ 9.0 |

Where the following parameters are defined to specify valid RB allocation ranges for the RB allocations regions in Tables 6.2.2.3.1-1 and 6.2.2.3.1-2:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBend = RBStart + LCRB - 1

RBStart,Low = Max(1, Floor(LCRB/2))

RBStart,High = NRB – RBStart,Low – LCRB

An RB allocation is an Outer RB allocation if

RBStart < RBStart,Low OR RBStart > RBStart,High OR LCRB > Ceil(NRB/2)

An RB allocation belonging to Table 6.2.2.3.1-1 is a Region 1 inner RB allocation if

RBstart ≥ Ceil(1/3 NRB) AND RBend < Ceil(2/3 NRB)

An RB allocation belonging to Table 6.2.2.3.1-2 is a Region 1 inner RB allocation if

RBstart ≥ Ceil(1/4 NRB) AND RBend < Ceil(3/4 NRB) AND LCRB ≤ Ceil(1/4 NRB)

An RB allocation is a Region 2 inner allocation if it is NOT an Outer allocation AND NOT a Region 1 inner allocation.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.4 apply.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.2.1.

##### 6.2.2.3.2	UE maximum output power reduction for power class 2

For power class 2, MPR specified in subclause 6.2.2.3.3 applies.

Table 6.2.2.3.2-1: Void

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.2.2.

##### 6.2.2.3.3	UE maximum output power reduction for power class 3

For power class 3, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

MPRnarrow = 2.5 dB, BWalloc,RB ≤ 1.44 MHz, and 0 ≤ RBstart < Ceil(1/3 NRB) or Ceil((2/3NRB) -LCRB) ≤ RBstart ≤ NRB-LCRB, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2.3.3-1 and Table 6.2.2.3.3-2.

Table 6.2.2.3.3-1: MPRWT for power class 3, BWchannel ≤ 200 MHz

| Modulation |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 2.0 |
|  | QPSK | 0.0 | ≤ 2.0 |
|  | 16 QAM | ≤ 3.0 | ≤ 3.5 |
|  | 64 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 5.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 |

Table 6.2.2.3.3-2: MPRWT for power class 3, BWchannel = 400 MHz

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 3.0 |
|  | QPSK | 0.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 |

Where the following parameters are defined to specify valid RB allocation ranges for RB allocations in Tables 6.2.2.3.3-1 and 6.2.2.3.3-2:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBend = RBStart + LCRB - 1

An RB allocation belonging to Table 6.2.2.3.3-1 is a Region 1 inner RB allocation if

RBstart ≥ Ceil(1/3 NRB) AND RBend < Ceil(2/3 NRB)

An RB allocation belonging to Table 6.2.2.3.3-2 is a Region 1 inner RB allocation if

RBstart ≥ Ceil(1/4 NRB) AND RBend < Ceil(3/4 NRB) AND LCRB ≤ Ceil(1/4 NRB)

An RB allocation is an Edge allocation if it is NOT a Region 1 inner allocation.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.2.3.

##### 6.2.2.3.4	UE maximum output power reduction for power class 4

For power class 4, MPR specified in sub-clause 6.2.2.3.3 applies.

Table 6.2.2.3.4-1: Void

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.2.4.

6.2.2.4	Test description

6.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.2.2.4.1-1 to Table 6.2.2.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.2.4.1-1: Test Configuration Table (Power Class 1, MPRnarrow)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 2 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 3 | Low |  |  |  | CP-OFDM 64 QAM | 2@0 |
| 4 | High |  |  |  | CP-OFDM 64 QAM | 2@NRB-2 |
| 5 | Low |  |  |  | CP-OFDM 64 QAM | 7@0 |
| 6 | High |  |  |  | CP-OFDM 64 QAM | 7@NRB-7 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-2. |  |  |  |  |  |  |

Table 6.2.2.4.1-2: Test Configuration Table (Power Class 1, MPRWT, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@0 |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@NRB-8 |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Mid |  |  |  | DFT-s-OFDM QPSK | Inner_Full_Region2 |
| 5 | Low |  |  |  | DFT-s-OFDM QPSK | 8@0 |
| 6 | High |  |  |  | DFT-s-OFDM QPSK | 8@NRB-8 |
| 7 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 8 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full_Region2 |
| 9 | Low |  |  |  | DFT-s-OFDM 16 QAM | 8@0 |
| 10 | High |  |  |  | DFT-s-OFDM 16 QAM | 8@NRB-8 |
| 11 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 12 | Low |  |  |  | DFT-s-OFDM 64 QAM | 8@0 |
| 13 | High |  |  |  | DFT-s-OFDM 64 QAM | 8@NRB-8 |
| 14 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 15 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Inner_Full_Region2 |
| 16 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full_Region2 |
| 17 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 18 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 19 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 20 | Low |  |  |  | CP-OFDM 16 QAM | 8@0 |
| 21 | High |  |  |  | CP-OFDM 16 QAM | 8@NRB-8 |
| 22 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 23 | Mid |  |  |  | CP-OFDM 16 QAM | Inner_Full_Region2 |
| 24 | Low |  |  |  | CP-OFDM 64 QAM | 8@0 |
| 25 | High |  |  |  | CP-OFDM 64 QAM | 8@NRB-8 |
| 26 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in clause 6.1-2. |  |  |  |  |  |  |

Table 6.2.2.4.1-3: Test Configuration Table (Power Class 1, MPRWT, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@0 |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@NRB-8 |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Full_Region2 |
| 5 | Mid |  |  |  | DFT-s-OFDM QPSK | Inner_Full_Region2 |
| 6 | Low |  |  |  | DFT-s-OFDM QPSK | 8@0 |
| 7 | High |  |  |  | DFT-s-OFDM QPSK | 8@NRB-8 |
| 8 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full_Region2 |
| 10 | Low |  |  |  | DFT-s-OFDM 16 QAM | 8@0 |
| 11 | High |  |  |  | DFT-s-OFDM 16 QAM | 8@NRB-8 |
| 12 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 13 | Low |  |  |  | DFT-s-OFDM 64 QAM | 8@0 |
| 14 | High |  |  |  | DFT-s-OFDM 64 QAM | 8@NRB-8 |
| 15 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 16 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full_Region2 |
| 17 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 18 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 19 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 20 | Low |  |  |  | CP-OFDM 16 QAM | 8@0 |
| 21 | High |  |  |  | CP-OFDM 16 QAM | 8@NRB-8 |
| 22 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 23 | Mid |  |  |  | CP-OFDM 16 QAM | Inner_Full_Region2 |
| 24 | Low |  |  |  | CP-OFDM 64 QAM | 8@0 |
| 25 | High |  |  |  | CP-OFDM 64 QAM | 8@NRB-8 |
| 26 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in clause 6.1-2. |  |  |  |  |  |  |

Table 6.2.2.4.1-4: Void

Table 6.2.2.4.1-5: Void

Table 6.2.2.4.1-6: Void

Table 6.2.2.4.1-7: Test Configuration Table (Power Class 2, 3 and 4, MPRnarrow, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 4 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2.2.4.1-8: Test Configuration Table (Power Class 2, 3 and 4, MPRWT, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 2 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 3 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full |
| 4 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 7 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Inner_Full |
| 8 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 9 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 10 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 11 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full |
| 12 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 13 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 14 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 15 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 16 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 17 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 18 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 19 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 20 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2.2.4.1-8a: Test Configuration Table (Power Class 2, 3, 4, MPRnarrow, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Right |
| 3 | Low |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Left |
| 4 | High |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Right |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2.2.4.1-9: Test Configuration Table (Power Class 2, 3 and 4, MPRWT, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 12 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 13 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 14 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 15 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 16 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 17 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 18 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 19 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 21 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.2.2.4.1-1 to Table 6.2.2.4.1-9.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

6.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.2.4.1-1 to Table 6.2.2.4.1-9. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in 6.2.2.5. EIRP test procedure is defined in Annex K.1.3. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

6.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

NOTE 2:	When switching to DFT-s-OFDM waveform, as specified in Table 6.2.2.4.1-1 to Table 6.2.2.4.1-9, send an NR RRCReconfiguration message according to TS 38.508-1 [10] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.2.2.5	Test requirement

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in following tables.

Table 6.2.2.5-1: UE Power Class test requirements for Power Class 1 (for Bands n257, n258, n261)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.4.1-1 | 1 | 40 | 14.4 | 7 | 18.6-TT | 55 |
|  | 2 | 40 | 14.4 | 7 | 18.6-TT | 55 |
|  | 3 | 40 | 10 | 5 | 25-TT | 55 |
|  | 4 | 40 | 10 | 5 | 25-TT | 55 |
|  | 5 | 40 | 10 | 5 | 25-TT | 55 |
|  | 6 | 40 | 10 | 5 | 25-TT | 55 |
| Table 6.2.2.4.1-2 | 1 | 40 | 5.5 | 5 | 29.5-TT | 55 |
|  | 2 | 40 | 5.5 | 5 | 29.5-TT | 55 |
|  | 3 | 40 | 5.5 | 5 | 29.5-TT | 55 |
|  | 4 | 40 | 3 | 2 | 35-TT | 55 |
|  | 5 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 6 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 7 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 8 | 40 | 4 | 3 | 33-TT | 55 |
|  | 9 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 10 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 11 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 12 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 13 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 14 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 15 | 40 | 5 | 4 | 31-TT | 55 |
|  | 16 | 40 | 4.5 | 4 | 31.5-TT | 55 |
|  | 17 | 40 | 7 | 5 | 28-TT | 55 |
|  | 18 | 40 | 7 | 5 | 28-TT | 55 |
|  | 19 | 40 | 7 | 5 | 28-TT | 55 |
|  | 20 | 40 | 7 | 5 | 28-TT | 55 |
|  | 21 | 40 | 7 | 5 | 28-TT | 55 |
|  | 22 | 40 | 7 | 5 | 28-TT | 55 |
|  | 23 | 40 | 5.5 | 5 | 29.5-TT | 55 |
|  | 24 | 40 | 7.5 | 5 | 27.5-TT | 55 |
|  | 256 | 40 | 7.5 | 5 | 27.5-TT | 55 |
|  | 26 | 40 | 7.5 | 5 | 27.5-TT | 55 |
| Table 6.2.2.4.1-3 | 1 | 40 | 5.5 | 5 | 29.5-TT | 55 |
|  | 2 | 40 | 5.5 | 5 | 29.5-TT | 55 |
|  | 3 | 40 | 5.5 | 5 | 29.5-TT | 55 |
|  | 4 | 40 | 3 | 2 | 35-TT | 55 |
|  | 5 | 40 | 3.5 | 3 | 33.5-TT | 55 |
|  | 6 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 7 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 8 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 9 | 40 | 4.5 | 4 | 31.5-TT | 55 |
|  | 10 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 11 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 12 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 13 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 14 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 15 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 16 | 40 | 5 | 4 | 31-TT | 55 |
|  | 17 | 40 | 7 | 5 | 28-TT | 55 |
|  | 18 | 40 | 7 | 5 | 28-TT | 55 |
|  | 19 | 40 | 7 | 5 | 28-TT | 55 |
|  | 20 | 40 | 7 | 5 | 28-TT | 55 |
|  | 21 | 40 | 7 | 5 | 28-TT | 55 |
|  | 22 | 40 | 7 | 5 | 28-TT | 55 |
|  | 23 | 40 | 6.5 | 5 | 28.5-TT | 55 |
|  | 24 | 40 | 9 | 5 | 26-TT | 55 |
|  | 25 | 40 | 9 | 5 | 26-TT | 55 |
|  | 26 | 40 | 9 | 5 | 26-TT | 55 |

Table 6.2.2.5-1a: UE Power Class test requirements for Power Class 1 (for Bands n260)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.4.1-1 | 1 | 38 | 14.4 | 7 | 16.6-TT | 55 |
|  | 2 | 38 | 14.4 | 7 | 16.6-TT | 55 |
|  | 3 | 38 | 10 | 5 | 23-TT | 55 |
|  | 4 | 38 | 10 | 5 | 23-TT | 55 |
|  | 5 | 38 | 10 | 5 | 23-TT | 55 |
|  | 6 | 38 | 10 | 5 | 23-TT | 55 |
| Table 6.2.2.4.1-2 | 1 | 38 | 5.5 | 5 | 27.5-TT | 55 |
|  | 2 | 38 | 5.5 | 5 | 27.5-TT | 55 |
|  | 3 | 38 | 5.5 | 5 | 27.5-TT | 55 |
|  | 4 | 38 | 3 | 2 | 33-TT | 55 |
|  | 5 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 6 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 7 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 8 | 38 | 4 | 3 | 31-TT | 55 |
|  | 9 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 10 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 11 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 12 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 13 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 14 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 15 | 38 | 5 | 4 | 29-TT | 55 |
|  | 16 | 38 | 4.5 | 4 | 29.5-TT | 55 |
|  | 17 | 38 | 7 | 5 | 26-TT | 55 |
|  | 18 | 38 | 7 | 5 | 26-TT | 55 |
|  | 19 | 38 | 7 | 5 | 26-TT | 55 |
|  | 20 | 38 | 7 | 5 | 26-TT | 55 |
|  | 21 | 38 | 7 | 5 | 26-TT | 55 |
|  | 22 | 38 | 7 | 5 | 26-TT | 55 |
|  | 23 | 38 | 5.5 | 5 | 27.5-TT | 55 |
|  | 24 | 38 | 7.5 | 5 | 25.5-TT | 55 |
|  | 25 | 38 | 7.5 | 5 | 25.5-TT | 55 |
|  | 26 | 38 | 7.5 | 5 | 25.5-TT | 55 |
| Table 6.2.2.4.1-3 | 1 | 38 | 5.5 | 5 | 27.5-TT | 55 |
|  | 2 | 38 | 5.5 | 5 | 27.5-TT | 55 |
|  | 3 | 38 | 5.5 | 5 | 27.5-TT | 55 |
|  | 4 | 38 | 3 | 2 | 33-TT | 55 |
|  | 5 | 38 | 3.5 | 3 | 31.5-TT | 55 |
|  | 6 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 7 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 8 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 9 | 38 | 4.5 | 4 | 29.5-TT | 55 |
|  | 10 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 11 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 12 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 13 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 14 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 15 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 16 | 38 | 5 | 4 | 29-TT | 55 |
|  | 17 | 38 | 7 | 5 | 26-TT | 55 |
|  | 18 | 38 | 7 | 5 | 26-TT | 55 |
|  | 19 | 38 | 7 | 5 | 26-TT | 55 |
|  | 20 | 38 | 7 | 5 | 26-TT | 55 |
|  | 21 | 38 | 7 | 5 | 26-TT | 55 |
|  | 22 | 38 | 7 | 5 | 26-TT | 55 |
|  | 23 | 38 | 6.5 | 5 | 26.5-TT | 55 |
|  | 24 | 38 | 9 | 5 | 24-TT | 55 |
|  | 25 | 38 | 9 | 5 | 24-TT | 55 |
|  | 26 | 38 | 9 | 5 | 24-TT | 55 |

Table 6.2.2.5-1b: Test Tolerance (Power class 1)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 3.38 dB, NTC 3.50 dB, ETC | 3.38 dB, NTC 3.50 dB, ETC |

Table 6.2.2.5-2: UE Power Class test requirements for Power Class 2

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.4.1-7 | 1 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 2 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 3 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 4 | 29 | 2.5 | 2 | 24.5-TT | 43 |
| Table 6.2.2.4.1-8 | 1 | 29 | 2 | 1.5 | 25.5-TT | 43 |
|  | 2 | 29 | 2 | 1.5 | 25.5-TT | 43 |
|  | 3 | 29 | 3 | 2 | 24-TT | 43 |
|  | 4 | 29 | 3.5 | 3 | 22.5-TT | 43 |
|  | 5 | 29 | 3.5 | 3 | 22.5-TT | 43 |
|  | 6 | 29 | 3.5 | 3 | 22.5-TT | 43 |
|  | 7 | 29 | 5 | 4 | 20-TT | 43 |
|  | 8 | 29 | 5.5 | 5 | 18.5-TT | 43 |
|  | 9 | 29 | 5.5 | 5 | 18.5-TT | 43 |
|  | 10 | 29 | 5.5 | 5 | 18.5-TT | 43 |
|  | 11 | 29 | 3.5 | 3 | 22.5-TT | 43 |
|  | 12 | 29 | 4 | 3 | 22-TT | 43 |
|  | 13 | 29 | 4 | 3 | 22-TT | 43 |
|  | 14 | 29 | 4 | 3 | 22-TT | 43 |
|  | 15 | 29 | 5 | 4 | 20-TT | 43 |
|  | 16 | 29 | 5 | 4 | 20-TT | 43 |
|  | 17 | 29 | 5 | 4 | 20-TT | 43 |
|  | 18 | 29 | 7.5 | 5 | 16.5-TT | 43 |
|  | 19 | 29 | 7.5 | 5 | 16.5-TT | 43 |
|  | 20 | 29 | 7.5 | 5 | 16.5-TT | 43 |
| Table 6.2.2.4.1-8a | 1 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 2 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 3 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 4 | 29 | 2.5 | 2 | 24.5-TT | 43 |
| Table 6.2.2.4.1-9 | 1 | 29 | 3 | 2 | 24-TT | 43 |
|  | 2 | 29 | 3 | 2 | 24-TT | 43 |
|  | 3 | 29 | 3 | 2 | 24-TT | 43 |
|  | 4 | 29 | 3 | 2 | 24-TT | 43 |
|  | 5 | 29 | 3 | 2 | 24-TT | 43 |
|  | 6 | 29 | 3 | 2 | 24-TT | 43 |
|  | 7 | 29 | 4.5 | 4 | 20.5-TT | 43 |
|  | 8 | 29 | 4.5 | 4 | 20.5-TT | 43 |
|  | 9 | 29 | 4.5 | 4 | 20.5-TT | 43 |
|  | 10 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 11 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 12 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 13 | 29 | 5 | 4 | 20-TT | 43 |
|  | 14 | 29 | 5 | 4 | 20-TT | 43 |
|  | 15 | 29 | 5 | 4 | 20-TT | 43 |
|  | 16 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 17 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 18 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 19 | 29 | 9 | 5 | 15-TT | 43 |
|  | 20 | 29 | 9 | 5 | 15-TT | 43 |
|  | 21 | 29 | 9 | 5 | 15-TT | 43 |

Table 6.2.2.5-3: UE Power Class test requirements for Power Class 3 (n257, 258, 261)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.4.1-7 | 1 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
| Table 6.2.2.4.1-8 | 1 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 5 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 6 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 7 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 8 | 22.4 | 5.5 | 5 | 11.9-TT-ΔMBP,n | 43 |
|  | 9 | 22.4 | 5.5 | 5 | 11.9-TT-ΔMBP,n | 43 |
|  | 10 | 22.4 | 5.5 | 5 | 11.9-TT-ΔMBP,n | 43 |
|  | 11 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 12 | 22.4 | 4 | 3 | 15.4-TT-ΔMBP,n | 43 |
|  | 13 | 22.4 | 4 | 3 | 15.4-TT-ΔMBP,n | 43 |
|  | 14 | 22.4 | 4 | 3 | 15.4-TT-ΔMBP,n | 43 |
|  | 15 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 16 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 17 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 18 | 22.4 | 7.5 | 5 | 9.9-TT-ΔMBP,n | 43 |
|  | 19 | 22.4 | 7.5 | 5 | 9.9-TT-ΔMBP,n | 43 |
|  | 20 | 22.4 | 7.5 | 5 | 9.9-TT-ΔMBP,n | 43 |
| Table 6.2.2.4.1-8a | 1 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
| Table 6.2.2.4.1-9 | 1 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 5 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 6 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 7 | 22.4 | 4.5 | 4 | 13.9-TT-ΔMBP,n | 43 |
|  | 8 | 22.4 | 4.5 | 4 | 13.9-TT-ΔMBP,n | 43 |
|  | 9 | 22.4 | 4.5 | 4 | 13.9-TT-ΔMBP,n | 43 |
|  | 10 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 11 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 12 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 13 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 14 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 15 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 16 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 17 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 18 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 19 | 22.4 | 9 | 5 | 8.4-TT-ΔMBP,n | 43 |
|  | 20 | 22.4 | 9 | 5 | 8.4-TT-ΔMBP,n | 43 |
|  | 21 | 22.4 | 9 | 5 | 8.4-TT-ΔMBP,n | 43 |
| Note 1: ΔMBP,n is the Multiband Relaxation factor declared by the UE for the tested band in table A.4.3.9-2 of TS38.508-2. This declaration shall fulfil the requirements in clause 6.2.1.1.3.3. Note 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant. Note 3: Max allowed sum of ΔMBP,n over all supported FR2 bands as defined in clause 6.2.1.1.3.3. Note 4: ΔMBP,n is 0 for single band UE. |  |  |  |  |  |  |

Table 6.2.2.5-3a: UE Power Class test requirements for Power Class 3 (n260)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.4.1-7 | 1 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 4 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
| Table 6.2.2.4.1-8 | 1 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 4 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 5 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 6 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 7 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 8 | 20.6 | 5.5 | 5 | 10.1-TT-ΔMBP,n | 43 |
|  | 9 | 20.6 | 5.5 | 5 | 10.1-TT-ΔMBP,n | 43 |
|  | 10 | 20.6 | 5.5 | 5 | 10.1-TT-ΔMBP,n | 43 |
|  | 11 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 12 | 20.6 | 4 | 3 | 13.6-TT-ΔMBP,n | 43 |
|  | 13 | 20.6 | 4 | 3 | 13.6-TT-ΔMBP,n | 43 |
|  | 14 | 20.6 | 4 | 3 | 13.6-TT-ΔMBP,n | 43 |
|  | 15 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 16 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 17 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 18 | 20.6 | 7.5 | 5 | 8.1-TT-ΔMBP,n | 43 |
|  | 19 | 20.6 | 7.5 | 5 | 8.1-TT-ΔMBP,n | 43 |
|  | 20 | 20.6 | 7.5 | 5 | 8.1-TT-ΔMBP,n | 43 |
| Table 6.2.2.4.1-8a | 1 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 4 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
| Table 6.2.2.4.1-9 | 1 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 4 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 5 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 6 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 7 | 20.6 | 4.5 | 4 | 12.1-TT-ΔMBP,n | 43 |
|  | 8 | 20.6 | 4.5 | 4 | 12.1-TT-ΔMBP,n | 43 |
|  | 9 | 20.6 | 4.5 | 4 | 12.1-TT-ΔMBP,n | 43 |
|  | 10 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 11 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 12 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 13 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 14 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 15 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 16 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 17 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 18 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 19 | 20.6 | 9 | 5 | 6.6-TT-ΔMBP,n | 43 |
|  | 20 | 20.6 | 9 | 5 | 6.6-TT-ΔMBP,n | 43 |
|  | 21 | 20.6 | 9 | 5 | 6.6-TT-ΔMBP,n | 43 |
| Note 1: ΔMBP,n is the Multiband Relaxation factor declared by the UE for the tested band in table A.4.3.9-2 of TS38.508-2. This declaration shall fulfil the requirements in clause 6.2.1.1.3.3. Note 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant. Note 3: Max allowed sum of ΔMBP,n over all supported FR2 bands as defined in clause 6.2.1.1.3.3. Note 4: ΔMBP,n is 0 for single band UE. |  |  |  |  |  |  |

Table 6.2.2.5-3b: Test Tolerance (Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 3.11 dB, NTC 3.30 dB, ETC | 3.11 dB, NTC 3.30 dB, ETC |

Table 6.2.2.5-4: UE Power Class test requirements for Power Class 4 (n257, 258, 261)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.4.1-7 | 1 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 2 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 3 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 4 | 34 | 2.5 | 2 | 29.5-TT | 43 |
| Table 6.2.2.4.1-8 | 1 | 34 | 2 | 1.5 | 30.5-TT | 43 |
|  | 2 | 34 | 2 | 1.5 | 30.5-TT | 43 |
|  | 3 | 34 | 3 | 2 | 29-TT | 43 |
|  | 4 | 34 | 3.5 | 3 | 27.5-TT | 43 |
|  | 5 | 34 | 3.5 | 3 | 27.5-TT | 43 |
|  | 6 | 34 | 3.5 | 3 | 27.5-TT | 43 |
|  | 7 | 34 | 5 | 4 | 25-TT | 43 |
|  | 8 | 34 | 5.5 | 5 | 23.5-TT | 43 |
|  | 9 | 34 | 5.5 | 5 | 23.5-TT | 43 |
|  | 10 | 34 | 5.5 | 5 | 23.5-TT | 43 |
|  | 11 | 34 | 3.5 | 3 | 27.5-TT | 43 |
|  | 12 | 34 | 4 | 3 | 27-TT | 43 |
|  | 13 | 34 | 4 | 3 | 27-TT | 43 |
|  | 14 | 34 | 4 | 3 | 27-TT | 43 |
|  | 15 | 34 | 5 | 4 | 25-TT | 43 |
|  | 16 | 34 | 5 | 4 | 25-TT | 43 |
|  | 17 | 34 | 5 | 4 | 25-TT | 43 |
|  | 18 | 34 | 7.5 | 5 | 1.5-TT | 43 |
|  | 19 | 34 | 7.5 | 5 | 1.5-TT | 43 |
|  | 20 | 34 | 7.5 | 5 | 1.5-TT | 43 |
| Table 6.2.2.4.1-8a | 1 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 2 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 3 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 4 | 34 | 2.5 | 2 | 29.5-TT | 43 |
| Table 6.2.2.4.1-9 | 1 | 34 | 3 | 2 | 29-TT | 43 |
|  | 2 | 34 | 3 | 2 | 29-TT | 43 |
|  | 3 | 34 | 3 | 2 | 29-TT | 43 |
|  | 4 | 34 | 3 | 2 | 29-TT | 43 |
|  | 5 | 34 | 3 | 2 | 29-TT | 43 |
|  | 6 | 34 | 3 | 2 | 29-TT | 43 |
|  | 7 | 34 | 4.5 | 4 | 25.5-TT | 43 |
|  | 8 | 34 | 4.5 | 4 | 25.5-TT | 43 |
|  | 9 | 34 | 4.5 | 4 | 25.5-TT | 43 |
|  | 10 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 11 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 12 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 13 | 34 | 5 | 4 | 25-TT | 43 |
|  | 14 | 34 | 5 | 4 | 25-TT | 43 |
|  | 15 | 34 | 5 | 4 | 25-TT | 43 |
|  | 16 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 17 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 18 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 19 | 34 | 9 | 5 | 20-TT | 43 |
|  | 20 | 34 | 9 | 5 | 20-TT | 43 |
|  | 21 | 34 | 9 | 5 | 20-TT | 43 |

Table 6.2.2.5-4a: UE Power Class test requirements for Power Class 4 (n260)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Table 6.2.2.4.1-7 | 1 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 2 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 3 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 4 | 31 | 2.5 | 2 | 26.5-TT | 43 |
| Table 6.2.2.4.1-8 | 1 | 31 | 2 | 1.5 | 27.5-TT | 43 |
|  | 2 | 31 | 2 | 1.5 | 27.5-TT | 43 |
|  | 3 | 31 | 3 | 2 | 26-TT | 43 |
|  | 4 | 31 | 3.5 | 3 | 24.5-TT | 43 |
|  | 5 | 31 | 3.5 | 3 | 24.5-TT | 43 |
|  | 6 | 31 | 3.5 | 3 | 24.5-TT | 43 |
|  | 7 | 31 | 5 | 4 | 22-TT | 43 |
|  | 8 | 31 | 5.5 | 5 | 20.5-TT | 43 |
|  | 9 | 31 | 5.5 | 5 | 20.5-TT | 43 |
|  | 10 | 31 | 5.5 | 5 | 20.5-TT | 43 |
|  | 11 | 31 | 3.5 | 3 | 24.5-TT | 43 |
|  | 12 | 31 | 4 | 3 | 24-TT | 43 |
|  | 13 | 31 | 4 | 3 | 24-TT | 43 |
|  | 14 | 31 | 4 | 3 | 24-TT | 43 |
|  | 15 | 31 | 5 | 4 | 22-TT | 43 |
|  | 16 | 31 | 5 | 4 | 22-TT | 43 |
|  | 17 | 31 | 5 | 4 | 22-TT | 43 |
|  | 18 | 31 | 7.5 | 5 | 18.5-TT | 43 |
|  | 19 | 31 | 7.5 | 5 | 18.5-TT | 43 |
|  | 20 | 31 | 7.5 | 5 | 18.5-TT | 43 |
| Table 6.2.2.4.1-8a | 1 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 2 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 3 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 4 | 31 | 2.5 | 2 | 26.5-TT | 43 |
| Table 6.2.2.4.1-9 | 1 | 31 | 3 | 2 | 26-TT | 43 |
|  | 2 | 31 | 3 | 2 | 26-TT | 43 |
|  | 3 | 31 | 3 | 2 | 26-TT | 43 |
|  | 4 | 31 | 3 | 2 | 26-TT | 43 |
|  | 5 | 31 | 3 | 2 | 26-TT | 43 |
|  | 6 | 31 | 3 | 2 | 26-TT | 43 |
|  | 7 | 31 | 4.5 | 4 | 22.5-TT | 43 |
|  | 8 | 31 | 4.5 | 4 | 22.5-TT | 43 |
|  | 9 | 31 | 4.5 | 4 | 22.5-TT | 43 |
|  | 10 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 11 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 12 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 13 | 31 | 5 | 4 | 22-TT | 43 |
|  | 14 | 31 | 5 | 4 | 22-TT | 43 |
|  | 15 | 31 | 5 | 4 | 22-TT | 43 |
|  | 16 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 17 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 18 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 19 | 31 | 9 | 5 | 17-TT | 43 |
|  | 20 | 31 | 9 | 5 | 17-TT | 43 |
|  | 21 | 31 | 9 | 5 | 17-TT | 43 |

## <<< Skip unchanged sections >>>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±4.89 dB (FR2a, NTC testing) ±5.09 dB (FR2b, NTC testing) ±5.17 dB (FR2a, ETC testing) ±5.37 dB (FR2b, ETC testing) PC1 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±[5.33] dB (FR2a, NTC testing) ±TBD dB (FR2b, NTC testing) ±TBD dB (FR2a, ETC testing) ±TBD dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.3-1 in TR 38.903) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP Max Device size ≤ 30 cm ±4.42 dB (FR2a, NTC testing) ±4.62 dB (FR2b, NTC testing) ±4.70 dB (FR2a, ETC testing) ±4.90 dB (FR2b, ETC testing) PC1 Max TRP Max Device size ≤ 30 cm ±[4.64] dB (FR2a, NTC testing) ± TBD dB (FR2b, NTC testing) ± TBD dB (FR2a, ETC testing) ± TBD dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.3-2 in TR 38.903) |
| 6.2.1.1_1 UE maximum output power – EIRP (Rel-16 and forward) | Same as 6.2.1.1 |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC3 Max Device size ≤ 30 cm ±4.60 dB (FR2a) ±5.20 dB (FR2b) PC1 Max Device size ≤ 30 cm TBD | MTSU = 1.00 x MU (from Table B.3-3 in TR 38.903) |
| 6.2.1.2_1 UE maximum output power – Spherical coverage (Rel16 and forward) | Same as 6.2.1.2 |  |
| 6.2.2 UE maximum output power reduction | PC3 Max Device size ≤ 30 cm ±4.92 dB (FR2a), NTC testing ±5.10 dB (FR2b), NTC testing ±5.20 dB (FR2a, ETC testing) ±5.38 dB (FR2b, ETC testing) PC1 Max Device size ≤ 30 cm ±5.30 dB (FR2a), NTC testing ±5.50 dB (FR2b), NTC testing ±5.57 dB (FR2a, ETC testing) ±5.77 dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.4-1 in TR 38.903) |
| 6.2.2_1 UE maximum output power reduction enhancements | Same as 6.2.2 |  |
| 6.2.3 UE maximum output power with additional requirements | Same as 6.2.2 |  |

## <<< Skip unchanged sections >>>

## F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 2.87 dB (FR2a, NTC) 2.87 dB (FR2b, NTC) 3.04 dB (FR2a, ETC) 3.04 dB (FR2b, ETC) PC1 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) [3.12] dB (FR2a, NTC) TBD dB (FR2b, NTC) TBD dB (FR2a, ETC) TBD dB (FR2b, ETC) Max EIRP 0 dB | PC3 Minimum peak EIRP TT = 0.60 x (MTSUIFF - 0.1) (FR2a) TT = 0.60 x (MTSUIFF - 0.3) (FR2b) PC1 Minimum peak EIRP TT = 0.60 x (MTSUIFF – [0.13]) (FR2a) TT = TBD (FR2b) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP IFF (Max Device size ≤ 30 cm) 2.65 dB (FR2a, NTC) 2.77 dB (FR2b, NTC) 2.82 dB (FR2a, ETC) 2.94 dB (FR2b, ETC) PC1 Max TRP IFF (Max Device size ≤ 30 cm) [2.78] dB (FR2a, NTC) TBD dB (FR2b, NTC) TBD dB (FR2a, ETC) TBD dB (FR2b, ETC) | Max TRP TT = 0.60 x MTSUIFF |
| 6.2.1.1_1 UE maximum output power – EIRP (Rel-16 and forward) | Same as 6.2.1.1 |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC1 TBD PC2 TBD PC3 IFF (Max Device size ≤ 30 cm) 2.58 dB (FR2a) 2.58 dB (FR2b) PC4 TBD | PC3 TT = 0.60 x (MTSUIFF - 0.3) (FR2a) TT = 0.60 x (MTSUIFF - 0.9) (FR2b) PC1 TBD |
| 6.2.1.2_1 UE maximum output power – Spherical coverage (Rel16 and forward) | Same as 6.2.1.2 |  |
| 6.2.2 UE maximum output power reduction | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.11 dB (FR2a, NTC) 3.11 dB (FR2b, NTC) 3.30 dB (FR2a, ETC) 3.30 dB (FR2b, ETC) PC1 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.38 dB (FR2a, NTC) 3.38 dB (FR2b, NTC) 3.50 dB (FR2a, ETC) 3.50 dB (FR2b, ETC) | Minimum peak EIRP PC3 TT = 0.65 x (MTSUIFF - 0.13) (FR2a) TT = 0.65 x (MTSUIFF - 0.31) (FR2b) PC1 TT = 0.65 x (MTSUIFF - 0.1) (FR2a) TT = 0.65 x (MTSUIFF - 0.3) (FR2b) |
| 6.2.2_1 UE maximum output power reduction enhancements | Same as 6.2.2 |  |
| 6.2.3 UE maximum output power with additional requirements | Same as 6.2.2 |  |

## <<< END OF CHANGES >>>