3GPP TSG-RAN5 Meeting #97	R5-226108
Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0385 | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PC1 MU - General Update in 38.903 section B.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PC1 MUs definition continues.The agreements made in sections B.3 to B.25 for specific test cases must be captured accordingly in section B.2.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Summarized the MU factors agreed for PC1 during this meeting. Original proposal is based on proposals made in R5-226109. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There might be incosnsitency between PC1 MU values listed and section B.2.2 and the values provided in sections B.3 to B.25. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## B.2.2	Measurement error contribution descriptions for IFF

### B.2.2.1	Positioning misalignment

See B.2.1.1.

The uncertainty value of positioning misalignment is estimated as below table and used across clause B.

Table B.2.2.1-1: Uncertainty value for positioning misalignment for IFF

| Power class | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- |
| PC1 | 0.02 | Normal | 2.00 | 0.01 |
| PC3 | 0.00 | Normal | 2.00 | 0.00 |

### B.2.2.2	Measure distance uncertainty

See B.2.1.2. For IFF1 this can be considered to be zero.

The uncertainty value of measure distance uncertainty is estimated as below table and used across clause B.

Table B.2.2.2-1: Uncertainty value for measure distance uncertainty for IFF

| Power class | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- |
| PC1, PC3 | 0.00 | Rectangular | 1.73 | 0.00 |

### B.2.2.3	Quality of Quiet Zone

See B.2.1.3.

The uncertainty value of quality of quiet zone is estimated as below table and used across clause B.

Table B.2.2.3-1: Uncertainty value for quality of quiet zone for IFF

| QZ size | Power class | Condition | Test case | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <= 30cm | PC1, PC3 | NC | NOTE1 | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  | ACLR (relative measurement) | 0.52 | Actual | 1.00 | 0.52 |
|  |  |  | SE (6GHz to 12.75GHz) | 0.7 | Actual | 1.00 | 0.7 |
|  |  |  | SE (12.75GHz to 23.45GHz) | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  | SE (23.45GHz to 40.8GHz) | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  | SE (40.8GHz to 66GHz) | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  | SE (66GHz to 80GHz) | 0.6 | Actual | 1.00 | 0.6 |
|  |  | ETC | NOTE1 | 0.9 | Actual | 1.00 | 0.9 |
|  |  |  | ACLR (relative measurement) | 0.52 | Actual | 1.00 | 0.52 |
| NOTE 1: The uncertainty in current row applies to maximum output power with EIRP and TRP, EIRP spherical coverage, MPR, configured output power with power boost, minimum output power, transmit OFF power, spectrum emission mask, reference sensitivity, adjacent selectivity, in-band blocking. |  |  |  |  |  |  |  |

### B.2.2.4	Mismatch

See B.2.1.4.

The uncertainty value of mismatch is estimated as below table and used across clause B.

Table B.2.2.4-2: Uncertainty value for mismatch for IFF

| QZ size | Power class | Condition | Test case | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |
| <= 30cm | PC3 | NC | Default | 1.30 | Actual | 1.00 | 1.30 |
|  |  |  | ACLR (relative measurement) | 1.84 | Actual | 1.00 | 1.84 |
|  |  |  | Tx SE (6GHz to 12.75GHz) | 1.5 | Actual | 1.00 | 1.5 |
|  |  |  | Tx SE (12.75GHz to 23.45GHz) | 1.5 | Actual | 1.00 | 1.5 |
|  |  |  | Tx SE (23.45GHz to 40.8GHz) | 1.4 | Actual | 1.00 | 1.4 |
|  |  |  | Tx SE (40.8GHz to 66GHz) | 2.3 | Actual | 1.00 | 2.3 |
|  |  |  | Tx SE (66GHz to 80GHz) | 2.3 | Actual | 1.00 | 2.3 |
|  |  |  | Rx SE (6GHz to 12.75GHz) | 1.6 | Actual | 1.00 | 1.6 |
|  |  |  | Rx SE (12.75GHz to 23.45GHz) | 1.6 | Actual | 1.00 | 1.6 |
|  |  |  | Rx SE (23.45GHz to 40.8GHz) | 1.5 | Actual | 1.00 | 1.5 |
|  |  |  | Rx SE (40.8GHz to 66GHz) | 2.3 | Actual | 1.00 | 2.3 |
|  |  |  | Rx SE (66GHz to 80GHz) | 2.3 | Actual | 1.00 | 2.3 |
|  |  | ETC | Default | 1.30 | Actual | 1.00 | 1.30 |
|  |  |  | ACLR (relative measurement) | 1.84 | Actual | 1.00 | 1.84 |
|  |  |  | SE |  |  |  |  |
|  | PC1 |  | Default | 1.30 | Actual | 1.00 | 1.30 |
|  |  |  | ACLR (relative measurement) | 1.84 | Actual | 1.00 | 1.84 |
|  |  |  | Tx SE (6GHz to 12.75GHz) | 1.5 | Actual | 1.00 | 1.5 |
|  |  |  | Tx SE (12.75GHz to 23.45GHz) | 1.5 | Actual | 1.00 | 1.5 |
|  |  |  | Tx SE (23.45GHz to 40.8GHz) | 1.4 | Actual | 1.00 | 1.4 |
|  |  |  | Tx SE (40.8GHz to 66GHz) | 2.3 | Actual | 1.00 | 2.3 |
|  |  |  | Tx SE (66GHz to 80GHz) | 2.3 | Actual | 1.00 | 2.3 |
|  |  |  | Rx SE (6GHz to 12.75GHz) | 1.6 | Actual | 1.00 | 1.6 |
|  |  |  | Rx SE (12.75GHz to 23.45GHz) | 1.6 | Actual | 1.00 | 1.6 |
|  |  |  | Rx SE (23.45GHz to 40.8GHz) | 1.5 | Actual | 1.00 | 1.5 |
|  |  |  | Rx SE (40.8GHz to 66GHz) | 2.3 | Actual | 1.00 | 2.3 |
|  |  |  | Rx SE (66GHz to 80GHz) | 2.3 | Actual | 1.00 | 2.3 |
|  |  | ETC | Default | 1.30 | Actual | 1.00 | 1.30 |
|  |  |  | ACLR (relative measurement) | 1.84 | Actual | 1.00 | 1.84 |
|  |  |  | SE |  |  |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |
| <= 30cm | PC1, PC3 | NC | All | 0.00 | U-shaped | 1.41 | 0.00 |
|  |  | ETC | All | 0.00 | U-shaped | 1.41 | 0.00 |

### B.2.2.5	Standing wave between DUT and measurement antenna

See B.2.1.5.

The uncertainty value of standing wave between the DUT and measurement antenna is estimated as below table and used across clause B.

Table B.2.2.5-1: Uncertainty value for standing wave between the DUT and measurement antenna for IFF

| Power class | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- |
| PC1, PC3 | 0.00 | U-shaped | 1.41 | 0.00 |

### B.2.2.6	Uncertainty of the RF power measurement equipment

See B.2.1.6.

The uncertainty value of RF power measurement equipment is estimated as below table and used across clause B.

Table B.2.2.6-1: Uncertainty value for RF power measurement equipment for IFF

| Power class | Test case | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| PC3 | MOP, MPR, Configured output power with power boost,SEM, ACLR | 2.16 | Normal | 2.00 | 1.08 |
|  | Minimum output power, OFF power | 2.50 | Normal | 2.00 | 1.25 |
|  | SE (6GHz to 12.75GHz) | 2.00 | Normal | 2.00 | 1.00 |
|  | SE (12.75GHz to 23.45GHz) | 2.16 | Normal | 2.00 | 1.08 |
|  | SE (23.45GHz to 40.8GHz) | 2.73 | Normal | 2.00 | 1.37 |
|  | SE (40.8GHz to 66GHz) | 4.00 | Normal | 2.00 | 2.00 |
|  | SE (66GHz to 80GHz) | 4.00 | Normal | 2.00 | 2.00 |
| PC1 | MOP, MPR, SEM, ACLR | 2.16 | Normal | 2.00 | 1.08 |
|  | Minimum output power, OFF power | 2.50 | Normal | 2.00 | 1.25 |
|  | TX SE (6GHz to 12.75GHz) | 2.00 | Normal | 2.00 | 1.00 |
|  | TX SE (12.75GHz to 23.45GHz) | 2.16 | Normal | 2.00 | 1.08 |
|  | TX SE (23.45GHz to 40.8GHz) | 2.73 | Normal | 2.00 | 1.37 |
|  | TX SE (40.8GHz to 66GHz) | 4.00 | Normal | 2.00 | 2.00 |
|  | TX SE (66GHz to 80GHz) | 4.00 | Normal | 2.00 | 2.00 |
|  | RX SE (6GHz to 12.75GHz) | 2.00 | Normal | 2.00 | 1.00 |
|  | RX SE (12.75GHz to 23.45GHz) | 2.16 | Normal | 2.00 | 1.08 |
|  | RX SE (23.45GHz to 40.8GHz) | 2.73 | Normal | 2.00 | 1.37 |
|  | RX SE (40.8GHz to 66GHz) | 4.00 | Normal | 2.00 | 2.00 |
|  | RX SE (66GHz to 80GHz) | 4.00 | Normal | 2.00 | 2.00 |

### B.2.2.7	Phase Curvature

See B.2.1.7. For IFF1 this can be considered to be zero.

The uncertainty value of phase curvature is estimated as below table and used across clause B.

Table B.2.2.7-1: Uncertainty value for phase curvature for IFF

| Power class | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- |
| PC1, PC3 | 0.00 | U-shaped | 1.41 | 0.00 |

### B.2.2.8	Amplifier Uncertainties

See B.2.1.8.

The uncertainty value of amplifier uncertainties is estimated as below table and used across clause B.

Table B.2.2.8-1: Uncertainty value for amplifier uncertainties for IFF

| Power class |  | Test case |  | Uncertainty value |  | Distribution of the probability |  | Divisor |  | Standard uncertainty (σ) [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |
| PC1 |  | Default |  | 2.10 |  | Normal |  | 2.00 |  | 1.05 |  |
|  |  | Relative power tolerance |  | TBD |  | Rectangular |  | 1.73 |  | TBD |  |
|  |  | SE (66GHz to 80GHz) |  | 3.0 |  | Normal |  | 2.00 |  | 1.50 |  |
|  |  | RX SE (66GHz to 80GHz) |  | 3.0 |  | Normal |  | 2.00 |  | 1.50 |  |
| PC3 |  | Default |  | 2.10 |  | Normal |  | 2.00 |  | 1.05 |  |
|  |  | Relative power tolerance |  | 0.5 |  | Rectangular |  | 1.73 |  | 0.29 |  |
|  |  | SE (66GHz to 80GHz) |  | 3.0 |  | Normal |  | 2.00 |  | 1.50 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |
| PC1, PC3 |  | Default |  | 0.00 |  | Normal |  | 2.00 |  | 0.00 |  |

### B.2.2.9	Random uncertainty

See B.2.1.9.

The uncertainty value of random uncertainty is estimated as below table and used across clause B.

Table B.2.2.9-1: Uncertainty value for random uncertainty for IFF

| Power class | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- |
| PC1, PC3 | 0.5 | Normal | 2.00 | 0.25 |

### B.2.2.10	Influence of XPD

See B.2.1.10.

The uncertainty value of influence of the XPD is estimated as below table and used across clause B.

Table B.2.2.10-2: Uncertainty value for influence of the XPD for IFF

| Power class | Test case | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| PC1, PC3 | Default | 0.01 | U-shaped | 1.41 | 0.00 |
|  | ACLR | 0.00 | U-shaped | 1.41 | 0.00 |
|  | SE (6GHz to 12.75GHz) | 0.09 | U-shaped | 1.41 | 0.064 |
|  | SE (12.75GHz to 23.45GHz) | 0.09 | U-shaped | 1.41 | 0.064 |
|  | SE (23.45GHz to 40.8GHz) | 0.01 | U-shaped | 1.41 | 0.00 |
|  | SE (40.8GHz to 66GHz) | 0.09 | U-shaped | 1.41 | 0.064 |
|  | SE (66GHz to 80GHz) | 0.09 | U-shaped | 1.41 | 0.064 |

### B.2.2.11	Insertion Loss Variation

See B.2.1.11.

The uncertainty value of insertion loss variantion is estimated as below table and used across clause B.

Table B.2.2.11-1: Uncertainty value for insertion loss variantion for IFF

| Power class | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |
| PC1, PC3 | 0.00 | Rectangular | 1.73 | 0.00 |
| Stage 1: Calibration measurement |  |  |  |  |
| PC1, PC3 | 0.00 | Rectangular | 1.73 | 0.00 |

### B.2.2.12	RF leakage (from measurement antenna to receiver/transmitter)

See B.2.1.12.

The uncertainty value of RF leakage is estimated as below table and used across clause B.

Table B.2.2.12-1: Uncertainty value for RF leakage for IFF

| Power class | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- |
| PC1, PC3 | 0.00 | Actual | 1.00 | 0.00 |

### B.2.2.13	Misalignment of positioning system

See B.2.1.13.

The uncertainty value of misalignment of positioning system is estimated as below table and used across clause B.

Table B.2.2.13-1: Uncertainty value for misalignment of positioning system for IFF

| Power class | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- |
| PC1, PC3 | 0.00 | Normal | 2.00 | 0.00 |

### B.2.2.14	Uncertainty of the Network Analyzer

See B.2.1.14.

The uncertainty value of uncertainty of the network analyzer is estimated as below table and used across clause B.

Table B.2.2.14-1: Uncertainty value for uncertainty of the network analyser for IFF

| Power class | Test case | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| PC1 | Default (6GHz to 40.8GHz) | 1.50 | Normal | 2.00 | 0.75 |
|  | SE (40.8GHz to 66GHz) | 1.70 | Normal | 2.00 | 0.85 |
|  | SE (66GHz to 80GHz) | 1.70 | Normal | 2.00 | 0.85 |
| PC3 | Default | 0.73 | Normal | 2.00 | 0.37 |
|  | Minimum output power, OFF power (EIRP, TRP), ACLR | 1.50 | Normal | 2.00 | 0.75 |
|  | SE (6GHz to 12.75GHz) | 0.90 | Normal | 2.00 | 0.45 |
|  | SE (12.75GHz to 23.45GHz) | 0.90 | Normal | 2.00 | 0.45 |
|  | SE (23.45GHz to 40.8GHz) | 1.50 | Normal | 2.00 | 0.75 |
|  | SE (40.8GHz to 66GHz) | 1.70 | Normal | 2.00 | 0.85 |
|  | SE (66GHz to 80GHz) | 1.70 | Normal | 2.00 | 0.85 |

### B.2.2.15	Uncertainty of the absolute gain of the calibration antenna

See B.2.1.15.

The uncertainty value of uncertainty of the absolute gain of the calibration antenna is estimated as below table and used across clause B.

Table B.2.2.15-1: Uncertainty value for uncertainty of the absolute gain of the calibration antenna for IFF

| Power class | Test case | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| PC1, PC3 | Default | 0.60 | Normal | 2.00 | 0.30 |
|  | SE (40.8GHz to 66GHz) | 1.70 | Normal | 2.00 | 0.85 |
|  | SE (66GHz to 80GHz) | 1.70 | Normal | 2.00 | 0.85 |

### B.2.2.16	Positioning and pointing misalignment between the reference antenna and the measurement antenna

See B.2.1.16.

The uncertainty value of positioning and pointing misalignment between the reference antenna and the measurement antenna is estimated as below table and used across clause B.

Table B.2.2.16-1: Uncertainty value for positioning and pointing misalignment between the reference antenna and the measurement antenna for IFF

| Power class | Test case | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| PC1, PC3 | Default | 0.01 | Rectangular | 1.73 | 0.00 |
|  | ACLR | 0.00 | Rectangular | 1.73 | 0.00 |
|  | SE | 0.05 | Rectangular | 1.73 | 0.03 |

### B.2.2.17	gNB emulator uncertainty

See B.2.1.17.

The uncertainty value of gNB emulator uncertainty is estimated as below table and used across clause B.

Table B.2.2.17-1: Uncertainty value for gNB emulator uncertainty for IFF

| Power class |  | Uncertainty value |  | Distribution of the probability |  | Divisor |  | Standard uncertainty (σ) [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC1 |  | 2.9 |  | Normal |  | 2.00 |  | 1.45 |  |
| PC3 |  | 2.9 |  | Normal |  | 2.00 |  | 1.45 |  |

### B.2.2.18	Phase centre offset of calibration

See B.2.1.18. For IFF1 this can be considered to be zero.

The uncertainty value of phase centre offset of calibration is estimated as below table and used across clause B.

Table B.2.2.18-1: Uncertainty value for phase centre offset of calibration for IFF

| Power class | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- |
| PC1, PC3 | 0.00 | Rectangular | 1.73 | 0.00 |

### B.2.2.19	Quality of the Quiet Zone for Calibration Process

See B.2.1.19.

The uncertainty value of quality of quiet zone for calibration process is estimated as below table and used across clause B.

Table B.2.2.19-1: Uncertainty value for quiet zone for calibration process for IFF

| QZ size | Power class | Condition | Test case | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <= 30cm | PC1, PC3 | NC | NOTE1 | 0.4 | Actual | 1.00 | 0.4 |
|  |  |  | ACLR (relative measurement) | 0.32 | Actual | 1.00 | 0.32 |
|  |  |  | SE (6GHz to 12.75GHz) | 0.7 | Actual | 1.00 | 0.7 |
|  |  |  | SE (12.75GHz to 23.45GHz) | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  | SE (23.45GHz to 40.8GHz) | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  | SE (40.8GHz to 66GHz) | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  | SE (66GHz to 80GHz) | 0.6 | Actual | 1.00 | 0.6 |
|  |  | ETC | NOTE1 | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  | ACLR (relative measurement) | 0.32 | Actual | 1.00 | 0.32 |
| NOTE 1: The uncertainty in current row applies to maximum output power with EIRP and TRP, EIRP spherical coverage, MPR, configured output power with power boost, minimum output power, transmit OFF power, spectrum emission mask, reference sensitivity, adjacent selectivity, in-band blocking. |  |  |  |  |  |  |  |

### B.2.2.20	Standing wave between reference calibration antenna and measurement antenna

See B.2.1.20.

The uncertainty value of standing wave between reference calibration antenna and measurement antenna is estimated as below table and used across clause B.

Table B.2.2.20-1: Uncertainty value for standing wave between reference calibration antenna and measurement antenna for IFF

| Power class | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- |
| PC1, PC3 | 0.00 | U-shaped | 1.41 | 0.00 |

### B.2.2.21	Influence of the calibration antenna feed cable (Flexing cables, adapters, attenuators, connector repeatability)

See B.2.1.21.

The uncertainty value of influence of the calibration antenna feed cable is estimated as below table and used across clause B.

Table B.2.2.21-1: Uncertainty value for influence of the calibration antenna feed cable for IFF

| Power class | Test case | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| PC1, PC3 | Default | 0.14 | Normal | 2.00 | 0.07 |
|  | SE (40.8GHz to 66GHz) | 0.28 | Normal | 2.00 | 0.14 |
|  | SE (66GHz to 80GHz) | 0.28 | Normal | 2.00 | 0.14 |

### B.2.2.22	Influence of TRP measurement grid

See B.2.1.22.

The uncertainty value of influence of TRP measurement grid is estimated as below table and used across clause B.

Table B.2.2.22-1: Uncertainty value for influence of TRP measurement grid for IFF

| Power class | Test case | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| PC1 (Note 1) | Default | 0.25 | Actual | 1.00 | 0.25 |
|  | SE | 0.25 | Actual | 1.00 | 0.25 |
| PC3 | Default | 0.25 | Actual | 1.00 | 0.25 |
|  |  | SE |  | 0.32 |  | Actual |  | 1.00 |  | 0.32 |  |
| Note 1: A finer measurement grid for SE (when compared to default) is required, i.e., a constant-step size grid with 2522 unique grid points (Dq=Df=5°) or a constant-density grid with 1600 grid points. |  |  |  |  |  |

### B.2.2.23	Influence of beam peak search grid

See B.2.1.23.

The uncertainty value of influence of beam peak search grid is estimated as below table and used across clause B.

Table B.2.2.23-1: Uncertainty value for influence of beam peak search grid for IFF

| Power class | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- |
| PC1, PC3 | 0.00 | Actual | 1.00 | 0.00 |

### B.2.2.24	Systematic error due to TRP calculation/quadrature

See B.2.1.24.

The uncertainty value of systematic error due to TRP calculation/quadrature is estimated as below table and used across clause B.

Table B.2.2.24-1: Uncertainty value for systematic error due to TRP calculation/quadrature for IFF

| Power class | Uncertainty value |
| --- | --- |
| PC1, PC3 | 0.00 |

### B.2.2.25	Multiple measurement antenna uncertainty

See B.2.1.25.

The uncertainty value of multiple measurement antenna uncertainty is estimated as below table and used across clause B.

Table B.2.2.25-1: Uncertainty value for multiple measurement antenna uncertainty for IFF

| Power class | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- |
| PC1, PC3 | 0.15 | Actual | 1.00 | 0.15 |

### B.2.2.26	DUT repositioning

See B.2.1.26.

The uncertainty value of DUT repositioning is estimated as below table and used across clause B.

Table B.2.2.26-1: Uncertainty value for DUT repositioning for IFF

| Power class | Test case | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| PC1 | TRP, spherical coverage | 0.00 | Rectangular | 1.73 | 0.00 |
|  | EIRP, EIS | 0.35 | Rectangular | 1.73 | 0.20 |
| PC3 | TRP, spherical coverage | 0.00 | Rectangular | 1.73 | 0.00 |
|  | EIRP, EIS | 0.08 | Rectangular | 1.73 | 0.05 |

### B.2.2.27	Influence of noise

See B.2.1.27.

The uncertainty value of influence of noise is estimated as below table and used across clause B.

Editor’s Note: For ACLR, all applicable configurations need to be added.

Table B.2.2.27-1: Uncertainty value for influence of noise for PC3 for IFF

| Test case |  | Frequency range |  | Noise floor |  | Minimum requirement |  | Estimated SNRtotal [dB/400MHz] |  | Relaxation |  | Influence of noise |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MOP-EIRP |  | FR2a |  | N/A |  | 20.7dBm/ChBW (22.4-1.7) |  | 16.33 (NOTE 1) |  | 0 |  | 0.1 |  |
|  |  | FR2b |  | N/A |  | 18.9dBm/ChBW (20.6-1.7) |  | 11.45 (NOTE 1) |  | 0 |  | 0.3 |  |
| MOP-TRP |  | FR2a |  | N/A |  | 23dBm/ChBW |  | 16.33 (NOTE 1) |  | 0 |  | 0.1 |  |
|  |  | FR2b |  | N/A |  | 23dBm/ChBW |  | 11.45 (NOTE 1) |  | 0 |  | 0.3 |  |
| MOP-Spherical |  | FR2a |  | N/A |  | 9.75dBm/ChBW (Spherical – MBR= 11.5-1.75) |  | 11.45 (NOTE 1) |  | 0 |  | 0.3 |  |
|  |  | FR2b |  | N/A |  | 7.6dBm/ChBW (Spherical – MBR=8-0.4) |  | 6.37 (NOTE 1) |  | 0 |  | 0.9 |  |
| MPR |  | FR2a |  | -7.6dBm/400MHz |  | 7.65dBm/ChBW (EIRP-MPB-MPR-T(MPR)=22.4-0.75-9-5) |  | 15.17 (NOTE 1) |  | 0 |  | 0.13 |  |
|  |  | FR2b |  | -5.5dBm/400MHz |  | 5.85dBm/ChBW (EIRP-MPB-MPR-T(MPR)=20.6-0.75-9-5) |  | 11.30 (NOTE 1) |  | 0 |  | 0.31 |  |
| Configured output power with power boost |  | FR2a |  | N/A |  | 21.7dBm/ChBW (22.4-1.7+1) |  | 16.33 (NOTE 1) |  | 0 |  | 0.1 |  |
|  |  | FR2b |  | N/A |  | 19.9dBm/ChBW (20.6-1.7+1) |  | 11.45 (NOTE 1) |  | 0 |  | 0.3 |  |
| Minimum output power |  | FR2a |  | -10.6dBm/400MHz |  | -13dBm |  | -2.54 (NOTE 1) |  | 8.4 |  | 1.0 (with relaxation) |  |
|  |  | FR2b |  | -5.5dBm/400MHz |  | -13dBm |  | -7.64 (NOTE 1) |  | 13.5 |  | 1.0 (with relaxation) |  |
| OFF power – TRP |  | FR2a |  | N/A |  | -35dBm/ChBW |  | -24.54 (NOTE 2) |  | 30.4 |  | 1.0 (with relaxation) |  |
|  |  | FR2b |  | N/A |  |  |  | <24.54 (NOTE 2) |  | N/A |  | Propose not to test |  |
| OFF power – EIRP |  | FR2a |  | -7.6dBm/400MHz |  | -30dBm/ChBW |  | -22.54 (NOTE 2) |  | 28.4 |  | 1.0 (with relaxation) |  |
|  |  | FR2b |  | -5.5dBm/400MHz |  |  |  | -24.64 (NOTE 2) |  | 30.5 |  | 1.0 (with relaxation) |  |
| Absolute power tolerance |  | Same as Minimum output power |  |  |  |  |  |  |  |  |  |  |  |
| Relative power tolerance |  | FR2a |  | -13.6dBm/100MHz |  | -7.6dBm/100MHz |  | 5.86 (NOTE 1) |  | 0 |  | 1.0 |  |
|  |  | FR2b |  | -11.5dBm/100MHz |  | -5.5dBm/100MHz |  | 5.86 (NOTE 1) |  | 0 |  | 1.0 |  |
| Aggregate power tolerance |  | Same as Relative power tolerance |  |  |  |  |  |  |  |  |  |  |  |
| Aggregate power tolerance |  | FR2a |  | -13.6dBm/100MHz |  | -7.6dBm/100MHz |  | 5.86 (NOTE 1) |  | 0 |  | 1.0 |  |
|  |  | FR2b |  | -11.5dBm/100MHz |  | -5.5dBm/100MHz |  | 5.86 (NOTE 1) |  | 0 |  | 1.0 |  |
| SEM |  | FR2a |  | N/A |  | -13dBm/1MHz |  | 8.14 (NOTE 1) |  | 0 |  | 0.62 |  |
|  |  | FR2b |  | N/A |  |  |  | 5.86 (NOTE 1) |  | 0 |  | 1.0 |  |
| ACLR (CP) |  | FR2a |  | -7.6dBm/400MHz |  | Highest testable MPR for 400MHz: 3dB 16.65dBm/ChBW (EIRP-MPB-MPR-T(MPR) =22.4-0.75-3-2) Actual lowest: 7.65dBm/ChBW (EIRP-MPB-MPR-T(MPR)=22.4-0.75-9-5) |  | 22.86 (with 3dB MPR) (NOTE 1) |  | 0 |  | N/A |  |
|  |  | FR2b |  | -5.5dBm/400MHz |  | Highest testable MPR for 400MHz: 2dB 16.35dBm/ChBW (EIRP-MPB-MPR-T(MPR) =20.6-0.75-2-1.5) Actual lowest: 5.85dBm/ChBW (EIRP-MPB-MPR-T(MPR)=20.6-0.75-9-5) |  | 21.86 (with 2dB MPR) (NOTE 1) |  | 0 |  | N/A |  |
| ACLR (ACP) |  | FR2a |  | -7.6dBm/400MHz |  | Highest testable MPR for 400MHz: 3dB -0.35dBm/ChBW (EIRP-MPB-MPR-T(MPR)-ACLR=22.4-0.75-3-2-17) Actual lowest: -9.35 dBm/ChBW (EIRP-MPB-MPR-T(MPR)-ACLR=22.4-0.75-9-5-17) |  | 5.86 (NOTE 1) |  | 0 |  | 1.0 |  |
|  |  | FR2b |  | -5.5dBm/400MHz |  | Highest testable MPR for 400MHz: 2dB 0.35dBm/ChBW (EIRP-MPB-MPR-T(MPR)-ACLR=20.6-0.75-2-1.5-16) Actual lowest: -10.15 dBm/ChBW (EIRP-MPB-MPR-T(MPR)-ACLR=20.6-0.75-9-5-16) |  | 5.86 (with 2dB MPR) (NOTE 1) |  | 0 |  | 1.0 |  |
| General Tx spurious |  | 6GHz <=f<=23.45GHz |  | N/A |  | -13dBm/1MHz |  | 10.0 (NOTE 1) |  | 0 |  | 0.41 |  |
|  |  | 23.45GHz<=f<=40GHz |  | N/A |  | -13dBm/1MHz |  | 10.0 (NOTE 1) |  | 0 |  |  |  |
|  |  | 40GHz<=f<=80GHz |  | N/A |  | -13dBm/1MHz |  | 10.0 (NOTE 1) |  | 0 |  |  |  |
| Tx spurious Co-existence |  | n260 (Aggressor band : n257, n261) |  | -23 |  | -2dBm/100MHz (-22dBm/MHz) |  | 0.86 (NOTE 2) |  | 5 |  | 1.0 (with relaxation) |  |
|  |  | n257, n261 (Aggressor band : n260) |  | -27.7 |  | -5dBm/100MHz (-25dBm/MHz) |  | 2.56 (NOTE 2) |  | 3.3 |  | 1.0 (with relaxation) |  |
|  |  | 23.6 GHz ≤ f ≤ 24.0GHz |  | -27.7 |  | 1dBm/200MHz (-22 dBm/MHz) |  | 5.56 (NOTE 2) |  | 0.3 |  |  |  |
|  |  | 36 GHz ≤ f ≤ 37GHz |  | -23dBm/MHz |  | 7dBm/1000MHz (-23dBm/MHz) |  | -0.14 (NOTE 2) |  | 6 |  | 1.0 (with relaxation) |  |
|  |  | 57 GHz ≤ f ≤ 66GHz |  | N/A |  | 2dBm/100MHz (-18dBm/MHz) |  | 5.86 (NOTE 1) |  | 0 |  | 1.0 |  |
| Additional spurious emission |  | NS_202 (7.25GHz <=f <=12.75GHz) |  | -40 dBm/MHz |  | -10dBm/100MHz (-30 dBm/MHz) |  | 10 (NOTE 1) |  | 0 |  | 0.41 |  |
|  |  | NS_202 (12.75GHz <=f <=23.45GHz) |  | -23 dBm/MHz |  | -10dBm/100MHz (-30 dBm/MHz) |  | -7.14 (NOTE 2) |  | 13 |  | 1.0 (with relaxation) |  |
|  |  | NS_202 (23.6GHz <=f <=24.0GHz) |  | -27.7 dBm/MHz |  | 1dBm/200MHz (-22 dBm/MHz) |  | 5.56 (NOTE 2) |  | 0.3 |  | 1.0 (with relaxation) |  |
|  |  | NS_202 (23.45GHz <=f <=40.8GHz) |  | -23 dBm/MHz |  | -10dBm/100MHz (-30 dBm/MHz) |  | -7.14 (NOTE 2) |  | 13 |  | 1.0 (with relaxation) |  |
|  |  | NS_202 (40.8GHz <=f <=66GHz) |  | -23 dBm/MHz |  | -10dBm/100MHz (-30 dBm/MHz) |  | -7.14 (NOTE 2) |  | 13 |  | 1.0 (with relaxation) |  |
|  |  | NS_203 (23.6GHz <=f <=24.0GHz) |  | -27.7 dBm/MHz |  | +1dBm/200MHz (-22dBm/MHz) |  | 5.56 (NOTE 2) |  | 0.3 |  | 1.0 (with relaxation) |  |
| Rx spurious |  | 6GHz <=f<=20GHz |  |  |  | -47dBm/1MHz |  | -4.34 (NOTE 2) |  | 10.2 |  | 1.0 dB for 23.45~40.8GHz, 0.64dB for 6~23.45 and 40.8~80 GHz. |  |
|  |  | 20GHz<=f<=40GHz |  |  |  | -47dBm/1MHz |  | -11.34 (NOTE 2) |  | 17.2 |  |  |  |
|  |  | 40GHz<=f<=80GHz |  |  |  | -47dBm/1MHz |  | -27.24 (NOTE 2) |  | 33.1 |  |  |  |
| NOTE 1: Estimated SNR is calculated based on agreed influence of noise. NOTE 2: Estimated SNR is calculated based on agreed relaxation value: Estimated SNR = 6dB - relaxation. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.2.2.27-2: Uncertainty value for influence of noise for PC1 for IFF

| Test case | Frequency range | Relaxation | Influence of noise |
| --- | --- | --- | --- |
| MOP-EIRP | FR2a | 0 | 0.13 |
|  | FR2b | TBD | TBD |
| MOP-TRP | FR2a | 0 | 0.13 |
|  | FR2b | TBD | TBD |
| MOP-Spherical | FR2a | 0 | 0.2 |
|  | FR2b | 0 | 0.35 |
| MPR | FR2a | 0 | 0.1 |
|  | FR2b | 0 | 0.3 |
| Minimum output power | FR2a | 0 | 0.3 |
|  | FR2b | 0 | 0.6 |
| OFF power – TRP | FR2a | Same as defined for PC3 in Table B.2.2.27-1 |  |
|  | FR2b | Same as defined for PC3 in Table B.2.2.27-1 |  |
| SEM | FR2a | 1.41 | 1.41 (NOTE 1, 2) |
|  | FR2b | 2.08 | 2.08 (NOTE 1, 3) |
| ACLR (ACP) | FR2a | 0 | 0.15 |
|  | FR2b | 0 | 0.3 |
| General Tx spurious | 6GHz <=f<12.5GHz | Same as defined for PC3 in Table B.2.2.27-1 |  |
|  | 12.5GHz<=f<66GHz |  |  | 0.95 |  |  | 0.95 (NOTE 1, 4) |  |
|  | 66GHz<=f<=80GHz | Same as defined for PC3 in Table B.2.2.27-1 |  |
| Rx spurious | 6GHz <=f<=20GHz | Same as defined for PC3 in Table B.2.2.27-1 |  |
|  | 20GHz<=f<=40GHz | Same as defined for PC3 in Table B.2.2.27-1 |  |
|  |  |  | 40GHz<=f<=80GHz |  |  | Same as defined for PC3 in Table B.2.2.27-1 |  |  |  |  |
| NOTE 1: values assuming up 6% of grid points with EIRP > 43dBm. NOTE 2: values assuming SNR = -6dB for points with EIRP > 43dBm / SNR = 8.14dB otherwise. NOTE 3: values assuming SNR = -8dB for points with EIRP > 43dBm / SNR = 6dB otherwise. NOTE 4: values assuming SNR = -4dB for points with EIRP > 43dBm / SNR = 10dB otherwise. |  |  |  |

### B.2.2.28	Systematic error related to beam peak search

See B.2.1.28.

The uncertainty value of systematic error related to beam peak search is estimated as below table and used across clause B.

Table B.2.2.28-1: Uncertainty value for systematic error related to beam peak search for IFF

| Power class | Uncertainty value |
| --- | --- |
| PC1 | 0.7 |
| PC3 | 0.5 |

### B.2.2.29	Influence of spherical coverage grid

See B.2.1.29.

The uncertainty value of influence of spherical coverage grid is estimated as below table and used across clause B.

Table B.2.2.29-1: Uncertainty value for influence of spherical coverage grid for IFF

| Power class | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- |
| PC1 | 0.13 | Actual | 1.00 | 0.13 |
| PC3 | 0.12 | Actual | 1.00 | 0.12 |

### B.2.2.30	Systematic error related to EIS spherical coverage

See B.2.1.30.

The uncertainty value of systematic error related to EIS spherical coverage is estimated as below table and used across clause B.

Table B.2.2.30-1: Uncertainty value for systematic error related to EIS spherical coverage for IFF

| Power class | Uncertainty value |
| --- | --- |
| PC1 | DL power step size, 0.2 |
| PC3 | DL power step size, 0.2 |

### B.2.2.31	Misalignment of DUT due to change of DUT orientation

See B.2.1.31.

The uncertainty value of misalignment of DUT due to change of DUT orientation is estimated as below table and used across clause B.

Table B.2.2.31-1: Uncertainty value for misalignment of DUT due to change of DUT orientation for IFF

| Power class |  | Uncertainty value |  | Distribution of the probability |  | Divisor |  | Standard uncertainty (σ) [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC1 |  | TBD |  | Actual |  | 1.00 |  | TBD |  |
| PC3 |  | 0.10 |  | Actual |  | 1.00 |  | 0.10 |  |

### B.2.2.32	Additional Impact of Interferer ACLR

See B.2.1.32.

The uncertainty value of additional Impact of Interferer ACLR is estimated as below table and used across clause B.

Table B.2.2.32-1: Uncertainty value for additional Impact of Interferer ACLR for IFF

| Power class | Uncertainty value |
| --- | --- |
| PC1, PC3 | 0.7 |

### B.2.2.33	Modulated Interferer uncertainty

See B.2.1.33.

The uncertainty value of modulated Interferer uncertainty is estimated as below table and used across clause B.

Table B.2.2.33-1: Uncertainty value for modulated Interferer uncertainty for IFF

| Power class |  | Uncertainty value |  | Distribution of the probability |  | Divisor |  | Standard uncertainty (σ) [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC1 |  | 2.9 |  | Normal |  | 2 |  | 1.45 |  |
| PC3 |  | 2.9 |  | Normal |  | 2 |  | 1.45 |  |

### B.2.2.34	Void

### B.2.2.35	Influence of offset antenna for blocker signal

See B.2.1.35.

### B.2.2.36	Uncertainty of the RF relative power measurement equipment

See B.2.1.36.

The uncertainty value of uncertainty of the RF relative power measurement equipment is estimated as below table and used across clause B.

Table B.2.2.36-1: Uncertainty value for uncertainty of the RF relative power measurement equipment for IFF

| Power class |  | Uncertainty value |  | Distribution of the probability |  | Divisor |  | Standard uncertainty (σ) [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC1 |  | TBD |  | Normal |  | 2 |  | TBD |  |
| PC3 |  | [0.4] |  | Normal |  | 2 |  | [0.2] |  |

## <<< END OF CHANGES >>>