3GPP TSG-RAN5 Meeting #97	R5-226121

# Toulouse, France, November 14th-18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1967 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RedCap RRM TC 16.3.2.1.3 - inter reestablish 1 Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add Addition of RedCap RRM TC 16.3.2.1.3 - inter reestablish 1 Rx |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RedCap RRM TC 16.3.2.1.3 - inter reestablish 1 Rx is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.2.1.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0214 TS 38.533 CR 2027 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

16.3.2.1.3	NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE

Editor’s Note : This test case is incomplete in following aspects :

-	TT analysis is still missing.

16.3.2.1.3.1	Test purpose

To verify that the NR inter-frequency RRC re-establishment delay in FR1 without known target cell is within the specified limits. This test will verify the requirements in 38.133 [6] clause 6.2.1B.

16.3.2.1.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards.

16.3.2.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.1.3.

16.3.2.1.3.4	Test description

16.3.2.1.3.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.1.2.4.1 with following exceptions:

-	Table 6.3.2.1.2.4.1-1 is replaced by Table 16.3.2.1.3.4.1-1.

-	Table 6.3.2.1.2.4.1-2 is replaced by Table 16.3.2.1.3.4.1-2.

-	Table 6.3.2.1.1.4.1-3 is replaced by Table 16.3.2.1.3.4.1-3

Table 16.3.2.1.3.4.1-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.3.2.1.3-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.1.3-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.1.3-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.1.3-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table 16.3.2.1.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.3.2.1.3.4.1-3: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1,2,3,4 | Cell1 |  |
|  | Neighbour cells |  | 1,2,3,4 | Cell2 |  |
| Final condition | Active cell |  | 1,2,3,4 | Cell2 |  |
| RF Channel Number |  |  | 1,2,3,4 | 1,2 |  |
| Time offset between cells |  |  | 1,4 | 3 ms | Asynchronous cells |
|  |  |  | 2,3 | 3 s | Synchronous cells |
| N310 |  | - | 1,2,3,4 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1,2,3,4 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1,2,3,4 | 0 | Radio link failure timer; |
| T311 |  | ms | 1,2,3,4 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1,2,3,4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1,2,4 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1,4 | SMTC.2 |  |
|  |  |  | 2,3 | SMTC.1 |  |
| DRX cycle length |  | s | 1,2,3,4 | OFF |  |
| PRACH configuration |  |  | 1,2,3,4 | PRACH.1 FR1 | Table A.7.1-1 |
| T1 |  | s | 1,2,3,4 | 5 |  |
| T2 |  | ms | 1,2,3,4 | 440 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1B in TS 38.133 [6], T310 and the period for UE turns off transmitter defined in clause 8.1B.5 in TS 38.133[6]) |
| T3 |  | s | 1,2,3,4 | 5 |  |

16.3.2.1.3.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.1.2.4.2.

16.3.2.1.3.4.3	Message contents

Same as the message contents given in 6.3.2.1.2.4.3 with the following exceptions:

-	Table 7.3.1-3 in 38.508-1 [14] is used with condition SMTC.2 for Test Configuration 16.3.2.1.3-4.

16.3.2.1.3.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.1.2.5 with following exceptions:

-	Table 6.3.2.1.2.5-1 is replaced by Table 16.3.2.1.3.5-1.

Table 16.3.2.1.3.5-1: Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| RF Channel Number |  | 1,2,3,4 | 1 |  |  | 2 |  |  |
| TDD configuration |  | 1,4 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET RMC |  | 1,4 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC |  | 1,4 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1,2,3,4 | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1,4 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial DL BWP configuration |  | 1,2,3,4 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1,2,3,4 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Active DL BWP confgiuration |  | 1,2,3,4 | DLBWP.1.1 RedCap | N/A | N/A | N/A | N/A | DLBWP.1.1 RedCap |
| Active UL BWP configuration |  | 1,2,3,4 | ULBWP.1.1 RedCap | N/A | N/A | N/A | N/A | ULBWP.1.1 RedCap |
| RLM-RS |  | 1,2,3,4 | SSB |  |  | SSB |  |  |
|  | dB | 1,2,3,4 | 4 | -infinity | -infinity | -infinity | -infinity | 7 |
| Note2 | dBm/SCS | 1,2,4 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4 | -98 |  |  |  |  |  |
|  | dB | 1,2,3,4 | 4 | -infinity | -infinity | -infinity | -infinity | 7 |
| SS-RSRP Note3 | dBm/SCS | 1,2,4 | -94 | -infinity | -infinity | -infinity | -infinity | -91 |
|  |  | 3 | -91 | -infinity | -infinity | -infinity | -infinity | -88 |
| Io | dBm/9.36 MHz | 1,2,4 | -64.59 | -70.05 | -70.05 | -70.05 | -70.05 | -62.26 |
|  | dBm/18.36 MHz | 3 | -61.66 | -67.12 | -67.12 | -67.12 | -67.12 | -59.33 |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

<End of modified section 1>