3GPP TSG-RAN5 Meeting #97	R5-226137

# Toulouse, France, November 14th-18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1983 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RedCap RRM TC 16.3.2.3.3 - LTE redirection 1 Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add RedCap RRM TC 16.3.2.3.3 - LTE redirection 1 Rx |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RedCap RRM TC 16.3.2.3.3 - LTE redirection 1 Rx is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.2.3.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0214 TS 38.533 CR 2027 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

16.3.2.3.3	NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 1 Rx UE

Editor’s Note : This test case is incomplete in following aspects :

-	TT analysis is still missing.

16.3.2.3.3.1	Test purpose

To verify RRC connection release with redirection from NR to E-UTRAN requirements specified in 38.133 [4] clause 6.2.3A.2.2.

16.3.2.3.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards.

16.3.2.3.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.2.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.3.3.

16.3.2.3.3.4	Test description

16.3.2.3.3.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.3.1.4.1 with following exceptions:

-	Table 6.3.2.3.1.4.1-1 is replaced by Table 16.3.2.3.3.4.1-1.

-	Table 6.3.2.3.1.4.1-2 is replaced by Table 16.3.2.3.3.4.1-2.

-	Table 6.3.2.3.1.4.1-3 is replaced by Table 16.3.2.3.3.4.1-3.

Table 16.3.2.3.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.2.3.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 16.3.2.3.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.3.2.3.3-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.3.2.3.3-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 16.3.2.3.3-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.3.2.3.3-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.3.2.3.3-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE FDD |
| 16.3.2.3.3-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.3.2.3.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.3.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.3.2.3.3.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2.3 |  |

16.3.2.3.3.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.3.1.4.2.

16.3.2.3.3.4.3	Message contents

Same as the message contents given in 6.3.2.3.1.4.3.

16.3.2.3.3.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.3.1.5 with following exceptions:

-	Table 6.3.2.3.1.5-1 is replaced by Table 16.3.2.3.3.5-1.

-	Table 6.3.2.3.1.5-2 is replaced by Table 16.3.2.3.3.5-2.

Table 16.3.2.3.3.5-1: Cell specific test parameters for NR Cell (Cell 1)

| Parameter |  |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode | Config 1,4 |  |  | FDD |  |
|  | Config 2,3,5,6 |  |  | TDD |  |
|  | Config 7,8 |  |  | HD-FDD |  |
| SSB Configuration | Config 1,2,4,5,7,8 |  |  | SSB.1 FR1 |  |
|  | Config 3,6 |  |  | SSB.1 RedCap FR1 |  |
| CSI-RS for tracking | Config 1,4,7,8 |  |  | TRS.1.1 FDD |  |
|  | Config 2,5 |  |  | TRS.1.1 TDD |  |
|  | Config 3,6 |  |  | TRS.1.2 TDD |  |
| TDD configuration | Config 1,4,7,8 |  |  | Not Applicable |  |
|  | Config 2,5 |  |  | TDDConf.1.1 |  |
|  | Config 3,6 |  |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,4,5,7,8 |  | MHz | 10: NRB,c = 52 |  |
|  | Config 3,6 |  |  | 20: NRB,c = 51 |  |
| BWP BW | Config 1,2,4,5,7,8 |  | MHz | 10: NRB,c = 52 |  |
|  | Config 3,6 |  |  | 20: NRB,c = 51 |  |
| DRX Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference | Config 1,4,7,8 |  |  | SR.1.1 FDD |  |
| measurement channel | Config 2,5 |  |  | SR.1.1 TDD |  |
|  | Config 3,6 |  |  | SR.2.1 TDD |  |
| CORESET Reference | Config 1,4,7,8 |  |  | CR.1.1 FDD |  |
| Channel | Config 2,5 |  |  | CR.1.1 TDD |  |
|  | Config 3,6 |  |  | CR.2.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC configuration | Config 1,2,4,5,7,8 |  |  | SMTC.1 FR1 |  |
|  | Config 3,6 |  |  | SMTC.2 FR1 |  |
| PDSCH/PDCCH | Config 1,2,4,5,7,8 |  | kHz | 15 kHz |  |
| subcarrier spacing | Config 3,6 |  |  | 30 kHz |  |
| PUCCH/PUSCH | Config 1,2,4,5,7,8 |  | kHz | 15 kHz |  |
| subcarrier spacing | Config 3,6 |  |  | 30 kHz |  |
| PRACH configuration |  |  |  | PRACH.1 FR1 |  |
| BWP configuration | Initial DL BWP |  |  | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 RedCap |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 RedCap |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 |  | Config 1,2,4,5,7,8 | dBm/SCS | -98 |  |
|  |  | Config 3,6 |  | -95 |  |
|  |  |  | dB | 4 | 4 |
|  |  |  | dB | 4 | 4 |
| IoNote3 |  | Config 1,2,4,5,7,8 | dBm/ 9.36MHz | -64.59 | -64.59 |
|  |  | Config 3,6 | dBm/ 18.36MHz | -61.66 | -61.66 |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table 16.3.2.3.3.5-2: Cell specific test parameters for E-UTRAN Cell (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1,2,3,4,5,6,7,8 | 2 |  |
| Duplex mode |  | 1,2,3,7 | FDD |  |
|  |  | 4,5,6,8 | TDD |  |
| TDD special subframe configurationNote1 |  | 4,5,6,8 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4,5,6,8 | 1 |  |
| BWchannel | MHz | 1,2,3,4,5,6,7,8 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PRACH ConfigurationNote2 |  | 1,2,3,7 | 4 |  |
|  |  | 4,5,6,8 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1,2,3,7 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4,5,6,8 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement |  | 1,2,3,7 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
| ChannelNote3 |  | 4,5,6,8 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote3 |  | 1,2,3,7 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4,5,6,8 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1,2,3,4,5,6,7,8 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -98 |  |
| Ês/Noc | dB | 1,2,3,4,5,6,7,8 | -Infinity | 4 |
| Ês/IotNote6 | dB | 1,2,3,4,5,6,7,8 | -Infinity | 4 |
| RSRPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -94 |
| SCH_RPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -94 |
| IoNote6 | dBm/9MHz | 1,2,3,4,5,6,7,8 | -70.22 | -64.76 |
| Propagation Condition |  | 1,2,3,4,5,6,7,8 | AWGN |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [24]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

<End of modified section 1>