3GPP TSG-RAN5 Meeting #97	R5-226144

# Toulouse, France, November 14th-18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1990 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RedCap RRM TC 16.5.1.2 - OOS non-DRX 2 Rx with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add RedCap RRM TC 16.5.1.2 - OOS non-DRX 2 Rx |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RedCap RRM TC 16.5.1.2 - OOS non-DRX 2 Rx is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.5.1.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0214 TS 38.533 CR 2027 2028 TS 38.903 CR 0396 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

16.5.1.2	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE

16.5.1.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell for 2Rx RedCap UE. This test will partly verify the FR1 radio link monitoring requirements in 38.133 [6] clause 8.1B.2.

16.5.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 RX from Release 17 onwards.

16.5.1.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 16.5.1.0.1 and 16.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.1.2.

16.5.1.2.4	Test description

Same as the test description given in clause 6.5.1.1.4.

16.5.1.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.1.1.4.1 with following exceptions:

-	Table 6.5.1.1.4.1-1 is replaced by Table 16.5.1.2.4.1-1.

-	Table 6.5.1.1.4.1-2 is replaced by Table 16.5.1.2.4.1-2.

-	Table 6.5.1.1.4.1-4 is replaced by Table 16.5.1.2.4.1-3.

Table 16.5.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.1.2-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, FDD |
| 16.5.1.2-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, TDD |
| 16.5.1.2-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, TDD |
| 16.5.1.2-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, HD-FDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.5.1.2.4.1-3: General test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| BWchannel |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |
|  |  | Config 3 |  | 20: NRB,c = 51 |
| DL initial BWP configuration |  | Config 1,2,3,4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1,2,3,4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1,2,3,4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1,2,3,4 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  | Config 1,2,3,4 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1,2,3,4 |  | PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1,4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.48 |
| T3 |  |  | s | 0.48 |
| D1 |  |  | s | 0.44 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

16.5.1.2.4.2	Test Procedure

Same as the test procedure given in clause 6.5.1.1.4.2.

16.5.1.2.4.3	Message Contents

Same as the message contents given in clause 6.5.1.1.4.3.

16.5.1.2.5	Test Requirement

Same as the test requirements given in clause 6.5.1.1.5 with following exceptions:

-	Table 6.5.1.1.5-1 is replaced by Table 16.5.1.2.5-1.

Table 16.5.1.2.5-1: Cell specific test parameters (Cell 1)

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1,2,4 | dB | 1.8Note 5 | -6.2Note 5 | -15.8Note 5 |
|  | Config 3 |  | 1.8Note 5 | -6.2Note 5 | -15.8Note 5 |
|  | Config 1,2,4 | dBm/15kHz | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
|  | Config 1,2,4 | dBm/SCS | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure 6.5.1.1.4-1. Note 5: Including the test tolerance given in Annex F. |  |  |  |  |  |

<End of modified section 1>