3GPP TSG-RAN5 Meeting #97	R5-226156

# Toulouse, France, November 14th-18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2002 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RedCap RRM TC 16.6.1.8 - gap DRX 2 Rx with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add RedCap RRM TC 16.6.1.8 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RedCap RRM TC 16.6.1.8 is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6.1.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0214 TS 38.533 CR 2027 2028 TS 38.903 CR 0399 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

16.6.1.8	NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE

16.6.1.8.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell identification and measurement period requirements in 38.133 [6] clauses 9.2B.6.1 and 9.2B.6.2.

16.6.1.8.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 onwards and supporting CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle.

16.6.1.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.8.

16.6.1.8.4	Test description

16.6.1.8.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.1.4.4.1 with following exceptions:

-	Table 6.6.1.4.4.1-1 is replaced by Table 16.6.1.8.4.1-1.

-	Table 6.6.1.4.4.1-2 is replaced by Table 16.6.1.8.4.1-2.

-	Table 6.6.1.4.4.1-3 is replaced by Table 16.6.1.8.4.1-3.

Table 16.6.1.8.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.8-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 16.6.1.8-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 16.6.1.8-3 | 30 kHz SSB SCS, 20MHz bandwidth, TDD duplex mode |
| 16.6.1.8-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.6.1.8.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

Table 16.6.1.8.4.1-3: General test parameters

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1,2,3,4 | Cell 1 |  |  |
| Neighbour cell |  | 1,2,3,4 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1,2,3,4 | 1: Cell 1 and Cell 2 |  |  |
| Measurement gap type |  | 1,2,3,4, | Per-UE gaps |  |  |
| Measurement gap repetition periodicity | ms | 1,2,3,4 | 40 |  |  |
| Measurement gap length | ms | 1,2,3,4 | 6 |  |  |
| Measurement gap offset | ms | 1,2,3,4 | 39 |  |  |
| SSB configuration |  | 1,2,4 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.1 RedCap FR1 |  |  |
| SMTC configuration |  | 1,2,3,4 | SMTC.1 RedCap FR1 |  |  |
| A3-Offset | dB | 1,2,3,4 | -4.5 |  |  |
| CP length |  | 1,2,3,4 | Normal |  |  |
| Hysteresis | dB | 1,2,3,4 | 0 |  |  |
| Time To Trigger | s | 1,2,3,4 | 0 |  |  |
| Filter coefficient |  | 1,2,3,4 | 0 |  | L3 filtering is not used |
| DRX | ms | 1,2,3,4 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  | 1,4 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1 |
|  |  | 2,3 | 3 us |  | Synchronous cells |
| T1 | s | 1,2,3,4 | 5 |  |  |
| T2 | s | 1,2,3,4 | 5 | 10 |  |

16.6.1.8.4.2	Test procedure

Same as the test procedure given in 6.6.1.4.4.2.

16.6.1.8.4.3	Message contents

Same as the message contents given in 6.6.1.4.4.3 with following exceptions:

-	Table 7.3.1-3 in 38.508-1 [14] is used with condition SMTC.1 RedCap FR1.

16.6.1.8.5	Test requirement

Same as the test requirement given in 6.6.1.4.5 with following exceptions:

-	Table 6.6.1.4.5-1 is replaced by Table 16.6.1.8.5-1.

-	The overall delay measured is expressed as:

Overall delays measured = TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap + TTI insertion uncertainty

  - TPSS/SSS_sync_intra_RedCap = 600 ms for Test 1, and TPSS/SSS_sync_intra_RedCap = 3200 ms for Test 2;

  - T SSB_measurement_period_intra_RedCap= 320 ms for Test 1, and TPSS/SSS_sync_intra_RedCap = 3200 ms for Test 2

  - TTI insertion uncertainty = 2 ms

As a result, the overall delay measured for Test 1 and Test 2 shall be less than a total of 922 ms and 6402 ms respectively.

Table 16.6.1.8.5-1: NR Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1,4 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1,4 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1,2 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1,2,3,4 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1,2,3,4 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1,2,3,4 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1,2,3,4 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1,2,3,4 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1,2,4 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1,2,3,4 | -98 |  |  |  |
|  | dB | 1,2,3,4 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1,2,3,4 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | 1,2,4 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/ 9.36 MHz | 1,2,4 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/ 18.36 MHz | 3 | -61.68 | -59.34 |  |  |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

<End of modified section 1>