3GPP TSG-RAN5 Meeting #97	R5-226206

# Toulouse, France, November 14th-18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2576 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to default configuration for aperiodic CSI-RS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In aperiodic CSI-RS based L1-RSRP TCs UE is expect to perform aperiodic CSI reporting based on aperiodic CSI-RS resource. In test parameters we have: - reportSlotOffsetList = 8 (i.e. the reporting offset = 8 slots), UE is expected to sent CSI report 8 slot in slot n+8 when it receives DCI triggering CSI reporting in slot n. - aperiodic CSI-RS RMC CSI-RS.1.3 FDD/CSI-RS.1.3 TDD/CSI-RS.2.3 TDD are used in test. For these CSI-RS RMCs aperiodicTriggeringOffset = 4 (i.e. CSI triggering offset = 4 slots) according to 38.533 Annex.A.1.2. Then the TE is expected to start transmit CSI-RS resources from slot n+4, which is also described in the test procedure. However, according to 38.214 the CSI-RS triggering offset is fixed to be 0 slot if the aperiodic CSI-RS resources are not type-D QCLed to some RSs. According to 38.533 QCL type D is only configured for FR2 TCI-state RMC. As a result, the correct CSI-triggering offset in 4.6.4.3 should be fixed to 0 slot. TE starting transmitting CSI-RS from slot n+4 may fail conformant UEs. We suggest change CSI-RS triggering offset = 0 slot in FR1 aperiodic CSI-RS default configurations |  |  |  |  |  |  |  |  |
| Summary of change: |  | - CSI-RS triggering offset is changed to 0 slot in FR1 aperiodic CSI-RS default configurations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	NZP-CSI-RS-ResourceSet for BM

Table 7.3.1-11B: NZP-CSI-RS-ResourceSet for BM

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-ResourceSetId | NZP-CSI-RS-ResourceSetId-BM |  |  |
|  | NZP-CSI-RS-ResourceSetId-BM with condition APERIODIC |  | APERIODIC |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for BM (0) | entry 1 |  |
|  | NZP-CSI-RS-ResourceId for BM (0) with condition APERIODIC |  | APERIODIC |
| NZP-CSI-RS-ResourceId[2] | NZP-CSI-RS-ResourceId for BM (1) | entry 2 |  |
|  | NZP-CSI-RS-ResourceId for BM (1) with condition APERIODIC |  | APERIODIC |
| } |  |  |  |
| aperiodicTriggeringOffset | 4 |  | APERIODIC AND FR2 |
|  | 0 |  | APERIODIC AND FR1 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

<End of modified section 1>