3GPP TSG-RAN5 Meeting #97	R5-226216

# Toulouse, France, November 14th-18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2037 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test frequency selection for DAPS HO TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to 38.533 Annex E.1.3.2. TE shall select test frequencies for intra-band inter-frequency DAPS HO in following way: - For FR1, source cell is configured using “low” frequency carrier for the band under test. For FR2, source cell is configured using “mid” frequency carrier for the band under test - For FR1, target cell is configured using “high” frequency carrier for the band under test. For FR2, target cell is configured using a non-overlapping test frequency adjacent to the serving cell frequency. However, according to 38.306, supporting DAPS HO is a per-BC UE capability. Then it’s possible that a UE only supports DAPS HO on an Intra-band contiguous band combination. In this case, the UE may fail the test since the test frequencies chosen according to Annex E.1.3.2 may be non-adjacent in certain bands. To fix this issue, we suggest specifiy test frequency selection criteria for DAPS HO TCs as follows: - To test intra-frequency DAPS HO, the test frequency selection criteria for Intra-frequency test cases specified in E.1.3.1 shall be used. - To test intra-band inter-frequency DAPS HO on intra-band contiguous BC, the test frequency selection criteria for intra-band contiguous CA specified in E.1.4.2 shall be used. - To test intra-band inter-frequency DAPS HO on intra-band non-contiguous BC, the test frequency selection criteria for intra-band non-contiguous CA specified in E.1.4.3 shall be used. - To test inter-band DAPS HO, the test frequency selection criteria for Inter-band carrier aggregation specified specified in E.1.4.1 shall be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test frequency selection criteria for DAPS HO TC are specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

E.1.7	NR sidelink test cases

Unless otherwise stated, for NR sidelink test cases the NR sidelink carrier shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14] for the corresponding band under test.

For con-current operation, unless otherwise stated, each of the NR Uu carriers and their respective cells shall be configured following the sample principles defined in E.1.2 and E.1.3.

E.1.8	DAPS handover test cases

For intra-frequency DAPS handover test cases, unless otherwise specified, the source cell and the target cell shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14] for the corresponding NR band under test.

For intra-band inter-frequency DAPS handover test cases to be tested on an intra-band contiguous band combination in FR1, unless otherwise stated, the source cell and the target cell shall be configured using the test frequency “Low” as defined in TS 38.508-1 [14], subclause 4.3.1.1.3 for the corresponding band under test (source carrier = CC1 and target carrier = CC2).

For intra-band inter-frequency DAPS handover test cases to be tested on an intra-band non-contiguous carrier aggregation in FR1, unless otherwise specified, the test frequency selection shall be done following the maximum Wgap principle, i.e. selecting the test frequencies (of the test frequencies defined in TS 38.508-1 [14]) with the widest frequency separation within the band under test.

For inter-band carrier aggregation test cases, unless otherwise specified, source cell and target cell shall be configured following the same principles defined in E.1.2 and E.1.3.

<End of modified section 1>