3GPP TSG-RAN5 Meeting #97	R5-226244
Toulouse, France, 14th – 18th November 2022

Agenda item:	5.4.17

Source:	Keysight Technologies

Title:	On TRP MU for Spurious Emissions

Document for:	Discussion and Endorsement

# Introduction

This contribution provides simulation analyses to determine the PC1 TRP MUs for spurious emissions.

# Discussion

The PC1 antenna assumptions have been previously agreed in [1], i.e., an 12x12 antenna array with 90° HPBW, and are summarized in Table 1 for the single-element and in Table 2 for the composite antenna array, respectively.

Table 1: Single Antenna Element Radiation Pattern for PC1

| Antenna element horizontal radiation pattern | AE,Hϕ=-min12ϕϕ3dB2,AmdB Am =25 dB |
| --- | --- |
| Horizontal half-power beamwidth of single element | ϕ3dB=90° |
| Antenna element vertical radiation pattern | AE,Vθ=-min12θ-90θ3dB2,SLAv SLAv =25 dB |
| Vertical half-power beamwidth of single array element | θ3dB=90° |
| Array element radiation pattern |  |
| Element gain without antenna losses | GE,max = 5 dBi |

Table 2: Composite Antenna Array Radiation Pattern for PC1

| Composite array radiation pattern in dB | the super position vector is given by: the weighting is given by: |
| --- | --- |
| Antenna array configuration (Row×Column) | 12 x 12 |
| Horizontal radiating element spacing | dh/λ = 0.5 |
| Vertical radiating element spacing | dv/λ = 0.5 |

This 12x12 antenna pattern based on these assumptions is shown in Figure 1.

Figure 1: PC1 Antenna Pattern with 12x12 Elements for the fundamental frequency (dH/l=dV/l=0.5)

For spurious emissions, the worst-case antenna pattern to be investigated is based on the 2nd harmonic with dH/l=dV/l=1 in Table 2 [2][3].

Figure 2: PC1 Antenna Pattern with 12x12 Elements for the 2nd harmonic (dH/l=dV/l=1)

Similar to previous TRP simulations, analyses with 10k different random rotations were performed to determine the measurement uncertainties of measurement grids on TRP. The results are summarized for the constant-step size grids in Table 3 and for the constant-density grids in Table 4. For the constant-step size grids, a maximum antenna offset of 6° was investigated as well together with the Jacobian quadrature as outlined in [4] and permitted in [3].

Table 3: Simulation results for the constant step size measurement grids

| Quadrature/TRP approach | Antenna Assumption | Number of unique grid points | Dq=Df [°] | Antenna Offset [°] | ¦Mean Error¦[dB] | Std. Deviation [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Classical sin(theta) | 12x12 (2nd harmonic) | 1106 | 7.5 | 0 | 0.12 | 1.03 |
| Clenshaw Curtis | 12x12 (2nd harmonic) | 1106 | 7.5 | 0 | 0.12 | 1.01 |
| Jacobian | 12x12 (2nd harmonic) | 1106 | 7.5 | 0 | 0.14 | 1.05 |
| Classical sin(theta) | 12x12 (2nd harmonic) | 2522 | 5 | 0 | 0.01 | 0.22 |
| Clenshaw Curtis | 12x12 (2nd harmonic) | 2522 | 5 | 0 | 0.01 | 0.21 |
| Jacobian | 12x12 (2nd harmonic) | 2522 | 5 | 0 | 0.00 | 0.21 |
| Jacobian | 12x12 (2nd harmonic) | 2522 | 5 | 6 | 0.00 | 0.23 |

Table 4: Simulation results for the constant density measurement grids

| Antenna Assumption | Number of grid points | ¦Mean Error¦ [dB] | Std. Deviation [dB] |
| --- | --- | --- | --- |
| 12x12 (2nd harmonic) | 500 | 0.28 | 1.57 |
| 12x12 (2nd harmonic) | 700 | 0.16 | 1.20 |
| 12x12 (2nd harmonic) | 1000 | 0.06 | 0.76 |
| 12x12 (2nd harmonic) | 1200 | 0.03 | 0.55 |
| 12x12 (2nd harmonic) | 1500 | 0.02 | 0.29 |
| 12x12 (2nd harmonic) | 1600 | 0.01 | 0.23 |
| 12x12 (2nd harmonic) | 1700 | 0.00 | 0.18 |

For PC3, it was previously found that the TRP measurement grids for in-band/OOB showed just a moderate increase in MU when applied to the 2nd harmonic pattern [2]. This increase was considered acceptable and subsequently endorsed for MTSU of spurious emissions test cases, e.g., Clause B.2.2.22 of [5].

Observation 1: For PC3, it was previously found that the TRP measurement grids for in-band/OOB showed just a moderate increase in MU when applied to the 2nd harmonic pattern.

Table B.2.2.22-1: Uncertainty value for influence of TRP measurement grid for IFF Power class; Test case; Uncertainty value; Distribution of the probability; Divisor; Standard uncertainty (σ) [dB]. PC1; Default; 0.25; Actual; 1.00; 0.25. SE; TBD; Actual; 1.00; TBD. PC3; Default; 0.25; Actual; 1.00; 0.25. SE; 0.32; Actual; 1.00; 0.32.

However, for PC1, it can be observed in Table 3 and Table 4 that the same measurement grids used for in-band/OOB applied to the worst-case, 2nd harmonic pattern yield a significant increase in MU that should be considered excessive and thus unacceptable.

Observation 2: For PC1, it is found that same measurement grids used for in-band/OOB applied to the worst-case, 2nd harmonic pattern yield a significant increase in MU.

It is therefore proposed to further refine the TRP measurement grids for spurious emissions while keeping the TRP measurement grid MU of 0.25 dB (same as for in-band/OOB), i.e., consider a constant-step size grid with 2522 unique grid points (Dq=Df=5°) or a constant-density grid with 1600 grid points [6].

Proposal 1: Further refine the PC1 TRP measurement grids for spurious emissions (when compared to in-band/OOB) to yield a 0.25 dB ‘Influence of TRP measurement grid’ MU, i.e., constant-step size grid with 2522 unique grid points (Dq=Df=5°) or a constant-density grid with 1600 grid points

The table in Table B.2.2.22-1 could thus be modified as follows

Table B.2.2.22-1: Uncertainty value for influence of TRP measurement grid for IFF Power class; Test case; Uncertainty value; Distribution of the probability; Divisor; Standard uncertainty (σ) [dB]. PC1 (Note 1); Default; 0.25; Actual; 1.00; 0.25. SE; 0.25; Actual; 1.00; 0.25. PC3; Default; 0.25; Actual; 1.00; 0.25. SE; 0.32; Actual; 1.00; 0.32. Note 1: A finer measurement grid for SE (when compared to default) is required, i.e., a constant-step size grid with 2522 unique grid points (Dq=Df=5°) or a constant-density grid with 1600 grid points..

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: For PC3, it was previously found that the TRP measurement grids for in-band/OOB showed just a moderate increase in MU when applied to the 2nd harmonic pattern.

Observation 2: For PC1, it is found that same measurement grids used for in-band/OOB applied to the worst-case, 2nd harmonic pattern yield a significant increase in MU.

Proposal 1: Further refine the PC1 TRP measurement grids for spurious emissions (when compared to in-band/OOB) to yield a 0.25 dB ‘Influence of TRP measurement grid’ MU, i.e., constant-step size grid with 2522 unique grid points (Dq=Df=5°) or a constant-density grid with 1600 grid points

# References

- R5-206618, PC1 Assumptions for CDF Curve for MOP EIRP Spherical Coverage, Qualcomm, 3GPP TSG-RAN5 Meeting #89-e, November 2020

- R5-195123, On the MU Term ‘Influence of TRP’ for spurious emissions, Keysight Technologies, 3GPP TSG-RAN WG5 #83, May 2019

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- R5-193689, TRP Integration with Offset Feed Antennas for IFF System, Keysight Technologies, 3GPP TSG-RAN WG5 #83, May 2019

- TR 38.903, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases

- R5-226109, Discussion on FR2 PC1 MU, Keysight Technologies, 3GPP TSG-RAN5 Meeting #97, November 2022