3GPP TSG-RAN5 Meeting #97	R5-226245
Toulouse, France, 14th – 18th November 2022

Agenda item:	5.5.1.12

Source:	Keysight Technologies

Title:	On AP96.25 RTS ATF Messages

Document for:	Discussion and Endorsement

# Introduction

This contribution provides our view on AP#96.25, i.e., the status of messages originally reserved for the two-stage LTE MIMO OTA test method.

# Discussion

In RAN5#96-e, discussions were held on a test function to limit Pcell power [1]. During the discussions, it was noted that proposed messages have been reserved already.

NOTE: RAN5 RF group should check and confirm whether the values 10010010 and 10010011 still need to be reserved for the two-stage MIMO OTA test method, because when I look into the latest (and old) version of TR 36978-d20, I see that those two values are not used.

Subsequently, an action point was created

| AP#96e.25 | RF | check and confirm whether the values 10010010 and 10010011 still need to be reserved for the two-stage LTE MIMO OTA test method | KEYS, Apple, Qualcomm, | R5-224078 | RAN5#97 | Open |
| --- | --- | --- | --- | --- | --- | --- |

to determine whether these reserved messages can be released or will still be needed eventually for the Radiated Two-Stage (RTS) LTE MIMO OTA test method.

RTS is a harmonized and permitted test method for LTE MIMO OTA testing [2][3] which relies on an antenna test function (ATF) defined in [4], specifically Clauses 6.17 and 6.18. Note 3 in Clause 6 lists 4 messages reserved for RTS

NOTE 3: E-UTRA, UTRA and GSM/GPRS test control messages use the same protocol discriminator value ("1111"). Following message type value series are reserved for GSM/GPRS testing commands as specified by TS 44.014 [33]: 0000xxxx, 0001xxxx and 0010xxxx where x represent 0 or 1. Following message type value series are reserved for UTRA testing commands as specified by TS 34.109 [11]: 0100xxxx where x represent 0 or 1. For E-UTRA and NB-IoT test commands the message type value series 1000xxxx and 1001xxxx are reserved, where the message type values 10010000, 10010001, 10010010 and 10010011 are reserved for possible future use by the antenna test function defined in TR 36.978 for use by the two-stage MIMO OTA test method. Should MIMO OTA conformance tests be defined in the future which are applicable to the two-stage method then the message definitions in TR 36.978 may be moved to this specification.

After internal review, the current status of these 4 messages is summarized in Table 1.

Table 1: Status of Reserved Messages for use by RTS

| Message | Status |
| --- | --- |
| 10010000 | Used for ANTENNA INFORMATION REQUEST (Clause 6.17 [4]) |
| 10010001 | Used for ANTENNA INFORMATION RESPONSE (Clause 6.18 [4]) |
| 10010010 | Was originally used in proprietary/pre-standard ATFs; not needed for 3GPP conformance testing and reservation of this message can be released |
| 10010011 | Was originally used in proprietary/pre-standard ATFs; not needed for 3GPP conformance testing and reservation of this message can be released |

It is therefore proposed to confirm that the reservation of messages 10010010 and 10010011 is no longer required while confirming the reservation of messages 10010000 and 10010001 defined in [4] already.

Proposal 1: Confirm that the reservation of messages 10010010 and 10010011 is no longer required while confirming the reservation of messages 10010000 and 10010001 defined in [4] already.

A CR to make appropriate change in the note 3 of Clause 6 in [4] is in [5].

# Conclusion

The following observations and conclusions were made in this contribution.

Proposal 1: Confirm that the reservation of messages 10010010 and 10010011 is no longer required while confirming the reservation of messages 10010000 and 10010001 defined in [4] already.

# References

- R5-224080, Discussion on message type for Power limit test functions, Keysight Technologies UK Ltd, Apple Portugal, 3GPP TSG-RAN5 Meeting #96-e, August 2022

- TR 37.977, Verification of radiated multi-antenna reception performance of User Equipment (UE)

- TR 36.978, User Equipment (UE) antenna test function definition for two-stage Multiple Input Multiple Output (MIMO) Over The Air (OTA) test method

- TS 36.509, Special conformance testing functions for User Equipment (UE)

- R5-226246, Clarification of RTS messages, NTT DOCOMO INC., 3GPP TSG-RAN5 Meeting #97, November 2022