3GPP TSG-RAN5 Meeting #97	R5-226303

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3233 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 6.1.2.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per the prose , table 6.1.2.11.3.3-1: SIB1 of NR cell 1 and NR cell 3 , SIB1 is getting updated for Cell 3 at Step 0 when Cell 3 is OFF. Cell 3 is getting powered up only at Step 3A. There is no point in broadcasting updated SIB1 for NR Cell3 when it is OFF. Hence it is proposed to set NR Cell 3 as non-suitable cell in T0 power level setting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Cell 3 is set as NonSuitable Cell before changing the system information on that cell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.1.2.11	Area Specific SIBs using systemInformationAreaID

6.1.2.11.1	Test Purpose (TP)

(1)

with { UE is in NR RRC_IDLE state }

ensure that {

when { SIB1 does not include si-RequestConfig and si-BroadcastStatus set to notBroadcasting }

then { UE sends RRCSystemInfoRequest message and acquires the necessary SIBs }

}

(2)

with { UE in NR_RRC Idle State }

ensure that {

when { UE reselects to neighbor cell with same systemInformationAreaID and SIB1 does not include si-RequestConfig and si-BroadcastStatus set to notBroadcasting }

then { UE applies system information parameters of previous cell for reselection purposes }

}

6.1.2.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.304, clause 5.2.4.4, 5.2.4.6 and 5.3.1. Unless otherwise stated these are Rel-15 requirements.

[TS 38.304, clause 5.2.4.2]

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NR inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8].

-	For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells of equal or lower priority;

-	Otherwise, the UE shall perform measurements of NR inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 38.133 [8].

[TS 38.304, clause 5.2.4.6]

If threshServingLowQ is broadcast in system information and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a higher priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority NR or EUTRAN RAT/frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT

Otherwise, cell reselection to a cell on a higher priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority NR frequency shall be based on ranking for intra-frequency cell reselection as defined in clause 5.2.4.6.

If threshServingLowQ is broadcast in system information and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a lower priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority NR or E-UTRAN RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a lower priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/frequency shall take precedence over a lower priority RAT/frequency if multiple cells of different priorities fulfil the cell reselection criteria.

If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an NR frequency, the highest ranked cell among the cells on the highest priority frequency(ies) meeting the criteria according to clause 5.2.4.6;

-	If the highest-priority frequency is from another RAT, the strongest cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

[TS 38.331, clause 5.2.2.2.1]

The UE shall apply the SI acquisition procedure as defined in clause 5.2.2.3 upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync completion, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification; and whenever the UE does not have a valid version of a stored SIB.

When the UE acquires a MIB or a SIB1 or an SI message in a serving cell as described in clause 5.2.2.3, and if the UE stores the acquired SIB, then the UE shall store the associated areaScope, if present, the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity, the systemInformationAreaID, if present, and the valueTag, if present, as indicated in the si-SchedulingInfo for the SIB. The UE may use a valid stored version of the SI except MIB, SIB1, SIB6, SIB7 or SIB8 e.g. after cell re-selection, upon return from out of coverage or after the reception of SI change indication.

NOTE:	The storage and management of the stored SIBs in addition to the SIBs valid for the current serving cell is left to UE implementation.

The UE shall:

1>	delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid;

1>	for each stored version of a SIB:

2>	if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

…

[TS 38.331, clause 5.2.2.3.3]

The UE shall:

…

1>	else:

2>	apply the timeAlignmentTimerCommon included in SIB1;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	initiate transmission of the RRCSystemInfoRequest message in accordance with 5.2.2.3.4;

2>	if acknowledgement for RRCSystemInfoRequest message is received from lower layers:

3>	acquire the requested SI message(s) as defined in subclause 5.2.2.3.2, immediately;

….

[TS 38.331, clause 5.2.2.3.4]

The UE shall set the contents of RRCSystemInfoRequest message as follows:

1>	set the requested-SI-List to indicate the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting.

The UE shall submit the RRCSystemInfoRequest message to lower layers for transmission.

6.1.2.11.3	Test description

6.1.2.11.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 3 and NR Cell 6.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR Cells.

UE:

None.

Preamble:

-	The UE is in NR RRC Idle mode (state 1N-A) on NR Cell 1 according to 38.508-1 [4] Table 4.4A.2-1.

6.1.2.11.3.2	Test procedure sequence

Table 6.1.2.11.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration T0 indicates the initial conditions for preamble. Configurations marked "T1" and "T2" is applied at the points indicated in the Main behaviour description in Table 6.1.2.11.3.2-3.

Table 6.1.2.11.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | NR Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -115 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -95 | -80 | Off | The power level values are assigned to satisfy SrxlevNRCell3 > ThreshX, HighP |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | Off | -88 | -80 | The power level values are assigned to satisfy RNRCell 3 < RNRCell 6 |

Table 6.1.2.11.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | NR Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | Off | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | Off | The power level values are assigned to satisfy SrxlevNRCell3 > ThreshX, HighP |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | Off | FFS | FFS | The power level values are assigned to satisfy RNRCell 3 < RNRCell 6 |

Table 6.1.2.11.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS changes SIB1 as specified as Table 6.1.2.11.3.3-1. Note: The sysinfo combination of NR cell 1 is changed to NR-4. | - | - | - | - |
| 0A | The SS notifies the UE of change of System Information on NR Cell 1 by send Short Message on PDCCH using P-RNTI. | <-- | NR PDCCH (DCI 1_0): Short Message | - | - |
| 1 | Void | - | - | - | - |
| 2 | UE sends RRCSystemInfoRequest message to NR Cell 1 to request SIB4. | --> | NR RRC: RRCSystemInfoRequest | 1 | P |
| - | EXCEPTION: After receving RRCSystemInfoRequest message in step 2, the SS ignores the reception of subsequent RRCSystemInfoRequest. | - | - | - | - |
| 3 | NR Cell 1 starts broadcasting SystemInformation message which including SIB4. | <-- | NR RRC: SystemInformation | - | - |
| 3A | The SS re-adjusts the SS/PBCH EPRE levels and according to row "T1" in table 6.1.2.11.3.2-1/2. | - | - | - | - |
| 4 | Wait 34s to let UE to perform inter-frequency cell reselection to NR Cell 3 (Note 1). | - | - | - | - |
| 4A | Does the UE send RRCSystemInfoRequest message on NR Cell 3 to request SIB4 in the next 5s? |  |  | 2 | F |
| 4B | The test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 3. | - | - | - | - |
| 5 | The SS re-adjusts the SS/PBCH EPRE levels and according to row "T2" in table 6.1.2.11.3.2-1/2. | - | - | - | - |
| 6 | Void |  |  |  |  |
| 6A | Wait 34s to let UE to perform inter-frequency cell reselection to NR Cell 6.(Note 1) | - | - | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 6? | - | - | 2 | P |
| Note 1: The wait time for cell reselection procedure is selected to cover Tdetect, NR_inter (32s) + TSI-NR (1280 ms) = 33.28s rounded up to 34s. |  |  |  |  |  |

6.1.2.11.3.3	Specific message contents

Table 6.1.2.11.3.3-1: SIB1 of NR cell 1 and NR cell 3 (All steps in Table 6.1.2.11.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 with Condition SIG AND FR1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| si-SchedulingInfo |  | SI-SchedulingInfo-Area | Table 6.1.2.11.3.3-2 |  |
| } |  |  |  |  |

Table 6.1.2.11.3.3-2: SI-SchedulingInfo-Area (Table 6.1.2.11.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-173 with Condition FR1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI-SchedulingInfo ::= SEQUENCE { |  |  |  |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo { | 2 entries |  |  |
| SchedulingInfo[1] SEQUENCE { |  | entry 1 |  |
| si-BroadcastStatus | broadcasting |  |  |
| si-Periodicity | rf32 |  |  |
| sib-MappingInfo SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo { | 1 entry |  |  |
| SIB-TypeInfo[1] SEQUENCE { |  | entry 1 |  |
| type | sibType2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| SchedulingInfo[2] SEQUENCE { |  | entry 2 |  |
| si-BroadcastStatus | not broadcasting |  |  |
| si-Periodicity | rf64 |  |  |
| sib-MappingInfo SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo { | 1 entry |  |  |
| SIB-TypeInfo[1] SEQUENCE { |  | entry 1 |  |
| type | sibType4 |  |  |
| valueTag | 0 |  |  |
| areaScope | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| systemInformationAreaID | ’0000 0000 0000 0000 0000 0011’B |  |  |
| } |  |  |  |

Table 6.1.2.11.3.3-3: RRCSystemInfoRequest (Step 2, Table 6.1.2.11.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSystemInfoRequest ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSystemInfoRequest-r15 SEQUENCE { |  |  |  |  |
| requested-SI-List |  | 01000000000000000000000000000000 | Bit corresponds to SIB4 in schedulingInfoList |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.1.2.11.3.3-4: SystemInformation (Step 3, Table 6.1.2.11.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-29 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SystemInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| systemInformation-r15 SEQUENCE { |  |  |  |  |
| sib-TypeAndInfo SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { |  | 1 entry |  |  |
| SIB4 SEQUENCE { |  | As per TS 38.508-1 [4] Table 4.6.2-3 |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { |  | 2 entries |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  |  | entry 1 |  |
| dl-CarrierFreq |  | Same downlink NR ARFCN as used for NR Cell 3 |  |  |
| smtc |  | SSB-MTC as defined in TS 38.508-1 [4] Table 4.6.3-185 with condition INTER-FREQ_ODD |  |  |
| } |  |  |  |  |
| InterFreqCarrierFreqInfo[2] SEQUENCE { |  |  | entry 2 |  |
| dl-CarrierFreq |  | Same downlink NR ARFCN as used for NR Cell 6 |  |  |
| smtc |  | SSB-MTC as defined in TS 38.508-1 [4] Table 4.6.3-185 with condition INTER-FREQ_ODD | smtc of the inter-freq carrier for NR Cell 6 is calculated with respect to NR Cell 3 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |