3GPP TSG-RAN5 Meeting #97	R5-226353

# Toulouse, France, Nov 14th - 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3245 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the CGI specific elements in UE-NR-Capability for MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update the CGI specific elements in UE-NR-Capability for MR-DC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the value/remark of CGI specific information elements in UE-NR-Capability for MR-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CGI specific information elements will not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

8.2.1.1.1	UE capability transfer / Success / EN-DC

<<UNCHANGED PARTS SKIPPED>>

Table 8.2.1.1.1.3.3-6: UE-NR-Capability (Table 8.2.1.1.1.3.3-5)

| Derivation Path: TS 38.331 [12], clause 6.3.3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| UE-NR-Capability ::= SEQUENCE { |  |  |  |  |  |  |  |
| accessStratumRelease |  | Checked |  | "rel-15" or higher |  |  |  |
| pdcp-Parameters SEQUENCE { |  |  |  |  |  |  |  |
| supportedROHC-Profiles SEQUENCE { |  |  |  |  |  |  |  |
| profile0x0000 |  | Not checked |  |  |  |  |  |
| profile0x0001 |  | Not checked |  |  |  |  |  |
| profile0x0002 |  | Not checked |  |  |  |  |  |
| profile0x0003 |  | Not checked |  |  |  |  |  |
| profile0x0004 |  | Not checked |  |  |  |  |  |
| profile0x0006 |  | Not checked |  |  |  |  |  |
| profile0x0101 |  | Not checked |  |  |  |  |  |
| profile0x0102 |  | Not checked |  |  |  |  |  |
| profile0x0103 |  | Not checked |  |  |  |  |  |
| profile0x0104 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| maxNumberROHC-ContextSessions |  | Not checked |  |  |  |  |  |
| uplinkOnlyROHC-Profiles |  | Not checked |  |  |  |  |  |
| <<UNCHANGED PARTS SKIPPED>> |  |  |  |  |  |  |  |
| eventB-MeasAndReport |  | Checked |  |  |  | pc_eventB_MeasAndReport |  |
| handoverFDD-TDD |  | Not checked |  |  |  |  |  |
| eutra-CGI-Reporting |  | Checked |  |  |  | pc_eutra_CGI_Reporting |  |
| nr-CGI-Reporting |  | Checked |  |  |  | pc_nr_CGI_Reporting |  |
| independentGapConfig |  | Checked |  |  |  | pc_independentGapConfig |  |
| periodicEUTRA-MeasAndReport |  | Checked |  |  |  | pc_periodicEUTRA_MeasAndReport |  |
| handoverFR1-FR2 |  | Not checked |  |  |  |  |  |
| maxNumberCSI-RS-RRM-RS-SINR |  | Not checked |  |  |  |  |  |
| nr-CGI-Reporting-ENDC |  | Checked |  |  |  | pc_nr_CGI_Reporting_ENDC |  |
| eutra-CGI-Reporting-NEDC |  | Not checked |  |  |  |  |  |
| eutra-CGI-Reporting-NRDC |  | Not checked |  |  |  |  |  |
| nr-CGI-Reporting-NEDC |  | Not checked |  |  |  |  |  |
| nr-CGI-Reporting-NRDC |  | Not checked |  |  |  |  |  |
| reportAddNeighMeasForPeriodic-r16 |  | Not checked |  |  |  |  |  |
| condHandoverParametersCommon-r16 |  | Not checked |  |  |  |  |  |
| nr-NeedForGap-Reporting-r16 |  | Not checked |  |  |  |  |  |
| <<UNCHANGED PARTS SKIPPED>> |  |  |  |  |  |  |  |

8.2.1.1.2	UE capability transfer / Success / NE-DC

<<UNCHANGED PARTS SKIPPED>>

Table 8.2.1.1.2.3.3-4: UE-NR-Capability (Table 8.2.1.1.2.3.3-3)

| Derivation Path: TS 38.331 [12], clause 6.3.3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| UE-NR-Capability ::= SEQUENCE { |  |  |  |  |  |  |  |
| accessStratumRelease |  | Checked |  | "rel-15" or higher |  |  |  |
| pdcp-Parameters SEQUENCE { |  |  |  |  |  |  |  |
| supportedROHC-Profiles SEQUENCE { |  |  |  |  |  |  |  |
| profile0x0000 |  | Not checked |  |  |  |  |  |
| profile0x0001 |  | Not checked |  |  |  |  |  |
| profile0x0002 |  | Not checked |  |  |  |  |  |
| profile0x0003 |  | Not checked |  |  |  |  |  |
| profile0x0004 |  | Not checked |  |  |  |  |  |
| profile0x0006 |  | Not checked |  |  |  |  |  |
| profile0x0101 |  | Not checked |  |  |  |  |  |
| profile0x0102 |  | Not checked |  |  |  |  |  |
| profile0x0103 |  | Not checked |  |  |  |  |  |
| profile0x0104 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| maxNumberROHC-ContextSessions |  | Not checked |  |  |  |  |  |
| uplinkOnlyROHC-Profiles |  | Not checked |  |  |  |  |  |
| <<UNCHANGED PARTS SKIPPED>> |  |  |  |  |  |  |  |
| handoverFDD-TDD |  | Not checked |  |  |  |  |  |
| eutra-CGI-Reporting |  | Checked |  |  |  | pc_eutra_CGI_Reporting |  |
| nr-CGI-Reporting |  | Checked |  |  |  | pc_nr_CGI_Reporting |  |
| independentGapConfig |  | Checked |  |  |  | pc_independentGapConfig |  |
| periodicEUTRA-MeasAndReport |  | Checked |  |  |  | pc_periodicEUTRA_MeasAndReport |  |
| handoverFR1-FR2 |  | Not checked |  |  |  |  |  |
| maxNumberCSI-RS-RRM-RS-SINR |  | Not checked |  |  |  |  |  |
| nr-CGI-Reporting-ENDC |  | Not checked |  |  |  |  |  |
| eutra-CGI-Reporting-NEDC |  | Checked |  |  |  | pc_eutra_CGI_Reporting_NEDC |  |
| eutra-CGI-Reporting-NRDC |  | Not checked |  |  |  |  |  |
| nr-CGI-Reporting-NEDC |  | Checked |  |  |  | pc_nr_CGI_Reporting_NEDC |  |
| nr-CGI-Reporting-NRDC |  | Not checked |  |  |  |  |  |
| reportAddNeighMeasForPeriodic-r16 |  | Not checked |  |  |  |  |  |
| condHandoverParametersCommon-r16 |  | Not checked |  |  |  |  |  |
| nr-NeedForGap-Reporting-r16 |  | Not checked |  |  |  |  |  |
| <<UNCHANGED PARTS SKIPPED>> |  |  |  |  |  |  |  |

<< END OF CHANGES >>