3GPP TSG-RAN5 Meeting #97	R5-226376

# Toulouse, France, 14th – 18th Nov 2022

Title:							Introduction of AWGN and signal flatness for FR2 to TS 38.533

Source:			Anritsu

Agenda Item:			5.4.17

Document for:			Endorsement

# 1.	Introduction

For TT analysis for FR2 RRC-based DL active BWP switch test cases [1], AWGN and signal flatness and signal-to-noise ratio uncertainty are used to calculate test system uncertainty in the test cases. However, AWGN and signal flatness is not defined for FR2 in TS 38.533 V17.4.0. This paper proposes to introduce AWGN and signal flatness for FR2 to TS 38.533.

For test case 5.5.6.2.1, the SS provides AWGN and one cell to the UE. Note that fading, minimum test time, and wireless cable mode are not used in this test case. The following parameters are specified in TS 38.521-4 Table F.2.1.2-1:

- AWGN flatness and signal flatness ±3.6 dB

- Signal-to-noise ratio uncertainty ±0.3 dB

Test System uncertainty on Cell1 = SQRT (0.32 + (0.25 * 3.6)2) = 0.94dB, round to 1.0dB

Considering the test system uncertainty, the minimum SNR (Es/Iot) of this test case is decreased to 18-1.0=17.0dB

# 2.	Discussion

As shown below, AWGN and signal flatness for FR2 was already discussed for Demod Performance test cases in TS 38.521-4. For FR2 RRM test cases, we propose to use the same value as Demod Performance test cases.

Table F.2.1.2-1: Maximum measurement uncertainty values for the test system for FR2 (up to 40 GHz) and Channel BW ≤ 400 MHz

| MU contributor |  | Unit |  | Value |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWconfig |  | dB |  | ±3.6 |  |  |  |
| gNB emulator Signal to noise ratio uncertainty |  | dB |  | ±0.3 |  |  |  |
| Impact on non-ideal isolation between branches for the wireless cable mode |  | dB |  | 0.60 for Rank1 0.45 for Rank2 |  | Systematic uncertainty |
| Fading profile power uncertainty |  | dB |  | ±0.5 for 1Tx ±0.7 for 2Tx |  |  |  |
| SNR uncertainty due to finite test time |  | dB |  | ±0.3 for PDSCH and doppler < 100Hz 0.0 for PDSCH and doppler ≥ ±0.4 for PDCCH |  |  |  |

Observation 1: AWGN and signal flatness is +/- 3.6 dB in FR2 Demod Performance test cases.

Proposal 1: Adopt +/- 3.6 dB for AWGN and signal flatness of FR2 RRM test cases.

# 3.	Conclusion

RAN5 is asked to endorse following proposals.

Proposal 1: Adopt +/- 3.6 dB for AWGN and signal flatness of FR2 RRM test cases.

# 4.	References

[1] R5-226375, “Test tolerance analysis for FR2 RRC-based DL active BWP switch”, Anritsu, RAN5#97