3GPP TSG-RAN5 Meeting #97	R5-226387

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2611 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of parameter related to SOR-CMCI for DL NAS transport and UL NAS transport message in 38.508-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCPSOR_CON-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN#96-e, WI of UE Conformance Test Aspects - Enhancement for the 5G Control Plane Steering of Roaming for UE in Connected mode has been approved, parameter related to SOR-CMCI for DL NAS transport and UL NAS transport message be introduced into 38.508-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding parameter related to SOR-CMCI for DL NAS transport and UL NAS transport message in 38.508-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The parameter related to SOR-CMCI for DL NAS transport and UL NAS transport message will be still missing and this WI willn’t be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

### 4.7.1	Contents of 5GMM messages

#### –	UL NAS transport

Table 4.7.1-10: UL NAS TRANSPORT

| Derivation Path: 24.501 clause 8.2.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| UL NAS TRANSPORT message identity |  | ‘0110 0111’B |  |  |
| Payload container type |  | Set according to specific message content |  |  |
| Payload container type |  | ‘0001’B | N1 SM information | INITIAL_PDU_REQUEST |
| Payload container type |  | ‘0100’B | SOR transparent container | 5GS_SOR |
| Spare half octet |  | '0000'B |  |  |
| Payload container |  | Set according to specific message content |  |  |
| Payload container |  | Present | The SOR transparent container in the payload container IE carries steering of roaming information. | 5GS_SOR |
| Length of payload container contents |  |  | 2 octet |  |
| Payload container contents |  |  |  |  |
| SOR transparent container |  | Present | The SOR transparent container carries steering of roaming information. |  |
| Length of SOR transparent container contents |  |  | 2 octet |  |
| SOR header |  |  |  | 1 octet |  |  |  |
| SOR data type |  | 1 | The SOR transparent container carries acknowledgement of successful reception of the steering of roaming information. |  |
| ME support of SOR-CMCI indicator (MSSI) value |  | If present: contents not checked |  |  |
| ME support of SOR-CMCI indicator (MSSI) value |  | 1 | SOR-CMCI supported by the ME | 5GS_SOR-CMCI |
| ME support of SOR-SNPN-SI indicator (MSSNPNSI) value |  | If present: contents not checked |  |  |
| SOR-MAC-IAUSF |  | Set to match the calculated SoR-MAC-IAUS as the way defined in TS 33.501 A.17 |  |  |
| Payload container |  | PDU SESSION ESTABLISHMENT REQUEST message |  | INITIAL_PDU_REQUEST |
| PDU session ID |  | If present: contents not checked |  |  |
| PDU session ID |  | Same PDU session ID as defined in the PDU SESSION ESTABLISHMENT REQUEST message in the Payload container |  | INITIAL_PDU_REQUEST |
| Old PDU session ID |  | If present: contents not checked |  |  |
| Request type |  | If present: contents not checked |  |  |
| Request type |  | ‘001’B | initial request | INITIAL_PDU_REQUEST |
| S-NSSAI |  | If present: contents not checked |  |  |
| DNN |  | If present: contents not checked | (NOTE 1) |  |
| Additional information |  | If present: contents not checked |  |  |
| MA PDU session information |  | If present: contents not checked |  |  |
| Release assistance indication |  | If present: contents not checked |  |  |
| NOTE 1: Although the contents of the IE is not required to be verified for PASS/FAIL purposes, the provided information shall be taken into account e.g. for the building the content of messages and allowing for specific UE behaviour as specified in Table 4.8.4-1. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| INITIAL_PDU_REQUEST | The UL NAS TRANSPORT message is used to transport a PDU SESSION ESTABLISHMENT REQUEST message to establish a new PDU session. |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	DL NAS transport

Table 4.7.1-11: DL NAS TRANSPORT

| Derivation Path: 24.501 clause 8.2.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| DL NAS TRANSPORT message identity |  | ‘0110 1000’B |  |  |
| Payload container type |  | Set according to specific message content |  |  |
| Payload container type |  | ‘0001’B | N1 SM information | 5GSM_MESSAGE |
| Payload container type |  | ‘0100’B | SOR transparent container | 5GS_SOR |
| Spare half octet |  | '0000'B |  |  |
| Payload container |  | Set according to specific message content |  |  |
| Payload container |  | 5GSM message |  | 5GSM_MESSAGE |
| Payload container |  | Present | The SOR transparent container in the payload container IE carries steering of roaming information. | 5GS_SOR |
| Length of payload container contents |  |  | 2 octet |  |
| Payload container contents |  |  |  |  |
| SOR transparent container |  | Present | The SOR transparent container carries steering of roaming information. |  |
| Length of SOR transparent container contents |  |  | 2 octet |  |
| SOR header |  |  |  |  |  | 1 octet |  |  |  |  |  |
| SOR data type |  | 0 | The SOR transparent container carries steering of roaming information. |  |
| List indication value |  | 1 | List of preferred PLMN/access technology combinations is provided |  |
| List type |  | 1 | The list type is a "PLMN ID and access technology list" |  |
| Acknowledgement (ACK) value |  | 1 | Acknowledgement requested |  |
| Additional parameters (AP) value |  | 0 | Additional parameters not included |  |
| Additional parameters (AP) value |  | 1 | Additional parameters included | 5GS_SOR-CMCI |
| SOR-MAC-IAUSF |  |  | Set to match the calculated SoR-MAC-IAUS as the way defined in TS 33.501 A.17 |  |  |  |  |  |  |  |  |
| CounterSOR |  |  | Value generated as per TS 33.501 Cl 6.14.2.3 |  |  |  |  |  |  |  |  |
| Length of PLMN ID and access technology list |  |  | 1 octet |  |
| PLMN ID and access technology list |  |  | Values determined by test implementation |  |
| SOR-CMCI indicator (SI) value |  | 0 | SOR-CMCI absent |  |
| SOR-CMCI indicator (SI) value |  | 1 | SOR-CMCI present | 5GS_SOR-CMCI |
| Store SOR-CMCI in ME indicator (SSCMI) value |  |  | Value determined by test implementation |  |
| SOR-SNPN-SI indicator (SSSI) value |  | 0 | subscribed SNPN or HPLMN indication that 'no change of the SOR-SNPN-SI stored in the UE is needed and thus no SOR-SNPN-SI is provided' |  |
| SOR-CMCI |  | Present |  | 5GS_SOR-CMCI |
| Length of SOR-CMCI contents |  |  | Length value determined by test implementation |  |
| SOR-CMCI rule 1 |  |  | Value determined by test implementation |  |
| Length of SOR-CMCI rule contents |  |  |  |  |  | Length value determined by test implementation |  |  |  |  |  |
| Tsor-cm timer value |  |  | 00100001 |  |  | 60s |  |  |  |  |  |
| Criterion type |  |  |  |  |  | Value determined by test implementation |  |  |  |  |  |
| Criterion value |  |  |  |  |  | Value determined by test implementation |  |  |  |  |  |
| SOR-SNPN-SI |  | Not Present |  |  |
| PDU session ID |  | Not Present |  |  |
| PDU session ID |  | Set to the same value as the PDU session ID of the 5GSM message in the Payload container. |  | 5GSM_MESSAGE |
| Additional information |  | Not Present |  |  |
| 5GMM cause |  | Not Present |  |  |
| Back-off timer value |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| 5GSM_MESSAGE | The DL NAS TRANSPORT message is used to transport a 5GSM message |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

[End of change]