3GPP TSG-RAN5 Meeting #97	R5-226388

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3253 | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction for test procedure sequence in 9.2.6.1.1.3.2 on TS38.523-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2022-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS.38.523-1, there is is editorial correction 5GS IMS test case 9.2.6.1 “UE-initiated de-registration / switch off”. Editorioal correction point is as follows. #1 Delete overlap description in step17 on Table 9.2.6.1.1.3.2-1: Main behaviour #2 Unify the font in all steps on Table 9.2.6.1.1.3.2-1: Main behaviour #3 Delete space in step30 on Table 9.2.6.1.1.3.2-1: Main behaviour |  |  |  |  |  |  |  |  |
| Summary of change: |  | There is editorial correction 5GS IMS test case 9.2.6.1 “UE-initiated de-registration / switch off”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will make a misreading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

##### 9.2.6.1.1	UE-initiated de-registration / switch off

9.2.6.1.1.1	Test Purpose (TP)

(1)

with { the UE in 5GMM-REGISTERED state }

ensure that {

when { the UE is switched off }

then { the UE shall send DEREGISTRATION REQUEST message with De-registration type IE indicated to "Switch off" }

}

(2)

with { the UE supports remove USIM without power down and in 5GMM-REGISTERED state }

ensure that {

when { the USIM is removed from the UE }

then { the UE shall send DEREGISTRATION REQUEST message with De-registration type IE indicated to "Switch off" }

}

(3)

with { the UE in 5GMM-DEREGISTERED-INTIATED state }

ensure that {

when { the first four expiries of the timer T3521 }

then { the UE shall retransmit the DEREGISTRATION REQUEST message and shall reset and restart timer T3521 }

}

(4)

with { the UE in 5GMM-DEREGISTERED-INTIATED state }

ensure that {

when { On the fifth expiry of timer T3521 }

then { the deregistration procedure shall be aborted and the UE perform local detach }

}

9.2.6.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 5.5.2.1, 5.5.2.2.1 and 5.5.2.2.6. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.2.1]

The de-registration procedure is used:

a)	by the UE to de-register for 5GS services over 3GPP access when the UE is registered over 3GPP access;;

b)	by the UE to de-register for 5GS services over 3GPP access, non-3GPP access, or both when the UE is registered in the same PLMN over both accesses;

c)	by the network to inform the UE that it is deregistered for 5GS services over 3GPP access when the UE is registered over 3GPP access;

d)	by the network to inform the UE that it is deregistered for 5GS services over 3GPP access, non-3GPP access, or both when the UE is registered in the same PLMN over both accesses; and

e)	by the network to inform the UE to re-register to the network.

The de-registration procedure with appropriate de-registration type shall be invoked by the UE:

a)	if the UE is switched off; and

b)	as part of the eCall inactivity procedure defined in subclause 5.5.3.

The de-registration procedure with appropriate de-registration type shall be invoked by the network:

a)	if the network informs whether the UE should re-register to the network.

The de-registration procedure with appropriate access type shall be invoked by the UE:

a)	if the UE wants to de-register for 5GS services over 3GPP access when the UE is registered over 3GPP access; or

b)	the UE wants to de-register for 5GS services over 3GPP access, non-3GPP access, or both when the UE is registered in the same PLMN over both accesses.

If the de-registration procedure is triggered due to USIM removal, the UE shall indicate "switch off" in the de-registration type IE.

If the de-registration procedure is requested by the UDM for a UE that has an emergency PDU session, the AMF shall not send a DEREGISTRATION REQUEST message to the UE.

If the de-registration procedure for 5GS services is performed, the PDU sessions, if any, for this particular UE are released locally without peer-to-peer signalling between the UE and the network.

The UE is allowed to initiate the de-registration procedure even if the timer T3346 is running.

NOTE:	When the UE has no PDU sessions over non-3GPP access, or the UE moves all the PDU sessions over a non-3GPP access to a 3GPP access, the UE and the AMF need not initiate de-registration over the non-3GPP access.

The AMF shall provide the UE with a non-3GPP de-registration timer.

[TS 24.501, clause 5.5.2.2.1]

The de-registration procedure is initiated by the UE by sending a DEREGISTRATION REQUEST message (see example in figure 5.5.2.2.1). The De-registration type IE included in the message indicates whether the de-registration procedure is due to a "switch off" or not. The access type included in the message indicates whether the de-registration procedure is:

a)	for 5GS services over 3GPP access when the UE is registered over 3GPP access only;

b)	for 5GS services over non-3GPP access when the UE is registered over non-3GPP access only; or

c)	for 5GS services over 3GPP access, non-3GPP access or both 3GPP access and non-3GPP access when the UE is registered in the same PLMN over both accesses.

If the UE has a valid 5G-GUTI, the UE shall populate the 5GS mobile identity IE with the valid 5G-GUTI. If the UE does not have a valid 5G-GUTI, the UE shall populate the 5GS mobile identity IE with its SUCI.

If the UE does not have a valid 5G-GUTI and it does not have a valid SUCI, then the UE shall populate the5GSmobile identity IE with its PEI.

If the de-registration request is not due to switch off and the UE is in the state 5GMM-REGISTERED or 5GMM-REGISTERED-INITIATED, timer T3521 shall be started in the UE after the DEREGISTRATION REQUEST message has been sent. The UE shall enter the state 5GMM-DEREGISTERED-INITIATED.

If the UE is to be switched off, the UE shall try for a period of 5 seconds to send the DEREGISTRATION REQUEST message. During this period, the UE may be switched off as soon as the DEREGISTRATION REQUEST message has been sent.

[TS 24.501, clause 5.5.2.2.6]

...

c)	T3521 timeout.

On the first four expiries of the timer, the UE shall retransmit the DEREGISTRATION REQUEST message and shall reset and restart timer T3521. On the fifth expiry of timer T3521, the de-registration procedure shall be aborted and the UE proceeds as follows:

9.2.6.1.1.3	Test description

9.2.6.1.1.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27.

UE:

-	None.

Preamble:

-	The UE is in state 3W-A on WLAN Cell 27 according to 38.508-1 [4].

9.2.6.1.1.3.2	Test procedure sequence

Table 9.2.6.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause switch off | - | - | - | - |
| 2 | Check: Does the UE transmit a DEREGISTRATION REQUEST with the De-registration type IE indicating "switch off"? | --> | DEREGISTRATION REQUEST | 1 | P |
| 3 | SS Transmits DEREGISTRATION ACCEPT | <-- | DEREGISTRATION ACCEPT | - | - |
| 4 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.3 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |
| 5 | The UE is switched on. | - | - | - | - |
| 6-15 | The registration procedure is completed by executing steps 1-10 of the UE registration procedure in TS 38.508-1 [4] table 4.5.2.2-3. | - | - | - | - |
| 16 | Cause UE to initiate deregistration. | - | - | - | - |
| 17 | The UE transmits a DEREGISTRATION REQUEST message. The UE starts timer T3521. | --> | DEREGISTRATION REQUEST | - | - |
| 18 | The SS does not respond to the DEREGISTRATION REQUEST message. | - | - | - | - |
| 19 | Check: When the timer T3521 expires does the UE re-transmit DETACH REQUEST message. Timer T3421 is re-started (1st expiry). | --> | DEREGISTRATION REQUEST | 3 | P |
| 20 | The SS does not respond to the DEREGISTRATION REQUEST message. | - | - | - | - |
| 21 | Check: When the timer T3521 expires does the UE re-transmit DEREGISTRATION REQUEST message. Timer T3521 is re-started (2nd expiry). | --> | DEREGISTRATION REQUEST | 3 | P |
| 22 | The SS does not respond to the DEREGISTRATION REQUEST message. | - | - | - | - |
| 23 | Check: When the timer T3521 expires does the UE re-transmit DEREGISTRATION REQUEST message. Timer T3521 is re-started (3rd expiry). | --> | DEREGISTRATION REQUEST | 3 | P |
| 24 | The SS does not respond to the DEREGISTRATION REQUEST message. | - | - | - | - |
| 25 | Check: When the timer T3521 expires does the UE re-transmit DEREGISTRATION REQUEST message. Timer T3521 is re-started (4th expiry). | --> | DEREGISTRATION REQUEST | 3 | P |
| 26 | The SS does not respond to the DETACH REQUEST message. | - | - | - | - |
| 27 | When the timer T3521 expires the UE aborts the detach procedure and performs a local detach (5th expiry). | - | - | 4 | P |
| 28 | SS Transmits PDU SESSION MODIFICATION COMMAND | <-- | PDU SESSION MODIFICATION COMMAND | - | - |
| 29 | Check: Does the UE transmit a PDU SESSION MODIFICATION COMPLETE? | --> | PDU SESSION MODIFICATION COMPLETE | 4 | F |
| 30 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.3 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |
| - | EXCEPTION: Steps 31 to 46 shall be implemented if the UE supports remove USIM without power down: pc_USIM_Removal = TRUE [29] | - | - | - | - |
| 31 | The UE is switched off. | - | - | - | - |
| 32 | The UE is switched on. | - | - | - | - |
| 33-42 | The registration procedure is completed by executing steps 1-10 of the UE registration procedure in TS 38.508-1 [4] table 4.5.2.2-3. | - | - | - | - |
| 43 | Cause removal of USIM from the UE without powering down. | - | - | - | - |
| 44 | Check: Does the UE transmit a DEREGISTRATION REQUEST with the De-registration type IE indicating "switch off"? | --> | DEREGISTRATION REQUEST | 2 | P |
| 45 | SS Transmits DEREGISTRATION ACCEPT | <-- | DEREGISTRATION ACCEPT | - | - |
| 46 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.3 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |

9.2.6.1.1.3.3	Specific message contents

Table 9.2.6.1.1.3.3-1: DEREGISTRATION REQUEST (Step 1 and step 44, Table 9.2.6.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| De-registration type |  |  |  |
| Switch off | ‘1’B |  |  |

[End of change]