3GPP TSG-RAN5 Meeting #97	R5-226411

# Toulouse, France, November 14 - 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1953 | rev |  | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Rx Requirements for CA_n66A-n77A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon, Qualcomm, Ericsson, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update reference sensitivity test requirement for CA_n66A-n77A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the following tables for CA_n66A-n77A: - Table 7.3A.0.3.2.1-1: ΔRIB,c due to CA (two bands) - Table 7.3A.0.4-1a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1 - Table 7.3A.1.5-1: Reference sensitivity requirement for 2DL CA - Table 7.3A.1_1.4.1-1: Test Configuration Table for inter-band 2DL CA exceptions - Table 7.3A.1_1.5-1: Reference sensitivity requirement for inter band CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core requirement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.0, 7.3A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0680 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### <Start of Changes>

#### 7.3A.0.3	ΔRIB,c for CA

##### 7.3A.0.3.1	General

For a UE supporting a CA configuration, the ΔRIB,c applies for both SC and CA operation.

##### 7.3A.0.3.2	ΔRIB,c for Inter-band CA

For the UE which supports inter-band carrier aggregation, the minimum requirement for reference sensitivity in subclause 7.3A.0 shall be increased by the amount given by ΔRIB,c defined in subclause 7.3A.0.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in subclause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied.

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in subclause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.521-3 [14] for the applicable operating bands.

7.3A.0.3.2.1	ΔRIB,c for two bands

Table 7.3A.0.3.2.1-1: ΔRIB,c due to CA (two bands)

| Inter-band CA configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1-n77 | n1 | 0.2 |
|  | n77 | 0.5 |
| CA_n1-n78 | n78 | 0.5 |
| CA_n2-n48 | n2 | 0.2 |
|  | n48 | 0.5 |
| CA_n2-n66 | n2 | 0.3 |
|  | n66 | 0.3 |
| CA_n2-n77 | n2 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n41 | n41 | 02 |
|  |  | 0.53 |
| CA_n3-n77 | n3 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n78 | n3 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n79 | n79 | 0.5 |
| CA_n5-n77 | n5 | 0.2 |
|  | n77 | 0.5 |
| CA_n5-n78 | n5 | 0.2 |
|  | n78 | 0.5 |
| CA_n7-n78 | n7 | 0.5 |
|  | n78 | 0.5 |
| CA_n8-n78 | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n8-n79 | n79 | 0.5 |
| CA_n24-n48 | n24 | 0.2 |
|  | n48 | 0.5 |
| CA_n24-n77 | n24 | 0.2 |
|  | n77 | 0.5 |
| CA n28-n75 | n28 | 0.2 |
| CA_n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n28-n79 | n28 | 0.2 |
|  | n79 | 0.5 |
| CA_n41-n781 | n78 | 0.5 |
| CA_n41-n79 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n48-n66 | n48 | 0.5 |
|  | n66 | 0.2 |
| CA_n48-n70 | n48 | 0.5 |
|  | n70 | 0.2 |
| CA_n66A-n77A | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n75-n78 | n78 | 0.5 |
| CA_n76-n78 | n78 | 0.5 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

7.3A.0.3.2.2	Void

7.3A.0.3.2.3	ΔRIB,c for three bands

Table 7.3A.0.3.2.3-1: ΔRIB,c due to CA (three bands)

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1-n78-n79 | n78 | 0.5 |
| CA_n26-n66-n70 | n26 | 0 |
|  | n66 | 0 |
|  | n70 | 0 |
| CA_n26-n70-n71 | n26 | 0 |
|  | n70 | 0 |
|  | n71 | 0 |
| CA_n48-n66-n70 | n48 | 0.5 |
|  | n66 | 0.2 |
|  | n70 | 0.2 |
| CA_n48-n66-n71 | n48 | 0.2 |
|  | n70 | 0.2 |
|  | n71 | 0.2 |
| CA_n48-n70-n71 | n48 | 0.2 |
|  | n70 | 0.2 |
|  | n71 | 0.2 |
| CA_n66-n70-n71 | n66 | 0 |
|  | n70 | 0 |
|  | n71 | 0 |

7.3A.0.3.2.4	ΔRIB,c for four bands

Table 7.3A.0.3.2.4-1: ΔRIB,c due to CA (four bands)

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |

#### 7.3A.0.4	Reference sensitivity exceptions due to UL harmonic interference for CA

Editor’s Note: Table 7.3A.0.4-1 format is different from 38.101-1 (V17.6.0) Table 7.3A.4-1. The old format will exist until RAN5 has final solutions on how to adopt RAN4 corresponding table for the minimum requirement of Reference sensitivity exceptions due to UL harmonic, and the resulted possible new format of Table 7.3A.1_1.4.1-1: Test Configuration Table for inter-band 2DL CA exceptions, and Table 7.3A.1_1.5-1: Reference sensitivity requirement for inter band CA.

Sensitivity degradation is allowed for a band in frequency range 1 if it is impacted by UL harmonic interference from another band in frequency range 1 of the same CA configuration.  Reference sensitivity exceptions are specified in Table 7.3A.0.4-1 with uplink configuration specified in Table 7.3A.0.4-2.

Table 7.3A.0.4-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

|  | MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHZ | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB |  | dB | dB | dB |
| n1 | n771,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n3 | n771,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.9 | 16.1 |  | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n3 | n781,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.9 | 16.1 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n5 | n774,5,13 |  | 10.5 | 8.9 | 7.8 | 7.2 | 6.5 | 5.1 | 4.2 | 3.5 | 2.8 | 2.3 | 2.1 | 1.4 |
| n5 | n776,7,13 |  | 10.4 | 8.9 | 7.8 | 6.7 | 6.0 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| n5 | n784,5 |  | 10.5 | 8.9 | 7.8 | 7.1 | 6.5 | 5.4 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
| n8 | n784,5 |  | 10.8 | 9.1 | 8.0 |  |  | 5.1 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
| n8 | n796,7 |  |  |  |  |  |  | [6.8] | 6.2 | [5.6] |  | 4.9 |  | 4.4 |
| n24 | n771,2,13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n773,13 |  | 1.1 | 0.8 | 0.3 | 0.1 |  |  |  |  |  |  |  |  |
| n28 | n751,2 | 28.1 | 25.3 | 24.0 | 22.8 |  |  |  |  |  |  |  |  |  |
|  | n786,7 |  | [10.4] | [8.9] | [7.8] |  |  | [4.7] | [3.7] | [3] |  | [1.7] | [1.2] | [0.7] |
| n66 | n481,2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.912 | 16.112 |  |  | 14.812 | 14.312 |
|  | n483 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n71 | n708,9 | 9.9 | 7.1 | 6.7 | 4.9 | 4.1 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n3-n77, CA_n3-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 5: The requirements should be verified for UL NR-ARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 7: The requirements should be verified for UL NR-ARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 10: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 11: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.2 apply unless otherwise specified). NOTE 12: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.0.4-1a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n2 | n48 | 5 | 15 | 25 (RBstart=0) | 5 | 27.1 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n48 | 10 | 15 | 50 (RBstart=0) | 1007 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n48 | 5 | 15 | 25 (RBstart=0) | 10 | 1.9 | NOTE 6 | UL2/DL1 near-miss |
| n2 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |

#### <skipped unchaged sections>

### 7.3A.1	Reference sensitivity power level for 2DL CA without exception

7.3A.1.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when no CA exceptions are allowed and single carrier requirements apply.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support NR 2DL CA.

7.3A.1.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.1.4	Test description

7.3A.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.1.4.1-1, 7.3A.1.4.1-2 and 7.3A.1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.1.4.1-1: Test Configuration Table for intra-band contiguous 2DL CA without exception

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range, High range |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 3) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest |  |  |
| Test Parameters CA Configurations |  |  |  |  |  |  |
| CA Configuration /NRB |  | DL Allocation |  | UL Allocation |  |  |
| PCC NRB | SCC NRB | CC MOD | PCC & SCC RB allocation | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
| Lowest NRB_agg (NOTE 4) | Lowest NRB_agg (NOTE 4) | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS | - |
| Highest NRB_agg (NOTE 4) | Highest NRB_agg (NOTE 4) | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS | - |
| Note 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. Note 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. Note 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested Note 4: In CA_n66B configuration with the same NRB_agg CC combination, PCC shall be selected as the lower CH BW Note 5: In a band where UE supports 4Rx, the test needs to be performed only with 4Rx antennas connected. |  |  |  |  |  |  |

Table 7.3A.1.4.1-2: Test Configuration Table for inter-band 2DL CA without exception

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Mid range for PCC and SCC with exceptions for CA configurations containing the following band combinations (Note 8): CA_n1-n77: Mid in band n1 and Low in band n77 CA_n3-n77: TBD in band 3 and TBD in band 77. CA_n8-nX: Low range for PCC in Band 8 CA_n70-n71: High range for PCC in band 71. CA_n3-n78: Mid in band 3 and High in band 78. CA_n5-n78: Mid in band 5 and High in band 78 CA_n29-n71: Low in band 29 and High in band 71.. |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2,3) |  |  |
|  | CA Configuration |  |  |  | PCC NRB | SCC NRB | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_nXA-nYA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | default | nY | default | Highest (Note 6) | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | default | nX | default | Highest (Note 6) | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A, X=1, Y=3. Note 4: REFSENS refers to the PCC bands and PCC NRB 's single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. Note 5: For band combinations including operating band without uplink band (as noted in Table 5.2-1), only the CA configuration where PCC band has uplink band shall be tested. Note 6: For NR band n70, DL 25 MHz / UL 15 MHz shall be configured (as specified in clause 5.3.6). Note 7: In a band where UE supports 4Rx, the test needs to be performed only with 4Rx antennas connected. Note 8: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. Note 9: CA_n29A-n71A is tested according to reference sensitivity levels specified in Clause 7.3A.1_1.5 due to cross band isolation exception specified in Table 7.3A.0.6-1 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.1.4.1-3: Test Configuration Table for intra-band non-contiguous 2DL CA without exception

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  |  | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  |  | UL Allocation (Note 2,3) |  |  |
|  | CA Configuration |  |  |  |  | PCC | Wgap / [MHz] | SCC |  | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  |  | SCC |  |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_nX(2A) Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | CC1 |  | nX | CC2 | Highest | Max (NOTE 4) | Lowest |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nX | CC1 |  | nX | CC2 | Highest NRB_agg (NOTE 5) | Max (NOTE 4) | Highest NRB_agg (NOTE 5) |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Test Settings for a CA_n71(2A) Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | CC1 |  | n71 | CC2 | 5MHz | 5.0 | 5MHz |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | 20@0 | - |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. Note 4: The Wgap is defined to be widest possible on band based on the PCC and SCC configuration Note 5: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested Note 6: In a band where UE supports 4Rx, the test needs to be performed only with 4Rx antennas connected. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Tables 7.3A.1.4.1-1, 7.3A.1.4.1-2 and 7.3A.1.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.3A.1.4.3.

7.3A.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 7.3A.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1-1 for C_RNTI to transmit the DL RMC according to Tables 7.3A.1.4.1-1, 7.3A.1.4.1-2 and 7.3A.1.4.1-3 on both PCC and SCC.  The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3A.1.4.1-1, 7.3A.1.4.1-2 and 7.3A.1.4.1-3 on PCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level to the appropriate REFSENS value defined in Tables 7.3.2.5-1, 7.3.2.5-2. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the throughput measurement. Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

7.	Measure the average throughput for each component carrier for a duration sufficient to achieve statistical significance according to Annex H.2A.

7.3A.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

7.3A.1.5	Test requirement

For 2DL carrier aggregation, test parameters are specified in table 7.3A.1.4.1-1, 7.3A.1.4.1-2 and 7.3A.1.4.1-3. For the CA configurations listed in table 7.3A.1.5-1, the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with reference power level specified in table 7.3.2.5-1 for non-SDL carrier for 2 Rx antenna port, in table 7.3.2.5-2 for non-SDL carrier for 4 Rx antenna port and in table 7.3A.1.5-2 for SDL carrier with following additional requirements:

The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For the UE which supports inter-band carrier aggregation, the test requirement for reference sensitivity shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.0.3 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

For intra-band non-contiguous 2 DL CA, the test requirement for shall be increased by ΔRIBNC given in Table 7.3A.0.2.2-1 for the SCC. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.1.5-1: Reference sensitivity requirement for 2DL CA

| Carrier aggregation type | DL CA configuration | UL CA configuration |
| --- | --- | --- |
|  | CA_n40B | - |
|  | CA_n41C | - |
| Intra-band contiguous 2DL CA | CA_n66B | - |
|  | CA_n77C | - |
|  | CA_n78B | - |
|  | CA_n78C | - |
|  | CA_n66(2A) | - |
| Intra-band non-contiguous 2DL CA | CA_n77(2A) | - |
|  | CA_n78(2A) | - |
|  | CA_n71(2A) | - |
|  | CA_n1A-n3A |  |
|  | CA_n1A-n77A | - |
|  | CA_n1A-n78A | - |
|  | CA_n2A-n48A | - |
|  | CA_n2A-n66A | - |
|  | CA_n2A-n77A | - |
|  | CA_n3A-n5A |  |
|  | CA_n3A-n41A |  |
|  | CA_n3A-n77A | - |
| Inter-band 2DL CA | CA_n3A-n78A | - |
|  | CA_n5A-n66A | - |
|  | CA_n5A-n77A | - |
|  | CA_n5A-n78A |  |
|  | CA_n8A-n78A | - |
|  | CA_n24A-n41A |  |
|  | CA_n24A-n48A |  |
|  | CA_n24A-n77A |  |
|  | CA_n28A-n41A | - |
|  | CA_n28A-n79A | - |
|  | CA_n41A-n79A | - |
|  | CA_n66A-n70A | - |
|  | CA_n66A-n71A | - |
|  | CA_n66A-n77A | - |
|  | CA_n70A-n71A | - |
| SDL configuration | CA_n29A-n66A | - |
|  | CA_n29A-n70A | - |
|  | CA_n29A-n71A (NOTE 1) | - |
| Note 1: CA_n29A-71A reference sensitivity requirement is tested in Clause 7.3A.1_1 due to cross band isolation exception specified in Table 7.3A.0.6-1. Other Rx cases shall be tested with reference sensitivity levels defined in Clause 7.3A.1_1.5. |  |  |

Table 7.3A.1.5-2: Reference sensitivity for SDL bands

| NR band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n29 | 15 | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |  |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.2.4.1-1, Table 7.3.2.4.1-2, Table 7.3.2.4.1-3, Table 7.3.2.5-1, Table 7.3.2.5-2 and Table 7.3A.1.4-1   with the reference sensitivity power level increased by ΔRIBNC  given in Table 7.3A.0.2.2-1 for the SCC(s). For aggregation of two downlink FDD carriers with one uplink carrier the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.0.2.2-1. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

For band combinations including operating bands without uplink band (as noted in Table 5.2-1), the requirements are specified in Table 7.3A.1.5-1 and for any band with uplink the uplink configuration specified in Table 7.3.2.4.1-2. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels, as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one-sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal, as described in Annex A.5.1.1/A.5.2.1). The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

### 7.3A.1_1	Reference sensitivity power level for 2DL CA exceptions

Editor’s Note: The following aspects are either missing or not yet determined:

- Test point analysis for CA_n3A-n5A IMD2 and IMD4 is currently missing in TR 38.905.

7.3A.1_1.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when CA exceptions are allowed.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.1_1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support NR 2DL CA

7.3A.1_1.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.1_1.4	Test description

7.3A.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.1_1.4.1-1: Test Configuration Table for inter-band 2DL CA exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | 1950 MHz (UL) | n3 | 1760 MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |  |
|  | n1 | Low | n3 | High | 5MHz | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 |  |  |
| Test Settings for CA_n1A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | 1965 MHz (UL) | n8 | 887,5 MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |  |
| Test Settings for CA_n1A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | Mid | n77 | 3900 MHz | 20 MHz | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 |  | - |
| 2 | n1 | Mid | n77 | 3870 MHz | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 |  | - |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / CBW |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  | UL Allocation (Note 2) |  |  |  |  |  |
|  | CA Configuration |  |  |  |  |  |  |  | PCC |  | SCC |  | CC MOD |  | PCC & SCC RB allocation |  |  | CC MOD |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
|  | PCC |  |  |  | SCC |  |  |  |  |  |  |  |  |  | PCC |  | SCC |  |  |  |  |  |  |
|  | Band |  | Range |  | Band |  | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 |  | 1950 MHz (UL) |  | n78 |  | 3710 MHz |  | 5 MHz |  | 10 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_3 |  | REFSENS_CA_3 |  |
| Test Settings for CA_n2A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 |  | 1860 MHz (UL) |  | n48 |  | 3700 MHz |  | 20 MHz |  | 20 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_1 |  | - |  |
| 2 | n2 |  | UL 1852.5/DL 1932.5 |  | n48 |  | 3625 MHz |  | 5 MHz |  | 20 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_3 |  | REFSENS_CA_3 |  |
| Test Settings for CA_n2A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 |  | UL 1855/DL 1935 |  | n66 |  | UL 1775/DL 2175 |  | 5 MHz |  | 5 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_3 |  | REFSENS_CA_3 |  |
| 2 | n2 |  | UL 1883.3/DL 1963.3 |  | n66 |  | UL 1750/DL 2150 |  | 5 MHz |  | 5 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_3 |  | REFSENS_CA_3 |  |
| Test Settings for CA_n2A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 |  | 1860 MHz (UL) |  | n77 |  | 3720 MHz |  | 20 MHz |  | 100 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_1 |  | - |  |
| 2 | n2 |  | 1860 MHz (UL) |  | n77 |  | 3700 MHz |  | 20 MHz |  | 20 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_1 |  | - |  |
| 3 | n2 |  | DL Mid |  | n77 |  | 3920 MHz |  | 20 MHz |  | 100 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | - |  | REFSENS_CA_2 |  |
| 4 | n2 |  | UL 1855/DL 1935 |  | n77 |  | 3790 MHz |  | 5 MHz |  | 10 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_3 |  | REFSENS_CA_3 |  |
| 5 | n2 |  | UL 1900/DL 1980 |  | n77 |  | 3720 MHz |  | 5 MHz |  | 10 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_3 |  | REFSENS_CA_3 |  |
| 6 | n2 |  | UL 1885/DL 1965 |  | n77 |  | 3810 MHz |  | 5 MHz |  | 10 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_3 |  | REFSENS_CA_3 |  |
| Test Settings for CA_n3A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1; n3; TBD; n77; TBD; Highest; Highest; CP-OFDM QPSK; Full RB; DFT-s-OFDM QPSK; REFSENS_CA_1; -. |
| 2; n3; 1721 MHz (UL); n5; 838 MHz; 10 MHz; 5 MHz; CP-OFDM QPSK; Full RB; DFT-s-OFDM QPSK; REFSENS_CA_3; REFSENS_CA_3. |
| Test Settings for CA_n3A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 |  | TBD |  | n77 |  | TBD |  | Highest |  | Highest |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_1 |  | - |  |
| 2 | n3 |  | TBD |  | n77 |  | TBD |  | Highest |  | 20 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_1 |  | - |  |
| Test Settings for a CA_n5A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 |  | UL 838/DL 883 |  | n66 |  | UL 1721/DL 2121 |  | 5 MHz |  | 5 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_3 |  | REFSENS_CA_3 |  |
| Test Settings for a CA_n5A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 |  | 834 MHz (UL) |  | n77 |  | 3336 MHz |  | 20 MHz |  | 100 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_1 |  | - |  |
| 2 | n5 |  | 834 MHz (UL) |  | n77 |  | 4170 MHz |  | 20 MHz |  | 20 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_1 |  | - |  |
| 3 | n5 |  | DL Mid |  | n77 |  | 3526 MHz |  | 10 MHz |  | 100 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | - |  | REFSENS_CA_2 |  |
| 4 | n5 |  | UL 844/ DL 889 |  | n77 |  | 3421 MHz |  | 5 MHz |  | 10 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_3 |  | REFSENS_CA_3 |  |
| 5 | n5 |  | UL 829/ DL 874 |  | n77 |  | 4190 MHz |  | 5 MHz |  | 10 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_3 |  | REFSENS_CA_3 |  |
| Test Settings for CA_n70A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 |  | Low |  | n70 |  | Low |  | 10 MHz |  | Highest |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_1 with RB start 10 |  | - |  |
| 2 | n71 |  | Low |  | n70 |  | Low |  | 5 MHz |  | 5 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_1 with RB start 10 |  | - |  |
| 3 |  | n70 |  | 1697.5 MHz (UL) |  | n71 |  | 695.5 MHz (UL) |  | 5 MHz |  | 5 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_3 |  | REFSENS_CA_3 |  |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | Mid | n78 | 3495 MHz | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n3 | Mid | n78 | 3465 MHz | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n3 | 1740 MHz (UL) | n78 | 3575 ;MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 4 | n3 | 1765 MHz (UL) | n78 | 3435 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n5A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | Mid | n78 | 3346 MHz | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n5 | Mid | n78 | 3316 MHz | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| Test Settings for CA_n7A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n7 | High | n78 | Low | 50 MHz | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | Mid | n78 | 3590 MHz | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n8 | 897.5 MHz (UL) | n78 | 3635 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n26A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | 1721 MHz (UL) | n26 | 838 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n29A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | High | n29 | Low | Highest | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 |  |
| 2 | n71 | High | n29 | Low | Highest | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 |  |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Test Settings for CA_n26A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n70 | 1707.5 MHz (UL) | n26 | 838 MHz (UL) | 5 MHz UL / 25 MHz DL | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n48A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | 3660 MHz (UL) | n66 | 1750 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n66 | High | n48 | Low | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n66 | High | n48 | Low | 5 MHz | 60 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n66 | High | n48 | Mid | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| Test Settings for CA_n48A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | 3695 MHz (UL) | n70 | 1697.5 MHz (UL) | 10 MHz | 15 MHz UL / 25 MHz DL | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n66A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | n66 | 1750 MHz (UL) | n71 | 675 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n66A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | 1720 MHz (UL) | n77 | 3440 MHz | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n66 | 1712.5 MHz (UL) | n77 | 3400 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n66 | UL 1775/ DL 2175 | n77 | 3950 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 4 | n66 | UL 1760/ DL 2160 | n77 | 3720 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. REFSENS_CA_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3A.0.4-2. REFSENS_CA_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3A.0.4-4a. REFSENS_CA_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to intermodulation interference due to 2UL CA according to table 7.3A.0.5-1 for PC3 and table 7.3A.0.5-1a for PC2. REFSENS_CA_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation for NR CA FR1 according to table 7.3A.0.6-2. Note 3: In a band where UE supports 4Rx, the test needs to be performed only with 4Rx antennas connected. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.1_1.4.1-2: Test Configuration Table for intra-band non-contiguous 2DL CA exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  |  | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  |  | UL Allocation (Note 2,3) |  |  |
|  | CA Configuration |  |  |  |  | PCC | Wgap / [MHz] | SCC |  | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  |  | SCC |  |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Test Settings for a CA_n71(2A) Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | CC1 |  | n71 | CC2 | 5MHz | 25.0 | 5MHz |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | 5@0 | - |
| 2 | n71 | CC1 |  | n71 | CC2 | 15MHz | 10.0 | 10MHz |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | 5@2 | - |
| 3 | n71 | CC1 |  | n71 | CC2 | 15MHz | 5.0 | 10MHz |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | 20@19 | - |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: REFSENS_CA_1 refers to the Uplink RB allocation for reference sensitivity exceptions according to table 7.3A.0.2.2-1 Note 4: The Wgap is defined to be widest possible on band based on the PCC and SCC configuration Note 5: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested Note 6: In a band where UE supports 4Rx, the test needs to be performed only with 4Rx antennas connected. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Tables 7.3A.1_1.4.1-1 and 7.3A.1_1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.3A.1_1.4.3.

7.3A.1_1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1.  Message contents are defined in clause 7.3A.1_1.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 7.3A.1_1.4.1-1 and 7.3A.1_1.4.1-2. on both PCC and SCC.  The SS sends downlink MAC padding bits on the DL RMC

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.1.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3A.1_1.5-1 and 7.3A.1_1.5-2 for PC3 CA, and in Table 7.3A.1_1.5-1a for PC2 CA. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the throughput measurement.  Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

7.	Measure the average throughput for each component carrier for a duration sufficient to achieve statistical significance according to Annex H.2A.

7.3A.1_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED and following exception:

For test points with “REFSENS_CA_3” UL configuration in table 7.3A.1_1.4.1-1, message exception in table 7.3A.1_1.4.3-1 applies.

Table 7.3A.1_1.4.3-1: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band 2UL CA |
|  | 23 |  | Power class 2 and Inter-band 2UL CA |

7.3A.1_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1 for PC3, and in Table 7.3A.1_1.5-1a for PC2. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 7.3A.1_1.5-1: Reference sensitivity requirement for inter band PC3 CA

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n1A-n3A | all | n1 | 15 | -100+23+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100 -2.7 +23+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n3 | 15 | -97+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | --97-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n3 | 15 |  |  |  |  |  |  | -82.3+1.4 +TT |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | -82.3-2.7+1.4+TT4 |  |  |  |  |  |  |
| CA_n1A-n8A | 1 | n1 | 15 | -100+6+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100 -2.7 +6+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n8 | 15 | -97+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -97-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n77A | all | n1 | 15 |  |  |  | -93.8 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -93.8 -2.7+TT4 |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.1 +13.8+TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -85.1 -2.2+13.8+TT4 |
|  | 2 | n77 | 15 |  |  |  | -92.2 +0.3+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -92.2 -2.2+0.3+TT4 |  |  |  |  |  |  |  |  |  |
| CA_n1A-n78A | 1 | n1 | 15 | -100 +8+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100 -2.7 +10.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  | -95.8 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -95.8 -2.2 +TT4 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n48A | 1 | n48 | 15 |  |  |  | -92.7 +0.3+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -94.9 +0.3+TT4 |  |  |  |  |  |  |  |  |  |
|  | 2 | n2 | 15 | -98.0 +12+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.7 +12+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n66A | 1 | n2 | 15 | -98.0 +20+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.7 +20+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 | -99.5 +4+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -102.2 +4+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n77A | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -85.1 +13.8+TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -87.3 +13.8+TT4 |
|  | 2 |  |  |  |  |  | -92.2 +0.3+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -94.4 +0.3+TT4 |  |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  | -91.8 +3.7+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -94.5+ 3.7+TT4 |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  | -98.0 +26+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.7 +28.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  | -98.0 +8+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.7 +10.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  |  | -98.0 +5+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.7 +5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n3A-n77A | All | n3 | 15 |  |  |  |  |  | -88.9 +TT |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | -88.9 -2.7 +TT4 |  |  |  |  |  |  |  |
|  | 1 | n77 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.1 +13.8 +TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -85.1 -2.2 +13.8 +TT4 |
|  | 2 | n77 | 15 |  | -95.3 +0.3 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -95.3 -2.2 +0.3 +TT4 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n66A | 1 | n5 | 15 | -98.0 +30+TT |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n77A | 1 | n77 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.1 +13.8+TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -87.3 +13.8+TT4 |
|  | 2 | n77 | 15 |  |  |  | -92.2 +0.3+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -94.4 +0.3+TT4 |  |  |  |  |  |  |  |  |  |
|  | 3 | n5 | 15 |  | -94.8 +4.0+TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | n5 | 15 | -98.0 +8.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 | n5 | 15 | -98.0 +5.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n70A-n71A | 1 | n70 | 15 |  |  |  |  | -92.7 +4.1 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -92.7 -2.7 +4.1 +TT4 |  |  |  |  |  |  |  |  |
|  | 2 | n70 | 15 | -100.0 +9.9 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.0 -2.7 +9.9 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n70 | 15 | -100.0 +5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.0 -2.7 +5 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | All | n71 | 15 | -97.2 +TT | -94.0 +TT |  |  |  |  |  |  |  |  |  |  |  |

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n3A-n78A | 1, 2 | n3 | 15 |  |  |  | -90.8 +TT |  | -88.9 +TT |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -90.8 -2.7 +TT4 |  | -88.9 -2.7+ TT4 |  |  |  |  |  |  |  |
|  | 1 | n78 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 +13.8 +TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 -2.2 +13.8 +TT4 |
|  | 2 | n78 | 15 |  |  |  | -92.7 +0.3 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -92.7 -2.2 +0.3 +TT4 |  |  |  |  |  |  |  |  |  |
|  | 3 | n3 | 15 | -97.0 +[26] +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.0 -2.7 +[28.7] +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | n3 | 15 | -97.0 +[8] +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.0 -2.7 +[10.7] +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3, 4 | n78 | 15 |  | -95.8 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -95.8 -2.2 +TT4 |  |  |  |  |  |  |  |  |  |  |  |

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n5A-n78A | 1, 2 | n5 | 15 |  |  |  | -86.8 +TT |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 +1.4 +TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 - 2.2 +1.4 +TT4 |
|  | 2 | n78 | 15 |  |  |  | -92.7 +7.8 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -92.7 -2.2 +7.8 +TT4 |  |  |  |  |  |  |  |  |  |
| CA_n7A-n78A | 1 | n7 | 30 |  |  |  |  |  |  |  | -81.5 +4.5 +TT |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | -81.5 -2.7+4.5 +TT4 |  |  |  |  |  |
|  | 1 | n78 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 +1.4 +TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 - 2.2 +1.4 +TT4 |
| CA_n8A-n78A | 1 | n8 | 15 |  |  |  | -85.8 + TT |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 +1.4 +TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 -2.2 +1.4 +TT4 |
|  | 2 | n8 | 15 | -97.0 + 8.3 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n78 | 15 |  | -95.8 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -95.8 -2.2 +TT4 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n26A-n66A | 1 | n26 | 15 | -97.55 + 30 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n26A-n70A | 1 | n26 | 15 | -97.55 + 30 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n70 | 15 |  |  |  |  | -92.7 + TT |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -92.7 - 2.7 +TT4 |  |  |  |  |  |  |  |  |
| CA_n29A-n71A | 1 | n29 | 15 | -97.0 + 17.5 + TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 |  |  |  | -86.0 + TT |  |  |  |  |  |  |  |  |  |
|  | 2 | n29 | 15 |  | -93.8 + 16.0 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n71 | 15 |  |  |  | -86.0 + TT |  |  |  |  |  |  |  |  |  |

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n48A-n66A | 1 | n48 | 15 | -99.0 + TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5 + 5.0 + TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -99.5 +5.0 -2.7 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n48 | 15 |  | -95.8 + 23.9 + TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 | -99.5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -99.5 -2.7 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n48 | 30 |  |  |  |  |  |  |  |  | -96.1 + 8.3 +TT |  |  |  |  |
|  | 3 | n66 | 15 | -99.5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -99.5 -2.7 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | n48 | 15 |  | -95.8 + 1.1. + TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | n66 | 15 | -99.5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -99.5 -2.7 +TT |  |  |  |  |  |  |  |  |  |  |  |  |

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n48A-n70A | 1 | n48 | 15 |  | -95.8 + TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n70 | 15 |  |  |  |  | -92.7 +26 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -92.7 - 2.7 +28.4 +TT4 |  |  |  |  |  |  |  |  |
| CA_n66A-n71A | 1 | n66 | 15 | -99.5 +5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -99.5 -2.7 +5 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 | -97.2 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n77A | 1 | n66 | 15 |  |  |  | -93.3+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -96.0+TT4 |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.1 +13.8+TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -87.3 +13.8+TT4 |
|  | 2 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  | -95.3 +1.1+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5 +1.1+TT4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n66 | 15 | -99.5 +31 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -102.2 +31 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n77 | 15 |  | -95.3 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | n66 | 15 | -99.5 +5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -102.2 +5 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | n77 | 15 |  | -95.3 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 5: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of changes>