3GPP TSG-RAN5 Meeting #97	R5-226477

# Toulouse, France, 14 – 18 Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2665 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inclusive Language Review of LTE RRM Section 4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2022-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Inclusive language review and update of 3GPP LTE RRM spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replaced ‘BlackCellList’ with ‘ExcludedCellList’ in Idle mode test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-Inclusive language remain in the spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.4.3, 4.3.2.1.4.3, 4.3.3.1.4.3, 4.3.4.1.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.3	E-UTRAN to UTRAN Cell Re-Selection

### 4.3.1	E-UTRAN FDD - UTRAN FDD cell re-selection

#### 4.3.1.1	E-UTRA FDD - UTRAN FDD cell reselection: UTRA FDD is of higher priority

##### 4.3.1.1.1	Test purpose

To verify that the UE is able to search and measure neighbouring UTRA FDD cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements when the UTRA is of higher priority.

##### 4.3.1.1.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 8 and forward that support UTRA FDD.

##### 4.3.1.1.3	Minimum conformance requirements

When the measurement rules indicate that UTRA FDD cells are to be measured, the UE shall measure CPICH Ec/Io and CPICH RSCP of detected UTRA FDD cells in the neighbour frequency list at the minimum measurement rate. The parameter NUTRA_carrier is the number of carriers used in the neighbour frequency list. The UE shall filter CPICH Ec/Io and CPICH RSCP measurements of each measured UTRA FDD cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

If the SServingCell of the E-UTRA serving cell is greater than Snonintrasearch then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in 3GPP TS 36.133 [4] clause 4.2.2 as Thigher_priority_search = (60 * Nlayers) seconds, where the parameter Nlayers is the total number of configured higher priority carrier frequencies.

If the SServingCell of the E-UTRA serving cell is less than or equal to Snonintrasearch then the UE shall search for and measure inter-RAT layers of higher or lower priority in preparation for possible re-selection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

The UE shall evaluate whether newly detectable UTRA FDD cells have met the re-selection criteria in 3GPP TS 36.304 [6] within time NUTRA_carrier * TdetectUTRA_FDD (as defined in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1) when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch when TreselectionRAT = 0 provided that the re-selection criteria is met by a margin of at least 6 dB.

Cells which have been detected shall be measured at least every NUTRA_carrier * TmeasureUTRA_FDD when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch.

When higher priority UTRA FDD cells are found by the higher priority search, they shall be measured at least every TmeasureUTRA_FDD. If re-selection to any higher priority cell is not triggered within (TevaluateUTRA_FDD + TreselectionRAT) after it is found in a higher priority search, the UE is not required to continue making measurements of the cell to evaluate the ongoing possibility of re-selection.

For a cell that has been already detected, but that has not been re-selected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA FDD cell has met re-selection criterion defined in 3GPP TS 36.304 [6] within NUTRA_carrier * TevaluateUTRA_FDD when TreselectionRAT = 0 as specified in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1 provided that the reselection criteria is met by a margin of at least 6dB.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.1 and A.4.3.1.1.

##### 4.3.1.1.4	Test description

4.3.1.1.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.22 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.3.1.1.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.1.1.4.3.

5.	There is one E-UTRA FDD cell and one UTRA FDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.1.1.4.1-1: General test parameters for E-UTRA FDD- higher priority UTRA FDD inter RAT cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell 1 | UE is on cell 1 in the initialisation phase, so that reselection to cell 2 occurs during T2 |
| T2 end condition | Active cell |  | Cell 2 | UE shall perform reselection to cell 2 during T2 |
|  | Neighbour cell |  | Cell 1 |  |
| T3 end condition | Active cell |  | Cell 1 | UE shall perform reselection to cell 1 during T3 |
|  | Neighbour cell |  | Cell 2 |  |
| E-UTRA PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in 3GPP TS 36.211 [9] |
| E_UTRA Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | >20 | During T1, cell 2 shall be powered off, and during the off time the primary scrambling code shall be changed, The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 |  | s | 85 | T2 needs to be defined so that cell re-selection reaction time is taken into account |
| T3 |  | s | 25 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

4.3.1.1.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. E-UTRA cell 1 is already identified by the UE prior to the start of the test. Before T1 the UE is camped on to cell 1. During T1, cell 2 shall be powered off, and during the off time the scrambling code shall be changed. At the start of T2 cell 2 becomes stronger than Threshx_high, the UE is expected to detect cell 2, send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Routing Area Update procedure on cell 2. At the start of T3 cell 2 becomes weaker than Threshserving_low, and the UE reselects to Cell 1.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Routing Area Update procedure".

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to duration T1 in Table 4.3.1.1.5-1 and 4.3.1.1.5-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

3.	During T1, Cell 2 shall be powered off and the SS shall set Cell 2 primary scrambling code = ((current cell 2 primary scrambling code - 50) mod 200 + 100).

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 4.3.1.1.5-1 and 4.3.1.1.5-2. The SS waits for random access requests information from the UE to perform cell re-selection procedure on the higher priority cell, Cell 2.

5.	If the UE responds on Cell 2 within 81s from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

6.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 7.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, skip to step 10.

7.	The SS shall switch the power setting from T2 to T3 as specified in Table 4.3.1.1.5-1 and 4.3.1.1.5-2.

8.	The SS waits for random access requests information from the UE to perform cell re-selection procedure on the lower priority cell, Cell 1.

9.	If the UE has re-selected Cell 1 within T3, after the re-selection or when T3 expires, skip to step 11.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, continue with step 10.

10.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

11.	Repeat step 2-10 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

4.3.1.1.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 and 3GPP TS 34.108 [24] clause 6.1.0b with the following exceptions:

Table 4.3.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-1 Table H.2.3-2 Table H.2.3-13 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-1 |

Table 4.3.1.1.4.3-2: System Information Block type 19:
Inter-RAT E-UTRAN FDD - UTRA FDD is of higher priority cell re-selection

| Derivation Path: 36.508 [7] clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| s-PrioritySearch1 | 31 (62dB) |  |  |
| s-PrioritySearch2 | 0 | default value is 0 |  |
| threshServingLow | 18 (36dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 2 (4 dB) |  |  |
| threshXlow | 25 (50 dB) |  |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

##### 4.3.1.1.5	Test requirement

Tables 4.3.1.1.4-1, 4.3.1.1.5-1 and 4.3.1.1.5-2 define the primary level settings including test tolerances for inter-RAT cell re-selection E-UTRA FDD to UTRA FDD test case (UTRA is of higher priority).

Table 4.3.1.1.5-1: Cell specific Test Parameters for Cell 1(E-UTRA FDD)

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in D.1.2 (OP.2 FDD) |  | OP.2 FDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qqualmin for UTRA neighbour cell | dB | -20 |  |  |
| Qrxlevmin for UTRA neighbour cell | dBm | -115 |  |  |
| Qrxlevmin | dBm | -140 |  |  |
|  | dBm/15 kHz | -98 |  |  |
| RSRP | dBm/15 KHz | -82.95 | -82.95 | -82.954 + TT1.3, is to be specified per individual test case |
|  | dB | 15.05 | 15.05 | 15.05 |
|  | dB | 15.05 | 15.05 | 15.05 |
| TreselectionEUTRAN | s | 0 |  |  |
| Snonintrasearch | dB | 50 |  |  |
| Threshx, high (Note 2) | dB | 40 |  |  |
| Propagation Condition |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |  |  |

Table 4.3.1.1.5-2: Cell specific test parameters for Cell 2 (higher priority UTRA FDD)

| Parameter | Unit | Cell 2 (UTRA) |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| UTRA RF Channel Number |  | Channel 2 |  |  |
| CPICH_Ec/Ior | dB | -10 |  |  |
| PCCPCH_Ec/Ior | dB | -12 |  |  |
| SCH_Ec/Ior | dB | -12 |  |  |
| PICH_Ec/Ior | dB | -15 |  |  |
| OCNS_Ec/Ior | dB | -0.941 |  |  |
|  | dB | -∞ | 11.90 | -5.70 |
|  | dBm/3,84 MHz | -70.10 |  |  |
| CPICH_Ec/Io | dB | -∞ | -10.27 | -16.74 |
| CPICH_RSCP | dBm | -∞ | -68.20 | -85.80 |
| Propagation Condition |  | AWGN |  |  |
| Qqualmin | dB | -20 |  |  |
| Qrxlevmin | dBm | -115 |  |  |
| QrxlevminEUTRA | dBm | -140 |  |  |
| UE_TXPWR_MAX_RACH | dBm | 21 |  |  |
| Treselection | s | 0 |  |  |
| Sprioritysearch1 | dB | 62 |  |  |
| Sprioritysearch2 | dB | 0 |  |  |
| Threshserving, low | dB | 36 |  |  |
| Threshx, low (Note) | dB | 50 |  |  |
| NOTE: This refers to the value of Threshx, low which is included in UTRA system information, and is a threshold for the E-UTRA target cell. |  |  |  |  |

The cell reselection delay to higher priority is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message on cell 2.

The cell re-selection delay to higher priority shall be less than 81 s.

NOTE:	The cell re-selection delay to higher priority cell can be expressed as: Thigher_priority_search +TevaluateUTRA_FDD + TSI-UTRA

Where:

Thigher_priority_search 	See section 4.2.2 of 3GPP TS 36.133 [4]; 60s is assumed in this test case

TevaluateUTRA-FDD	See Table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1

TSI-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 80.48 s for higher priority cell search, allow 81 s for higher priority cell reselection in the test case.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 4.3.1.2	E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority

##### 4.3.1.2.1	Test purpose

To verify that the UE is able to search and measure neighbouring UTRA FDD cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements when the UTRA is of lower priority.

##### 4.3.1.2.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 8 and forward that support UTRA FDD.

##### 4.3.1.2.3	Minimum conformance requirements

When the measurement rules indicate that UTRA FDD cells are to be measured, the UE shall measure CPICH Ec/Io and CPICH RSCP of detected UTRA FDD cells in the neighbour frequency list at the minimum measurement rate. The parameter NUTRA_carrier_FDD is the number of carriers used in the neighbour frequency list. The UE shall filter CPICH Ec/Io and CPICH RSCP measurements of each measured UTRA FDD cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

If the SServingCell of the E-UTRA serving cell is greater than Snonintrasearchthen the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in 3GPP TS 36.133 [4] clause 4.2.2 as Thigher_priority_search = (60 * Nlayers) seconds, where the parameter Nlayers is the total number of configured higher priority carrier frequencies.

If the SServingCell of the E-UTRA serving cell is less than or equal to Snonintrasearch then the UE shall search for and measure inter-RAT layers of higher or lower priority in preparation for possible re-selection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

The UE shall evaluate whether newly detectable UTRA FDD cells have met the re-selection criteria in 3GPP TS 36.304 [6] within time NUTRA_carrier_FDD * TdetectUTRA_FDD (as defined in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1) when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch when TreselectionRAT = 0 provided that the re-selection criteria is met by a margin of at least 6 dB.

Cells which have been detected shall be measured at least every NUTRA_carrier * TmeasureUTRA_FDD when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch.

When higher priority UTRA FDD cells are found by the higher priority search, they shall be measured at least every TmeasureUTRA_FDD. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of re-selection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell.

For a cell that has been already detected, but that has not been re-selected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA FDD cell has met re-selection criterion defined in 3GPP TS 36.304 [6] within NUTRA_carrier * TevaluateUTRA_FDD when TreselectionRAT = 0 as specified in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1 provided that the reselection criteria is met by a margin of at least 6dB.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.1 and A.4.3.1.2.

##### 4.3.1.2.4	Test description

4.3.1.2.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.22 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands..

2.	The general test parameter settings are set up according to Table 4.3.1.2.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.1.2.4.3.

5.	There is one E-UTRA FDD cell and one UTRA FDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.1.2.4.1-1: General test parameters for EUTRA FDD -
lower priority UTRA FDD inter RAT cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 | E-UTRAN cell |
| T1 end condition | Active cells |  | Cell1 | UE shall perform reselection to cell 1 during T1 for subsequent iterations of the test |
|  | Neighbour cell |  | Cell2 |  |
| T2 end condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T2 |
|  | Neighbour cell |  | Cell1 |  |
| E-UTRA PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in 3GPP TS 36.211 [9] |
| E_UTRA Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | 85 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 25 | T2 need to be defined so that cell re-selection reaction time is taken into account. |

4.3.1.2.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one UTRA FDD carrier. In the test there are two successive time periods, with time duration of T1 and T2 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. At T1 the UE is camped on to Cell 1. Cell 2 is of lower priority than Cell 1. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Routing Area Update procedure".

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to T1 in Table 4.3.1.2.5-1 and 4.3.1.2.5-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts. If the UE is already camped in Cell 1, wait until T1expires and skip to step 5.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

4.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 5.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, skip to step 9.

5.	The SS shall switch the power setting from T1 to T2 as specified in Table 4.3.1.2.5-1 and 4.3.1.2.5-2.

6.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

7.	If the UE responds on Cell 2 during time duration T2 within 21 seconds from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, skip to step 10.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, continue with step 9.

9.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

10.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

4.3.1.2.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 and 3GPP TS 34.108 [24] clause 6.1.0b with the following exceptions:

Table 4.3.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-5 Table H.2.3-6 Table H.2.3-13 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-1 |

Table 4.3.1.2.4.3-2: System Information Block type 19: Inter-RAT E-UTRAN FDD - UTRA FDD is of lower priority cell re-selection

| Derivation Path: 36.508 [7] clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 21 (42dB) |  |  |
| s-PrioritySearch2 | 0 | default value is 0 |  |
| threshServingLow | 0 (0dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 24 (48 dB) |  |  |
| threshXlow | 1 (2 dB) | Default value |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

##### 4.3.1.2.5	Test requirement

Tables 4.3.1.2.4.1-1, 4.3.1.2.5-1 and 4.3.1.2.5-2 define the primary level settings including test tolerances for E-UTRAN FDD- UTRAN FDD inter-RAT cell re-selection test case which UTRA is of lower priority.

Table 4.3.1.2.5-1: Cell specific test parameters for cell 1 (E-UTRA)

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in D.1.2 (OP.2 FDD) |  | OP.2 FDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qqualmin for UTRA neighbour cell | dB | -20 |  |  |
| Qrxlevmin for UTRA neighbour cell | dBm | -115 |  |  |
| Qrxlevmin | dBm | -140 |  |  |
|  | dBm/15 kHz | -99.35 |  |  |
| RSRP | dBm/15 KHz | -84.95 | -103.05 |  |
|  | dB | 14.40 | -3.70 |  |
|  | dB | 14.40 | -3.70 |  |
| TreselectionEUTRAN | s | 0 |  |  |
| Snonintrasearch | dB | Not sent |  |  |
| Threshserving, low | dB | 44 |  |  |
| Threshx, low (Note 2) | dB | 42 |  |  |
| Propagation Condition |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |  |  |

Table 4.3.1.2.5-2: Cell specific test parameters for Cell 2(Lower priority UTRA FDD)

| Parameter | Unit | Cell 2 (UTRA) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | Channel 2 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| OCNS_Ec/Ior | dB | -0.941 |  |
|  | dB | 13.80 | 13.80 |
|  | dBm/3,84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -10.18 | -10.18 |
| CPICH_RSCP | dBm | -66.20 | -66.20 |
| Propagation Condition |  | AWGN |  |
| Qqualmin | dB | -20 |  |
| Qrxlevmin | dBm | -115 |  |
| QrxlevminEUTRA | dBm | -140 |  |
| UE_TXPWR_MAX_RACH | dBm | 21 |  |
| Treselection | s | 0 |  |
| Sprioritysearch1 | dB | 42 |  |
| Sprioritysearch2 | dB | 0 |  |
| Threshx, high (Note) | dB | 48 |  |
| NOTE: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell. |  |  |  |

The cell reselection delay to lower priority is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message on cell 2.

The cell re-selection delay to lower priority shall be less than 21 s.

NOTE:	The cell re-selection delay to lower priority cell can be expressed as: TevaluateUTRA_FDD + TSI-UTRA

Where:

TevaluateUTRA-FDD	See Table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1

TSI-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 20.48 s for lower priority cell reselection, allow 21 s.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 4.3.1.3	E-UTRAN FDD - UTRAN FDD cell re-selection in fading propagation conditions: UTRA FDD is of lower priority

##### 4.3.1.3.1	Test purpose

To verify that the UE is able to search and measure neighbouring UTRA FDD cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements when the UTRA is of lower priority. The E-UTRA cell is in fading propagation conditions and the UTRA cell is in AWGN propagation conditions.

##### 4.3.1.3.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 8 and forward that support UTRA FDD.

##### 4.3.1.3.3	Minimum conformance requirements

When the measurement rules indicate that UTRA FDD cells are to be measured, the UE shall measure CPICH Ec/Io and CPICH RSCP of detected UTRA FDD cells in the neighbour frequency list at the minimum measurement rate. The parameter NUTRA_carrier_FDD is the number of carriers used in the neighbour frequency list. The UE shall filter CPICH Ec/Io and CPICH RSCP measurements of each measured UTRA FDD cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

If the SServingCell of the E-UTRA serving cell is greater than Snonintrasearch then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in 3GPP TS 36.133 [4] clause 4.2.2 as Thigher_priority_search = (60 * Nlayers) seconds, where the parameters Nlayers is the total number of configured higher priority carrier frequencies.

If the SServingCell of the E-UTRA serving cell is less than or equal to Snonintrasearch then the UE shall search for and measure inter-RAT layers of higher or lower priority in preparation for possible re-selection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

The UE shall evaluate whether newly detectable UTRA FDD cells have met the re-selection criteria in 3GPP TS 36.304 [6] within time NUTRA_carrier_FDD * TdetectUTRA_FDD (as defined in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1) when the SServingCell of the E-UTRA serving cell is greater than Snonintrasearch when TreselectionRAT = 0 provided that the re-selection criteria is met by a margin of at least 6 dB.

Cells which have been detected shall be measured at least every NUTRA_carrier * TmeasureUTRA_FDD when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch.

When higher priority UTRA FDD cells are found by the higher priority search, they shall be measured at least every TmeasureUTRA_FDD. If, after it detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of re-selection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell.

For a cell that has been already detected, but that has not been re-selected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA FDD cell has met re-selection criterion defined in 3GPP TS 36.304 [6] within NUTRA_carrier * TevaluateUTRA_FDD when TreselectionRAT = 0 as specified in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1 provided that the reselection criteria is met by a margin of at least 6dB.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.1 and A.4.3.1.3.

##### 4.3.1.3.4	Test description

4.3.1.3.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and faders and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A figure A.26 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.3.1.3.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.1.3.4.3.

5.	There is one E-UTRA FDD cell and one UTRA FDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.1.3.4.1-1: General test parameters for EUTRA FDD- 
lower priority UTRA FDD inter RAT cell re-selection test case in fading conditions

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 | E-UTRAN Cell. |
| T1 end condition | Active cells |  | Cell1 | UE shall perform reselection to cell 1 during T1 for subsequent iterations of the test. |
|  | Neighbour cell |  | Cell2 |  |
| T3 end condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T3 |
|  | Neighbour cell |  | Cell1 |  |
| E-UTRA PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in 3GPP TS 36.211 [9] |
| E_UTRA Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | <85 | T1 need to be defined so that cell re-selection reaction time is taken into account. T1 is terminated when the UE starts to send preambles to cell 1 |
| T2 |  | s | 64 | The start of T2 is defined as the time when the UE starts to send PRACH preambles to cell 1 |
| T3 |  | s | <25 | T3 need to be defined so that cell re-selection reaction time is taken into account. T3 is terminated when the UE starts to send preambles to cell 2 |
| T4 |  | s | 64 | The start of T4 is defined as the time when the UE starts to send PRACH preambles to cell 2 |

4.3.1.3.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one UTRA FDD carrier. In the test there are four successive time periods, with time duration of T1, T2, T3 and T4 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. At T1 the UE is camped on to Cell 1. Cell 2 is of lower priority than Cell 1. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2. Time duration T2 and T4 are not used for cell re-selection in the test.

In the following test procedure, "UE responds on Cell 1" means "UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure according to 3GPP TS 36.508 [7] clause 4.5A.2" and "UE responds on Cell 2" means "UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Routing Area Update procedure".

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to duration T0 in Table 4.3.1.3.5-1 and 4.3.1.3.5-2.

3.	Set the parameters according to T2 in Table 4.3.1.3.5-3 and 4.3.1.3.5-4. Propagation conditions are set according to Annex B clauses B.1.1 and B.2.2. T2 starts.

4.	The SS monitors for possible random access request information from the UE to perform cell re-selection on Cell 2.

5.	If the UE responds on Cell 2 within T2, then count a fail for the event "Hold out on Cell 1" and skip to step 10.
Otherwise, count a success for the event "Hold out on Cell 1" and after T2 expires continue with step 6.

6.	The SS shall switch the power setting from T2 to T3 as specified in Table 4.3.1.3.5-3 and 4.3.1.3.5-4.

7.	The SS waits for random access request information from the UE to perform cell re-selection on Cell 2.

8.	If the UE responds on Cell 2 during time duration T3 within 21 seconds from the beginning of time period T3, then count a success for the event "Re-select lower priority Cell 2".
Otherwise, count a fail for the event "Re-select lower priority Cell 2".

9.	If the UE responds on Cell 2 within T3, at the moment of the request-reception continue with step 10.
Otherwise, after T3 expires skip to step 17.

10.	The SS shall switch the power setting from T3 to T4 as specified in Table 4.3.1.3.5-3 and 4.3.1.3.5-4.

11.	The SS monitors for possible random access request information from the UE to perform cell re-selection on Cell 1.

12.	If the UE responds on Cell 1 within T4, then count a fail for the event "Hold out on Cell 2" and skip to step 17.
Otherwise, count a success for the event "Hold out on Cell 2" and after T4 expires continue with step 13.

13.	The SS shall switch the power setting from T4 to T1 as specified in Table 4.3.1.3.5-3 and 4.3.1.3.5-4.

14.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

15.	If the UE responds on Cell 1 within T1, at the moment of the request-reception skip to step 17.
Otherwise, after T1 expires continue with step 16.

16.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1. Set the parameters according to duration T0 in Table 4.3.1.3.5-1 and 4.3.1.3.5-2.

17.	Repeat step 3-16 until a test verdict has been achieved.
Each of the events "Hold out on Cell 1", "Re-select lower priority Cell 2" and "Hold out on Cell 2" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

4.3.1.3.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 and 3GPP TS 34.108 [24] clause 6.1.0b with the following exceptions:

Table 4.3.1.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-5 Table H.2.3-6 Table H.2.3-13 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-1 |

Table 4.3.1.3.4.3-2: System Information Block type 19:
Inter-RAT E-UTRAN FDD - UTRA FDD is of lower priority cell re-selection in fading conditions

| Derivation Path: 36.508 [7] clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 21 (42dB) |  |  |
| s-PrioritySearch2 | 0 | default value is 0 |  |
| threshServingLow | 0 (0dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 22 (44 dB) |  |  |
| threshXlow | 1 (2 dB) | Default value |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

##### 4.3.1.3.5	Test requirement

Tables 4.3.1.3.4.1-1, 4.3.1.3.5-1, 4.3.1.3.5-2, 4.3.1.3.5-3 and 4.3.1.3.5-4 define the primary level settings including test tolerances for E-UTRAN FDD- UTRAN FDD inter-RAT cell re-selection test case which UTRA is of lower priority. Note that the time period for T0 is system implementation dependent.

Table 4.3.1.3.5-1: Cell specific test parameters for Cell 1 (Lower priority E-UTRA FDD)

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
|  |  | T0 |
| E-UTRA RF Channel number |  | 1 |
| BWchannel | MHz | 10 |
| Antenna Configuration |  | 1x2 |
| OCNG Patterns defined in D.1.2 (OP.2 FDD) |  | OP.2 FDD |
| PSS_RA | dB | 0 |
| SSS_RA | dB | 0 |
| PCFICH_RB | dB | 0 |
| PHICH_RA | dB | 0 |
| PHICH_RB | dB | 0 |
| PDCCH_RA | dB | 0 |
| PDCCH_RB | dB | 0 |
| PDSCH_RA | dB | 0 |
| PDSCH_RB | dB | 0 |
| OCNG_RANote 1 | dB | 0 |
| OCNG_RBNote 1 | dB | 0 |
| Qqualmin for UTRA neighbour cell | dB | -20 |
| Qrxlevmin for UTRA neighbour cell | dBm | -115 |
| Qrxlevmin | dBm | -140 |
|  | dBm/15 kHz | -104 |
| RSRP | dBm/15 KHz | -82 |
|  | dB | 22 |
|  | dB | 22 |
| TreselectionEUTRAN | s | 0 |
| Snonintrasearch | dB | Not sent |
| Threshserving, low | dB | 44 |
| Threshx, low (Note 2) | dB | 42 |
| Propagation Condition |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |

Table 4.3.1.3.5-2: Cell specific test parameters for Cell 2 (Lower priority UTRA FDD)

| Parameter | Unit | Cell 2 (UTRA) |
| --- | --- | --- |
|  |  | T0 |
| UTRA RF Channel Number |  | Channel 2 |
| CPICH_Ec/Ior | dB | -10 |
| PCCPCH_Ec/Ior | dB | -12 |
| SCH_Ec/Ior | dB | -12 |
| PICH_Ec/Ior | dB | -15 |
| OCNS_Ec/Ior | dB | -0.941 |
|  | dB | 13.80 |
|  | dBm/3,84 MHz | -70 |
| CPICH_Ec/Io | dB | -10.18 |
| CPICH_RSCP | dBm | -66.20 |
| Propagation Condition |  | AWGN |
| Qqualmin | dB | -20 |
| Qrxlevmin | dBm | -115 |
| QrxlevminEUTRA | dBm | -140 |
| UE_TXPWR_MAX_RACH | dBm | 21 |
| Treselection | s | 0 |
| Sprioritysearch1 | dB | 42 |
| Sprioritysearch2 | dB | 0 |
| Threshx, high (Note) | dB | 44 |
| NOTE: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell |  |  |

Table 4.3.1.3.5-3: Cell specific test parameters for cell 1 (E-UTRA)

| Parameter | Unit | Cell 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |  | T3 |  | T4 |  |  |
| E-UTRA RF Channel number |  | 1 |  |  |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  |  |  |  |  |
| OCNG Patterns defined in D.1.2 (OP.2 FDD) |  | OP.2 FDD |  |  |  |  |  |  |
| PSS_RA | dB | 0 |  |  |  |  |  |  |
| SSS_RA | dB | 0 |  |  |  |  |  |  |
| PCFICH_RB | dB | 0 |  |  |  |  |  |  |
| PHICH_RA | dB | 0 |  |  |  |  |  |  |
| PHICH_RB | dB | 0 |  |  |  |  |  |  |
| PDCCH_RA | dB | 0 |  |  |  |  |  |  |
| PDCCH_RB | dB | 0 |  |  |  |  |  |  |
| PDSCH_RA | dB | 0 |  |  |  |  |  |  |
| PDSCH_RB | dB | 0 |  |  |  |  |  |  |
| OCNG_RANote 1 | dB | 0 |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB | 0 |  |  |  |  |  |  |
| Qqualmin for UTRA neighbour cell | dB | -20 |  |  |  |  |  |  |
| Qrxlevmin for UTRA neighbour cell | dBm | -115 |  |  |  |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |  |  |  |  |
|  | dBm/15 kHz | -104 |  |  |  |  |  |  |
| RSRP | dBm/15 KHz | -82 | -82 |  | -107 |  | -107 |  |
|  | dB | 22 | 22 |  | -3 |  | -3 |  |
|  | dB | 22 | 22 |  | -3 |  | -3 |  |
| TreselectionEUTRAN | s | 0 |  |  |  |  |  |  |
| Snonintrasearch | dB | Not sent |  |  |  |  |  |  |
| Threshserving, low | dB | 44 |  |  |  |  |  |  |
| Threshx, low (Note 2) | dB | 42 |  |  |  |  |  |  |
| Propagation Condition |  | ETU70 |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |  |  |  |  |  |  |

Table 4.3.1.3.5-4: Cell specific test parameters for Cell 2 (Lower priority UTRA FDD)

| Parameter | Unit | Cell 2 (UTRA) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 |
| UTRA RF Channel Number |  | Channel 2 |  |  |  |
| CPICH_Ec/Ior | dB | -10 |  |  |  |
| PCCPCH_Ec/Ior | dB | -12 |  |  |  |
| SCH_Ec/Ior | dB | -12 |  |  |  |
| PICH_Ec/Ior | dB | -15 |  |  |  |
| OCNS_Ec/Ior | dB | -0.941 |  |  |  |
|  | dB | 13.80 | 13.80 | 13.80 | 13.80 |
|  | dBm/3,84 MHz | -70 |  |  |  |
| CPICH_Ec/Io | dB | -10.18 | -10.18 | -10.18 | -10.18 |
| CPICH_RSCP | dBm | -66.20 | -66.20 | -66.20 | -66.20 |
| Propagation Condition |  | AWGN |  |  |  |
| Qqualmin | dB | -20 |  |  |  |
| Qrxlevmin | dBm | -115 |  |  |  |
| QrxlevminEUTRA | dBm | -140 |  |  |  |
| UE_TXPWR_MAX_RACH | dBm | 21 |  |  |  |
| Treselection | s | 0 |  |  |  |
| Sprioritysearch1 | dB | 42 |  |  |  |
| Sprioritysearch2 | dB | 0 |  |  |  |
| Threshx, high (Note) | dB | 44 |  |  |  |
| NOTE: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell |  |  |  |  |  |

The probability of re-selection from Cell 1 to Cell 2 during T2 observed during testing shall be less than 10%.

The probability of re-selection from Cell 2 to Cell 1 during T4 observed during testing shall be less than 10%.

The cell reselection delay to lower priority is defined as the time from the beginning of time period T3, to the moment when the UE camps on Cell 2 and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message on Cell 2. In order to evaluate re-selection delay, the SS first needs to verify that the UE is camped on Cell 1 at the start of T3.

The cell re-selection delay to lower priority shall be less than 21 s.

NOTE:	The cell re-selection delay to lower priority cell can be expressed as: TevaluateUTRA_FDD + TSI-UTRA

Where:

TevaluateUTRA-FDD	See Table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1

TSI-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 20.48 s for lower priority cell reselection, allow 21 s.

For the test to pass, all the events above shall pass.

The statistical pass/ fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

#### 4.3.1.4	E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority for 5MHz bandwidth

##### 4.3.1.4.1	Test purpose

Same test purpose as in clause 4.3.1.2.1.

##### 4.3.1.4.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 8 and forward that support only E-UTRA bands within band group FDD_N and UTRA FDD.

##### 4.3.1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 4.3.1.2 with the following exceptions:

-	Instead of A.4.3.1.2  use A.4.3.1.4.

##### 4.3.1.4.4	Test description

4.3.1.4.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 5MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.22.

2.	The general test parameter settings are set up according to Table 4.3.1.4.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.1.4.4.3.

5.	There is one E-UTRA FDD cell and one UTRA FDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.1.4.4.1-1: General test parameters for EUTRA FDD- 
lower priority UTRA FDD inter RAT cell re-selection test case for 5MHz bandwidth

Same as Table 4.3.1.2.4.1-1.

4.3.1.4.4.2	Test procedure

Same test procedure as in clause 4.3.1.2.4.2 with the following exceptions:

-	Instead of Table 4.3.1.2.5-1  use Table 4.3.1.4.5-1.

4.3.1.4.4.3	Message contents

Same message contents as 4.3.1.2.4.3 with the following exceptions:

Table 4.3.1.4.4.3-1: System Information Block type 19:
Inter-RAT E-UTRAN FDD - UTRA FDD is of lower priority cell re-selection for 5MHz bandwidth

| Derivation Path: 36.508 [7] clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 21 (42dB) |  |  |
| s-PrioritySearch2 | 0 | default value is 0 |  |
| threshServingLow | 0 (0dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 25 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 24 (48 dB) |  |  |
| threshXlow | 1 (2 dB) | Default value |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

##### 4.3.1.4.5	Test requirement

Same test requirement as in clause 4.3.1.2.5 with the following exceptions:

Tables 4.3.1.4.4.1-1, 4.3.1.4.5-1 and 4.3.1.4.5-2 define the primary level settings including test tolerances for E-UTRAN FDD- UTRAN FDD inter-RAT cell re-selection test case which UTRA is of lower priority.

Table 4.3.1.4.5-1: Cell specific test parameters for cell 1 (E-UTRA) for 5MHz bandwidth

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 5 |  |
| OCNG Patterns defined in D.1.16 (OP.16 FDD) |  | OP.16 FDD |  |
| NOTE: See Table 4.3.1.2.5-1 for the other parameters. |  |  |  |

#### 4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection

##### 4.3.1.5.1	Test purpose

To verify that the UE is able to search and measure neighbouring UTRA FDD cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements for increased UE carrier monitoring UTRA when the UTRA FDD cells are of lower priority than E-UTRA serving cell.

##### 4.3.1.5.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 12 and forward that support increased carrier monitoring UTRA.

##### 4.3.1.5.3	Minimum conformance requirements

The cell reselection delay is defined as the time from the beginning of a relevant time period, to the moment when the UE camps on the target cell, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on the target cell.

When the measurement rules indicate that UTRA FDD cells are to be measured, the UE shall measure CPICH Ec/Io and CPICH RSCP of detected UTRA FDD cells in the neighbour frequency list at the minimum measurement rate specified in this section. The UE shall filter CPICH Ec/Io and CPICH RSCP measurements of each measured UTRA FDD cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

If the UE is not configured with eDRX_IDLE cycle or configured with eDRX_IDLE cycle not longer than 20.48 s, the UE shall evaluate whether newly detectable UTRA FDD cells in normal performance group have met the reselection criteria in 3GPP TS 36.304 within time NUTRA_carrier,normal * TdetectUTRA_FDD , and evaluate whether newly detectable UTRA FDD cells in reduced performance group have met the reselection criteria in 3GPP TS 36.304 within time 6 * NUTRA_carrier,reduced * TdetectUTRA_FDD when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when TreselectionRAT = 0 provided that the reselection criteria is met by a margin of at least 6dB for reselections based on RSCP, or a margin of at least 3dB for reselections based on Ec/Io. If the UE is configured with eDRX_IDLE cycle longer than 20.48 s, the UE shall evaluate whether newly detectable UTRA FDD cells in normal performance group have met the reselection criteria in 3GPP TS 36.304 within time (NUTRA_carrier,normal) * TdetectUTRA_FDD , and when Srxlev < 3 dB or Squal < 3 dB evaluate whether newly detectable UTRA FDD cells in reduced performance group have met the reselection criteria in 3GPP TS 36.304 within time 6 * NUTRA_carrier,reduced * TdetectUTRA_FDD when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when TreselectionRAT = 0 provided that the reselection criteria is met by a margin of at least 6dB for reselections based on RSCP, or a margin of at least 3dB for reselections based on Ec/Io.

If the UE is not configured with eDRX_IDLE cycle or configured with eDRX_IDLE cycle not longer than 20.48 s, cells which have been detected shall be measured at least every NUTRA_carrier,normal * TmeasureUTRA_FDD for the cells in normal performance group, and at least every 6 * NUTRA_carrier,reduced * TmeasureUTRA_FDD for the cells in reduced performance group when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ. If the UE is configured with eDRX_IDLE cycle longer than 20.48 s, cells which have been detected shall be measured at least every (NUTRA_carrier,normal) * TmeasureUTRA_FDD for the cells in normal performance group, and when Srxlev < 3 dB or Squal < 3 dB at least every 6 * NUTRA_carrier,reduced * TmeasureUTRA_FDD for the cells in reduced performance group when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

When higher priority UTRA FDD cells are found by the higher priority search, they shall be measured at least every Tmeasure,UTRA_FDD. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE is not configured with eDRX_IDLE cycle or configured with eDRX_IDLE cycle not longer than 20.48 s, for a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA FDD cell has met reselection criterion defined in 3GPP TS 36.304 [1] within NUTRA_carrier,normal * TevaluateUTRA_FDD if the cell is in normal performance group and within 6 * NUTRA_carrier,reduced * TevaluateUTRA_FDD if the cell is in reduced performance group when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB for reselections based on RSCP, or a margin of at least 3dB for reselections based on Ec/Io. If the UE is configured with eDRX_IDLE cycle longer than 20.48 s, for a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA FDD cell has met reselection criterion defined in 3GPP TS 36.304 [1] within (NUTRA_carrier,normal) * TevaluateUTRA_FDD if the cell is in normal performance group and when Srxlev < 3 dB or Squal < 3 dB within 6 * NUTRA_carrier,reduced * TevaluateUTRA_FDD if the cell is in reduced performance group when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB for reselections based on RSCP, or a margin of at least 3dB for reselections based on Ec/Io.

If Treselection timer has a non zero value and the UTRA FDD cell is satisfied with the reselection criteria which are defined in [1], the UE shall evaluate this UTRA FDD cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

For UE not configured with eDRX_IDLE cycle, TdetectUTRA_FDD, TmeasureUTRA_FDD and TevaluateUTRA_FDD are specified in Table 4.2.2.5.1-1. For UE configured with eDRX_IDLE cycle, TdetectUTRA_FDD, TmeasureUTRA_FDD and TevaluateUTRA_FDD are specified in Table 4.2.2.5.1-2, where the requirements apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of TdetectUTRA_FDD, TmeasureUTRA_FDD and TevaluateUTRA_FDD when multiple PTWs are used.

Table 4.3.1.5.3-1: TdetectUTRA_FDD, TmeasureUTRA_FDD, and TevaluateUTRA_FDD

| DRX cycle length [s] | TdetectUTRA_FDD [s] | TmeasureUTRA_FDD [s] (number of DRX cycles) | TevaluateUTRA_FDD [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 30 | 5.12 (16) | 15.36 (48) |
| 0.64 |  | 5.12 (8) | 15.36 (24) |
| 1.28 |  | 6.4(5) | 19.2 (15) |
| 2.56 | 60 | 7.68 (3) | 23.04 (9) |

Table 4.3.1.5.3-2: TdetectUTRA_FDD, TmeasureUTRA_FDD and TevaluateUTRA_FDD for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | TdetectUTRA_FDD [s] (number of DRX cycles) | TmeasureUTRA_FDD [s] (number of DRX cycles) | TevaluateUTRA_FDD [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
| 5.12 ≤ eDRX_IDLE cycle length ≤ 2621.44 | 0.32 | ≥1.28 (1) | Note 3 (23) | 0.96 (3) | Note 3 (9) |
|  | 0.64 | ≥2.56 (2) |  | 1.92 (3) | Note 3 (9) |
|  | 1.28 | ≥3.84 (3) |  | 3.84 (3) | Note 3 (9) |
|  | 2.56 | ≥7.68 (6) |  | 7.68 (3) | Note 3 (9) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of 3GPP TS 24.008 [34]. NOTE 3: The time is calculated depending on the number N of DRX cycles as follows: |  |  |  |  |  |

For higher priority cells, a UE may optionally use a shorter value for TmeasureUTRA_FDD, which shall not be less than Max(0.64 s, one DRX cycle).

UE which support increased UE carrier monitoring UTRA according to the capabilities in [5,14] shall additionally be capable of monitoring at least

-	Depending on UE capability, 6 FDD UTRA carriers, and

-	Depending on UE capability, 7 TDD UTRA carriers, and

In addition to the requirements defined above, a UE supporting E-UTRA measurements in RRC_IDLE state and supporting Increased UE carrier monitoring E-UTRA or increased UE carrier monitoring UTRA according to the capabilities in [5,14] shall be capable of monitoring a total of at least 13 carrier frequency layers, which includes serving layer, comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD, GSM (one GSM layer corresponds to 32 cells), cdma2000 1x and HRPD layers.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.1, 4.2.2.9a and A.4.3.1.5

##### 4.3.1.5.4	Test description

4.3.1.5.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure 54 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands..

2.	The general test parameter settings are set up according to Table 4.3.1.5.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.1.5.4.3.

5.	There is one E-UTRA FDD cell and two UTRA FDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.1.5.4.1-1: General test parameters for E-UTRAN FDD- UTRAN FDD inter frequency cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| T0 | Active cell |  | Cell 1 | T0 is repeated on each repetition of the test. In T0 the test equipment selects frequencies for cell 2 and 3, and then time is allowed for the UE to identify the neighbour cells. See cell specific parameters for detailed settings. |
| T1 start condition | Active cell |  | Cell 1 |  |
| T1 end condition | Active cell |  | Cell 2 | UE shall perform reselection to cell 2 during T1 |
|  | Neighbour cell |  | Cell 1, cell 3 |  |
| T2 end condition | Active cell |  | Cell 1 | UE shall perform reselection to cell 1 during T2 |
|  | Neighbour cell |  | Cell 2, cell 3 |  |
| T3 end condition | Active cell |  | Cell 3 | UE shall perform reselection to cell 3 during T3 |
|  | Neighbour cell |  | Cell 1, cell 2 |  |
| T4 end condition | Active cell |  | Cell 1 | UE shall perform reselection to cell 1 during T4 |
|  | Neighbour cell |  | Cell 2, cell 3 |  |
| UE configured E-UTRA RF Channel Number |  |  | 1 | Serving cell and six UTRA FDD carrier frequencies are used in the UE neighbour cell list. Frequencies 5,6 and 7 are indicated to have reduced performance |
| UE configured UTRA RF Channel Number |  |  | 2,3,4,5,6,7 |  |
| Test equipment configuration |  |  | Cell 1 uses E-UTRA RF channel number 1 Cells 2,3,4 are randomly selected to use different frequencies selected from frequencies 2,3,4,5,6,7 |  |
| PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in 3GPP TS 36.211 |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T0 |  | s | (test equipment frequency selection and configuration time) + 960 | Initialisation time need to be defined so that cell detection time is taken into account. |
| T1 |  | s | 60 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 25 | T2 need to be defined so that cell re-selection reaction time is taken into account |
| T3 |  | s | 350 | T3 need to be defined so that cell re-selection reaction time is taken into account. |
| T4 |  | s | 25 | T4 need to be defined so that cell re-selection reaction time is taken into account |

4.3.1.5.4.2	Test procedure

The test scenario comprises of indicating 6 UTRA FDD interfrequency cells on 6 different carriers in the neighbour list of cell 1, cells 2 and 3. Each repetition of the test consists of five successive time periods, with time duration of T0, T1, T2, T3 and T4 respectively. In the initialisation phase and at the start of each repetition of T0, the test equipment provides signals for cell 1 (serving cell), and selects frequencies for cells 2 and 3 which are chosen from the 6 inter-RAT layers which are configured in the UE neighbour cell list as described in general and cell specific parameters. The neighbour lists of cells 2 and 3 shall include the frequency of cell 1 in the normal performance group and shall exclude other frequencies configured to the UE in the test.

Cell 1, 2 and 3 are identified by the UE during time period T0. Cell 1, cell 2 and cell 3 all belong to different tracking areas.  Furthermore, UE has not registered with network for the tracking area containing cell 2 or 3. Cells 2 and 3 all have lower absolute priority than cell 1.

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to T0 in Table 4.3.1.5.5-1 and 4.3.1.5.5-2. Propagation conditions are set according to Annex B clause B.1.1. T0 starts. SS provides signals for cell 1 and selects frequencies for cells 2 and 3. If the UE is already camped in Cell 1, wait until T0 expires and skip to step 5.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1

4.	If the UE has re-selected Cell 1 within T0, after the re-selection or when T0 expires, continue with step 5.
Otherwise, if T0 expires and the UE has not yet re-selected Cell 1, skip to step 24.

5.	The SS shall switch the power setting from T0 to T1 as specified in Table 4.3.1.5.5-1 and 4.3.1.5.5-2.

6.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

7.	If the UE responds on the Cell 2 during time duration T1 within 59 seconds from the beginning of time period T1, then count a success for the event "Re-select Cell 2". Otherwise count a fail for the event "Re-select Cell 2".

8. 	If the UE has re-selected Cell 2 within T1, after the re-selection or when T1 expires, skip to step 10.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 2, continue with step 9.

9. 	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

10.	The SS shall switch the power setting from T1 to T2 as specified in Table 4.3.1.5.5-1 and 4.3.1.5.5-2.

11.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

12.	If the UE responds on the Cell 1 during time duration T2 within 21 seconds from the beginning of time period T2, then count a success for the event "Re-select Cell 1". Otherwise count a fail for the event "Re-select Cell 1".

13.	If the UE has re-selected Cell 1 within T2, after the re-selection or when T2 expires, skip to step 15.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 1, continue with step 14.

14.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

15.	The SS shall switch the power setting from T2 to T3 as specified in Table 4.3.1.5.5-1 and 4.3.1.5.5-2.

16.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 3.

17.	If the UE responds on the Cell 3 during time duration T3 within 347 seconds from the beginning of time period T3, then count a success for the event "Re-select Cell 3". Otherwise count a fail for the event "Re-select Cell 3".

18.	If the UE has re-selected Cell 3 within T3, after the re-selection or when T3 expires, skip to step 20.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 3, continue with step 19.

19. Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

20.	The SS shall switch the power setting from T3 to T4 as specified in Table 4.3.1.5.5-1 and 4.3.1.5.5-2.

21.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

22.	If the UE responds on the Cell 1 during time duration T4 within 21 seconds from the beginning of time period T4, then count a success for the event "Re-select Cell 1". Otherwise count a fail for the event "Re-select Cell 1".

23.	If the UE has re-selected Cell 1 within T4, after the re-selection or when T4 expires, skip to step 25.
Otherwise, if T4 expires and the UE has not yet re-selected Cell 1, continue with step 24.

24. Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell

25.	Repeat step 2-25 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

4.3.1.5.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 with the following exceptions:

Table 4.3.1.5.4.3-1: Common Exception messages for E-UTRAN FDD- UTRAN FDD inter-rat event triggered reporting for increased carrier monitoring UTRA with reduced performance group configured, non DRX test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-5 Table H.2.3-6 Table H.2.3-13 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-1 |

Table 4.3.1.5.4.3-2: UTRA SystemInformationBlockType19:
Inter-RAT E-UTRAN FDD - UTRA FDD (UTRA FDD is of lower priority) cell re-selection

| Derivation Path: 36.508 [7] clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 31 (62dB) |  |  |
| s-PrioritySearch2 | 0 | default value is 0 |  |
| threshServingLow | 18 (36dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 4.3.1.5.4.3-3: SystemInformationBlockType6:
Inter-RAT E-UTRAN FDD/TDD - UTRA FDD (UTRA FDD is of lower priority) cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-5 SystemInformationBlockType6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| t-ReselectionUTRA | 0 |  |  |
| } |  |  |  |
| CarrierFreqUTRA-FDD-Ext-r12 ::= SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| threshX-Low-r12 | 24 (52dB) | 50 is actual value in dB (25 * 2 dB) |  |
| q-RxLevMin-r12 | -57 (-115 dBm) | -115 is actual value in dBm (-57 * 2 + 1 dBm) |  |
| p-MaxUTRA-r12 | 21 (21 dBm) |  |  |
| q-QualMin-r12 | -20 (-20 dB) |  |  |
| reducedMeasPerformance-r12 | TRUE for Cell 3 | If neighbouring inter-frequency cell is configured for reduced measurement performance |  |
| } |  |  |  |

Table 4.3.1.5.4.3-4: SystemInformationBlockType3:
Inter-RAT E-UTRAN FDD/TDD - UTRA FDD (UTRA is of lower priority) cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 62 |  |  |
| threshServingLow | 36 |  |  |
| cellReselectionPriority | 3 |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 (-140 dBm) |  |  |
| t-ReselectionEUTRA | 0 |  |  |
| } |  |  |  |

##### 4.3.1.5.5	Test requirement

Tables 4.3.1.5.4.1-1, 4.3.1.5.5-1 and 4.3.1.5.5-2 define the primary level settings including test tolerances for inter-RAT cell re-selection E-UTRA FDD to UTRA FDD test case for increased UTRA channel monitoring (UTRA is of lower priority).

Table 4.3.1.5.5-1: Cell specific test parameters for E-UTRAN FDD- 
UTRAN FDD inter-RAT cell reselection test case in AWGN cell 1 (E-UTRAN)

| Parameter | Unit | Cell 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 |  | T2 | T3 | T4 |
| E-UTRA RF Channel number |  | 1 |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB = 25 10MHz: NRB = 50 |  |  |  |  |  |
| Io | dBm/4.5MHz(25RB) dBm/9Mhz (50RB) | -59.06 -56.05 |  | -71.77 -68.76 | -59.06 -56.05 | -71.77 -68.76 | -59.06 -56.05 |
| PDSCH parameters: DL Reference Measurement Channel |  | OP.16 FDD (5MHz) OP.2 FDD (10MHz) |  |  |  |  |  |
| PBCH RA | dB | 0 |  |  |  |  |  |
| PBCH RB | dB |  |  |  |  |  |  |
| PSS RA | dB |  |  |  |  |  |  |
| SSS RA | dB |  |  |  |  |  |  |
| PCFICH RB | dB |  |  |  |  |  |  |
| PHICH RA | dB |  |  |  |  |  |  |
| PHICH RB | dB |  |  |  |  |  |  |
| PDCCH RA | dB |  |  |  |  |  |  |
| PDCCH RB | dB |  |  |  |  |  |  |
| PDSCH RA | dB |  |  |  |  |  |  |
| PDSCH RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |  |  |  |
| Note 2 | dBm | -98 |  |  |  |  |  |
| RSRP Note 3 | dBm | -84 | -101.7 |  | -84 | -101.7 | -84 |
|  | dB | 14 | -3.7 |  | 14 | -3.7 | 14 |
|  | dB | 14 | -3.7 |  | 14 | -3.7 | 14 |
| TreselectionEUTRAN | s | 0 |  |  |  |  |  |
| Snonintrasearch | dB | 62 |  |  |  |  |  |
| Threshserving, low | dB | 44 |  |  |  |  |  |
| Threshx, low (Note 4) | dB | 40 |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |  |  |  |  |  |

Table 4.3.1.5.5-2: Cell specific test parameters for cells 2 and 3 (UTRA)

| Parameter | Unit | Cell 2 |  |  |  |  | Cell 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T2 | T3 | T4 | T0 | T1 | T2 | T3 | T4 |
| UTRA RF Channel Number |  | Randomly selected from 1, 2, 3 such that cell 2 is in the normal performance group |  |  |  |  | Randomly selected from 4, 5, 6 such that cell 3 is in the reduced performance group |  |  |  |  |
| CPICH_Ec/Ior | dB | -10 |  |  |  |  | -10 |  |  |  |  |
| PCCPCH_Ec/Ior | dB | -12 |  |  |  |  | -12 |  |  |  |  |
| SCH_Ec/Ior | dB | -12 |  |  |  |  | -12 |  |  |  |  |
| PICH_Ec/Ior | dB | -15 |  |  |  |  | -15 |  |  |  |  |
| OCNS_Ec/Ior | dB | -0.941 |  |  |  |  | 0.941 |  |  |  |  |
|  | dB | +11 | +11.8 | +11.8 | -5 | +11 | +11 | -5 | +11 | +11.8 | +11.8 |
|  | dBm/3,84 MHz | -70 |  |  |  |  | -70 |  |  |  |  |
| CPICH_Ec/Io | dB | -10.33 | -10.28 | -10.28 | -16.19 | -10.33 | -10.33 | -16.19 | -10.33 | -10.28 | -10.28 |
| CPICH_RSCP | dBm | -69 | -68.2 | -68.2 | -85 | -69 | -69 | -85 | -69 | -68.2 | -68.2 |
| Propagation Condition |  | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Qqualmin | dB | -20 |  |  |  |  | -20 |  |  |  |  |
| Qrxlevmin | dBm | -115 |  |  |  |  | -115 |  |  |  |  |
| QrxlevminEUTRA | dBm | -140 |  |  |  |  | -140 |  |  |  |  |
| UE_TXPWR_MAX_RACH | dBm | 21 |  |  |  |  | 21 |  |  |  |  |
| Treselection | s | 0 |  |  |  |  | 0 |  |  |  |  |
| Sprioritysearch1 | dB | 40 |  |  |  |  | 40 |  |  |  |  |
| Sprioritysearch2 | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| Threshx, high (Note) | dB | 50 |  |  |  |  | 50 |  |  |  |  |
| NOTE: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell. |  |  |  |  |  |  |  |  |  |  |  |

The cell reselection delay is defined as the time from the beginning of a relevant time period, to the moment when the UE camps on the target cell, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on the target cell.

The reselection delays shall meet the requirements in table 4.3.1.5.5-1

Table 4.3.1.5.5-3: FFS

| Time phase | Target cell | Requirement for reselection delay (seconds) |
| --- | --- | --- |
| T0 | Cell 1 |  |
| T1 | Cell 2 (normal performance group) | 59 |
| T2 | Cell 1 (normal performance group) | 21 |
| T3 | Cell 3 (reduced performance group) | 347 |
| T4 | Cell 1 (normal performance group) | 21 |

NOTE:	The cell re-selection delay to a normal performance group cell can be expressed as: (NUTRA_carrier,normal) * TevaluateUTRA_FDD + TSI-UTRA and to a reduced performance group cell can be expressed as: 6 * NUTRA_carrier,reduced * TevaluateUTRA_FDD + TSI-UTRA.

Where:

TevaluateUTRA-FDD	See Table 4.2.2.5.1-1

TSI-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 58.88 s for normal performance group reselection, allow 59 s, and gives a total of 346.88 s for reduced performance group reselection, allow 347 s for reduced performance group in the test case. For reselections back to cell 1 since only one frequency is configured, the requirement is Tevaluate,E-UTRAN_Inter, + TSI = 20.48s, allow 21s.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

### 4.3.2	E-UTRAN FDD - UTRAN TDD cell re-selection

#### 4.3.2.1	Test purpose

To verify that the UE is able to search and measure neighbouring UTRAN TDD cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements when the UTRA cell is of lower priority.

#### 4.3.2.2	Test applicability

This test applies to all types of release 8 and forward E-UTRA FDD UEs that support release 9 and forward UTRA TDD.

#### 4.3.2.3	Minimum conformance requirements

##### 4.3.2.3.1	3.84Mcps TDD option

There are no requirements so this is not tested.

##### 4.3.2.3.2	1.28Mcps TDD option

When the measurement rules indicate that UTRA TDD cells are to be measured, the UE shall measure P-CCPCH RSCP of detected UTRA TDD cells in the neighbour frequency list at the minimum measurement rate specified in this section. The parameter NUTRA_carrier_TDD is the number of carriers in the neighbour frequency list. The UE shall filter P-CCPCH RSCP measurements of each measured UTRA TDD cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period. P-CCPCH RSCP of UTRAN TDD cells shall not be filtered over a longer period than that specified in 3GPP TS 36.133 [4] table 4.2.2.5.2-1.

If the SServingCellof the E-UTRA serving cell is greater than Snonintrasearch then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in section 36.133 [4] clauses 4.2.2.

If the SServingCellof the E-UTRA serving cell is less than or equal to Snonintrasearch then the UE shall search for and measure inter-RAT layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

The UE shall evaluate whether newly detectable UTRA TDD cells have met the reselection criteria in 3GPP TS 36.304 within time (NUTRA_carrier_TDD) * TdetectUTRA_TDD when the SServingCell of the E-UTRA serving cell (or other cells on the same frequency layer) is less than Snonintrasearch when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB.

Cells which have been detected shall be measured at least every (NUTRA_carrier_TDD) * TmeasureUTRA_TDD when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch.

When higher priority UTRA TDD cells are found by the higher priority search, they shall be measured at least every Tmeasure,UTRA_TDD. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell.

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA TDD cell has met reselection criterion defined in 3GPP TS 36.304 [6] within NUTRA_carrier_TDD *TevaluateUTRA_TDD when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6 dB.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause 4.2.2.5.2 and A.4.3.2.

##### 4.3.2.3.3	7.68Mcps TDD option

There are no requirements so this is not tested.

#### 4.3.2.4	Test description

##### 4.3.2.4.1	3.84Mcps TDD option

There are no requirements so this is not tested.

##### 4.3.2.4.2	1.28Mcps TDD option

4.3.2.4.2.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10 MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A figure A.22 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.3.2.4.2.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.2.4.2.3.

5.	There is one E-UTRA FDD cell and one UTRA TDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.2.4.2.1-1: General test parameters for E-UTRA FDD to UTRA
(1.28 Mcps TDD OPTION) Cell Re-selection

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell 1 | E-UTRA FDD cell |
| T1 end condition | Active cell |  | Cell1 | UE shall perform reselection to cell 1 during T1 for subsequent iterations of the test. |
|  | Neighbour cell |  | Cell2 | 1.28 Mcps TDD OPTION cell |
| T2 end condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T2 |
|  | Neighbour cell |  | Cell1 | E-UTRA FDD cell |
| CP length of cell 1 |  |  | normal |  |
| E-UTRA PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in 3GPP TS 36.211 |
| Time offset between cells |  |  | 3 ms | Asynchronous cells 3ms or 92160*Ts |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure. |
| Treselection |  | s | 0 |  |
| DRX cycle length |  | s | 1,28 |  |
| HCS |  |  | Not used |  |
| T1 |  | s | 85 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 25 |  |

4.3.2.4.2.2	Test procedure

The test consists of one active cell and one neighbour cell. In the test there are two successive time periods, with time duration of T1 and T2 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the test. Cell 1 and Cell 2 belong to different tracking areas. The UTRA TDD layer is configured at a lower priority than the E-UTRA FDD layer.

The ranking of the cells shall be made according to the cell reselection criteria specified in 3GPP TS 36.304.

In the following test procedure "UE responds" means "UE starts transmitting the SYNCH-UL sequence in the UpPTS for sending RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure".

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to T1 in Table 4.3.2.5.2-1 and 4.3.2.5.2-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts. If the UE is already camped in Cell 1, wait until T1expires and skip to step 5.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on the higher priority cell, Cell 1.

4.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 5.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, skip to step 9.

5.	The SS shall switch the power setting from T1 to T2 as specified in Table 4.3.2.5.2-1 and 4.3.2.5.2-2.

6.	The SS waits for random access requests information from the UE to perform cell re-selection on the lower priority cell, Cell 2.

7.	If the UE responds on lower priority cell, Cell 2 during time duration T2 within 21 seconds from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, skip to step 10.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, continue with step 9.

9.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

4.3.2.4.2.3	Message contents

Message contents are according TS 36.508 [7] clause 4.6 and 3GPP TS 34.108 [24] clause 6.1.0b with the following exceptions:

Table 4.3.2.4.2.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-7 Table H.2.3-8 Table H.2.3-14 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-1 |

Table 4.3.2.4.2.3-2: System Information Block type 19:
E-UTRA FDD- lower priority UTRA TDD inter RAT cell re-selection test case

| Derivation Path: 36.508 clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 0 (0dB) |  |  |
| s-PrioritySearch2 | Not present | default value is 0 |  |
| threshServingLow | 0 (0dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 23 (46 dB) |  |  |
| threshXlow | 1 (2 dB) | Default value |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

##### 4.3.2.4.3	7.68 Mcps TDD option

There are no requirements so this is not tested.

#### 4.3.2.5	Test requirement

##### 4.3.2.5.1	3.84Mcps TDD option

There are no requirements so this is not tested.

##### 4.3.2.5.2	1.28Mcps TDD option

Tables 4.3.2.4.2-1, 4.3.2.5.2-1 and 4.3.2.5.2-2 defines the primary level settings including test tolerances for E-UTRAN FDD to UTRA TDD cell re-selection test case.

Table 4.3.2.5.2-1: Cell specific test parameters for
cell re-selection E-UTRA FDD to UTRA TDD test case (cell 1)

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RB | dB |  |  |
| SSS_RB | dB |  |  |
| PCFICH_PA | dB |  |  |
| PHICH_PA | dB |  |  |
| PHICH_PB | dB |  |  |
| PDCCH_PA | dB |  |  |
| PDCCH_PB | dB |  |  |
| PDSCH_PA | dB |  |  |
| PDSCH_PB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Qrxlevmin | dBm/15kHz | -140 | -140 |
|  | dBm/15kHz | -98 |  |
| RSRP | dBm/15kHz | -86.9 | -101.1 |
|  | dB | 11.1 | -3.1 |
| Snonintrasearch | dB | Not sent |  |
| Threshserving, low | dB | 46 (-94dBm) |  |
| Threshx, low (Note2) | dB | 24 (-79dBm) |  |
| Propagation Condition |  | AWGN |  |
| NOTE 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA TDD target cell. |  |  |  |

Table 4.3.2.5.2-2: Cell specific test parameters for
cell re-selection E-UTRA FDD to UTRA TDD test case (cell 2)

| Parameter |  | Unit | Cell 2 (UTRA) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Timeslot Number |  |  | 0 |  |  | DwPTS |  |
|  |  |  | T1 |  | T2 | T1 | T2 |
| UTRA RF Channel Number (Note1) |  |  | Channel 2 |  |  |  |  |
| PCCPCH_Ec/Ior | dB |  | -3 |  | -3 |  |  |
| DwPCH_Ec/Ior | dB |  |  |  |  | 0 | 0 |
| OCNS_Ec/Ior | dB |  | -3 |  | -3 |  |  |
|  | dB |  | 11 |  | 11 | 11 | 11 |
|  | dBm/1.28 MHz |  | -80 |  |  |  |  |
| PCCPCH RSCP | dBm |  | -72 | -72 |  | n.a. | n.a. |
| Propagation Condition |  |  | AWGN |  |  |  |  |
| Qrxlevmin | dBm |  | -103 |  |  |  |  |
| Qoffset1s,n | dB |  | C1, C2: 0 |  |  |  |  |
| Qhyst1s | dB |  | 0 |  |  |  |  |
| Threshx, high (Note2) | dB |  | 46 (-94dBm) |  |  |  |  |
| NOTE 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency's channel number. NOTE 2: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell. |  |  |  |  |  |  |  |

The cell reselection delay to lower priority is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send the SYNCH-UL sequence in the UpPTS for sending the RRC CONNECTION REQUEST message on cell 2.

The cell re-selection delay to lower priority shall be less than 21 s.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

NOTE:	The cell re-selection delay to lower priority cell can be expressed as: TevaluateUTRA_TDD + TSI-UTRA

Where:

TevaluateUTRA_TDD	19.2s, as specified in 3GPP TS 36.133 [4] table 4.2.2.5.2-1

TSI-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 20.48 s, allow 21 s for lower priority cell reselection in the test case.

##### 4.3.2.5.3	7.68 Mcps TDD option

There are no requirements so this is not tested.

### 4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon

#### 4.3.2A.1	Test purpose

This test is to verify the requirement for the E-UTRA FDD to UTRA TDD inter-RAT cell reselection requirements for increased UE carrier monitoring. UTRA TDD cells are of lower priority than E-UTRA serving cell.

#### 4.3.2A.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 12 and forward that support increased carrier monitoring UTRA.

#### 4.3.2A.3	Minimum conformance requirements

The cell reselection delay is defined as the time from the beginning of a relevant time period, to the moment when the UE camps on the target cell, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on the target cell.

When the measurement rules indicate that UTRA TDD cells are to be measured, the UE shall measure P-CCPCH RSCP of detected UTRA TDD cells in the neighbour frequency list at the minimum measurement rate specified in this section. The UE shall filter P-CCPCH RSCP measurements of each measured UTRA TDD cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period. If the UE is not configured with eDRX_IDLE cycle, P-CCPCH RSCP of UTRAN TDD cells shall not be filtered over a longer period than that specified in table 4.2.2.5.2-1. If the UE is configured with eDRX_IDLE cycle, P-CCPCH RSCP of UTRAN TDD cells shall not be filtered over a longer period than that specified in table 4.2.2.5.2-2.

If the UE is not configured with eDRX_IDLE cycle or configured with eDRX_IDLE cycle not longer than 20.48 s, the UE shall evaluate whether newly detectable UTRA TDD cells in normal performance group have met the reselection criteria in 3GPP TS 36.304 within time NUTRA_carrier_TDD,normal * TdetectUTRA_TDD, and evaluate whether newly detectable UTRA TDD cells in reduced performance group have met the reselection criteria in 3GPP TS 36.304 within time 6 * NUTRA_carrier_TDD,reduced * TdetectUTRA_TDD when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ  when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB. If the UE is configured with eDRX_IDLE cycle longer than 20.48 s, the UE shall evaluate whether newly detectable UTRA TDD cells in normal performance group have met the reselection criteria in 3GPP TS 36.304 within time (NUTRA_carrier_TDD,normal) * TdetectUTRA_TDD, and when Srxlev < 3 dB or Squal < 3 dB evaluate whether newly detectable UTRA TDD cells in reduced performance group have met the reselection criteria in 3GPP TS 36.304 within time 6 * NUTRA_carrier_TDD,reduced * TdetectUTRA_TDD when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ  when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB.

If the UE is not configured with eDRX_IDLE cycle or configured with eDRX_IDLE cycle not longer than 20.48 s, cells which have been detected shall be measured at least every NUTRA_carrier_TDD,normal * TmeasureUTRA_TDD for the cells in normal performance group, and at least every 6 * NUTRA_carrier_TDD,reduced * TmeasureUTRA_TDD for the cells in reduced performance group, when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ. If the UE is configured with eDRX_IDLE cycle longer than 20.48 s, cells which have been detected shall be measured at least every (NUTRA_carrier_TDD,normal) * TmeasureUTRA_TDD for the cells in normal performance group, and when Srxlev < 3 dB or Squal < 3 dB at least every 6 * NUTRA_carrier_TDD,reduced * TmeasureUTRA_TDD for the cells in reduced performance group, when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

When higher priority UTRA TDD cells are found by the higher priority search, they shall be measured at least every Tmeasure,UTRA_TDD. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE is not configured with eDRX_IDLE cycle or configured with eDRX_IDLE cycle not longer than 20.48 s, for a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA TDD cell has met reselection criterion defined in [1] within NUTRA_carrier_TDD,normal *TevaluateUTRA_TDD if the cell is in normal performance group and within 6 * NUTRA_carrier_TDD,reduced * TevaluateUTRA_TDD if the cell is in reduced performance group when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB. If the UE is configured with eDRX_IDLE cycle longer than 20.48 s, for a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA TDD cell has met reselection criterion defined in [1] within NUTRA_carrier_TDD,normal *TevaluateUTRA_TDD if the cell is in normal performance group and when Srxlev < 3 dB or Squal < 3 dB within 6 * NUTRA_carrier_TDD,reduced * TevaluateUTRA_TDD if the cell is in reduced performance group when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB.

If Treselection timer has a non zero value and the UTRA TDD cell is satisfied with the reselection criteria which are defined in [1], the UE shall evaluate this UTRA TDD cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

For UE not configured with eDRX_IDLE cycle, TdetectUTRA_TDD, TmeasureUTRA_TDD and TevaluateUTRA_TDD are specified in Table 4.3.2A.3-1. For UE configured with eDRX_IDLE cycle, TdetectUTRA_TDD, TmeasureUTRA_TDD and TevaluateUTRA_TDD are specified in Table 4.3.2A.3-2, where the requirements apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of TdetectUTRA_TDD, TmeasureUTRA_TDD and TevaluateUTRA_TDD when multiple PTWs are used.

Table 4.3.2A.3-1: TdetectUTRA_TDD, TmeasureUTRA_TDD and TevaluateUTRA_TDD

| DRX cycle length [s] | TdetectUTRA_TDD [s] | TmeasureUTRA_TDD [s] (number of DRX cycles) | TevaluateUTRA_TDD [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 30 | 5.12 (16) | 15.36 (48) |
| 0.64 |  | 5.12 (8) | 15.36 (24) |
| 1.28 |  | 6.4(5) | 19.2 (15) |
| 2.56 | 60 | 7.68 (3) | 23.04 (9) |

Table 4.3.2A.3-2: TdetectUTRA_TDD, TmeasureUTRA_TDD and TevaluateUTRA_TDD for
UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | TdetectUTRA_TDD [s] (number of DRX cycles) | TmeasureUTRA_TDD [s] (number of DRX cycles) | TevaluateUTRA_TDD [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
| 5.12 ≤ eDRX_IDLE cycle length ≤ 2621.44 | 0.32 | ≥1.28 (1) | Note 3 (23) | 0.96 (3) | Note 3 (9) |
|  | 0.64 | ≥2.56 (2) |  | 1.92 (3) | Note 3 (9) |
|  | 1.28 | ≥3.84 (3) |  | 3.84 (3) | Note 3 (9) |
|  | 2.56 | ≥7.68 (6) |  | 7.68 (3) | Note 3 (9) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of 3GPP TS 24.008 [34]. NOTE 3: The time is calculated depending on the number N of DRX cycles as follows: |  |  |  |  |  |

For higher priority cells, a UE may optionally use a shorter value for TmeasureUTRA_TDD, which shall not be less than Max(0.64 s, one DRX cycle).

UE which support increased UE carrier monitoring UTRA according to the capabilities in [5,14] shall additionally be capable of monitoring at least

-	Depending on UE capability, 6 FDD UTRA carriers, and

-	Depending on UE capability, 7 TDD UTRA carriers, and

In addition to the requirements defined above, a UE supporting E-UTRA measurements in RRC_IDLE state and supporting Increased UE carrier monitoring E-UTRA or increased UE carrier monitoring UTRA according to the capabilities in [5,14] shall be capable of monitoring a total of at least 13 carrier frequency layers, which includes serving layer, comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD, GSM (one GSM layer corresponds to 32 cells), cdma2000 1x and HRPD layers.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.2, 4.2.2.9a and A.4.3.2A

#### 4.3.2A.4	Test description

##### 4.3.2A.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10 MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A figure 54 for UE with 2Rx RF band and Annex A, Figure TBS for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.3.2A.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.2A.4.3.

5.	There is one E-UTRA FDD cell and two UTRA TDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.2A.4.1-1: General test parameters for E-UTRA FDD to
UTRA TDD inter-RAT cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| T0 | Active cell |  | Cell1/ | T0 is repeated on each repetition of the test. In T0 the test equipment selects frequencies for cell 2, 3 and then time is allowed for the UE to identify the neighbour cells. See cell specific parameters for detailed settings. |
| T1 start condition | Active cell |  | Cell 1 |  |
| T1 end condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T1 |
|  | Neighbour cell |  | Cell1 |  |
| T2 end condition | Active cell |  | Cell1 | UE shall perform reselection to cell 1 during T2 |
| T3 end condition | Active cell |  | Cell3 | UE shall perform reselection to cell 3 during T3 |
|  | Neighbour cell |  | Cell1 |  |
| T4 end condition | Active cell |  | Cell1 | UE shall perform reselection to cell 1 in T4 so that next repetition of test can start from T0 |
| UE configured UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5, 6, 7 | Seven UTRA TDD carrier frequencies are used in the UE neighbour cell list. Frequencies 4, 5, 6, and 7 are indicated to have reduced performance |
| Test equipment configuration |  |  | Cell 1, 2, 3 | Cell 1 uses E-UTRA RF channel number 1 Cells 2 is randomly selected to use different frequencies selected from UTRA frequencies 1, 2, 3. Cells 3 is randomly selected to use different frequencies selected from UTRA frequencies 4, 5, 6, 7. |
| CP length of cell 1 |  |  | normal |  |
| PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in 3GPP TS 36.211 [16] |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Treselection |  | s | 0 |  |
| HCS |  |  | Not used |  |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T0 |  | s | (Test equipment frequency selection and configuration time) + 960 | T0 is defined so that the Test equipment selects frequencies and configures the cells, then the UE cell detection time is taken into account. |
| T1 |  | s | 60 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 25 | T2 need to be defined so that cell re-selection reaction time is taken into account |
| T3 |  | s | 500 | T3 need to be defined so that cell re-selection reaction time is taken into account. |
| T4 |  | s | 25 | T4 need to be defined so that cell re-selection reaction time is taken into account |

##### 4.3.2A.4.2	Test procedure

The test scenario comprises of indicating 7 UTRA TDD inter-RAT cells on 7 different carriers in the neighbour list of cell 1 as given in tables 4.3.2A.5-1,.4.3.2A.5-2 and 4.3.2A.5-3. Each repetition of the test consists of five successive time periods, with time duration of T0, T1, T2, T3, and T4 respectively. In the initialisation phase and at the start of each repetition of T0, the test equipment provides signals for cell 1 (E-UTRA serving cell), and selects frequencies for cells 2 and 3 which are chosen from the 7 inter-RAT layers which are configured in the UE neighbour cell list as described in general and cell specific parameters. The neighbour lists of cells 2 and 3 shall include the frequency of cell 1 in the normal performance group.

Cell 1, 2 and 3 are identified by the UE during time phase T0. Cell 1, cell 2 and cell 3 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2 or 3.

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to T0 in Table 4.3.2A.5-1, 4.3.2A.5-2 and 4.3.2A.5-3. Propagation conditions are set according to Annex B clause B.1.1. T0 starts. SS provides signals for cell 1 and selects frequencies for cells 2 and 3. If the UE is already camped in Cell 1, wait until T0 expires and skip to step 5.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1

4.	If the UE has re-selected Cell 1 within T0, after the re-selection or when T0 expires, continue with step 5.
Otherwise, if T0 expires and the UE has not yet re-selected Cell 1, skip to step 24.

5.	The SS shall switch the power setting from T0 to T1 as specified in Table 4.3.2A.5-1, 4.3.2A.5-2 and 4.3.2A.5-3.

6.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

7.	If the UE responds on the Cell 2 during time duration T1 within 58.9 seconds from the beginning of time period T1, then count a success for the event "Re-select Cell 2". Otherwise count a fail for the event "Re-select Cell 2".

8. 	If the UE has re-selected Cell 2 within T1, after the re-selection or when T1 expires, skip to step 10.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 2, continue with step 9.

9. 	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

10.	The SS shall switch the power setting from T1 to T2 as specified in Table 4.3.2A.5-1, 4.3.2A.5-2 and 4.3.2A.5-3.

11.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

12.	If the UE responds on the Cell 1 during time duration T2 within 20.5 seconds from the beginning of time period T2, then count a success for the event "Re-select Cell 1". Otherwise count a fail for the event "Re-select Cell 1".

13.	If the UE has re-selected Cell 1 within T2, after the re-selection or when T2 expires, skip to step 15.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 1, continue with step 14.

14.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

15.	The SS shall switch the power setting from T2 to T3 as specified in Table 4.3.1.5.5-1 and 4.3.1.5.5-2.

16.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 3.

17.	If the UE responds on the Cell 3 during time duration T3 within 462.1 seconds from the beginning of time period T3, then count a success for the event "Re-select Cell 3". Otherwise count a fail for the event "Re-select Cell 3".

18.	If the UE has re-selected Cell 3 within T3, after the re-selection or when T3 expires, skip to step 20.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 3, continue with step 19.

19.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

20.	The SS shall switch the power setting from T3 to T4 as specified in Table 4.3.2A.5-1, 4.3.2A.5-2 and 4.3.2A.5-3..

21.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

22.	If the UE responds on the Cell 1 during time duration T4 within 20.5 seconds from the beginning of time period T4, then count a success for the event "Re-select Cell 1". Otherwise count a fail for the event "Re-select Cell 1".

23.	If the UE has re-selected Cell 1 within T4, after the re-selection or when T4 expires, skip to step 25.
Otherwise, if T4 expires and the UE has not yet re-selected Cell 1, continue with step 24.

24.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell

25.	Repeat step 2-24 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

##### 4.3.2A.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 with the following exceptions:

Table 4.3.2A.4.3-1: Common Exception messages for E-UTRAN FDD- UTRAN FDD inter-rat event triggered reporting for increased carrier monitoring UTRA with reduced performance group configured, non DRX test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-7 Table H.2.3-6 Table H.2.3-13 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-1 |

Table 4.3.2A.4.33-2: System Information Block type 19: Inter-RAT E-UTRAN FDD - UTRA TDD is of lower priority cell re-selection

| Derivation Path: 36.508 [7] clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 21 (42dB) |  |  |
| s-PrioritySearch2 | 0 | default value is 0 |  |
| threshServingLow | 0 (0dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 24 (48 dB) |  |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 4.3.2A.4.3-3: SystemInformationBlockType6:
Inter-RAT E-UTRAN FDD/TDD - UTRA TDD is of lower priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-5 SystemInformationBlockType6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CarrierFreqUTRA-FDD-Ext-r12 ::= SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| threshX-Low-r12 | 24 |  |  |
| q-RxLevMin-r12 | -70 (-140 dBm) | -140 is actual value in dBm (-70 * 2 + 1 dBm) |  |
| p-MaxUTRA-r12 | 21 (21 dBm) |  |  |
| reducedMeasPerformance-r12 | TRUE for Cell 3 | If neighbouring inter-frequency cell is configured for reduced measurement performance |  |
| } |  |  |  |

Table 4.3.2A.4.3-4: SystemInformationBlockType3:
Inter-RAT E-UTRAN FDD/TDD - UTRA FDD is of lower priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 46 |  |  |

#### 4.3.2A.5	Test requirement

Tables 4.3.2A.4.1-1, 4.3.2A.5-1, 4.3.2A.5-2 and 4.3.2A.5-3 defines the primary level settings including test tolerances for E-UTRAN FDD to UTRA TDD cell re-selection test case.

Table 4.3.2A.5-1: E-UTRA Cell specific test parameters for
E-UTRA FDD to UTRA TDD inter-RAT cell reselection test case in AWGN

| Parameter | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel number |  | 1 |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB = 25 10MHz: NRB = 50 |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RA NOTE 1 | dB |  |  |  |  |  |
| OCNG_RB NOTE 1 | dB |  |  |  |  |  |
| NOTE 2 | dBm/15kHz | -98 |  |  |  |  |
|  | dB | 11.05 | -3.05 | 11.05 | -3.05 | 11.05 |
| NOTE 3 | dB | 11.05 | -3.05 | 11.05 | -3.05 | 11.05 |
| RSRP NOTE 3 | dBm/15kHz | -86.95 | -101.05 | -86.95 | -101.05 | -86.95 |
| Qrxlevmin | dBm/15kHz | -140 |  |  |  |  |
| Snonintrasearch | dB | Not sent |  |  |  |  |
| Threshserving, low | dB | 46 (-94dBm) |  |  |  |  |
| Threshx, low NOTE 4 | dB | 24 (-79dBm) |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Es/Iot and RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA TDD target cell. |  |  |  |  |  |  |

Table 4.3.2A.5-2: UTRA TDD Cell specific test parameters for
E-UTRA FDD to UTRA TDD inter-RAT cell reselection test case in AWGN

| Parameter | Unit | Cell 2 (UTRA TDD) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Timeslot Number |  | 0 |  |  |  |  | DwPTS |  |  |  |  |
|  |  | T0 | T1 | T2 | T3 | T4 | T0 | T1 | T2 | T3 | T4 |
| UTRA RF Channel Number NOTE 1 |  | Randomly selected from 1, 2, 3 such that cell 2 is in the normal performance group |  |  |  |  |  |  |  |  |  |
| PCCPCH_Ec/Ior | dB | -3 |  |  |  |  |  |  |  |  |  |
| DwPCH_Ec/Ior | dB |  |  |  |  |  | 0 |  |  |  |  |
| OCNS_Ec/Ior | dB | -3 |  |  |  |  |  |  |  |  |  |
|  | dBm/ 1.28 MHz | -80 |  |  |  |  |  |  |  |  |  |
|  | dB | -3 | 11 | -3 | -3 | -3 | -3 | 11 | -3 | -3 | -3 |
| PCCPCH RSCP | dBm | -86 | -72 | -86 | -86 | -86 | n.a. |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Qrxlevmin | dBm | -103 |  |  |  |  |  |  |  |  |  |
| Qoffset1s,n | dB | C1, C2: 0 |  |  |  |  |  |  |  |  |  |
| Qhyst1s | dB | 0 |  |  |  |  |  |  |  |  |  |
| Sprioritysearch1 | dB | 24 (-79dBm) |  |  |  |  |  |  |  |  |  |
| Sprioritysearch2 | dB | 0 |  |  |  |  |  |  |  |  |  |
| Threshx, high NOTE 2 | dB | 46 (-94dBm) |  |  |  |  |  |  |  |  |  |
| SsearchE-UTRA | dB | Not send |  |  |  |  |  |  |  |  |  |
| Time offset to cell1 | ms | 3 |  |  |  |  |  |  |  |  |  |
| NOTE 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency's channel number. NOTE 2: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.2A.5-3: UTRA TDD Cell specific test parameters for
E-UTRA FDD to UTRA TDD inter-RAT cell reselection test case in AWGN

| Parameter | Unit | Cell 3 (UTRA TDD) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Timeslot Number |  | 0 |  |  |  |  | DwPTS |  |  |  |  |
|  |  | T0 | T1 | T2 | T3 | T4 | T0 | T1 | T2 | T3 | T4 |
| UTRA RF Channel Number NOTE 1 |  | Randomly selected from 4, 5, 6, 7 such that cell 3 is in the reduced performance group |  |  |  |  |  |  |  |  |  |
| PCCPCH_Ec/Ior | dB | -3 |  |  |  |  |  |  |  |  |  |
| DwPCH_Ec/Ior | dB |  |  |  |  |  | 0 |  |  |  |  |
| OCNS_Ec/Ior | dB | -3 |  |  |  |  |  |  |  |  |  |
|  | dBm/ 1.28 MHz | -80 |  |  |  |  |  |  |  |  |  |
|  | dB | -3 | -3 | -3 | 11 | -3 | -3 | -3 | -3 | 11 | -3 |
| PCCPCH RSCP | dBm | -86 | -86 | -86 | -72 | -86 | n.a. |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Qrxlevmin | dBm | -103 |  |  |  |  |  |  |  |  |  |
| Qoffset1s,n | dB | C1, C2: 0 |  |  |  |  |  |  |  |  |  |
| Qhyst1s | dB | 0 |  |  |  |  |  |  |  |  |  |
| Sprioritysearch1 | dB | 24 (-79dBm) |  |  |  |  |  |  |  |  |  |
| Sprioritysearch2 | dB | 0 |  |  |  |  |  |  |  |  |  |
| Threshx, high NOTE 2 | dB | 46 (-94dBm) |  |  |  |  |  |  |  |  |  |
| SsearchE-UTRA | dB | Not send |  |  |  |  |  |  |  |  |  |
| Time offset to cell1 | ms | 3 |  |  |  |  |  |  |  |  |  |
| Time offset to cell2 | μs | 3 |  |  |  |  |  |  |  |  |  |
| NOTE 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency's channel number. NOTE 2: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell |  |  |  |  |  |  |  |  |  |  |  |

The cell reselection delay is defined as the time from the beginning of a relevant time period, to the moment when the UE camps on the target cell, and starts to send the SYNCH-UL sequence in the UpPTS on cell 2, 3 for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on the target cell.

The reselection delays shall meet the requirements in table 4.3.2A.5-4

Table A.4.3.2A.5-4: Test requirements for E-UTRA FDD to UTRA TDD inter-RAT cell reselection

| Time phase | Target cell | Requirement for reselection delay (seconds) |
| --- | --- | --- |
| T1 | Cell 2 (normal performance group) | 58.9 |
| T3 | Cell 3 (reduced performance group) | 462.1 |

The rate of correct cell reselections observed during repeated tests shall be at least 90%, with a successful reselection counted if it is within the required time regardless of the carrier frequencies involved. At least 90% of reselections to the reduced performance group shall be within the required time, and at least 90% of reselections to the normal performance group shall be within the required time with a confidence level of 95%.

NOTE:	The cell re-selection delay to a normal performance group cell can be expressed as: NUTRA_carrier_TDD,normal *TevaluateUTRA_TDD + TSI_UTRA, and to a reduced performance group cell can be expressed as: 6 * NUTRA_carrier_TDD,reduced * TevaluateUTRA_TDD + TSI_UTRA,

Where:

TevaluateUTRA_TDD	19.2s, See Table 4.2.2.5.2-1

TSI_UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 3 * 19.2 + 1.28 = 58.88 s for normal performance group reselection and 6 * 4 * 19.2 + 1.28 = 462.08s for reduced performance group reselection, allow 58.9s for normal performance group and 462.1s for reduced performance group in the test case.

Since only one E-UTRA frequency is configured and signal level of UTRA cell is lower than threshold of Sprioritysearch, the UE shall select back to cell 1 (E-UTRA cell) within Kcarrier * TevaluateEUTRA + TSI = 19.2 + 1.28 = 20.48s.

### 4.3.3	E-UTRAN TDD - UTRAN FDD cell re-selection

#### 4.3.3.1	Test purpose

To verify that the UE is able to search and measure neighbouring UTRA FDD cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements when the UTRA is of lower priority.

#### 4.3.3.2	Test applicability

This test applies to all types of E-UTRA TDD UE release 8 and forward that support UTRA FDD.

#### 4.3.3.3	Minimum conformance requirements

When the measurement rules indicate that UTRA FDD cells are to be measured, the UE shall measure CPICH Ec/Io and CPICH RSCP of detected UTRA FDD cells in the neighbour frequency list at the minimum measurement rate. The parameter NUTRA_carrier_FDD is the number of carriers in the neighbour frequency list. The UE shall filter CPICH Ec/Io and CPICH RSCP measurements of each measured UTRA FDD cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

If the SServingCell of the E-UTRA serving cell is greater than Snonintrasearch then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in 3GPP TS 36.133 [4] clause 4.2.2 as Thigher_priority_search = (60 * Nlayers) seconds, where the parameter Nlayers is the total number of configured higher priority carrier frequencies.

If the SServingCell of the E-UTRA serving cell is less than or equal to Snonintrasearch then the UE shall search for and measure inter-RAT layers of higher or lower priority in preparation for possible re-selection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

The UE shall evaluate whether newly detectable UTRA FDD cells have met the re-selection criteria in 3GPP TS 36.304 [6] within time NUTRA_carrier_FDD * TdetectUTRA_FDD (as defined in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1) when the SServingCell of the E-UTRA serving cell is greater than Snonintrasearch when TreselectionRAT = 0 provided that the re-selection criteria is met by a margin of at least 6 dB.

Cells which have been detected shall be measured at least every NUTRA_carrier * TmeasureUTRA_FDD when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch.

When higher priority UTRA FDD cells are found by the higher priority search, they shall be measured at least every TmeasureUTRA_FDD. If re-selection to any higher priority cell is not triggered within (TevaluateUTRA_FDD + TreselectionRAT) after it is found in a higher priority search, the UE is not required to continue making measurements of the cell to evaluate the ongoing possibility of re-selection.

For a cell that has been already detected, but that has not been re-selected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA FDD cell has met re-selection criterion defined in 3GPP TS 36.304 [6] within NUTRA_carrier * TevaluateUTRA_FDD as defined in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1 when TreselectionRAT = 0.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.1 and A.4.3.3.

#### 4.3.3.4	Test description

##### 4.3.3.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.22 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.3.3.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.3.4.3.

5.	There is one E-UTRA TDD cell and one UTRA FDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.3.4.1-1: General test parameters for EUTRA TDD- 
lower priority UTRA FDD inter RAT cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 | E-UTRAN cell |
| T1 end condition | Active cells |  | Cell1 | UE shall perform reselection to cell 1 during T1 for subsequent iterations of the test |
|  | Neighbour cell |  | Cell2 |  |
| T2 end condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T2 |
|  | Neighbour cell |  | Cell1 |  |
| E-UTRA PRACH configuration |  |  | 53 | As specified in table 5.7.1-2 in 3GPP TS 36.211 |
| Uplink-downlink configuration of cell 1 |  |  | 1 | As specified in table 4.2.2 in 3GPP TS 36.211 |
| Special subframe configuration of cell 1 |  |  | 6 | As specified in table 4.2.1 in 3GPP TS 36.211 |
| E_UTRA Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | 85 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 25 | T2 need to be defined so that cell re-selection reaction time is taken into account. |

##### 4.3.3.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one UTRA FDD carrier. In the test there are two successive time periods, with time duration of T1 and T2 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 2 is of lower priority than Cell 1.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure".

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to T1 in Table 4.3.3.5-1 and 4.3.3.5-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts. If the UE is already camped in Cell 1, wait until T1 expires and skip to step 5.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

4.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 5.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, skip to step 9.

5.	The SS shall switch the power setting from T1 to T2 as specified in Table 4.3.3.5-1 and 4.3.3.5-2.

6.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

7.	If the UE responds on Cell 2 during time duration T2 within 21 seconds from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, skip to step 10.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, continue with step 9.

9.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

10.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

##### 4.3.3.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 and 3GPP TS 34.108 [24] clause 6.1.0b with the following exceptions:

Table 4.3.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-5 Table H.2.3-6 Table H.2.3-13 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-2 |

Table 4.3.3.4.3-2: System Information Block type 19: EUTRA TDD- 
lower priority UTRA FDD inter RAT cell re-selection test case

| Derivation Path: 36.508 clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 21 (42dB) |  |  |
| s-PrioritySearch2 | 0 | default value is 0 |  |
| threshServingLow | 0 (0dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 24 (48 dB) |  |  |
| threshXlow | 1 (2 dB) | Default value |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

#### 4.3.3.5	Test requirement

Tables 4.3.3.4.1-1, 4.3.3.5-1 and 4.3.3.5-2 define the primary level settings including test tolerances for E-UTRAN TDD- UTRAN FDD intra frequency cell re-selection test case which UTRA is of lower priority.

Table 4.3.3.5-1: Cell specific test parameters for Cell 1(E-UTRA TDD)

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in D.2.1 (OP.2 TDD) |  | OP.2 TDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qqualmin for UTRA neighbour cell | dB | -20 |  |  |
| Qrxlevmin for UTRA neighbour cell | dBm | -115 |  |  |
| Qrxlevmin | dBm | -140 |  |  |
|  | dBm/15 kHz | -99.35 |  |  |
| RSRP | dBm/15 KHz | -84.95 | -103.05 |  |
|  | dB | 14.40 | -3.70 |  |
|  | dB | 14.40 | -3.70 |  |
| TreselectionEUTRAN | s | 0 |  |  |
| Snonintrasearch | dB | Not sent |  |  |
| Threshserving, low | dB | 44 |  |  |
| Threshx, low (Note 2) | dB | 42 |  |  |
| Propagation Condition |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |  |  |

Table 4.3.3.5-2: Cell specific test parameters for Cell 2(Lower priority UTRA FDD)

| Parameter | Unit | Cell 2 (UTRA) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | Channel 2 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| OCNS_Ec/Ior | dB | -0.941 |  |
|  | dB | 13.80 | 13.80 |
|  | dBm/3,84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -10.18 | -10.18 |
| CPICH_RSCP | dBm | -66.20 | -66.20 |
| Propagation Condition |  | AWGN |  |
| Qqualmin | dB | -20 |  |
| Qrxlevmin | dBm | -115 |  |
| QrxlevminEUTRA | dBm | -140 |  |
| UE_TXPWR_MAX_RACH | dBm | 21 |  |
| Treselection | S | 0 |  |
| Sprioritysearch1 | dB | 42 |  |
| Sprioritysearch2 | dB | 0 |  |
| Threshx, high (Note) | dB | 48 |  |
| NOTE: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell. |  |  |  |

The cell reselection delay to lower priority is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message on cell 2.

The cell re-selection delay to lower priority test requirement in this case is expressed as:

Cell re-selection delay to lower priority = TevaluateUTRA_FDD + TSI-EUTRA

TevaluateUTRA_FDD = 19.2 s; as specified in 3GPP TS 36.133 [4] clause 4.2.2.5

TSI-EUTRA = 1280 ms; as specified in 3GPP TS 36.133 [4] clause 4.2.2.4

The cell re-selection delay to lower priority shall be less than a total of 21 seconds in this test case (note: this gives a total of 20.48 seconds but the test allows 21 seconds).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

### 4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection

#### 4.3.3A.1	Test purpose

This test is to verify the requirement for the E-UTRA TDD to UTRA FDD inter-RAT cell reselection requirements for increased UE carrier monitoring UTRA. UTRA FDD cells are of lower priority than E-UTRA serving cell.

#### 4.3.3A.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 12 and forward that support increased carrier monitoring UTRA.

#### 4.3.3A.3	Minimum conformance requirements

Same as 4.3.1.5.3.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.1, 4.2.2.9a and A.4.3.3A

#### 4.3.3A.4	Test description

##### 4.3.3A.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10 MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A figure 54 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.3.3A.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.3A.4.3.

5.	There is one E-UTRA FDD cell and two UTRA TDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.3A.4-1: General test parameters for E-UTRAN TDD- 
UTRAN FDD inter frequency cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| T0 | Active cell |  | Cell 1 | T0 is repeated on each repetition of the test. In T0 the test equipment selects frequencies for cell 2, 3 and then time is allowed for the UE to identify the neighbour cells. See cell specific parameters for detailed settings. |
| T1 start condition | Active cell |  | Cell 1 |  |
| T1 end condition | Active cell |  | Cell 2 | UE shall perform reselection to cell 2 during T1 |
|  | Neighbour cell |  | Cell 1, cell 3 |  |
| T2 end condition | Active cell |  | Cell 1 | UE shall perform reselection to cell 1 during T2 |
|  | Neighbour cell |  | Cell 2, cell 3 |  |
| T3 end condition | Active cell |  | Cell 3 | UE shall perform reselection to cell 3 during T3 |
|  | Neighbour cell |  | Cell 1, cell 2 |  |
| T4 end condition | Active cell |  | Cell 1 | UE shall perform reselection to cell 1 during T4 |
|  | Neighbour cell |  | Cell 2, cell 3 |  |
| UE configured E-UTRA RF Channel Number |  |  | 1 | Serving cell and six UTRA FDD carrier frequencies are used in the UE neighbour cell list. Frequencies 5,6 and 7 are indicated to have reduced performance |
| UE configured UTRA RF Channel Number |  |  | 2,3,4,5,6,7 |  |
| Test equipment configuration |  |  | Cell 1 uses E-UTRA RF channel number 1 Cells 2,3,4 are randomly selected to use different frequencies selected from frequencies 2,3,4,5,6,7 |  |
| PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in 3GPP TS 36.211 |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T0 |  | s | (test equipment frequency selection and configuration time) + 960 | Initialisation time need to be defined so that cell detection time is taken into account. |
| T1 |  | s | 60 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 25 | T2 need to be defined so that cell re-selection reaction time is taken into account |
| T3 |  | s | 350 | T3 need to be defined so that cell re-selection reaction time is taken into account. |
| T4 |  | s | 25 | T4 need to be defined so that cell re-selection reaction time is taken into account |

Table 4.3.3A.4-2: General test parameters for EUTRA TDD- 
UTRA FDD inter RAT cell re-selection test case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Uplink-downlink configuration of cell 1 |  | 1 | As specified in table 4.2.2 in 3GPP TS 36.211 |
| Special subframe configuration of cell 1 |  | 6 | As specified in table 4.2.1 in 3GPP TS 36.211 |

##### 4.3.3A.4.2	Test procedure

The test scenario comprises of indicating 6 UTRA FDD interfrequency cells on 6 different carriers in the neighbour list of cell 1, cells 2 and 3. Each repetition of the test consists of five successive time periods, with time duration of T0, T1, T2, T3 and T4 respectively. In the initialisation phase and at the start of each repetition of T0, the test equipment provides signals for cell 1 (serving cell), and selects frequencies for cells 2 and 3 which are chosen from the 6 inter-RAT layers which are configured in the UE neighbour cell list as described in general and cell specific parameters. The neighbour lists of cells 2 and 3 shall include the frequency of cell 1 in the normal performance group and shall exclude the other frequencies configured to the UE in the test. Cell 1, 2 and 3 are identified by the UE during time period T0. Cell 1, cell 2 and cell 3 all belong to different tracking areas.  Furthermore, UE has not registered with network for the tracking area containing cell 2 or 3. Cells 2 and 3 all have lower absolute priority than cell 1.

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to T0 in Table 4.3.1.5.5-1 and 4.3.1.5.5-2. Propagation conditions are set according to Annex B clause B.1.1. T0 starts. SS provides signals for cell 1 and selects frequencies for cells 2 and 3. If the UE is already camped in Cell 1, wait until T0 expires and skip to step 5.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1

4.	If the UE has re-selected Cell 1 within T0, after the re-selection or when T0 expires, continue with step 5.
Otherwise, if T0 expires and the UE has not yet re-selected Cell 1, skip to step 24.

5.	The SS shall switch the power setting from T0 to T1 as specified in Tables 4.3.3A.5-1, 4.3.3A.5-2 and 4.3.3A.5-3.

6.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

7.	If the UE responds on the Cell 2 during time duration T1 within 59 seconds from the beginning of time period T1, then count a success for the event "Re-select Cell 2". Otherwise count a fail for the event "Re-select Cell 2".

8. 	If the UE has re-selected Cell 2 within T1, after the re-selection or when T1 expires, skip to step 10.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 2, continue with step 9.

9. 	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

10.	The SS shall switch the power setting from T1 to T2 as specified in Tables 4.3.3A.5-1, 4.3.3A.5-2 and 4.3.3A.5-3.

11.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

12.	If the UE responds on the Cell 1 during time duration T2 within 21 seconds from the beginning of time period T2, then count a success for the event "Re-select Cell 1". Otherwise count a fail for the event "Re-select Cell 1".

13.	If the UE has re-selected Cell 1 within T2, after the re-selection or when T2 expires, skip to step 15.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 1, continue with step 14.

14. Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

15.	The SS shall switch the power setting from T2 to T3 as specified in Tables 4.3.3A.5-1, 4.3.3A.5-2 and 4.3.3A.5-3.

16.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 3.

17.	If the UE responds on the Cell 3 during time duration T3 within 347 seconds from the beginning of time period T3, then count a success for the event "Re-select Cell 3". Otherwise count a fail for the event "Re-select Cell 3".

18.	If the UE has re-selected Cell 3 within T3, after the re-selection or when T3 expires, skip to step 20.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 3, continue with step 19.

19.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

20.	The SS shall switch the power setting from T3 to T4 as specified in Tables 4.3.3A.5-1, 4.3.3A.5-2 and 4.3.3A.5-3.

21.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

22.	If the UE responds on the Cell 1 during time duration T4 within 21 seconds from the beginning of time period T4, then count a success for the event "Re-select Cell 1". Otherwise count a fail for the event "Re-select Cell 1".

23.	If the UE has re-selected Cell 1 within T4, after the re-selection or when T4 expires, skip to step 25.
Otherwise, if T4 expires and the UE has not yet re-selected Cell 1, continue with step 24.

24.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell

25.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

##### 4.3.3A.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 with the following exceptions:

Table 4.3.3A.4.3-1: Common Exception messages for E-UTRAN FDD- UTRAN FDD inter-rat event triggered reporting for increased carrier monitoring UTRA with reduced performance group configured, non DRX test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-5 Table H.2.3-6 Table H.2.3-13 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-1 |

Table 4.3.3A.4.3-2: System Information Block type 19:
Inter-RAT E-UTRAN FDD - UTRA FDD is of lower priority cell re-selection

| Derivation Path: 36.508 [7] clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 21 (42dB) |  |  |
| s-PrioritySearch2 | 0 | default value is 0 |  |
| threshServingLow | 0 (0dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 24 (48 dB) |  |  |
| threshXlow | 1 (2 dB) | Default value |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 4.3.3A.4.3-3: SystemInformationBlockType6:
Inter-RAT E-UTRAN FDD/TDD - UTRA FDD is of lower priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-5 SystemInformationBlockType6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CarrierFreqUTRA-FDD-Ext-r12 ::= SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| threshX-Low-r12 | 24 (48 dB) | 48 is actual value in dB (24 * 2 dB) |  |
| q-RxLevMin-r12 | -70 (-140 dBm) | -140 is actual value in dBm (-70 * 2 + 1 dBm) |  |
| p-MaxUTRA-r12 | 21 (21 dBm) |  |  |
| q-QualMin-r12 | -20 (-20 dB) |  |  |
| reducedMeasPerformance-r12 | TRUE for Cell 3 | If neighbouring inter-frequency cell is configured for reduced measurement performance |  |
| } |  |  |  |

#### 4.3.3A.5	Test requirement

Tables 4.3.2A.4.1-1, 4.3.3A.5-1, 4.3.3A.5-2 and 4.3.3A.5-3 defines the primary level settings including test tolerances for E-UTRAN FDD to UTRA TDD cell re-selection test case.

Table 4.3.3A.5-1: Cell specific test parameters for E-UTRAN TDD- 
UTRAN FDD inter-RAT cell reselection test case in AWGN cell 1 (E-UTRAN)

| Parameter | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel number |  | 1 |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB = 25 10MHz: NRB = 50 |  |  |  |  |
| Io | dBm/4.5MHz(25RB) dBm/9Mhz (50RB) | -59.06 -56.05 | -71.77 -68.76 | -59.06 -56.05 | -71.77 -68.76 | -59.06 -56.05 |
| PDSCH parameters: DL Reference Measurement Channel |  | OP.10 TDD(5MHz) OP.2 TDD (10MHz) |  |  |  |  |
| PBCH RA | dB | 0 |  |  |  |  |
| PBCH RB | dB |  |  |  |  |  |
| PSS RA | dB |  |  |  |  |  |
| SSS RA | dB |  |  |  |  |  |
| PCFICH RB | dB |  |  |  |  |  |
| PHICH RA | dB |  |  |  |  |  |
| PHICH RB | dB |  |  |  |  |  |
| PDCCH RA | dB |  |  |  |  |  |
| PDCCH RB | dB |  |  |  |  |  |
| PDSCH RA | dB |  |  |  |  |  |
| PDSCH RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |  |  |
| Note 2 | dBm | -98 |  |  |  |  |
| RSRP Note 3 | dBm | -84 | -101.7 | -84 | -101.7 | -84 |
|  | dB | 14 | -3.7 | 14 | -3.7 | 14 |
|  | dB | 14 | -3.7 | 14 | -3.7 | 14 |
| TreselectionEUTRAN | s | 0 |  |  |  |  |
| Snonintrasearch | dB | 62 |  |  |  |  |
| Threshserving, low | dB | 44 |  |  |  |  |
| Threshx, low (Note 4) | dB | 40 |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |  |  |  |  |

Table 4.3.3A.5-2: Cell specific test parameters for cells 2 and 3 (UTRA)

| Parameter | Unit | Cell 2 |  |  |  |  | Cell 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T2 | T3 | T4 | T0 | T1 | T2 | T3 | T4 |
| UTRA RF Channel Number |  | Randomly selected from 1, 2, 3 such that cell 2 is in the normal performance group |  |  |  |  | Randomly selected from 4, 5, 6 such that cell 3 is in the reduced performance group |  |  |  |  |
| CPICH_Ec/Ior | dB | -10 |  |  |  |  | -10 |  |  |  |  |
| PCCPCH_Ec/Ior | dB | -12 |  |  |  |  | -12 |  |  |  |  |
| SCH_Ec/Ior | dB | -12 |  |  |  |  | -12 |  |  |  |  |
| PICH_Ec/Ior | dB | -15 |  |  |  |  | -15 |  |  |  |  |
| OCNS_Ec/Ior | dB | -0.941 |  |  |  |  | 0.941 |  |  |  |  |
|  | dB | +11 | +11.8 | +11.8 | -5 | +11 | +11 | -5 | +11 | +11.8 | +11.8 |
|  | dBm/3,84 MHz | -70 |  |  |  |  | -70 |  |  |  |  |
| CPICH_Ec/Io | dB | -10.33 | -10.28 | -10.28 | -16.19 | -10.33 | -10.33 | -16.19 | -10.36 | -10.28 | -10.28 |
| CPICH_RSCP | dBm | -69 | -68.2 | -68.2 | -85 | -69 | -69 | -85 | -69 | -68.2 | -68.2 |
| Propagation Condition |  | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Qqualmin | dB | -20 |  |  |  |  | -20 |  |  |  |  |
| Qrxlevmin | dBm | -115 |  |  |  |  | -115 |  |  |  |  |
| QrxlevminEUTRA | dBm | -140 |  |  |  |  | -140 |  |  |  |  |
| UE_TXPWR_MAX_RACH | dBm | 21 |  |  |  |  | 21 |  |  |  |  |
| Treselection | s | 0 |  |  |  |  | 0 |  |  |  |  |
| Sprioritysearch1 | dB | 40 |  |  |  |  | 40 |  |  |  |  |
| Sprioritysearch2 | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| Threshx, low (Note) | dB | 50 |  |  |  |  | 50 |  |  |  |  |
| NOTE: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell. |  |  |  |  |  |  |  |  |  |  |  |

The cell reselection delay is defined as the time from the beginning of a relevant time period, to the moment when the UE camps on the target cell, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on the target cell.

The reselection delays shall meet the requirements in table 4.3.3A.5-3.

Table 4.3.3A.5-3

| Time phase | Target cell | Requirement for reselection delay (seconds) |
| --- | --- | --- |
| T0 | Cell 1 |  |
| T1 | Cell 2 (normal performance group) | 59 |
| T2 | Cell 1 (normal performance group) | 21 |
| T3 | Cell 3 (reduced performance group) | 347 |
| T4 | Cell 1 (normal performance group) | 21 |

NOTE:	The cell re-selection delay to a normal performance group cell can be expressed as: (NUTRA_carrier,normal) * TevaluateUTRA_FDD + TSI-UTRA and to a reduced performance group cell can be expressed as: 6 * NUTRA_carrier,reduced * TevaluateUTRA_FDD + TSI-UTRA.

Where:

TevaluateUTRA-FDD	See Table 4.2.2.5.1-1

TSI-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 58.88 s for normal performance group reselection, allow 59 s, and gives a total of 346.88 s for reduced performance group reselection, allow 347 s for reduced performance group in the test case. For reselections back to cell 1 since only one frequency is configured, the requirement is Tevaluate,E-UTRAN_Inter, + TSI = 20.48, allow 21s.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

### 4.3.4	E-UTRAN TDD - UTRAN TDD cell re-selection

#### 4.3.4.1	E-UTRA TDD - UTRAN TDD cell re-selection: UTRA is of higher priority

##### 4.3.4.1.1	Test purpose

To verify that the UE is able to search and measure neighbouring UTRA TDD cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements when the UTRA is of higher priority.

##### 4.3.4.1.2	Test applicability

This test applies to all types of release 8 and forward E-UTRA TDD UEs that support release 9 and forward UTRA TDD.

##### 4.3.4.1.3	Minimum conformance requirements

When the measurement rules indicate that UTRA TDD cells are to be measured, the UE shall measure P-CCPCH RSCP of detected UTRA TDD cells in the neighbour frequency list at the minimum measurement rate. The parameter NUTRA_carrier_TDD is the number of carriers in the neighbour frequency list. The UE shall filter P-CCPCH RSCP measurements of each measured UTRA TDD cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

If the SServingCell of the E-UTRA serving cell is greater than Snonintrasearch then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in 3GPP TS 36.133 [4] clause 4.2.2 as Thigher_priority_search = (60 * Nlayers) seconds, where the parameter Nlayers is the total number of configured higher priority carrier frequencies.

If the SServingCell of the E-UTRA serving cell is less than or equal to Snonintrasearch then the UE shall search for and measure inter-RAT layers of higher, or lower priority in preparation for possible re-selection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

The UE shall evaluate whether newly detectable UTRA TDD cells have met the re-selection criteria in 3GPP TS 36.304 [6] within time (NUTRA_carrier_TDD) * TdetectUTRA_TDD (as defined in table 4.2.2.5.2-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.2) when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch when TreselectionRAT = 0 provided that the reselection criteria is met by a margin of at least 6dB.

Cells which have been detected shall be measured at least every (NUTRA_carrier_TDD) * TmeasureUTRA_TDD when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch.

When higher priority UTRA TDD cells are found by the higher priority search, they shall be measured at least every TmeasureUTRA_TDD. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of re-selection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell.

For a cell that has been already detected, but that has not been re-selected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA TDD cell has met re-selection criterion defined in 3GPP TS 36.304 [6] within NUTRA_carrier_TDD * TevaluateUTRA_TDD when TreselectionRAT = 0.as specified in table 4.2.2.5.2-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.2 provided that the reselection criteria is met by a margin of at least 6dB.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.2 and A.4.3.4.1.

##### 4.3.4.1.4	Test description

4.3.4.1.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B/eNodeB emulator) and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A figure A.22 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.3.4.1.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.4.1.4.3.

5.	There is one E-UTRA TDD cell and one UTRA TDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.4.1.4.1-1: General test parameters for E-UTRAN to
UTRAN (1.28 Mcps TDD OPTION) Cell Re-selection

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell 1 | UE is on cell 1 in the initialisation phase, so that reselection to cell 2 occurs during the first T2 phase |
| T2 end condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T2 |
|  | Neighbour cell |  | Cell1 |  |
| T3 end condition | Active cell |  | Cell1 | UE shall perform reselection to cell 1 during T3 |
|  | Neighbour cell |  | Cell2 |  |
| Uplink-downlink configuration of cell 1 |  |  | 1 | As specified in table 4.2.2 in 3GPP TS 36.211 [9] |
| Special subframe configuration of cell 1 |  |  | 6 | As specified in table 4.2.1 in 3GPP TS 36.211 [9] |
| PRACH configuration of cell 1 |  |  | 53 | As specified in table 5.7.1-3 in 3GPP TS 36.211 [9] |
| CP length of cell 1 |  |  | Normal |  |
| Time offset between cells |  |  | 3 ms | Asynchronous cells 3ms or 92160*Ts |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure. |
| Treselection |  | s | 0 |  |
| DRX cycle length |  | s | 1,28 |  |
| HCS |  |  | Not used |  |
| T1 |  | s | >20 | During T1, cell 2 shall be powered off, and during the off time the primary scrambling code shall be changed, The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 |  | s | 85 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 25 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

4.3.4.1.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. E-UTRA cell 1 is already identified by the UE prior to the start of the test. Before T1 the UE is camped on to cell 1. During T1, cell 2 shall be powered off, and during the off time the cell id shall be changed. At starting T2 cell 2 becomes stronger than Threshx_high, the UE is expected to detect cell 2, send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2. At the start of T3, cell 2 becomes weaker than Threshserving_low, and the UE reselects to Cell 1.

In the following test procedure "UE responds" means "UE starts transmitting the SYNCH-UL sequence in the UpPTS for sending RRC CONNECTION REQUEST message to perform a Routing Area Update procedure".

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to duration T1 in Table 4.3.4.1.5-1and 4.3.4.1.5-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

3.	During T1, cell 2 shall be powered off and the SS shall set Cell 2 cell parameter id = (current cell 2 cell parameter id +4) mod 16.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 4.3.4.1.5-1 and 4.3.4.1.5-2. The SS waits for random access requests information from the UE to perform cell re-selection procedure on the higher priority cell, Cell 2.

5.	If the UE responds on Cell 2 within 81s from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

6.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 7.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, skip to step 10.

7.	The SS shall switch the power setting from T2 to T3 as specified in Table 4.3.4.1.5-1 and 4.3.4.1.5-2.

8.	The SS waits for random access requests information from the UE to perform cell re-selection procedure on the lower priority cell, Cell 1.

9.	If the UE has re-selected Cell 1 within T3, after the re-selection or when T3 expires, skip to step 11.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, continue with step 10.

10.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

11.	Repeat step 2-10 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

4.3.4.1.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 and 3GPP TS 34.108 [24] clause 6.1.0b with the following exceptions:

Table 4.3.4.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-3 Table H.2.3-4 Table H.2.3-14 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-2 |

Table 4.3.4.1.4.3-2: System Information Block type 19:
E-UTRA TDD- higher priority UTRA TDD inter RAT cell re-selection test case

| Derivation Path: 36.508 clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| s-PrioritySearch1 | 31 (62dB) |  |  |
| s-PrioritySearch2 | Not present | default value is 0 |  |
| threshServingLow | 12 (24dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 2 (4 dB) |  |  |
| threshXlow | 23 (46 dB) | Default value |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

##### 4.3.4.1.5	Test requirement

Tables 4.3.4.1.4-1, 4.3.4.1.5-1 and 4.3.4.1.5-2 define the primary level settings including test tolerances for cell re-selection E-UTRA TDD to UTRA TDD test case (UTRA is of higher priority).

Table 4.3.4.1.5-1: Cell specific test requirement parameters for
cell re-selection E-UTRA TDD to UTRA TDD test case (cell 1)

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in D.2.2 (OP.2 TDD) |  | OP.2 TDD |  |  |
| PBCH_RA | dB | 0 | 0 | 0 |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qrxlevmin | dBm | -140 | -140 | -140 |
|  | dBm/15kHz | -98 |  |  |
| RSRP | dBm/15kHz | -86.9 | -86.9 | -86.9 |
|  | dB | 11.1 | 11.1 | 11.1 |
| Threshx, high (Note2) | dB | 24(-79dBm) |  |  |
| Snonintrasearch | dB | 46 |  |  |
| Propagation Condition |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |  |  |

Table 4.3.4.1.5-2: Cell specific test requirement parameters for
cell re-selection E-UTRA TDD to UTRA TDD test case (cell 2)

| Parameter | Unit | Cell 2 (UTRA) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Timeslot Number |  | 0 |  |  | DwPTS |  |  |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| UTRA RF Channel Number (Note1) |  | Channel 2 |  |  |  |  |  |
| PCCPCH_Ec/Ior | dB | -3 | -3 | -3 |  |  |  |
| DwPCH_Ec/Ior | dB |  |  |  | 0 | 0 | 0 |
| OCNS_Ec/Ior | dB | -3 | -3 | -3 |  |  |  |
|  | dB | -inf | 11 | -3 | -inf | 11 | -3 |
|  | dBm/1.28 MHz | -80 |  |  |  |  |  |
| PCCPCH RSCP | dBm | -inf | -72 | -86 | n.a. |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Qrxlevmin | dBm | -103 |  |  |  |  |  |
| Qoffsets,n | dB | C1, C2: 0 |  |  |  |  |  |
| Qhysts | dB | 0 |  |  |  |  |  |
| Snonintrasearch | dB | Not sent |  |  |  |  |  |
| Threshserving, low | dB | 24 (-79dBm) |  |  |  |  |  |
| Threshx, low (Note2) | dB | 46 (-94dBm) |  |  |  |  |  |
| NOTE 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency's channel number. NOTE 2: This refers to the value of Threshx, low which is included in UTRA system information, and is a threshold for the E-UTRA target cell. |  |  |  |  |  |  |  |

The cell reselection delay to higher priority is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send the SYNCH-UL sequence in the UpPTS for sending the RRC CONNECTION REQUEST message on cell 2.

The cell re-selection delay to higher priority shall be less than 81 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE: The cell re-selection delay to higher priority cell can be expressed as: Thigher_priority_search + TevaluateUTRA_TDD + TSI_UTRA,

Where:

Thigher_priority_search 60s, 	See TS 36.133 [4] section 4.2.2.5

TevaluateUTRA_TDD 19.2s, 	See TS 36.133 [4] Table 4.2.2.5.2-1

TSI_UTRA 	Maximum repetition period of relevant system info blocks that need to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 80.48 s, allow 81 s for higher priority cell reselection in the test case.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 4.3.4.2	E-UTRAN TDD - UTRAN TDD cell re-selection: UTRA is of lower priority

##### 4.3.4.2.1	Test purpose

To verify that the UE is able to search and measure neighbouring UTRA TDD cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements when the UTRA is of lower priority.

##### 4.3.4.2.2	Test applicability

This test applies to all types of release 8 and forward E-UTRA TDD UEs that support release 9 and forward UTRA TDD.

##### 4.3.4.2.3	Minimum conformance requirements

When the measurement rules indicate that UTRA TDD cells are to be measured, the UE shall measure P-CCPCH RSCP of detected UTRA TDD cells in the neighbour frequency list at the minimum measurement rate. The parameter NUTRA_carrier_TDD is the number of carriers in the neighbour frequency list. The UE shall filter P-CCPCH RSCP measurements of each measured UTRA TDD cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

If the SServingCell of the E-UTRA serving cell is greater than Snonintrasearch then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in 3GPP TS 36.133 [4] clause 4.2.2 as Thigher_priority_search = (60 * Nlayers) seconds, where the parameter Nlayers is the total number of configured higher priority carrier frequencies.

If the SServingCell of the E-UTRA serving cell is less than or equal to Snonintrasearch then the UE shall search for and measure inter-RAT layers of higher, or lower priority in preparation for possible re-selection. In this scenario, the minimum rate at which the UE is required to search for and measure such layers is not reduced and shall be the same as that defined below for lower priority RATs.

The UE shall evaluate whether newly detectable UTRA TDD cells have met the re-selection criteria in 3GPP TS 36.304 [6] within time (NUTRA_carrier_TDD) * TdetectUTRA_TDD (as defined in table 4.2.2.5.2-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.2) when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch when TreselectionRAT = 0 provided that the reselection criteria is met by a margin of at least 6dB.

Cells which have been detected shall be measured at least every (NUTRA_carrier_TDD) * TmeasureUTRA_TDD when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch.

When higher priority UTRA TDD cells are found by the higher priority search, they shall be measured at least every TmeasureUTRA_TDD. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of re-selection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell.

For a cell that has been already detected, but that has not been re-selected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA TDD cell has met re-selection criterion defined in 3GPP TS 36.304 [6] within NUTRA_carrier_TDD * TevaluateUTRA_TDD when TreselectionRAT = 0 as specified in table 4.2.2.5.2-1of 3GPP TS 36.133 [4] provided that the reselection criteria is met by a margin of at least 6dB.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.2 and A.4.3.4.2.

##### 4.3.4.2.4	Test description

4.3.4.2.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B/eNodeB emulator) and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A figure A.22 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.3.4.2.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.4.2.4.3.

5.	There is one E-UTRA TDD cell and one UTRA TDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.4.2.4.1-1: General test parameters for E-UTRAN to
UTRAN (1.28 Mcps TDD OPTION) Cell Re-selection

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell 1 | E-UTRAN TDD Cell |
| T1 end condition | Active cell |  | Cell1 | UE shall perform reselection to cell 1 during T1 for subsequent iterations of the test. |
|  | Neighbour cell |  | Cell2 | 1.28 Mcps TDD OPTION cell |
| T2 end condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T2 |
|  | Neighbour cell |  | Cell1 | E-UTRA TDD cell |
| Uplink-downlink configuration of cell 1 |  |  | 1 | As specified in table 4.2.2 in 3GPP TS 36.211 [9] |
| Special subframe configuration of cell 1 |  |  | 6 | As specified in table 4.2.1 in 3GPP TS 36.211 [9] |
| PRACH configuration of cell 1 |  |  | 53 | As specified in table 5.7.1-3 in 3GPP TS 36.211 [9] |
| CP length of cell 1 |  |  | Normal |  |
| Time offset between cells |  |  | 3 ms | Asynchronous cells 3ms or 92160*Ts |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure. |
| Treselection |  | s | 0 |  |
| DRX cycle length |  | s | 1,28 |  |
| HCS |  |  | Not used |  |
| T1 |  | s | 85 |  |
| T2 |  | s | 25 |  |

4.3.4.2.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one UTRA TDD carrier. In the test there are two successive time periods, with time duration of T1 and T2 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. At T1 the UE is camped on to Cell 1. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2.

In the following test procedure "UE responds" means "UE starts transmitting the SYNCH-UL sequence in the UpPTS for sending RRC CONNECTION REQUEST message to perform a Routing Area Update procedure".

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to T1 in Table 4.3.4.2.5-1 and 4.3.4.2.5-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts. If the UE is already camped in Cell 1, wait until T1 expires and skip to step 5.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

4.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 5.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, skip to step 9.

5.	The SS shall switch the power setting from T1 to T2 as specified in Table 4.3.4.2.5-1 and 4.3.4.2.5-2.

6.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

7.	If the UE responds on Cell 2 during time duration T2 within 21 seconds from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, skip to step 10.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, continue with step 9.

9.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

10.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

4.3.4.2.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 and 3GPP TS 34.108 [24] clause 6.1.0b with the following exceptions:

Table 4.3.4.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-7 Table H.2.3-8 Table H.2.3-14 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-2 |

Table 4.3.4.2.4.3-2: System Information Block type 19: E-UTRA FDD- lower priority UTRA TDD inter RAT cell re-selection test case

| Derivation Path: 36.508 clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 0 (0dB) |  |  |
| s-PrioritySearch2 | Not present | default value is 0 |  |
| threshServingLow | 0 (0dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 23 (46 dB) |  |  |
| threshXlow | 1 (2 dB) | Default value |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

##### 4.3.4.2.5	Test requirement

Tables 4.3.4.2.4.1-1, 4.3.4.2.5-1 and 4.3.4.2.5-2 define the primary level settings including test tolerances for E-UTRAN TDD- UTRAN TDD intra frequency cell re-selection test case which UTRA is of lower priority.

Table 4.3.4.2.5-1: Cell specific test requirement parameters for
cell re-selection E-UTRA TDD to UTRA TDD test case (cell 1)

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in D.2.2 (OP.2 TDD) |  | OP.2 TDD |  |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RA(Note1) | dB |  |  |
| OCNG_RB(Note1) | dB |  |  |
| Qrxlevmin | dBm | -140 | -140 |
|  | dBm/15kHz | -98 |  |
| RSRP | dBm/15kHz | -86.9 | -101.1 |
|  | dB | 11.1 | -3.1 |
| Snonintrasearch | dB | Not sent |  |
| Threshserving, low | dB | 46 (-94dBm) |  |
| Threshx, low (Note2) | dB | 24 (-79dBm) |  |
| Propagation Condition |  | AWGN |  |
| NOTE 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA target cell |  |  |  |

Table 4.3.4.2.5-2: Cell specific test requirement parameters for
cell re-selection E-UTRA TDD to UTRA TDD test case (cell 2)

| Parameter | Unit | Cell 2 (UTRA) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Timeslot Number |  | 0 |  | DwPTS |  |
|  |  | T1 | T2 | T1 | T2 |
| UTRA RF Channel Number (Note1) |  | Channel 2 |  |  |  |
| PCCPCH_Ec/Ior | dB | -3 | -3 |  |  |
| DwPCH_Ec/Ior | dB |  |  | 0 | 0 |
| OCNS_Ec/Ior | dB | -3 | -3 |  |  |
|  | dB | 11 | 11 | 11 | 11 |
|  | dBm/1.28 MHz | -80 |  |  |  |
| PCCPCH RSCP | dBm | -72 | -72 | n.a. | n.a. |
| Propagation Condition |  | AWGN |  |  |  |
| Qrxlevmin | dBm | -103 |  |  |  |
| Qoffsets,n | dB | C1, C2: 0 |  |  |  |
| Qhysts | dB | 0 |  |  |  |
| Threshx, high (Note2) | dB | 46 (-94dBm) |  |  |  |
| NOTE 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency's channel number. NOTE 2: This refers to the value of Threshx, high which is included in UTRA system information and is a threshold for the E-UTRA target cell. |  |  |  |  |  |

The cell re-selection delay to lower priority is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2 and starts to send the SYNCH-UL sequence in the UpPTS for sending the RRC CONNECTION REQUEST message to perform a Routing Area Update procedure on Cell 2.

The cell re-selection delay to lower priority shall be less than 21 s.

NOTE:	The cell re-selection delay to lower priority cell can be expressed as: TevaluateUTRA_TDD + TSI_UTRA,

Where:

TevaluateUTRA_TDD	19.2s, See Table 4.2.2.5.2-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1

TSI_UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 20.48 s, allow 21 s for lower priority cell reselection in the test case. For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority

##### 4.3.4.3.1	Test purpose

To verify that the UE is able to search and measure neighbouring UTRA TDD cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements when the UTRA is of lower priority. The E-UTRA cell is in fading propagation conditions and the UTRA cell is in AWGN propagation conditions.

##### 4.3.4.3.2	Test applicability

This test applies to all types of release 8 and forward E-UTRA TDD UEs that support release 9 and forward UTRA TDD.

##### 4.3.4.3.3	Minimum conformance requirements

When the measurement rules indicate that UTRA TDD cells are to be measured, the UE shall measure P-CCPCH RSCP of detected UTRA TDD cells in the neighbour frequency list at the minimum measurement rate specified in this section. The parameter NUTRA_carrier_TDD is the number of carriers in the neighbour frequency list. The UE shall filter P-CCPCH RSCP measurements of each measured UTRA TDD cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period. P-CCPCH RSCP of UTRAN TDD cells shall not be filtered over a longer period than that specified in table 4.3.4.3.3-1.

The UE shall evaluate whether newly detectable UTRA TDD cells have met the reselection criteria in 3GPP TS 36.304 [6] within time (NUTRA_carrier_TDD) * TdetectUTRA_TDD when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB.

Cells which have been detected shall be measured at least every (NUTRA_carrier_TDD) * TmeasureUTRA_TDD when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch.

When higher priority UTRA TDD cells are found by the higher priority search, they shall be measured at least every Tmeasure,UTRA_TDD. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell.

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA TDD cell has met reselection criterion defined in 3GPP TS 36.304 [6] within NUTRA_carrier_TDD *TevaluateUTRA_TDD when Treselection = 0 as specified in table 4.3.4.3.3-1 provided that the reselection criteria is met by a margin of at least 6dB.

Table 4.3.4.3.3-1: TdetectUTRA_TDD, TmeasureUTRA_TDD and TevaluateUTRA_TDD

| DRX cycle length [s] | TdetectUTRA_TDD [s] | TmeasureUTRA_TDD [s] (number of DRX cycles) | TevaluateUTRA_TDD [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 30 | 5.12 (16) | 15.36 (48) |
| 0.64 |  | 5.12 (8) | 15.36 (24) |
| 1.28 |  | 6.4(5) | 19.2 (15) |
| 2.56 | 60 | 7.68 (3) | 23.04 (9) |

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.2 and A.4.3.4.3.

##### 4.3.4.3.4	Test description

4.3.4.3.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and faders and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A figure A.26 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.3.4.3.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.4.3.4.3.

5.	There is one E-UTRA TDD cell and one UTRA TDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.4.3.4.1-1: General test parameters for EUTRA TDD- 
lower priority UTRA TDD inter RAT cell re-selection test case in fading conditions

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 | E-UTRAN cell |
| T1 end condition | Active cells |  | Cell1 | UE shall perform reselection to cell 1 during T1 for subsequent iterations of the test |
|  | Neighbour cell |  | Cell2 |  |
| T3 end condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T3 |
|  | Neighbour cell |  | Cell1 |  |
| E-UTRA PRACH configuration |  |  | 53 | As specified in table 5.7.1-3 in 3GPP TS 36.211 [9] |
| Uplink-downlink configuration of cell 1 |  |  | 1 | As specified in table 4.2.2 in 3GPP TS 36.211 [9] |
| Special subframe configuration of cell 1 |  |  | 6 | As specified in table 4.2.1 in 3GPP TS 36.211 [9] |
| E_UTRA Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | <85 | T1 need to be defined so that cell re-selection reaction time is taken into account. T1 is terminated when the UE starts to send preambles to cell 1 |
| T2 |  | s | 64 | The start of T2 is defined as the time when the UE starts to send PRACH preambles to cell 1 |
| T3 |  | s | <25 | T3 need to be defined so that cell re-selection reaction time is taken into account. T3 is terminated when the UE starts to send PRACH preambles to cell 2 |
| T4 |  | s | 64 | The start of T4 is defined as the time when the UE starts to send PRACH preambles to cell 2 |

4.3.4.3.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one UTRA TDD carrier. In the test there are four successive time periods, with time duration of T1, T2, T3 and T4 respectively. Both E-UTRA Cell 1 and UTRA Cell 2 are already identified by the UE prior to the start of the test. At T1 the UE is camped on to Cell 1. Cell 2 is of lower priority than Cell 1. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2. Time duration T2 and T4 are not used for cell re-selection in the test.

In the following test procedure, "UE responds on Cell 1" means "UE starts transmitting preamble on PRACH for sending RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure according to 3GPP TS 36.508 [7] clause 4.5A.2" and "UE responds on Cell 2" means "UE starts transmitting the SYNCH-UL sequence in the UpPTS for sending RRC CONNECTION REQUEST message to perform a Routing Area Update procedure".

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to duration T0 in Table 4.3.4.3.5-1 and 4.3.4.3.5-2.

3.	Set the parameters according to T2 in Table 4.3.4.3.5-3 and 4.3.4.3.5-4. Propagation conditions are set according to Annex B clause B.1.1 and B.2.2. T2 starts.

4.	The SS monitors for possible random access request information from the UE to perform cell re-selection on Cell 2.

5.	If the UE responds on Cell 2 within T2, then count a fail for the event "Hold out on Cell 1" and skip to step 10.
Otherwise, count a success for the event "Hold out on Cell 1" and after T2 expires continue with step 6.

6.	The SS shall switch the power setting from T2 to T3 as specified in Table 4.3.4.3.5-3 and 4.3.4.3.5-4.

7.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

8.	If the UE responds on Cell 2 during time duration T3 within 21 seconds from the beginning of time period T3, then count a success for the event "Re-select lower priority Cell 2".
Otherwise, count a fail for the event "Re-select lower priority Cell 2".

9.	If the UE responds on Cell 2 within T3, at the moment of the request-reception continue with step 10.
Otherwise, after T3 expires skip to step 17.

10.	The SS shall switch the power setting from T3 to T4 as specified in Table 4.3.4.3.5-3 and 4.3.4.3.5-4.

11.	The SS monitors for possible random access request information from the UE to perform cell re-selection on Cell 1.

12.	If the UE responds on Cell 1 within T4, then count a fail for the event "Hold out on Cell 2" and skip to step 17.
Otherwise, count a success for the event "Hold out on Cell 2" and after T4 expires continue with step 13.

13.	The SS shall switch the power setting from T4 to T1 as specified in Table 4.3.4.3.5-3 and 4.3.4.3.5-4.

14.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

15.	If the UE responds on Cell 1 within T1, at the moment of the request-reception skip to step 17.
Otherwise, after T1 expires continue with step 16.

16.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.
Set the parameters according to duration T0 in Table 4.3.4.3.5-1 and 4.3.4.3.5-2.

17.	Repeat step 3-16 until a test verdict has been achieved.
Each of the events "Hold out on Cell 1", "Re-select lower priority Cell 2" and "Hold out on Cell 2" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

4.3.4.3.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 and 3GPP TS 34.108 [24] clause 6.1.0b with the following exceptions:

Table 4.3.4.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-5 Table H.2.3-8 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-2 |

Table 4.3.4.3.4.3-2: System Information Block type 3 (1.28 Mcps TDD):
Inter-RAT E-UTRA TDD - UTRA TDD is of lower priority cell re-selection

| Derivation Path: 34.108 clause 6.1.0b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - SIB4 Indicator | TRUE |  |  |
| - Cell identity | 0000 0000 0000 0000 0000 0000 0001B |  |  |
| - Cell selection and re-selection info |  |  |  |
| - Mapping info | Not present |  |  |
| - Cell selection and reselection quality measure | (no data) |  |  |
| - CHOICE mode | TDD |  |  |
| - Sintrasearch | 10 dB |  |  |
| - Sintersearch | 10 dB |  |  |
| - SsearchHCS | Not present |  |  |
| - RAT List | Not present |  |  |
| - Qrxlevmin | -103 dBm |  |  |
| - Qhyst1s | 0 dB |  |  |
| - Treselections | 0 seconds |  |  |
| - HCS Serving cell information | Not present |  |  |
| - Maximum allowed UL TX power | 21dBm |  |  |

Table 4.3.4.3.4.3-3: System Information Block type 19:
Inter-RAT E-UTRA TDD - UTRA TDD is of lower priority cell re-selection

| Derivation Path: 36.508 clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 0 (0dB) |  |  |
| s-PrioritySearch2 | Not present | default value is 0 |  |
| threshServingLow | 0 (0dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 22 (44 dB) |  |  |
| threshXlow | 1 (2 dB) | Default value |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

##### 4.3.4.3.5	Test requirement

Tables 4.3.4.3.4.1-1, 4.3.4.3.5-1, 4.3.4.3.5-2, 4.3.4.3.5-3 and 4.3.4.3.5-4 define the primary level settings including test tolerances for E-UTRA TDD- UTRA TDD inter-RAT cell re-selection test case which UTRA is of lower priority.

Table 4.3.4.3.5-1: Cell specific test parameters for Cell 1 (E-UTRA TDD)

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
|  |  | T0 |
| E-UTRA RF Channel number |  | 1 |
| BWchannel | MHz | 10 |
| Antenna Configuration |  | 1x2 |
| OCNG Patterns defined in D.2.2 (OP.2 TDD) |  | OP.2 TDD |
| PSS_RA | dB | 0 |
| SSS_RA | dB | 0 |
| PCFICH_RB | dB | 0 |
| PHICH_RA | dB | 0 |
| PHICH_RB | dB | 0 |
| PDCCH_RA | dB | 0 |
| PDCCH_RB | dB | 0 |
| PDSCH_RA | dB | 0 |
| PDSCH_RB | dB | 0 |
| OCNG_RANote 1 | dB | 0 |
| OCNG_RBNote 1 | dB | 0 |
| Qrxlevmin for UTRA neighbour cell | dBm | -103 |
| Qrxlevmin | dBm | -140 |
|  | dBm/15 kHz | -104 |
| RSRP | dBm/15 kHz | -82 |
|  | dB | 22 |
|  | dB | 22 |
| TreselectionEUTRAN | s | 0 |
| Snonintrasearch | dB | Not sent |
| Threshserving, low | dB | 44 |
| Threshx, low (Note 2) | dB | 24 |
| Propagation Condition |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |

Table 4.3.4.3.5-2: Cell specific test parameters for Cell 2 (UTRA TDD)

| Parameter | Unit | Cell 2 (UTRA) |
| --- | --- | --- |
|  |  | T0 |
| Timeslot Number |  | 0 |
| UTRA RF Channel Number (Note 1) |  | Channel 2 |
| PCCPCH_Ec/Ior | dB | -3 |
| DwPCH_Ec/Ior | dB |  |
| OCNS_Ec/Ior | dB | -3 |
|  | dB | 13 |
|  | dBm/1.28 MHz | -80 |
| PCCPCH RSCP | dBm | -70 |
| Propagation Condition |  | AWGN |
| Qrxlevmin | dBm | -103 |
| QrxlevminEUTRA | dBm | -140 |
| UE_TXPWR_MAX_RACH | dBm | 21 |
| Treselection | s | 0 |
| Threshx, high (Note 2) | dB | 44 |
| NOTE 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency's channel number. NOTE 2: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell |  |  |

Table 4.3.4.3.5-3: Cell specific test parameters for cell 1 (E-UTRA)

| Parameter | Unit | Cell 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel number |  | 1 |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| OCNG Patterns defined in D.2.2 (OP.2 TDD) |  | OP.2 TDD |  |  |  |
| PSS_RA | dB | 0 |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Qrxlevmin for UTRA neighbour cell | dBm | -103 |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |  |
|  | dBm/15 kHz | -104 |  |  |  |
| RSRP | dBm/15 KHz | -82 | -82 | -107 | -107 |
|  | dB | 22 | 22 | -3 | -3 |
|  | dB | 22 | 22 | -3 | -3 |
| TreselectionEUTRAN | s | 0 |  |  |  |
| Snonintrasearch | dB | Not sent |  |  |  |
| Threshserving, low | dB | 44 |  |  |  |
| Threshx, low (Note 2) | dB | 24 |  |  |  |
| Propagation Condition |  | ETU70 |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |  |  |  |

Table 4.3.4.3.5-4: Cell specific test parameters for cell 2 (UTRA)

| Parameter | Unit | Cell 2 (UTRA) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Timeslot Number |  | 0 |  |  |  |  |  |  | DwPTS |  |  |  |
|  |  | T1 |  | T2 |  | T3 |  | T4 | T1 | T2 | T3 | T4 |
| UTRA RF Channel Number (Note1) |  | Channel 2 |  |  |  |  |  |  |  |  |  |  |
| PCCPCH_Ec/Ior | dB | -3 |  |  |  |  |  |  |  |  |  |  |
| DwPCH_Ec/Ior | dB |  |  |  |  |  |  |  | 0 |  |  |  |
| OCNS_Ec/Ior | dB | -3 |  |  |  |  |  |  |  |  |  |  |
|  | dB | 13 |  | 13 |  | 13 |  | 13 | 13 | 13 | 13 | 13 |
|  | dBm/1.28 MHz | -80 |  |  |  |  |  |  |  |  |  |  |
| PCCPCH RSCP | dBm | -70 | -70 |  | -70 |  | -70 |  | n.a. | n.a. | n.a. | n.a. |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |  |
| Qrxlevmin | dBm | -103 |  |  |  |  |  |  |  |  |  |  |
| QrxlevminEUTRA | dBm | -140 |  |  |  |  |  |  |  |  |  |  |
| UE_TXPWR_MAX_RACH | dBm | 21 |  |  |  |  |  |  |  |  |  |  |
| Treselection | s | 0 |  |  |  |  |  |  |  |  |  |  |
| Threshx, high (Note2) | dB | 44 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency's channel number. NOTE 2: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell |  |  |  |  |  |  |  |  |  |  |  |  |

The probability of reselection from cell 1 to cell 2 during T2 observed during testing shall be less than 10%

The probability of reselection from cell 2 to cell 1 during T4 observed during testing shall be less than 10%

The cell reselection delay to lower priority is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 2, and starts to send the SYNCH-UL sequence in the UpPTS for sending the RRC CONNECTION REQUEST message on cell 2. In order to evaluate reselection delay, the system simulator first needs to verify that the UE is camped on cell 1 at the start of T3

The cell re-selection delay to lower priority cell can be expressed as:

Cell re-selection delay to lower priority = TevaluateUTRA_TDD + TSI-UTRA

TevaluateUTRA_TDD	19.2s, as specified in 3GPP TS 36.133 [4] Table 4.2.2.5.2-1

TSI-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

The cell re-selection delay to lower priority shall be less than a total of 21 seconds in this test case (note: this gives a total of 20.48 seconds but the test allows 21 seconds).

For the test to pass, all the events above shall pass.

The statistical pass/ fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

#### 4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon

##### 4.3.4.4.1	Test purpose

This test is to verify the requirement for the E-UTRA TDD to UTRA TDD inter-RAT cell reselection requirements for increased UE carrier monitoring. UTRA TDD cells are of lower priority than E-UTRA serving cell.

##### 4.3.4.4.2	Test applicability

This test applies to all types of E-UTRA TDD UE release 12 and forward that support increased carrier monitoring UTRA.

##### 4.3.4.4.3	Minimum conformance requirements

Same as 4.3.2A.3.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.1, 4.2.2.9a and A.4.3.4.4

##### 4.3.4.4.4	Test description

4.3.4.4.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10 MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A figure 54 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.3.4.4.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.2A.4.2.3.

5.	There is one E-UTRA TDD cell and two UTRA TDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.4.4.4.1-1: General test parameters for E-UTRA TDD to
UTRA TDD inter-RAT cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| T0 | Active cell |  | Cell1 | T0 is repeated on each repetition of the test. In T0 the test equipment selects frequencies for cell 2, 3 and then time is allowed for the UE to identify the neighbour cells. See cell specific parameters for detailed settings. |
| T1 start condition | Active cell |  | Cell 1 |  |
| T1 end condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T1 |
|  | Neighbour cell |  | Cell1 |  |
| T2 end condition | Active cell |  | Cell1 | UE shall perform reselection to cell 1 during T2 |
| T3 end condition | Active cell |  | Cell3 | UE shall perform reselection to cell 3 during T3 |
|  | Neighbour cell |  | Cell1 |  |
| T4 end condition | Active cell |  | Cell1 | UE shall perform reselection to cell 1 in T4 so that next repetition of test can start from T0 |
| UE configured UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5, 6, 7 | Seven UTRA TDD carrier frequencies are used in the UE neighbour cell list. Frequencies 4, 5, 6, and 7 are indicated to have reduced performance |
| Test equipment configuration |  |  | Cell 1, 2, 3 | Cell 1 uses E-UTRA RF channel number 1 Cells 2 is randomly selected to use different frequencies selected from UTRA frequencies 1, 2, 3. Cells 3 is randomly selected to use different frequencies selected from UTRA frequencies 4, 5, 6, 7. |
| Uplink-downlink configuration of cell 1 |  |  | 1 | As specified in table 4.2.2 in 3GPP TS 36.211 [16] |
| Special subframe configuration of cell 1 |  |  | 6 | As specified in table 4.2.1 in 3GPP TS 36.211 [16] |
| CP length of cell 1 |  |  | normal |  |
| PRACH configuration |  |  | 53 | As specified in table 5.7.1-3 in 3GPP TS 36.211 [16] |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Treselection |  | s | 0 |  |
| HCS |  |  | Not used |  |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T0 |  | s | (Test equipment frequency selection and configuration time) + 960 | T0 is defined so that the Test equipment selects frequencies and configures the cells, then the UE cell detection time is taken into account. |
| T1 |  | s | 60 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 25 | T2 need to be defined so that cell re-selection reaction time is taken into account |
| T3 |  | s | 500 | T3 need to be defined so that cell re-selection reaction time is taken into account. |
| T4 |  | s | 25 | T4 need to be defined so that cell re-selection reaction time is taken into account |

4.3.4.4.4.2	Test procedure

The test scenario comprises of indicating 7 UTRA TDD inter-RAT cells on 7 different carriers in the neighbour list of cell 1 as given in tables  4.3.4.4.4.1-1, 4.3.4.4.5-1, 4.3.4.4.5-2 and 4.3.4.4.5-3.. Each repetition of the test consists of five successive time periods, with time duration of T0, T1, T2, T3, and T4 respectively. In the initialisation phase and at the start of each repetition of T0, the test equipment provides signals for cell 1 (E-UTRA serving cell), and selects frequencies for cells 2 and 3 which are chosen from the 7 inter-RAT layers which are configured in the UE neighbour cell list as described in general and cell specific parameters. The neighbour lists of cells 2 and 3 shall include the frequency of cell 1 in the normal performance group as well as the other UTRA TDD frequencies configured to the UE in the test.

Cell 1, 2, and 3 are identified by the UE during time phase T0. Cell 1, cell 2, and cell 3 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2 or 3.

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to T0 in Tables 4.3.4.4.5-1, 4.3.4.4.5-2 and 4.3.4.4.5-3. Propagation conditions are set according to Annex B clause B.1.1. T0 starts. SS provides signals for cell 1 and selects frequencies for cells 2 and 3. If the UE is already camped in Cell 1, wait until T0 expires and skip to step 5.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1

4.	If the UE has re-selected Cell 1 within T0, after the re-selection or when T0 expires, continue with step 5.
Otherwise, if T0 expires and the UE has not yet re-selected Cell 1, skip to step 24.

5.	The SS shall switch the power setting from T0 to T1 as specified in Tables 4.3.4.4.5-1, 4.3.4.4.5-2 and 4.3.4.4.5-3.

6.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

7.	If the UE responds on the Cell 2 during time duration T1 within 58.9 seconds from the beginning of time period T1, then count a success for the event "Re-select Cell 2". Otherwise count a fail for the event "Re-select Cell 2".

8. 	If the UE has re-selected Cell 2 within T1, after the re-selection or when T1 expires, skip to step 10.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 2, continue with step 9.

9. 	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

10.	The SS shall switch the power setting from T1 to T2 as specified in Table 4.3.4.4.5-1, 4.3.4.4.5-2 and 4.3.4.4.5-3.

11.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

12.	If the UE responds on the Cell 1 during time duration T2 within 20.5 seconds from the beginning of time period T2, then count a success for the event "Re-select Cell 1". Otherwise count a fail for the event "Re-select Cell 1".

13.	If the UE has re-selected Cell 1 within T2, after the re-selection or when T2 expires, skip to step 15.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 1, continue with step 14.

14.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

15.	The SS shall switch the power setting from T2 to T3 as specified in Tables 4.3.4.4.5-1, 4.3.4.4.5-2 and 4.3.4.4.5-3.

16.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 3.

17.	If the UE responds on the Cell 3 during time duration T3 within 462.1 seconds from the beginning of time period T3, then count a success for the event "Re-select Cell 3". Otherwise count a fail for the event "Re-select Cell 3".

18.	If the UE has re-selected Cell 3 within T3, after the re-selection or when T3 expires, skip to step 20.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 3, continue with step 19.

19.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

20.	The SS shall switch the power setting from T3 to T4 as specified in Tables 4.3.4.4.5-1, 4.3.4.4.5-2 and 4.3.4.4.5-3.

21.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

22.	If the UE responds on the Cell 1 during time duration T4 within 20.5 seconds from the beginning of time period T4, then count a success for the event "Re-select Cell 1". Otherwise count a fail for the event "Re-select Cell 1".

23.	If the UE has re-selected Cell 1 within T4, after the re-selection or when T4 expires, skip to step 25.
Otherwise, if T4 expires and the UE has not yet re-selected Cell 1, continue with step 24.

24.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell

25.	Repeat step 2-24 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

4.3.4.4.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 with the following exceptions:

Table 4.3.4.4.4.3-1: Common Exception messages for E-UTRAN FDD- UTRAN FDD inter-rat event triggered reporting for increased carrier monitoring UTRA with reduced performance group configured, non DRX test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-7 Table H.2.3-6 Table H.2.3-13 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-1 |

Table 4.3.4.4.4.3-2: System Information Block type 19:
Inter-RAT E-UTRAN TDD - UTRA TDD is of lower priority cell re-selection

| Derivation Path: 36.508 [7] clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 12 (24dB) |  |  |
| s-PrioritySearch2 | 0 | default value is 0 |  |
| threshServingLow | 46 |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 46 |  |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 4.3.4.4.4.3-3: SystemInformationBlockType6:
Inter-RAT E-UTRAN TDD - UTRA TDD is of lower priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-5 SystemInformationBlockType6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CarrierFreqUTRA-FDD-Ext-r12 ::= SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| threshX-Low-r12 | 24 |  |  |
| q-RxLevMin-r12 | -51 (-103 dBm) | -103 is actual value in dBm (-70 * 2 + 1 dBm) |  |
| p-MaxUTRA-r12 | 21 (21 dBm) |  |  |
| reducedMeasPerformance-r12 | TRUE for Cell 3 | If neighbouring inter-frequency cell is configured for reduced measurement performance |  |
| } |  |  |  |

Table 4.3.4.4.4.3-4: SystemInformationBlockType3:
Inter-RAT E-UTRAN TDD - UTRA FDD is of lower priority cell re-selection

| Derivation Path: 3GPP TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 46 |  |  |

##### 4.3.4.4.5	Test requirement

Tables 4.3.4.4.4.1-1, 4.3.4.4.5-1, 4.3.4.4.5-2 and 4.3.4.4.5-3 defines the primary level settings including test tolerances for E-UTRAN TDD to UTRA TDD cell re-selection test case.

Table 4.3.4.4.5-1: E-UTRA Cell specific test parameters for
E-UTRA TDD to UTRA TDD inter-RAT cell reselection test case in AWGN

| Parameter | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel number |  | 1 |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB = 25 10MHz: NRB = 50 |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RA Note 1 | dB |  |  |  |  |  |
| OCNG_RB Note 1 | dB |  |  |  |  |  |
| Note 2 | dBm/15kHz | -98 |  |  |  |  |
|  | dB | 11.05 | -3.05 | 11.05 | -3.05 | 11.05 |
| Note 3 | dB | 11.05 | -3.05 | 11.05 | -3.05 | 11.05 |
| RSRP Note 3 | dBm/15kHz | -86.95 | -101.05 | -86.95 | -101.05 | -86.95 |
| Qrxlevmin | dBm/15kHz | -140 |  |  |  |  |
| Snonintrasearch | dB | Not sent |  |  |  |  |
| Threshserving, low | dB | 46 (-94dBm) |  |  |  |  |
| Threshx, low Note 4 | dB | 24 (-79dBm) |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Es/Iot and RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA TDD target cell. |  |  |  |  |  |  |

Table 4.3.4.4.5-2: UTRA TDD Cell specific test parameters for
E-UTRA TDD to UTRA TDD inter-RAT cell reselection test case in AWGN

| Parameter | Unit | Cell 2 (UTRA TDD) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Timeslot Number |  | 0 |  |  |  |  | DwPTS |  |  |  |  |
|  |  | T0 | T1 | T2 | T3 | T4 | T0 | T1 | T2 | T3 | T4 |
| UTRA RF Channel Number Note1 |  | Randomly selected from 1, 2, 3 such that cell 2 is in the normal performance group |  |  |  |  |  |  |  |  |  |
| PCCPCH_Ec/Ior | dB | -3 |  |  |  |  |  |  |  |  |  |
| DwPCH_Ec/Ior | dB |  |  |  |  |  | 0 |  |  |  |  |
| OCNS_Ec/Ior | dB | -3 |  |  |  |  |  |  |  |  |  |
|  | dBm/ 1.28 MHz | -80 |  |  |  |  |  |  |  |  |  |
|  | dB | -3 | 11 | -3 | -3 | -3 | -3 | 11 | -3 | -3 | -3 |
| PCCPCH RSCP | dBm | -86 | -72 | -86 | -86 | -86 | n.a. |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Qrxlevmin | dBm | -103 |  |  |  |  |  |  |  |  |  |
| Qoffset1s,n | dB | C1, C2: 0 |  |  |  |  |  |  |  |  |  |
| Qhyst1s | dB | 0 |  |  |  |  |  |  |  |  |  |
| Sprioritysearch1 | dB | 24 (-79dBm) |  |  |  |  |  |  |  |  |  |
| Sprioritysearch2 | dB | 0 |  |  |  |  |  |  |  |  |  |
| Threshx, low Note2 | dB | 46 (-94dBm) |  |  |  |  |  |  |  |  |  |
| SsearchE-UTRA | dB | Not send |  |  |  |  |  |  |  |  |  |
| Time offset to cell1 | ms | 3 |  |  |  |  |  |  |  |  |  |
| NOTE 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency's channel number. NOTE 2: This refers to the value of Threshx, low which is included in UTRA system information, and is a threshold for the E-UTRA target cell |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.4.4.5-3

| Parameter | Unit | Cell 3 (UTRA TDD) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Timeslot Number |  | 0 |  |  |  |  | DwPTS |  |  |  |  |
|  |  | T0 | T1 | T2 | T3 | T4 | T0 | T1 | T2 | T3 | T4 |
| UTRA RF Channel Number Note1 |  | Randomly selected from 4, 5, 6, 7 such that cell 3 is in the reduced performance group |  |  |  |  |  |  |  |  |  |
| PCCPCH_Ec/Ior | dB | -3 |  |  |  |  |  |  |  |  |  |
| DwPCH_Ec/Ior | dB |  |  |  |  |  | 0 |  |  |  |  |
| OCNS_Ec/Ior | dB | -3 |  |  |  |  |  |  |  |  |  |
|  | dBm/ 1.28 MHz | -80 |  |  |  |  |  |  |  |  |  |
|  | dB | -3 | -3 | -3 | 11 | -3 | -3 | -3 | -3 | 11 | -3 |
| PCCPCH RSCP | dBm | -86 | -86 | -86 | -72 | -86 | n.a. |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Qrxlevmin | dBm | -103 |  |  |  |  |  |  |  |  |  |
| Qoffset1s,n | dB | C1, C2: 0 |  |  |  |  |  |  |  |  |  |
| Qhyst1s | dB | 0 |  |  |  |  |  |  |  |  |  |
| Sprioritysearch1 | dB | 24 (-79dBm) |  |  |  |  |  |  |  |  |  |
| Sprioritysearch2 | dB | 0 |  |  |  |  |  |  |  |  |  |
| Threshx, high Note2 | dB | 46 (-94dBm) |  |  |  |  |  |  |  |  |  |
| SsearchE-UTRA | dB | Not send |  |  |  |  |  |  |  |  |  |
| Time offset to cell1 | ms | 3 |  |  |  |  |  |  |  |  |  |
| Time offset to cell2 | μs | 3 |  |  |  |  |  |  |  |  |  |
| NOTE 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency's channel number. NOTE 2: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell. |  |  |  |  |  |  |  |  |  |  |  |

The cell reselection delay is defined as the time from the beginning of a relevant time period, to the moment when the UE camps on the target cell, and starts to send the SYNCH-UL sequence in the UpPTS on cell 2, 3 for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on the target cell.

The reselection delays shall meet the requirements in table 4.3.4.4.5-4.

Table 4.3.4.4.5-4: Test requirements for E-UTRA TDD to UTRA TDD inter-RAT cell reselection

| Time phase | Target cell | Requirement for reselection delay (seconds) |
| --- | --- | --- |
| T1 | Cell 2 (normal performance group) | 58.9 |
| T3 | Cell 3 (reduced performance group) | 462.1 |

The rate of correct cell reselections observed during repeated tests shall be at least 90%, with a successful reselection counted if it is within the required time regardless of the carrier frequencies involved. At least 90% of reselections to the reduced performance group shall be within the required time, and at least 90% of reselections to the normal performance group shall be within the required time with a confidence level of 95%..

NOTE:	The cell re-selection delay to a normal performance group cell can be expressed as: NUTRA_carrier_TDD,normal *TevaluateUTRA_TDD + TSI_UTRA, and to a reduced performance group cell can be expressed as: 6 * NUTRA_carrier_TDD,reduced * TevaluateUTRA_TDD + TSI_UTRA,

Where:

TevaluateUTRA_TDD	19.2s, See Table 4.2.2.5.2-1

TSI_UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 3 * 19.2 + 1.28 = 58.88 s for normal performance group reselection and 6 * 4 * 19.2 + 1.28 = 462.08s for reduced performance group reselection, allow 58.9s for normal performance group and 462.1s for reduced performance group in the test case.

Since only one E-UTRA frequency is configured and signal level of UTRA cell is lower than threshold of Sprioritysearch, the UE shall select back to cell 1 (E-UTRA cell) within Kcarrier * TevaluateEUTRA + TSI = 19.2 + 1.28 = 20.48s.