3GPP TSG-RAN5 Meeting #97	R5-226613

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0486 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to annexure A.25 for Video conference creation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following the changes of R5-223463 (based on response LS C1-213557), in conferencing scenarios where resources are already reserved during dialog creation, a UE can confirm resource reservation in either the PRACK for 183 Session Progress, or an additional subsequent UPDATE request. However conference creation sequence A.25 is based on sequence A.15.1 which mandates SDP content in UPDATE only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Steps of A.15.1 are brought into the conference creation sequence A.25 and modified to allow PRACK to include SDP content and made the following UPDATE steps conditionally optional. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant device may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CR R5s220849 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# A.25	Video Conference Creation / 5GS

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | UE sends INVITE with the first SDP offer. |
| 2 |  |  | 100 Trying | SS sends a 100 Trying provisional response. |
| 3 |  |  | 183 Session Progress | SS sends an SDP answer. |
| 4 |  |  | PRACK | UE acknowledges reception of 183 Session Progress., with optional SDP offer |
| 5 |  |  | 200 OK | SS responds to PRACK. |
|  |  |  | Exception : Step 6-7 describe optional behaviour depending on UE implementation. The steps will be executed if no SDP offer was included in PRACK at step 4 |  |
| 6 |  |  | UPDATE | UE sends a second SDP offer in an UPDATE request. |
| 7 |  |  | 200 OK | SS responds to UPDATE. |
| 8 | <- |  | 200 OK | The SS responds INVITE with 200 OK and gives the final conference URI within the response |
| 9 | -> |  | ACK | The UE acknowledges the receipt of 200 OK for INVITE |
|  |  |  | EXCEPTION: steps 10–13 describe optional behaviour depending on UE configuration. The SS shall wait up to 3s for the SUBSCRIBE of step 10 |  |
| 10 | -> |  | SUBSCRIBE | UE subscribes the conference event |
| 11 | <- |  | 200 OK | SS responds to the subscription |
| 12 | <- |  | NOTIFY | SS sends the initial state of the conference event to the UE |
| 13 | -> |  | 200 OK | UE responds to the NOTIFY |

Specific Message Contents

The specific message contents for steps 1–7 is otherwise identical to what have been specified in Annex A.15.1, but with the exceptions as below:

INVITE (Step 1)

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | sip:mmtel@conf-factory appended with px_IMS_HomeDomainName |
| To |  |
| addr-spec | sip:mmtel@conf-factory appended with px_IMS_HomeDomainName |

183 Session in Progress for INVITE (Step 3)

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | sip:temporary@conf-factory. appended with px_IMS_HomeDomainName |
| feature-param | isfocus |
| Record-Route |  |
| rec-route | <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: protected server port of SS;lr> |

PRACK (Step 4)

| Header/param | Value/remark |
| --- | --- |
| Require | (present, if Message-Body is present) |
| option-tag | precondition |
| Message-body | (if present, shall be the same as message body in UPDATE at step 6 of Annex A.15.1) |

200 OK for PRACK (Step 5)

| Header/param | Value/remark |
| --- | --- |
| Require | (present, if Message-Body is present) |
| option-tag | precondition |
| Content-Type |  |
| media-type | (present, if SDP message-Body was present in PRACK at step 4) |
|  | application/sdp |
| Content-Length | (present if SDP message-Body was present in PRACK at step 4) |
| value | length of message body |
| Message-body | (present, if SDP message-Body was present in PRACK at step 4) same as message body in 200 OK for UPDATE at step 7 of Annex A.15.1 |

200 OK for INVITE (Step 8)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in Annex A.3.1 of TS 34.229-1 [2], applying conditions A1 and A5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Record-Route |  |
| rec-route | Same value as in the 183 response |
| Contact |  |
| addr-spec | sip:final@conf-factory. appended with px_IMS_HomeDomainName |
| feature-param | Isfocus |

ACK (Step 9)

Use the default message “ACK” in Annex A.2.7 of TS 34.229-1 [2], applying conditions A1 and A3, with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | sip:final@conf-factory. appended with px_IMS_HomeDomainName |

SUBSCRIBE (Step 10)

Use the default message “SUBSCRIBE for conference event package” in Annex A.5.1 of TS 34.229-1 [2], applying conditions A1 and A7.

200 OK (Step 11)

Use the default message “200 OK for SUBSCRIBE” in Annex A.5.2 of TS 34.229-1 [2], applying condition A1.

NOTIFY (Step 12)

Use the default message “MT NOTIFY for conference event package” in Annex A.5.3 of TS 34.229-1 [2], applying condition A5.

200 OK (Step 13)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in Annex A.3.1 of TS 34.229-1 [2], applying conditions A5 and A22.